3GPP TSG-RAN WG4 Meeting # 100-e	R4-2113887

Electronic Meeting, Aug. 16-27, 2021

Source:	ZTE Corporation

Title:	On RLM and RLF relaxation for UE power saving

Agenda item:	9.14.2

Document for:	Discussion

# 1	Introduction

In RAN4 98-e, work plan and general aspects for the new work item UE power saving enhancement were discussed. In RAN4 98-bis-e and RAN4 99-e, discussions were more focused on technical issues. Technical discussions related to RLM and RLF were also triggered during the meeting and companies’ views were summarized in WF [1], the content copied below:

- This paper discusses the open issues listed above in the WF and provide our view.

2	Discussions

We give our views on the above issues one by one.

As to the general principle when defining relaxation cases, we should keep in mind that the basic functions are more critical than enhancements, which means negative system level impact due to RLM/BFD relaxation should be minimized. Power saving on the UE side is for sure desirable, but not at a cost of degraded system performance.

Negative system level impact due to RLM/BFD relaxation should be minimized.

Issue 2-2-2: Good serving cell quality criteria for RLM/BFD: predefined or configured threshold

As to whether the thresholds for RLM/BFD are pre-defined or configured, our opinion is that this shall be configured by the network. There are many application scenarios and many different use cases for R17 NR networks, for instance for vehicular communications, SL communications and so on. Thus, a pre-defined threshold doesn’t give enough flexibility.

Our thinking is that the threshold shall be configured by the network to the UE.

The thresholds are configured to the UE by the network.

Issue 2-2-5/2-2-6: Low mobility criteria of RLM/BFD relaxation

The current agreement is that the UE verifies whether the low mobility criterion is fulfilled or not based on the RSRP variation and/or SINR variation, provided that the variation thresholds are configured by the NW. Our proposal is that the low mobility criterion cannot be represented by a single variable such as RSRP or SINR, so a combined metric is best.

Use RSRP variation and SINR variation as the variation thresholds for low mobility criterion.

Issue 2-3-2: Exiting criteria of RLM relaxation – Additional

Here we hold a similar position as last meeting, that the UE shall exit relaxation mode when the radio link quality of the serving cell is worse than a certain threshold, which is higher than Qout. The threshold can be configured by network with margin, or can be FFS.

The UE shall exit relaxation mode when the radio link quality of the serving cell is worse than a certain threshold, which is higher than Qout. The threshold can be configured by network with margin.

3	Conclusion

Proposal 1: Negative system level impact due to RLM/BFD relaxation should be minimized.

- Proposal 2: The thresholds are configured to the UE by the network..

Proposal 3: Use RSRP variation and SINR variation as the variation thresholds for low mobility criterion.

Proposal 4: The UE shall exit relaxation mode when the radio link quality of the serving cell is worse than a certain threshold, which is higher than Qout. The threshold can be configured by network with margin.

References

- R4-2108351, WF on RLM/BFD relaxation for UE Power Saving enhancements, MediaTek Inc.