3GPP TSG-RAN WG4 Meeting #100-e	R4-2113942

# E-meeting, 16nd Aug, 2021 – 27th Aug, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.141-1 | CR | 0242 | rev |  | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Maintenance CR to TS 38.141-1: NR-U BS conformance testing requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_unlic-Perf |  |  |  |  |  | Date: |  |  | 2021-08-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - The following NOTE 2 for NR-U BS REFSENS requirement in section 7.2.5 is misleading, since FRC defined for NR-U BS REFSENS include from G-FR1-A1-12 to G-FR1-A1-19 instead of only G-FR1-A1-12 and G-FR1-A1-19. NOTE 2: PREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each single interlace of FRC G-FR1-A1-12 and G-FR1-A1-19, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. - In Table Table Table 7.3.5-2b and Table Table 7.3.5-2c NOTE, FRC should be placed as interlaced instead of consecutive. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update the NOTE 2 in Table 7.2.5-2a and Table 7.2.5-2b,Table 7.2.5-3a,Table 7.2.5-3b as following to aligned with dynamic range part: - This requirement shall be met for each interleaved application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. - In Table 7.3.5-2b: and Table 7.3.5-2c: NOTE, FRC consecutive is placed by interlaced. - Add one more sentence in Table Table 7.3.5-3b and Table Table 7.3.5-3c NOTE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NR-U BS conformance testing requirement is not defined correctly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.5, 7.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of change>

7.2.5	Test requirements

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in annex A.1 with parameters specified in table 7.2.5-1 for Wide Area BS, in table 7.2.5-2 for Medium Range BS and in table 7.2.5-3 for Local Area BS in any operating band except for band n46 and n96.

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in annex A.2 with parameters specified in table 7.2.5-2a for Medium Range BS and in table 7.3.5-3a for Local Area BS, for band n46.

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in annex A.2 with parameters specified in table 7.2.5-2b for Medium Range BS and in table 7.2.5-3b for Local Area BS, for band n96.

The reference sensitivity level requirements for NB-IoT are specified in clause 7.2.5 of TS 36.141 [24].

Table 7.2.5-1: NR Wide Area BS reference sensitivity levels

| BS channel | Sub-carrier | Reference | Reference sensitivity power level, PREFSENS (dBm) |  |  |
| --- | --- | --- | --- | --- | --- |
| bandwidth (MHz) | spacing (kHz) | measurement channel | f ≤ 3.0 GHz | 3.0 GHz < f ≤ 4.2 GHz | 4.2 GHz < f ≤ 6.0 GHz |
| 5, 10, 15 | 15 | G-FR1-A1-1 (Note 1) | -101 | -100.7 | -100.5 |
|  |  | G-FR1-A1-10 (Note 3) | -101 (Note 2) | -100.7 (Note 2) | -100.5 (Note 2) |
| 10, 15 | 30 | G-FR1-A1-2 (Note 1) | -101.1 | -100.8 | -100.6 |
| 10, 15 | 60 | G-FR1-A1-3 (Note 1) | -98.2 | -97.9 | -97.7 |
| 20, 25, 30, 40, | 15 | G-FR1-A1-4 (Note 1) | -94.6 | -94.3 | -94.1 |
| 50 |  | G-FR1-A1-11 (Note 4) | -94.6 (Note 2) | -94.3 (Note 2) | -94.1 (Note 2) |
| 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 30 | G-FR1-A1-5 (Note 1) | -94.9 | -94.6 | -94.4 |
| 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 60 | G-FR1-A1-6 (Note 1) | -95 | -94.7 | -94.5 |
| NOTE 1: PREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. NOTE 2: The requirements apply to BS that supports NB-IoT operation in NR in-band. NOTE 3: PREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for a single instance of G-FR1-A1-10 mapped to the 24 NR resource blocks adjacent to the NB-IoT PRB, and for each consecutive application of a single instance of G-FR1-A1-1 mapped to disjoint frequency ranges with a width of 25 resource blocks each. NOTE 4: PREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for a single instance of G-FR1-A1-11 mapped to the 105 NR resource blocks adjacent to the NB-IoT PRB, and for each consecutive application of a single instance of G-FR1-A1-4 mapped to disjoint frequency ranges with a width of 106 resource blocks each. |  |  |  |  |  |

Table 7.2.5-2: NR Medium Range BS reference sensitivity levels

| BS channel | Sub-carrier | Reference | Reference sensitivity power level, PREFSENS (dBm) |  |  |
| --- | --- | --- | --- | --- | --- |
| bandwidth (MHz) | spacing (kHz) | measurement channel (Note 5) | f ≤ 3.0 GHz | 3.0 GHz < f ≤ 4.2 GHz | 4.2 GHz < f ≤ 6.0 GHz |
| 5, 10, 15 | 15 | G-FR1-A1-1 (Note 1) | -96 | -95.7 | -95.5 |
|  |  | G-FR1-A1-10 (Note 3) | -96 (Note 2) | -95.7 (Note 2) | -95.5 (Note 2) |
| 10, 15 | 30 | G-FR1-A1-2 (Note 1) | -96.1 | -95.8 | -95.6 |
| 10, 15 | 60 | G-FR1-A1-3 (Note 1) | -93.2 | -92.9 | -92.7 |
| 20, 25, 30, 40, | 15 | G-FR1-A1-4 (Note 1) | -89.6 | -89.3 | -89.1 |
| 50 |  | G-FR1-A1-11 (Note 4) | -89.6 (Note 2) | -89.3 (Note 2) | -89.1 (Note 2) |
| 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 30 | G-FR1-A1-5 (Note 1) | -89.9 | -89.6 | -89.4 |
| 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 60 | G-FR1-A1-6 (Note 1) | -90 | -89.7 | -89.5 |
| NOTE 1: PREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. NOTE 2: The requirements apply to BS that supports NB-IoT operation in NR in-band. NOTE 3: PREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for a single instance of G-FR1-A1-10 mapped to the 24 NR resource blocks adjacent to the NB-IoT PRB, and for each consecutive application of a single instance of G-FR1-A1-1 mapped to disjoint frequency ranges with a width of 25 resource blocks each. NOTE 4: PREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for a single instance of G-FR1-A1-11 mapped to the 105 NR resource blocks adjacent to the NB-IoT PRB, and for each consecutive application of a single instance of G-FR1-A1-4 mapped to disjoint frequency ranges with a width of 106 resource blocks each. NOTE 5: These reference measurement channels are not applied for band n46 and n96. |  |  |  |  |  |

Table 7.2.5-2a: NR Medium Range BS reference sensitivity levels for band n46

| BS channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel | Reference sensitivity power level, PREFSENS (dBm) |
| --- | --- | --- | --- |
| 10 | 15 | G-FR1-A1-12 (NOTE 2) | -101.5 |
|  | 30 | G-FR1-A1-13 (NOTE 2) | -99.2 |
|  | 60 | G-FR1-A1-3 (NOTE 1) | -92.4 |
| 20 | 15 | G-FR1-A1-14 (NOTE 2) | -98.6 |
|  | 30 | G-FR1-A1-15 (NOTE 2) | -95.6 |
|  | 60 | G-FR1-A1-6 (NOTE 1) | -89.2 |
| 40 | 15 | G-FR1-A1-16 (NOTE 2) | -95.5 |
|  | 30 | G-FR1-A1-17 (NOTE 2) | -92.5 |
|  | 60 | G-FR1-A1-6 (NOTE 1) | -89.2 |
| 60 | 30 | G-FR1-A1-18 (NOTE 2) | -90.9 |
|  | 60 | G-FR1-A1-6 (NOTE 1) | -89.2 |
| 80 | 30 | G-FR1-A1-19 (NOTE 2) | -89.6 |
|  | 60 | G-FR1-A1-6 (NOTE 1) | -89.2 |
| NOTE 1: PREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. NOTE 2: PREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each interleaved application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. |  |  |  |

Table 7.2.5-2b: NR Medium Range BS reference sensitivity levels for band n96

| BS channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel | Reference sensitivity power level, PREFSENS (dBm) |
| --- | --- | --- | --- |
| 20 | 15 | G-FR1-A1-14 (NOTE 2) | -97.6 |
|  | 30 | G-FR1-A1-15 (NOTE 2) | -94.6 |
|  | 60 | G-FR1-A1-6 (NOTE 1) | -88.2 |
| 40 | 15 | G-FR1-A1-16 (NOTE 2) | -94.5 |
|  | 30 | G-FR1-A1-17 (NOTE 2) | -91.5 |
|  | 60 | G-FR1-A1-6 (NOTE 1) | -88.2 |
| 60 | 30 | G-FR1-A1-18 (NOTE 2) | -89.9 |
|  | 60 | G-FR1-A1-6 (NOTE 1) | -88.2 |
| 80 | 30 | G-FR1-A1-19 (NOTE 2) | -88.6 |
|  | 60 | G-FR1-A1-6 (NOTE 1) | -88.2 |
| NOTE 1: PREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. NOTE 2: PREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each interleaved application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. |  |  |  |

Table 7.2.5-3: NR Local Area BS reference sensitivity levels

| BS channel | Sub-carrier | Reference | Reference sensitivity power level, PREFSENS (dBm) |  |  |
| --- | --- | --- | --- | --- | --- |
| bandwidth (MHz) | spacing (kHz) | measurement channel (Note 5) | f ≤ 3.0 GHz | 3.0 GHz < f ≤ 4.2 GHz | 4.2 GHz < f ≤ 6.0 GHz |
| 5, 10, 15 | 15 | G-FR1-A1-1 (Note 1) | -93 | -92.7 | -92.5 |
|  |  | G-FR1-A1-10 (Note 3) | -93 (Note 2) | -92.7 (Note 2) | -92.5 (Note 2) |
| 10, 15 | 30 | G-FR1-A1-2 (Note 1) | -93.1 | -92.8 | -92.6 |
| 10, 15 | 60 | G-FR1-A1-3 (Note 1) | -90.2 | -89.9 | -89.7 |
| 20, 25, 30, 40, | 15 | G-FR1-A1-4 (Note 1) | -86.6 | -86.3 | -86.1 |
| 50 |  | G-FR1-A1-11 (Note 4) | -86.6 (Note 2) | -86.3 (Note 2) | -86.1 (Note 2) |
| 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 30 | G-FR1-A1-5 (Note 1) | -86.9 | -86.6 | -86.4 |
| 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 60 | G-FR1-A1-6 (Note 1) | -87 | -86.7 | -86.5 |
| NOTE 5: These reference measurement channels are not applied for band n46 and n96. |  |  |  |  |  |

Table 7.2.5-3a: NR Local Area BS reference sensitivity levels for band n46

| BS channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel | Reference sensitivity power level, PREFSENS (dBm) |
| --- | --- | --- | --- |
| 10 | 15 | G-FR1-A1-12 (NOTE 2) | -98.5 |
|  | 30 | G-FR1-A1-13 (NOTE 2) | -96.2 |
|  | 60 | G-FR1-A1-3 (NOTE 1) | -89.4 |
| 20 | 15 | G-FR1-A1-14 (NOTE 2) | -95.6 |
|  | 30 | G-FR1-A1-15 (NOTE 2) | -92.6 |
|  | 60 | G-FR1-A1-6 (NOTE 1) | -86.2 |
| 40 | 15 | G-FR1-A1-16 (NOTE 2) | -92.5 |
|  | 30 | G-FR1-A1-17 (NOTE 2) | -89.5 |
|  | 60 | G-FR1-A1-6 (NOTE 1) | -86.2 |
| 60 | 30 | G-FR1-A1-18 (NOTE 2) | -87.9 |
|  | 60 | G-FR1-A1-6 (NOTE 1) | -86.2 |
| 80 | 30 | G-FR1-A1-19 (NOTE 2) | -86.6 |
|  | 60 | G-FR1-A1-6 (NOTE 1) | -86.2 |
| NOTE 1: PREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. NOTE 2: PREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each interleaved application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. |  |  |  |

Table 7.2.5-3b: NR Local Area BS reference sensitivity levels for band n96

| BS channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel | Reference sensitivity power level, PREFSENS (dBm) |
| --- | --- | --- | --- |
| 20 | 15 | G-FR1-A1-14 (NOTE 2) | -94.6 |
|  | 30 | G-FR1-A1-15 (NOTE 2) | -91.6 |
|  | 60 | G-FR1-A1-6 (NOTE 1) | -85.2 |
| 40 | 15 | G-FR1-A1-16 (NOTE 2) | -91.5 |
|  | 30 | G-FR1-A1-17 (NOTE 2) | -88.5 |
|  | 60 | G-FR1-A1-6 (NOTE 1) | -85.2 |
| 60 | 30 | G-FR1-A1-18 (NOTE 2) | -86.9 |
|  | 60 | G-FR1-A1-6 (NOTE 1) | -85.2 |
| 80 | 30 | G-FR1-A1-19 (NOTE 2) | -85.6 |
|  | 60 | G-FR1-A1-6 (NOTE 1) | -85.2 |
| NOTE 1: PREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. NOTE 2: PREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each interleaved application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. |  |  |  |

<Next of change>

7.3.5	Test requirements

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in annex A.2 with parameters specified in table 7.3.2-1 for Wide Area BS, in table 7.3.2-2 for Medium Range BS and in table 7.3.2-3 for Local Area BS in any operating band except for band n46 and n96.

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in annex A.2 with parameters specified in table 7.3.5-2b for Medium Range BS and in table 7.3.5-3b for Local Area BS, for band n46.

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in annex A.2 with parameters specified in table 7.3.5-2c for Medium Range BS and in table 7.3.5-3c for Local Area BS, for band n96.

For NB-IoT operation in NR in-band, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in Annex A of TS 36.141 [24] with parameters specified in table 7.3.5-1a for Wide Area BS, in table 7.3.5-2a for Medium Range BS and in table 7.3.5-3a for Local Area BS.

Table 7.3.5-1: Wide Area BS dynamic range

| BS channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) / BWConfig | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| 5 | 15 | G-FR1-A2-1 | -70.4 | -82.5 | AWGN |
|  | 30 | G-FR1-A2-2 | -71.1 |  |  |
| 10 | 15 | G-FR1-A2-1 | -70.4 | -79.3 | AWGN |
|  | 30 | G-FR1-A2-2 | -71.1 |  |  |
|  | 60 | G-FR1-A2-3 | -68.1 |  |  |
| 15 | 15 | G-FR1-A2-1 | -70.4 | -77.5 | AWGN |
|  | 30 | G-FR1-A2-2 | -71.1 |  |  |
|  | 60 | G-FR1-A2-3 | -68.1 |  |  |
| 20 | 15 | G-FR1-A2-4 | -64.2 | -76.2 | AWGN |
|  | 30 | G-FR1-A2-5 | -64.2 |  |  |
|  | 60 | G-FR1-A2-6 | -64.5 |  |  |
| 25 | 15 | G-FR1-A2-4 | -64.2 | -75.2 | AWGN |
|  | 30 | G-FR1-A2-5 | -64.2 |  |  |
|  | 60 | G-FR1-A2-6 | -64.5 |  |  |
| 30 | 15 | G-FR1-A2-4 | -64.2 | -74.4 | AWGN |
|  | 30 | G-FR1-A2-5 | -64.2 |  |  |
|  | 60 | G-FR1-A2-6 | -64.5 |  |  |
| 40 | 15 | G-FR1-A2-4 | -64.2 | -73.1 | AWGN |
|  | 30 | G-FR1-A2-5 | -64.2 |  |  |
|  | 60 | G-FR1-A2-6 | -64.5 |  |  |
| 50 | 15 | G-FR1-A2-4 | -64.2 | -72.1 | AWGN |
|  | 30 | G-FR1-A2-5 | -64.2 |  |  |
|  | 60 | G-FR1-A2-6 | -64.5 |  |  |
| 60 | 30 | G-FR1-A2-5 | -64.2 | -71.3 | AWGN |
|  | 60 | G-FR1-A2-6 | -64.5 |  |  |
| 70 | 30 | G-FR1-A2-5 | -64.2 | -70.7 | AWGN |
|  | 60 | G-FR1-A2-6 | -64.5 |  |  |
| 80 | 30 | G-FR1-A2-5 | -64.2 | -70.1 | AWGN |
|  | 60 | G-FR1-A2-6 | -64.5 |  |  |
| 90 | 30 | G-FR1-A2-5 | -64.2 | -69.5 | AWGN |
|  | 60 | G-FR1-A2-6 | -64.5 |  |  |
| 100 | 30 | G-FR1-A2-5 | -64.2 | -69.1 | AWGN |
|  | 60 | G-FR1-A2-6 | -64.5 |  |  |
| NOTE: The wanted signal mean power is the power level of a single instance of the corresponding reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. |  |  |  |  |  |

Table 7.3.5-1a: Wide Area BS dynamic range for NB-IoT operation in NR in-band

| BS channel bandwidth (MHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) / BWConfig | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 5 |  |  | -82.5 |  |
| 10 |  |  | -79.3 |  |
| 15 |  |  | -77.5 |  |
| 20 | FRC A15-1 in |  | -76.2 |  |
| 25 | Annex A.15 in | -99.4 | -75.2 | AWGN |
| 30 | TS 36.141 [24] |  | -74.4 |  |
| 40 |  |  | -73.1 |  |
| 50 |  |  | -72.1 |  |
| 5 |  |  | -82.5 |  |
| 10 |  |  | -79.3 |  |
| 15 | FRC A15-2 in |  | -77.5 |  |
| 20 | Annex A.15 in | -105.3 | -76.2 | AWGN |
| 25 | TS 36.141 [24] |  | -75.2 |  |
| 30 |  |  | -74.4 |  |
| 40 |  |  | -73.1 |  |
| 50 |  |  | -72.1 |  |

Table 7.3.5-2: Medium Range BS dynamic range

| BS channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel (Note 2) | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) / BWConfig | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| 5 | 15 | G-FR1-A2-1 | -65.4 | -77.5 | AWGN |
|  | 30 | G-FR1-A2-2 | -66.1 |  |  |
| 10 | 15 | G-FR1-A2-1 | -65.4 | -74.3 | AWGN |
|  | 30 | G-FR1-A2-2 | -66.1 |  |  |
|  | 60 | G-FR1-A2-3 | -63.1 |  |  |
| 15 | 15 | G-FR1-A2-1 | -65.4 | -72.5 | AWGN |
|  | 30 | G-FR1-A2-2 | -66.1 |  |  |
|  | 60 | G-FR1-A2-3 | -63.1 |  |  |
| 20 | 15 | G-FR1-A2-4 | -59.2 | -71.2 | AWGN |
|  | 30 | G-FR1-A2-5 | -59.2 |  |  |
|  | 60 | G-FR1-A2-6 | -59.5 |  |  |
| 25 | 15 | G-FR1-A2-4 | -59.2 | -70.2 | AWGN |
|  | 30 | G-FR1-A2-5 | -59.2 |  |  |
|  | 60 | G-FR1-A2-6 | -59.5 |  |  |
| 30 | 15 | G-FR1-A2-4 | -59.2 | -69.4 | AWGN |
|  | 30 | G-FR1-A2-5 | -59.2 |  |  |
|  | 60 | G-FR1-A2-6 | -59.5 |  |  |
| 40 | 15 | G-FR1-A2-4 | -59.2 | -68.1 | AWGN |
|  | 30 | G-FR1-A2-5 | -59.2 |  |  |
|  | 60 | G-FR1-A2-6 | -59.5 |  |  |
| 50 | 15 | G-FR1-A2-4 | -59.2 | -67.1 | AWGN |
|  | 30 | G-FR1-A2-5 | 59.8 |  |  |
|  | 60 | G-FR1-A2-6 | -59.5 |  |  |
| 60 | 30 | G-FR1-A2-5 | -59.2 | -66.3 | AWGN |
|  | 60 | G-FR1-A2-6 | -59.5 |  |  |
| 70 | 30 | G-FR1-A2-5 | -59.2 | -65.7 | AWGN |
|  | 60 | G-FR1-A2-6 | -59.5 |  |  |
| 80 | 30 | G-FR1-A2-5 | -59.2 | -65.1 | AWGN |
|  | 60 | G-FR1-A2-6 | -59.5 |  |  |
| 90 | 30 | G-FR1-A2-5 | -59.2 | -64.5 | AWGN |
|  | 60 | G-FR1-A2-6 | -59.5 |  |  |
| 100 | 30 | G-FR1-A2-5 | -59.2 | -64.1 | AWGN |
|  | 60 | G-FR1-A2-6 | -59.5 |  |  |
| NOTE 1: The wanted signal mean power is the power level of a single instance of the corresponding reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. NOTE 2: These reference measurement channels are not applied for band n46 and n96. |  |  |  |  |  |

Table 7.3.5-2a: Medium Range BS dynamic range for NB-IoT operation in NR in-band

| BS channel bandwidth (MHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) / BWConfig | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 5 |  |  | -77.5 |  |
| 10 |  |  | -74.3 |  |
| 15 |  |  | -72.5 |  |
| 20 | FRC A15-1 in |  | -71.2 |  |
| 25 | Annex A.15 in | -94.4 | -70.2 | AWGN |
| 30 | TS 36.141 [24] |  | -69.4 |  |
| 40 |  |  | -68.1 |  |
| 50 |  |  | -67.1 |  |
| 5 |  |  | -77.5 |  |
| 10 |  |  | -74.3 |  |
| 15 | FRC A15-2 in |  | -72.5 |  |
| 20 | Annex A.15 in | -100.3 | -71.2 | AWGN |
| 25 | TS 36.141 [24] |  | -70.2 |  |
| 30 |  |  | -69.4 |  |
| 40 |  |  | -68.1 |  |
| 50 |  |  | -67.1 |  |

Table 7.3.5-2b: Medium Range BS dynamic range for band n46

| BS channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) / BWConfig | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| 10 | 15 | G-FR1-A2-7 | -72.5 | -74.3 | AWGN |
|  | 30 | G-FR1-A2-8 | -70.3 |  |  |
|  | 60 | G-FR1-A2-3 | -63.1 |  |  |
| 20 | 15 | G-FR1-A2-9 | -69.5 | -71.2 | AWGN |
|  | 30 | G-FR1-A2-10 | -66.5 |  |  |
|  | 60 | G-FR1-A2-6 | -59.5 |  |  |
| 40 | 15 | G-FR1-A2-11 | -66.4 | -68.1 | AWGN |
|  | 30 | G-FR1-A2-12 | -63.4 |  |  |
|  | 60 | G-FR1-A2-6 | -59.5 |  |  |
| 60 | 30 | G-FR1-A2-13 | -61.6 | -66.3 | AWGN |
|  | 60 | G-FR1-A2-6 | -59.5 |  |  |
| 80 | 30 | G-FR1-A2-14 | -60.4 | -65.1 | AWGN |
|  | 60 | G-FR1-A2-6 | -59.5 |  |  |
| NOTE: The wanted signal mean power is the power level of a single instance of the corresponding reference measurement channel. This requirement shall be met for each interleaved application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. |  |  |  |  |  |

Table 7.3.5-2c: Medium Range BS dynamic range for band n96

| BS channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) / BWConfig | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| 20 | 15 | G-FR1-A2-9 | -68.5 | -70.2 | AWGN |
|  | 30 | G-FR1-A2-10 | -65.5 |  |  |
|  | 60 | G-FR1-A2-6 | -58.5 |  |  |
| 40 | 15 | G-FR1-A2-11 | -65.4 | -67.1 | AWGN |
|  | 30 | G-FR1-A2-12 | -62.4 |  |  |
|  | 60 | G-FR1-A2-6 | -58.5 |  |  |
| 60 | 30 | G-FR1-A2-13 | -60.6 | -65.3 | AWGN |
|  | 60 | G-FR1-A2-6 | -58.5 |  |  |
| 80 | 30 | G-FR1-A2-14 | -59.4 | -64.1 | AWGN |
|  | 60 | G-FR1-A2-6 | -58.5 |  |  |
| NOTE: The wanted signal mean power is the power level of a single instance of the corresponding reference measurement channel. This requirement shall be met for each interleaved application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. |  |  |  |  |  |

Table 7.3.5-3: Local Area BS dynamic range

| BS channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel (Note 2) | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) / BWConfig | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| 5 | 15 | G-FR1-A2-1 | -62.4 | -74.5 | AWGN |
|  | 30 | G-FR1-A2-2 | -63.1 |  |  |
| 10 | 15 | G-FR1-A2-1 | -62.4 | -71.3 | AWGN |
|  | 30 | G-FR1-A2-2 | -63.1 |  |  |
|  | 60 | G-FR1-A2-3 | -60.1 |  |  |
| 15 | 15 | G-FR1-A2-1 | -62.4 | -69.5 | AWGN |
|  | 30 | G-FR1-A2-2 | -63.1 |  |  |
|  | 60 | G-FR1-A2-3 | -60.1 |  |  |
| 20 | 15 | G-FR1-A2-4 | -56.2 | -68.2 | AWGN |
|  | 30 | G-FR1-A2-5 | -56.2 |  |  |
|  | 60 | G-FR1-A2-6 | -56.5 |  |  |
| 25 | 15 | G-FR1-A2-4 | -56.2 | -67.2 | AWGN |
|  | 30 | G-FR1-A2-5 | -56.2 |  |  |
|  | 60 | G-FR1-A2-6 | -56.5 |  |  |
| 30 | 15 | G-FR1-A2-4 | -56.2 | -66.4 | AWGN |
|  | 30 | G-FR1-A2-5 | -56.2 |  |  |
|  | 60 | G-FR1-A2-6 | -56.5 |  |  |
| 40 | 15 | G-FR1-A2-4 | -56.2 | -65.1 | AWGN |
|  | 30 | G-FR1-A2-5 | -56.2 |  |  |
|  | 60 | G-FR1-A2-6 | -56.5 |  |  |
| 50 | 15 | G-FR1-A2-4 | -56.2 | -64.1 | AWGN |
|  | 30 | G-FR1-A2-5 | -56.2 |  |  |
|  | 60 | G-FR1-A2-6 | -56.5 |  |  |
| 60 | 30 | G-FR1-A2-5 | -56.2 | -63.3 | AWGN |
|  | 60 | G-FR1-A2-6 | -56.5 |  |  |
| 70 | 30 | G-FR1-A2-5 | -56.2 | -62.7 | AWGN |
|  | 60 | G-FR1-A2-6 | -56.5 |  |  |
| 80 | 30 | G-FR1-A2-5 | -56.2 | -62.1 | AWGN |
|  | 60 | G-FR1-A2-6 | -56.5 |  |  |
| 90 | 30 | G-FR1-A2-5 | -56.2 | -61.5 | AWGN |
|  | 60 | G-FR1-A2-6 | -56.5 |  |  |
| 100 | 30 | G-FR1-A2-5 | -56.2 | -61.1 | AWGN |
|  | 60 | G-FR1-A2-6 | -56.5 |  |  |
| NOTE 1: The wanted signal mean power is the power level of a single instance of the corresponding reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. NOTE 2: These reference measurement channels are not applied for band n46 and n96. |  |  |  |  |  |

Table 7.3.5-3a: Local Area BS dynamic range for NB-IoT operation in NR in-band

| BS channel bandwidth (MHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) / BWConfig | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 5 |  |  | -74.5 |  |
| 10 |  |  | -71.3 |  |
| 15 | FRC A15-1 in |  | -69.5 |  |
| 20 | Annex A.15 in | -91.4 | -68.2 | AWGN |
| 25 | TS 36.141 [24] |  | -67.2 |  |
| 30 |  |  | -66.4 |  |
| 40 |  |  | -65.1 |  |
| 50 |  |  | -64.1 |  |
| 5 |  |  | -74.5 |  |
| 10 |  |  | -71.3 |  |
| 15 | FRC A15-2 in |  | -69.5 |  |
| 20 | Annex A.15 in | -97.3 | -68.2 | AWGN |
| 25 | TS 36.141 [24] |  | -67.2 |  |
| 30 |  |  | -66.4 |  |
| 40 |  |  | -65.1 |  |
| 50 |  |  | -64.1 |  |

Table 7.3.5-3b: Local Area BS dynamic range for band n46

| BS channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) / BWConfig | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| 10 | 15 | G-FR1-A2-7 | -69.5 | -71.3 | AWGN |
|  | 30 | G-FR1-A2-8 | -67.3 |  |  |
|  | 60 | G-FR1-A2-3 | -60.1 |  |  |
| 20 | 15 | G-FR1-A2-9 | -66.5 | -68.2 | AWGN |
|  | 30 | G-FR1-A2-19 | -63.5 |  |  |
|  | 60 | G-FR1-A2-6 | -56.5 |  |  |
| 40 | 15 | G-FR1-A2-11 | -63.4 | -65.1 | AWGN |
|  | 30 | G-FR1-A2-12 | -60.4 |  |  |
|  | 60 | G-FR1-A2-6 | -56.5 |  |  |
| 60 | 30 | G-FR1-A2-13 | -58.6 | -63.3 | AWGN |
|  | 60 | G-FR1-A2-6 | -56.5 |  |  |
| 80 | 30 | G-FR1-A2-14 | -57.4 | -62.1 | AWGN |
|  | 60 | G-FR1-A2-6 | -56.5 |  |  |
| NOTE: The wanted signal mean power is the power level of a single instance of the corresponding reference measurement channel. This requirement shall be met for each interleaved application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. |  |  |  |  |  |

Table 7.3.5-3c: Local area BS dynamic range for band n96

| BS channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) / BWConfig | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| 20 | 15 | G-FR1-A2-9 | -65.5 | -67.2 | AWGN |
|  | 30 | G-FR1-A2-19 | -62.5 |  |  |
|  | 60 | G-FR1-A2-6 | -55.5 |  |  |
| 40 | 15 | G-FR1-A2-11 | -62.4 | -64.1 | AWGN |
|  | 30 | G-FR1-A2-12 | -59.4 |  |  |
|  | 60 | G-FR1-A2-6 | -55.5 |  |  |
| 60 | 30 | G-FR1-A2-13 | -57.6 | -62.3 | AWGN |
|  | 60 | G-FR1-A2-6 | -55.5 |  |  |
| 80 | 30 | G-FR1-A2-14 | -56.4 | -61.1 | AWGN |
|  | 60 | G-FR1-A2-6 | -55.5 |  |  |
| NOTE: The wanted signal mean power is the power level of a single instance of the corresponding reference measurement channel. This requirement shall be met for each interleaved application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. |  |  |  |  |  |

<End of change>