3GPP TSG-RAN WG4 Meeting # 100-e 	R4-2113972

Electronic Meeting, August 16th-27th, 2021

Agenda Item: 9.20.3.4

Source: MediaTek Inc.

Title: RRM measurements relaxation

Document for: Discussion and decision

# Introduction

In this contribution paper, the impact of the RRM measurement relaxation for reduced capability (RedCap) on the Rel-17 specification are discussed. The WI in [1] has the following work item:

- Specify support for the following RRM measurement relaxations for neighbouring cells for RedCap devices: for RRC_Idle/Inactive/Connected [RAN2, RAN4]: - Specify measurement (RSRP/RSRQ) based stationary criterion and not-at-cell-edge criterion [RAN2] - Enabling/disabling of RRM measurement relaxation should be under the network’s control. Specify both broadcast and dedicated signalling for enabling/disabling of RRM measurement relaxation. - Specify UE requirements for RRM measurement relaxation [RAN4] - No RRM measurement relaxations are specified for the serving cell.

Also, RAN2 has the following agreements [2]:

Agreements online: - Continue discussion on Rel-17 not-at-cell-edge criterion in RRC_IDLE/INACTIVE within two options: - Option 1) Reuse Rel-16 not-at-cell-edge criterion with the same thresholds (i.e., SSearchThresholdP / SSearchThresholdQ) - Option 2) Reuse Rel-16 not-at-cell-edge criterion with the different thresholds

# Discussion on the stationary and not-at-cell edge criteria

In this section, the RRM measurement relaxations are discussed for the stationary and not-at-cell edge criteria. As given in the WID, the RRM measurement relaxations are for both inactive/idle and connected modes, hence this section is divided into two main parts as follows:

# The stationary and not-at-cell edge criteria for inactive mode

The stationary and not-at-cell edge criteria are new criteria for RedCap UEs in 5G NR Rel-17. This new criteria allow the UEs to improve the power saving. Therefore, to achieve that, further DRX relaxation compared to the low mobility and not-at-cell edge criteria must be considered. Given that the relaxation factor used for the low mobility criterion and not-at-cell edge criterion is K1 = 3 [3], hence the new relaxation factor (J2) for the stationary criterion and not-at-cell edge criterion must be larger than 3 (i.e. J2 > K1). Nevertheless, RAN2 has not reached a conclusion on whether to reuse the not-at-cell edge criterion from release 16 or introduce a new Rel-17 not-at-cell-edge criterion. Therefore, the new relaxation factor (J2) shall be used for the stationary criterion until a consensus is reached on whether to introduce a new Rel-17 not-at-cell edge criterion. Once RAN2 has reached conclusion and if the conclusion is to reuse the Rel-16 not-at-cell edge then there is no need to introduce new DRX requirements for the not-at-cell edge criterion, however, if a new Rel-17 not-at-cell edge criterion is introduced then new DRX requirements are needed, where the latter shall use the new scaling factor (J2).

- Support a new relaxation factor to be used in the DRX cycles requirements for the stationary criterion in IDLE/INACTIVE mode. In addition, if a new Rel-17 not-at-cell edge criterion is introduced then the same relaxation factor of the stationary criterion shall be used for the RRM measurement relaxations of Rel-17 not-at-cell edge criterion.

Furthermore, the existing RRM measurements requirement for the scenario when both low mobility criterion and not-at-cell edge criterion are satisfied is that: the UE can stop measurements for up to an hour [3]. Also, given that the stationary and not-at-cell edge criteria should be further relaxed compared to low mobility and not-at-cell edge criteria, hence the period of time that the UE can stop monitoring neighbouring cell should be longer than an hour.

- Support extending the time period in which the UE can stop its measurements for cell reselection in IDLE/INACTIVE mode, when both stationary criterion and not-at-cell edge criterion are satisfied.

# The stationary and not-at-cell edge criteria for connected mode

There is no current agreements from RAN2 regarding connected mode, hence RAN4 can wait until RAN2 reach a consensus.

- RAN4 to wait for RAN2 until they reach to agreement on RRM relaxation in CONNECTED mode.

# Summary

In this contribution, the RRM measurement relaxation for RedCap are discussed and hence we have the following proposals:

Proposal 1: Support a new relaxation factor to be used in the DRX cycles requirements for the stationary criterion in IDLE/INACTIVE mode. In addition, if a new Rel-17 not-at-cell edge criterion is introduced then the same relaxation factor of the stationary criterion shall be used for the RRM measurement relaxations of Rel-17 not-at-cell edge criterion.

Proposal 2: Support extending the time period in which the UE can stop its measurements for cell reselection in IDLE/INACTIVE mode, when both stationary criterion and not-at-cell edge criterion are satisfied.

Proposal 3: RAN4 to wait for RAN2 until they reach to agreement on RRM relaxation in CONNECTED mode.

# References

- RP-211574, “Revised WID on support of reduced capability NR devices”.

- R2-2106472, “Report from Break-out session on R17 NTN and REDCAP”.

- 3GPP TS 38.133, “NR; Requirements for support of radio resource management (Release 17)”, V17.2.0.