# 3GPP TSG-RAN WG4 Meeting # 100-e	R4-2114108

# Electronic Meeting, August 16-27, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | Draft | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on TC of HO for NR-U R17 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_unlic-Perf |  |  |  |  |  | Date: |  |  | 2021-08-01 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - Dedicated CORESET RMC configuration is missing in A.11.2.1.1 to A.11.2.1.8 - RMSI CORESET RMC configuration is TBD in A.11.2.1.6 to A.11.2.1.8 - The DL CCA probability and UL CCA probability are not aligned with the lastest agreements. in A.11.2.1.4 to A.11.2.1.8 - The CCA model is not defined for LBE and FBE in A.11.2.1.5 - PRACH configuration is not defined for CCA operation - The note should be added to state that the UE is only required to be tested in one of the supported test configurations - CCA probability, PRACH configuration is not defined in A.12.2.1.1 - Noc configuration in A.12.2.1.1 is not correct - According to the agreements in RAN4#99e, the applicability rule should be added for HO from NR-U to E-UTRA - Typos need to be fixed |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add dedicated CORESET RMC configuration in A.11.2.1.1 to A.11.2.1.8 - Add RMSI CORESET RMC configuration in A.11.2.1.6 to A.11.2.1.8 - Correct DL CCA probability and UL CCA probability in A.11.2.1.4 to A.11.2.1.8 - Add CCA model in A.11.2.1.5 - Add PRACH configuration - Add CCA probability, PRACH configuration in A.12.2.1.1 - Correct Noc configuration in A.12.2.1.1. - Add note that the UE is only required to be tested in one of the supported test configurations - Add note in A.11.2.1.7 and A.11.2.1.8 about the applicability rule - Typos fixed |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test cases are wrong |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.11.2.1.1 to A.11.2.1.8 and A.12.2.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### <Start of Change 1>

### A.11.2.1	Handover

#### A.11.2.1.1	Intra-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA; known target cell

##### A.11.2.1.1.1	Test Purpose and Environment

This test is to verify the requirement for the NR intra frequency handover requirements from FR1 carrier under CCA to FR1 carrier under CCA specified in clause 6.1B.1.2.

##### A.11.2.1.1.2	Test Parameters

Supported test configurations are shown in table A.11.2.1.1.2-1. Both handover delay and interruption length are tested by using the parameters in table A.11.2.1.1.2-2, and A.11.2.1.1.2-3.

The test consists of three successive time periods, with time durations of T1, T2 and T3 respectively. At the start of time duration T1, the UE may not have any timing information of cell 2.

NR shall send a RRC message implying handover to cell 2. The RRC message implying handover shall be sent to the UE during period T2, after the UE has reported Event A3. T3 is defined as the end of the last TTI containing the RRC message implying handover.

Table A.11.2.1.1.2-1: Intra-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA test configurations

| Config | Description |
| --- | --- |
| 1 | Source cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode Target cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |

Table A.11.2.1.1.2-2: General test parameters Intra-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 | On the carrier under CCA |
|  | Neighbouring cell |  | Cell 2 | On the carrier under CCA |
| Final condition | Active cell |  | Cell 2 | On the carrier under CCA |
| DL CCA model | Dynamic channel accessNote 1, 3 |  | As specified in clause A.3.26.2.1 |  |
|  | Semi-static channel access Note 2, 3 |  |  |  |
| UL CCA model | Dynamic channel access Note 1, 3 |  | As specified in clause A.3.26.2.2 |  |
|  | Semi-static channel access Note 2,3 |  |  |  |
| A3-Offset |  | dB | 0 |  |
| Hysteresis |  | dB | 0 |  |
| Time To Trigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| Time offset between cells |  |  | 3 s | Synchronous cells |
| T304 |  | ms | 500 |  |
| LCCA_DL |  |  | 5 |  |
| WCCA_DL |  | ms | T304 |  |
| LCCA_UL |  |  | 5 |  |
| WCCA_UL |  | ms | T304 |  |
| T1 |  | s | 5 |  |
| T2 |  | s | 5 |  |
| T3 |  | s | ≥ Tinterrupt | Tinterrupt is defined in clause 6.1B.1.2 |
| NOTE 1: For a UE supporting dynamic channel access and network configuring dynamic channel occupancy. NOTE 2: For a UE supporting semi-static channel access and network configuring semi-static channel occupancy. NOTE 3: For a UE supporting both semi-static and dynamic channel access, the UE can be tested under dynamic channel occupancy only. |  |  |  |  |

Table A.11.2.1.1.2-3: Cell specific test parameters for NR FR1-FR1 Intra frequency handover test case

| Parameter |  |  | Unit | Cell 1 |  |  |  |  |  | Cell 2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 |  | T2 |  | T3 |  | T1 |  | T2 |  | T3 |
| NR RF Channel Number |  |  |  | 1 |  |  |  |  |  | 1 |  |  |  |  |
| PCCA_DL for dynamic channel access Note 4,6 |  |  | - | PCCA_DL_1=0.75 PCCA_DL_2=0.75 |  |  |  |  |  | PCCA_DL_1=0.75 PCCA_DL_2=0.75 |  |  |  |  |
| PCCA_DL for semi-static channel access Note 5,6 |  |  | - | PCCA_DL=0.9375 |  |  |  |  |  | PCCA_DL=0.9375 |  |  |  |  |
| PCCA_UL for dynamic channel access Note 4,6 |  |  | - | 0.75 |  |  |  |  |  | 0.75 |  |  |  |  |
| PCCA_UL for semi-static channel access Note 5,6 |  |  | - | 0.87 |  |  |  |  |  | 0.87 |  |  |  |  |
| TDD configuration |  | Config 1 |  | TDDConf.1.1 CCA |  |  |  |  |  |  |  |  |  |  |
| BWchannel |  | Config 1 |  | 40: NRB,c = 106 |  |  |  |  |  |  |  |  |  |  |
| BWP BW |  | Config 1 |  | 40: NRB,c = 106 |  |  |  |  |  |  |  |  |  |  |
| DRX Cycle |  |  | ms | Not Applicable |  |  |  |  |  |  |  |  |  |  |
| PDSCH Reference |  | Config 1 |  | SR.1.1 CCA |  |  |  |  |  |  |  |  |  |  |
| CORESET Reference Channel |  | Config 1 |  | CR.1.1 CCA |  |  |  |  |  |  |  |  |  |  |
| Dedicated CORESET RMC configuration |  | Config 1 |  | CCR.1.1 CCA |  |  |  |  |  |  |  |  |  |  |
| TRS configuration |  | Config 1 |  | TRS.1.1 TDD |  |  |  |  |  |  |  |  |  |  |
| OCNG Patterns |  |  |  | OP.1 |  |  |  |  |  |  |  |  |  |  |
| SMTC Configuration |  |  |  | SMTC.1 |  |  |  |  |  |  |  |  |  |  |
| DBT window configuration |  | Config 1 |  | DBT.1 |  |  |  |  |  |  |  |  |  |  |
| SSB configuration for semi-static channel accessNote 4, 6 |  | Config 1 |  | SSB.1 CCA |  |  |  |  |  |  |  |  |  |  |
| SSB configuration for dynamic channel accessNote 5, 6 |  | Config 1 |  | SSB.2 CCA |  |  |  |  |  |  |  |  |  |  |
| ssb-PositionQCL |  | Config 1 |  | [1] |  |  |  |  |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1 | kHz | 30 kHz |  |  |  |  |  |  |  |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  | Config 1 | kHz | 30 kHz |  |  |  |  |  |  |  |  |  |  |
| PRACH configuration |  |  |  | FR1 PRACH configuration 1 under CCA |  |  |  |  |  |  |  |  |  |  |
| BWP configuration |  | Initial DL BWP |  | DLBWP.0.1 |  |  |  |  |  |  |  |  |  |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |  |  |  |  |  |  |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |  |  |  |  |  |  |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | -98 |  |  |  |  |  |  |  |  |  |  |
| Note2 | Config 1 |  | dBm/SCS | -95 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | dB | 8 | -3.3 |  | -3.3 |  | -Infinity |  | 2.36 |  | 2.36 |  |
|  |  |  | dB | 8 | 8 |  | 8 |  | -Infinity |  | 11 |  | 11 |  |
| SSB_RP | Config 1 |  | dBm/SCS | -87 | -87 |  | -87 |  | -Infinity |  | -84 |  | -84 |  |
| IoNote3 | Config 1 |  | dBm/ 38.16MHz | -55.31 | -50.96 |  | -50.96 |  | -55.31 |  | -50.96 |  | -50.96 |  |
| Propagation condition |  |  | - | AWGN |  |  |  |  | AWGN |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 5: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. Note 6: For a UE supporting both semi-static and dynamic channel access, the UE can be tested under dynamic channel occupancy only. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

##### A.11.2.1.1.3 Test Requirements

The UE shall start to transmit the PRACH to Cell 2 less than Tinterrupt from the beginning of time period T3, where Tinterrupt is defined in clause 6.1B.1.2

The rate of correct handovers observed during repeated tests shall be at least 90%.

NOTE:	The handover delay can be expressed as: RRC procedure delay + Tinterrupt, where:

Tinterrupt = Tsearch + TIU + Tprocessing  + T∆ + Tmargin

Tsearch = 0.

Tprocessing = 20 ms.

Tmargin = 2 ms.

T∆ = (1+ L2) *20 ms.

TIU = (1+ L3)*10 + 10 ms

RRC procedure delay = 10 ms and is specified in clause 12 in TS 38.331 [2], L2 is the number of SMTC occasions not available at the UE during the time tracking period where L2  LCCA_DL, and L3 is the number of consecutive SSB to PRACH occasion association periods during which no PRACH occasion is available for PRACH transmission due to UL CCA failure, where L3  LCCA_UL. L3 = 0 for Type 2C UL channel access procedure as defined in TS 37.213 [33]. The interruption time considering the potential extensions caused by L1, L2 , L3  and by the UL CCA failure detection/recovery mechanism is limited by the T304 timer. The UE behaviour at the T304 timer expiry is detailed in TS 38.331 [2]. Test equipment should make sure that LCCA_DL and LCCA_UL are not exceeded during a test by monitoring the number of CCA failures and preventing additional CCA failures from happening after LCCA_DL or LCCA_UL is reached.

#### A.11.2.1.2	Intra-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA; unknown target cell

##### A.11.2.1.2.1	Test Purpose and Environment

This test is to verify the requirement intra frequency handover requirements from FR1 carrier under CCA to FR1 carrier under CCA specified in clause 6.1B.1.2.

##### A.11.2.1.2.2	Test Parameters

Supported test configurations are shown in table A.11.2.1.2.2-1. Both handover delay and interruption length are tested by using the parameters in table A.11.2.1.2.2-2, and A.11.2.1.2.2-3.

The test scenario comprises of two carriers and one cell on each carrier. No gap patterns are configured in the test case. The test consists of two successive time periods, with time durations of T1, T2 respectively. At the start of time duration T1, the UE does not have any timing information of cell 2. Starting T2, cell 2 becomes detectable and the UE receives a RRC handover command from the network. The start of T2 is the instant when the last TTI containing the RRC message implying handover is sent to the UE.

Table A.11.2.1.2.2-1: Intra-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA test configurations

| Config | Description |
| --- | --- |
| 1 | Source cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode Target cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |

Table A.11.2.1.2.2-2: General test parameters Intra-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 | On the carrier under CCA |
|  | Neighbouring cell |  | Cell 2 | On the carrier under CCA |
| Final condition | Active cell |  | Cell 2 | On the carrier under CCA |
| DL CCA model | Dynamic channel accessNote 1, 3 |  | As specified in clause A.3.26.2.1 |  |
|  | Semi-static channel access Note 2, 3 |  |  |  |
| UL CCA model | Dynamic channel access Note 1, 3 |  | As specified in clause A.3.26.2.2 |  |
|  | Semi-static channel access Note 2,3 |  |  |  |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| Time offset between cells |  |  | 3 s | Synchronous cells |
| T304 |  | ms | 500 |  |
| LCCA_DL |  |  | 5 |  |
| WCCA_DL |  | ms | T304 |  |
| LCCA_UL |  |  | 5 |  |
| WCCA_UL |  | ms | T304 |  |
| T1 |  | s | 5 |  |
| T2 |  | s | ≥ Tinterrupt | Tinterrupt is defined in clause 6.1B.1.2 |
| NOTE 1: For a UE supporting dynamic channel access and network configuring dynamic channel occupancy. NOTE 2: For a UE supporting semi-static channel access and network configuring semi-static channel occupancy. NOTE 3: For a UE supporting both semi-static and dynamic channel access, the UE can be tested under dynamic channel occupancy only. |  |  |  |  |

Table A.11.2.1.2.2-3: Cell specific test parameters for NR FR1-FR1 Intra frequency handover test case

| Parameter |  |  | Unit | Cell 1 |  |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 |  | T2 |  | T1 |  | T2 |
| NR RF Channel Number |  |  |  | 1 |  |  |  | 1 |  |  |
| PCCA_DL for dynamic channel access Note 4,6 |  |  | - | PCCA_DL_1=0.75 PCCA_DL_2=0.75 |  |  |  | PCCA_DL_1=0.75 PCCA_DL_2=0.75 |  |  |
| PCCA_DL for semi-static channel access Note 5,6 |  |  | - | PCCA_DL=0.9375 |  |  |  | PCCA_DL=0.9375 |  |  |
| PCCA_UL for dynamic channel access Note 4,6 |  |  | - | 0.75 |  |  |  | 0.75 |  |  |
| PCCA_UL for semi-static channel access Note 5,6 |  |  | - | 0.87 |  |  |  | 0.87 |  |  |
| TDD configuration |  | Config 1 |  | TDDConf.1.1 CCA |  |  |  |  |  |  |
| BWchannel |  | Config 1 |  | 40: NRB,c = 106 |  |  |  |  |  |  |
| BWP BW |  | Config 1 |  | 40: NRB,c = 106 |  |  |  |  |  |  |
| DRX Cycle |  |  | ms | Not Applicable |  |  |  |  |  |  |
| PDSCH Reference |  | Config 1 |  | SR.1.1 CCA |  |  |  |  |  |  |
| CORESET Reference Channel |  | Config 1 |  | CR.1.1 CCA |  |  |  |  |  |  |
| Dedicated CORESET RMC configuration |  | Config 1 |  | CCR.1.1 CCA |  |  |  |  |  |  |
| TRS configuration |  | Config 1 |  | TRS.1.2 TDD |  |  |  |  |  |  |
| OCNG Patterns |  |  |  | OP.1 |  |  |  |  |  |  |
| SMTC Configuration |  |  |  | SMTC.1 |  |  |  |  |  |  |
| DBT window configuration |  | Config 1 |  | DBT.1 |  |  |  |  |  |  |
| SSB configuration for semi-static channel access Note 4, 6 |  | Config 1 |  | SSB.1 CCA |  |  |  |  |  |  |
| SSB configuration for dynamic channel access Note 5, 6 |  | Config 1 |  | SSB.2 CCA |  |  |  |  |  |  |
| ssb-PositionQCL |  | Config 1 |  | [1] |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1 | kHz | 30 kHz |  |  |  |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  | Config 1 | kHz | 30 kHz |  |  |  |  |  |  |
| PRACH configuration |  |  |  | FR1 PRACH configuration 1 under CCA |  |  |  |  |  |  |
| BWP configuration |  | Initial DL BWP |  | DLBWP.0.1 |  |  |  |  |  |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |  |  |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |  |  |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | -98 |  |  |  |  |  |  |
| Note2 | Config 1 |  | dBm/SCS | -95 |  |  |  |  |  |  |
|  |  |  | dB | 8 | -0.64 |  | -Infinity |  | -0.64 |  |
|  |  |  | dB | 8 | 8 |  | -Infinity |  | 8 |  |
| SSB_RP | Config 1 |  | dBm/SCS | -87 | -87 |  | -Infinity |  | -87 |  |
| IoNote3 | Config 1 |  | dBm/ 38.16MHz | -55.31 | -52.60 |  | -55.31 |  | -52.60 |  |
| Propagation condition |  |  | - | AWGN |  |  | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 5: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. Note 6: For a UE supporting both semi-static and dynamic channel access, the UE can be tested under dynamic channel occupancy only. |  |  |  |  |  |  |  |  |  |  |

##### A.11.2.1.2.3	Test Requirements

The UE shall start to transmit the PRACH to Cell 2 less than Tinterrupt from the beginning of time period T3, where Tinterrupt is defined in clause 6.1B.1.2

The rate of correct handovers observed during repeated tests shall be at least 90%.

NOTE:	The handover delay can be expressed as: RRC procedure delay + Tinterrupt, where:

Tinterrupt = Tsearch + TIU + Tprocessing  + T∆ + Tmargin

Tsearch = (1+L1)* 20 ms.

Tprocessing = 20 ms.

Tmargin = 2 ms.

T∆ = (1+ L2) *20 ms.

TIU = (1+ L3)*10 + 10 ms

RRC procedure delay = 10 ms and is specified in clause 12 in TS 38.331 [2], L1 is the number of SMTC occasions not available at the UE during the intra-frequency detection period, L2 is the number of SMTC occasions not available at the UE during the time tracking period, where L1 + L2  LCCA_DL, and L3 is the number of consecutive SSB to PRACH occasion association periods during which no PRACH occasion is available for PRACH transmission due to UL CCA failure, where L3  LCCA_UL. L3 = 0 for Type 2C UL channel access procedure as defined in TS 37.213 [33]. The interruption time considering the potential extensions caused by L1, ,L2 , L3  and by the UL CCA failure detection/recovery mechanism is limited by the T304 timer.

#### A.11.2.1.3	Inter-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA; unknown target cell

##### A.11.2.1.3.1	Test Purpose and Environment

This test is to verify the requirement for inter frequency handover requirements from FR1 carrier under CCA to FR1 carrier under CCA specified in clause 6.1B.1.2.

##### A.11.2.1.3.2	Test Parameters

Supported test configurations are shown in table A.11.2.1.3.2-1. Both handover delay and interruption length are tested by using the parameters in table A.11.2.1.3.2-2, and A.11.2.1.3.2-3.

The test scenario comprises of two carriers and one cell on each carrier. No gap patterns are configured in the test case. The test consists of two successive time periods, with time durations of T1, T2 respectively. At the start of time duration T1, the UE does not have any timing information of cell 2. Starting T2, cell 2 becomes detectable and the UE receives a RRC handover command from the network. The start of T2 is the instant when the last TTI containing the RRC message implying handover is sent to the UE.

Table A.11.2.1.3.2-1: Inter-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA test configurations

| Config | Description |
| --- | --- |
| 1 | Source cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode Target cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |

Table A.11.2.1.3.2-2: General test parameters Inter-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 | On the carrier under CCA |
|  | Neighbouring cell |  | Cell 2 | On the carrier under CCA |
| Final condition | Active cell |  | Cell 2 | On the carrier under CCA |
| DL CCA model | Dynamic channel accessNote 1, 3 |  | As specified in clause A.3.26.2.1 |  |
|  | Semi-static channel access Note 2, 3 |  |  |  |
| UL CCA model | Dynamic channel access Note 1, 3 |  | As specified in clause A.3.26.2.2 |  |
|  | Semi-static channel access Note 2,3 |  |  |  |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| T304 |  | ms | 500 |  |
| LCCA_DL |  |  | 5 |  |
| WCCA_DL |  | ms | T304 |  |
| LCCA_UL |  |  | 5 |  |
| WCCA_UL |  | ms | T304 |  |
| T1 |  | s | 5 |  |
| T2 |  | s | Tinterrupt | Tinterrupt is defined in clause 6.1B.1.2 |
| NOTE 1: For a UE supporting dynamic channel access and network configuring dynamic channel occupancy. NOTE 2: For a UE supporting semi-static channel access and network configuring semi-static channel occupancy. NOTE 3: For a UE supporting both semi-static and dynamic channel access, the UE can be tested under dynamic channel occupancy only. |  |  |  |  |

Table A.11.2.1.3.2-3: Cell specific test parameters for NR FR1-FR1 Inter frequency handover test case

| Parameter |  |  | Unit | Cell 1 |  |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 |  | T2 |  | T1 |  | T2 |
| NR RF Channel Number |  |  |  | 1 |  |  |  | 2 |  |  |
| PCCA_DL for dynamic channel access Note 4,6 |  |  | - | PCCA_DL_1=0.75 PCCA_DL_2=0.75 |  |  |  | PCCA_DL_1=0.75 PCCA_DL_2=0.75 |  |  |
| PCCA_DL for semi-static channel access Note 5,6 |  |  | - | PCCA_DL=0.9375 |  |  |  | PCCA_DL=0.9375 |  |  |
| PCCA_UL for dynamic channel access Note 4,6 |  |  | - | 0.75 |  |  |  | 0.75 |  |  |
| PCCA_UL for semi-static channel access Note 5,6 |  |  | - | 0.87 |  |  |  | 0.87 |  |  |
| TDD configuration |  | Config 1 |  | TDDConf.1.1 CCA |  |  |  |  |  |  |
| BWchannel |  | Config 1 |  | 40: NRB,c = 106 |  |  |  |  |  |  |
| BWP BW |  | Config 1 |  | 40: NRB,c = 106 |  |  |  |  |  |  |
| DRX Cycle |  |  | ms | Not Applicable |  |  |  |  |  |  |
| PDSCH Reference |  | Config 1 |  | SR.1.1 CCA |  |  |  |  |  |  |
| CORESET Reference Channel |  | Config 1 |  | CR1.1 CCA |  |  |  |  |  |  |
| Dedicated CORESET RMC configuration |  | Config 1 |  | CCR.1.1 CCA |  |  |  |  |  |  |
| TRS configuration |  | Config 1 |  | TRS.1.2 TDD |  |  |  |  |  |  |
| OCNG Patterns |  |  |  | OP.1 |  |  |  |  |  |  |
| SMTC Configuration |  |  |  | SMTC.1 |  |  |  |  |  |  |
| DBT window configuration |  | Config 1 |  | DBT.1 |  |  |  |  |  |  |
| SSB configuration for semi-static channel access Note 4, 6 |  | Config 1 |  | SSB.1 CCA |  |  |  |  |  |  |
| SSB configuration for dynamic channel access Note 5, 6 |  | Config 1 |  | SSB.2 CCA |  |  |  |  |  |  |
| ssb-PositionQCL |  | Config 1 |  | [1] |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1 | kHz | 30 kHz |  |  |  |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  | Config 1 | kHz | 30 kHz |  |  |  |  |  |  |
| PRACH configuration |  |  |  | FR1 PRACH configuration 1 under CCA |  |  |  |  |  |  |
| BWP configuration |  | Initial DL BWP |  | DLBWP.0.1 |  |  |  |  |  |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |  |  |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |  |  |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | -98 |  |  |  |  |  |  |
| Note2 | Config 1 |  | dBm/SCS | -95 |  |  |  |  |  |  |
|  |  |  | dB | 4 | 4 |  | -Infinity |  | 5 |  |
|  |  |  | dB | 4 | 4 |  | -Infinity |  | 5 |  |
| SSB_RP | Config 1 |  | dBm/SCS | -91 | -91 |  | -Infinity |  | -90 |  |
| IoNote3 | Config 1 |  | dBm/ 38.16MHz | -58.49 | -58.49 |  | -63.94 |  | -57.75 |  |
| Propagation condition |  |  | - | AWGN |  |  | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 5: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. Note 6: For a UE supporting both semi-static and dynamic channel access, the UE can be tested under dynamic channel occupancy only. |  |  |  |  |  |  |  |  |  |  |

##### A.11.2.1.3.3 Test Requirements

The UE shall start to transmit the PRACH to Cell 2 less than Tinterrupt from the beginning of time period T3, where Tinterrupt is defined in clause 6.1B.1.2

The rate of correct handovers observed during repeated tests shall be at least 90%.

NOTE:	The handover delay can be expressed as: RRC procedure delay + Tinterrupt, where:

Tinterrupt = Tsearch + TIU + Tprocessing  + T∆ + Tmargin

Tsearch = (3+L1’)* 20 ms.

Tprocessing = 20 ms.

Tmargin = 2 ms.

T∆ = (1+ L2) *20 ms.

TIU = (1+ L3)*10 + 10 ms

RRC procedure delay = 10 ms and is specified in clause 12 in TS 38.331 [2], L1’ is the number of SMTC occasions not available at the UE during the inter-frequency detection period, L2 is the number of SMTC occasions not available at the UE during the time tracking period, where L1’ + L2  LCCA_DL, and L3 is the number of consecutive SSB to PRACH occasion association periods during which no PRACH occasion is available for PRACH transmission due to UL CCA failure, where L3  LCCA_UL. L3 = 0 for Type 2C UL channel access procedure as defined in TS 37.213 [33]. The interruption time considering the potential extensions caused by , L1´,L2 , L3  and by the UL CCA failure detection/recovery mechanism is limited by the T304 timer.

#### A.11.2.1.4	Inter-frequency handover from FR1 carrier under CCA to FR1; known target cell

##### A.11.2.1.4.1	Test Purpose and Environment

This test is to verify the requirement for the NR with CCA FR1-NR FR1 handover requirements specified in clause 6.1.1.2.

##### A.11.2.1.4.2	Test Parameters

Supported test configurations are shown in table A.11.2.1.4.2-1. Both handover delay and interruption length are tested by using the parameters in table A.11.2.1.4.2-2, and A.11.2.1.4.2-3.

The test consists of three successive time periods, with time durations of T1 T2 and T3 respectively. At the start of time duration T1, the UE may not have any timing information of cell 2.

NR with CCA shall send a RRC message implying handover to cell 2. The RRC message implying handover shall be sent to the UE during period T2, after the UE has reported Event A3. T3 is defined as the end of the last TTI containing the RRC message implying handover.

Table A.11.2.1.4.2-1: Handover from NR with CCA FR1 to NR FR1 test configuration

| Config | Description |
| --- | --- |
| 1 | Source cell: NR with CCA 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | Source cell: NR with CCA 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | Source cell: NR with CCA 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode Target cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.11.2.1.4.2-2: General test parameters handover from NR with CCA FR1 to NR FR1

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 | NR cell with CCA |
|  | Neighbouring cell |  | Cell 2 | NR cell |
| Final condition | Active cell |  | Cell 2 |  |
| DL CCA model |  |  | As specified in clause A.3.26.2.1 |  |
| UL CCA model |  |  | As specified in clause A.3.26.2.2 |  |
| A3-Offset |  | dB | 0 |  |
| Hysteresis |  | dB | 0 |  |
| Time To Trigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| Time offset between cells |  |  | 3 s | Synchronous cells |
| T1 |  | s | 5 |  |
| T2 |  | s | 5 |  |
| T3 |  | s | 1 |  |

Table A.11.2.1.4.2-3: Cell specific test parameters for NR with CCA FR1 – NR FR1 handover test case

| Parameter |  |  | Unit | Test | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | configuration | T1 | T2 | T3 | T1 | T2 | T3 |
| NR RF Channel Number |  |  |  | 1,2,3 | 1 |  |  | 2 |  |  |
| Duplex mode |  |  |  | 1 | TDD |  |  | FDD |  |  |
|  |  |  |  | 2 | TDD |  |  | TDD |  |  |
|  |  |  |  | 3 | TDD |  |  | TDD |  |  |
| TDD configuration |  |  |  | 1 | TDDConf.1.1 CCA |  |  | Not Applicable |  |  |
|  |  |  |  | 2 | TDDConf.1.1 CCA |  |  | TDDConf.1.1 |  |  |
|  |  |  |  | 3 | TDDConf.1.1 CCA |  |  | TDDConf.2.1 |  |  |
| BWchannel |  |  | MHz | 1 | 40: NRB,c = 106 |  |  | 10: NRB,c = 52 |  |  |
|  |  |  |  | 2 | 40: NRB,c = 106 |  |  | 10: NRB,c = 52 |  |  |
|  |  |  |  | 3 | 40: NRB,c = 106 |  |  | 40: NRB,c = 106 |  |  |
| BWP BW |  |  | MHz | 1 | 40: NRB,c = 106 |  |  | 10: NRB,c = 52 |  |  |
|  |  |  |  | 2 | 40: NRB,c = 106 |  |  | 10: NRB,c = 52 |  |  |
|  |  |  |  | 3 | 40: NRB,c = 106 |  |  | 40: NRB,c = 106 |  |  |
| DRX Cycle |  |  | ms | 1,2,3 | Not Applicable |  |  |  |  |  |
| PDSCH Reference measurement channel |  |  |  | 1 | SR.1.1 CCA |  |  | SR.1.1 FDD |  |  |
|  |  |  |  | 2 | SR.1.1 CCA |  |  | SR.1.1 TDD |  |  |
|  |  |  |  | 3 | SR.1.1 CCA |  |  | SR2.1 TDD |  |  |
| CORESET Reference Channel |  |  |  | 1 | CR1.1 CCA |  |  | CR.1.1 FDD |  |  |
|  |  |  |  | 2 | CR1.1 CCA |  |  | CR.1.1 TDD |  |  |
|  |  |  |  | 3 | CR1.1 CCA |  |  | CR2.1 TDD |  |  |
| Dedicated CORESET RMC configuration |  |  |  | 1 | CCR.1.1 CCA |  |  | CCR.1.1 FDD |  |  |
|  |  |  |  | 2 | CCR.1.1 CCA |  |  | CCR.1.1 TDD |  |  |
|  |  |  |  | 3 | CCR.1.1 CCA |  |  | CCR.2.1 TDD |  |  |
| DL CCA probability for semi-static channel access (PCCA_DL)DL CCA probability PCCA_DL |  |  |  | 1,2,3 | 0.9375 |  |  | N/A |  |  |
| DL CCA probability for dynamic channel access (PCCA_DL_1) |  |  |  | 1,2,3 | 0.75 |  |  | N/A |  |  |
| DL CCA probability for dynamic channel access (PCCA_DL_2) |  |  |  | 1,2,3 | 0.75 |  |  | N/A |  |  |
| UL CCA probability for semi-static channel access PCCA_UL |  |  |  | 1,2,3 | 0.75 |  |  | N/A |  |  |
| UL CCA probability for dynamic static channel access PCCA_UL |  |  |  | 1,2,3 | 0.87 |  |  |  |  |  |
| TRS configuration |  |  |  | 1 | TRS.1.2 TDD |  |  | TRS.1.1 FDD |  |  |
|  |  |  |  | 2 | TRS.1.2 TDD |  |  | TRS.1.1 TDD |  |  |
|  |  |  |  | 3 | TRS.1.2 TDD |  |  | TRS.1.2 TDD |  |  |
| OCNG Patterns |  |  |  |  | OP.1 |  |  |  |  |  |
| SMTC Configuration |  |  |  |  | SMTC.1 |  |  |  |  |  |
| SSB Configuration |  | Semi-static channel access |  | 1,2 | SSB.1 CCA (As defined in A.3.10A ) |  |  | SSB.1 FR1 |  |  |
|  |  | Dynamic channel access |  |  | SSB.2 CCA (As defined in A.3.10A ) |  |  |  |  |  |
|  |  | Semi-static channel access |  | 3 | SSB.1 CCA (As defined in A.3.10A ) |  |  | SSB.2 FR1 |  |  |
|  |  | Dynamic channel access |  |  | SSB.2 CCA (As defined in A.3.10A ) |  |  |  |  |  |
| DBT window configuration |  |  |  |  | As defined in A.3.28.1 |  |  | Not applicable |  |  |
| PDSCH/PDCCH subcarrier spacing |  |  | kHz | 1,2 | 30 kHz |  |  | 15 kHz |  |  |
|  |  |  |  | 3 | 30 kHz |  |  | 30 kHz |  |  |
| PUCCH/PUSCH subcarrier spacing |  |  | kHz | 1,2 | 30 kHz |  |  | 15 kHz |  |  |
|  |  |  |  | 3 | 30 kHz |  |  | 30 kHz |  |  |
| PRACH configuration |  |  |  |  | FR1 PRACH configuration 1 under CCA |  |  | FR1 PRACH configuration 1 |  |  |
| BWP configuration |  | Initial DL BWP |  | 1,2,3 | DLBWP.0.1 |  |  |  |  |  |
|  |  | Dedicated DL BWP |  | 1,2,3 | DLBWP.1.1 |  |  |  |  |  |
|  |  | Initial UL BWP |  | 1,2,3 | ULBWP.0.1 |  |  |  |  |  |
|  |  | Dedicated UL BWP |  | 1,2,3 | ULBWP.1.1 |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 1,2,3 | 0 |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  | 1,2,3 |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  | 1,2,3 |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  | 1,2,3 |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  | 1,2,3 |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  | 1,2,3 |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  | 1,2,3 |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote1 |  |  |  | 1,2,3 |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRSNote1 |  |  |  | 1,2,3 |  |  |  |  |  |  |
| Note2 |  |  | dBm/SCS | 1,2 | [-101] |  |  | -98 |  |  |
|  |  |  |  | 3 | [-101] |  |  | -95 |  |  |
|  |  |  | dB | 1,2,3 | 8 | -3.3 | -3.3 | -Infinity | 2.36 | 2.36 |
|  |  |  | dB | 1,2,3 | 8 | 8 | 8 | -Infinity | 11 | 11 |
| SSB_RP | Config 1 |  | dBm/SCS | 1,2,3 | -90 | -90 | -90 | -Infinity | -87 | -87 |
| IoNote3 | Config 1 |  | dBm/ 9.36MHz | 1,2,3 | -61.41 | -57.06 | -57.06 | -61.41 | -57.06 | -57.06 |
| Propagation condition |  |  | - | 1,2,3 | AWGN |  |  | AWGN |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For UE supporting both semi-static and dynamic cannel access, the UE must be tested under both dynamic and semi-static channel occupancy configurations. |  |  |  |  |  |  |  |  |  |  |

##### A.11.2.1.4.3 Test Requirements

The UE shall start to transmit the PRACH to Cell 2 less than 112 ms from the beginning of time period T3.

The rate of correct handovers observed during repeated tests shall be at least 90%.

NOTE:	The handover delay can be expressed as: RRC procedure delay + Tinterrupt, where:

RRC procedure delay = 10 ms and is specified in clause 12 in TS 38.331 [2].

Tinterrupt = 102 ms in the test. Tinterrupt is defined in clause 6.1.1.2.2.

This gives a total of 112 ms.

#### A.11.2.1.5	Inter-frequency handover from FR1 carrier under CCA to FR1; unknown target cell

##### A.11.2.1.5.1	Test Purpose and Environment

This test is to verify the requirement for the NR with CCA FR1-NR FR1 handover requirements specified in clause 6.1.1.2.

##### A.11.2.1.5.2	Test Parameters

Supported test configurations are shown in table A.11.2.1.5.2-1. Both handover delay and interruption length are tested by using the parameters in table A.11.2.1.5.2-2, and A.12.2.1.7.2-3.

The test scenario comprises of two carriers and one cell on each carrier. Cell 1 is the NR with CCA cell and Cell 2 is an NR neighbour cell. No gap patterns are configured in the test case. The test consists of two successive time periods, with time durations of T1 and T2 respectively. At the start of time duration T1, the UE does not have any timing information of cell 2.

Starting T2, cell 2 becomes detectable and the UE receives a RRC handover command from the network. The start of T2 is the instant when the last TTI containing the RRC message implying handover is sent to the UE.

Table A.11.2.1.5.2-1: Handover from NR with CCA FR1 to NR FR1 test configuration

| Config | Description |
| --- | --- |
| 1 | Source cell: NR with CCA 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | Source cell: NR with CCA 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | Source cell: NR with CCA 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode Target cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.11.2.1.5.2-2: General test parameters handover from NR with CCA FR1 to NR FR1

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 | NR cell with CCA |
|  | Neighbouring cell |  | Cell 2 | NR cell |
| Final condition | Active cell |  | Cell 2 |  |
| DL CCA model |  |  | As specified in clause A.3.26.2.1 |  |
| UL CCA model |  |  | As specified in clause A.3.26.2.2 |  |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| Time offset between cells |  |  | 3 s | Synchronous cells |
| T1 |  | s | 5 |  |
| T2 |  | s | 5 |  |

Table A.11.2.1.5.2-3: Cell specific test parameters for NR with CCA FR1 – NR FR1 handover test case

| Parameter | Unit | Configuration | Cell 1 |  |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |  | T1 | T2 |
| NR RF Channel Number |  | 1, 2, 3 | 1 |  |  | 2 |  |
| Duplex mode |  | 1 | TDD |  |  | FDD |  |
|  |  | 2 | TDD |  |  | TDD |  |
|  |  | 3 | TDD |  |  | TDD |  |
| DL CCA model |  | 1, 2, 3 | As specified in clause A.3.26.2.1 |  |  | N/A |  |
| UL CCA model |  | 1, 2, 3 | As specified in clause A.3.26.2.2 |  |  |  |  |
| TDD configuration |  | 1 | TDDConf.1.1 CCA |  |  | Not Applicable |  |
|  |  | 2 | TDDConf.1.1 CCA |  |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.1.1 CCA |  |  | TDDConf.2.1 |  |
| BWchannel | MHz | 1 | 40: NRB,c = 106 |  |  | 10: NRB,c = 52 |  |
|  |  | 2 | 40: NRB,c = 106 |  |  | 10: NRB,c = 52 |  |
|  |  | 3 | 40: NRB,c = 106 |  |  | 40: NRB,c = 106 |  |
| BWP BW | MHz | 1 | 40: NRB,c = 106 |  |  | 10: NRB,c = 52 |  |
|  |  | 2 | 40: NRB,c = 106 |  |  | 10: NRB,c = 52 |  |
|  |  | 3 | 40: NRB,c = 106 |  |  | 40: NRB,c = 106 |  |
| DRX Cycle | ms |  | Not Applicable |  |  |  |  |
| PDSCH Reference measurement channel |  | 1 | SR.1.1 CCA |  |  | SR.1.1 FDD |  |
|  |  | 2 | SR.1.1 CCA |  |  | SR.1.1 TDD |  |
|  |  | 3 | SR.1.1 CCA |  |  | SR2.1 TDD |  |
| CORESET Reference Channel |  | 1 | CR2.1 TDD |  |  | CR.1.1 FDD |  |
|  |  | 2 | CR2.1 TDD |  |  | CR.1.1 TDD |  |
|  |  | 3 | CR2.1 TDD |  |  | CR2.1 TDD |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 CCA |  |  | CCR.1.1 FDD |  |
|  |  | 2 | CCR.1.1 CCA |  |  | CCR.1.1 TDD |  |
|  |  | 3 | CCR.1.1 CCA |  |  | CCR.2.1 TDD |  |
| TRS configuration |  | 1 | TRS.1.2 TDD |  |  | TRS.1.1 FDD |  |
|  |  | 2 | TRS.1.2 TDD |  |  | TRS.1.1 TDD |  |
|  |  | 3 | TRS.1.2 TDD |  |  | TRS.1.2 TDD |  |
| DL CCA probability for semi-static channel access (PCCA_DL)DL CCA probability PCCA_DL |  | 1, 2, 3 | 0.9375 |  |  | N/A |  |
| DL CCA probability for dynamic channel access (PCCA_DL_1) |  | 1, 2, 3 | 0.75 |  |  | N/A |  |
| DL CCA probability for dynamic channel access (PCCA_DL_2) |  | 1, 2, 3 | 0.75 |  |  | N/A |  |
| UL CCA probability for semi-static channel access PCCA_UL |  | 1, 2, 3 | 0.75 |  |  | N/A |  |
| UL CCA probability for dynamic static channel access PCCA_UL |  | 1, 2, 3 | 0.87 |  |  | N/A |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  |  |  |  |
| SMTC Configuration |  | 1, 2, 3 | SMTC.1 |  |  |  |  |
| DBT window configuration |  | 1, 2, 3 | As defined in A.3.28.1 |  |  | N/A |  |
| SSB configuration | Semi-static channel access | 1,2 | SSB.1 CCA (As defined in A.3.10A ) |  |  | SSB.1 FR1 |  |
|  | Dynamic channel access |  | SSB.2 CCA (As defined in A.3.10A ) |  |  |  |  |
|  | Semi-static channel access | 3 | SSB.1 CCA (As defined in A.3.10A ) |  |  | SSB.2 FR1 |  |
|  | Dynamic channel access |  | SSB.2 CCA (As defined in A.3.10A ) |  |  |  |  |
| ssb-PositionQCL |  |  | [1] |  |  | N/A |  |
| PDSCH/PDCCH subcarrier spacing | kHz | 1 | 30 kHz |  |  | 15 kHz |  |
|  |  | 2 | 30 kHz |  |  | 15 kHz |  |
|  |  | 3 | 30 kHz |  |  | 30 kHz |  |
| PUCCH/PUSCH subcarrier spacing | kHz | 1 | 30 kHz |  |  | 15 kHz |  |
|  |  | 2 | 30 kHz |  |  | 15 kHz |  |
|  |  | 3 | 30 kHz |  |  | 30 kHz |  |
| PRACH configuration |  | 1,2,3 | FR1 PRACH configuration 1 |  | FR1 PRACH configuration 1 under CCA |  |  |
| BWP configuration | Initial DL BWP | 1,2,3 | DLBWP.0.1 |  |  |  |  |
|  | Dedicated DL BWP | 1,2,3 | DLBWP.1.1 |  |  |  |  |
|  | Initial UL BWP | 1,2,3 | ULBWP.0.1 |  |  |  |  |
|  | Dedicated UL BWP | 1,2,3 | ULBWP.1.1 |  |  |  |  |
| EPRE ratio of PSS to SSS | dB | 1,2,3 | 0 |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | 1,2,3 |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | 1,2,3 |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  | 1,2,3 |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | 1,2,3 |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | 1,2,3 |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  | 1,2,3 |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote1 |  | 1,2,3 |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRSNote1 |  | 1,2,3 |  |  |  |  |  |
| Note2 | dBm/SCS | 1,2,3 | [-101] |  |  | -98 |  |
|  |  |  | [-101] |  |  | -95 |  |
|  | dB | 1,2,3 | -Infinity | 5 |  | 4 | 4 |
|  | dB | 1,2,3 | -Infinity | 5 |  | 4 | 4 |
| SSB_RP | dBm/SCS | 1,2,3 | -Infinity | -93 |  | -94 | -94 |
|  |  |  | -Infinity | -90 |  | -91 | -91 |
| IoNote3 | dBm/ 9.36MHz | 1,2,3 | -70.05 | -63.85 |  | -64.59 | -64.59 |
|  | dBm/ 38.16MHz | 1,2,3 | -63.94 | -57.75 |  | -58.49 | -58.49 |
| Propagation condition | - | 1,2,3 | AWGN |  |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For UE supporting both semi-static and dynamic cannel access, the UE must be tested under both dynamic and semi-static channel occupancy configurations. |  |  |  |  |  |  |  |

##### A.11.2.1.5.3 Test Requirements

The UE shall start to transmit the PRACH to Cell 2 less than 132 ms from the beginning of time period T2.

The rate of correct handovers observed during repeated tests shall be at least 90%.

NOTE:	The handover delay can be expressed as: RRC procedure delay + Tinterrupt, where:

RRC procedure delay = 10 ms and is specified in clause 12 in TS 38.331 [2].

Tinterrupt = 122 ms in the test. Tinterrupt is defined in clause 6.1.1.2.2.

This gives a total of 132 ms.

#### A.11.2.1.6	Inter-frequency handover from FR1 to FR1 carrier under CCA; unknown target cell

##### A.11.2.1.6.1	Test Purpose and Environment

This test is to verify the requirement for inter frequency handover requirements from FR1 to FR1 carrier under CCA specified in clause 6.1B.1.2.

##### A.11.2.1.6.2	Test Parameters

Supported test configurations are shown in table A.11.2.1.6.2-1. Both handover delay and interruption length are tested by using the parameters in table A.11.2.1.6.2-2, and A.11.2.1.6.2-3.

The test scenario comprises of two carriers and one cell on each carrier. No gap patterns are configured in the test case. The test consists of two successive time periods, with time durations of T1, T2 respectively. At the start of time duration T1, the UE does not have any timing information of cell 2. Starting T2, cell 2 becomes detectable and the UE receives a RRC handover command from the network. The start of T2 is the instant when the last TTI containing the RRC message implying handover is sent to the UE.

Table A.11.2.1.6.2-1: Inter-frequency handover from FR1 to FR1 carrier under CCA test configurations

| Configuration | Description of a cell with CCA | Description of a cell without CCA |
| --- | --- | --- |
| 1 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |  |

Table A.11.2.1.6.2-2: General test parameters Inter-frequency handover from FR1 to FR1 carrier under CCA

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 |  |
|  | Neighbouring cell |  | Cell 2 | On the carrier under CCA |
| Final condition | Active cell |  | Cell 2 | On the carrier under CCA |
| DL CCA model |  |  | As specified in clause A.3.26.2.1 |  |
| UL CCA model |  |  | As specified in clause A.3.26.2.2 |  |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| T1 |  | s | 5 |  |
| T2 |  | s | Tinterrupt | Tinterrupt is defined in clause 6.1B.1.2 |

Table A.11.2.1.6.2-3: Cell specific test parameters for NR FR1-FR1 Inter frequency handover test case

| Parameter |  | Unit | Configuration | Cell 1 |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 |  | T2 |
| NR RF Channel Number |  |  | 1, 2, 3 | 1 |  | 2 |  |  |
| DL CCA probability for semi-static channel access (PCCA_DL)DL CCA probability PCCA_DL |  |  | 1, 2, 3 | N/A |  | 0.9375 |  |  |
| DL CCA probability for dynamic channel access (PCCA_DL_1) |  |  | 1, 2, 3 | N/A |  | 0.75 |  |  |
| DL CCA probability for dynamic channel access (PCCA_DL_2) |  |  | 1, 2, 3 | N/A |  | 0.75 |  |  |
| UL CCA probability for semi-static channel access PCCA_UL |  |  | 1, 2, 3 | N/A |  | 0.75 |  |  |
| UL CCA probability for dynamic static channel access PCCA_UL |  |  | 1, 2, 3 | N/A |  | 0.87 |  |  |
| TDD configuration |  |  | 1 | N/A |  | TDDConf.1.1.CCA |  |  |
|  |  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1.CCA |  |  |
|  |  |  | 3 | TDDConf.1.2 |  | TDDConf.1.1.CCA |  |  |
| BWchannel |  |  | 1 | 10: NRB,c = 52 |  | 40: NRB,c = 106 |  |  |
|  |  |  | 2 | 10: NRB,c = 52 |  | 40: NRB,c = 106 |  |  |
|  |  |  | 3 | 40: NRB,c = 106 |  | 40: NRB,c = 106 |  |  |
| BWP BW |  |  | 1 | 10: NRB,c = 52 |  | 40: NRB,c = 106 |  |  |
|  |  |  | 2 | 10: NRB,c = 52 |  | 40: NRB,c = 106 |  |  |
|  |  |  | 3 | 40: NRB,c = 106 |  | 40: NRB,c = 106 |  |  |
| DRX Cycle |  | ms |  | Not Applicable |  |  |  |  |
| PDSCH Reference |  |  | 1 | SR.1.1 FDD |  | SR.1.1 CCA |  |  |
|  |  |  | 2 | SR.1.1 TDD |  | SR.1.1 CCA |  |  |
|  |  |  | 3 | SR.2.1 TDD |  | SR.1.1 CCA |  |  |
| CORESET Reference Channel |  |  | 1 | CR.1.1 FDD |  | CR.1.1 CCA |  |  |
|  |  |  | 2 | CR.1.1 TDD |  | CR.1.1 CCA |  |  |
|  |  |  | 3 | CR.2.1 TDD |  | CR.1.1 CCA |  |  |
| Dedicated CORESET RMC configuration |  |  | 1 | CCR.1.1 FDD |  | CCR.1.1 CCA |  |  |
|  |  |  | 2 | CCR.1.1 TDD |  | CCR.1.1 CCA |  |  |
|  |  |  | 3 | CCR.2.1 TDD |  | CCR.1.1 CCA |  |  |
| TRS configuration |  |  | 1 | TRS.1.1 FDD |  | TRS.1.2 TDD |  |  |
|  |  |  | 2 | TRS.1.1 TDD |  | TRS.1.2 TDD |  |  |
|  |  |  | 3 | TRS.1.2 TDD |  | TRS.1.2 TDD |  |  |
| OCNG Patterns |  |  | 1, 2, 3 | OP.1 |  |  |  |  |
| SMTC Configuration |  |  | 1, 2, 3 | SMTC.1 |  |  |  |  |
| DBT window configuration |  |  | 1, 2, 3 | N/A |  | As defined in A.3.28.1 |  |  |
| SSB configuration |  |  | 1, 2 | SSB.1 FR1 |  | SSB.1 CCA for semi-static channel access; SSB.2 CCA for dynamic channel access; |  |  |
|  |  |  | 3 | SSB.2 FR1 |  | SSB.1 CCA for semi-static channel access; SSB.2 CCA for dynamic channel access; |  |  |
| ssb-PositionQCL |  |  |  | N/A |  | [1] |  |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | 1 | 15 kHz |  | 30 kHz |  |  |
|  |  |  | 2 | 15 kHz |  | 30 kHz |  |  |
|  |  |  | 3 | 30 kHz |  | 30 kHz |  |  |
| PUCCH/PUSCH subcarrier spacing |  | kHz | 1 | 15 kHz |  | 30 kHz |  |  |
|  |  |  | 2 | 15 kHz |  | 30 kHz |  |  |
|  |  |  | 3 | 30 kHz |  | 30 kHz |  |  |
| PRACH configuration |  |  |  | FR1 PRACH configuration 1 |  |  | FR1 PRACH configuration 1 CCA |  |
| BWP configuration | Initial DL BWP |  | 1, 2, 3 | DLBWP.0.1 |  |  |  |  |
|  | Dedicated DL BWP |  | 1, 2, 3 | DLBWP.1.1 |  |  |  |  |
|  | Initial UL BWP |  | 1, 2, 3 | ULBWP.0.1 |  |  |  |  |
|  | Dedicated UL BWP |  | 1, 2, 3 | ULBWP.1.1 |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB |  | 0 |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz |  | -98 |  |  |  |  |
| Note2 | Config 1 | dBm/SCS | 1, 2 | -98 |  |  |  |  |
|  |  |  | 3 | -95 |  |  |  |  |
|  |  | dB |  | 4 | 4 | -Infinity |  | 5 |
|  |  | dB |  | 4 | 4 | -Infinity |  | 5 |
| SSB_RP | Config 1 | dBm/SCS | 1, 2 | -94 | -94 | -Infinity |  | -93 |
|  |  |  | 3 | -91 | -91 | -Infinity |  | -90 |
| IoNote3 | Config 1 | dBm/ 9.36MHz | 1, 2 | -64.59 | -64.59 | -70.05 |  | -63.85 |
|  |  | dBm/ 38.16MHz | 3 | -58.49 | -58.49 | -63.94 |  | -57.75 |
| Propagation condition |  | - |  | AWGN |  | AWGN |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For UE supporting both semi-static and dynamic cannel access, the UE must be tested under both dynamic and semi-static channel occupancy configurations. |  |  |  |  |  |  |  |  |

##### A.11.2.1.6.3 Test Requirements

The UE shall start to transmit the PRACH to Cell 2 less than Tinterrupt from the beginning of time period T3, where Tinterrupt is defined in clause 6.1B.1.2

The rate of correct handovers observed during repeated tests shall be at least 90%.

NOTE:	The handover delay can be expressed as: RRC procedure delay + Tinterrupt, where:

RRC procedure delay = 10 ms and is specified in clause 12 in TS 38.331 [2], L1’ is the number of SMTC occasions not available at the UE during the inter-frequency detection period, L2 is the number of SMTC occasions not available at the UE during the time tracking period, and L3 is the number of consecutive SSB to PRACH occasion association periods during which no PRACH occasion is available for PRACH transmission due to UL CCA failure. L3 = 0 for Type 2C UL channel access procedure as defined in TS 37.213 [33]. The interruption time considering the potential extensions caused by L1, L1´,L2 , L3  and by the UL CCA failure detection/recovery mechanism is limited by the T304 timer. The UE behaviour at the T304 timer expiry is detailed in TS 38.331 [2].

#### A.11.2.1.7	 SA NR FR1 carrier under CCA - E-UTRAN handover with known target cell

##### A.11.2.1.7.1	Test Purpose and Environment

The purpose of this set of tests is to verify that the UE can make correct inter-RAT E-UTRAN handover when operating in standalone (SA) operation with PCell in FR1 carrier under CCA. This test shall verify the NR to E-UTRAN handover requirements as specified in clause 6.1.2.1.

The test comprises of one NR carrier under CCA and one E-UTRA carrier. There are two cells and one cell on each carrier. Cell 1 is the NR PCell and Cell 2 is an inter-RAT E-UTRAN neighbour cell. The test consists of three successive time periods, with time durations of T1, T2 and T3 respectively. At the start of time duration T1, the UE does not have any timing information of Cell 2. Starting T2, Cell 2 becomes detectable and the UE is expected to detect and send a measurement report. Gap pattern configuration with id #0 as specified in Table 9.1.2-1 is configured before T2 begins to enable inter-RAT frequency monitoring.

A RRC message implying handover shall be sent to the UE during period T2 after the UE has reported Event B2. The start of T3 is the instant when the last TTI containing the RRC message implying handover is sent to the UE. The handover message shall contain Cell 2 as the target cell.

Supported test configurations are shown in table A.11.2.1.7-1. General test parameters are provided in Table A.11.2.1.7-2. Cell specific test parameters for Cell 1 and Cell 2 are provided in Tables A.11.2.1.7-3 and A.11.2.1.7-4 respectively.

Table A.11.2.1.7-1: Supported test configurations for SA inter-RAT E-UTRAN handover tests

| Configuration | Description of a cell with CCA | Description of a cell without CCA |
| --- | --- | --- |
| 1 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode | LTE 10 MHz bandwidth, TDD duplex mode |
| 2 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode | LTE 10 MHz bandwidth, FDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations. Note 2: The UE supporting SA operation only on NR band(s) with shared spectrum access is required to be tested. |  |  |

Table A.11.2.1.7-2: General test parameters for SA inter-RAT E-UTRAN handover

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  |  | 1 | 1 NR carrier frequency is used in the test |
| LTE RF Channel Number |  |  | 2 | 1 E-UTRAN carrier frequency is used in the test |
| Initial conditions | Active cell |  | Cell 1 | NR cell on a carrier under CCA |
|  | Neighbouring cell |  | Cell 2 | E-UTRAN cell |
| Final condition | Active cell |  | Cell 2 |  |
| DL CCA model |  |  | As specified in clause A.3.26.2.1 |  |
| UL CCA model |  |  | As specified in clause A.3.26.2.2 |  |
| NR measurement quantity |  |  | SS-RSRP |  |
| E-UTRAN measurement quantity |  |  | RSRP |  |
| b2-Threshold1 |  | dBm | As specified in Table A.11.2.1.7-3 | Absolute NR SS-RSRP threshold for event B2 |
| b2-Threshold2EUTRAN |  | dBm | -98 | Absolute E-UTRAN RSRP threshold for event B2 |
| Hysteresis |  | dB | 0 |  |
| TimeToTrigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| DRX |  |  | OFF | Non-DRX test |
| Access Barring Information |  | - | Not sent | No additional delays in random access procedure |
| Time offset between cells |  |  | 3 ms | Asynchronous cells |
| Gap pattern configuration Id |  |  | 0 | As specified in Table 9.1.2-1 started before T2 starts |
| T1 |  | s | 5 |  |
| T2 |  | s | 5 |  |
| T3 |  | s | 1 |  |

Table A.11.2.1.7-3: Cell specific test parameters for SA inter-RAT E-UTRA handover (Cell 1)

| Parameter |  | Unit | Configuration | Cell 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T3 |
| RF channel number |  |  | 1, 2 | 1 |  |  |
| TDD Configuration |  |  | 1, 2 | TDDConf.1.1.CCA |  |  |
| DL CCA probability for semi-static channel access (PCCA_DL)DL CCA probability PCCA_DL |  |  | 1, 2 | 0.9375 |  |  |
| DL CCA probability for for dynamic static channel access (PCCA_DL_1) |  |  | 1, 2 | 0.75 |  |  |
| DL CCA probability for for dynamic static channel access (PCCA_DL_2) |  |  | 1, 2 | 0.75 |  |  |
| UL CCA probability for semi-static channel access PCCA_UL |  |  | 1, 2 | 0.75 |  |  |
| UL CCA probability for dynamic static channel access PCCA_UL |  |  | 1, 2 | 0.87 |  |  |
| BWchannel |  |  | 1, 2 | 40: NRB,c = 106 (TDD) |  |  |
| PDSCH reference measurement channel |  |  | 1, 2 | SR.1.1 CCA |  |  |
| CORESET reference channel |  |  | 1, 2 | CR.1.1 CCA |  |  |
| Dedicated CORESET RMC configuration |  |  | 1, 2 | CCR.1.1 CCA |  |  |
| TRS configuration |  |  | 1, 2 | TRS.1.2 TDD |  |  |
| OCNG patternNote1 |  |  | 1, 2 | OP.1 |  |  |
| BWP | Initial DL BWP |  | 1, 2 | DLBWP.0.1 |  |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  |  |
| SMTC configuration |  |  | 1, 2 | SMTC.1 |  |  |
| DBT window configuration |  |  | 1, 2 | As defined in A.3.28.1 |  |  |
| SSB configuration |  |  | 1, 2 | SSB.1 CCA for semi-static channel access; SSB.2 CCA for dynamic channel access; |  |  |
| b2-Threshold1 |  | dBm | 1, 2 | -93 |  |  |
| EPRE ratio of PSS to SSS |  | dB | 1, 2 | 0 |  |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |  |  |  |
| NocNote2 |  | dBm/15 KHz | 1, 2 | -100 | -104 | -100 |
| NocNote2 |  | dBm/SCS | 1, 2 | -97 | -101 | -97 |
| Ês/Noc |  | dB | 1, 2 | 12 | 0 | -4 |
| Ês/IotNote3 |  | dB | 1, 2 | 12 | 0 | -4 |
| SS-RSRPNote3 |  | dBm/SCS | 1, 2 | -85 | -101 | -101 |
| IoNote3 |  | dBm/38.16 MHz | 1, 2 | -53.68 | -66.9448 | -64.49 |
| Propagation condition |  |  | 1, 2 | AWGN |  |  |
| Antenna Configuration and Correlation Matrix |  |  | 1, 2 | 1x2 Low |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Ês/Iot, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For UE supporting both semi-static and dynamic cannel access, the UE must be tested under both dynamic and semi-static channel occupancy configurations. |  |  |  |  |  |  |

Table A.11.2.1.7-4: Cell specific test parameters for SA inter-RAT E-UTRA handover (Cell 2)

| Parameter | Unit | Configuration | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |
| RF channel number |  | 1, 2 | 2 |  |  |
| Duplex mode |  | 1 | FDD |  |  |
|  |  | 2 | TDD |  |  |
| TDD special subframe configurationNote1 |  | 2 | 6 |  |  |
| TDD uplink-downlink configurationNote1 |  | 2 | 1 |  |  |
| BWchannel | MHz | 1, 2 | 10 MHz: NRB,c = 50 |  |  |
| PRACH ConfigurationNote2 |  | 1 | 4 |  |  |
|  |  | 2 | 53 |  |  |
| PDSCH parameters: DL Reference Measurement ChannelNote3 |  | 1 | 10 MHz: R.3 FDD |  |  |
|  |  | 2 | 10 MHz: R.0 TDD |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote3 |  | 1 | 10 MHz: R.6 FDD |  |  |
|  |  | 2 | 10 MHz: R.6 TDD |  |  |
| OCNG PatternsNote3 |  | 1 | 10 MHz: OP.10 FDD |  |  |
|  |  | 2 | 10 MHz: OP.1 TDD |  |  |
| PBCH_RA | dB | 1, 2 | 0 |  |  |
| PBCH_RB |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |
| OCNG_RANote4 |  |  |  |  |  |
| OCNG_RBNote4 |  |  |  |  |  |
| NocNote5 | dBm/15kHz | 1, 2 | -98 |  |  |
| Ês/Noc | dB | 1, 2 | -Infinity | 8 | 78 |
| Ês/IotNote6 | dB | 1, 2 | -Infinity | 78 | 78 |
| RSRPNote6 | dBm/15kHz | 1, 2 | -Infinity | -90 | -90 |
| SCH_RPNote6 | dBm/15kHz | 1, 2 | -Infinity | -90 | -90 |
| IoNote6 | dBm/9MHz | 1, 2 | -67.21 +10log(NRB,c/100) | -58.57 +10log(NRB,c/100) | -58.57 +10log(NRB,c/100) |
| Propagation Condition |  | 1, 2 | AWGN |  |  |
| Antenna Configuration and Correlation Matrix Note7 |  | 1, 2 | 1x2 Low |  |  |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [23]. Note 2: PRACH configurations are specified in table 5.7.1-2 and table 5.7.1-3 in TS 36.211 [23]. Note 3: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [15] respectively. Note 4: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 5: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 6: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 7: Propagation condition and correlation matrix are defined in clause B.2 in TS 36.101 [25]. |  |  |  |  |  |

##### A.11.2.1.7.2	Test Requirements

The UE shall start to transmit the PRACH to Cell 2 less than 85 ms from the beginning of time period T3.

The rate of correct handovers observed during repeated tests shall be at least 90%.

NOTE:	The handover delay can be expressed as: RRC procedure delay + Tinterrupt, where:

RRC procedure delay = 50 ms and is specified in clause 6.1.2.1.

Tinterrupt = 35 ms in the test; Tinterrupt is defined in clause 6.1.2.1.

This gives a total of 85 ms.

#### A.11.2.1.8	SA NR FR1 carrier under CCA - E-UTRAN handover with unknown target cell

##### A.11.2.1.8.1	Test Purpose and Environment

The purpose of this set of tests is to verify that the UE can make correct inter-RAT E-UTRAN handover when operating in standalone (SA) operation with PCell in FR1 carrier under CCA. This test shall verify the NR to E-UTRAN handover requirements for the case when the target E-UTRAN cell is unknown as specified in clause 6.1.2.1.

The test comprises of one NR carrier under CCA and one E-UTRA carrier. There are two cells and one cell on each carrier. Cell 1 is the NR PCell and Cell 2 is an inter-RAT E-UTRAN neighbour cell. The test consists of two successive time periods, with time durations of T1 and T2 respectively. At the start of time duration T1, the UE does not have any timing information of Cell 2. Starting T2, Cell 2 becomes detectable. No Gap pattern shall be configured.

A RRC message implying handover shall be sent to the UE during period T1. The start of T2 is the instant when the last TTI containing the RRC message implying handover is sent to the UE. The handover message shall contain Cell 2 as the target cell.

Supported test configurations are shown in table A.11.2.1.8-1. General test parameters are provided in Table A.11.2.1.8-2. Cell specific test parameters for Cell 1 and Cell 2 are provided in Tables A.11.2.1.8-3 and A.11.2.1.8-4 respectively.

Table A.11.2.1.8-1: Supported test configurations for SA inter-RAT E-UTRAN handover tests

| Configuration | Description of a cell with CCA | Description of a cell without CCA |
| --- | --- | --- |
| 1 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode | LTE 10 MHz bandwidth, TDD duplex mode |
| 2 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode | LTE 10 MHz bandwidth, FDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations. Note 2: The UE supporting SA operation only on NR band(s) with shared spectrum access is required to be tested. |  |  |

Table A.11.2.1.8-2: General test parameters for SA inter-RAT E-UTRAN handover

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  |  | 1 | 1 NR carrier frequency is used in the test |
| LTE RF Channel Number |  |  | 2 | 1 E-UTRAN carrier frequency is used in the test |
| Initial conditions | Active cell |  | Cell 1 | NR cell on a carrier under CCA |
|  | Neighbouring cell |  | Cell 2 | E-UTRAN cell |
| Final condition | Active cell |  | Cell 2 |  |
| DL CCA model |  |  | As specified in clause A.3.26.2.1 |  |
| UL CCA model |  |  | As specified in clause A.3.26.2.2 |  |
| NR measurement quantity |  |  | SS-RSRP |  |
| DRX |  |  | OFF | Non-DRX test |
| Access Barring Information |  | - | Not sent | No additional delays in random access procedure |
| Time offset between cells |  |  | 3 ms | Asynchronous cells |
| T1 |  | s | 5 |  |
| T2 |  | s | 1 |  |

Table A.11.2.1.8-3: Cell specific test parameters for SA inter-RAT E-UTRA handover (Cell 1)

| Parameter |  | Unit | Configuration | Cell 1 |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 |
| RF channel number |  |  | 1, 2 | 1 |  |
| DL CCA probability for semi-static channel access (PCCA_DL)DL CCA probability PCCA_DL |  |  | 1, 2 | 0.9375 |  |
| DL CCA probability for dynamic channel access (PCCA_DL_1) |  |  | 1, 2 | 0.75 |  |
| DL CCA probability for dynamic channel access (PCCA_DL_2) |  |  | 1, 2 | 0.75 |  |
| UL CCA probability for semi-static channel access PCCA_UL |  |  | 1, 2 | 0.75 |  |
| UL CCA probability for dynamic static channel access PCCA_UL |  |  | 1, 2 | 0.87 |  |
| TDD Configuration |  |  | 1, 2 | TDDConf.1.1.CCA |  |
| BWchannel |  | MHz | 1, 2 | 40: NRB,c = 106 (TDD) |  |
| PDSCH reference measurement channel |  |  | 1, 2 | SR.1.1 CCA |  |
| CORESET reference channel |  |  | 1, 2 | CR.1.1 CCA |  |
| Dedicated CORESET RMC configuration |  |  | 1, 2 | CCR.1.1 CCA |  |
| TRS configuration |  |  | 1, 2 | TRS.1.2 TDD |  |
| OCNG patternNote1 |  |  | 1, 2 | OP.1 |  |
| BWP | Initial DL BWP |  | 1, 2 | DLBWP.0.1 |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  |
| SMTC configuration |  |  | 1, 2 | SMTC.1 |  |
| DBT window configuration |  |  | 1, 2 | As defined in A.3.28.1 |  |
| SSB configuration |  |  | 1, 2 | SSB.1 CCA for semi-static channel access; SSB.2 CCA for dynamic channel access; |  |
| EPRE ratio of PSS to SSS |  | dB | 1, 2 | 0 |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |  |  |
| NocNote2 |  | dBm/15 KHz | 1, 2 | -98 |  |
| NocNote2 |  | dBm/SCS | 1, 2 | -95 |  |
| Ês/Noc |  | dB | 1, 2 | 0 | 0 |
| Ês/IotNote3 |  | dB | 1, 2 | 0 | 0 |
| SS-RSRPNote3 |  | dBm/SCS | 1, 2 | -95 | -95 |
| IoNote3 |  | dBm/38.16 MHz | 1, 2 | -60.94 | -60.94 |
| Propagation condition |  |  | 1, 2 | AWGN |  |
| Antenna Configuration and Correlation Matrix |  |  | 1, 2 | 1x2 Low |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Ês/Iot, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For UE supporting both semi-static and dynamic cannel access, the UE must be tested under both dynamic and semi-static channel occupancy configurations. |  |  |  |  |  |

Table A.11.2.1.8-4: Cell specific test parameters for SA inter-RAT E-UTRA handover (Cell 2)

| Parameter | Unit | Configuration | Cell 2 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| RF channel number |  | 1, 2 | 2 |  |
| Duplex mode |  | 1 | FDD |  |
|  |  | 2 | TDD |  |
| TDD special subframe configurationNote1 |  | 2 | 6 |  |
| TDD uplink-downlink configurationNote1 |  | 2 | 1 |  |
| BWchannel | MHz | 1, 2 | 10 MHz: NRB,c = 50 |  |
| PRACH ConfigurationNote2 |  | 1 | 4 |  |
|  |  | 2 | 53 |  |
| PDSCH parameters: DL Reference Measurement ChannelNote3 |  | 1 | 10 MHz: R.3 FDD |  |
|  |  | 2 | 10 MHz: R.0 TDD |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote3 |  | 1 | 10 MHz: R.6 FDD |  |
|  |  | 2 | 10 MHz: R.6 TDD |  |
| OCNG PatternsNote3 |  | 1 | 10 MHz: OP.10 FDD |  |
|  |  | 2 | 10 MHz: OP.1 TDD |  |
| PBCH_RA | dB | 1, 2 | 0 |  |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote4 |  |  |  |  |
| OCNG_RBNote4 |  |  |  |  |
| NocNote5 | dBm/15kHz | 1, 2 | -98 |  |
| Ês/Noc | dB | 1, 2 | -Infinity | 7 |
| Ês/IotNote6 | dB | 1, 2 | -Infinity | 7 |
| RSRPNote6 | dBm/15kHz | 1, 2 | -Infinity | -91 |
| SCH_RPNote6 | dBm/15kHz | 1, 2 | -Infinity | -91 |
| IoNote6 | dBm/9MHz | 1, 2 | -70.22 | -62.43 |
| Propagation Condition |  | 1, 2 | AWGN |  |
| Antenna Configuration and Correlation Matrix Note7 |  | 1, 2 | 1x2 Low |  |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [23]. Note 2: PRACH configurations are specified in table 5.7.1-2 and table 5.7.1-3 in TS 36.211 [23]. Note 3: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [15] respectively. Note 4: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 5: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 6: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 7: Propagation condition and correlation matrix are defined in clause B.2 in TS 36.101 [25]. |  |  |  |  |

##### A.11.2.1.8.2	Test Requirements

The UE shall start to transmit the PRACH to Cell 2 less than 165 ms from the beginning of time period T2.

The rate of correct handovers observed during repeated tests shall be at least 90%.

NOTE:	The handover delay can be expressed as: RRC procedure delay + Tinterrupt, where:

RRC procedure delay = 50 ms and is specified in clause 6.1.2.1.

Tinterrupt = 115 ms in the test; Tinterrupt is defined in clause 6.1.2.1.

This gives a total of 165 ms.

### <End of Change 1>