3GPP TSG-RAN WG4 Meeting #100-e	R4- 2114151

Electronic Meeting, 16 – 27 August, 2020

Source:	Huawei, HiSilicon

Title:	TP to TR 37.717-21-11 for DC_7-7-28-n1

Agenda item:	8.15.2

Document for:	Approval

# Background

This contribution provides text proposal on the EN-DC band combination DC_7-7-28-n1 as defined in the basket WI [1].

# Text Proposal

##### ---Start of changes---

## 5.x	DC_7-7-28_n1

### 5.x.1	Configurations for DC

Table 5.x.1-1: Inter-band DC configurations (three bands)

| DC Configuration | Uplink DC configuration |
| --- | --- |
| DC_7A-7A-28A_n1A | DC_28A_n1A DC_7A_n1A |

### 5.x.2	Co-existence studies

Co-existence studies have been performed for DC_7-28_n1 as well as for lower order combinations.

### 5.x.3	∆TIB and ∆RIB values

Table 5.x.3-1: ΔTIB,c

| Inter-band DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_7-7-28_n1 | 7 | 0.6 |
|  | 28 | 0.6 |
|  | n1 | 0.5 |

Table 5.x.3-2: ΔRIB

| Inter-band DC configuration | E-UTRA or NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_7-7-28_n1 | 7 | 0 |
|  | 28 | 0.2 |
|  | n1 | 0 |

### 5.x.4	Reference sensitivity exceptions

The MSD requirements can reuse the same requirements for DC_7A-28A_n1A.

##### ---End of changes---

# Reference

[1]	RP-211371, “WID revision: Dual Connectivity (DC) of 2 bands LTE inter-band CA (2DL/1UL) and 1 NR band (1DL/1UL)”, Huawei, HiSilicon