3GPP TSG-RAN WG4 Meeting #100-e		R4-2114184

E-meeting, 16 Aug – 27 Aug, 2021

Agenda item:	9.6.2.2

Source:	Intel Corporation

Title:	Discussion on Tx EVM requirements for DL 1024QAM

Document for:	Discussion

# Introduction

In the RAN4 #99-e meeting WF with simulation assumption for DL 1024QAM was agreed [1]. In this paper we provide simulation result and views on the feasibility of DL 1024QAM and correspondent Tx EVM requirements.

# Discussion

## Link-level performance

In this section we provide link-level simulation results, using the simulation assumptions agreed in [1] and summarized in Table 1.

Table 1. Link level simulation assumptions

| Parameter | Value |
| --- | --- |
| Carrier frequency | 2 GHz |
| CBW/SCS | 40 MHz CBW + 15 kHz SCS |
| Allocated RBs | Full allocation |
| PDSCH configuration | Type A mapping, Start symbol 1, Duration 13 (for D slots) |
| DMRS configuration | Type 1, Single symbol, additional DMRS: pos1 |
| Symbol type | CP-OFDM |
| Number of HARQ transmission | 8 HARQ processes, maximum 4 transmissions |
| RV sequence | {0,2,3,1} |
| Propagation conditions | Case 1: TDL-A 10 ns, Maximum Doppler frequency: 5Hz Case 2: TDL-A 10 ns, Maximum Doppler frequency: 5Hz |
| Antenna configuration | 2x4, Low correlation |
| FRC configuration | Fixed rank 1, Fixed rank 2 256QAM: MCS 24, 25, 26, 27 from MCS index table 2 (Table 5.1.3.1-2, TS 38.114) 1024QAM: MCS 23, 24, 25, 26 from MCS index table captured in R1-2102088 |
| UE Receiver assumptions | Practical channel estimation MMSE-IRC |
| Tx EVM | 2%, 2.5%, 3% |
| Rx EVM | 0%, 1.5%, 3%. |

Below in Figure 1 we compare performance of 256QAM and 1024QAM for different values of Tx EVM and Rx EVM  for Rank 1 and Rank 2 for TDL-A channel model. In Figure 2 the same comparison is conducted for TDL-D channel model. The purpose of the comparison is to analyse the value of Tx EVM required to achieve sufficient performance improvement of 1024QAM over 256QAM. In the figures for each scenario the analysed curve is an envelope of the throughput curves for four MCSs. In other words, for each SNR point the best MCS out of four (MCS 24-27 for 256QAM and MCS 23-26 for 1024QAM) is selected.

| Rx EVM = 0%, TDL-A channel |  |
| --- | --- |
| Rx EVM = 1.5%, TDL-A channel |  |
| Rx EVM = 3%, TDL-A channel |  |
| Figure 1. Simulation results for TDL-A channel model |  |

| Rx EVM = 1.5%, TDL-D channel |  |
| --- | --- |
| Rx EVM = 3%, TDL-D channel |  |
| Figure 2. Simulation results for TDL-D channel model |  |

Observation 1: From link level results we can conclude:

TDL-A channel

    - MIMO rank 1: sufficient performance improvement is observed for SNR > 26-29 dB depending on EVM conditions.

    - MIMO rank 2:

      - for Rx EVM = 3% there is no performance improvement for SNR < 35dB

      - for Rx EVM < 3% small performance improvement can be observed for SNR > 31-35dB depending on EVM conditions.

TDL-D channel

    - MIMO rank 1: sufficient performance improvement is observed for SNR > 25 dB

    - MIMO rank 2: sufficient performance improvement is observed for SNR > 30-34 dB depending on EVM conditions

Observations 2: From analysis of required Tx EVM value to achieve benefits of 1024QAM we can conclude that the 1024QAM performance in case of Tx EVM 2.5% is slightly better than in case of Tx EVM 3% for rank 1 and sufficiently better for rank 2.

Proposal 1: Define Tx EVM requirements equal to 2.5%

## 1024QAM testability

In this section we analyze under which assumptions 70% of max T-put metric is achievable for DL 1024QAM for SNR<38dB. The corresponding simulation results are summarized in Table 2 and Table 3 for TDL-A and TDL-D channel models respectively. Here “N/A” means that 70% of max T-put is not achievable for SNR < 38 dB

Table 2. SNR for 70% of Max T-put for different 1024QAM MCSs in TDL-A channel

| Rank configuration | Rx EVM | Tx EVM | MCS 23 | MCS 24 | MCS 25 | MCS 26 |
| --- | --- | --- | --- | --- | --- | --- |
| Rank 1 | 0.0% | 2.0% | 23.3 | 24.8 | 26.6 | 29.4 |
|  |  | 2.5% | 23.8 | 25.3 | 27.3 | 31.1 |
|  |  | 3.0% | 24.5 | 26.3 | 28.7 | N/A |
|  | 1.5% | 2.0% | 24.3 | 25.7 | 27.5 | 31.1 |
|  |  | 2.5% | 24.7 | 26.5 | 28.7 | 33.1 |
|  |  | 3.0% | 25.2 | 27.2 | 30.1 | N/A |
|  | 3.0% | 2.0% | 24.9 | 26.7 | 28.9 | 33.1 |
|  |  | 2.5% | 25.5 | 27.4 | 30.2 | N/A |
|  |  | 3.0% | 26.3 | 28.6 | 32 | N/A |
| Rank 2 | 0.0% | 2.0% | 29.3 | 31 | 33.1 | 37.3 |
|  |  | 2.5% | 30.2 | 32.4 | 35.4 | N/A |
|  |  | 3.0% | 31.6 | 34.9 | N/A | N/A |
|  | 1.5% | 2.0% | 30.9 | 33 | 35.7 | N/A |
|  |  | 2.5% | 32.1 | 34.9 | N/A | N/A |
|  |  | 3.0% | 33.9 | N/A | N/A | N/A |
|  | 3.0% | 2.0% | 32.9 | 35.9 | N/A | N/A |
|  |  | 2.5% | 34.5 | N/A | N/A | N/A |
|  |  | 3.0% | 37.1 | N/A | N/A | N/A |

Table 3. SNR for 70% of Max T-put for different 1024QAM MCSs in TDL-D channel

| Rank configuration | Rx EVM | Tx EVM | MCS 23 | MCS 24 | MCS 25 | MCS 26 |
| --- | --- | --- | --- | --- | --- | --- |
| Rank 1 | 1.5% | 2.5% | 23.3 | 25.0 | 27.0 | 31.0 |
|  |  | 3.0% | 24.1 | 25.7 | 28.4 | 34.6 |
|  | 3.0% | 2.5% | 24.5 | 26.1 | 28.4 | 33.0 |
|  |  | 3.0% | 24.9 | 26.9 | 29.5 | N/A |
| Rank 2 | 1.5% | 2.5% | 28.8 | 31.2 | 35.1 | N/A |
|  |  | 3.0% | 30.5 | 34.8 | N/A | N/A |
|  | 3.0% | 2.5% | 30.5 | 33.7 | N/A | N/A |
|  |  | 3.0% | 32.6 | N/A | N/A | N/A |

Observations #3: From analysis on testability of DL 1024QAM we can conclude:

    - SNR operating point is rather sensitive to Tx/Rx EVM value

    - DL 1024QAM can not be tested for rank 2 + MCS 26. For rank 2 + MCS 25 and rank 1 + MCS 26 the 70% of max T-put is achievable only under conditions of low Tx and Rx EVM

# Conclusion

In this paper we provide simulation result and views on the feasibility of DL 1024QAM and correspondent Tx EVM requirements. The following observations and proposals were made:

Observation 1: From link level results we can conclude:

TDL-A channel

    - MIMO rank 1: sufficient performance improvement is observed for SNR > 26-29 dB depending on EVM conditions.

    - MIMO rank 2:

      - for Rx EVM = 3% there is no performance improvement for SNR < 35dB

      - for Rx EVM < 3% small performance improvement can be observed for SNR > 31-35dB depending on EVM conditions.

TDL-D channel

    - MIMO rank 1: sufficient performance improvement is observed for SNR > 25 dB

    - MIMO rank 2: sufficient performance improvement is observed for SNR > 30-34 dB depending on EVM conditions

Observations 2: From analysis of required Tx EVM value to achieve benefits of 1024QAM we can conclude that the 1024QAM performance in case of Tx EVM 2.5% is slightly better than in case of Tx EVM 3% for rank 1 and sufficiently better for rank 2.

Proposal 1: Define Tx EVM requirements equal to 2.5%

Observations #3: From analysis on testability of DL 1024QAM we can conclude:

    - SNR operating point is rather sensitive to Tx/Rx EVM value

    - DL 1024QAM can not be tested for rank 2 + MCS 26. For rank 2 + MCS 25 and rank 1 + MCS 26 the 70% of max T-put is achievable only under conditions of low Tx and Rx EVM

# References

- R4-2108077, WF on Link level simulation assumptions, CATT, RAN4 #99e, May 2021