3GPP TSG-RAN Meeting #100e	R4-2114208

# Electronic Meeting, August 16 – 27, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.171 | CR | 0013 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Frequency bands for testing of A-GNSS sensitivity requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf, TEI16 |  |  |  |  |  | Date: |  |  | 2021-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In some configurations, the performance of the A-GNSS sensitivity test may be influenced by the bearer band used for the test scenario. However, the A-GNSS test requirements and scenarios do not specify any specific bands or band-combinations in which the test cases should be validated. RAN5 currently tests the A-GNSS (and A-GPS) sensitivity requirements in each UE supported frequency band of the relevant cellular RAT. However, this is unnecessary and may result in excessive testing time (in particular for EN-DC operation mode) since not all bearer bands affect the GNSS reception bands. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - For NR single carrier, only the frequency bands which can generate 2nd order harmonics and other distortions falling into the GNSS reception bands need to be tested. - For EN-DC, only those EN-DC configurations that can generate second or third order intermodulation (IM) products falling into the GNSS reception bands need to be tested. - The EN-DC configurations are divided into groups with similar IMD level and risks. For each group, only one of the EN-DC configurations in the group need to be tested. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It remains unclear which frequency bands and band combinations should be used for verifying A-GNSS sensitivity requirements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | B.1.7, B.1.12 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS/TR 36.171 CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR 37.571 CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | See R4-2100021 for additional background: "LS on Frequency Bands for testing of A-GNSS Sensitivity requirements in NR and LTE" (RAN5). See R4-2108232 for RAN4 agreements: "WF on frequency bands for testing of A-GNSS Sensitivity requirements in NR and LTE". |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## B.1.7	E-UTRA or NR frequency and frequency error

In all test cases other than Sensitivity in clause 5.1 with E-UTRA frequency, the E-UTRA frequency used shall be the mid-range for the E-UTRA operating band. The E-UTRA frequency with respect to the GNSS carrier frequency shall be offset by +0.025 PPM.

In all test cases other than Sensitivity in clause 5.1 with NR, the NR frequency used shall be as specified in TS 38.508-1 [20], clause 4.3.1. The NR frequency with respect to the GNSS carrier frequency shall be offset by + 0.025 PPM.

Editor's Note: What does this 0.025 ppm offset "with respect to the GNSS carrier frequency" mean? What does "NR frequency" and "GNSS carrier frequency" refer to?

For verifying the sensitivity requirements in clause 5.1 with NR single carrier, the sensitivity tests shall be performed in each frequency band listed in Table B.1.7-1. The NR frequency and channel configuration shall be selected to ensure second order harmonics and other distortion will fall into the GNSS receiver bands as defined in clause B.1.12.2 for the particular GNSS. Table B.1.7-2 defines exemplary test frequencies and RB settings for GPS L1/Galileo E1 for the frequency band listed in Table B.1.7-1.

NOTE : 	If the DUT does not support any of the frequency bands listed in Table B.1.7-1 the sensitivity tests in clause 5.1 can be performed in any frequency band supported by the DUT.

Table B.1.7-1: NR operating bands for verifying GNSS sensitivity

| NR operating bands | n13, n14, n24, n79, n96 |
| --- | --- |

Table B.1.7-2: Exemplary NR configuration for verifying the sensitivity requirements for GPS L1/Galileo E1

| NR Band | Bandwidth (SCS) [MHz] | UL Fc [MHz] | UL ARFCN | Total No of UL RB’s | Start UL RB No. |
| --- | --- | --- | --- | --- | --- |
| n13 | 10 (15 kHz) | 782.00 | 156400 | 52 | 0 |
| n14 | 10 (15 kHz) | 793.00 | 158600 | 52 | 0 |
| n24 | 5 (15 kHz) | 1629.00 | 325800 | 25 | 0 |
| n79 | 100 (30 kHz) | 4744.20 | 716280 | 273 | 0 |
| n96 | 80 (30 kHz) | 6312.70 | 820846 | 217 | 0 |

[…]

## B.1.12	EN-DC band combinations for testing A-GNSS sensitivity

#### B.1.12.1	EN-DC band combination groups

The A-GNSS sensitivity requirements in EN-DC operation mode with uplink assigned to E-UTRA and NR frequency bands the requirements in clause 5.1 can be verified by one EN-DC band combination in each of the applicable Frequency Group Combination specified in Table B.1.12.1-1. The A-GNSS sensitivity requirements for the remaining applicable EN-DC band combinations in each Frequency Group Combination are considered to be verified by using the one EN-DC band combination in each Frequency Group Combination. The Frequency Groups are defined in Table B.1.12.1-2.

The applicable EN-DC band combinations for verifying A-GNSS sensitivity requirements in EN-DC operation mode are specified in clause B.1.12.2.

Table B.1.12.1-1: EN-DC band combination groups for verifying A-GNSS sensitivity requirements in EN-DC operation mode

| Frequency Group Combination | EN-DC Band Combinations |
| --- | --- |
| Group VHF-VHF | NA |
| Group VHF-LB | NA |
| Group VHF-MLB | NA |
| Group VHF-MB | NA |
| Group VHF-HB | NA |
| Group VHF-UHB1 | NA |
| Group VHF-UHB2 | NA |
| Group LB-VHF | NA |
| Group LB-LB | DC_5A_n12A DC_5A_n71A DC_8A_n20A DC_8A_n28A DC_12A_n5A DC_20A_n8A DC_20A_n28A DC_28A_n5A DC_28A_n8A DC_71A_n5A DC_20A_n83A |
| Group LB-MLB | DC_20A_n50A DC_20A_n51A DC_28A_n51A DC_28A_n50A |
| Group LB-MB | DC_5A_n2A DC_5A_n66A DC_8A_n1A DC_8A_n3A DC_8A_n34A DC_8A_n39A DC_12A_n2A DC_12A_n25A DC_12A_n66A DC_18A_n3A DC_20A_n1A DC_20A_n3A DC_26A_n25A DC_28A_n3A DC_71A_n66A DC_8A_n80A DC_20A_n80A |
| Group LB-HB | DC_5A_n7A DC_5A_n38A DC_5A_n40A DC_8A_n40A DC_8A_n41A DC_12A_n7A DC_12A_n38A DC_12A_n41A DC_20A_n7A DC_20A_n38A DC_20A_n41A DC_26A_n41A DC_28A_n7A DC_28A_n40A DC_28A_n41A DC_71A_n38A |
| Group LB-UHB1 | DC_5A_n48A DC_5A_n78A DC_8A_n77A DC_8A_n78A DC_12A_n78A DC_18A_n77A DC_18A_n78A DC_19A_n77A DC_19A_n78A DC_20A_n77A DC_20A_n78A DC_26A_n77A DC_26A_n78A DC_28A_n77A DC_28A_n78A DC_71A_n48A DC_71A_n78A |
| Group LB-UHB2 | DC_5A_n79A DC_8A_n79A DC_18A_n79A DC_19A_n79A DC_26A_n79A DC_28A_n79A |
| Group MLB-VHF | NA |
| Group MLB-LB | DC_11A_n28A |
| Group MLB-MLB | NA |
| Group MLB-MB | DC_11A_n3A |
| Group MLB-HB | NA |
| Group MLB-UHB1 | DC_11A_n77A DC_11A_n78A DC_21A_n77A DC_21A_n78A |
| Group MLB-UHB2 | DC_11A_n79A DC_21A_n79A |
| Group MB-VHF | NA |
| Group MB-LB | DC_1A_n5A DC_1A_n8A DC_1A_n20A DC_1A_n28A DC_1A_n71A DC_2A_n5A DC_2A_n12A DC_2A_n71A DC_3A_n5A DC_3A_n8A DC_3A_n20A DC_3A_n28A DC_3A_n71A DC_66A_n5A DC_66A_n12A DC_66A_n71A DC_3A_n82A |
| Group MB-MLB | DC_1A_n50A DC_1A_n51A DC_3A_n50A DC_3A_n51A |
| Group MB-MB | DC_1A_n3A DC_2A_n66A DC_3A_n1A DC_3A_n34A DC_66A_n2A DC_66A_n25A DC_1A_n80A DC_2A_n2A2 DC_66A_n66A2 DC_3A_n3A2 DC_3A_n84A |
| Group MB-HB | DC_1A_n7A DC_1A_n38A DC_1A_n40A DC_1A_n41A DC_2A_n7A DC_2A_n38A DC_2A_n41A DC_3A_n7A DC_3A_n38A DC_3A_n40A DC_3A_n41A DC_4A_n38A DC_4A_n41A DC_25A_n41A DC_39A_n40A DC_39A_n41A DC_66A_n7A DC_66A_n38A DC_66A_n41A |
| Group MB-UHB1 | DC_1A_n77A DC_1A_n78A DC_2A_n48A DC_2A_n78A DC_3A_n77A DC_3A_n78A DC_4A_n78A DC_39A_n78A DC_66A_n48A DC_66A_n78A |
| Group MB-UHB2 | DC_1A_n79A DC_3A_n79A DC_39A_n79A |
| Group HB-VHF | NA |
| Group HB-LB | DC_7A_n5A DC_7A_n8A DC_7A_n20A DC_7A_n28A DC_7A_n71A DC_30A_n5A DC_41A_n28A |
| Group HB-MLB | DC_7A_n51A |
| Group HB-MB | DC_7A_n1A DC_7A_n3A DC_7A_n66A DC_30A_n2A DC_30A_n66A DC_40A_n1A DC_41A_n3A DC_7A_n80A DC_38A_n3A |
| Group HB-HB | DC_7A_n40A DC_40A_n41A DC_7A_n7A2 DC_41A_n41A |
| Group HB-UHB1 A(NOTE 1): | DC_40A_n77A DC_40A_n78A |
| B(NOTE 2): | DC_7A_n78A DC_38A_n78A DC_41A_n78A |
| C(NOTE 3): | DC_7A_n77A DC_41A_n77A |
| Group HB-UHB2 | DC_40A_n79A DC_41A_n79A |
| Group UHB1-VHF | NA |
| Group UHB1-LB | NA |
| Group UHB1-MLB | NA |
| Group UHB1-MB | NA |
| Group UHB1-HB | NA |
| Group UHB1-UHB1 | NA |
| Group UHB1-UHB2 | NA |
| Group UHB2-VHF | NA |
| Group UHB2-LB | NA |
| Group UHB2-MLB | NA |
| Group UHB2-MB | NA |
| Group UHB2-HB | NA |
| Group UHB2-UHB1 | NA |
| Group UHB2-UHB2 | NA |
| NOTE 1: This sub-group generates IM2. NOTE 2: This sub-group generates IM3 NOTE 3: This sub-group generates IM2 and IM3. |  |

Table B.1.12.1-2: Definition of Frequency Groups

| Frequency Group | Frequency Range (MHz) |
| --- | --- |
| VHF | 400.0 – 458.0 |
| LB | 662.0 – 916.0 |
| MLB | 1426.0 – 1518.0 |
| MB | 1626.0 – 2025.0 |
| HB | 2300.0 – 2690.0 |
| UHB1 | 3300.0 – 4201.0 |
| UHB1 | 4400.0 – 5000.0 |

#### B.1.12.2	Applicable EN-DC band combinations for verifying A-GNSS sensitivity requirements

The A-GNSS sensitivity requirements in clause 5.1 when in EN-DC operation mode shall be verified for EN-DC band combinations that can generate second or third order intermodulation products falling into the following GNSS receiver bands for the particular GNSS (where supported by the UE):

-	GPS L1 C/A:				1574.3970 – 1576.4430 MHz

-	Galileo E1 / GPS L1C:	1573.3740 – 1577.4660 MHz

-	GLONASS G1:			1597.5515 – 1605.8860 MHz

-	BDS B1I:					1559.0520 – 1563.1440 MHz

For each frequency group combination in Table B.1.12.2-1 only one EN-DC band combination need to be tested for the supported GNSS.

Table B.1.12.2-1: EN-DC band combinations for verifying A-GNSS sensitivity requirements

| Frequency Group Combination | EN-DC band combinations |  |  |
| --- | --- | --- | --- |
|  | GPS L1 / Galileo E1 | GLONASS G1 | BDS B1 |
| Group LB-LB | DC_20A_n28A DC_28A_n5A DC_20A_n83A | DC_8A_n28A DC_20A_n28A DC_28A_n8A DC_20A_n83A | DC_5A_n12A DC_12A_n5A DC_20A_n28A DC_28A_n5A DC_20A_n83A |
| Group LB-MLB | NA | NA | NA |
| Group LB-MB | NA | NA | NA |
| Group LB-HB | DC_5A_n40A DC_28A_n40A | DC_8A_n41A DC_28A_n40A | DC_5A_n40A DC_28A_n40A |
| Group LB-UHB1 | DC_8A_n77A DC_8A_n78A DC_20A_n77A DC_20A_n78A | DC_5A_n78A DC_8A_n77A DC_8A_n78A DC_20A_n77A DC_20A_n78A DC_26A_n77A DC_26A_n78A | DC_8A_n77A DC_8A_n78A |
| Group LB-UHB2 | NA | NA | NA |
| Group MLB-LB | NA | NA | NA |
| Group MLB-MLB | NA | NA | NA |
| Group MLB-MB | NA | NA | NA |
| Group MLB-HB | NA | NA | NA |
| Group MLB-UHB1 | NA | NA | NA |
| Group MLB-UHB2 | DC_11A_n79A DC_21A_n79A | DC_11A_n79A DC_21A_n79A | DC_11A_n79A DC_21A_n79A |
| Group MB-LB | NA | NA | NA |
| Group MB-MLB | NA | NA | NA |
| Group MB-MB | DC_1A_n3A DC_2A_n66A DC_3A_n1A DC_66A_n2A DC_66A_n25A DC_1A_n80A DC_3A_n84A | DC_1A_n3A DC_2A_n66A DC_3A_n1A DC_66A_n2A DC_66A_n25A DC_1A_n80A DC_3A_n84A | DC_1A_n3A DC_2A_n66A DC_3A_n1A DC_66A_n2A DC_66A_n25A DC_1A_n80A DC_3A_n84A |
| Group MB-HB | DC_1A_n40A | DC_1A_n7A | DC_1A_n7A |
| Group MB-UHB1 | DC_1A_n77A DC_1A_n78A DC_2A_n78A DC_3A_n77A DC_3A_n78A DC_4A_n78A DC_39A_n78A DC_66A_n78A | DC_1A_n77A DC_1A_n78A DC_2A_n78A DC_3A_n77A DC_3A_n79A DC_4A_n78A DC_39A_n78A DC_66A_n78A | DC_1A_n77A DC_1A_n78A DC_2A_n78A DC_3A_n77A DC_3A_n79A DC_4A_n78A DC_39A_n78A DC_66A_n78A |
| Group MB-UHB2 | DC_3A_n79A | NA | DC_3A_n79A |
| Group HB-LB | NA | DC_7A_n8A | NA |
| Group HB-MLB | NA | NA | NA |
| Group HB-MB | DC_40A_n1A | DC_40A_n1A | DC_40A_n1A |
| Group HB-HB | NA | NA | NA |
| Group HB-UHB1 A: | DC_40A_n77A | DC_40A_n77A | DC_40A_n77A |
| B: | DC_7A_n78A DC_38A_n78A DC_41A_n78A | DC_7A_n78A DC_38A_n78A DC_41A_n78A | DC_7A_n78A DC_38A_n78A DC_41A_n78A |
| C: | DC_7A_n77A DC_41A_n77A | DC_7A_n77A DC_41A_n77A | DC_7A_n77A DC_41A_n77A |
| Group HB-UHB2 | NA | NA | NA |

#### B.1.12.3	Test frequencies for EN-DC band combinations

For verifying the sensitivity requirements in clause 5.1 in EN-DC operation mode, the E-UTRA and NR frequency and channel configuration shall be selected to ensure the intermodulation products fall into the GNSS receiver bands as defined in clause B.1.12.2 for the particular GNSS.

Table B.1.12.3-1 shows exemplary test frequencies and RB settings for GPS L1/Galileo E1 for one EN-DC band combination in each of the Frequency Group Combinations defined in clause B1.12.2.

Table B.1.12.3-1: Exemplary EN-DC Test Configuration for GPS L1/Galileo E1 for one EN-DC Band Combination per Frequency Group Combination

| Frequency Group Combination | EN-DC Band Combination | Bandwidth LTE / NR (SCS) [MHz] | UL Fc LTE / NR [MHz] | UL ARFCN LTE / NR | No. of UL RBs LTE / NR | Start UL RB LTE / NR |
| --- | --- | --- | --- | --- | --- | --- |
| LB-LB | DC_20A_n28A | 5 / 5 (15 kHz) | 834.50 / 740.50 | 24175 / 148100 | 6 / 6 | 9 / 9 |
| LB-HB | DC_5A_n40A | 5 / 5 (15 kHz) | 826.50 / 2397.50 | 20425 / 479500 | 6 / 6 | 0 / 19 |
| LB-UHB1 | DC_8A_n77A | 3 / 10 (30 kHz) | 881.50 / 3337.50 | 21465 / 622500 | 6 / 3 | 0 / 10 |
| MLB-UHB2 | DC_11A_n79A | 5 / 40 (30 kHz) | 1429.40 / 4434.50 | 22765 / 695633 | 6 / 3 | 9 /51 |
| MB-MB | DC_1A_n3A | 5 / 5 (15 kHz) | 1922.50 / 1747.50 | 18025 / 349500 | 6 / 6 | 0 / 9 |
| MB-HB | DC_1A_n40A | 5 / 5 (15 kHz) | 1939.50 / 2305.50 | 18195 / 460900 | 6 / 6 | 9 / 9 |
| MB-UHB1 | DC_1A_n77A | 5 / 10 (30 kHz) | 1922.50 / 3497.50 | 18025 / 633166 | 6 / 3 | 9 / 10 |
| MB-UHB2 | DC_3A_n79A | 3 / 40 (30 kHz) | 1711.50 / 4980.00 | 19215 / 732000 | 6 / 3 | 4 / 101 |
| HB-MB | DC_40A_n1A | 5 / 5 (15 kHz) | 2340.50 / 1939.50 | 38695 / 387900 | 6 / 6 | 9 / 9 |
| HB-UHB1 | DC_7A_n78A | 5 / 10 (30 kHz) | 2502.50 / 3427.50 | 20775 / 628500 | 6 / 3 | 3 / 10 |
| HB-UHB1 | DC_7A_n77A | 5 / 10 (30 kHz) | 2502.50 / 4077.50 | 20775 / 671833 | 6 / 3 | 9 / 10 |