3GPP TSG- RAN WG#4 Meeting #100-e	R4-2114236

# Electronic Meeting,  16-27 August 2021

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.880 | CR | 0001 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Cr for 37.880: n71 filter data |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | T-Mobile USA |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R-4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_HPUE_FWVM-Core |  |  |  |  |  | Date: |  |  | 2021-08-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Filter data for n71 needed as per the WID. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adds data for n71 filters, as well as updated and additional specs for other filters. Also added the definition of FWA and additional filter data for B5 and B12. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | n71 filter data, definition of FWA and low power filter data for B5 and B12 will be missing from the TR. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.3, 7.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<First Changed Section>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

3GPP	3rd Generation Partnership Project

ACLR	Adjacent Channel Leakage Ratio

A-MPR	Additional Maximum Power Reduction

BB	Base Band

BS	Base Station

BW	Bandwidth

CL	Coupling Loss

DL	Downlink

FWA	Fixed Wireless Access

HPUE	High Power User Equipment

IF	Intermediate Frequency

ITU	International Telecommunication Union

LTE	Long Term Evolution

MOP	Maximum Output Power

MPR	Allowed Maximum Power Reduction

MTC	Machine-Type Communications

NB-IoT	Narrowband – Internet of Things

NB	Narrowband

NR	New Radio

OOBE	Out Of Band Emissions

PA	Power Amplifier

PC	Power Class

PRB	Physical Resource Block

PSNB	Public Safety Narrowband

RAN4	Radio Access Network Working Group 4

REFSENS	Reference Sensitivity

RF	Radio Frequency

Rx	Receiver

SW	Software

TR	Technical Report

Tx	Transmitter

UE	User Equipment

<Next Changed Section>

# 7	UE hardware

Editor note: This clause the third objective of the study item.

## 7.1	New RF components

During Rel-11 when band 14 PC1 requirements were developed Technical report Public safety broadband high power User Equipment (UE) for band 14 [3] and a contribution [11] discussed necessary hardware changes to realize PC1 vehicular mobile form factor compared to PC3 normal handheld formfactor. These studies in [3][11] are considered to be relevant also to this WI targeting high-power UE operation for fixed-wireless/vehicle-mounted use cases in LTE bands 5 and 12 and NR band n71.

Main take away from [3][11] is that in order to minimize implementation complexity and utilize the Power Class 3 eco-system it was considered that changes to the baseband IC and RF IC should be avoided and only changes to the discrete RF combining front end elements are considered. This manifests mainly to need of frontend filters with better power handling capabilities. Suitable technologies mentioned were ceramic-filters and cavity-filters. Furthermore power amplifiers that are capable for PC1 operation are obviously necessity. A few filter examples are presented below with input power capabilities of 43 dBm which obviously in more than needed for PC1 operation even considering the post PA-losses but this demonstrates that capable technology is available.

Band 5 duplex-filter  UMD005A (ctscorp.com)

Band 5/partial Band 26 duplex-filter UMD026B (ctscorp.com)

Band 12 duplex-filter  UMD012A (ctscorp.com)

Band 71/n71 duplex-filter UMD071A (ctscorp.com)

In addition to the above filters with 43 dBm capability, there are also filters with input power capabilities of 38 dBm, that are smaller and may be more appropriate for FWA devices.

Band 5 duplex-filter USD005A (ctscorp.com)

Band 5/full Band 26 duplex-filter  USD026A (ctscorp.com)

Band 12 duplex-filter USD012A (ctscorp.com)

Band 71/n71 Band 85 duplex-filter USD7185A (ctscorp.com)

<End of changes>