3GPP TSG-RAN WG4 Meeting #100-e	R4-2114272

Electronic Meeting, 16 – 27 August, 2021

Title: 	Discussion on remaining issues for PRS-RSRP measurement requirements

Source: 	Huawei, HiSilicon

Agenda item:	6.1.6.1.2

Document for:	Discussion

Introduction

RRM requirements for PRS-RSRP measurement were discussed in RAN4#99-e, and the outcomes are captured in the WF [1]. Based on [1] the following issues are to be further discussed:

- Applicable requirements for PRS-RSRP configured for DL-TDOA

In this paper we will provide our views on the above open issues for PRS-RSRP measurement.

Discussion

- Applicable requirements for PRS-RSRP configured for DL-TDOA - Option 1 (HW, Nokia, Intel) - Requirements for RSTD in clause 9.9.2 apply - Option 2a (ZTE, vivo, OPPO, Ericsson) - Requirements for PRS-RSRP in clause 9.9.3 apply - Option 2b (OPPO) - Current requirements in clause 9.9.3 also apply for the case when PRS-RSRP is measured for DL-TDOA, except the following scenarios: - If handover occurs while PRS-RSRP measurements for Multi-RTT are being performed, or - If other cell change impacting SRS configuration occurs while PRS-RSRP measurements for Multi-RTT are being performed - When PRS-RSRP is configured for multi-RTT, UE Rx-Tx time difference and PRS-RSRP measurements are performed over the same measurement period

In RAN4#98-bis-e, it was agreed that when PRS-RSRP is configured for DL-TDOA, RSTD and PRS-RSRP measurements are performed over the same measurement period.

In our view, this agreement means that when PRS-RSRP is configured for DL-TDOA, the measurement period requirements will be specified in clause 9.9.2 of 38.133 which defines RSTD measurement period, or more accurately the measurement period for DL-TDOA. In this sense, there is no need to apply the requirements in clause 9.9.3 for PRS-RSRP measurement configured for DL-TDOA.

This would also resolve the compatibility issue in option 2b, e.g. in case of HO, UE would continue the PRS-RSRP measurement if configured for DL-TDOA (same as RSTD), and would restart the PRS-RSRP measurement if configured for multi-RTT (same as UE Rx-Tx).

It is noted that with the current wording, some requirements in clause 9.9.3 would apply for PRS-RSRP configured for DL-TDOA, i.e. in the end of the clause 9.9.3.5, the following two paragraphs are about PRS-RSRP configured for DL-TDOA and multi-RTT, and we believe they should be removed.

When the PRS-RSRP measurement is configured together with UE Rx-Tx time difference measurement, the UE behaviour at a serving cell (SpCell or SCell) change for the PRS-RSRP measurement is the same as the UE behaviour for the UE Rx-Tx time difference measurement specified in clause 9.9.4.5, and the PRS-RSRP measurement shall meet the accuracy requirements in clause 10.1.24. When the PRS-RSRP measurement is configured together with RSTD measurement, the UE behaviour at a serving cell (SpCell or SCell) change for the PRS-RSRP measurement is the same as the UE behaviour for the RSTD measurement specified in clause 9.9.2.5, and the PRS-RSRP measurement shall meet the accuracy requirements in clause 10.1.24.

Proposal 1: Remove requirements related to PRS-RSRP for DL-TDOA or multi-RTT in clause 9.9.3.

Conclusions

In this paper we provided our views on remaining issues for PRS-RSRP measurement requirements.

Proposal 1: Remove requirements related to PRS-RSRP for DL-TDOA or multi-RTT in clause 9.9.3.

Reference

- R4-2108294, WF on UE PRS measurement requirements, Huawei, HiSilicon