3GPP TSG-RAN4 Meeting # 100-e	R4-2114321

# Electronic Meeting,  16th– 27th August 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.176-1 | CR | <CR#> | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on conducted performance specification of IAB - Others |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IAB |  |  |  |  |  | Date: |  |  | 2021-8-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | 3.1: there are terminology for OTA and not used in conducted perforamnce specification, should be removed 3.2: some symbol related to antenna connector should be removed 4.13: text improvement is needed |  |  |  |  |  |  |  |  |
| Summary of change: |  | 3.1: Remove terminology for OTA 3.2: Remove some symbol related to antenna connector of Type 1-C 4.13: text improved |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Errors in specificaitons. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 3.2, 4.13 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## < start of changes >

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

aggregated IAB channel bandwidth: the RF bandwidth in which a IAB-DU or IAB-MT transmits and receives multiple contiguously aggregated carriers. The aggregated IAB channel bandwidth is measured in MHz

active transmitter unit: transmitter unit which is ON, and has the ability to send modulated data streams that are parallel and distinct to those sent from other transmitter units to one or more IAB type 1-H TAB connectors at the transceiver array boundary

basic limit: emissions limit relating to the power supplied by a single transmitter to a single antenna transmission line in ITU-R SM.329 [5] used for the formulation of unwanted emission requirements for FR1

beam: beam (of the antenna) is the main lobe of the radiation pattern of an antenna array

NOTE:	For certain antenna array, there may be more than one beam.

Channel edge: lowest or highest frequency of the NR carrier, separated by the IAB-MT channel bandwidth or IAB-DU channel bandwidth.

Carrier aggregation: aggregation of two or more component carriers in order to support wider transmission bandwidths

Carrier aggregation configuration: a set of one or more operating bands across which the IAB-DU or IAB-MT aggregates carriers with a specific set of technical requirements

co-location reference antenna: a passive antenna used as reference for co-location requirements

Contiguous spectrum: spectrum consisting of a contiguous block of spectrum with no sub-block gap(s).

fractional bandwidth: fractional bandwidth FBW is defined as FBW=200∙FFBWhigh-FFBWlowFFBWhigh+FFBWlow%

highest carrier: The carrier with the highest carrier frequency transmitted/received in a specified frequency band.

IAB-donor: gNB that provides network access to UEs via a network of backhaul and access links.

IAB-DU channel bandwidth: RF bandwidth supporting a single IAB-DU RF carrier with the transmission bandwidth configured in the uplink or downlink

NOTE 1:	The IAB-DU channel bandwidth is measured in MHz and is used as a reference for transmitter and receiver RF requirements.

NOTE 2:	It is possible for the IAB to transmit to and/or receive from one or more UE bandwidth parts that are smaller than or equal to the IAB transmission bandwidth configuration, in any part of the IAB transmission bandwidth configuration.

IAB-DU RF Bandwidth: RF bandwidth in which an IAB-DU transmits and/or receives single or multiple carrier(s) within a supported operating band

IAB-DU RF Bandwidth edge: frequency of one of the edges of the IAB-DU RF Bandwidth.

IAB-MT channel bandwidth: RF bandwidth supporting a single IAB-MT RF carrier with the transmission bandwidth configured in the uplink or downlink

NOTE 1:	The IAB-MT channel bandwidth is measured in MHz and is used as a reference for transmitter and receiver RF requirements.

IAB-MT RF Bandwidth: RF bandwidth in which an IAB-MT transmits and/or receives single or multiple carrier(s) within a supported operating band

NOTE: 	 In single carrier operation, the IAB-MT RF Bandwidth is equal to the IAB-MT channel bandwidth.

IAB-MT RF Bandwidth edge: frequency of one of the edges of the IAB-MT RF Bandwidth.

IAB RF Bandwidth: RF bandwidth in which an IAB-DU or IAB-MT transmits and/or receives single or multiple carrier(s) within a supported operating band

IAB RF Bandwidth edge: frequency of one of the edges of the IAB RF Bandwidth.

IAB type 1-H: IAB-DU or IAB-MT operating at FR1 with a requirement set consisting of conducted requirements defined at individual TAB connectors and OTA requirements defined at RIB

IAB type 1-O: IAB-DU or IAB-MT operating at FR1 with a requirement set consisting only of OTA requirements defined at the RIB

IAB type 2-O: IAB-DU or IAB-MT operating at FR2 with a requirement set consisting only of OTA requirements defined at the RIB

inter-band gap: The frequency gap between two supported consecutive operating bands.

Inter RF Bandwidth gap: frequency gap between two consecutive IAB-DU or IAB-MT RF Bandwidths that are placed within two supported operating bands

lowest Carrier:	The carrier with the lowest carrier frequency transmitted/received in a specified frequency band.

maximum carrier output power: mean power level measured per carrier at the indicated interface, during the transmitter ON period in a specified reference condition

measurement bandwidth: RF bandwidth in which an emission level is specified

multi-band connector: TAB connector of IAB type 1-H associated with a transmitter or receiver that is characterized by the ability to process two or more carriers in common active RF components simultaneously, where at least one carrier is configured at a different operating band than the other carrier(s) and where this different operating band is not a sub-band or superseding-band of another supported operating band

multi-band RIB: operating band specific RIB associated with a transmitter or receiver that is characterized by the ability to process two or more carriers in common active RF components simultaneously, where at least one carrier is configured at a different operating band than the other carrier(s) and where this different operating band is not a sub-band or superseding-band of another supported operating band

Non-contiguous spectrum: spectrum consisting of two or more sub-blocks separated by sub-block gap(s).

operating band: frequency range in which NR operates (paired or unpaired), that is defined with a specific set of technical requirements

NOTE:	The operating band(s) for an IAB-DU and IAB-MT are declared by the manufacturer

Parent node: IAB-MT's next hop neighbour node; the parent node can be IAB-node or IAB-donor.

Radio Bandwidth: frequency difference between the upper edge of the highest used carrier and the lower edge of the lowest used carrier

rated carrier output power: mean power level associated with a particular carrier the manufacturer has declared to be available at the indicated interface, during the transmitter ON period in a specified reference condition

rated total output power: mean power level associated with a particular operating band the manufacturer has declared to be available at the indicated interface, during the transmitter ON period in a specified reference condition

requirement set:	 one of the NR requirement sets as defined for IAB type 1-H, IAB type 1-O, and IAB type 2-O

single-band connector: IAB type 1-H TAB connector supporting operation either in a single operating band only, or in multiple operating bands but does not meet the conditions for a multi-band connector.

sub-band: A sub-band of an operating band contains a part of the uplink and downlink frequency range of the operating band.

sub-block: one contiguous allocated block of spectrum for transmission and reception by the same IAB-DU or IAB-MT

NOTE:	There may be multiple instances of sub-blocks within a IAB RF Bandwidth.

sub-block gap: frequency gap between two consecutive sub-blocks within a IAB RF Bandwidth, where the RF requirements in the gap are based on co-existence for un-coordinated operation

superseding-band: A superseding-band of an operating band includes the whole of the uplink and downlink frequency range of the operating band.

TAB connector: transceiver array boundary connector

TAB connector RX min cell group: operating band specific declared group of TAB connectors to which IAB type 1-H conducted RX requirements are applied

NOTE:	Within this definition, the group corresponds to the group of TAB connectors which are responsible for receiving a cell when the IAB type 1-H setting corresponding to the declared minimum number of cells with reception on all TAB connectors supporting an operating band, but its existence is not limited to that condition

TAB connector TX min cell group: operating band specific declared group of TAB connectors to which IAB type 1-H conducted TX requirements are applied.

NOTE:	Within this definition, the group corresponds to the group of TAB connectors which are responsible for transmitting a cell when the IAB type 1-H setting corresponding to the declared minimum number of cells with transmission on all TAB connectors supporting an operating band, but its existence is not limited to that condition

transceiver array boundary: conducted interface between the transceiver unit array and the composite antenna

transmission bandwidth: RF Bandwidth of an instantaneous transmission from an IAB-DU or IAB-MT, measured in resource block units

transmitter OFF period: time period during which the IAB-DU or IAB-MT transmitter is not allowed to transmit

transmitter ON period: time period during which the IAB-DU or IAB-MT transmitter is transmitting data and/or reference symbols

transmitter transient period: time period during which the transmitter is changing from the OFF period to the ON period or vice versa

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

	Percentage of the mean transmitted power emitted outside the occupied bandwidth on the assigned channel

BWChannel	BS channel bandwidth

BWChannel_CA	Aggregated BS channel bandwidth, expressed in MHz. BWChannel_CA= Fedge_high- Fedge_low.

BWConfig	Transmission bandwidth configuration, expressed in MHz, where BWConfig = NRB x SCS x 12 kHz

BWtot	Total RF bandwidth

f	Separation between the channel edge frequency and the nominal -3 dB point of the measuring filter closest to the carrier frequency

fmax	f_offsetmax minus half of the bandwidth of the measuring filter

ΔfOBUE	Maximum offset of the operating band unwanted emissions mask from the downlink operating band edge

ΔfOOB	Maximum offset of the out-of-band boundary from the uplink operating band edge

FC	RF reference frequency on the channel raster

FC,block, high	Fc of the highest transmitted/received carrier in a sub-block

FC,block, low	Fc of the lowest transmitted/received carrier in a sub-block

FC_low	The Fc of the lowest carrier, expressed in MHz

FC_high	The Fc of the highest carrier, expressed in MHz

Fedge_low	The lower edge of aggregated BS channel bandwidth, expressed in MHz. Fedge_low = FC_low - Foffset_low

Fedge_high	The upper edge of aggregated BS channel bandwidth, expressed in MHz. Fedge_high = FC_high + Foffset_high.

Foffset_high	Frequency offset from FC_high to the upper Base Station RF Bandwidth edge, or from FC,block, high to the upper sub-block edge

Foffset_low	Frequency offset from FC_low to the lower Base Station RF Bandwidth edge, or from FC,block, low to the lower sub-block edge

FDL_low	The lowest frequency of the downlink operating band

FDL_high	The highest frequency of the downlink operating band

f_offset	Separation between the channel edge frequency and the centre of the measuring filter

f_offsetmax	The offset to the frequency ΔfOBUE outside the downlink operating band

FDL_low	The lowest frequency of the downlink operating band

FDL_high	The highest frequency of the downlink operating band

FUL_low	The lowest frequency of the uplink operating band

FUL_high	The highest frequency of the uplink operating band

Iuant	gNB internal logical interface between the implementation specific O&M function and the RET antennas and TMAs control unit function of the gNB

Ncells	The declared number corresponding to the minimum number of cells that can be transmitted by an BS type 1-H in a particular operating band

NRB	Transmission bandwidth configuration, expressed in resource blocks

NRXU,active	The number of active receiver units. The same as the number of demodulation branches to which compliance is declared for chapter 8 performance requirements

NRXU,counted	The number of active receiver units that are taken into account for conducted Rx spurious emission scaling, as calculated in clause 7.6.1

NRXU,countedpercell	The number of active receiver units that are taken into account for conducted RX spurious emissions scaling per cell, as calculated in clause 7.6.1

NTXU,counted	The number of active transmitter units as calculated in clause 6.1, that are taken into account for conducted TX output power limit in clause 6.2.1, and for unwanted TX emissions scaling

NTXU,countedpercell	The number of active transmitter units that are taken into account for conducted TX emissions scaling per cell, as calculated in clause 6.1

Pmax,c,TABC	The maximum carrier output power per TAB connector

Prated,c,sys	The sum of Prated,c,TABC for all TAB connectors for a single carrier

Prated,c,TABC	The rated carrier output power per TAB connector

Prated,t,TABC	The rated total output power declared at TAB connector

PREFSENS	Conducted Reference Sensitivity power level

Wgap	Sub-block gap or Inter RF Bandwidth gap size

## << Unchanged part is omitted>>

## 4.13 	Test efficiency optimization

When manufacture declares the same RF implementation for IAB-MT and IAB-DU (D.IAB-1) and the declarations in table 4.13-1 are the same for IAB-DU and IAB-MT, it is sufficient to test only IAB-MT or IAB-DU with the test requirement applicability according to Table 4.13-2 for Tx requirements and Table 4.13-3 for Rx requirements.

For IAB type 1-H it is required that the DUT selection between requirements follows following rules:

-	Out of maximum output transmit power, modulation quality and ACLR, operating band unwanted emissions and transmitter general spurious emissions, IAB-DU and IAB-MT are required to be the DUT at least once,

-	Out of receiver requirements of reference sensitivity, receiver spurious, receiver intermodulation, IAB-DU and IAB-MT are required to be the DUT at least once.

In some cases, the test requirements are the same but the MU for the IAB-MT is larger than for the IAB-DU. In cases where the test efficiency optimization is applicable the lower MU value should be used.

Table 4.13-1: Declarations required to be the same for IAB-DU and IAB-MT for test efficiency optimization to apply

| Declaration identifier | Declaration | Additional conditions | Applicability |  |
| --- | --- | --- | --- | --- |
|  |  |  | IAB-DU type 1-H | IAB-MT type 1-H |
| D.2 | IAB class | Medium range IAB-DU can apply test efficiency optimization with wide area IAB-MT other declarations in this table are the same. | x | x |
| D.3 | Operating bands and frequency ranges |  | x | x |
| D.11 | Maximum IAB RF Bandwidth |  | x | x |
| D.12 | Maximum IAB RF Bandwidth for multi-band operation |  | x | x |
| D.13 | Total RF bandwidth (BWtot) |  | x | x |
| D.14 | NR supported channel bandwidths and SCS |  | x | x |
| D.15 | CA only operation |  | x | x |
| D.16 | Single or multiple carrier |  | x | x |
| D.17 | Maximum number of supported carriers per operating band in single band operation |  | x | x |
| D.18 | Maximum number of supported carriers per operating band in multi-band operation | Required only in case multi-band operation is supported | x | x |
| D.19 | Total maximum number of supported carriers in multi-band operation |  | x | x |
| D.20 | Other band combination multi-band restrictions |  | x | x |
| D.21 | Rated carrier output power (Prated,c,AC, or Prated,c,TABC) |  | x | x |
| D.22 | Rated total output power (Prated,t,AC, or Prated,t,TABC) |  | x | x |
| D.23 | Rated multi-band total output power, Prated,MB,TABC |  | x | x |
| NOTE: Manufacturer declarations applicable per IAB requirement set were marked as "x". Manufacturer declarations not applicable per IAB requirement set were marked as "n/a". |  |  |  |  |

Table 4.13-2: Test requirement applicability for TX requirements

| Tx requirement |  | Test efficiency optimization applicable | Test requirement applicability (Note) |
| --- | --- | --- | --- |
| Maximum output power |  | Yes |  |
| Output power dynamics (only for IAB-DU) |  | No | n/a |
| Output power dynamics (only for IAB-MT) |  | No | n/a |
| Transmitter OFF power |  | Yes |  |
| Transient period |  | Yes |  |
| IAB-DU Frequency Error |  | No | n/a |
| IAB-MT Frequency Error |  | No | n/a |
| Modulation quality |  | Yes |  |
| Time alignment error (only for IAB-DU) |  | No | n/a |
| Occupied bandwidth |  | Yes |  |
| ACLR |  | Yes |  |
| Operating band unwanted emission |  | Yes |  |
| Transmitter spurious emission | General requirement | Yes |  |
|  | Additional spurious emissions | Yes |  |
|  | Co-location with other base stations | Yes |  |
| OTA transmitter intermodulation |  | Yes |  |
| Note: Test requirement applicability defines how to select whether IAB-DU or IAB-MT test requirement is applied. In case no applicability definition is provided or despite the applicability definition test requirement is the same for IAB-DU and IAB-MT, either can apply. |  |  |  |

Table 4.13-3: Test requirement applicability for receiver requirements

| Rx requirement |  | Test efficiency optimization applicable | Test requirement applicability (Note) |
| --- | --- | --- | --- |
| Reference sensitivity |  | Yes |  |
| Dynamic range (only for IAB-DU) |  | No | n/a |
| Adjacent channel selectivity |  | Yes | IAB-MT |
| In-band blocking |  | Yes | IAB-MT |
| Out-of-band blocking | General requirement | Yes | IAB-MT |
|  | Co-location requirement | Yes |  |
| Receiver spurious emissions |  | Yes |  |
| Receiver intermodulation |  | Yes |  |
| In-channel selectivity (only for IAB-DU) |  | No | n/a |
| Note: Test requirement applicability defines how to select whether IAB-DU or IAB-MT test requirement is applied. In case no applicability definition is provided or despite the applicability definition test requirement is the same for IAB-DU and IAB-MT, either can apply. |  |  |  |

## < end of changes >