3GPP TSG-RAN WG4 Meeting # 100-e 			R4-2114371

# Electronic Meeting, August 16-27, 2021

Source: 		Nokia, Nokia Shanghai Bell

Title: 			On 1900MHz RMR RAN4 BS RF requirements impact due to ECC Decision (20)02

Agenda Item: 	8.5.3

Document for:	Approval

# Introduction

During RAN#92e meeting, a revised Work Item [1] has been approved on introduction of Rail Mobile Radio (RMR) 1900MHz spectrum (Uplink/Downlink: 1900 MHz – 1910 MHz). This document discusses the impact to RAN4 BS RF requirements due to ECC Decision (20)02.

# Discussion

Below is the summary of relevant ECC Decision 20(02) requirements relevant to objectives of this Work Item. Proposals have been made how they should be addressed (on top of generic changes applicable to introduction of each 3GPP frequency band) in RAN4 specifications.

Table 1: General in-block requirement
mandatory for uncoordinated deployment

| RMR channel BW | Maximum e.i.r.p. |
| --- | --- |
| 10 MHz | = 65 dBm/10MHz (Note 1) |
| Note 1: In case an administration wishes to allow higher e.i.r.p., coordination or other mitigation measures are required. |  |

Table 2: Baseline requirement

| Frequency range | e.i.r.p. limit |
| --- | --- |
| 1920-1980 MHz | -43 dBm/5MHz |

Proposal 1: Since additional ECC BS transmitter requirements are not conducted, it is proposed to define them as 3GPP BS declarations.

Table 3: Requirements on wideband RMR BS receiver characteristics

| Parameter | Value |
| --- | --- |
| Level of the wanted signal | RefSens + 3 dB |
| Maximum 5 MHz LTE interfering signal in 1805-1880 MHz | -20 dBm |
| The antenna connector of the BS receiver is the reference point. The reference sensitivity (RefSens) is the minimum mean power received at the antenna connector at which a specified minimum performance shall be met. These requirements cover both blocking and third-order intermodulation. |  |

Proposal 2: It is proposed to consider new blocking requirement (conducted) for 1900MHz RMR band.

# Conclusion

This document proposes how to address ECC requirements in 3GPP specifications. The following proposals are made:

Proposal 1: Since additional ECC BS transmitter requirements are not conducted, it is proposed to define them as 3GPP BS declarations.

Proposal 2: It is proposed to consider new blocking requirement (conducted) for 1900MHz RMR band.

# References

[1] RP-211542	Revised WID on introduction of Rail Mobile Radio (RMR) 1900MHz spectrum; UIC; RAN#92-e