3GPP TSG-RAN WG4 Meeting # 100-e	R4-2114409

# Electronic Meeting, August 16-27, 2021

Source: 	Huawei

Title: 	Further discussion on channel raster aspects for RMR900

Agenda Item:

Document for:	Discussion

# Introduction

During previous RAN4#99-e meeting discussion on the channel raster aspects for RMR 900 work item [1] was initiated in [3]. In this contribution provide analysis of the channel raster aspects of the RMR900 topic.

# Channel raster considerations

ECC decision [2] states that 919.6 MHz is the lower edge of the lowest RB for RMR900:

According to the WID [4], RMR900 is requested to define 5MHz channel bandwidth with 15 kHz SCS. Referring to the TS 38.104 section 5.3, the following parameters apply:

NRB = 25

Minimum guardband = 242.5 kHz

Our interpretation of the above-referred ECC decision is depicted on the picture below, where the NR channel bandwidth and transmission bandwidth configuration were embedded:

- Lowest edge of the lowest RB is marked at 919.6 MHz

- Spectrum block starts at 919.4 MHz

- Tthe left most PRB would be at 919.4 + 0.242 MHz = 919.642 MHz

- The lower edge of the lowest RB is according to [2], i.e. ≥ 919.6 MHz

The above figure indicate that spectrum available for the RMR900 DL is 919.6 – 925 MHz, and such frequency range shall be captured by the NR-ARFCN.

Observation 1: the NR-ARFCN allocation for the RMR900 DL shall reflect the 919.6 to 925 MHz range.

# Conclusion

Based on the discussion, the following observations on channel raster were formulated:

Observation 1: the NR-ARFCN allocation for the RMR900 DL shall reflect the 919.6 to 925 MHz range.

# References

[1] 	RP-210878		New WID on introduction of 900 MHz spectrum to 5G NR applicable for Rail Mobile Radio, UIC

[2]	ECC Decision (20)02	Harmonised use of the paired frequency bands 874.4-880.0 MHz and 919.4-925.0 MHz and of the unpaired frequency band 1900-1910 MHz for Railway Mobile Radio (RMR), November 2020

[3]	R4-2111054	Discussion on channel raster aspects for RMR900, RAN4#99-e

[4]	RP-211495	Introduction of 900MHz NR band for Europe for Rail Mobile Radio (RMR), RAN#92-e