3GPP TSG-RAN WG4 Meeting#100-e	R4-2114474

Online, 16th Aug – 27th Aug, 2021

Source: 	Qualcomm Incorporated

Title: 	Simulation Results for Application Layer Throughput Tests

Agenda item:	10.7.2

Document for:	Discussion

- Introduction

In RAN4#99-e, RAN4 agreed on simulation assumptions in [1] to study the feasibility of defining requirements with link adaptation. In this paper, we provide simulation results based on [1].

- Simulation Results

Based on simulation assumptions in [1], below are the link adaptation results for each case:

Table 1: Link Adaptation Throughput results for FR1 FDD 10MHz, 15kHz

| SNR (dB) | 2x2 |  |  |  | 2x4 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Median CQI | Median Rank | BLER (%) | Throughput (Mbps) | Median CQI | Median Rank | BLER (%) | Throughput (Mbps) |
| 0 | 3 | 1 | 23.4% | 3.28 | 3 | 1 | 5.3% | 5.88 |
| 2 | 4 | 1 | 13.0% | 4.6 | 5 | 1 | 8.6% | 7.56 |
| 4 | 4 | 1 | 8.0% | 6.21 | 4 | 2 | 12.0% | 9.98 |
| 6 | 5 | 1 | 7.9% | 7.95 | 5 | 2 | 10.7% | 13.11 |
| 8 | 6 | 1 | 8.9% | 9.88 | 6 | 2 | 9.6% | 16.53 |
| 10 | 8 | 1 | 9.9% | 12.16 | 6 | 2 | 6.4% | 20.24 |
| 12 | 8 | 1 | 10.0% | 14.35 | 8 | 2 | 7.3% | 23.75 |
| 14 | 6 | 2 | 10.6% | 17.01 | 9 | 2 | 7.9% | 27.74 |
| 16 | 6 | 2 | 11.7% | 19.67 | 10 | 2 | 9.3% | 31.64 |
| 18 | 8 | 2 | 13.0% | 22.42 | 10 | 2 | 13.2% | 34.49 |
| 20 | 9 | 2 | 16.2% | 24.8 | 12 | 2 | 15.9% | 37.43 |

Table 2: Link Adaptation Throughput results for FR1 TDD 40MHz, 30kHz

| SNR (dB) | 2x2 |  |  |  | 2x4 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Median CQI | Median Rank | BLER (%) | Throughput (Mbps) | Median CQI | Median Rank | BLER (%) | Throughput (Mbps) |
| 0 | 3 | 1 | 16.2% | 10.64 | 4 | 1 | 7.8% | 17.77 |
| 2 | 4 | 1 | 15.4% | 14.35 | 3 | 2 | 4.3% | 23.37 |
| 4 | 4 | 1 | 15.8% | 19.67 | 3 | 2 | 4.0% | 30.78 |
| 6 | 6 | 1 | 14.8% | 25.37 | 5 | 2 | 7.4% | 42.18 |
| 8 | 6 | 1 | 8.6% | 32.49 | 5 | 2 | 6.1% | 53.39 |
| 10 | 7 | 1 | 4.8% | 39.52 | 6 | 2 | 1.3% | 64.89 |
| 12 | 8 | 1 | 4.7% | 46.93 | 7 | 2 | 0.3% | 75.43 |
| 14 | 6 | 2 | 6.1% | 53.68 | 8 | 2 | 0.1% | 86.36 |
| 16 | 6 | 2 | 4.5% | 62.23 | 8 | 2 | 0.3% | 98.8 |
| 18 | 7 | 2 | 2.2% | 71.63 | 10 | 2 | 6.4% | 111.15 |
| 20 | 8 | 2 | 1.1% | 82.65 | 11 | 2 | 11.2% | 120.65 |

Table 3: Link Adaptation Throughput results for FR2 TDD 100MHz, 120kHz, 2x2

| SNR (dB) | Median CQI | Median Rank | BLER (%) | Throughput (Mbps) |
| --- | --- | --- | --- | --- |
| 0 | 5 | 1 | 20.2% | 16.53 |
| 2 | 6 | 1 | 10.4% | 22.99 |
| 4 | 7 | 1 | 7.4% | 30.12 |
| 6 | 8 | 1 | 6.4% | 38.76 |
| 8 | 9 | 1 | 6.2% | 46.84 |
| 10 | 10 | 1 | 8.0% | 56.05 |
| 12 | 11 | 1 | 8.7% | 68.12 |
| 14 | 9 | 2 | 8.0% | 80.85 |
| 16 | 9 | 2 | 7.4% | 94.15 |

- Alignment Results Criteria

In [1], one of the open issues was alignment results criteria.

- Alignment Results Criteria

  - Option 1: Absolute throughput span within X% of average throughput across companies at a given SNR.

    - Decide X based on simulation results. Possible values of X = [5]% or [10]%.

  - Option 2: SNR G±Gspan can be reached for the T% of maximum throughput

    - Maximum throughput is derived with TBS corresponding to CQI index 15 with rank 2 for 2Rx/4Rx UE.

    - Decide Gspan based on simulation results. Candidate option is Gspan = [2.5] dB.

We make following observations.

Observation 1: Throughput span in % is very large for lower SNR regime since absolute throughput itself is very low.

Observation 2: If RAN4 decides to define requirements for both Rank1 and Rank2 regime, SNR at T% of maximum throughput (Option 2) may be better for alignment.

Observation 3: If RAN4 decides to define requirements for both only Rank2 regime, as asked by RAN5, both options can be used for alignment.

Proposal 1: Use Option 2 for alignment results criteria.

- Conclusions

This paper provides simulation results for studying LA throughput requirements in RAN4. Following has been proposed.

Observation 1: Throughput span in % is very large for lower SNR regime since absolute throughput itself is very low.

Observation 2: If RAN4 decides to define requirements for both Rank1 and Rank2 regime, SNR at T% of maximum throughput (Option 2) may be better for alignment.

Observation 3: If RAN4 decides to define requirements for both only Rank2 regime, as asked by RAN5, both options can be used for alignment.

Proposal 1: Use Option 2 for alignment results criteria.

# References

[1] R4-2108102, “WF on 5G NR UE Application Layer Data Throughput Performance”, 3GPP TSG RAN WG4 Meeting #99-e, Online, May 19 – May 27, 2021.