3GPP TSG-RAN WG4 Meeting #100-e                                         R4-2114507

Electronic Meeting, 16 – 27 August 2021

Source: 	Huawei, HiSilicon

Title: 	On specific HPUE power class capability for NR V2X

Agenda Item:	9.15.6.1

Document for:	Approval

# Introduction

NR V2X power class capability was further discussed in last RAN4 meeting [1].

- Issue 1-1: NR V2X PC2 power class capability

- Options

  - OP1: Define power class capability for PC2

  - OP2: Define power class capability for both PC3 and PC2

  - OP3: No need to define new power class capability for NR-V2X

No conclusion was drawn for the these options above. This contribution provides our consideration on the issue.

# Discussion

As discussed in past meetings, there is a specific SL parameter for LTE-V2X to report the PC2 power class capability in TS 36.306.

#### 4.3.21.22 v2x-HighPower-r14 This parameter indicates whether the UE supports maximum transmit power associated with Power class 2 V2X UE for V2X sidelink transmission in a band, see TS 36.101 [6].

Though the capability is a SL parameter, it is reported via Uu RRC messages to the network.

BandParametersTxSL-r14 ::= SEQUENCE {

v2x-BandwidthClassTxSL-r14		V2X-BandwidthClassSL-r14,

v2x-eNB-Scheduled-r14			ENUMERATED {supported}				OPTIONAL,

v2x-HighPower-r14				ENUMERATED {supported}				OPTIONAL

}

Observation 1: Power class 2 UE capability for LTE-V2X is reported via Uu RRC message.

Since the capability is reported to network, as PC2 is also supported by NR-V2X, we think similar capability is also needed for NR-V2X.

Proposal 1: Specific PC2 power class for NR V2X needs to be defined.

Unlike LTE-V2X, unicast is supported by NR-V2X. And there are some parameters are exchanged between V2X UE to UE as below.

BandSidelinkPC5-r16 ::=           SEQUENCE {

freqBandSidelink-r16              FreqBandIndicatorNR,

--15-1

sl-Reception-r16                  SEQUENCE {

harq-RxProcessSidelink-r16        ENUMERATED {n16, n24, n32, n64},

pscch-RxSidelink-r16              ENUMERATED {value1, value2},

scs-CP-PatternRxSidelink-r16      CHOICE {

fr1-r16                           SEQUENCE {

scs-15kHz-r16                     BIT STRING (SIZE (16))                            OPTIONAL,

scs-30kHz-r16                     BIT STRING (SIZE (16))                            OPTIONAL,

scs-60kHz-r16                     BIT STRING (SIZE (16))                            OPTIONAL

},

fr2-r16                           SEQUENCE {

scs-60kHz-r16                     BIT STRING (SIZE (16))                            OPTIONAL,

scs-120kHz-r16                    BIT STRING (SIZE (16))                            OPTIONAL

}

}                                                                                           OPTIONAL,

extendedCP-RxSidelink-r16         ENUMERATED {supported}                                    OPTIONAL

}                                                                                               OPTIONAL,

--15-10

sl-Tx-256QAM-r16                  ENUMERATED {supported}                                        OPTIONAL,

--15-12

lowSE-64QAM-MCS-TableSidelink-r16 ENUMERATED {supported}                                        OPTIONAL,

...,

[[

--15-14

csi-ReportSidelink-r16                SEQUENCE {

csi-RS-PortsSidelink-r16              ENUMERATED {p1, p2}

}                                                                                               OPTIONAL,

--15-19

rankTwoReception-r16                  ENUMERATED {supported}                                    OPTIONAL,

--15-23

sl-openLoopPC-RSRP-ReportSidelink-r16 ENUMERATED {supported}                                    OPTIONAL,

--13-1

sl-Rx-256QAM-r16                      ENUMERATED {supported}                                    OPTIONAL

]]

}

In our view, the power class information does not need to be included in the PC5 RRC messages, it should still be reported via Uu RRC message.

Proposal 2: The new PC2 power class for NR-V2X should be reported via Uu RRC message.

Since PC2 HPUE NR-V2X is considered in RAN4 as a Rel-16 left over issue, the capability should be introduced from Rel-16.

Proposal 3: The new PC2 power class for NR-V2X should be introduced from Rel-16.

Proposal 4: LS should be sent to RAN2 on the agreements for the new PC2 power class capability for NR-V2X.

# Conclusion

This contribution provides our consideration on specific HPUE power class capability for NR-V2X. We have the following observations and proposals.

Observation 1: Power class 2 UE capability for LTE-V2X is reported via Uu RRC message.

Proposal 1: Specific PC2 power class for NR V2X needs to be defined.

Proposal 2: The new PC2 power class for NR-V2X should be reported via Uu RRC message.

Proposal 3: The new PC2 power class for NR-V2X should be introduced from Rel-16.

Proposal 4: LS should be sent to RAN2 on the agreements for the new PC2 power class capability for NR-V2X.

A draft LS is provided in [2].

# References

[1] R4-2107872, Way forward on PC2 NR V2X, Huawei, HiSiliocn

[2] R4-2114508, draft LS on new power class 2 capability for NR-V2X, Huawei, HiSilicon