3GPP TSG-RAN WG4 Meeting # 100-e	R4-2114693

Electronic Meeting, August 16-27, 2021

Title: 	Some co-existence simulation results for 57-71 GHz

Source: 	CATT

Agenda item:	9.16.6

Document for:	Discussion

- Introduction

This contribution provides our further simulation results for 71 GHz co-existence according to the simulation assumption in last meeting [1].

- Discussion

The simulation assumption in last meeting’s WF is as following,

Table 1: Proposed list of coexistence simulation parameters

| System Parameters | Deployment | Indoor office C in TR 38.808 Dense urban scenario A in TR 38.808 |
| --- | --- | --- |
|  | Carrier Frequency | 60 GHz, 70 GHz |
|  | Channel BW | 100MHz and 400MHz |
|  | SCS | 120KHz for 100MHz and 960KHz for 400MHz |
|  | Number of active UEs | 1 |
|  | Channel model | InH open office model in TR 38.901 Umi model in TR 38.901 |
|  | LBT | No LBT considered (optional: consider LBT) |
| BS | (Mg, Ng, M, N, P) | (1,1,4,8,2) for indoor deployment (1,1,16,16,2) for dense urban deployment |
|  | (dv, dh) | (0.5 λ, 0.5 λ) |
|  | Antenna element gain | 5 dBi |
|  | Antenna element radiation pattern | Indoor: Table A.2.1-7 in TR 38.802 for ceiling mount UMi: Table 7.3-1 in TR 38.901 |
|  | EIRP limit | 40 dBm for indoor deployment 52.8 dBm for dense urban deployment |
|  | Noise Figure | 13 dB |
| UE | (Mg, Ng, M, N, P) | (1,2,2,8,2) |
|  | (dv, dh) | (0.5 λ, 0.5 λ) |
|  | Antenna element gain | 5 dBi |
|  | Antenna element radiation pattern | Indoor and UMi: Table A.2.1-8 in TR 38.802 |
|  | EIRP limit | 20 dBm |
|  | Noise figure | 13 dB |
|  | LoS/ NLoS | LoS probability model defined in TR 38.803 |

Simulation was done for the two scenarios: Indoor office C and Dense urban scenario A. ACIR simulation results for the throughput loss of the two scenarios are provided in 2.1 and 2.2. The detail result curves are captured in annex.

  - Indoor

The ACIR simulation results for 5% throughput loss, indoor scenario are shown in Table 2.1-1 for DL and Table 2.1-2 for UL. The deployment is coordinated.

Table 2.1-1: ACIR simulation results for indoor scenario, DL 5% throughput loss

|  | 100 MHz | 400 MHz |
| --- | --- | --- |
| 60 GHz average | 13.5 dB | 13.5 dB |
| 60 GHz edge | 22.5 dB | 22.5 dB |
| 70 GHz average | 15 dB | 15 dB |
| 70 GHz edge | 23 dB | 23 dB |

Table 2.1-2: ACIR simulation results for indoor scenario, UL 5% throughput loss

|  | 100 MHz | 400 MHz |
| --- | --- | --- |
| 60 GHz average | 0 dB | 0 dB |
| 60 GHz edge | 2 dB | 1dB |
| 70 GHz average | 0 dB | 0 dB |
| 70 GHz edge | 4.5 dB | 3 dB |

  - Dense urban

The ACIR simulation results for 5% throughput loss, dense urban scenario are shown in Table 2.1-1 for DL and Table 2.1-2 for UL. Both the uncoordinated and coordinated are simulated, only coordinated results are captured in the following tables because the coordinated results are more stringent.

Table 2.2-1: ACIR simulation results for dense urban scenario, DL 5% throughput loss

|  | 100 MHz | 400 MHz |
| --- | --- | --- |
| 60 GHz average | 8.5 | 6.5 |
| 60 GHz edge | 0 | 0 |
| 70 GHz average | 10.5 | 8.5 |
| 70 GHz edge | 0 | 0 |

Table 2.2-2: ACIR simulation results for dense urban scenario, UL 5% throughput loss

|  | 100 MHz | 400 MHz |
| --- | --- | --- |
| 60 GHz average | 0 | 0 |
| 60 GHz edge | 0 | 0 |
| 70 GHz average | 0 | 0 |
| 70 GHz edge | 0 | 0 |

- Summary

This contribution provides some simulation results according to the simulation assumption in last meeting. The confirmation of the simulation assumption and the calibration between different companies are needed.

Reference

[1] R4-2107915, WF on 60 GHz coexistence, ACLR, and ACS, Qualcomm

Annex: Simulation results

Indoor scenario:

Dense urban scenario：