3GPP TSG-RAN WG4 Meeting #100-e 												R4-2114709

Electronic Meeting, 12th – 20th April, 2021

Agenda item:			8.2

Source:	Moderator (Apple Inc)

Title:	Email discussion summary for [100-e][109] NR_6GHz_unlic_full

Document for:	Information

# Introduction

3GPP Rel-16 NR-U WI specified how the NR technology can be used on the unlicensed spectrum thus offering more resources in frequency bands, such as 5GHz and 6GHz.  5GHz is a well-known band for the unlicensed operation, but 6GHz is a relative new band usage of which was approved recently in different regulatory regions. While the 6GHz band for the US is already part of the Rel-16 core functionality, current 3GPP specifications do not support it for other countries, such as South Korea and Canada, which have finalised their regulatory requirements only recently. Thus, RAN#92 meeting approved a new WI aim of which is to enable support of the 6GHz unlicensed band for those countries and regions that have finalised recently the corresponding regulatory requirements.

The scope of this email discussion will cover three major areas:

-	WI work plan and related aspects.

-	Summary of the current regulatory status, i.e. which country requirements are common and which ones are completely new.

-	System related aspects, such as band plan, required NS values, A-MPR values, etc.

# Topic #1: Introduction of operation in full unlicensed band 5925-7125MHz for NR

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2112343 | Apple | - RAN4#100: - analyse the band plan for the countries, which are in the scope of the WI, to conclude whether band n96 can be re-used; - analyse regulatory requirements for the countries, which are in the scope of the WI, to conclude which existing NS values can be re-used and how many new NS values might be needed; - start working on the required A-MPR values; - RAN4#101: - continue technical work on the required NS values and associated A-MPR requirements; - agree on preliminary A-MPR values; - introduce running CRs for required NS values and associated A-MPR requirements. - RAN4#102: - agree the final CRs implemented all necessary NS values and A-MPR requirements. Proposal: Agree the proposed work plan for this WI |
| R4-2113065 | Huawei, HiSilicon | Proposal: The introduction of VLP should be addressed in the WI in term of power class and regulatory requirements for Canada, Brazil and South Korea. |
| R4-2112344 | Apple | a) All countries, which are in the scope of this WI, allow the full range of 5925-7125MHz for the unlicensed operation. b) Latin America countries Brazil, Peru and Chile share same parameters for the low power indoor (LPI) mode, which in turn is identical to the US FCC LPI mode for the 6GHz band. c) Unlike Brazil, outdoor operation for the unlicensed 6GHz band is not allowed in Peru and Chile. d) Canada allows three different modes for the 6GHz band: standard power (with AFC), low power indoor and very low power. And it is worth noting that Canadian SP and LPI mode parameters are slightly different when compared to US FCC. e) Even though both Brazil and Canada have the VLP mode defined by the local regulators, they have different parameters (and are different when compared to e.g. EU/CEPT VLP). f) South Korea allows both LPI and VLP modes, whereupon the VLP mode parameters are identical to the EU/CEPT regulations, but the LPI mode parameters are not the same as in e.g. US or EU/CEPT. Proposal 1: Re-use 3GPP band n96 for Canada, Peru, Chile, Brazil and South Korea. Proposal 2: Since Peru, Chile and Brazil have the same requirements for the LPI mode, which in turn is identical to the US LPI mode, re-use existing NS_53 for those countries. Proposal 3: Since Canada and South Korea have different LPI mode requirements, two new NS values are needed. Proposal 4: Defer introduction of the VLP mode for Brazil, Canada and South Korea. |
| R4-2113066 | Huawei, HiSilion | For Canada LPI operation, new NS may be needed to address in-band 5 dBm/MHz PSD limit for PC5 UE For Canada VLP operation, new power class and new NS may be needed to address in-band 5 dBm/MHz PSD limit. For Brazil LPI operation, NS_53 may be reused. For Brazil VLP operation, new power class and new NS may be needed to address in-band -5 dBm/MHz PSD limit and out of band -27 dBm/MHz limit. For Peru and Chile LPI operation, new NS may be needed to address in-band -1 dBm/MHz PSD limit for PC5 UE For South Korea LPI operation, new NS may be needed to address in-band 2 dBm/MHz PSD limit and out of band limits for PC5 UE. For South Korea VLP operation, new power class and new NS may be needed to address in-band 1 dBm/MHz PSD limit and out of band limits. |
| R4-2112972 | LG Electronics | Observation 1: In the case of South Korea, the legacy RF requirements for n96 UE can be reused, however, there are differences for the in-band emissions requirements between Korea’s technical standards and the 3GPP standards. Observation 2: The following 2 options are available for NS value for LPI UE in Korea 5925-7125MHz. Option 1: RAN4 can reuse NS_53 for Korea NR-U operation in 5925~7125MHz. Option 2: For the NR-U operation in Korea, RAN4 can define New NS value (NS_xx) to optimize the A-MPR requirements. Proposal 1: To comply Korea regulatory requirements, RAN4 needs to specify the new NS value (NS_xx) to optimize the A-MPR requirements. Proposal 2: The operating band perspective for NR-U operation in Korea, n96 NR band will be reused. Observation 3: To support VLP operating mode, new power class in n96 is needed to specify in TS38.101-1. Also, VLP operating mode with +14/+17dBm Power class were requested in EU/UK/Canada/Brazil. The power class 6(+14dBm) was already specified in TS36.101 to support NB-IoT UE. Observation 4: For VLP operating mode, RAN4 needs to specify the A-MPR requirements with new NS_xy to meet the additional emission requirements (in-band emission and out-of-band emission). Proposal 3: To support VLP operating mode, RAN4 shall specify the new power class (Power class6: +14dBm) in n96 and specify the A-MPR requirements with New NS_xy to meet the additional emission requirements. |
| R4-2112345 | Apple | Proposal 1: Define A-MPR for NR-U PC5 LPI in Canada as provided in Table 2.1-1. Proposal 2: Define A-MPR for NR-U PC5 LPI in South Korea as provided in Table 2.2-1. |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 1-1 (WI work plan and scope)

Only one contribution (R4-2112343) was submitted proposing the work plan for this WI.

Issue 1-1-1: Workplan

- Proposals

  - Option 1: WI work plan as proposed in R4-2112343.

- Recommended WF

  - Agree the WI work plan proposed in R4-2112343.

### Sub-topic 1-2 (Overview of regulatory requirements)

In this sub-topic the general regulatory related aspects are handled. Since not all the countries share same regulatory parameters for the 6GHz unlicensed band, this sub-topic aims at analyzing which countries have same parameters and which parameters are different.

The following paper, submitted to this WI, make a summary of the regulatory related parameters: R4-2113065, R4-2112344, R4-2113066, R4-2112972.

Issue 1-2-1: Summary of regulatory parameters

- Recommended WF

  - Based on the presented information, it is suggested to endorse the following table which summarises which countries have or share the same parameters (max EIRP, PSD, OOBE) and which countries have new ones.

| Country | Mode |  |  |
| --- | --- | --- | --- |
|  | SP | LPI | VLP |
| Canada | (new) | (new) | (new) |
| Brazil | N/A | same as US LPI | (new) |
| Peru | N/A | same as US LPI | N/A |
| Chile | N/A | same as US LPI | N/A |
| South Korea | N/A | (new) | same as EU/CEPT VLP |
| NOTE 1: Peru and Chile LPI regulatory parameters are the same as US LPI, but there is no -27dBm/MHz out-of-band emission requirement. NOTE 2: South Korea VLP parameters are the same as in EU/CEPT, but out-of-band emission requirement is set to -34dBm/MHz outside the operational range. |  |  |  |

### Sub-topic 1-3 (System related aspects)

In this sub-topic the regulatory requirements are “translated” into specific 3GPP system parameters. The following issues are considered: band plan, LPI and VLP, required NS values, A-MPR values.

Input from the following papers is considered: R4-2113065, R4-2112344, R4-2112972, R4-2112345.

Issue 1-3-1: Band plan

- Proposals

  - Option 1: Band n96 is used for South Korea (R4-2112972), Canada, Brazil, Chile and Peru (R4-2112344).

  - (no other options were proposed)

- Recommended WF

  - Agree that existing 3GPP band n96 is used for South Korea, Canada, Brazil, Chile and Peru.

Issue 1-3-2: Canada

- Proposals

  - SP mode: no need for a new NS flag for NR-U PC5 (R4-2113066 and R4-2112344).

  - LPI mode: a new NS flag is needed (R4-2113066 and R4-2112344)

  - VLP mode: different views expressed on whether VLP should be added now (R4-2113066 and R4-2112344)

- Recommended WF

  - Agree that no NS flag is needed for the SP mode in Canada (NOTE: can be confirmed further by simulations).

  - Agree that a new NS flag is needed for the LPI mode in Canada

  - VLP should be discussed further.

Issue 1-3-3: Brazil

- Proposals

  - LPI mode: re-use NS_53 (US LPI) for Brazil (R4-2113066 and R4-2112344).

  - VLP mode: different views expressed on whether VLP should be added now (R4-2113066 and R4-2112344)

- Recommended WF

  - Agree that NS_53 can be re-used for LPI in Brazil.

  - VLP should be discussed further.

Issue 1-3-4: Peru and Chile

- Proposals

  - LPI mode: while R4-2112344 states that no NS flag is needed because the requirements are the same as in Brazil and US, R4-2113066 mentions that a new NS flag may be needed.

- Recommended WF

  - Check further whether Peru and Chile can re-use NS_53 (similar to Brazil LPI).

Issue 1-3-5: South Korea

- Proposals

  - LPI mode: a new NS value is needed (R4-2112344 and R4-2112972).

  - VLP mode: different views expressed on whether VLP should be added now (R4-2112344 and R4-2112972)

- Recommended WF

  - A new NS value is needed for LPI in South Korea.

  - VLP should be discussed further.

Issue 1-3-6: A-MPR values

- Proposals

  - Option 1: A-MPR values for the LPI mode in Canada and South Korea (R4-2112345).

  - (no other A-MPR simulations are submitted)

- Recommended WF

  - Agree tentatively proposed A-MPR values, subject for further checking and corrections.

## Companies views’ collection for 1st round

### Open issues

| Company | Comments |
| --- | --- |
| XXX | THIS IS A TEMPLATE, DO NOT CHANGE IT, ADD A NEW ROW BELOW Issue 1-1-1 (Work plan): Issue 1-2-1 (Summary of the regulatory requirements): Issue 1-3-1 (Band plan) Issue 1-3-2 (Canada) Issue 1-3-3 (Brazil) Issue 1-3-4 (Peru and Chile) Issue 1-3-5 (South Korea) Issue 1-3-6 (A-MPR values) |
| Skyworks | Issue 1-1-1 (Work plan): Agree with proposed WF Issue 1-2-1 (Summary of the regulatory requirements): Agree with proposed WF Issue 1-3-1 (Band plan): Agree with proposed WF Issue 1-3-2 (Canada): Agree with proposed WF. On VLP we need to understand the fall back mechanism between indoor to outdoor operation. In any case we do not see that a specific power class should be defined as a PC5 UE may have to support VLP mode when being outdoors and connected to an indoor BS. Issue 1-3-3 (Brazil): Agree with proposed WF. On VLP we need to understand the fall back mechanism between indoor to outdoor operation. In any case we do not see that a specific power class should be defined as a PC5 UE may have to support VLP mode when being outdoors and connected to an indoor BS. Issue 1-3-4 (Peru and Chile): can reuse same NS that US LPI Issue 1-3-5 (South Korea): Agree with proposed WF. On VLP we need to understand the fall back mechanism between indoor to outdoor operation. In any case we do not see that a specific power class should be defined as a PC5 UE may have to support VLP mode when being outdoors and connected to an indoor BS. Issue 1-3-6 (A-MPR values): values are consistent with n96 US numbers and the higher in band PSD, since only 20MHz needs A-MPR it may be worth clarifying that only when there is one sub-band transmitted wideband operation needs to use the respective (full/partial) A-MPR. |
| Huawei | Issue 1-1-1 (Work plan): in general it is ok, but the introduction of VLP should be addressed in the WI in term of power class and regulatory requirements for Canada, Brazil and South Korea, which should be include in the work plan. Issue 1-2-1 (Summary of the regulatory requirements): For Peru and Chile, there is no OOB emission is required which is not the same as UP LPI Issue 1-3-1 (Band plan) Issue 1-3-2 (Canada) OK with proposed WF Issue 1-3-3 (Brazil) OK with proposed WF Issue 1-3-4 (Peru and Chile) Need further check Issue 1-3-5 (South Korea) OK with proposed WF |
| Nokia | Issue 1-1-1 (Work plan) - Fine with the proposed Work plan, just wondering if it is a bit optimistic. Issue 1-2-1 (Summary of the regulatory requirements): - We are fine to tentatively captured the table, but it seems some further checking is needed. Issue 1-3-1 (Band plan) – Ok with proposed WF Issue 1-3-2 (Canada) – Ok with proposed WF Issue 1-3-3 (Brazil) – Ok with proposed WF Issue 1-3-4 (Peru and Chile) – Ok with proposed WF to further check Issue 1-3-5 (South Korea) – Ok with proposed WF Issue 1-3-6 (A-MPR values) – Ok with proposed WF |
| LGE | Issue 1-1-1 (Work plan): We would like to add a discussion about the new power class considering VLP mode(Korea, Canada, Brazil) to the work plan. Power class 6(+14dBm) was already specified in TS36.101 to support NB-IoT UE. Issue 1-2-1 (Summary of the regulatory requirements): we propose as below. Country; Mode. SP; LPI; VLP. Canada; (new); (new); (new). Brazil; N/A; same as US LPI (NS_53); (new). Peru; N/A; New NS or NS_53; N/A. Chile; N/A; New NS or NS_53; N/A. South Korea; N/A; (new); (new)Note2. Note2: In VLP mode, there are differences for the out-of-band emissions requirements between Korea’s technical standards and the EU/CEPT standards. - South Korea’s OOBE limit(@VLP): -34 dBm/MHz(f ≤ 5925MHz, f ≥ 6445MHz) - EU/CEPT’s OOBE limit(@VLP): -45 dBm/MHz(below 5935MHz); Issue 1-3-1 (Band plan): Agree with moderator recommended WF Issue 1-3-2 (Canada), Issue 1-3-3 (Brazil), Issue 1-3-5 (South Korea): To support VLP operating mode(Korea, Canada, Brazil), RAN4 shall specify the new power class (Power class6: +14dBm) in n96 and specify the A-MPR requirements with New NS_xy to meet the additional emission requirements. The Power class 6(+14dBm) was already specified in LTE in TS36.101 to support NB-IoT UE. So it can be specify in NR specification to support VLP mode. Issue 1-3-4 (Peru and Chile): Agree with moderator recommended WF to further check Issue 1-3-6 (A-MPR values): we think that RAN4 needs additional A-MPR data from other interested companies to support LPI mode with different emission requirements. |
| Apple | Issue 1-1-1 (Work plan): As a rapporteur company, we would like to differentiate a bit between the work plan, i.e, the timeline, and the scope. The intention of the workplan proposal was to present what we should do by each meeting in terms of A-MPR simulations, running CRs, etc to meet the target deadline. In that sense to have or not have the VLP is merely the question for the scope of the work, and the overall workload, but not the timeline. Issue 1-2-1 (Summary of the regulatory requirements): As pointed by Huawei and LGE, for Peru and Chile, the OOBE requirements are not specified. And VLP for South Korea have different OOBE requirements when comparing to EU/CEPT. So, it should be indeed analyzed further whether we can re-use e.g. NS_53 for Peru and Chile (see our comments below). Issue 1-3-1 (Band plan) Re-use band n96. Differences in regional requirements, if any, can be handled with NS values. Issue 1-3-2 (Canada) VLP: Our view is that introduction of a new power class goes beyond the scope of this WI and available TUs. The same discussion took place in the 6GHz for CEPT/EU WI, and no conclusion was reached. Issue 1-3-3 (Brazil) VLP: Our view is that introduction of a new power class goes beyond the scope of this WI and available TUs. The same discussion took place in the 6GHz for CEPT/EU WI, and no conclusion was reached. And please note that Brazil has 17dBm max EIRP for VLP, which is different comparing to Canada or EU/CEPT. Issue 1-3-4 (Peru and Chile) LPI: The difference between Peru/Chile and Brazil/US is that the latter countries have -27dBm/MHz OOBE requirement, which Peru/Chile do not have. However, it means that we can safely re-use NS_53 for Peru/Chile because A-MPR values for US were derived with stricter assumptions. The only potential concern in this case would be that we allow somewhat larger A-MPR values for Peru/Chile than needed. However, our analysis indicates that even for LPI in US, we are limited by PSD, not by OOBE. So, same A-MPR values eventually apply for Peru/Chile. We can have an action point for the next meeting to triple-check it by re-running some simulations. Issue 1-3-5 (South Korea) VLP: Our view is that introduction of a new power class goes beyond the scope of this WI and available TUs. The same discussion took place in the 6GHz for CEP/TEU WI, and no conclusion was reached. Since VLP in South Korea is very close to EU/CEPT VLP, only OOBE are slightly different, this should be ideally aligned with the 6GHz EU WI. Issue 1-3-6 (A-MPR values) We of course welcome other companies to submit their A-MPR results. To expedite the work, we suggest following the same principle as for the 6GHz EU WI, i.e. calibrate first LPI A-MPR values. |

### CRs/TPs comments collection

For close-to-finalize WIs and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| XXX | Company A |
|  | Company B |
| YYY | Company A |
|  | Company B |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Issue #1-1-1 (work plan) | Tentative agreements: There were several comments concerning the workplan and the VLP mode. The moderator understanding is that the workplan reflects merely the timeline and phases we have to perform to meet the target deadline, and the scope does not exclude the VLP mode. Candidate options: Agree with the proposed work plan with an assumption that VLP related discussions are in the scope of this WI. Recommendations for 2nd round: No further discussions during the 2nd round. |
| Issue #1-2-1 (summary of regulatory requirements) | Tentative agreements: A summary table was presented, for which several companies noted that even though Peru/Chile have same max EIPR and PSD as for Brazil/US, the latter countries have also the OOBE requirements. Same comment is for the VLP mode in South Korea. Candidate options: The summary table is updated as follows. Country; Mode. SP; LPI; VLP. Canada; (new); (new); (new). Brazil; N/A; same as US LPI; (new). Peru; N/A; (new) Note1; N/A. Chile; N/A; (new) Note1; N/A. South Korea; N/A; (new); (new)Note2. NOTE 1: Peru and Chile LPI regulatory parameters are the same as US/Brazil LPI, but there is no -27dBm/MHz out-of-band emission requirement. NOTE 2: South Korea VLP parameters are the same as in EU/CEPT, but out-of-band emission requirement is set to -34dBm/MHz outside the operational range Recommendations for 2nd round: Companies are welcome to check further requirements to ensure that we have a consistent summary. A similar table can be discussed but with the NS values to summarize which NS values can be re-used. |
| Issue #1-3-1 (band plan) | Tentative agreements: Several companies agreed to use re-use existing band n96 for new countries, there were no objections to re-use that band. Candidate options: Re-use band n96 for new countries (Canada, South Korea, Brazil, Peru, Chile) Recommendations for 2nd round: No further discussions during the 2nd round. |
| Issue #1-3-2 (Canada) | Tentative agreements: Companies are Ok with the proposal not to have a new value for SP in Canada (re-using US SP NS_54) and to have a new NS value for the LPI mode. Candidate options: Agree to re-use US SP NS_54 and introduce a new NS value for LPI in Canada. Recommendations for 2nd round: Continue discussions for the VLP mode. |
| Issue #1-3-3 (Brazil) | Tentative agreements: Companies are Ok with the proposal to re-use NS_53 for LPI in Brazil. Candidate options: Agree to re-use US NS_53 for LPI in Brazil. Recommendations for 2nd round: Continue discussions for the VLP mode. |
| Issue #1-3-4 (Peru and Chile) | Tentative agreements: Peru and Chile do not have OOBE requirements, which are present for US and Brazil. According to the feedback from one company, in case of 1dBm/MHz PSD limite, the A-MPR values we are usually PSD limited, so in this case the same A-MPR values would also apply to Peru/Chile. Candidate options: Either add a new NS value for Peru/Chile or re-use NS_53. Recommendations for 2nd round: Check further whether same A-MPR values and NS_53 can be re-used. |
| Issue #1-3-5 (South Korea) | Tentative agreements: Companies are Ok to have a new NS value for LPI in South Korea. Candidate options: Add a new NS value for the LPI in South Korea, Recommendations for 2nd round: Continue discussions for the VLP mode. |
| Issue #1-3-6 (A-MPR values) | Tentative agreements: Companies are generally Ok with the proposed A-MPR values for Canada and South Korea. These are of course tentative/preliminary values, which can be calibrated further based on more simulation results, which companies can submit for the next meeting. For instance as commented by one company for LPI in Canada, since only 20MHz needs A-MPR it may be worth clarifying that only when there is one sub-band transmitted wideband operation needs to use the respective (full/partial) A-MPR. Candidate options: Capture tentative values, e.g. in the WF document, which can be revised later. Recommendations for 2nd round: No need for further discussions this meeting. |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round (if applicable)

# Recommendations for Tdocs

## 1st round

New tdocs

| Title | Source | Comments |
| --- | --- | --- |
| WF on introduction of the full unlicensed band | Apple Inc. |  |

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

## 2nd round

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-210xxxx | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-210xxxx | WF on … | YYY | Agreeable, Revised, Noted |  |
| R4-210xxxx | LS on … | ZZZ | Agreeable, Revised, Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents

# Annex

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| Skyworks Solutions Inc. | Dominique Brunel | Doinique.brunel@skyworksinc.com |
| Huawei | Liehai Liu | liuliehai@huawei.com |
| LG Electronics | Jaehyuk jang | jh1.jang@lge.com |
| Apple Inc. | Alex Sayenko | asayenko@apple.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)