3GPP TSG-RAN WG4 Meeting # 100-e 													R4-2114723

Electronic Meeting, 16th – 27th August 2021

Agenda item:			11.8, 8.42

Source:	Moderator (Huawei)

Title:	Email discussion summary for [100-e][123] LTE_NR_Other_basket

Document for:	Information

# Introduction

Briefly introduce background, the scope of this email discussion (e.g. list of treated agenda items) and provide some guidelines for email discussion if necessary.

List of candidate target of email discussion for 1st round and 2nd round

- 1st round: TBA

- 2nd round: TBA

In this email thread, AI 8.42 and 11.8 are discussed. One CR from AI 11.7.2 is added for the 2nd round discussion. The topics are divided into:

- NB-IoT related

- LTE-MTC related

- Downlink interruption for band combinations to conduct dynamic Tx Switching

- A-MPR for LTE Band 24 for UE categories M1 and M2

# Topic #1: NB-IoT

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2112280 | Nokia, Nokia Shanghai Bell | Proposal 1: The current NB-IoT RB power dynamic range for QPSK RB transmission in NB-IoT should apply to 16QAM RB for in-band or guard band operation or NB-IoT operation in NR in-band. There is no need for changes to the specifications nor new test cases to define NB-IoT RB power dynamic range for 16QAM. Proposal 2: An EVM limit of 12.5% should be specified for NB-IoT 16QAM transmission. Proponent of a different EVM requirement should provide technical justifications (e.g., preliminary simulation results) in next RAN4 meeting. Proposal 3: To define at least one of the new uplink FRC for 16QAM as shown in table 3 above. |
| R4-2113620 | Nokia, Nokia Shanghai Bell | Proposal 1: Numerically, the NB-IoT IBE mask for pi/2-BPSK and QPSK should not change significantly. Proposal 2: Define the second General formula of NB-IoT IBE mask as 20 log10EVM-3-5⋅(Δtone-1)/LCtone. Proposal 3: Define the first General formula of NB-IoT IBE mask as 20∙log10EVM-10∙log10Ntone/LCtone. Also, we repeated the EVM limit proposal: Proposal 4: For NB-IoT UL with 16-QAM modulation, use the EVM limit of 12.5 %. |
| R4-2114002 | Nokia, Nokia Shanghai Bell | In NB-IoT, current in-band emission mask assumes pi/2-BPSK or QPSK modulation. Higher modulations would need a stricter mask due to their higher target RX SNR. This CR changes the first and second General formula of the in-band emission mask. The requested changes make the mask stricter for 16-QAM without affecting pi/2-BPSK and QPSK. Make the NB-IoT in-band emission mask (first and second General formula) dependent on the EVM limit. |
| R4- 2114216 | Huawei, HiSilicon | Proposal 1: Update the max SNR in the 1st term and include the EVM to the 2nd term of the general part of the in-band emission requirement. Proposal 2: As a trade-off, consider the IBE mask defined in Table 3 as the new IBE requirements for both QPSK and 16QAM. Parameter description; Unit; Limit (NOTE 1); Applicable Frequencies. General; dB; max20∙log10EVM-10∙log10Ntone/LCtone, 20∙log10EVM-3-5∙∆tone-1/LCtone, -57dBm/(3.75 kHz or 15 kHz)-Ptone; Any non-allocated (NOTE 2). |
| R4- 2114217 | Huawei, HiSilicon | Proposal 1: The existing RB power dynamic range requirements are applicable for both QPSK and 16QAM. There is no need to change the wording of the current BS RF specifications, nor add any new test cases for 16QAM. Proposal 2: Set the EVM limit of 16QAM to 12.5% and add notes to the rated output power declarations to allow optional back-off for NB-IoT 16QAM DL operation in standalone mode. Proposal 3: It is unnecessary to add 16QAM FRCs for NB-IoT BS dynamic range tests. |
| R4- 2114218 | Huawei, Ericsson | The Rel-17 additional enhancements for NB-IoT and LTE-MTC WI was approved at RAN#86 and updated at RAN#88e, with RAN1 completion due by June 2021 (RAN#91) and overall core part completion due by September 2021 (RAN#92). In RAN#92e, the TU and WID were updated [1][2]. This contribution presents the rapporteurs’ work plan for the WI. It does not affect or limit the scope of the approved WID or the exchange of information the working groups can undertake. Note that there may be changes in the proposed work plan due to schedule updates in the responsible working groups. |
| R4-2114345 | Ericsson | Proposal-1: Adding 16QAM support on EVM fo TX signal quality for NB. Observation#1: There should be no coverage impact on legacy NB-IoT device due to the 16QAM introduction. Observation#2: 16QAM should be considered in BS manufacture needs to declare the guard band support for 5MHz channel. Proposal-2:Allow the power boosting of 6dB for NB-IoT carrier signal mixing the QPSK tone and 16QAM tone wih with in-band and guard band operation for a LTE and NR carrier. Proposal-3: Considering 16QAM is supported starting from 3 tones, existing dynamic range requirement for QPSK and 1 tone is stringent enough and there is no need to introduce 16 QAM FRC for BS dynamic range. |
| R4-2114346 | Ericsson | Observation: The first item in general IBE requirement formula decide the IBE requirement for legacy NB-IoT device and when legacy NB-IoT device coexist with NB-IoT device with 16QAM capability, the legacy NB-IoT device dominates the noise emission at the non-allocated tones. Proposal: Only consider updating the second item of the IBE requirement with different modulation schemes. |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 1-1 BS RF Requirements

Sub-topic description: The impact to BS RF requirements in support of 16QAM are discussed here. It appears that there are good convergence on the requirements for RB power dynamic range and the EVM limit for 16QAM, leaving some minor issues to be resolved. Regarding the BS Rx dynamic range test, there are still two different views.

Open issues and candidate options before e-meeting:

Issue 1-1-1: NB-IoT RB power dynamic range

Issue 1-1-2: EVM limit for 16QAM DL

Issue 1-1-3: 16QAM FRC for BS Rx dynamic range

### Sub-topic 1-2 UE RF Requirements

Sub-topic description: The impact to UE RF requirements in support of 16QAM are discussed here. There’s convergence on the EVM limit for 16QAM. The main sticking point is the IBE requirements. One company proposes to update only the 2nd term of the general IBE requirement, but two companies support to update both the 1st and 2nd term. And new MPR results based on the updated IBE are to be discussed.

Open issues and candidate options before e-meeting:

Issue 1-2-1: IBE mask

Issue 1-2-2: MPR for 16QAM

### Sub-topic 1-3 Updated Work Plan

Sub-topic description: The work plan for the WI of additional enhancements for NB-IoT and MTC is discussed here. Based on the decisions in RAN#92, the Oct meeting is cancelled and one meeting is added to Jan 2022. The work plan is updated accordingly.

Open issues and candidate options before e-meeting:

Issue 1-3-1: Update of Work Plan

## Companies views’ collection for 1st round

### Open issues

One of the two formats, i.e. either example 1 or 2 can be used by moderators.

Issue 1-1-1: NB-IoT RB power dynamic range

- Tentative Agreement: Reuse the existing NB-IoT RB power dynamic range requirements for 16QAM for in-band or guard band operation or NB-IoT operation in NR in-band. Whether any changes to the current specifications or new test cases for 16QAM are needed is to be confirmed.

- Proposals

  - Option 1: No changes to the specifications, nor new test cases for 16QAM RB power dynamic range.

  - Option 2: Need changes (to be proposed)

  - Option 3: Others (to be proposed)

- Recommended WF

  - Consider whether the existing requirements in the current specifications can already cover 16QAM

| Company | Comments |
| --- | --- |
| Nokia | Support Option 1. |
| Huawei | Option 1. |
| Ericsson | Option 1. |

Issue 1-1-2: EVM limit for 16QAM DL

- Tentative Agreements: Define the EVM limit for 16QAM as 12.5% and discuss if power back-off is allowed for 16QAM in standalone mode

- Proposals

  - Option 1: Max power back-off is allowed for 16QAM in stand-alone mode, which is subject to vendor’s declaration (i.e., rated Power)

  - Option 2: Max power back-off is not allowed

  - Option 3: Others (to be proposed)

- Recommended WF

  - Discuss the above options

| Company | Comments |
| --- | --- |
| Nokia | Support Option 2. |
| Huawei | Option 1. Please note that if a BS is capable of transmitting at full power, it won’t be affected by option 1 at all. It is only a provision for some legacy base stations in the field operating at standalone mode. For operators, they could either upgrade the hardware or not support 16QAM. A reasonable tradeoff would be to reuse the legacy hardware with some power back-off for 16QAM. The impact of option 1 is minimal. |
| Ericsson | Option 2. The rated power for standalone mode is declarable, if the NB carrier rated power too high and cannot support 16QAm, why just declare it with lower value for the new BS? In that case, with declared lower rated power, the new NB-Iot standalone can support 16QAM; For the deployed BS with supporting NB-IoT standalone, if the max power backoff power would be allowed, the deployed NB device coverage would be impacted. So considering the legacy NB device coexisting with 16QAM supported NB device, it is still preferable that the output power of BS configuring NB-IoT standalone carrier is not changed. |
| Huawei | It seems there might be some misunderstanding here. In our paper, we propose to add notes to TS 36.141 in the manufacture’s declarations section. The notes are to allow lower rated power to be declared for 16QAM for NB-IoT BS in standalone mode, which is a similar treatment for 1024QAM. In-band/guard band mode won’t be affected, nor will QPSK in any mode. To Ericsson: with the above clarification, would you still object? |

Issue 1-1-3: 16QAM FRC for BS Rx dynamic range

- Proposals

  - Option 1: Define new uplink FRC(s) for 16QAM

  - Option 2: New 16QAM FRC is not needed

- Recommended WF

  - Consider whether the existing tests are sufficient and what the added benefits are by defining 16QAM FRCs.

| Company | Comments |
| --- | --- |
| Nokia | Option 1 at least for stand-alone operation, as simulation results in R4-2112280 show that around 10dB higher SNR is required for 16QAM compared to QPSK, and as discussed in R4-2114217, legacy base stations may be optimized for NB-IoT DL of early releases with QPSK as the highest order modulation, leaving little margin to support higher receive power for 16QAM. |
| Huawei | To Nokia: our concern on legacy base stations is on the transmitter side, while the issue here is for the receiver. Anyway, if you think 16QAM might be challenging for legacy base stations, why don’t you allow power back-off in issue 1-1-2? We prefer option 2, but we’re also open to discuss option 1 further. |
| Ericsson | Option 2. We are not sure to introduce 2 FRC and test the same receiver twice would add value. Perhaps the Demod performance will add NB-IoT 16QAM case later on and the blocking test to test the receiver linearity. |
| Nokia | To Huawei: the difference compared to rated output power is that rated output power is a declared value from the manufacturer, while receiver dynamic range is a minimum requirement where all BS declared to support 16QAM shall meet; thus a legacy stand-alone NB-IoT BS declared to support 16QAM shall be tested with 16QAM operation, otherwise there is no guarantee that it can still meet the minimum requirement with 16QAM operation. To Ericsson: demod performance is another option, but receiver dynamic range aim to test more than the receiver linearity, e.g. receiver EVM. |
| Huawei | The 16QAM receiver performance should be tested in demod part. The existing Rx dynamic range tests already check the receiver EVM. The question is how much extra value is by adding 16QAM FRC. For example, no 64QAM FRC is added to NR/EUTRA in addition to the 16QAM FRC. |

Issue 1-2-1: IBE mask

- Tentative Agreements: Update the 2nd term of the general IBE mask using EVM, whether the 1st term should be updated is TBC.

- Proposals

  - Option 1: Do not change the 1st term of the general IBE mask

  - Option 2: Update the 1st term as proposed by Nokia and Huawei

  - Option 3: Others (to be proposed)

- Recommended WF

  - Discuss the above options

| Company | Comments |
| --- | --- |
| Nokia | Option 2. 16-QAM has a higher target RX SNR and thus higher target RX power than the lower-order modulations. Therefore, applying the same first General IBE mask formula to both 16-QAM and lower-order modulations would mean that a higher absolute emission level would be permitted for 16-QAM. By using the EVM limit in the formula, we can conveniently make the mask stricter for 16-QAM while keeping the mask practically unchanged for the lower-order modulations. |
| Huawei | Option 2. We share the same view as Nokia. |
| Ericsson | Fine with option 2. |

Issue 1-2-2: MPR for 16QAM

- The MPR simulation results are summarized as follows.

Table 1.3.1-1: MPR for 16QAM (power class 3)

| Tone spacing | Number of tones | Allocated tones | MPR [dB] R4-2109948 | MPR [dB] R4-2111295 | MPR [dB] R4-2113620:A | MPR [dB] R4-2113620:B |
| --- | --- | --- | --- | --- | --- | --- |
| 15 kHz | 3 | 0-2 | 2.1 | 2 | 1.5 | 1.5 |
|  |  | 3-5 | 2.0 | 2 | 1.5 | 1.5 |
|  |  | 6-8 | 2.0 | 2 | 1.5 | 1.5 |
|  |  | 9-11 | 2.1 | 2 | 1.5 | 1.5 |
|  | 6 | 0-5 | 1.6 | 2 | 1.1 | 1.3 |
|  |  | 6-11 | 1.6 | 2 | 1.1 | 1.3 |
|  | 12 | 0-11 | 1.9 | 2.7 | 2.6 | 2.6 |

Table 1.3.1-2: MPR for 16QAM (power class 5)

| Tone spacing | Number of tones | Allocated tones | MPR [dB] R4-2109948 | MPR [dB] R4-2111295 | MPR [dB] R4-2113620:A | MPR [dB] R4-2113620:B |
| --- | --- | --- | --- | --- | --- | --- |
| 15 kHz | 3 | 0-2 | 2.6 | 2.3 | 1.4 | 1.4 |
|  |  | 3-5 | 2.5 | 2.3 | 1.4 | 1.4 |
|  |  | 6-8 | 2.5 | 2.2 | 1.4 | 1.4 |
|  |  | 9-11 | 2.6 | 2.2 | 1.4 | 1.4 |
|  | 6 | 0-5 | 2.1 | 2.2 | 1.0 | 1.3 |
|  |  | 6-11 | 2.1 | 2.1 | 1.0 | 1.3 |
|  | 12 | 0-11 | 2.4 | 3.0 | 2.5 | 2.5 |

- Proposal #1: MPR for 16QAM PC3 and PC5

  - Option 1: Consider all simulation results and take average

  - Option 2: Consider the simulation results under the same calibration point as well as conformance constraints (such as same IBE mask), and take average

  - Option 3: Others (to be proposed)

- Proposal #2: MPR for 16QAM PC6

  - Option 1: To be discussed in future meetings

  - Option 2: Others (to be proposed)

- Recommended WF

  - Discuss the above proposals, MPR for PC3 and PC5 may share the same table or have separate tables

| Company | Comments |
| --- | --- |
| Nokia | Proposal #1: Option 3, average MPR results with the same IBE mask but with possibly different calibration points. Nokia’s earlier R4-2109948 should be ignored since we have corrected the PA model calibration so that QPSK with specified MPR meets the emission requirements, including OBW. Also, based on the decision on the IBE mask, the averaging should include either R4-2113620:A (first General IBE mask formula unchanged) or R4-2113620:B (EVM limit applied in the first General IBE mask formula). Thus, two MPR values remain for each tone allocation. Proposal #2: Option 1 In the spec, PC3 and PC5 can have separate MPR columns in a common table. |
| Huawei | Proposal #1: in view of the above clarification, we tend to agree with Nokia’s proposal. Since we both prefer the same IBE mask, we could take the following averages for PC3 and PC5. PC3: Tone spacing; Number of tones; Allocated tones; MPR [dB] R4-2111295; MPR [dB] R4-2113620:B; Average [dB]. 15 kHz; 3; 0-2; 2; 1.5; 1.8. 3-5; 2; 1.5; 1.8. 6-8; 2; 1.5; 1.8. 9-11; 2; 1.5; 1.8. 6; 0-5; 2; 1.3; 1.7. 6-11; 2; 1.3; 1.7. 12; 0-11; 2.7; 2.6; 2.7. PC5: Tone spacing; Number of tones; Allocated tones; MPR [dB] R4-2111295; MPR [dB] R4-2113620:B; Average [dB]. 15 kHz; 3; 0-2; 2.3; 1.4; 1.9. 3-5; 2.3; 1.4; 1.9. 6-8; 2.2; 1.4; 1.8. 9-11; 2.2; 1.4; 1.8. 6; 0-5; 2.2; 1.3; 1.8. 6-11; 2.1; 1.3; 1.7. 12; 0-11; 3.0; 2.5; 2.8. Since the values are quite close after averaging, we might use one table for both PC3 and PC5. For example: Tone spacing; Number of tones; Allocated tones; MPR proposal [dB]. 15 kHz; 3; 0-2; 1.8. 3-5; 1.8. 6-8; 1.8. 9-11; 1.8. 6; 0-5; 1.8. 6-11; 1.8. 12; 0-11; 2.8. Proposal #2: option 1. |
| Qualcomm | We had some issues with the simulation and were not able to deliver results on time. We would like to make a decision only in 2nd round on the numbers so that we have a bit more time to check the values. |

Issue 1-3-1: Work Plan

- Proposals

  - Option 1: Approve the work plan

  - Option 2: Need changes (to be proposed)

  - Option 3: Others (to be proposed)

- Recommended WF

  - Discuss the above options

| Company | Comments |
| --- | --- |
| Nokia | Support Option 3, as the work plan will need to continuously adjusted following RAN1 discussion and decision. |
| Huawei | Option 1. The work plan can always be updated when necessary. To Nokia: can you please clarify what needs to be done to the work plan in this meeting? |
| Nokia | To Huawei: we meant the work plan will need to be continuously adjusted following RAN1 discussion and decision in the coming RAN4 meetings. |

### CRs/TPs comments collection

For close-to-finalize WIs and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2114002 CR on NB-IoT IBE mask to allow 16-QAM | Huawei: it’s subject to the outcome of the discussions on IBE mask. |
|  | <Moderator>: According to the comments received, the CR appears to be agreeable. However, should we wait for the outcome of the discussion on EVM and MPR? Otherwise, the IBE mask would be updated in the spec for 16QAM while EVM and MPR might be missing. Suggest to endorse the CR, not approve it right now. |
| YYY | Company A |
|  | Company B |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic #1-1 | Tentative agreements: For issue 1-1-1, consensus is reached, i.e., Reuse the existing NB-IoT RB power dynamic range requirements for 16QAM for in-band or guard band operation or NB-IoT operation in NR in-band. And no changes to the specifications, nor new test cases for 16QAM RB power dynamic range are needed. For issue 1-1-2, the majority view is to set the EVM limit for 16QAM to 12.5%. Whether BS vendor is allowed to declare different rated output power for 16QAM is TBC. For issue 1-1-3, there’s no consensus yet on whether to define new 16QAM FRC for BS Rx dynamic range tests. Candidate options: Recommendations for 2nd round: Continue to discuss issue 1-1-2 and 1-1-3 in the 2nd round. |
| Sub-topic #1-2 | Tentative agreements: For issue 1-2-1, consensus is reached that the IBE mask should be updated using the formula proposed by Nokia and Huawei. For issue 1-2-2, one 16QAM MPR table for both PC3 and PC5 is proposed after averaging the simulation results provided by two companies. A third company requests more time to check. Candidate options: Recommendations for 2nd round: Companies are encouraged to check the proposed MPR table and reach consensus in this meeting. |
| Sub-topic #1-3 | Tentative agreements: For issue 1-3-1, no objection is raised against the work plan. One company commented that the work plan will need to be continuously adjusted following RAN1 discussion and decision in the coming RAN4 meetings. Candidate options: Recommendations for 2nd round: |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2114002 CR on NB-IoT IBE mask to allow 16-QAM | According to the comments received, the CR appears to be agreeable. However, it’s better to wait for the outcome of the discussion on EVM and MPR, and include all the UE RF requirements for 16QAM as one package. Otherwise, the IBE mask might be updated in the spec for 16QAM while the EVM and MPR requirements are missing. Suggest to endorse the CR, not approve it right now. |

## Discussion on 2nd round (if applicable)

### Open issues

Issue 1-1-2: EVM limit for 16QAM DL

- Can BS manufacturers declare lower rated output power for 16QAM in standalone mode?

- Proposals

  - Option 1: Yes

  - Option 2: No

  - Option 3: Others (to be proposed)

| Company | Comments |
| --- | --- |

Issue 1-1-3: 16QAM FRC for BS Rx dynamic range

- Are 16QAM FRCs needed for BS Rx dynamic range tests?

- Proposals

  - Option 1: Yes

  - Option 2: No

  - Option 3: Others (to be proposed)

| Company | Comments |
| --- | --- |

Issue 1-2-2: MPR for 16QAM

Consider the MPR table below as a candidate for 16QAM PC3 and PC5

| Tone spacing | Number of tones | Allocated tones | MPR proposal [dB] |
| --- | --- | --- | --- |
| 15 kHz | 3 | 0-2 | 1.8 |
|  |  | 3-5 | 1.8 |
|  |  | 6-8 | 1.8 |
|  |  | 9-11 | 1.8 |
|  | 6 | 0-5 | 1.8 |
|  |  | 6-11 | 1.8 |
|  | 12 | 0-11 | 2.8 |

- Proposals

  - Option 1: Agree with the MPR table

  - Option 2: Need revisions (please propose)

  - Option 3: Others (to be proposed)

| Company | Comments |
| --- | --- |

## Summary for 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

# Topic #2: LTE-MTC

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2114344 | Ericsson | Observation#1: If the output power were kept the same for PUSCH sub-PRB but reduced for PRACH, PUCCH and full-PRB PUSCH, there will be an MCL loss for the channels subject to a power reduction which translates into a coverage loss. Observation#2: From control channel coverage point of view, the new power behaviour UE will be the same with a power class UE of which the rated power is 3 dB less. Observation#3: If only PUSCH sub-PRB transmissions were boosted, there might not be benefits in terms of coverage since the MCL of full-PRB PUSCH and other physical channels would remain the same. Observation#4: From a resource utilization perspective, simulation results showed that even if a 3dB power boosting were applied to sub-PRB using ℼ /2-BPSK no gain would be observed with respect to a non-boosted sub-PRB transmission using QPSK. Observation#5: Reducing the full-PRB transmission power generally is against the UE rated power definition. Proposal-1: Follow the framework of NR pi/2 BPSK power boosting if RAN4 decides that there is an overall gain from the subPRB boosting. Observation#6: If RAN4 decided for the subPRB power boosting, it will be possible to boost power for 2 out 3 tone subPRB transmission thanks to low PAPR characteristic. Proposal-2: Focus on PC5 CAT-M1 device for the potential power boosting to PC3 on subPRB transmission. |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 2-1 Feasibility study on max power reduction for PRACH, PUCCH, and full-PRB PUSCH

Sub-topic description: According to the agreed WF, two options are going to be explored for the feasibility study.

Open issues and candidate options before e-meeting:

Issue 2-1: Feasibility study on max power reduction

## Companies views’ collection for 1st round

### Open issues

Issue 2-1: Feasibility study on max power reduction

- Proposals

  - Option 1: Legacy UE power class, New MPR for sub-PRB allocation, and new MPR for PRACH, PUCCH and full-PRB PUSCH

  - Option 2: Legacy UE power class, legacy MPR for PRACH, PUCCH, and full-PRB PUSCH, new MPR for sub-PRB allocation

    - Proposal-1: Follow the framework of NR pi/2 BPSK power boosting if RAN4 decides that there is an overall gain from the subPRB boosting.

    - Proposal-2: Focus on PC5 CAT-M1 device for the potential power boosting to PC3 on subPRB transmission

- Recommended WF

  - Discuss the above options

| Company | Comments |
| --- | --- |
| Qualcomm | We would support Option 2 but we still need to understand the details better. Why not create a new power class altogether? |
| Ericsson | Seems only Ericsson contribute this topic for several meetings in a row, if no other companies would commit to contribute this topic in future meetings, suggestion is to conclude this topic in this meeting. We can try to discuss below in a potential WF to conclude this: - One company does not see the apparent gain to introduce this feature at network and thus it is recommended from RAN4 not pursue this feature in RAN4 and an updated WID to remove the below objective will be followed: - For UEs supporting PUSCH sub-PRB resource allocation, study and if found feasible, specify support power reduction for PRACH, PUCCH, and full-PRB PUSCH, with a maximum reduction of e.g. 3 dB below sub-PRB PUSCH power. [LTE-MTC] [RAN4] |
| Sony | Option 2 is preferred. As Qualcomm mention, a new power class could be an option to study further? |
| Huawei | We support Option 1 because we believe that’s the correct interpretation of the WID objective, i.e. power reduction, not power boost. We need more time to study the topic. We’re also fine with Ericsson’s suggestion, i.e. remove the related objective from the WID due to lack of interest from companies. |

### CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| XXX | Company A |
|  | Company B |
| YYY | Company A |
|  | Company B |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic# 2-1 | Views are still diverged on this topic. And there seems to be lack of interest from companies. Tentative agreements: Candidate options: A potential WF to conclude this is proposed by Ericsson as follows: - One company does not see the apparent gain to introduce this feature at network and thus it is recommended from RAN4 not pursue this feature in RAN4 and an updated WID to remove the below objective will be followed: - For UEs supporting PUSCH sub-PRB resource allocation, study and if found feasible, specify support power reduction for PRACH, PUCCH, and full-PRB PUSCH, with a maximum reduction of e.g. 3 dB below sub-PRB PUSCH power. [LTE-MTC] [RAN4] Recommendations for 2nd round: Discuss the potential WF. |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round (if applicable)

Issue 2-1: Feasibility study on max power reduction

No apparent gain is observed to introduce this feature at network and thus it is recommended not to pursue this feature in RAN4 and an updated WID to remove the below objective will be followed:

- For UEs supporting PUSCH sub-PRB resource allocation, study and if found feasible, specify support power reduction for PRACH, PUCCH, and full-PRB PUSCH, with a maximum reduction of e.g. 3 dB below sub-PRB PUSCH power. [LTE-MTC] [RAN4]

- Proposals

  - Option 1: Agree with the proposal

  - Option 2: Disagree

  - Option 3: Others (to be proposed)

| Company | Comments |
| --- | --- |

## Summary for 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

# Topic #3: Downlink interruption for band combinations to conduct dynamic Tx Switching

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2111924 | CATT | TR 37.867 v0.4.0 |
| R4-2112496 | China Telecom | CR to 38.101-1 Introduce DL interruption clarification for CA conduting Tx Switching |

## Companies views’ collection for 1st round

### CRs/TPs comments collection

For close-to-finalize WIs and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2111924 | [Moderator]: No new input, can be withdrawn? |
|  | Company B |
| R4-2112496 | [Moderator]: big CR for email approval, or can be withdrawn? |
|  | Company B |

## Summary for 1st round

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

# Topic #4: A-MPR for LTE Band 24 for UE categories M1 and M2

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2112317 | Ligado Networks | Reason for change: Adding LTE Band 24 for CAT-M1/M2 Summary of change: Adding A-MPR requirement for LTE Band 24 for CAT-M1/M2 |

### CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2112317 CR for adding A-MPR for LTE Band 24 for UE categories M1 and M2 | Company A |
|  | Company B |

Summary for 2nd round (if applicable)

# Recommendations for Tdocs

## 1st round

New tdocs

| Title | Source | Comments |
| --- | --- | --- |
| WF on BS and RF requirements in support of NB-IoT 16QAM | Huawei, HiSilicon | To capture the agreements in this meeting |
| WF on max power reduction for PRACH, PUCCH, and full-PRB PUSCH | Ericsson |  |

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2112280 | Proposals on BS RF requirements for support of 16QAM in NB-IoT | Nokia, Nokia Shanghai Bell | To be noted |  |
| R4-2113620 | MPR for NB-IoT 16-QAM with modified IBE | Nokia, Nokia Shanghai Bell | To be noted |  |
| R4-2114002 | CR on NB-IoT IBE mask to allow 16-QAM | Nokia, Nokia Shanghai Bell | To be endorsed | Wait for the outcome of MPR and EVM discussions. Approve all UE RF requirements for 16QAM as one package. |
| R4-2114216 | Discussion on in-band emission requirements for 16QAM NB-IoT Uplink | Huawei, HiSilicon | To be noted |  |
| R4-2114217 | Further discussion on BS RF requirements for 16QAM NB-IoT Downlink | Huawei, HiSilicon | To be noted |  |
| R4-2114218 | Work plan of Rel-17 enhancements for NB-IoT and LTE-MTC | Huawei, Ericsson | To be endorsed |  |
| R4-2114344 | RF impact analysis on Rel-17 eMTC WID | Ericsson | To be noted |  |
| R4-2114345 | BS RF impact analysis on R17 NB_IoT | Ericsson | To be noted |  |
| R4-2114346 | UE RF impact analysis on R17 NB_IoT | Ericsson | To be noted |  |
| R4-2111924 | TR 37.867 v0.4.0 | CATT | To be withdrawn |  |
| R4-2112496 | CR to 38.101-1 Introduce DL interruption clarification for CA conduting Tx Switching | China Telecom | To be withdrawn |  |
| R4-2112317 | CR for adding A-MPR for LTE Band 24 for UE categories M1 and M2 | Ligado Networks | To be returned to | Moved from [148] |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

## 2nd round

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-210xxxx | WF on … | YYY | Agreeable, Revised, Noted |  |
| R4-210xxxx | LS on … | ZZZ | Agreeable, Revised, Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents