3GPP TGS-RAN WG4 Meeting #100e	R4-2114752
Electronic Meeting, Aug. 16 – 27, 2021

Title:	LS on lower 6GHz NR unlicensed operation for Europe

Response to:

Release:	Rel-17

Work Item:	NR_6GHz_unlic_EU

Source:	TSG RAN WG4

To:	TSG RAN

Contact person:	Johannes Hejselbaek

Johannes.hejselbaek@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

According to the WID for the Introduction of lower 6GHz NR unlicensed operation for Europe RAN4 is tasked to [1]:

- Depending on the details of the European regulatory requirements, determine whether they are best handled by relevant updates (if any) of band n96 or whether a new band is needed.

  - If a new band is needed, determine the band plan for unlicensed operation in the range 5945-6425 MHz

Therefor the two options list below have been discussed:

- Option 1: Re-using already defined band n96, for the frequency range 5945 MHz to 6425 MHz

  - FFS if additional notes and/or clarifications are needed. Regional specific requirements to be included in relevant specifications.

- Option 2: Defining a new band n[xx], for the frequency range 5945 MHz to 6425 MHz

  - On top of specific requirements provided by ECC, the new band shall reuse requirements already defined for n96, where possible.

During the RAN4#98, RAN4#98bis and RAN4#99 the two options have been discussed without any resolution. As a result, the matter was raised to RAN discussion at RAN#92. At RAN#92 no conclusion was achieved and RAN4 was tasked to compare option 1 (Re-using already defined band n96) and option 2 (Defining a new band n[xx]) regarding requirements and signalling at RAN4#100 meeting and to bring a comparison back to RAN#93e. The decision between the two options was then to take place at RAN#93. This LS conveys the requested comparison and related observations.

RAN4 has agreed that:

- Both option 1and option 2 are technically feasible and can support unlicensed operation for Europe in the frequency range 5945 MHz to 6425 MHz.

- The same hardware of UE as for n96 may be reused on the frequency range 5945MHz to 6425MHz no matter whether to define a new band or define new NS for the existing n96.

- The necessary changes in UE specification 38.101-1 and BS specification 38.104 is detailed in table 1 and 2 below.

In the tables below it is indicated whether or not specification(s) need changes (i.e. amendments or clarifications) or not (i.e. can be re-used as is). Further, comments on actions needed or concerns have been added.

Table 1 – Comparison of UE specification (TS 38.101-1) impact for the two options

| Affected clause | Requirement | Option 1 Re-using already defined band n96 | Option 2 Defining a new band n[xx], |
| --- | --- | --- | --- |
| 5.2 | Operating bands | No changes needed Existing band n96 is re-used FUL_low: Limited to 5945 MHz FUL_high: Limited to 6424 MHz FDL_low: Limited to 5945 MHz FDL_high: Limited to 6424 MHz | Changes needed A new band [nXX] is added FUL_low: Defined as 5945 MHz FUL_high: Defined as 6424 MHz FDL_low: Defined as 5945 MHz FDL_high: Defined as 6424 MHz |
| 5.2A.1, 5.2A.2 | Intra-band CA and inter-band CA | Potential re-use of band n96 combinations limited to the frequency range available in Europe. | Potential duplicating of band combinations with new band number. |
| 5.3.5 | UE channel bandwidth per operating band | No changes needed Existing channel bandwidth will be re-used. Including 100MHz CBW. | Changes needed Duplicating and re-using of the n96 channel bandwidth for the new band in the bandwidth table. Including 100MHz CBW. |
| 5.4.2.3 | Channel raster | No changes needed Existing channel rasters are re-used (with a NOTE if needed limiting the range for the EU/CEPT region) | Changes needed Duplicating and re-using of the n96 channel rasters, that correspond to the 5945-6425MHz range |
| 5.4.3.3 | Sync raster | No changes needed Existing sync rasters are re-used (with a NOTE if needed limiting the range for the EU/CEPT region) | Changes needed Duplicating and re-using of the n96 sync rasters corresponding to the 5945-6425MHz range |
| 5.5A.1 | Configurations for intra-band contiguous CA | No changes needed Band n96 intra-band CA configurations will be re-used, including being standardised NR-U UL CA. | Changes needed Duplicating and re-using of n96 intra-band CA configurations, including being standardised NR-U UL CA. |
| 5.5A.3.1 | Configurations for inter-band CA (two bands) | Potential re-use of band n96 combinations limited to the frequency range available in Europe. | Potential duplicating of band combinations with new band number. |
| 6.2F.1 | UE maximum output power | Specification changes New NS value(s) corresponding to the EU/CEPT regulations | Specification changes New NS value(s) corresponding to the EU/CEPT regulations |
|  |  | Some companies expresses concern with having multiple NS values defined for different regions for a single band and stated that European regulatory conformance cannot be meet with this approach. However, other companies do not share these concerns and note that other bands defined within the 3GPP specification also have multiple NSs for different regions. |  |
| 6.2F.2 | UE maximum output power reduction | No changes needed | No changes needed |
| 6.2F.3 | UE additional maximum output power reduction | Changes needed New NS value(s) added but only applicable for the 5945-6425 MHz range restricting operation within this frequency range. | Changes needed New NS value(s) for the new band |
| 7.3F | Reference sensitivity | No changes needed Band n96 requirements are re-used. Including REFSENS exceptions for band combinations in 38.101-1 and 38.101-3 | Changes needed Duplicating and re-using of band n96 requirements. Including REFSENS exceptions for band combinations in 38.101-1 and 38.101-3 |
| 7.6F.2 | In-band blocking characteristics | FFS | FFS |
| 7.6F.3 | Out-of-band Blocking characteristics | FFS | FFS |

Table 2– Comparison of BS specification (TS 38.104) impact for the two options

| Affected clause | Requirement | Option 1 Re-using band n96 | Option 2 Defining a new band n[xx] |
| --- | --- | --- | --- |
| 5.2 | Operating band | No changes needed Existing band n96 is re-used FUL_low: Limited to 5945 MHz FUL_high: Limited to 6424 MHz FDL_low: Limited to 5945 MHz FDL_high: Limited to 6424 MHz | Changes needed A new band [nXX] is added FUL_low: Defined as 5945 MHz FUL_high: Defined as 6424 MHz FDL_low: Defined as 5945 MHz FDL_high: Defined as 6424 MHz |
| 5.4.2.3 | Channel raster entries for each operating band | No changes needed Existing channel bandwidth will be re-used. Including 100MHz CBW. | Changes needed Duplicating and re-using of the n96 channel bandwidth for the new band in the bandwidth table. Including 100MHz CBW. |
| 5.4.3.3 | Synchronization raster entries for each operating band | No changes needed Existing channel rasters are re-used (with a NOTE if needed limiting the range for the EU/CEPT region) | Changes needed Duplicating and re-using of the n96 channel rasters, that correspond to the 5945-6425MHz range |
| 6.6.1 | General | FFS if the definition of Foube have to be clarified for the range 5945-6425 MHz when operated in EU | FFS if Foube needs redefining for new band |
| 6.6.3.2 | ACLR | No changes needed | Changes needed Duplicating and re-using from n96 and add the new band number |
| 6.6.5.2 | UEM | FFS | FFS |
| 7.2.2 | REFSENS | No changes needed | Changes needed Duplicating and re-using from n96 and add the new band number |
| 7.3.2 | Dynamic range | No changes needed | Changes needed Duplicating and re-using from n96 and add the new band number |
| 7.4.1.2 | ACS requirement | No changes needed | Changes needed Duplicating and re-using from n96 and add the new band number |
| 7.4.2 | In-band blocking | Changes needed Modify the requirement for 5945-6425 so note is not based on operating band but on restricted frequency range (and potentially a modified Foube) when operating in EUFFS | Changes needed Clarify Foube requirements for new band with the range 5945-6425 MHz |
| 7.5.2 | Out-of-band blocking | Changes needed Modify the requirement for 5945-6425 so note is not based on operating band but on restricted frequency range (and potentially a modified Foobb) when operating in EUFFS | Changes needed Clarify Foobb requirements for new band with the range 5945-6425 MHz |
| 7.5.3 | Co-location minimum requirements | Changes needed Paragraph 4 and Note 3 would have to be modified to take into account the range 5945-6425 MHz when operated in EU (not the operating band as the exclusions would only exist for the allocated sub-band). | No change needed |
| 7.6.2 | Receiver spurious emissions | Changes needed Modify the requirement for 5945-6425 so note is not based on operating band but on restricted frequency range (and potentially a modified Foube) when operating in EU | No change needed |
| 7.7.2 | Intermodulation requirement | No changes needed | Changes needed Duplicating and re-using from n96 and add the new band number |
| 7.8.2 | ICS requirement | No changes needed | Changes needed Duplicating and re-using from n96 and add the new band number |

# 2	Actions

To 3GPP RAN

ACTION: RAN4 respectfully asks RAN to take the information detailed above into account.

# 3	Dates of next TSG RAN WG4 meetings

TSG RAN WG4 Meeting #101-e	November 1 – 12, 2021	          E-meeting

TSG RAN WG4 Meeting #102-e	February 21 – 25, 2022	          TBC

# 4	References

- RP-210762, Introduction of lower 6GHz NR unlicensed operation for Europe, WID