3GPP TSG-RAN WG4 Meeting # 100-e	 												R4-2115016

Electronic Meeting, 16-27 August, 2021

Agenda item:			8.26

Source:	Moderator (Ericsson)

Title:	Email discussion summary for [100-e][116] NR_bands_R17_BWs

Document for:	Information

# Introduction

The basket WI was agreed in RAN#88e meeting to manage all requests related to adding new channel BW in existing NR bands.

This agenda item will handle all contributions related to this WI:

- Endorsement of the updated WI including the new requests submitted for this meeting:

  - Adding 70MHz in band n40.

  - Adding 70 and 90MHz in band n97, and 100MHz for BS in band n97.

  - Adding 10, 20, 30, 70 and 90MHz in band n79.

- Start or continue discussion on:

  - Adding 25, 30 and 40 MHz CBW in band n2.

  - Adding 25 MHz CBW in band n5.

  - Adding 25 and 30MHz in band n71.

  - Adding 100MHz in bands n46 and n96

# Topic #1: Rapporteur inputs

This topic is aiming endorsing the updated WI with new requests submitted for this meeting.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2113737 | Ericsson | WID revision including new requests made for this meeting |
| R4-2113738 | Ericsson | Big CR to TS 38.104 This CR will merge all draft CRs endorsed in the 1st / 2nd round. |
| R4-2113739 | Ericsson | Big CR to TS 38.101-1 This CR will merge all draft CRs endorsed in the 1st / 2nd round. |
| R4-2112474 | TTA | The status of 5G local network spectrum for vertical service in the Republic of Korea |

## Open issues summary

### Sub-topic 1-1

Sub-topic description: A new request has been submitted for this meeting.

Open issues and candidate options before e-meeting: Endorsed the revised WID

Issue 1-1: New request adding 70MHz in band n40.

- Proposals

  - Comments are welcome.

- Recommended WF

  - Approve the new request and endorse the revised WID.

### Sub-topic 1-2

Sub-topic description: A new request has been submitted for this meeting.

Open issues and candidate options before e-meeting: Endorsed the revised WID

Issue 1-2: New request adding 70, 90, 100 MHz for UE and 70, 90 MHz for BS in band n97.

- Proposals

  - Comments are welcome.

- Recommended WF

  - Approve the new request and endorse the revised WID.

### Sub-topic 1-3

Sub-topic description: A new request has been submitted for this meeting.

Open issues and candidate options before e-meeting: Endorsed the revised WID

Issue 1-3: New request adding 10, 20, 30, 70 and 90MHz in band n79.

- Proposals

  - Comments are welcome.

- Recommended WF

  - Approve the new request and endorse the revised WID.

## Companies views’ collection for 1st round

### Open issues

| Company | Comments |
| --- | --- |
| XXX | Issue 1-1: Issue 1-2: Issue 1-3: Others: |
| ZTE | Issue 1-3: For introduction of 10,20,30MHz in band 79, since GSCN for n79 is based on minimum channel bandwidth 40MHz with step size 16, however if lower than 40MHz is introduced, then GSCN step size should be updated which might have impact on UE access procedure, we would like to further discuss this view with more BS and UE vendors how to handle this issue. |

### CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| NA |  |

## Summary for 1st round

### Open issues

|  | Status summary |
| --- | --- |
| Sub-topic #1-1 | Tentative agreements: No comment received in the 1st round, this proposal could be endorsed by RAN4. Recommendations for 2nd round: NA |
| Sub-topic #1-2 | Tentative agreements: No comment received in the 1st round, this proposal could be endorsed by RAN4. Recommendations for 2nd round: NA |
| Sub-topic #1-3 | One company has some concerns (GSCN step size and potential impact on UE access procedure) with this request and is looking for feedbacks from other delegates. Recommendations for 2nd round: Please provide feedback on ZTE concerns and any recommendation on how to address this. |

### CRs/TPs

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |

## Discussion on 2nd round (if applicable)

Issue 1-3: New request adding 10, 20, 30, 70 and 90MHz in band n79 - GSCN step size and potential impact on UE access procedure

Check also R4-2112474 which gives important background to this request.

| Company | Comments |
| --- | --- |
| Samsung | Thank you for the comment on the GSCN for n79. So far, we believe the most straightforward way to resolve the issue would be changing the step size from 16 to 1 as the band of n77 and n78. However, once the revised WID is approved, we can also discuss other possible solutions given the potential conditions for the new channel bandwidth. We will try to contact offline with more BS and UE vendors how to handle this issue before the next meeting. |

## Summary on 2nd round (if applicable)

|  | Status summary |
| --- | --- |
| Issue 1-3 | Tentative agreements: The GSCN concern should be further discussed when working on the introduction of the new channel BW for n79. The request could be endorsed. |

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| R4-2113737 | To be endorsed |
| R4-2113738 | For email approval |
| R4-2113739 | For email approval |

# Topic #2: Band n2 – 25, 30 and 40MHz

This topic is focusing on adding 25, 30 and 40MHz CBW support in band n2.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2111745 | AT&T | Draft CR to TS 38.101-1 |
| R4-2111747 | AT&T | Draft CR to TS 38.104 |

## Open issues summary

No issue has been raised for this topic.

- Recommended WF: Moderator’s recommendation is to focus on providing comment to the proposed draft CRs and endorse them.

## Companies views’ collection for 1st round

### Open issues

Issue 2-1:  NA

| Company | Comments |
| --- | --- |
| XXX |  |

### CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2111745 | Draft CR to 38.101-1 Introduction of Additional CBWs for Band n2 |
|  | Skyworks: We are fine with the proposed REFSENS levels (Table 7.3.2-1) and UL Configuration proposals (Table 7.3.2-3). Editorial comment: Table 7.3.2-1 header rows could not be displayed when opening the draftCR on our version of Word. May be an issue related to management of landscape format? |
|  | AT&T: Thanks for the confirmation on the REFSENS levels and UL configurations. We can see the header row for Table 7.3.2-1 but for some reason it is splitting across pages and seems to be in the top margin area. Perhaps, Draft View or Web Layout views could help. |
|  | Qualcomm: We are fine with the REFSENS values/ULconfiguration. |
|  | Murata: We are ok to the REFSENS values and UL configurations. |
| R4-2111747 | Draft CR to 38.104 Introduction of Additional CBWs for Band n2 |
|  | Company A |

## Summary for 1st round

### Open issues

|  | Status summary |
| --- | --- |
| Sub-topic#1 | NA |

### CRs/TPs

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2111745 | Companies are fine with the proposed values in the CR. Regarding the editorial comment, if confirmed, it could be fixed when writing the big CR. This draft CR is endorsable. |
| R4-2111747 | No comment has been received, the draft CR is endorsable. |

## Discussion on 2nd round (if applicable)

NA

# Topic #3: Band n5 –25 MHz

This topic is focusing on adding 25MHz CBW support in band n5.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2111746 | AT&T | Draft CR to 38.101-1 Introduction of 25MHz CBW for Band n5 |
| R4-2111748 | AT&T | Draft CR to 38.104 Introduction of 25MHz CBW for Band n5 |
| R4-2114584 | Skyworks Solutions Inc. | Proposal 1: Adopt changes highlighted in yellow in Table 1 for Table 5.3.5-1 (subclause 5.3.5). Proposal 2: Adopt following highlighted text proposal based on [3] for subclause 5.3.6 and highlighted changes in Table 2 for Table 5.3.6-1. Proposal 3: Adopt the highlighted changes in Table 4 for Table 7.3.2-1 and in Table 5 for Table 7.3.2-3 for sub-clause 7.3.2. |

## Open issues summary

### Sub-topic 3-1

Sub-topic description: Draft CRs

- Proposals: There are few differences between the draft CR to TS 38.101-1 from AT&T and the proposed changes from Skyworks, they are listed here after:

- Clause 5.3.6: Skyworks proprose the following additional change:

“In FDD, the confinement is defined as a maximum deviation to the Tx-Rx carrier center…”

- REFSENS values: there is 0.1dB difference between the 2 proposals, a compromise should be easy to find.

- Skyworks propose to add a new note 8 with the REFSENS values:

NOTE 8: For n5, REFSENS is specified at [881.5] MHz downlink carrier frequency.

- The new note 5 added to the RB allocation has different wording:

  - AT&T’s proposal:

NOTE 5:	The highest valid UL configuration with TX-RX separation (Table 5.4.4-1) equal to 0 shall be used.

  - Skyworks’ proposal:

NOTE 5:   For band n5, uplink channel bandwidth is restricted to 20MHz and [836.5]MHz uplink carrier frequency shall be used.

- Recommended WF: Moderator’s recommendation would be to focus on providing comment to the proposed draft CRs directly and possibly endorse them after revision.

## Companies views’ collection for 1st round

### Open issues

Sub topic 3-1: Draft CRs

| Company | Comments |
| --- | --- |
| XXX |  |

CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2111746 | Draft CR to 38.101-1 Introduction of Additional CBWs for Band n5 |
|  | Skyworks: 1) The addition word “maximum” in Clause 5.3.6 (proposal 2 R4-2114584) does not need to be captured in R4-2111746 as long as it is captured in bigCR that introduces 35MHz-45MHz CBW. Note this change was agreed in WF R4-2107819 – we make proposal in R4-2112023. 2) REFSENS levels (Table 7.3.2-1): we are fine with the proposed levels, 3) UL configuration for REFSENS – Table 7.3.2-3 NOTE 5, we prefer this CR’s version rather than our P3 note 8 / Note 5. We would like to propose small changes which make NOTE 5 generic enough to apply to all bands for which REFSENS is specified for asymmetric UL/DL and where UL – DL frequency separation is set to the default duplex distance/default TX/RX separation as: NOTE 5: For this DL channel bandwidth, the highest valid UL configuration with TX-RX separation (Table 5.4.4-1) equal to 0 shall be used channel bandwidth specified in Table 5.3.6-1 and the default Tx-Rx frequency separation specified in Table 5.4.4-1 shall be used. Advantage of this approach: - RAN4 no longer has to specify the exact UL and DL carrier frequencies for each band of operation where asymmetric UL/DL operation has been agreed. Currently RAN4 there are ongoing discussions for bands n8 20MHz UL/35MHz, n25 40MHz UL. 45MHz DL and n71 20MHz UL /35MHz DL. Adopting this NOTE 5 proposal means that Note 8 in Table 7.3.2-1 (R4-2114584) can be removed, and specific carrier frequencies no longer need to be captured, making this NOTE5 future proof in case more such cases are requested. - It may allow RAN5 to test reference sensitivity across multiple channels if needed. Pre-condition to adopt NOTE5 proposal: - Agree that for REFSENS for bands with asymmetric UL/DL CBW, the UL/DL carrier frequencies are set at their default Tx-Rx frequency separation value as specified in Table 5.4.4-1. - Agree on adopting the REFSENS levels that correspond to such UL Fc / DL Fc frequency separation. |
|  | AT&T: Responses to Skyworks comments. 1) We are OK to capture the addition of the word “maximum” in clause 5.3.6 in our draft CR. This will help with traceability for the big CR. We will upload a revision with this change. 2) Thanks for the confirmation. 3) We are OK to update NOTE 5 as suggested. We will upload a revision with this change. |
|  | Skyworks: Thank you for revision 1 of draft CR R4-2111746. This is agreeable to us. The updated Note 5 will help simplify the 35MHz/45MHz bigCR discussed in thread [117]. |
|  | Qualcomm: We are fine the revision of the CR. |
|  | Murata: We are ok to the revision. |
| R4-2111748 | Draft CR to 38.104 Introduction of Additional CBWs for Band n5 |
|  | Company A |

## Summary for 1st round

### Open issues

|  | Status summary |
| --- | --- |
| Sub-topic#1 | NA |

### CRs/TPs

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2111746 | This draft CR shall be revised. A draft revision has already been proposed and it seems agreeable by everyone. If no further comment in the 2nd round, the revision will then be endorsed directly. |
| R4-2111748 | No comment received in the 1st round, to be endorsed. |

## Discussion on 2nd round (if applicable)

No discussion is expected in the 2nd round, a revision to the draft CR has already been proposed and seems acceptable by everyone.

Sub topic 3-1: Draft CRs

| Company | Comments |
| --- | --- |
| Skyworks | Rev1 R4-2111746: Based on offline discussions with AT&T, it is proposed to: - further improve NOTE 5 by adding the clarification highlighted in yellow below. This removes any ambiguity about which UL RBs shall be used for 25MHZ REFSENS verification. So, for n5 25MHz DL, NOTE5 indicates that UL should be configured with 20MHz and that the UL RB Configuration of 20MHz shall be used, ie 20RB/10RBs for SCS 15 / SCS 30, NOTE 5: For this DL channel bandwidth, the UL configuration of the highest UL channel bandwidth specified in Table 5.3.6-1 and the default Tx-Rx frequency separation specified in Table 5.4.4-1 shall be used. - Remove the UL RB specified in the column 25MHz and replace with text “NOTE 5”. These changes make this CR aligned with the proposed revision2 bigCR in [117]. |
| AT&T | Rev 2 R4-2111746 was posted to the Round 2 folder to align with the proposal above. |

## Summary on 2nd round (if applicable)

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| R4-2114916 (Revision of R4-2111746) | Skyworks’ comments have been addressed in Rev2 of R4-2111746. No comment has been received after. To be endorsed |

# Topic #4: Band n71 – 25 and 30 MHz

This topic is focusing on adding 25 and 30MHz CBW support in band n71.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2112737 | Murata | Observation 1: With limiting the UL BW to 20MHz, PA Tx noise can be significantly reduced. Proposal 1: For Band n71 25MHz and 30MHz channel bandwidth operation, restrict the UL channel bandwidth to 20MHz, and keep existing duplex offset between center of RX and TX channel BWs. Proposal 2: REFSENS and UL configurations for band n71 25MHz and 30MHz Channel bandwidth operation are proposed highlighted as Table 3 and Table 4. Operating Band; SCS kHz; 5 MHz (dBm); 10 MHz (dBm); 15 MHz (dBm); 20 MHz (dBm); 25 MHz (dBm); 30 MHz (dBm); 35 MHz (dBm). n71; 15; -97.2; -94; -91.6; -86; -83.6; -82.3; [-80.7]. 30; -94.3; -91.9; -87.4; -83.7; -82.4; [-80.8]. 60. Operating Band; SCS kHz; 5 MHz (dBm); 10 MHz (dBm); 15 MHz (dBm); 20 MHz (dBm); 25 MHz (dBm); 30 MHz (dBm); 35 MHz (dBm). n71; 15; 25; 25; 20; 20; 20; 20; [20]. 30; 12; 10; 10; 10; 10; [10]. 60. |

## Open issues summary

### Sub-topic 4-1

Sub-topic description: Symmetric or asymmetric UL/DL for 25 and 30MHz channel BW.

Issue 4-1: channel BW in UL:  asymmetric (20 MHz) or symmetric.

- Proposals: Channel BW in UL should be limited to 20MHz for 25 and 30MHz.

  - Yes (Murata)

  - No

- Recommended WF

  - Further discussion would be needed to collect other companies’ view.

Issue 4-2: REFSENS limits and RB allocations

- Proposals: Following option should be further discussed

  - Option 1:

| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n71 | 15 | -97.2 | -94 | -91.6 | -86 | -83.6 | -82.3 | [-80.7] |
|  | 30 |  | -94.3 | -91.9 | -87.4 | -83.7 | -82.4 | [-80.8] |
|  | 60 |  |  |  |  |  |  |  |

| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n71 | 15 | 25 | 25 | 20 | 20 | 20 | 20 | [20] |
|  | 30 |  | 12 | 10 | 10 | 10 | 10 | [10] |
|  | 60 |  |  |  |  |  |  |  |

  - Other

- Recommended WF

  - TBA

## Companies views’ collection for 1st round

### Open issues

Issue 4-1: channel BW in UL:  asymmetric (20 MHz) or symmetric

| Company | Comments |
| --- | --- |
| Skyworks | Additional measurements/data is needed before we can accept REFSENS levels for 25MHz and 30MHz CBW. We can present data at next meeting for n71. |
| Qualcomm | Qualcomm would like to bring contribution with analysis to the next meeting. |
| Murata | We agree with recommended WF. |
| Apple | We would like to ask for time until next meeting to provide an analysis on both scenarios. |

Issue 4-2: REFSENS values and RB allocations

| Company | Comments |
| --- | --- |
| Skyworks | It seems reasonable to adopt the same UL RB configurations for 25MHz and 30MHz than what RAN4 has agreed for 35MHz. If RAN4 agrees that REFSENS is specified assuming that UL and DL carrier frequencies are set to their default Tx-Rx frequency separation value specified in Table 5.4.4-1, we propose to adopt the generic NOTE 5 for n71 as follows (based on comments for n5 25MHz): NOTE 5: For this DL channel bandwidth, the highest UL channel bandwidth specified in Table 5.3.6-1 and the default Tx-Rx frequency separation specified in Table 5.4.4-1 shall be used. |
| Qualcomm | The UL configuration is reasonable |
| Murata | We agree with Skyworks comments. |
| Apple | We would like to ask for time until next meeting to provide an analysis for the REFSENS for 25 and 30 MHz. In terms of the RB allocation, the numbers seem reasonable. |

CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |

## Summary for 1st round

### Open issues

|  | Status summary |
| --- | --- |
| Sub-topic#1 | REFSENS values and RB allocations Tentative agreements: The RB allocation seems agreeable by everyone. REFSENS values discussion should be postponed to next meeting, waiting for additional inputs. Candidate options: Recommendations for 2nd round: Capture the RB allocation in a WF |

### CRs/TPs

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | NA |

## Discussion on 2nd round (if applicable)

Sub topic 4-1: WF

| Company | Comments |
| --- | --- |
| Skyworks | Draft WF on n71 REFSENS configuration Thank you for bringing this WF. The agreement reflects our views for UL RB configuration, we are fine with it. Please note that when time comes to file CR, it would be good to align the way UL configuration is captured with the way we coordinated n5 25MHz CR with [117] bigCR. One suggestion is to adopt this table for UL RB configurations. In [117] bigCR, the NOTE 5 is simply added to cells corresponding to 35MHz CBW. Operating band / SCS / Channel bandwidth / Duplex-mode. Operating Band; SCS kHz; 5 MHz (dBm); 10 MHz (dBm); 15 MHz (dBm); 20 MHz (dBm); 25 MHz (dBm); 30 MHz (dBm); 35 MHz (dBm); 40 MHz (dBm); 45 MHz (dBm); 50 MHz (dBm); Duplex Mode. n71; 15; 25; 25; 20; 20; NOTE 5; NOTE 5; FDD. 30; 12; 10; 10; NOTE 5; NOTE 5. 60. NOTE 1: UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.3.2-1).\ NOTE 5: For this DL channel bandwidth, the UL configuration of the highest UL channel bandwidth specified in Table 5.3.6-1 and the default Tx-Rx frequency separation specified in Table 5.4.4-1 shall be used.. One question: at end of round 1, have we clearly reached agreement that for DL 25MHz and DL 30MHz uplink CBW is restricted to 20MHz? This agreement does not seem to be captured in the WF. We would like to suggest the following slide 3 corrections: Proposal 1: For 25MHz and 30MHz downlink CBW operation, it is agreed that UL CBW is restricted to 20MHz, Proposal 2: The following uplink RB configurations for 25 and 30 MHz n71 REFSENS are agreed (we leave it up to proponent to adopt either the proposed WF table format or the suggested table format hereabove that makes use of NOTE 5) Proposal 3: Interested companies are requested to bring additional REFSENS measurements and analysis for 25 and 30 MHz n71 to RAN4#101 based on the uplink configuration above. |
| Apple | We are ok with the proposals made by Skyworks for the WF. |

## Summary on 2nd round (if applicable)

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| R4-2114914 | Above comments have been captured in the final draft of the WF. To be approved |

# Topic #5: NR-U bands n46 and n96 – 100 MHz channel BW

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-1111835 | Charter Communications, Inc | Observation 1: It can be noticed that for 60 MHz channel bandwidth configurations, the channel rasters were defined to fall inside the 80 MHz channel bonding configurations in Wi-Fi. This assures fair co-existence with both technologies. Observation 2: There are several co-existence issues with the proposed channel rasters for 100 MHz channel bandwidth in [1] and [2]. Observation 3: Wideband multi-channel access operations for 100 MHz channel bandwidth needs to consider multiple CAT4 LBT procedures to insure fair co-existence with Wi-Fi. Proposal 1: RAN4 should consider no 100 MHz channel bandwidth configuration in NR-U can overlap two 80 MHz Wi-Fi channel bonding, only 4- 100 MHz channel rasters (5200, 5300, 5520 and 5865 MHz) for NR-U in 5 GHz (n46). Observation4: In 6 GHz in the US, contiguous 80 MHz Wi-Fi channel bonds from U-NII-5 to U-NII-8 can be observed Proposal 2: RAN4 should consider 2-100 MHz NR-U channel bandwidth configurations per 160 MHz Wi-Fi channels (5995 MHz, 6475 MHz, 6055 MHz, 6535 MHz, 6155 MHz, 6635 MHz, 6215 MHz, 6315 MHz, 6695 MHz, 6795 MHz, 6375 MHz, 6855 MHz, 6955 MHz, 7055 MHz and 7075MHz) in 6 GHz (n96). |
| R4-2112031 | CableLabs | Observation 1: the new 100 MHz bandwidth for NR-U introduces a new case of channel puncturing: interior triple puncture. Proposal 1: re-use SEMs up to 80 MHz bandwidth agreed during RAN4 #93 (presented in Section 2 and Figure 1) for the 100 MHz bandwidth edge puncture, interior single puncture and interior double puncture SEMs. Proposal 2: SEM case 1 (-31 dBr) should be used with priority since it has the smallest SINR degradation (compared with the other 2 cases under analysis), see Table 1 and Figure 3. Proposal 3: SEM case 2 (-28 dBr) may be considered alternatively albeit the victim SINR degradation remains a concern, see Table 1 and Figure 3. Proposal 4: another alternative solution is to avoid triple punctured case for the 100 MHz bandwidth. The maximum interior punctured bandwidth is 40 MHz. |
| R4-2112301 | Skyworks Solutions Inc. | Proposal: - 100MHz channel SEM is simply introduced by changing Table 6.5F.2.2-1 in 38.101-1 to its simplified version below with brackets removed - The rest of the specification does not need changes and the image leakage exception to -28dBr and the carrier leakage exception to -28dBc is maintained as already specified in section 6.5F.2.2.1 in 38.101-1 |
| R4-2112302 | Skyworks Solutions Inc. | Observation 1 on channelization rule: Deriving from the 60MHz NR-U channelization rule, NR-U 100MHz channels should not overlap with two 160MHz Wi-Fi channels and there are four overlapping 100MHz channels per 160MHz Wi-Fi channel. Proposal 1 on missing 60 and 80MHz channels: Missing channels are verified and if confirmed added to 38.101-1 Proposal 2 on 100MHz channelization rule: - The 100MHz channel shall not overlap two 160 MHz Wi-Fi channels rule is adopted. - The number of 100MHz channels per 160MHz channels may be restricted to less than four - Channel frequencies and NREF values can be derived from Table 1 based on adopted rule |
| R4-2113067 | Huawei, HiSilicon | Proposal 1: Channel raster for 100MHz CBW in NRU as listed in Table 1 is proposed for n46. Proposal 2: existing MPR values can be applied for 100 MHz channel bandwidth Proposal 3: The BS spectrum emission mask for 100MHz channel bandwidths in NRU should be supported as below: - for full bandwidth transmission, the general spectrum emission mask in NRU is applied. - for single non-transmitted channel in the middle, the emission mask of the puncture center is limited at -23dBr - for multiple non-transmitted channels in the middle, the emission mask in the middle is floored at -25dBr |
| R4-2113664 | Mediatek India Technology Pvt. | Observation 1: RF filter can reject out-of-band interferers but not feasible to deal with in-band interferers. ACS can be achieved by combining rejection capabilities from IF (Intermediate frequency) analog and digital filters. Observation 2: In TS 38.101-1, the Table 7.5-2 and Table 7.5-5 indicate that ACS for NR bands with FDL_low ≥ 3300 MHz is 33dB and the BW of adjacent interferer is assumed to be the same as desired signal BW. Thus, regarding implementation of IF filters’ rejection capability, the upper bound of ACS could be 33dB. Observation 3: For co-existence scenarios, the NR-U ACS requirement can be better or equal to Wi-Fi adjacent-interferer rejection capability, as shown below. Wi-Fi’s rejection requirement to adjacent interferer is 16dB. Proposal 1: To consider adopting NR-U ACS of 17.5 dB for 100MHz CBW, which is equal to Wi-Fi rejection capability for adjacent interferer. RX parameter; Units; Channel bandwidth. 20 MHz; 40 MHz; 60 MHz; 80 MHz; 100 MHz. ACS; dB; 24; 21; 19.2; 18; [17.5]. |
| R4-2113937 | ZTE Corporation | Proposal 1: propose to define 100MHz BW for NR-U. Proposal 2: to adopt the channel raster for 100MHz in Table 1. Proposal 3: to adopt the intra-carrier GB for 100MHz in Table 2. CBW; SCS [KHz]; GB@80MHz; GB@20MHz; Intra-band carrier PRB configuration. 100MHz; 30; 845KHz; 805KHz; 6PRB(51~56) 5PRB(107~111) 6PRB(162~167) 6PRB(218~223). 60; 1370KHz; 970KHz; 4PRB(25~28) 3PRB(53~55) 4PRB(80~83) 4PRB(108~111). Note: PRB index is starting from 1.. Proposal 4: to adopt SEM with three non-transmitted carriers for 100MHz in Table 3. Three non-transmitted channel (N=60MHz). Point; A; B; C; D; E; F. DeltaFrequency; [MHz]; -0.5N; -0.5N+1; -0.25N; 0.25N; 0.5N-1; 0.5N. Absolute Frequency; [MHz]; -30; -29; -15; 15; 29; 30. Delta Power; [dBr]; 0; -20; -25; -25; -20; 0. Proposal 5: to further discuss the related BS/UE RF requirement. |
| R4-2114202 | Qualcomm Incorporated | Proposal 1: Adopt the channel raster in Table 1 for NR-U 100 MHz CBW in band n46. Nominal channel freq (MHz); NREF (NR-ARFCN); FREF (MHz). 5200; 746668; 5200.02. 5220; 748000; 5220.00. 5240; 749332; 5239.98. 5260; 750668; 5260.02. 5280; 752000; 5280.00. 5300; 753332; 5299.98. 5520; 768000; 5520.00. 5540; 769332; 5539.98. 5560; 770668; 5560.02. 5580; 772000; 5580.00. 5600; 773332; 5599.98. 5620; 774668; 5620.02. 6540; 776000; 5640.00. 5660; 777332; 5659.98. 5680; 778668; 5680.02. 5785; 785668; 5785.02. 5805; 787000; 5805.00. 5825; 788332; 5824.98. 5845; 789668; 5845.02. 5865; 791000; 5865.00. Table 1: Channel raster for NRU 100 MHz in band n46 Proposal 2: Adopt the channel raster in Table 2 for NR-U 100 MHz CBW in band n96. Nominal channel freq (MHz); NREF (NR-ARFCN); FREF (MHz). 5995; 799668; 5995.02. 6015; 801000; 6015.00. 6035; 802332; 6034.98. 6055; 803668; 6055.02. 6155; 810332; 6154.98. 6175; 811668; 6175.02. 6195; 813000; 6195.00. 6215; 814332; 6214.98. 6315; 821000; 6315.00. 6335; 822332; 6334.98. 6355; 823668; 6355.02. 6375; 825000; 6375.00. 6475; 831668; 6475.02. 6495; 833000; 6495.00. 6515; 834332; 6514.98. 6535; 835668; 6535.02. 6635; 842332; 6634.98. 6655; 843668; 6655.02. 6675; 845000; 6675.00. 6695; 846332; 6694.98. 6795; 853000; 6795.00. 6815; 854332; 6814.98. 6835; 855668; 6835.02. 6855; 857000; 6855.00. 6955; 863668; 6955.02. 6975; 865000; 6975.00. 6995; 866332; 6994.98. 7015; 867668; 7015.02. 7035; 869000; 7035.00. 7055; 870332; 7054.98. 7075; 871668; 7075.02. Table 2: Channel raster for NRU 100 MHz in band n96 |

## Open issues summary

### Sub-topic 5-1: channelization

Sub-topic description: Channel raster for 100MHz channel BW. There are many views on channelization for 100MHz, some companies are making their proposals based on general rules. Before going in each value selection, it’s preferrable to align on generic rules as much as possible.

Issue 5-1-1: Channelization for 100MHz – general rules

- Proposals: .

  - Option 1 (Charter)

    - One 100MHz NR-U should only overlap one 80 MHz Wi-Fi channel for n46.

    - Consider only two 100 MHz NR-U per 160 MHz Wi-Fi channel in n96.

  - Option 2 (Skyworks, Qualcomm?)

    - One 100MHz NR-U should not overlap two 160 MHz Wi-Fi channels for n46 and n96.

    - Consider only four 100 MHz per 160MHz Wi-Fi channel for n46 and n96.

  - Other option

- Recommended WF

  - Further discussion would be needed to collect other companies’ view and converge to some agreements.

Issue 5-1-2: Missing channels for 60 and 80MHz channel BW

- Proposals: .

  - Option 1: Based on n46 NREF values, a valid 60MHz channel at 5845.02MHz (NREF=789668) and a 80MHz channel at 5854.98MHz (NREF=790332) corresponding to an existing Wi-Fi channel are missing. (Skyworks)

- Recommended WF

  - TBA

### Sub-topic 5-2: SEM

Sub-topic description: The SEM for 100MHz channel BW shall be further discussed and more specifically when puncturing channels inside a 100MHz NR-U signal.

Issue 5-2-1: General SEM for 100MHz channel BW

- Proposals: The SEM for 100MHz channel BW without any punctured carrier shall be the same SEM as for channel BW up to 80 MHz

  - Yes (CableLabs, Skyworks, Huawei, ZTE)

  - No

- Recommended WF

  - The general SEM for 100MHz channel BW shall be the same SEM as for up to channel BW up to 80 MHz, as specified in TS 38.101-1 clause 6.5F.2.2.

Issue 5-2-2: SEM for edge puncture, single puncture and double puncture

- Proposals: For edge puncture, interior single and double puncture, re-use the same SEM for up to 80 MHz

  - Yes (CableLabs, Skyworks, Huawei)

  - No

- Recommended WF

  - The SEM for edge puncture, interior single and double puncture shall be the same the SEM for up to 80 MHz, as specified in TS 38.101-1.

Issue 5-2-3: Triple punctured channel in the 100MHz channel

- Proposals: Avoid triple punctured case for 100 MHz channel BW

  - Yes (CableLabs)

  - No

- Recommended WF

  - TBA

Issue 5-2-4: SEM for triple puncture

- Proposals: For interior triple puncture (depending on agreement on issue XX) the SEM should be

  - Option1: -31dBr at 20MHz from the edges (CableLabs (preferred))

  - Option2: -28dBr at 10MHz from the edges (CableLabs (as a compromise))

  - Option3: -25dBr at 15MHz from the edges (Huawei, ZTE)

  - Option 4: -28dBr floor in-band as already agreed for other channel BW punctures to accommodate image issues (Skyworks)

  - Other

- Recommended WF

  - TBA

Issue 5-2-5: SEM table simplification

- Proposals: SEM requirement table 6.5F.2.2-1

  - Option 1: Simplify the table using formula (Skyworks):

  - Option 2: Keep existing table

- Recommended WF

  - TBA

### Sub-topic 5-3: Others

Sub-topic description: The following issues are related to other RF requirements when introducing 100MHz channel BW for NR-U.

Issue 5-3-1: ACS

- Proposals: The NR-U ACS value for 100MHz channel BW:

  - Option 1: 17.5 dB (MediaTek)

  - Other

- Recommended WF

  - TBA

Issue 5-3-2: MPR

- Proposals: Existing MPR values can be applied to 100 MHz channel BW

  - Yes (Huawei)

  - No

- Recommended WF

  - Existing MPR values should be applicable to 100MHz channel BW as well.

Issue 5-3-3: Intra-carrier guard band

- Proposals: Existing MPR values can be applied to 100 MHz channel BW

  - Option 1 (ZTE)

| CBW | SCS [KHz] | GB@80MHz | GB@20MHz | Intra-band carrier PRB configuration |
| --- | --- | --- | --- | --- |
| 100MHz | 30 | 845KHz | 805KHz | 6PRB(51~56) 5PRB(107~111) 6PRB(162~167) 6PRB(218~223) |
|  | 60 | 1370KHz | 970KHz | 4PRB(25~28) 3PRB(53~55) 4PRB(80~83) 4PRB(108~111) |
| Note: PRB index is starting from 1. |  |  |  |  |

  - other

- Recommended WF

  - TBA.

## Companies views’ collection for 1st round

### Open issues

Sub-topic 5-1: Channelization

Issue 5-1-1: Channelization for 100MHz – general rules

| Company | Comments |
| --- | --- |
| Skyworks | Confirm that Skyworks and Qualcomm channelization rules for 100MHz are the same. Even in WiFi channelization rule is related to upward channel bandwidths so it is not acceptable that 100MHz channels are constrained by 80MHz channels. The rule should be 100MHz channels shall not overlap two 160MHz channels. |
| Charter Communications Inc | Option 1 For n46 ( 5GHz), there is a significant incumbency for Wi-Fi in this band and having 100 MHz channel rasters in NR-U overlapping two 80 MHz channel bonding configurations in Wi-Fi is devastating and will not honored the agreement in revised WID. There has been studies in the US bu University of Chicago that shows that co-existence issues between technologies in this band causes as much as 90% of throughput degradation For n96 (6 GHz), there isn’t an incumbency issue is this band as it is newly created. As a compromise agreement having 2-100 MHz nr-U rasters falling inside 160 MHz Wi-I channel bonding is a good compromise. Limiting this to 2-100 MHz channel rasters rather than 4-100 MHz channel rasters provides some fairness as Wi-Fi channel can dynamically arrange themselves to utilize the unused spectrum. |
| Qualcomm | In RAN92#-e it was agreed that when introducing NR-U 100 MHz “the same importance shall be given to ensuring co-existence with other technologies (e.g. WiFi) as for other channel bandwidths.” In our view, the channelization rules proposed by Skyworks and Qualcomm follow the spirit of this agreement. Charter argues that one 100 MHz NR-U channel should not be allowed to overlap with two 80 MHz Wifi channels. By that logic, a 60 MHz NR-U channel should not have been allowed to overlap with two 40 MHz Wifi channels. However, the NR-U 60 MHz channel raster does allow such overlap and, following the same principle, NR-U 100 MHz should be allowed to overlap two 80 MHz Wifi channels. We agree that the channel rasters should ensure fair co-existence between both technologies. However, relegating NR-U 100 MHz to band-edge channel allocations with stricter emission limits would not be fair. |
| CableLabs | We support option 1. Wi-Fi networks have been deployed years before NR-U was defined. There are only seven 80 MHz Wi-Fi channels in band n46. It is detrimental to Wi-Fi networks and users if a 100 MHz NR-U channel blocks two 80 MHz Wi-Fi channels. And it is not a fair coexistence scenario in the unlicensed band. Based on the ECC Report 302 and FCC R&O DOC-363490A1, 50% of the Wi-Fi channels use 80 MHz bandwidth, and only 10% of the Wi-Fi channels use 40 MHz bandwidth. We understand Qualcomm’s point that 60 MHz NR-U channel rasters should not have been allowed to overlap 40 MHz Wi-Fi channels. However, 80 MHz Wi-Fi bandwidth is much more important than 40 MHz bandwidth. In addition, 60 MHz NR-U channel blocks two 40 MHz Wi-Fi channels makes the spectrum utilization near 75%. 100 MHz NR-U channel blocks two 80 MHz Wi-Fi channels decreases the spectrum utilization to 62.5%. Again, 50% of Wi-Fi channels use 80 MHz bandwidth. 100 MHz NR-U channel rasters block two 80 MHz Wi-Fi channels reduce the spectrum efficiency in the unlicensed band. |
| Nokia | We support option 2 with similar arguments as provided by Qualcomm |
| Huawei | We are ok with option 2. Similar approach can apply as 60 MHz. |
| Intel | First of all, we would like to note that 3GPP has defined two modes of operation in terms of co-existence with WiFi - one where NR-U/LAA coexist with WI-Fi, and one where the network ensures that Wi-Fi or other incumbent is absent, and this is indicated to a UE via a parameter called absenceOfAnyOtherTechnology-r16. Also, NR-U supports FBE mode with semi-static channel occupancy and in accordance to RAN1 specifications it is intended for environments where the absence of other technologies is guaranteed (e.g., by level of regulations, private premises policies). Therefore, we think that two types of deployment scenarios are possible and channel raster design should take this into account: - Scenario #1: Environments where the absence of other technologies is guaranteed - In our understanding in such scenarios no technical issues are expected for NR-U to use 100 MHz CBW with a flexible raster. So, Option 2 can be adopted or alternatively a more flexible channel raster (e.g., 20 MHz grid) can be considered. - Scenario #2: 3GPP/WiFi coexistence scenarios - For such scenario a certain level of fair co-existence with WiFi should be considered. Option 1 will almost preclude using NR-U 100 MHz CBW and also push 100MHz allocations to the band edge with stricter requirements. From our point of view Option 2, when one 100MHz NR-U does not overlap two 160 MHz Wi-Fi channels will provide a fair coexistence. Furthermore, we think that two 100 MHz per 160MHz Wi-Fi channel can be considered as a good compromise solution for this scenario. In summary our view is 1) RAN4 can define different channel rasters for Scenario #1 and Scenario #2 (i.e. for environments where the absence of other technologies is guaranteed and for environments with presence of other technologies) 2) RAN4 can define channel raster locations to allow for greater flexibility for NR-U(e.g. based on option 2) and add specification text that would state that certain raster locations are only for use in environments, where the absence of other technologies is guaranteed (e.g., by level of regulations, private premises policies). It would be the responsibility of the network owner to ensure that this requirement is respected. 3) For environments “where the absence of other technologies is guaranteed” use Option 2 or more flexible channel raster (e.g., 20 MHz grid) 4) For environments “with presence of other technologies” use modified Option 2 with two 100 MHz per 160MHz Wi-Fi channel |
| Charter Communications Inc | To Intel’s comments, Scenario #1 cannot be guarantee, especially in 5 GHz due to the high level of incumbency. There are examples and studies that have been done where this condition has not been guarantee. Having a 100 MHz NR-U raster blocking two 80 Mhz Wi-Fi channels does not ensure co-existence in 5 GHz |
| ZTE | Option 2, we tend to agree with skyworks that 100MHz carrier placement should not be restricted by 80MHz. |
| HPE | For n46 (5 GHz), we support Option 1 and have grave reservations about coexistence with Wi-Fi networks and the efficient use of the band if a 100 MHz NR-U channel were configured to overlap with two 80 MHz Wi-Fi channels. Based on our laboratory and field experience with LAA, as well the extensive body of simulations and measured results published by others, the other proposed options would result neither in fair coexistence nor in efficient use of spectrum. For n96 (6 GHz), we have the same concerns about fair coexistence with Wi-Fi and efficient use of spectrum. However, we also have more fundamental concerns regarding incumbent protection in the 6 GHz band. As far as we are aware, the impact of 100 MHz channel configurations on protected incumbents has never been presented or evaluated in either the EU or US rulemaking process. The deployment of multiple technologies operating with unaligned channel rasters some with bandwidths that have not been studied or even presented for review, has the potential to substantially degrade incumbent operation and should not be considered. |
| Apple | Generally speaking, one of the reasons for having channelization raster restrictions to ensure better co-existence with other systems, e.g. WIFI. From that point of view, Option 1 provides better co-exsitence as there will be fewer cases when one 100MHz chanel overlaps with several bonded channels. For better co-existence with WIFI, NR-U already can use 80MHz or 80+80MHz carrier aggregation so the use case for 100MHz is anyway somewhat limited. |

Issue 5-1-2: Missing channels for 60 and 80MHz

| Company | Comments |
| --- | --- |
| Skyworks | After verifying NREFs for 100MHz we can confirm that the 60MHZ and 80MHZ channels are missing and the missing 80mMHz channel correspond to a valid 80MHZ WiFi channel. If agreeable to all, the specification should be fixed even if this is not directly linked to 100MHz. |
| Charter Communications Inc | It looks like option 1 is ok |
| Qualcomm | It appears that incorrect (?) NR-U raster points were added in TS 38.101-1 instead of the missing channels. The channels in question are NREF=787000 (5805 MHz) for CBW=60 MHz and NREF = 786332 (5795 MHz) for CBW=80 MHz. It seems these channels should be replaced by the ones listed in option 1. |
| CableLabs | This is probably due to the U-NII-4 band released by FCC recently. We agree on the 60MHz channel raster at 5845.02MHz (NREF=789668) and the 80MHz channel raster at 5854.98MHz (NREF=790332). |
| Nokia | We are fine with this correction. |

Sub-topic 5-2: SEM

Issue 5-2-1: General SEM for 100MHz channel BW

| Company | Comments |
| --- | --- |
| Skyworks | Agree with the way FW and suggests that simplified equation based table is used. Note that current table has errors. |
| Charter Communications Inc | - We agree with the proposal (The SEM for 100MHz channel BW without any punctured carrier shall be the same SEM as for channel BW up to 80 MHz) |
| CableLabs | We agree. |
| Nokia | Agree with WF |
| Huawei | agree |
| ZTE | We agree with that. |
| Apple | Yes, it is ok to follow same principles as the existing 80MHz SEM mask has. |

Issue 5-2-2: SEM for edge puncture, single puncture and double puncture

| Company | Comments |
| --- | --- |
| Skyworks | Agree with the way FW |
| Charter Communications Inc. | - We agree with the proposal (For edge puncture, interior single and double puncture, re-use the same SEM for up to 80 MHz) |
| CableLabs | We agree. |
| Nokia | Agree with WF |
| Huawei | agree |
| Apple | Yes, for edge puncture, interior single and double puncture, re-use the same SEM for up to 80 MHz. |

Issue 5-2-3: Triple punctured channel in the 100MHz channel

| Company | Comments |
| --- | --- |
| Skyworks | We do not see why triple punctures should be avoided. They are on the BS side only (as only possible for non-contiguous sub-bands) |
| Charter Communications Inc. | We agree with the proposal (Avoid triple punctured case for 100 MHz channel BW) |
| CableLabs | This is unnecessary of Issue 5-2-4 could achieve a consensus. Avoiding the triple punctured channel is an alternative solution for Issue 5-2-4. Similar solutions were adopted by Wi-Fi for punctured channel SEM topics. |
| Nokia | To our understanding there is nothing precluding triple punctures. This as long as we follow the principal of the harmonized standard as has been used for the development of the SEM in general for NR-U. For triple punctures the requirements are copied in below from EN 301 893: When there are two or more unused channels in between used channels (all belonging to the same group of adjacent channels configured for multi-channel operation) and these unused channels are adjacent to each other, an additional channel edge mask as in figure 3 shall be applied only at the lowest and highest channel edge of the group of unused adjacent channels. UM is the total bandwidth of used channels adjacent to the unused channel(s). |
| Huawei | We support to include triple punctures. |
| Intel | We like to understand the motivation preventing the triple punctures. A puncturing is based on each LBT assessment and is independent process from other LBT. |
| ZTE | Similar view as Skyworks, why tripe punctures cannot been enabled. |
| Apple | So, the use case is that the network performs 1-0-0-0-1 transmission i.e. three intermediate channels are not transmitted at all. We can put a restriction on the network side to disable triple-puncture, but that could be too restrictive for the network. |

Issue 5-2-4: SEM for triple puncture

| Company | Comments |
| --- | --- |
| Skyworks | As discussed in our paper a -28dBr floor but the slope should not reach -28dBr at 10MHz offset like suggested in CableLabs document but rather at 12.5MHz offset. |
| Charter Communications Inc. | We agree with - Option1: -31dBr at 20MHz from the edges (CableLabs (preferred)) - Option2: -28dBr at 10MHz from the edges (CableLabs (as a compromise)) |
| CableLabs | We provided a comparative simulation study in R4-2112031. The -25 dBr SEM corresponds to large SINR degradation. We prefer -31 dBr and could accept -28 dBr. We are checking the -28 dBr 12.5 MHz offset that Skyworks proposed. |
| Nokia | To our understanding the proposal from Skyworks follows the principal of the harmonized standard as described above. If this is the case, we see no reason for changing our approach so far for NR-U of reusing the EN 301 893 SEM |
| Huawei | We do not see the reason to not to align with ETSI BRAN mask |
| Apple | Having -28dBr as a floor in-band value is a good compromise because this is what we already have for the edge puncture case. The “slope” can be discussed further. |

Issue 5-2-5: SEM table simplification

| Company | Comments |
| --- | --- |
| Skyworks | Existing table cannot be kept as 100MHz nees to be added and there are errors to be fixed. Given this is would be better to adopt a BW related equation based approach like happening for NR. |
| Charter Communications Inc. | Proposed table cannot be agreed until a resolution or consensus agreement is obtained in issue 5-2-4 given that in the equation for 100 Mhz channel bandwidth the limit results yields -28 dBr. |
| CableLabs | Thanks Skyworks for re-defining the formulas. Let’s solve issues 5-2-3 and 5-2-4 first. The simplification may not be necessary if issue 5-2-3 is agreed. |
| Nokia | We are okay with simplifying the SEM table as proposed by Skyworks. We can keep [100MHz] as tentative if that resolves the concerns from Charter and CableLabs |

Sub-topic 5-3: Others

Issue 5-3-1: ACS

| Company | Comments |
| --- | --- |
| MediaTek | Option 1: 17.5 dB with square brackets from our Tdoc’s Table. I missed to put wording in our proposal. The intention is we think [17.5] dB ACS for 100MHz CBW. If there is any RAN4 colleagues’ contribution in next meeting for further discussion. It is also appreciated. |
| Apple | The NR-U ACS value for 100 MHz should be defined as 17 dB not 17.5 dB, considering the log-scale from the definition of NR-U ACS 24 dB at 20 MHz. The scaling follows the previous agreement for the definition of NR-U ACS for the other CBWs. |

Issue 5-3-2: MPR

| Company | Comments |
| --- | --- |
| Skyworks | In our early NRU evaluation we also looked into 100MHz and can confirm that existing MPR is applicable to 100MHz. however, depending on SEM mask adopted, MPR for 100MHz wideband operation may need revisiting. In our contribution we also discussed whether A-MPR should focus on n96 NS53/54 only as a starting point or tackle any upcoming regulation in 6GHz band. |
| Charter Communications Inc. | We are ok revisiting the MPR for 100 MHz wideband operation. |
| Huawei | In our evaluation the existing MPR can be reused. |
| Apple | From the UE perspective, we can adopt exiting A-MPR values from the 80MHz channel, but practically speaking that will allow a UE to back-off more than it should. Thus, ideally new A-MPR values for the 100MHz channel should be derived. |

Issue 5-3-3: Intra-carrier guard band

| Company | Comments |
| --- | --- |
| XXX |  |
| ZTE | This issue should be resolved similar as 40MHz, 60MHz, 80MHz; |
| Apple | ZTE proposal is to have the following pattern, 50-6-50-6-50-5-50-6-50, which results in asymmetrical numbers. So for the 30kHz SCS, we may consider a smaller center bin which will result in same intra-carrier guard bands between all the bins: 50-6-50-6-49-6-50-6-50 For the 60kHz SCS, the guard bands may look as presented below. We already have 5RB intra-carrier guard band for the 80MHz channel, so it is not clear why should have 3-4RBs for the 100MHz channel. 23-5-23-5-23-5-23-5-23 |

CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |

## Summary for 1st round

### Open issues

|  | Status summary |
| --- | --- |
| Issue 5-1-1 | Channelization for 100MHz – general rules Tentative agreements:4 companies prefer option 1 and 6 companies prefer option 2. Candidate options: Recommendations for 2nd round: Capture any possible agreements for the next steps, with the 2 options in the WF. |
| Issue 5-1-2 | Missing channels for 60 and 80MHz channel BW Tentative agreements: General agreement that the proposed raster values for 60 and 80 MHz are missing. But they might replace NREF=787000 (5805 MHz) for CBW=60 MHz and NREF = 786332 (5795 MHz) for CBW=80 MHz Candidate options: Add values: for 60MHz channel at 5845.02MHz (NREF=789668) and for 80MHz channel at 5854.98MHz (NREF=790332) Recommendations for 2nd round: Confirm if those values should replace existing values: NREF=787000 (5805 MHz) for CBW=60 MHz and NREF = 786332 (5795 MHz) for CBW=80 MHz Final agreement to be captured in the WF. |
| Issue 5-2-1 | General SEM for 100MHz channel BW Tentative agreements: General agreement that the SEM for 100MHz channel BW without any punctured carrier shall be the same SEM as for channel BW up to 80 MHz Candidate options: Recommendations for 2nd round: To be captured in the WF. |
| Issue 5-2-2 | SEM for edge puncture, single puncture and double puncture Tentative agreements: General agreement that for edge puncture, interior single and double puncture, re-use the same SEM for up to 80 MHz Candidate options: Recommendations for 2nd round: To be captured in the WF |
| Issue 5-2-3 | Triple punctured channel in the 100MHz channel Tentative agreements:6 companies consider triple puncture shall be supported, one company is against and one company commented triple puncture could be supported if an agreement could found on 5-2-4. Candidate options: Assume triple puncture should be supported (an agreement might be found on 5-2-4). Recommendations for 2nd round: to be captured in the WF. |
| Issue 5-2-4 | SEM for triple puncture Tentative agreements: Option 4 is preferred by 4 companies arguing this is compliant with the ETSI harmonized standard. 2 companies have different opinion and prefer options 1 or 2. Candidate options: Recommendations for 2nd round: To be further discussed, the different options and next steps to be captured in the WF. |
| Issue 5-2-5 | SEM table simplification Tentative agreements: The simplification seems acceptable, pending on solving issues 5-2-3/5-2-4. Candidate options: Recommendations for 2nd round: The simplification might be captured in the WF, removing 100MHz for the time being (waiting for the resolution of 5-2-3/5-2-4). |
| Issue 5-3-1 | ACS Tentative agreements: No consensus so far Candidate options: 17 dB and [17.5 dB] have been proposed Recommendations for 2nd round: Capture the 2 options in the WF if agreement in the 2nd round. |
| Issue 5-3-2 | MPR Tentative agreements: 2 companies consider same MPR is applicable for 100MHz, one company is open for considering another value and one company thinks MPR should be ideally reconsidered. There is no strong view suggesting MPR shall be reviewed then. Candidate options: Existing MPR values can be applied to 100 MHz channel BW Recommendations for 2nd round: Capture the tentative agreement in the WF. |
| Issue 5-3-3 | Intra-carrier guard band Tentative agreements: Candidate options: Recommendations for 2nd round: To be further discussed. Capture any agreement or next steps in the WF. |

### CRs/TPs

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | NA |

## Discussion on 2nd round (if applicable)

Further discussion could be captured in the following tables, but it would be more efficient to focus on the WF.

Sub-topic 5-1: Channelization

Issue 5-1-1: Channelization for 100MHz – general rules

| Company | Comments |
| --- | --- |
| Charter Communications Inc. | In our option 1, we have already made compromises in n96. We cannot make compromises in n46 as the impact for incumbents is extremely impactful. Another option for considerations is CA combination with a 40 MHz channel bandwidth and a 60 MHz channel bandwidth in n46 in addition to 100 Mhz bandwidth should only overlap one 80 MHz Wi-Fi channel |
| Broadcom | We support Charter’s position above |
| CableLabs | We support option 1. We agree with what Apple mentioned in the draft R4-2114915 WF document. 100 MHz bandwidth and many other bandwidth options could be achieved by CA/DC. Option 2 does not bring any noticeable benefits but could cause harmful coexistence issues. |
| HPE | We concur with Charter, Broadcom, and CableLabs in supporting Option 1. Option 1 already represents a compromise providing efficient use of the band without unfairly disadvantaging any technology. It would allow a single 100 MHz channel overlapping with only one 80 MHz channel and also allow the aggregation of multiple narrower-bandwidth channels. The other option would degrade Wi-Fi performance, result in less efficient use of spectrum resources, and bring no clear benefit. As noted in our previous comments, we have considerably less latitude in n96, where the risk of negative impact on incumbents posed by 100 MHz channels has not been assessed. |
| Apple | Our general understanding is that the whole point in defining and limiting existing channel raster points for the shared access bands is to have better co-existence. So, it is not clear why we first introduced the concept of “ensuring better co-existence”, and now start exploring options that go in the opposite direction. As commented earlier, NR-U can use DL CA and aggregate several 20-80MHz carriers that will be perfectly aligned with WIFI; not mentioning the fact that ongoing NR-U UL CA work also considers only 20-80MHz channels. From that perspective, Option 1 is a better choice because Option 2 does not bring any noticeable benefits. |
| Skyworks | As commented in the WF if we are OK to discuss restriction on the number of 100MHz channels in relation to coexistence aspects we do not agree to a rule that would be linked to a lower channel BW as it does not apply to WiFi channel bonding rules where not only 160MHz overlaps 2 80MHz channels but 11be 320MHz channels can overlap each other over 160MHz so multiple 320MHz channels overlap 160 or 80MHz channels. 100+60MHz combination and 80+80 MHz combination should be allowed where an 160MHz WiFi channel is allowed. For n46 100MHz channels can be discussed on coexistence aspects but not on channel bonding rule or spectrum efficiency. if it happens that we are only left with channels with large AMPR it will be better to not have 100MHZ channels in n46 and save the RAN4 work. |

Issue 5-1-2: Missing channels for 60 and 80MHz

| Company | Comments |
| --- | --- |
| Charter Communications Inc | We agree with adding the values for 60MHz channel at 5845.02MHz (NREF=789668) and for 80MHz channel at 5854.98MHz (NREF=790332). It can replaced existing values |
| Broadcom | We are fine with the proposed correction. |
| CableLabs | We agree to add the missing channel rasters. |
| Slyworks | As clarified in the WF these channel shall not replace but be added to existing list as the other channels are legal (but subject to AMPR for OOB) |

Sub-topic 5-2: SEM

Issue 5-2-4: SEM for triple puncture

| Company | Comments |
| --- | --- |
| Charter Communications Inc | We still agree with option 1 or option 2 as a compromise. Question: How does option 4 differ from option 2? |
| Broadcom | We prefer Option 1. We can consider Option 2 as a compromise. |
| CableLabs | After offline discussions with Skyworks, we think options 2 and 4 are the same: -28 dBr at 10 MHz from the transmission channel edge. We support option 2 or 4. |
| Apple | What is the actual difference between Option 2 and Option 4? |
| skyworks | After clarification on the differences between the 3 options in CableLabs paper we agree Summary option 2 ( same as our proposal) as it is already supported by the current spec. |

Sub-topic 5-3: Others

Issue 5-3-1: ACS

| Company | Comments |
| --- | --- |
| Charter Communications Inc | We agree with 17dB |
| Broadcom | The value should be 17dB |
| Apple | We support the option 17 dB. |

Issue 5-3-3: Intra-carrier guard band

| Company | Comments |
| --- | --- |
| Apple | For some reason options proposed by Apple were not captured. The following pattern for 30kHz provides better symmetry: 50-6-50-6-49-6-50-6-50 As for the 60kHz, the following pattern is fully aligned with the 80MHz channel where 5RB gap is also used. 23-5-23-5-23-5-23-5-23 |

## Summary on 2nd round (if applicable)

|  | Status summary |
| --- | --- |
| Issue 5-1-1 | No agreement, the different options have been captured in the WF. |
| Issue 5-1-2 | Agreement captured in the WF. |
| Issue 5-2-4 | No agreement, the different options have been captured in the WF. |
| Issue 5-3-1 | Agreement captured in the WF. |
| Issue 5-3-3 | No agreement, the different options have been captured in the WF. |

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| R4-2114915 | To be approved |

# Recommendations for Tdocs

## 1st round

New tdocs

| Title | Source | Comments |
| --- | --- | --- |
| WF on adding 25 and 30 MHz channel BW in NR band n71 | T-Mobile USA |  |
| WF on adding 100 MHz channel BW in NR-U bands n46 and n96. | Qualcomm |  |

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| General |  |  |  |  |
| R4-2113737 | Revised Basket WID on adding channel bandwidth support to existing NR bands | Ericsson |  | Wait for 2nd round |
| R4-2113738 | Big CR to TS 38.104: Adding channel BW support in existing NR bands | Ericsson |  | For e-mail approval |
| R4-2113739 | Big CR to TS 38.101-1: Adding channel BW support in existing NR bands | Ericsson |  | For e-mail approval |
| Band n2 – 25, 30 and 40 MHz |  |  |  |  |
| R4-2111745 | Draft CR to 38.101-1 Introduction of Additional CBWs for Band n2 | AT&T | To be endorsed |  |
| R4-2111747 | Draft CR to 38.104 Introduction of Additional CBWs for Band n2 | AT&T | To be endorsed |  |
| Band n5 – 25 MHz |  |  |  |  |
| R4-2111746 | Draft CR to 38.101-1 Introduction of 25MHz CBW for Band n5 | AT&T | To be revised |  |
| R4-2111748 | Draft CR to 38.104 Introduction of 25MHz CBW for Band n5 | AT&T |  | Could be endorsed but wait for revision of R4-2111746 |
| R4-2114584 | Adding 25MHz CBW to Band n5 | Skyworks Solutions Inc. | To be noted |  |
| Band n71 – 25 and 30 MHz |  |  |  |  |
| R4-2112737 | REFSENS of n71 for 25MHz and 30MHz channel bandwidth | Murata | To be noted |  |
| NR-U bands n46 and n96 – 100 MHz |  |  |  |  |
| R4-1111835 | Co-existence challenges with NR-U 100MHz channel bandwidth and other technologies in 5 GHz (n46) and 6 GHz (n96) | Charter Communications, Inc | To be noted |  |
| R4-2112031 | NR-U Punctured Channel SEM for 100 MHz Bandwidth | CableLabs | To be noted |  |
| R4-2112301 | NRU 100MHz SEM mask including wideband operation | Skyworks Solutions Inc. | To be noted |  |
| R4-2112302 | NRU 100MHz channelization | Skyworks Solutions Inc. | To be noted |  |
| R4-2113067 | 100 MHz channel bandwidth for NR-U | Huawei, HiSilicon | To be noted |  |
| R4-2113664 | Discussion of ACS requirement for NR-U CBW of 100MHz | Mediatek India Technology Pvt. | To be noted |  |
| R4-2113937 | Further discussion on the introduction of 100MHz for NR-U | ZTE Corporation | To be noted |  |
| R4-2114202 | Introducing NR-U 100 MHz carrier bandwidth in bands n46 and n96 | Qualcomm Incorporated | To be noted |  |

## 2nd round

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| General |  |  |  |  |
| R4-2113737 | Revised Basket WID on adding channel bandwidth support to existing NR bands | Ericsson | To be endorsed |  |
| R4-2113738 | Big CR to TS 38.104: Adding channel BW support in existing NR bands | Ericsson | For email approval |  |
| R4-2113739 | Big CR to TS 38.101-1: Adding channel BW support in existing NR bands | Ericsson | For email approval |  |
| Band n5 – 25 MHz |  |  |  |  |
| R4-2114916 | Draft CR to 38.101-1 Introduction of 25MHz CBW for Band n5 | AT&T | To be endorsed | Revision of R4-2111746 |
| R4-2111748 | Draft CR to 38.104 Introduction of 25MHz CBW for Band n5 | AT&T | To be endorsed |  |
| Band n71 – 25 and 30 MHz |  |  |  |  |
| R4-2114914 | WF on adding 25 and 30 MHz channel BW in NR band n71 | T-Mobile USA | To be approved |  |
| NR-U bands n46 and n96 – 100 MHz |  |  |  |  |
| R4-2114915 | WF on adding 100 MHz channel BW in NR-U bands n46 and n96. | Qualcomm | To be approved |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents

# Annex

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| Skyworks Solutions Inc. (NRU) | Dominique Brunel | Dominique.brunel@skyworksinc.com |
| Skyworks Solutions Inc. (new CBW) | Laurent Noel | Laurent.noel@skyworksinc.com |
| AT&T | Ron Borsato | ronald.borsato@att.com |
| Nokia | Hisashi Onozawa | hisashi.onozawa@nokia.com |
| Huawei | Liehai Liu | liuliehai@huawei,com |
| Murata | Shinya Hitomi | hitomishinya@murata.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)