3GPP TSG-RAN WG4 Meeting # 100-e 										  	      R4-2115028

Electronic Meeting, 16 - 27 Aug, 2021

Agenda item:			9.3.2.2, 9.3.2.3, 9.3.2.7.1

Source:	Moderator (China Telecom)

Title:	Email discussion summary for [100-e][128] NR_RF_FR1_enh_Part_2

Document for:	Information

Introduction

This email thread covers the Rel-17 Tx switching maintenance in AI 9.3.2.2 & 9.3.2.3, and the clarification of Tx switching scenarios in AI 9.3.2.7.1.

List of candidate target of email discussion for 1st round and 2nd round:

- 1st round:

  - Review and comment the recommended WF in section 1.2 and 2.2.

- 2nd round:

  - 1 sub-thread on the WF, with email title ‘[100-e][128] NR_RF_FR1_enh_Part_2 - WF’ (led by CTC)

    - WF on UL-MIMO coherence for Rel-17 Tx switching

  - 1 sub-thread on the LS, with email title ‘[100-e][128] NR_RF_FR1_enh_Part_2 - LS’ (led by E///)

    - Draft LS on TX switching with multiple TAG

Note: For quick turnaround in responding to comments, it is recommended to send company comments in email body of each sub-thread instead of adding them in the summary document. Moderator will add all the email comments into the summary document.

Topic #1: Rel-17 Tx switching maintenance

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2112228 | China Telecom | This contribution discusses the UE capability on UL MIMO coherence for the three Rel-17 Tx switching scenarios, with the following observations and proposals: Observation 1: For 3CC 1Tx-2Tx switching, the difference with Rel-16 1Tx-2Tx switching is that the CC number is increased from 1 to 2 for the band with 2Tx, and the UE ability on maintaining UL-MIMO coherence for the 2 CCs in the same band can be the same. Proposal 1: For 3CC 1Tx-2Tx switching, if UE capability on UL-MIMO coherence is needed, the Rel-16 per BC UE capability can be applied, i.e., the same capability applies to both Rel-16 and Rel-17 1Tx-2Tx switching. Observation 2: For 2CC 2Tx-2Tx switching, it is not quite sure: a) whether the UE behavior for switching is the same or not compared to Rel-16 1Tx-2Tx switching, and b) whether the UE ability on maintaining UL-MIMO coherence for the 2 CCs in different bands is the same or not. Proposal 2: For 2CC 2Tx-2Tx switching, if UE capability on UL-MIMO coherence is needed, further confirm whether the Rel-16 per BC UE capability can be applied for both CCs in different bands. Observation 3: For 3CC 2Tx-2Tx switching, the difference with 2CC 2Tx-2Tx switching is that the CC number is increased from 1 to 2 in one of the bands, and the UE ability on maintaining UL-MIMO coherence for the 2 CCs in the same band can be the same. Proposal 3: For 3CC 2Tx-2Tx switching, if UE capability on UL-MIMO coherence is needed, the capability for 2CC 2Tx-2Tx switching can be applied, i.e., the same capability applies to both 2CC and 3CC 2Tx-2Tx switching. Based on the discussion in RAN4, if any agreements w.r.t. UE capability and UL-MIMO coherence exception will be reached, LS to RAN2 and RAN1 is needed. |
| R4-2112824 | Ericsson | Draft LS on modification of Pcmax for UL CA with uplink Tx switching capability |

Open issues summary

Sub-topic 1-1: UL MIMO coherence for Rel-17 Tx switching

Issue 1-1: UL MIMO coherence for Rel-17 Tx switching

- Background

  - For Rel-16 1Tx-2Tx switching between two carriers, it was identified that the coherence between two antenna ports can or cannot be maintained when the Tx chain(s) are used to transmit on an other band for different types of UE architectures.

  - As a result, it was agreed to introduce UE capability to differentiate the two types of UEs in RAN4 #99e, with the RAN4 CR agreed in R4-2109582 and the LS to RAN1/2 approved in R4-2107765.

- Summary of Tx switching scenarios

| Tx switching |  | Band A | Band B |  |
| --- | --- | --- | --- | --- |
|  |  | CC1 | CC2 | CC3 |
| Rel-16 | 2CC 1Tx-2Tx | 1Tx | 2Tx |  |
| Rel-17 | 3CC 1Tx-2Tx | 1Tx | 2Tx | 2Tx |
|  | 2CC 2Tx-2Tx | 2Tx | 2Tx |  |
|  | 3CC 2Tx-2Tx | 2Tx | 2Tx | 2Tx |

Issue 1-1A: For 3CC 1Tx-2Tx switching

- Proposals:

  - Option 1: For 3CC 1Tx-2Tx switching, if UE capability on UL-MIMO coherence is needed, the Rel-16 per BC UE capability can be applied, i.e., the same capability applies to both Rel-16 and Rel-17 1Tx-2Tx switching. (CTC)

    - CTC: For 3CC 1Tx-2Tx switching, the difference with Rel-16 1Tx-2Tx switching is that the CC number is increased from 1 to 2 for the band with 2Tx, and the UE ability on maintaining UL-MIMO coherence for the 2 CCs in the same band can be the same.

Issue 1-1B: For 2CC 2Tx-2Tx switching

- Proposals:

  - Option 1: For 2CC 2Tx-2Tx switching, if UE capability on UL-MIMO coherence is needed, further confirm whether the Rel-16 per BC UE capability can be applied for both CCs in different bands. (CTC)

    - CTC: For 2CC 2Tx-2Tx switching, it is not quite sure: a) whether the UE behavior for switching is the same or not compared to Rel-16 1Tx-2Tx switching, and b) whether the UE ability on maintaining UL-MIMO coherence for the 2 CCs in different bands is the same or not.

Issue 1-1C: For 3CC 2Tx-2Tx switching

- Proposals:

  - Option 1: For 3CC 2Tx-2Tx switching, if UE capability on UL-MIMO coherence is needed, the capability for 2CC 2Tx-2Tx switching can be applied, i.e., the same capability applies to both 2CC and 3CC 2Tx-2Tx switching. (CTC)

    - CTC: For 3CC 2Tx-2Tx switching, the difference with 2CC 2Tx-2Tx switching is that the CC number is increased from 1 to 2 in one of the bands, and the UE ability on maintaining UL-MIMO coherence for the 2 CCs in the same band can be the same.

- Recommended WF

  - Encourage feedback on UL MIMO coherence for different Rel-17 Tx switching scenarios.

| Company | Comments |
| --- | --- |
| OPPO | Issue 1-1A: For 3CC 1Tx-2Tx switching Option 1. In 3CC 1T-2T switching, UE in the high band will use one PA to cover both CCs in order to support UL MIMO, thus, coherence is same with current 2Tx chain assumption. Issue 1-1B: For 2CC 2Tx-2Tx switching The coherence in the two bands could be different, and Rel-16 capability cannot be reused since it is per-band combination capability rather than per band per band combination (Rel-17 2 band case) capability. Issue 1-1C: For 3CC 2Tx-2Tx switching Option 1 with issue 1-1B assumption. |
| China Telecom | Issue 1-1A: For 3CC 1Tx-2Tx switching Option 1. Issue 1-1B: For 2CC 2Tx-2Tx switching Depends on inputs from chipset/UE side. Issue 1-1C: For 3CC 2Tx-2Tx switching Option 1. |
| ZTE | Issue 1-1A: For 3CC 1Tx-2Tx switching Option 1. For 1Tx-2Tx switching (for both 2CC or 3CC), the coherence only applies to the band with 2Tx, so no matter 2CC or 3CC, it should be the same. Issue 1-1B: For 2CC 2Tx-2Tx switching For 2Tx-2Tx switching, both bands work in MIMO mode, so if there is a switching in the time window, then the coherence may not be maintained. Issue 1-1C: For 3CC 2Tx-2Tx switching Option 1. 2CC and 3CC may have the same capability in 2Tx-2Tx switching. |
| Qualcomm | Issue 1-1A: Agree with option 1, Issue 1-1B: Agree, this should be clarified. Our view is that it may depend on implementation, some frequency generation circuitry maybe impacted by switching and some may not depending on the frequencies and implementation choices. Issue 1-1C: Agree with option 1 |
| CMCC | Issue 1-1A: For 3CC 1Tx-2Tx switching Option 1 Issue 1-1B: For 2CC 2Tx-2Tx switching If the coherence cannot be maintained in this case, then the Rel-16 signaling cannot be reused. Issue 1-1C: For 3CC 2Tx-2Tx switching Option1 |
| vivo | Issue 1-1A: For 3CC 1Tx-2Tx switching Option 1 Issue 1-1B: For 2CC 2Tx-2Tx switching Still not clear in current stage, may depend on implementation. More conservative way might be define new capability, or postpone the decision to have further analysis. Issue 1-1C: For 3CC 2Tx-2Tx switching Option1 |
| Huawei | Issue 1-1A: For 3CC 1Tx-2Tx switching Option 1 Issue 1-1B: For 2CC 2Tx-2Tx switching It depends on UE implementation. Agree it should be further clarified. Issue 1-1C: For 3CC 2Tx-2Tx switching Option1 |

Companies views’ collection for 1st round

Open issues

Provided in section 1.2

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic 1-1: UL MIMO coherence for Rel-17 Tx switching | Issue 1-1: UL MIMO coherence for Rel-17 Tx switching Issue 1-1A: For 3CC 1Tx-2Tx switching Tentative agreements: - Option 1: For 3CC 1Tx-2Tx switching, if UE capability on UL-MIMO coherence is needed, the Rel-16 per BC UE capability can be applied, i.e., the same capability applies to both Rel-16 and Rel-17 1Tx-2Tx switching. (CTC, OPPO, ZTE, QC, CMCC, vivo, HW) Issue 1-1B: For 2CC 2Tx-2Tx switching Candidate options: - Option 1: For 2CC 2Tx-2Tx switching, if UE capability on UL-MIMO coherence is needed, further clarify whether the Rel-16 per BC UE capability can be applied for both CCs in different bands. (CTC, [ZTE], QC, vivo, HW) - Option 2: The Rel-16 signaling cannot be reused (OPPO, CMCC) Recommendations for 2nd round: Further discuss and check if additional agreements can be reached. If more time is needed for internal checking, can also postpone the decision to the next meeting. Issue 1-1C: For 3CC 2Tx-2Tx switching Tentative agreements: - Option 1: For 3CC 2Tx-2Tx switching, if UE capability on UL-MIMO coherence is needed, the capability for 2CC 2Tx-2Tx switching can be applied, i.e., the same capability applies to both 2CC and 3CC 2Tx-2Tx switching. (CTC, OPPO, ZTE, QC, CMCC, vivo, HW) |

Discussion on 2nd round

R4-2114957	WF on UL-MIMO coherence for Rel-17 Tx switching

Type: other		For: Approval
					Source: China Telecom

Abstract:

This contribution provides the WF.

Discussion:

| Company | Comment |
| --- | --- |
| OPPO | It seems in the 1st round most companies agree that whether the two CC can keep same coherence or not depends on UE implementation. That means for some UE this can be maintained, and for other UE the two CC can have different coherence capability. Therefore, the signaling shall cover both kinds of UE implementation. However, current Rel-16 capability is defined as per band combination capability according to LS R4-2107765. Therefore, Rel-16 signaling cannot be reused. To summary, maybe the outcome can be as below: - For 2CC 2Tx-2Tx switching, UE capability on UL-MIMO coherence at CC1 and CC2 can be different which is up to UE implementation; - UL-MIMO coherence signaling shall support different coherence capability at CC1 and CC2; - Whether Rel-16 signaling can be reused is FFS after RAN2 specify this Rel-16 capability in 38.331. |
| China Telecom | To my understanding, in round 1, most companies commented that it needs to be further checked or clarified. As we know, this is related to UE architecture and how Tx switching is conducted, which usually need some time for internal checking. That is how the “Recommendations for 2nd round” comes from. But for sure, if in this meeting, all companies agree to define different coherence capability, from moderator perspective, I will be more than happy to conclude this issue in this week. |
| China Telecom | Not sure about other companies’ view. To be safe, I kept the options for 2CC 2Tx-2Tx switching in draft WF on UL-MIMO coherence for R17 Tx switching_r1.docx, and please let me know if any suggestions. |

Recommendation:		Agreeable

R4-2112824	Draft LS on modification of Pcmax for UL CA with uplink Tx switching capability

Type: LS out		For: Approval
					to RAN
					Source: Ericsson

Abstract:

Draft LS to inform RAN on the implementation of the CR endorsed at RAN#91-e (implemented in the latest version by RAN4 for RAN#93-e)

Moderator’s note:

This tdoc was not included in round 1 discussion, since it was originally submitted to AI 9.3.1 and moderator did not notice when it was moved to AI 9.3.2.2.

Meanwhile, the corresponding CR in R4-2112814 (Modification of Pcmax for UL CA with uplink Tx switching capability) was concluded as Not pursued in the Main Session report, so moderator recommends to note the LS as well.

Recommendation:		Noted

Topic #2: Clarification of Tx switching scenarios

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2112825 | Ericsson | Draft LS to RAN1 and RAN2 to ask about possible impact on RAN1 and RAN2 specification of a removal of the single-TAG restriction for CA. |

Open issues summary

Sub-topic 2-1: TX switching with multiple TAG for UL CA

Issue 2-1: TX switching with multiple TAG for UL CA

- Proposals

  - LS to RAN1/2 (E///)

    - RAN4 asks RAN1/2 whether removal of the Rel-16 single-TAG restriction for UL CA with TX switching has any impact on RAN1/2 specifications with an UL timing difference between carriers up to a symbol duration (SCS = 30k) as seen at the UE.

- Recommended WF

  - Encourage feedback from companies

| Company | Comments |
| --- | --- |
| OPPO | It seems this LS is for Rel-16 Tx switching, which in our understanding should be stable and no changes is expected especially for extending the scope of a frozen WID. Maybe further discussion can happen in future Rel-18 discussions. For now, not clear of the necessity. |
| Nokia | We understand the motivation, but now Rel-17 is on-going and Rel-16 WI is completed. It is not the appropriate timing to introduce this into Rel-16. |
| ZTE | Single TAG is an important assumption for the work on Tx switching in Rel-16. If this assumption is lifted, we may need to revisit and check agreements made under this assumption in RAN4 before we send an LS to RAN1/2. |
| Qualcomm | Removing the single TAG would make the switching location ambiguous. Why this comes up now is unclear. Prefer not to make the agreement. |
| vivo | Propose not to consider multiple TAG for UL CA in current stage. |
| Ericsson | Combinations of UL-MIMO and UL CA are enabled for the collocated scenario with the single-TAG assumption with UE capable of switching across two TX chains. But it is not possible for a non-collocated case. CA band combinations with cells in bands below 2 GHz and cells in bands at e.g. 3.5 GHz are common and non-collocation is a common scenario due to the different cell sizes in these bands. Hence the multiple-TAG general case for combination of UL-MIMO and UL CA is therefore a common deployment case. The single-TAG is an unnecessary restriction of deployment scenarios for the feature unless there are hurdles in the RAN1 and RAN2 standards. To Qualcomm: the position of the switching period is not ambiguous for the multiple-TAG case, the switching period precedes time T0 (the start of the uplink transmission) just as for the single-TAG case according to the time masks proposed: The gNB scheduler ensures that there is no transmissions in the switching periods by not scheduling the last UL symbol before the switching gap (maximum one symbol at MRTD < 30 us). The timing of an UL transmission at T0 is determined by the corresponding DL carrier, hence there may be slight UL timing shifts also for the collocated case due to DL timing errors. The timing requirements on T0 – Toffset w r t the received DCI are not modified. Moreover, The MRTD up to 30 us does not modify the DL interruptions allowed. We are proposing to send an LS to RAN1 and RAN2 to ask about any implications. If confirmed by RAN1 and RAN2 that there are no changes, it is straightforward to accommodate the multiple-TAG case. The time masks have to be modified as shown in R4-2112825 for the two cases along the same principles as the single-TAG case. A capability might be needed to handle legacy implication with single-TAG. Since CA band combinations with cells in bands below 2 GHz and cells in bands at e.g. 3.5 GHz are common and non-collocation is a common scenario, we see no reason not to ask RAN1 and RAN2 about implications. If none we can combine UL CA and UL-MIMO for Ues supporting TX switching for the non-collocated case common in deployments. A most useful enhancement of the feature. |
| Huawei | It’s clear in the scope of the completed WI and the on-going WI that the scenario is for single-TAG. The proposal for scenarios out of the scope could be considered in future release if the scenario is justified. |

Companies views’ collection for 1st round

Open issues

Provided in section 2.2

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic 2-1: TX switching with multiple TAG for UL CA | Issue 2-1: TX switching with multiple TAG for UL CA Candidate options: - Option 1: Send LS to RAN1/2 (E///) - RAN4 asks RAN1/2 whether removal of the Rel-16 single-TAG restriction for UL CA with TX switching has any impact on RAN1/2 specifications with an UL timing difference between carriers up to a symbol duration (SCS = 30k) as seen at the UE. - Option 2: Not send the LS for Rel-16 (Nokia, ZTE) - ZTE: If single TAG assumption is lifted, we may need to revisit and check agreements made under this assumption in RAN4 before we send an LS to RAN1/2. - Option 3: Not send the LS for Rel-16 and Rel-17 (OPPO, QC, vivo, HW) - QC: Removing the single TAG would make the switching location ambiguous. Why this comes up now is unclear. Recommendations for 2nd round: Continue discussion. |

Discussion on 2nd round

R4-2114958	Draft LS on TX switching with multiple TAG

Type: LS out		For: Approval
					to RAN
					Source: Ericsson

Abstract:

Draft LS to inform RAN on the implementation of the CR endorsed at RAN#91-e (implemented in the latest version by RAN4 for RAN#93-e)

Discussion:

E///: This is to kick off the discussion on and LS and TX switching with multiple TAG to allow combinations of UL CA and UL-MIMO in 3.5 GHz also for the non-collocated scenario.

The current version of the LS in

https://www.3gpp.org/ftp/tsg_ran/WG4_Radio/TSGR4_100-e/Docs/R4-2112825.zip

While we have the floor, a comment on the options:

Ericsson: on Option 2, ”Not send the LS for Rel-16” and ’if the single TAG assumption is lifted, we may need to revisit and check agreements made under this assumption in RAN4 before we send an LS to RAN1/2’

The TX switching feature was first discussed for SUL band combinations for which is single-TAG always applies (since a single cell); this is not the case for the UL CA band combinations subsequently added. Therefore we should not unnecessarily impose a single-TAG restriction on TX switching for UL CA unless there are hurdles in the RAN1 and RAN2 standards, the RAN4 time masks can be modified to accommodate multiple-TAG and the DL interruption capability would not be affected by the maximum MRTD of CA.

Why not start by asking RAN1 and RAN2 and remove any unnecessary deployment restrictions if possible?

On option 3, ”Not send the LS for Rel-16 and Rel-17” and ’Removing the single TAG would make the switching location ambiguous.’

In our view, the position of the switching period is not ambiguous for the multiple-TAG case. The switching period precedes the time T0 (the start of the UL transmission following a triggered UL switch) just as for the single-TAG case according to the time masks proposed in the LS. The gNB scheduler ensures that there is no transmissions in the switching periods by not scheduling the last UL symbol before the switching gap (maximum one symbol at MRTD < 30 us). The timing of an UL transmission at T0 is determined by the corresponding DL carrier timing, hence there may be slight UL timing shift also for the collocated case due to DL timing errors.

Since CA band combinations with cells in bands below 2 GHz and cells in bands at e.g. 3.5 GHz are common and non-collocation therefore a common scenario due to different cell sizes, we see no reason not to start by asking RAN1 and RAN2 about implications of removing the single-TAG restriction. Does this common deployment scenario need any justification? If no implication we could combine UL CA and UL-MIMO for UEs supporting TX switching for the non-collocated case common in deployments. A most useful enhancement of the feature by a straightforward change in RAN4 specifications. Can be done in TEI.

  - Encourage feedback from companies

| Company | Comments |
| --- | --- |
| T-Mobile USA | We support sending the LS. We understand the concerns with the timing of this request, but it would be good to have RAN1 and RAN2 confirm if there would be implications or not. If there are implications for RAN1 and RAN2, then how this is treated in the future might be different than if there are no RAN1 and RAN2 impacts. |
| Nokia | Wouldn’t it be more appropriate to have a clear objective in a certain WI and ask RAN1/2 what we want to know? |

Latest status:

Revision of LS was not uploaded.

Recommend to withdraw the revision and Note the original one in R4-2112825.

Recommendation:		Withdrawn

- Recommendations for Tdocs

  - 1st round

New tdocs

| Title | Source | Comments |
| --- | --- | --- |
| WF on … | YYY |  |
| LS on … | ZZZ | To: RAN_X; Cc: RAN_Y |
| WF on UL-MIMO coherence for Rel-17 Tx switching | China Telecom |  |

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-210xxxx | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-2112228 | Discussion on UL MIMO coherence for Rel-17 Tx switching | China Telecom | Noted |  |
| R4-2112825 | Draft LS on TX switching with multiple TAG | Ericsson | Revised |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

  - 2nd round

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2114957 | WF on UL-MIMO coherence for Rel-17 Tx switching | China Telecom | agreeable |  |
| R4-2112825 | Draft LS on TX switching with multiple TAG | Ericsson | Noted | Changed from “Revised” to “Noted” |
| R4-2114958 | Draft LS on TX switching with multiple TAG | Ericsson | Withdrawn | Revision of LS was not uploaded. Recommend to withdraw the revision and Note the original one in R4-2112825. |
| R4-2112824 | Draft LS on modification of Pcmax for UL CA with uplink Tx switching capability | Ericsson | Noted | The corresponding CR in R4-2112814 was Noted. |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents

- Annex

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| OPPO | Jinqiang Xing | xingjinqiang@oppo.com |
| Nokia | Hiromasa Umeda | hiromasa.umeda@nokia.com |
| China Telecom | Shan YANG | yangshan@chinatelecom.cn |
| ZTE | Aijun Cao | Cao.aijun@zte.com.cn |
| CMCC | Xiaoran ZHANG | zhangxiaoran@chinamobile.com |
| Ericsson | Christian Bergljung | christian.bergljung@ericsson.com |
| Huawei | Ye Liu | leo.liuye@huawei.com |
| T-Mobile USA | Bill Shvodian | bill.shvodian@t-mobile.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)