3GPP TSG-RAN WG4 Meeting # 100-e 														R4-2115034

Electronic Meeting, 16th – 27th August, 2021

Agenda item:			6.1.3.1, 9.15.1, 9.15.2, 9.15.3, 9.15.4 & 9.15.7

Source:	Moderator (LG Electronics)

Title:	Email discussion summary for [100-e][134] NRSL_enh_Part1

Document for:	Information

Introduction

In this paper, RAN4 treats the remaining issues for 5G V2X UE in Rel-16. Also mainly deal with the SL_enh_part1 in Rel-17 for operating bands, system parameters, and UE transmitter/Receiver requirements for SL enhancement including 5G V2X enhancement and Public safety using PC5 operation.

The provided technical docs list of email discussion are shown in Reference in the end of the paper.

Candidate target of email discussion for 1st round is listed as follows.

- 1st round: Discuss A-MPR requirements to meet the FCC regulatory requirements in n47 for 5G V2X RF maintenance. Also, RAN4 treats the n14 coexistence evaluation to protect B13 and legacy n14 system for in-coverage NW scenarios.

  - Topic #1: Maintenance of 5G V2X UE

  - Sub-Topic #1-1: Update of A-MPR for FCC regulation with  NS_52

    - Sub-Topic #1-1-1: Discussion paper from Huawei (10400) & LGE (12843)

    - Sub-Topic #1-1-2: Draft CR from Huawei (13411)

  - Sub-Topic #1-2: Correction CR for 5G V2X UE

    - Sub-Topic #1-2-1: Draft CR from LGE (12888)

  - Topic #2: SL enhancements

  - Sub-Topic #2-1: Coexistence evaluation for PS operation in licensed band

    - Sub-Topic #2-1-1: Protection of B13/n13 UE by n14 PS operation with PC1/PC3

    - Sub-Topic #2-1-2: Protection of legacy n14 Uu system in in-coverage NW scenarios

  - Sub-Topic #2-2: System parameters

    - Sub-Topic #2-2-1: Channel raster & Sync. raster

  - Sub-Topic #2-3: SL enh. RF requirements

    - Sub-Topic #2-3-1: Pcmax definition for SL Enhancement UE

    - Sub-Topic #2-3-2: REFSENS requirements

- 2nd round: The following issues will be further discussed

  - Topic #1: Maintenance of 5G V2X UE

    - Sub topic #1-1: Update of A-MPR for FCC regulation with NS_52

  - Topic #2: SL enhancement

    - Sub-topic #2-1: Coexistence evaluation for PS operation in licensed band

    - Sub-Topic #2-2: Sync. raster of SL operation in licensed band

    - Sub-Topic #2-3: SL enh. RF requirements

| Tdoc number | Title | Source |
| --- | --- | --- |
| R4-2114977 | WF on FCC regulation requirements for 5G V2X service | Huawei |
| R4-2114978 | WF on n14 coexistence evaluation for NR PS UE | Ericsson |
| R4-2114979 | WF on Pemax definition and NR PS REFSENS in licensed band | LG Electronics |
| R4-2112767 | TR38.785 v0.3.0 TR Update for SL enhancement in Rel-17 | LG Electronics France |
| R4-2114981 | TP on updating REFSENS requirements for NR SL enhancement | LG Electronics France |
| R4-2114980 | TP on sync raster for SL licensed bands | CATT |
| R4-2114337 | CR on NR V2X Pcmax in TS38.101-1 in Rel-17 | Ericsson |
| R4-2115086 | LS on FCC regulation of the C-V2X emission limits on 47 CFR Parts 90 for V2X service in 5850-5925 MHz | LG Electronics |

- Topic #1: Maintenance of 5G V2X UE

Main technical topic overview. The structure can be done based on sub-agenda basis.

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2110400 | Huawei | Observation 1: The specified emissions limits in FCC regulation are not what RAN4 specified in clause 6.5E.2.3.2 from TS 38.101-1. Observation 2: Currently, there is no 40MHz ITS spectrum allocation based on FCC regulatory. Proposal 2: Companies are encouraged to further check the FCC regulation. It’s up to RAN4 how to address this mismatching issue. |
| R4-2112843 | LGE | In FCC, they allocated the 30MHz frequency range (5895~5925MHz) for vehicular system as shown in Figure 1 [1]. Figure 1. 5.9GHz ITS/Unlicensed band Current FCC regulation in 90.381 for C-V2X emission limits Proposal 1: RAN4 would update A-MPR requirements based on current FCC emission limits. And revise the A-SEM requirements by NS_52 in Section 6.5E.2.3.2 in TS38.101-1. Proposal 2: For A-MPR simulation campaign, RAN4 can complete the A-MPR issue until RAN4 #101-e meeting. |
| R4-2112888 | LGE | CR to modify some editorial correction as follow - Editorial correction of SL MIMO in maximum output power - For inter-band con-current V2X operation, SL can operated in licensed band not n47. So update the related RF requirements. - Update reference FRC Tables for REFSENS |
| R4-2113411 | Huawei | Based on the latest FCC regulation, the ITS operation using C-V2X technology is in frequency range 5895~5925 instead of 5865~5905. So remove ASEM requirements for NS_52. |

Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Based on provided contributions, RAN4 mainly treats A-MPR requirements based on FCC regulatory requirements by NS_52 in 1st round.

  - Sub-Topic #1-1: Update of A-MPR for FCC regulation with NS_52

    - Sub-Topic #1-1-1: Discussion paper from Huawei (10400) & LGE (12843)

    - Sub-Topic #1-1-2: Draft CR from Huawei (13411)

  - Sub-Topic #1-2: Correction CR for 5G V2X UE

    - Sub-Topic #1-2-1: Draft CR from LGE (12888)

    - Sub-topic #1-1

Sub-topic description: Update of A-MPR for FCC regulation with NS_52

Open issues and candidate options before e-meeting:

Issue 1-1-1: Is it acceptable to update A-MPR and A-SEM requirements considering current spurious emission limits in 90.381 in FCC regulations in unlicensed ITS spectrum (n47)?

- Proposals

  - Option 1: Yes, RAN4 update A-MPR and A-SEM requirements with current spurious emission limits in 90.381 in FCC regulation

  - Option 2: No.

- Recommended WF

  - Option 1

Issue 1-1-2: Based on draft CR (R4-2113411) by Huawei, RAN4 can remove all A-MPR and A-SEM requirements by NS_52

- Proposals

  - Option 1: Yes, RAN4 can remove the related RF requirements at this meeting.

  - Option 2: No, RAN4 can still keep the related RF requirements and they will be replaced by updating A-MPR and A-SEM requirements at Nov. RAN4 #101-e meeting.

- Recommended WF

  - TBA

    - Sub-topic #1-2

Sub-topic description: Correction CR for 5G V2X UE

Open issues and candidate options before e-meeting:

Issue 1-2-1: Are the proposed contents in Draft CR (R4-2112888, LGE) acceptable?

- Proposals

  - Option 1: Allow the proposed contents based on the reason for changes.

  - Editorial correction of SL MIMO in maximum output power

  - For inter-band con-current V2X operation, SL can be operated in a licensed band, not n47. So update the related RF requirements.

  - Update reference FRC Tables for REFSENS

  - Option 2:  Further check the detailed contents. And further discuss in 2nd round

- Recommended WF

  - TBA.

Companies views’ collection for 1st round

    - Open issues

One of the two formats, i.e. either example 1 or 2 can be used by moderators.

Sub topic 1-1: Update of A-MPR for FCC regulation with NS_52

Issue 1-1-1: Is it acceptable to update A-MPR and A-SEM requirements considering current spurious emission limits in 90.381 in FCC regulations in unlicensed ITS spectrum (n47)?

| Company | Comments |
| --- | --- |
| CATT | Option 1. |
| LGE | Option 1 |
| Huawei | Option 1. |
| vivo | We suggest to check with FCC, then go with Option 1. FCC only allocate 30MHz in total for V2X. However, 40MHz is defined in the spec, which is from the demand for 5GAA. This mismatching issue, will also have an impact the channel bandwidths for V2X, too. We need to check this issue with FCC first. LGE : what is check point in FCC? LGE and Huawei shared the current FCC regulation. It is not aligned with current 3GPP specification. |
| Qualcomm | Option2: No. From our understanding the spurious emissions limits in 90.381 are currently only a proposal and may change. We suggest waiting until these spurious emissions specs are finalized before updating the A-MPR and A-SEM requirements. LGE: When we expect to finalize the FCC regulation? This is Rel-16 Specification, currently RAN4 treat Rel-17 SL enh WI that will be completed in 6 months. |
| OPPO | If limits still under discussion in FCC, then need to wait. |

Issue 1-1-2: Based on draft CR (R4-2113411) by Huawei, RAN4 can remove all A-MPR and A-SEM requirements by NS_52

| Company | Comments |
| --- | --- |
| CATT | OK with both options. |
| LGE | Prefer option 2 since if RAN4 remove the A-MPR and A-SEM for FCC regulation, then it can be misunderstood that there was no additional emission requirements in US. Or TBD is not good approach for 5G V2X UE in Rel-16 specification. So we prefer replace the specification in November meeting. |
| Huawei | Option 1. RAN4 can’t leave the incorrect ASEM requirements in current specification. A-SEM for FCC isn’t suitable to derive AMPR requirements directly. It’s too ambitious that RAN4 can specify ASEM and AMPR requirements together in RAN4#101. |
| vivo | Option 2. This change request can wait after we check with FCC. |
| Qualcomm | Option 3: We do not mind removing the RF requirements for NS_52 at this meeting. However, we do not think the new spurious emissions specs for NS_52 are final. So we feel that we should wait for them to be finalized before attempting to generate new A-MPR numbers for NS_52. |

Sub topic 1-2: Correction CR for 5G V2X UE

Issue 1-2-1: Are the proposed contents in Draft CR (R4-2112888, LGE) acceptable?

| Company | Comments |
| --- | --- |
| CATT | Support this CR. |
| LGE | Option 1. Support this CR |
| vivo | OK with this CR. |
| Qualcomm | Option 1: ok with CR |

    - CRs/TPs comments collection

For close-to-finalize WIs and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2112888 | vivo: OK with this CR. |
|  | Company B |
|  | … |
| R4-2113411 | Vivo: This change request can wait after we check with FCC. |
|  | Company B |
|  | … |

Summary for 1st round

    - Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1-1: Update of A-MPR for FCC regulation with NS_52 | Issue 1-1-1: Is it acceptable to update A-MPR and A-SEM requirements considering current spurious emission limits in 90.381 in FCC regulations in unlicensed ITS spectrum (n47)? In 1st round, RAN4 discussed the current 5G V2X requirmeents is not aligned with FCC regulation since only 30MHz is available in US. It was discussed from May 2021. In May meeting, RAN4 agreed to wait and check the FCC regulation until this meeting. But proponent of A-SEM(in FCC) in Rel-16 did not provide any document to explain the current action in FCC. As moderator, propose as follow - Candidate options - Option 1: RAN4 update A-MPR and A-SEM requirements based on the current spurious emission limits in 90.381 in FCC regulation. - Option 2: RAN4 still wait to finalize FCC regulation. Keep the current 3GPP specification when the revised regulation will be released. - Recommendations for 2nd round: Based on the candidate two options, RAN4 can further discuss in 2nd round. |
|  | Issue 1-1-2: Based on draft CR (R4-2113411) by Huawei, RAN4 can remove all A-MPR and A-SEM requirements by NS_52 This CR is related in Issue 1-1-1. If RAN4 make consensus with option 1 in issue 1-1-1, then follow options will be discussed in 2nd round. But Option2 is chosen in Issue 1-1-1, then this discussion is not necessary. - Candidate options - Option 1: RAN4 can remove the related RF requirements at this meeting. - Option 2: RAN4 can still keep the related RF requirements and they will be replaced by updating A-MPR and A-SEM requirements at Nov. RAN4 #101-e meeting. - Recommendations for 2nd round: According to Issue 1-1-1 decision, RAN4 can further discuss above two options. |
| Sub-Topic#1-2: Correction CR for 5G V2X UE | Issue 1-2-1: Are the proposed contents in Draft CR (R4-2112888, LGE) acceptable and reasonable? In 1st round all companies support the draft CR for 5G V2X maintenance. So it can be ageed. - Tentative Agreements: The Draft CR will be agreed and shadowing CR will be provided. - Recommendations for 2nd round: Completed. |

    - CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2112888 | The Draft CR will be endorsed. |
| R4-2112892 | Mirror CR of R4-2112888 for Rel-17. Mirror CR will be provided in 2nd round. |
| R4-2113411 | Pending the decision. It will be further discussed in 2nd round. |
| R4-2113412 | Mirror CR of R4-2113412 for Rel-17. It is depend on the decision of mother CR (R4-2113411) |

Discussion on 2nd round (if applicable)

    - Open issues (if applicable)

Sub topic 1-1: Update of A-MPR for FCC regulation with NS_52

Issue 1-1-1: Is it acceptable to update A-MPR and A-SEM requirements considering current spurious emission limits in 90.381 in FCC regulations in unlicensed ITS spectrum (n47)?

- Option 1: RAN4 update A-MPR and A-SEM requirements based on the current spurious emission limits in 90.381 in FCC regulation.

- Option 2: RAN4 still wait to finalize FCC regulation. Keep the current 3GPP specification when the revised regulation will be released.

| Company | Comments |
| --- | --- |
| LGE | Option1. The 40MHz spurious emission from 5GAA and Qualcomm was proposed in 2019 in 3GPP and FCC. FCC known that this situation. And finally they do not accept the 40MHz usage in US. LGE really would like to know that the current FCC situation. So I requested to QC to provide the current activity and status in last RAN4 meeting. But, QC did not provide any Tdoc in this meeting. Why RAN4 wait to the FCC regulation changes which is not guaranteed to the completion date. So, RAN4 can update the A-MPR and A-SEM by NS_52 in TS38.101-1 based on the current FCC A-SEM and define the A-MPR to meet the requirements. We think that A-SEM is based on the 90.381 emission limits. Interested companies can provide the A-MPR simulation based on the RAN4 A-MPR simulation assumptions. |
| Qualcomm | We do not think that we should update A-MPR values for NS 52 when the current FCC spurious emissons values are not finalized [to LGE] We did not realized that you had specifically requested us to look into this. However, we did make inquires into the latest FCC position and the information we received is that only the band location is settled and the spurious emissions values are still not finalized. We are also not sure when these specs will be finalized. We are agreeable to sending an LS to the FCC if the group decides that this is appropriate. |
| Huawei, HiSilicon | If there is no formal FCC spurious emissions available, the A-MPR and NS_52 are not correct in the specification. But removing this part in the spec may cause some compatibility issue. For the draft LS, we feel it may not be appropriate for 3GPP to ask when the rules of FCC will be issued. |
| LGE | RAN4 do not wait to the endless discussion for that. So the sending LS can encourage FCC to make decision. |

Issue 1-1-2: Based on draft CR (R4-2113411) by Huawei, RAN4 can remove all A-MPR and A-SEM requirements by NS_52

This CR is related in Issue 1-1-1. If RAN4 make consensus with option 1 in issue 1-1-1, then follow options will be discussed in 2nd round. But Option2 is chosen in Issue 1-1-1, then this discussion is not necessary.

- Option 1: RAN4 can remove the related RF requirements at this meeting.

- Option 2: RAN4 can still keep the related RF requirements and they will be replaced by updating A-MPR and A-SEM requirements at Nov. RAN4 #101-e meeting.

| Company | Comments |
| --- | --- |
| LGE | Option 2. RAN4 will replace the RF requiremens based on the current FCC regulation. |
| Qualcomm | Option 3: Can remove the current NS 52 requirements, but do not generate new values until the spurious emissions values in the FCC spec is finalized. |
| Huawei, HiSilicon | We think option 1 may cause some compatibility issue. Since the formal FCC rule is not available, we may put the existing A-MPR requirements in brackets, once the regulation is updated, the requirements can be updated accordingly. A Note could be considered that the requirements are pending update and test is not needed for the current A-MPR requirement for NS_52. In that sense, the LS to FCC is not needed. To HW from LGE: RAN4 defer the all FCC regulation issues when they feedback to 3GPP. Based on the feedback, RAN4 can work for specification to comply the FCC regulation. |

    - Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

Sub topic 1-1: Update of A-MPR for FCC regulation with NS_52

Issue 1-1-1: Is it acceptable to update A-MPR and A-SEM requirements considering current spurious emission limits in 90.381 in FCC regulations in unlicensed ITS spectrum (n47)?

Tentative agreement:

- RAN4 still wait to finalize FCC regulation. Keep the current 3GPP specification when the revised regulation will be released.

Issue 1-1-2: Based on draft CR (R4-2113411) by Huawei, RAN4 can remove all A-MPR and A-SEM requirements by NS_52

Tentative agreement:

- RAN4 can still keep the related RF requirements with brackets and it will be replaced for the A-MPR and A-SEM requirements based on the FCC updating emission limits. And A Note could be considered that the requirements are pending update and test is not needed for the current A-MPR requirement for NS_52.

| Tdoc number | Title | Source | Recommendation |
| --- | --- | --- | --- |
| R4-2114977 | WF on FCC regulation requirements for 5G V2X service | Huawei | To be Approved |
| R4-2115086 | LS on FCC regulation of the C-V2X emission limits on 47 CFR Parts 90 for V2X service in 5850-5925 MHz | LG Electronics | To be Approved |

- Topic #2: SL enhancements UE

Main technical topic overview. The structure can be done based on sub-agenda basis.

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2111940 | CATT | TP on sync raster for SL licensed band - Duplexer mode for n79 is changed from TDD to HD in Table 7.1.1-1 - Synchronization raster in session 7.3.2 was changed as follow The synchronization raster defined in sub-clause 5.4.3 in TS 38.101-1 is applied to each licensed operating band for NR SL enhancement. |
| R4-2111941 | CATT | TP on REFSENS for SL enhancement Observation 1: Uu REFSENS requirements of band n14 are more relaxed (3dB higher) than those of other bands (e.g. band n1). The downlink operating band of band n14 does not have an adequately large separation (20MHz spacing) from uplink operating band so that REFSENS should be degraded to compensate self-interference between uplink transmission and downlink reception within band n14. Observation 2: SL REFSENS for band n14 (without degradation) are more relaxed than Uu REFSENS for band n1 (without degradation). Proposal 1: To adopt the principle defined in TR 38.886 and to specify SL REFSENS for band n14 in Table 3. NR Operating band / SCS / Channel bandwidth / Duplex-mode. V2X Band; SCS kHz; 5MHz (dBm); 10MHz (dBm); 20MHz (dBm); 30MHz (dBm); 40MHz (dBm); Duplex Mode. n14; 15; -99.0; -96.5; FDD. 30; -96.1. 60. |
| R4-2112842 | LGE | TP on updating REFSENS requirements for NR SL enhancement Proposal 1: RAN4 can reuse the REFSENS requirements of NR UE for NR V2X UE when the operating band has narrow UL-DL frequency gap (up to 50MHz frequency gap). Proposal 2: For the number of RB size for REFSENS of SL enhancement UE in sidelink Tx configuration, RAN4 can choose a RB size from uplink configuration for NR UE that is close to the allowed LCRB allocation. NR Operating band / SCS / Channel bandwidth / Duplex-mode. V2X Band; SCS kHz; 5MHz (dBm); 10MHz (dBm); 20MHz (dBm); 30MHz (dBm); 40MHz (dBm); Duplex Mode. n14; 15; -97.0; -93.8; HD. 30; -94.1. 60. |
| R4-2111942 | CATT | On coexistence evaluation necessity in band n14 This Tdoc is not available |
| R4-2112606 | Xiaomi | On channelization for licensed and un-licensed band This Tdoc is not available |
| R4-2112767 | LGE | TR38.785 v0.3.0 TR update for SL enh. In Rel-17 It will be reserved to capture the all approved TPS and WF in this meeting |
| R4-2112840 | LGE | Consideration on NR PS and LTE PS different point for n14 SL enhancement coexistence study perspective Observation 1: LTE PS UE already considered with PC1 and PC3 operation whether protect adjacent carrier such as B13 UE and legacy LTE system or not. Observation 2: RAN4 did not consider OLPC scheme for coexistence evaluation in ProSe WI. But most general assumption will be reused for coexistence evaluation. Observation 3: For the NR PS UE operation perspective in n14, RAN4 consider same coexistence scenarios in Figure 1. Figure 1. Coexistence scenarios between LTE UE (B13) and LTE PS UE (B14) Observation 4: OLPC scheme will be mitigate the interference problem between legacy system and new NR PS operation. Also RAN1 allowed the power control schemes for NR SL operation as follow - OLPC is applied by based on SL pathloss in unlicensed band - OLPC is applied by based on DL pathloss or SL pathloss in licensed band - There was no power control scheme if SL-PowerControl-r16 field is not signaled. Observation 5: Based on RAN1/RAN2 specification, the worst scenario is to consider no power control scheme in NR public safety operation for SL enhancement WI. Proposal 1: Based on above observation, RAN4 do not need the additional coexistence evaluation in n14 to protect band 13 UE and LTE/NR legacy system. |
| R4-2112990 | vivo | Discussion on coexistence evaluation necessity in n14 Proposal 1: To decide whether PC1 should be supported in band n14 for SL transmission before we conclude the coexistence studies in band 14. Observation 1: In LTE ProSe, there is no additional requirements defined in band 14 for the protection of band 13 UE. Proposal 2: No need to perform additional co-existence evaluation to protect band 13. Proposal 3: NR PS operation in in-coverage scenario can be allowed in band n14 without additional coexistence evaluation with legacy LTE/NR system. |
| R4-2114337 | Ericsson | CR on NR V2X Pcmax in TS38.101-1 in Rel-17 Adding the associated serving cell and IE P-Max in Pcmax formula in 6.2E.4.1 - For the total transmitted power PCMAX,PSSCH/PSCCH , PEMAX,c is the value given by IE P-Max for serving cell c, defined by [7], when UE is associated with a serving cell on the NR V2X carrier; PEMAX,c is the value given by IEsl-maxTxPower, defined by TS 38.331, when the UE is not associated with a serving cell on the NR V2X carrier . |
| R4-2114335 | Ericsson | Coexisting simulation assumption for public safety UC and protection of B13 Figure 1: Network configuration for the sidelink carrier/BWP and Uu carrier/BWP Proposal-1: RAN4 discuss if the configuration (c ) or (d) in Figure 1 should be considered in sidelink n14 coexisting with B13 scenario without power control. Proposal-2: RAN4 further discuss the potential differences listed above between NR V2X and LTE ProSe for public safety service and conclude whether or not the coexisting simulation conclusion could apply to the NR v2x also Candidate different points - The SINR table is different with LTE and NR; should discuss if the SINR to throughput table are similar between LTE and NR V2X. - There is no power control used in TR 36.843, while power control is defined in NR V2X when configured as one BWP in serving cell; - The D2D RB allocation is 2 RB in TR 36.843, should discuss if it could be the same for NR V2X considering the traffic model - Check the ACIR model is the same with NR V2X UE in TR 36.843. |

Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Based on provided contributions, RAN4 mainly treats n14 coexistence evaluation to protect B13 and legacy n14 system for in-coverage NW scenarios. Also, RAN4 discusses the detailed general parameters (operating bands, system parameters, operating channel BWs ) and UE RF Tx/Rx requirements for SL enhancement UE.

  - Topic #2: SL enhancements

  - Sub-Topic #2-1: Coexistence evaluation for PS operation in licensed band

    - Sub-Topic #2-1-1: Protection of B13/n13 UE by n14 PS operation with PC1/PC3

    - Sub-Topic #2-1-2: Protection of legacy n14 Uu system in in-coverage NW scenarios

  - Sub-Topic #2-2: System parameters

    - Sub-Topic #2-2-1: Channel raster & Sync. raster

  - Sub-Topic #2-3: SL enh. RF requirements

    - Sub-Topic #2-3-1: Pcmax definition for SL Enhancement UE

    - Sub-Topic #2-3-2: REFSENS requirements

    - Sub-topic #2-1

Sub-topic description: Coexistence evaluation for PS operation in licensed band

Open issues and candidate options before e-meeting:

Issue 2-1-1: Protection of B13/n13 UE by n14 PS operation with PC1/PC3

- Proposals

  - Option 1: Based on LTE ProSe coexistence evaluation results, RAN4 does not need to consider the additional coexistence evaluation to protect B13/n13 UE.

  - Option 2: RAN4 needs to consider the additional coexistence evaluation to protect B13/n13 UE.

- Recommended WF

  - TBA

Issue 2-1-2:  Protection of legacy n14 Uu system in in-coverage NW scenarios

- Proposals

  - Option 1: Based on LTE ProSe coexistence evaluation results, RAN4 does not need to consider the additional coexistence evaluation to protect the legacy n14 Uu system in in-coverage NW scenarios.

  - Option 2: RAN4 needs to consider the additional coexistence evaluation to protect the legacy n14 Uu system in in-coverage NW scenarios.

- Recommended WF

  - TBA

    - Sub-topic #2-2

Sub-topic description: System parameters

Open issues and candidate options before e-meeting:

Issue 2-2-1: Channel raster & Sync. raster

- Proposals

  - Option 1: TP(R4-2111940, CATT) for synchronization raster contents are fine in a licensed band based on agreed WF (R4-2108001) at the last RAN4 meeting.

  - Option 2:  Keep the original contents for sync. raster

- Recommended WF

  - Option1 is agreeable.

    - Sub-topic #2-3

Sub-topic description: SL enh. RF requirements

Open issues and candidate options before e-meeting:

Issue 2-3-1: Pcmax definition for SL Enhancement UE

- Proposals

  - Option 1: Based on Draft CR (R4-2114337, Ericsson), RAN4 allows to define the additional Pemax which is given by serving cell c for SL enh. UE in a licensed band.

  - Option 2: Further discuss whether define the additional Pemax which is given by serving cell c for SL enh. UE in a licensed band or not.

- Recommended WF

  - TBA

Issue 2-3-2:  REFSENS requirements

- Proposals

  - Option 1: Based on TP (R4-2111941, CATT), RAN4 follow the REFSENS equation in TR38.886 to derive REFSENS requirements for SL enh UE in FDD band.

  - Option 2: Based on TP (R4-2112842, LGE), RAN4 reuse the REFSENS requirements and configuration of NR UE for SL enh UE in a specific FDD band which has a narrow UL-DL frequency gap (up to 50MHz).

- Recommended WF

  - TBA

Companies views’ collection for 1st round

    - Open issues

One of the two formats, i.e. either example 1 or 2 can be used by moderators.

Sub topic 2-1: Coexistence evaluation for PS operation in licensed band

Issue 2-1-1: Protection of B13/n13 UE by n14 PS operation with PC1/PC3

| Company | Comments |
| --- | --- |
| CATT | Prefer Option 1. |
| EAB | It is difficult to reach directly the conclusion without some discussion on the coexisting simulation assumption difference as indicated in previous meeting WF. In our paper, we think the network configuration for the NR SL needs to be clarified as there are two cases where NR SL has no associated cell and thus there is no OLPC control. If such network configuration would be allowed, the coexisting simulation should be done and option 2 should be preferable. Another discussion point is how much different between NR SINR- throughput table and LTE SINR -throughput table, if the difference is minor and network operator confirm the configuration ( case c and d ) not supported, we think option 1 would be ok. |
| LGE | Option 1, If RAN4 consider worst coexistence evaluation scenarios, it is almost same as LTE D2D coexistence scenarios. So RAN4 do not need to additional coexistence evaluation to protect B13/n13. The different between NR SINR and LTE SINR is quite small based on difference between NR V2X and LTE V2X in Rel-16. In D2D coexistence, RAN4 already considered no OLPC as worst scenarios. It is same between LTE D2D and NR PS scenarios. |
| Huawei | Tend to agree with Ericssion. Some discussion and clarification is still needed before RAN4 confirms that there is no issue for the co-existence cases. |
| vivo | Option 1. |
| Qualcomm | Option 1: |
| OPPO | E/// and HW comment is valid. And comparison is needed before conclusion is made. |

Issue 2-1-2:  Protection of legacy n14 Uu system in in-coverage NW scenarios

| Company | Comments |
| --- | --- |
| CATT | Prefer Option 1. |
| EAB | Situation here is similar with the sissue 2-1-1, however, there is no PC considered and thus from OLPC point of view, the LTE coexisting provide a worst case simulation. There are still some coexisting simulation difference identified in our paper, for example , the NR SL SINR- throughput table, the #RB for the public safety etc, if such difference could be regarded as minor factors, the conclusion will be option 1, or if the such difference in simulation assumption would be major, the option 2 would be needed. |
| LGE | Option 1. If RAN4 consider worst coexistence evaluation scenarios, it is almost same as LTE D2D coexistence scenarios. So RAN4 do not need to additional coexistence evaluation to protect legacy system in n14 in in-NW scenarios. |
| Huawei | Tend to agree with Ericssion. |
| vivo | Option 1. |
| Qualcomm | Option 1 |

Sub topic 2-2: System parameters

Issue 2-2-1: Channel raster & Sync. raster

| Company | Comments |
| --- | --- |
| CATT | Support option 1. Option 1 is derived based on the agreed WF (R4-2108001). |
| LGE | Option 1 |
| Xiaomi | Option 1. |
| Huawei | Option 1. |
| vivo | One comment: For the following change, does this mean the sync raster for the licensed bands apply for the frequency location of S-SSB? The synchronization raster defined in subclause 5.4.3 in TS 38.101-1 is applied to each licensed operating band for NR SL enhancement. |
| Qualcomm | Option 1 |

Sub topic 2-3: SL enhancement RF requirements

Issue 2-3-1: Pcmax definition for SL Enhancement UE

| Company | Comments |
| --- | --- |
| CATT | Support option 1. |
| EAB | Option 1. The associated cell is valid configuration specified in OLPC in RAN1 specificaiton. Seems removing it in RAN4 specificaiton is not properly and spefications will break. |
| LGE | Option 1 |
| Xiaomi | Option 1. |
| Huawei | Option 2. We think Pemax should use the IE specifically defined for SL, however, to consider both in coverage and out-of-coverage scenarios, the ambiguous term of serving cell could be removed. |
| vivo | In email thread [136], Issue 3-2 configured output power for intra-band con-current operation relates to the issue here. We can discuss them together. |

Issue 2-3-2:  REFSENS requirements

| Company | Comments |
| --- | --- |
| CATT | Support option 1. REFSENS in Option 1 is derived based on general principle defined in 38.886. For Band n14, SL is operated in uplink resource with half duplex mode so that there is no simultaneous Tx/Rx. Uu reference sensitivity is actually degraded to compensate self-interference between simultaneous Tx/Rx. But SL has a different case and Uu REFSENS should not be directly reused. |
| EAB | Option 1, Agree with CATT. HD-FDD mode has no TX leakage to receiver thus REFSENS should be improved. |
| LGE | Prefer option 2. n14 V2X UE have same Duplexer as n14 legacy UE. So duplexer performance for islolation perspective in n14 receiving frequency is same. |
| Huawei | Option 1. Agree with the explanation by CATT. |
| vivo | Prefer Option 1. |
| Qualcomm | Option 3: REFSENS requirements for n14 should follow the NR Uu values. If other Uu bands are introduced in the future the REFSENS for those bands should be looked at on a case by case basis. We do not agree with tightening the SLREFSENS in n14 compared to values for Uu due to RX only operation without a detailed budget showing this to be feasible. Also, according to the latest coexistence scenario there is a possibility that PS does not operate in isolation in band n14, other UEs in close proximity that are both transmitting and receiving may also be present in the n14 band that could potentially interfere with the PS receivers. |
| CHTTL | Suggest to focus on REFSENS for n14 only, cuz some of the options/ proposals seems a little bit generic to other bands. |
| AT&T | Option 1. We agree with CATT that band n14 SL REFSENS should not consider degradation to compensate for self-interference due to simulataneous Tx/Rx. In fact, for LTE ProSe case, REFSENS for SL operation was also better than Uu REFSENSE. |

    - CRs/TPs comments collection

For close-to-finalize WIs and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-211940 | Company A |
|  | Company B |
|  | … |
| R4-211941 | Qualcomm: Do not agree. N14 REFSEN values should be the same as the n14 Uu REFSENS values |
|  | Company B |
|  | … |
| R4-2112767 | TR38.785 v0.3.0 TR update for SL enh. In Rel-17 It will be reserved to capture all approved TPs and WFs in this meeting |
| R4-2112842 | Qualcomm: We do not agree with this TP. We do not know why there is a distinction between narrow and wide Ul-DL gap. We think that the n14 values for Uu REFSENS can be reused for n14 SL and other bands can be discusses on a case by case basis. LGE: to QC, the TP content is only focus on the generalized REFSENS for candidate FDD band for SL operation. We can remove the generalized contents and focus on n14 NR PS UE. This is aligned with QC intentsion. |
|  | Company B |
|  | … |
| R4-2114337 | Company A |
|  | Company B |
|  | … |

Summary for 1st round

    - Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#2-1: Coexistence evaluation for PS operation in licensed band | Issue 2-1-1: Protection of B13/n13 UE by n14 PS operation with PC1/PC3 In 1st round, RAN4 discussed whether the NR PS n14 UE protect legacy B13/n13 UE or not. Some companies (4) prefer to reuse the coexistence evaluation of LTE D2D coexistence results since PC1/3 D2D UE was analyzed to protect the adjacent carrier. And some companies (3) prefer to further clarify to conclude coexistence evaluation necessity. So, propose three options as following - Candidate options - Option 1: Based on LTE ProSe coexistence evaluation results, RAN4 does not need to consider the additional coexistence evaluation to protect B13/n13 UE. - Option 2: Further discuss the difference between LTE Prose coexistence parameters and assumtpions and NR PS operating. Based on RAN4 discussion, RAN4 conclude whether coexistence simulation is needed or not in this meeting. - Recommendations for 2nd round: Based on two candidate options, RAN4 can further discuss whether the additional coexistence evaluation is needed to protect B13/n13 UE or not. |
|  | Issue 2-1-2: Protection of legacy n14 Uu system in in-coverage NW scenarios In 1st round, RAN4 discussed whether the NR PS n14 UE protect legacy B14/n14 system or not. Some companies (4) prefer to reuse the coexistence evaluation of LTE D2D coexistence results since PC1/3 D2D UE was analyzed to protect the adjacent carrier. And some companies (2) prefer to further clarify to conclude coexistence evaluation necessity. So, propose three options as following - Candidate options - Option 1: Based on LTE ProSe coexistence evaluation results, RAN4 does not need to consider the additional coexistence evaluation to protect the legacy n14 Uu system in in-coverage NW scenarios. - Option 2: Further discuss the difference between LTE Prose coexistence parameters and assumtpions and NR PS operating. Based on RAN4 discussion, RAN4 conclude whether coexistence simulation is needed or not in this meeting. - Recommendations for 2nd round: Based on two candidate options, RAN4 can further discuss whether the additional coexistence evaluation is needed to protect legacy B14/n14 system or not in in-NW coverage. |
| Sub-Topic#2-2: System parameters | Issue 2-2-1: Channel raster & Sync. raster In 1st round, most companies (6) support the TP(R4-2111940, CATT). One claraification point is that the sync raster for the licensed bands apply for the frequency location of S-SSB? - Candidate options - Option 1: The Sync. raster of SL operation in licensed bands will be applied for the frequency location of S-SSB of SL operation. - Option 2: Need further discuss the Sync. raster of frequency location in licensed bands. - Tentative Agreements: - TP(R4-2111940, CATT) for synchronization raster contents are fine in a licensed band based on agreed WF (R4-2108001) at the last RAN4 meeting. - Recommendations for 2nd round: In 2nd round, the TP content will be further check based on the RAN4 discussion |
| Sub-topic#2-3: SL enh. UE RF requirements | Issue 2-3-1: Pcmax definition for SL Enhancement UE In 1st round, most companies (4) support the Draft CR(R4-2114337, Ericsson). But one company want to need further discussion whether define the additional Pemax which is given by serving cell c for SL enh. UE. So RAN4 can further discuss the Pcmax for SL enh. UE. - Candidate options - Option 1: Based on Draft CR (R4-2114337, Ericsson), RAN4 allows to define the additional Pemax which is given by serving cell c for SL enh. UE in a licensed band. - Option 2: Further discuss whether define the additional Pemax which is given by serving cell c for SL enh. UE for SL enh. UE and intra-band V2X con-current V2X UE in a licensed band or not. - Recommendations for 2nd round: In 2nd round, RAN4 can further discuss how to define the Pcmax in licensed band. It will be also related in intra-band V2X con-current operation UE in a licensed band. |
|  | Issue 2-3-2: REFSENS requirements In 1st round, most companies (5) support to reuse the 5G V2X REFSENS equation in TR38.886. but 2 companies propose to reuse REFSENS of NR Uu band. - Candidate options - Option 1: For n14 NR PS UE, RAN4 follow the REFSENS equation in TR38.886 to derive REFSENS requirements for SL enh UE in FDD band. - Option 2: For n14 NR PS UE, RAN4 reuse the REFSENS requirements and configuration of NR UE for SL enh UE. - Recommendations for 2nd round: Based on above two candidate options, RAN4 can further discuss how to define the n14 PS UE’ REFSENS requirements. |

    - CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2111940 | Need to revised TP to address the sync. raster of SL UE in the licensed bands apply for the frequency location of S-SSB. It will be addressed in issue 2-2-1. |
| R4-2111941 | Pending the decision. It will be further discussed in issue 2-3-2 in 2nd round. |
| R4-2112767 | TR will be treated in 2nd round to capture the all approved TPs in this meeting |
| R4-2112842 | Pending the decision. It will be further discussed in issue 2-3-2 in 2nd round. |
| R4-2114337 | Pending the decision. It will be further discussed in issue 2-3-1 in 2nd round. |

Discussion on 2nd round (if applicable)

    - Open issues (if applicable)

Sub-topic #2-1: Coexistence evaluation for PS operation in licensed band

Issue 2-1-1: Protection of B13/n13 UE by n14 PS operation with PC1/PC3

- Option 1: Based on LTE ProSe coexistence evaluation results, RAN4 does not need to consider the additional coexistence evaluation to protect B13/n13 UE.

- Option 2: Further discuss the difference between LTE Prose coexistence parameters and assumtpions and NR PS operating. Based on RAN4 discussion, RAN4 conclude whether coexistence simulation is needed or not in this meeting.

| Company | Comments |
| --- | --- |
| LGE | Option 1. The different point between NR PS and LTE D2D are listed as follow from Ericsson - The SINR table: It was already verified between LTE V2X and NR V2X. almost same - There is no power control used in TR 36.843, while power control is defined in NR V2X when configured as one BWP in serving cell; No power control is worst scenarios for the coexistence evaluation. And RAN4 shall consider worst scenarios for the coexistence evaluation to introduce the new system. So it was same between NR PS and LTE D2D. - The D2D RB allocation is 2 RB in TR 36.843, should discuss if it could be the same for NR V2X considering the traffic model : NR PS UE shall assisned at leat 10RB for the SL operation in NR. So, 2RB allocation is worst case for the adjacent carrier interference perspective. So, NR PS’s interference is smaller than LTE D2D. - Check the ACIR model, To verify the coexistence analysis, RAN4 assumed TR36.942 for the ACIR model. So it was same between LTE D2D and NR PS. Hence, RAN4 can reuse the LTE D2D coexistence evaluation as worst scenarios. In NR PS, RAN4 can expected the smaller interference problems compare to LTE D2D coexistence results. |
| Qualcomm | Option 1 |
| Ericsson | The coexisting study for B13 and B14 in LTE is in TR36.837, and TR 36.843 does not cover this. I agree on the SINR table, RB allocation and ACIR with LGE. For power control, as TR 36.843 model the power control so the coexisting dependency on LTE can only be valid with the same power control assumption. Maybe operator could comment on this. |
| AT&T | Option 1. |
| Ericsson | Option 1 is fine to us. |

Issue 2-1-2:  Protection of legacy n14 Uu system in in-coverage NW scenarios

- Option 1: Based on LTE ProSe coexistence evaluation results, RAN4 does not need to consider the additional coexistence evaluation to protect the legacy n14 Uu system in in-coverage NW scenarios.

- Option 2: Further discuss the difference between LTE Prose coexistence parameters and assumtpions and NR PS operating. Based on RAN4 discussion, RAN4 conclude whether coexistence simulation is needed or not in this meeting.

| Company | Comments |
| --- | --- |
| LGE | It is same view in Issue 2-1-1. |
| Qualcomm | Option 1 |
| Ericsson | Based on the LGE reply, we are also ok with option 1. |
| AT&T | Option 1. |

Sub-topic #2-2: System parameters

Issue 2-2-1: Sync. raster of SL operation in licensed band

- Option 1: The Sync. raster of SL operation in licensed bands will be applied for the frequency location of S-SSB of SL operation.

- Option 2: Need further discuss the Sync. raster of frequency location in licensed bands.

- Option 3: Keep the RAN4 agreements for Synchronization Raster in Rel-16.

| Company | Comments |
| --- | --- |
| Vivo | If Option 1 is the common understanding among companies, it should be captured in the TP on sync raster for SL enhancement. We also want to point out: In Rel-16, we agreed no sync raster for both licensed bands and unlicensed bands for NR V2X. However, in Rel-17, sync raster will be applied for the frequency location of S-SSB. If we make this an agreement, should we inform RAN1 of this agreement as we did in Rel-16? The conclusion on sync raster design will have an impact on RAN1. |
| CATT | We have provided the updated TP with one sentence added: The synchronization raster defined in subclause 5.4.3 in TS 38.101-1 is applied to each licensed operating band for NR SL enhancement. The synchronization raster of SL operation in licensed band is applied for the frequency location of S-SSB. Companies please check if this is acceptable. |
| LGE | We are fine with revision. For the Rel-16, RAN4 agreed no sync. raster definition for both licensed and unlicensed band. But it is not mean that there was no sync. raster for SL operation UE. For n38 SL UE, the synchronization raster will be reused of NR Uu. This is our view for the clarification of it. From the QC comment, I awared the reason that RAN4 did not define the Sync. raster in both licensed band and unlicensed band in Rel-16. Specially FDD band has some problem for the sync. raster. So option 3 is best way in licensed band. |
| Qualcomm | Option 2: Following is an agreement from RAN1AH1901: Agreements: - The frequency location for S-SSB is (pre-) configured - Note: it implies that there is no intended hypothses detection in frequency location of S-SSB performed by the UE for a carrier in a given band - Note: the potential frequency locations for the (pre-)configured frequency location may be restricted, up to RAN4 Our interpretation of the above is that RAN1 does not have any concept of synch raster for SL and also for FDD bands the synch raster for Uu is only defined for the DL band. For SL on n14 where only the UL band is used it would not be possible to position the S-SSB on the synch raster as there is no synch raster defined for UL. For this reason we believe that it would be best not to define a synch raster for licensed bands. |

Sub-topic #2-3: SL enahncement RF requirements

Issue 2-3-1: Pcmax definition for SL Enhancement UE

- Option 1: Based on Draft CR (R4-2114337, Ericsson), RAN4 allows to define the additional Pemax which is given by serving cell c for SL enh. UE in a licensed band.

- Option 2: Further discuss whether define the additional Pemax which is given by serving cell c for SL enh. UE for SL enh. UE and intra-band V2X con-current V2X UE in a licensed band or not.

| Company | Comments |
| --- | --- |
| CATT | Option 1. |
| LGE | Option 1 |
| Xiaomi | We have a comment here as: For the case as mentioned in Ericsson’s CR that “when UE is associated with a serving cell on the NR V2X carrier”, does it mean the intra-band concurrent operation for the same carrier? If so, then this situation should be stated for the sub-clause for intra-band concurrent operation configured power, but not in the general part. LGE: to Xiaomi: This is for SL enh. UE single operation in n14 in [134] email thread. So RAN4 do not consider the intra-band concurrent operation for the same carrier usage in this email thread. You mentioned issues will be treated in configured Tx power requirements in [135] e-mail trhead. So RAN4 need to distinguish the single carrier operation and intra-band con-current operation. Secondly, this might also imply that the power class of SL transmission is limited by the IE p-max for intra-band concurrent operation. We are wondering if this is the common understanding of the group. |
| Qualcomm | Option 1 |
| Huawei, HiSilicon | Option 2. Since there is specific IE for SL, we see no reason to separate the IEs for different scenarios. In addition, IE sl-maxTxPower is associated with Cond CBR, in our view, that is not the correct IE refered for this power limitation scenario. |

Issue 2-3-2:  REFSENS requirements

- Option 1: For n14 NR PS UE, RAN4 follow the REFSENS equation in TR38.886 to derive REFSENS requirements for SL enh UE in FDD band.

- Option 2: For n14 NR PS UE, RAN4 reuse the REFSENS requirements and configuration of NR UE for SL enh UE.

| Company | Comments |
| --- | --- |
| CATT | Option 1. As lots of companies presented in 1st round, SL in Band n14 operates with HD instead of FDD for Uu. HD SL doesn’t have Tx leakage into receiver side so no need to degrade REFSENS requirements for SL. Also AT&T pointed out that for LTE ProSe SL REFSENS was also better than Uu REFSENS. So Uu REFSENS does not have to be a reference to decide SL REFSENS. RAN4 should not ignore the actual situation just by looking at Uu REFSENS requirements. For possible interference from Uu UE mentioned by QC, it falls in intra-band V2X and Uu operation instead of SL only case. RAN4 could further study REFSENS for intra-band V2X operation in n14. If such interference is identified, sensitivity degradation can be introduced for intra-RAT case in future. |
| LGE | Option 2. For n14 NR PS UE, it has same Duplexer as n14 legacy UE. also RAN4 allocated SL transmission configuration for REFSENS requirements test. So same REFSENS was expected. For the D2D Prose, the agreed REFSENS equiation is as follow RefSensD2D = RefSensWAN+ ∆SNR(D2D-WAN) + ∆IL(UL-DL)+ 10log10(LCRB/NRB). In here, the ∆SNR(D2D-WAN) and LCRB is different between D2D and WAN. So the REFSENS defined as different. Also considered 4 retransmission to REFSENS. But, when we consider same LCRB and no retransmission, then same REFSENS is expected. |
| Qualcomm | Option 2: We think that though the TX is not operating while n14 is receiving there are additional RF losses due to additional switches in this implementation where the n14 SL and n14 Uu will be sharing the same RF circuitry. Also, there may be additional losses in the duplexer TX path which now is being used for both TX and RX. For these reasons we think the REFSENS should be kept the same as for Uu. If we want to tighten the REFSENS requirement for SL then a detailed budget should be provided to show the impact of the TX on the current REFSENS values and the extra losses mentioned above should also be considered. |
| Huawei, HiSilicon | Option 1. We share similar view as CATT. |
| AT&T | Option 1. To clarify our statement on LTE ProSe REFSENS, we are referring to the direct communication REFSENS. In addition, the reference measurement channel for direct communication uses the same LCRB and no retransmissions as for Uu operation. Even in this case, REFSENS in HD-FDD mode is better than Uu performance. To AT&T: I attendted RAN4 meeting to specify REFSENS of D2D communication. In here RAN4 allow 4 HARQ with full RBs. So REFSENS of D2D UE is higher about 3.8dB compare to LTE UE. Table A.6.2-2: Fixed Reference measurement channel for ProSe Direct Communication receiver requirements Parameter; Unit; Value. Channel bandwidth; MHz; 1.4; 3; 5; 10; 15; 20. Allocated resource blocks; 25; 50. Subcarriers per resource block; 12; 12. Packets per SA period; 1; 1. Modulation; QPSK; QPSK. Transport Block Size; 2216; 4392. Transport block CRC; Bits; 24; 24. Maximum number of HARQ transmissions; 4; 4. Binary Channel Bits; Bits; 7200; 14400. Max. Throughput averaged over 1 SA period of 40ms; kbps; 55.4; 109.8. UE Category; ≥ 1; ≥ 1. Note 1: For PSSCH transmission, the last symbol shall be punctured as per TS 36.211. NOTE 2: Throughput (in kbps) will depend on SA period configuration. |

    - Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

Sub-topic #2-1: Coexistence evaluation for PS operation in licensed band

Issue 2-1-1: Protection of B13/n13 UE by n14 PS operation with PC1/PC3

Tentative agreement

- Option 1: Based on LTE ProSe coexistence evaluation results, RAN4 does not need to consider the additional coexistence evaluation to protect B13/n13 UE.

Issue 2-1-2:  Protection of legacy n14 Uu system in in-coverage NW scenarios

Tentative agreement

- Option 1: Based on LTE ProSe coexistence evaluation results, RAN4 does not need to consider the additional coexistence evaluation to protect the legacy n14 Uu system in in-coverage NW scenarios.

Sub-topic #2-2: System parameters

Issue 2-2-1: Sync. raster of SL operation in licensed band

Tentative agreement

- Option 3: Keep the RAN4 agreements for Synchronization Raster in Rel-16.

Sub-topic #2-3: SL enahncement RF requirements

Issue 2-3-1: Pcmax definition for SL Enhancement UE

GTW Agreement

- Further check if RAN4 can adopt Option 1 for SL Enh. UE in n14  and if the existing IEsl-maxTxPower can be used

Issue 2-3-2:  REFSENS requirements

Tentative Agreement

- RAN4 need further discussion for REFSENS requirements in n14.

| Tdoc number | Title | Source | Recommendation |
| --- | --- | --- | --- |
| R4-2114978 | WF on n14 coexistence evaluation for NR PS UE | Ericsson | To be Approved |
| R4-2114979 | WF on Pemax definition and NR PS REFSENS in licensed band | LGE | To be Approved |

- Recommendations for Tdocs

1st round

New tdocs

| Title | Source | Comments |
| --- | --- | --- |
| WF on FCC regulation requirements for 5G V2X service | Huawei |  |
| WF on n14 coexistence evaluation for NR PS UE | Ericsson |  |
| WF on Pemax definition and NR PS REFSENS in licensed band | LGE |  |

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2112843 | Consideration on A-SEM requirements by NS_52 based on FCC regulation | LG Electronics France | To be noted |  |
| R4-2112888 | Draft CR for correction of 5G V2X UE RF requirements in Rel-16 | LG Electronics France | To be endorsed |  |
| R4-2112892 | Draft CR for correction of 5G V2X UE RF requirements in Rel-17 | LG Electronics France | To be endorsed | Mirror CR will be provided in 2nd round |
| R4-2113411 | Draft CR for 38.101-1 to remove the ASE requirements for NS_52 (Rel-16) | Huawei, HiSilicon | To be returned | Further discussed in 2nd round |
| R4-2113412 | Draft CR for 38.101-1 to remove the ASE requirements for NS_52 (Rel-17) | Huawei, HiSilicon | To be returned | Further discussed in 2nd round |
| R4-2111940 | TP on sync raster for SL licensed bands | CATT | To be revised (R4-2114980) |  |
| R4-2111941 | TP on REFSENS for SL enhancement | CATT | To be noted | Further discussed in 2nd round. It will be addressed in LGE revised TP. |
| R4-2111942 | On coexistence evaluation necessity in band n14 | CATT | To be noted |  |
| R4-2112767 | TR38.785 v0.3.0 TR Update for SL enhancement in Rel-17 | LG Electronics France | To be returned | TR will be treated in 2nd round to capture the all approved TPs. |
| R4-2112840 | Consideration on NR PS and LTE PS different point for n14 SL enhancement coexistence study perspective | LG Electronics France | To be noted |  |
| R4-2112842 | TP on updating REFSENS requirements for NR SL enhancement | LG Electronics France | To be revised (r4-2114981) | Further discussed in 2nd round |
| R4-2112990 | Discussion on coexistence evaluation necessity in n14 | vivo | To be noted |  |
| R4-2114335 | Coexisting simulation assumption for public safety UC and protection of B13 | Ericsson | To be noted |  |
| R4-2114337 | CR on NR V2X Pcmax in TS38.101-1 in Rel-17 | Ericsson | To be returned |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

2nd round

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2114977 | WF on FCC regulation requirements for 5G V2X service | Huawei | To be Approved |  |
| R4-2114978 | WF on n14 coexistence evaluation for NR PS UE | Ericsson | To be Approved |  |
| R4-2114979 | WF on Pemax definition and NR PS REFSENS in licensed band | LG Electronics | To be Approved |  |
| R4-2112767 | TR38.785 v0.3.0 TR Update for SL enhancement in Rel-17 | LG Electronics France | To be Agreed | Update TR contents based on the approved TPs in [134][135][136] email thread |
| R4-2114981 | TP on updating REFSENS requirements for NR SL enhancement | LG Electronics France | To be Noted | [ ] is added |
| R4-2114980 | TP on sync raster for SL licensed bands | CATT | To be Noted |  |
| R4-2113411 | Draft CR for 38.101-1 to remove the ASE requirements for NS_52 (Rel-16) | Huawei, HiSilicon | To be Postponed | Further discuss in next RAN4 meeting |
| R4-2113412 | Draft CR for 38.101-1 to remove the ASE requirements for NS_52 (Rel-17) | Huawei, HiSilicon | To be Postponed | Further discuss in next RAN4 meeting |
| R4-2114337 | CR on NR V2X Pcmax in TS38.101-1 in Rel-17 | Ericsson | To be Postponed | Further discuss in next RAN4 meeting |
| R4-2115086 | LS on FCC regulation of the C-V2X emission limits on 47 CFR Parts 90 for V2X service in 5850-5925 MHz | LGE | To be Approved | To verify the emission limits for C-V2X UE in 5895~5925MHz |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents

Annex

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| LG Electronics | Suhwan Lim | Suhwan.lim@lge.com |
| CATT | Yuan Gao | gaoyuan@catt.cn |
| vivo | Shuai Zhou | shuai.zhou@vivo.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)