3GPP TSG-RAN WG4 Meeting # 100-e																 R4-2115035

Electronic Meeting, August. 16-27, 2021

Agenda item:			9.15.5

Source:	Moderator (CATT)

Title:	Email discussion summary for [100-e][135] NRSL_enh_Part_2

Document for:	Information

Introduction

This email discussion summary will continue to focus on operating scenarios, synchronous operation, and RF requirements for intra-band V2X operation.

The agenda items involved are as follows:

9.15.5	Partially used SL operation with NR Uu operating bands	[NR_SL_enh-Core]

9.15.5.1	FDM operation	[NR_SL_enh-Core]

9.15.5.2	TDM operation	[NR_SL_enh-Core]

9.15.5.3	Synchronous operation between NR Uu and NR SL in a TDD band	[NR_SL_enh-Core]

9.15.5.4	Others

The candidate targets of this email discussion for 1st round and 2nd round:

- 1st round

  - Companies to provide comments on each sub-topic and try to converge.

  - Companies to check TPs and provide comments

  - Assign the corresponding WF after the 1st round discussion.

- 2nd round

  - Capture the agreements and open issues if any in WF and further discuss the WF.

  - Recommend the final status of the WFs and TPs.

Topic #1: Operating scenarios for intra-band V2X operation

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2112769 | LG Electronics France | Title: RF requirements for intra-band con-current V2X operation with NR PC5 and NR Uu in a licensed band Proposal 1: RAN4 specify ON/OFF Time Mask for TDM operation in same carrier as shown in Figure 2-1 to Figure 2-3. Proposal 2: For the ON/OFF time mask for TDM operation in different carrier, RAN4 specify the ON/OFF Time mask in Figure 2-4 (b). |
| R4-2112771 | LG Electronics France | Title: TP on RF requirements for intra-band con-current V2X operation in licensed band |
| R4-2112991 | vivo | Title: Discussion on issues for intra-band con-current operation Proposal 1：No need to introduce the frequency separation for the case Uu and SL are in different channels for intra-band con-current operation. Proposal 2: RAN4 discuss the necessity of the restriction of not allowing the con-current od SL reception and Uu transmission and its impact on the specification. Observation 1: RAN1 defined the priority rule for simultaneous transmission case, not for the switching case for Uu and SL transmissions discussed in RAN4. Proposal 3: If RAN4 decide the switching period position based on the priority rule defined in RAN1, the impact on RAN1 should also be considered. Proposal 4: RAN4 should make a clear decision whether to allow intra-band con-current V2X operation for FDD band. |
| R4-2111944 | CATT | Title: On V2X intra-band con-current operation Observation 1: In terms of intra-band contiguous and non-contiguous CA, it should be noted that the minimum requirements only apply for non-simultaneous Tx/ Rx between all carrier for TDD combinations. In other words, frequency separation is not taken into account from standard standpoint to protect Rx reception from Tx transmission for intra-band CA. Proposal 1: Not allow simultaneous UL Tx and SL Rx for non-contiguous V2X intra-band con-current operation like contiguous case. Proposal 2: Send LS to RAN1 to indicate RAN4 agreements w.r.t contiguous and non-contiguous V2X con-current operation within a licensed TDD band. |
| Rev_R4-2112604 | Xiaomi | Title: Further discussion on FDM intra-band concurrent operation Observation 1: The guard period is designed for sidelink TX and RX switching. Observation 2: The guard period should both cover the transient period and the TTA. Observation 3: For SCS as 15kHz and 30kHz the guard period is long enough, however, for SCS as 60kHz, 2 symbols might be needed . Observation 4: Currently RRM session is considering decreasing the NTA_offset to keep the guard period unchanged as one symbol. Observation 5: For case 2 as sidelink aligned to UL, the guard period is long enough to cover the transient period. Proposal 1: Wait for the RRM session discussion as well as the RAN1 reply LS to finalize the guard period discussion. |
| R4-2114338 | Ericsson | Title: FDM operation for partially used SL operation in licensed band Proposal-1: Confirm that no in-device coexisting study means that it is not allowed the con-current reception of SL and Uu transmission for both adjacent and non-adjacent carrier cases in TDD band. Observation-1: To allow the con-current SL reception and Uu transmission, the TDD and FDD band has the same issue where the 79 dB isolation will be needed between SL receiving signal and Uu transmission signal. Observation-2: RAN4 need to study how to achieve the isolation if con-current SL reception and Uu transmission is allowed in general. Proposal-2: As the FDD band and TDD band has the same coexisting issue for con-current SL reception and Uu transmission, the same decision should be applied to both TDD and FDD band. |
| R4-2111943 | CATT | Title: TP on intra-band V2X operation |
| Rev_R4-2112605 | Xiaomi | Title: Further discussion on TDM intra-band concurrent operation Observation 1: The overlap of UL and SL occurs during SL to UL switching and for UL to SL switching there is no interference. Observation 2: The new method as figure 3 do not need to change the guard period design and with less spec effort. Observation 3: Similar scheduling restriction is needed. Observation 4: The switching period should be defined by RF session and can be informed to RRM session for their discussion of scheduling restriction. Observation 5: Even without the timing advance issue, the switching period still need to be discussed. Observation 6: 150us switching period can be used as starting point for NR and NR sidelink switching for intra-band concurrent operation in TDD band. Observation 7: Considering the timing advance, placing the switching time at the NR sidelink slot can fully use the guard period to save system overhead. Proposal: Propose the switching time as figure 3 shown as keep the guard period design unchanged and place the switching time at NR SL slot when switching from NR SL to NR slot. |
| R4-2114506 | Huawei, HiSilicon | Title: On TDM operation for NR SL Observation 1: The switching time discussed in Rel-16 is for cases of different carriers between LTE SL and NR SL, which is not appropriate Rel-17 for switching case in same carrier for Uu and SL Proposal 1: It is proposed to consider switching time separately for cases of same carrier and different carriers Observation 2: If SL transmission timing is aligned with received DL timing, to avoid interference between Uu and SL, guard period should be considered Proposal 2: It is proposed to wait for the response from RAN1 on SL timing alignment issue to decide whether guard period should be considered |
| R4-2111945 | CATT | Title: On time mask for Uu and SL switching Proposal 1: To consider the time mask in Figure 1 and Figure 2 (15kHz/30kHz SCS) for SL and Uu switching with the same carrier. Proposal 2: To consider the time mask in Figure 3 and Figure 4 for SL and Uu switching with different carriers without dual PA capability. |
| R4-2114251 | Qualcomm Incorporated | Title: Con-current reception of SL and Uu transmission in licensed band Proposal: Do not allow con-current NR UL Transmission and NR SL reception with non-adjacent carrier for FDM operation in a TDD band. |

Open issues summary

Based on above contributions, the following sub-topics and issues regarding operating scenarios for partially used SL operation are summarized.

- Sub-topic 1-1: Intra-band V2X operation (TDM)

- Issue 1-1-1: Switching time for same carrier and different carrier

- Issue 1-1-2: Switching time length

- Issue 1-1-3: Switching time position

- Issue 1-1-4: Time mask for same carrier

- Issue 1-1-5: Time mask for different carriers

- Sub-topic 1-2: Intra-band V2X con-current operation (FDM)

- Issue 1-2-1: In-device coexistence

- Issue 1-2-2: Non-adjacent carrier in TDD band

- Issue 1-2-3: Whether to consider intra-band V2X con-current operation in FDD band

- Issue 1-2-4: Intra-band V2X con-current operation in FDD band

- Issue 1-2-5: Whether to send LS to RAN1 to indicate RAN4 agreements w.r.t contiguous and non-contiguous V2X con-current operation within a licensed TDD band.

Sub-topic 1-1: Intra-band V2X operation (TDM)

Issue 1-1-1: Switching time for same carrier and different carrier

- Proposals

  - Option 1: To consider switching time separately for cases of same carrier and different carriers.

- Recommended WF

  - Option 1 can be agreed.

Issue 1-1-2: Switching time length

- Proposals

  - Option 1: 150us switching time can be used as starting point for NR and NR sidelink switching for intra-band concurrent operation in TDD band.

- Recommended WF

  - Moderator: If switching period (120us) for LTE SL and NR SL switching can be reused, the switching time for NR Uu and NR SL switching should be 140us (120+10+10).

Issue 1-1-3: Switching time position

- Proposals

  - Option 1: If RAN4 decide the switching period position based on the priority rule defined in RAN1, the impact on RAN1 should also be considered.

  - Option 2: No impact on RAN1 in case RAN4 decide the switching period position based on the priority rule defined in RAN1.

- Recommended WF

  - Need more discussion.

Issue 1-1-4: Time mask for same carrier

- Proposals

  - Option 1: RAN4 specify ON/OFF Time Mask for TDM operation in same carrier as proposed in paper R4-2112769 and R4-2111945.

Figure 1: Time mask for NR Uu switching to NR SL

Figure 2: Time mask for NR SL switching to NR Uu (15kHz/30kHz SCS)

Figure 3: Time mask for NR SL switching to NR Uu (60kHz SCS)

- Recommended WF

  - Option 1 can be agreed.

Issue 1-1-5: Time mask for different carriers

- Proposals

  - Option 1: To consider the time mask in Figure 4 for SL and Uu switching with different carriers without dual PA capability proposed in paper R4-2112769.

Figure 4: Time mask for TDM operation in different carrier from NR SL/NR Uu to NR Uu/NR SL

  - Option 2: To consider the time mask in Figure 5 and Figure 6 for SL and Uu switching with different carriers without dual PA capability as proposed in paper R4-2111945.

Figure 5: Time mask for NR SL (higher priority) switching to NR Uu without dual PA capability

Figure 6: Time mask for NR Uu (higher priority) switching to NR SL without dual PA capability

- Recommended WF

  - Check whether the scheduling restriction similar to LTE V2X and NR V2X switching defined in Rel-16 RRM session should be used or not.

Sub-topic 1-2: Intra-band V2X con-current operation (FDM)

Issue 1-2-1: In-device coexistence

- Proposals

  - Option 1: Confirm that no in-device coexisting study means that it is not allowed the con-current reception of SL and Uu transmission for both adjacent and non-adjacent carrier cases in TDD band.

- Recommended WF

  - Option 1 can be agreed.

Issue 1-2-2: Non-adjacent carrier in TDD band

- Proposals

  - Option 1: Do not allow simultaneous UL Tx and SL Rx for non-contiguous V2X intra-band con-current operation like contiguous case.

  - Option 2: Introduce frequency separation to allow simultaneous UL Tx and SL Rx with an adequate isolation.

- Recommended WF

  - Need more discussion.

Issue 1-2-3: Whether to consider intra-band V2X con-current operation in FDD band

- Proposals

  - Option 1: Yes

  - Option 2: No

- Recommended WF

  - Moderator: In last meeting, operators clarified that NR PS operating in band n14 includes not only out-of-coverage but in-coverage scenarios. Whether to consider intra-band V2X con-current operation for band n14 should be based on operator request.

Issue 1-2-4: Intra-band V2X con-current operation in FDD band

- Proposals

  - Option 1: As the FDD band and TDD band has the same coexisting issue for con-current SL reception and Uu transmission, the same decision should be applied to both TDD and FDD band.

  - Option 2: More study is needed for con-current SL reception and Uu transmission in FDD band.

- Recommended WF

  - Need more discussion.

Issue 1-2-5: Whether to send LS to RAN1 to indicate RAN4 agreements w.r.t contiguous and non-contiguous V2X con-current operation within a licensed TDD band.

- Proposals

  - Option 1: Yes

  - Option 2: No

- Recommended WF

  - Need more discussion.

Companies views’ collection for 1st round

Open issues

Issue 1-1-1: Switching time for same carrier and different carrier

| Company | Comments |
| --- | --- |
| Xiaomi | Option 1. Agree to define switching time with same carrier and different carrier cases. |
| CATT | Option 1. Single RF chain is considered as baseline for TDM operation. Switching time should be considered separately for same carrier and different carriers. Specifically, for the same carrier, only transient period can be included. For the different carriers, switching period plus transient period should be included. |
| Huawei | Option 1. |
| LGE | Option 1. |
| vivo | OK with Option 1. |
| Qualcomm | Option 2: We think that this needs further discussion. We should agree on what are all the possible configuration changes that can occur when switching between Uu and SL. We believe that both the same carrier and different carrier cases will have a switching period as well as a transient period. There are several contributors to switching period due to differences between Uu and SL such as: - LO tuning time - Bandwidth changes - RB allocation changes - PA power differences - Baseband settings. For the same carrier the absence of LO switching may not shorten the switching time adequately for it to be less than 1 symbol in duration for all SCS values if all the other above-mentioned changes (except LO tuning time) need be done. Suggest WF: - Companies agree on are all the possible configuration changes needed when switching from Uu to SL and visa-versa - Companies research their capabilities and present their views at RAN4#101-e with the goal of seeing if switching can be achieved in 1 symbol or less. - From results it will become apparent if switching with same or different carriers should be treated the same or not. |
| OPPO | Option 1. |

Issue 1-1-2: Switching time length

| Company | Comments |
| --- | --- |
| Xiaomi | Agree with 120us switching period as starting point. |
| CATT | If switching period (120us) for LTE SL and NR SL switching can be reused, the switching time for NR Uu and NR SL switching should be 140us (120+10+10). |
| Huawei | Agree with the recommended WF. |
| LGE | Switching period (140us) for NR Uu and NR SL in a licensed band is considered |
| vivo | Agree with CATT. 140 us can be used for NR Uu and SL switching. |
| Qualcomm | Option 2:We think that 210us would be a good starting point for NR Uu to NR SL switching time. As the candidate switching times under consideration (i.e. 150us & 210us) are larger than 1 symbol guard period we should agree on asking the RRM session to decide on the scheduling restriction that should apply to switching between Uu and SL. Based on the RRM agreement switching time can be discussed further. Suggested WF - As both candidate switching times are greater than 1 symbol ask RRM session to decide on a scheduling restriction that would apply - Based on RRM agreement switching time can be discussed further |
| OPPO | WF is ok. |

Issue 1-1-3: Switching time position

| Company | Comments |
| --- | --- |
| Xiaomi | We don’t see the impact to RAN1. However, for the position, in our paper, it depends on the case as SL to NR or NR to SL switching. Before RAN1 reply LS on the SL timing, we cannot agree with the priority rule at this stage. |
| CATT | Option 2. |
| Huawei | Option 2. |
| LGE | The position will be decided based on RAN1/RAN2 agreement. So it will not impact to any other WG. |
| vivo | Option 1. The priority rule defined in RAN1 is not for the switching case between Uu and SL. However, if RAN4 decides to use the priority to the switching case, we should check with RAN1 first. |
| Qualcomm | Option 3: RAN4 decide switching position based on priority rule defined in RAN1. We do not see any RAN1 impact but are agreeable to discuss this further. |
| OPPO | Option 2. |

Issue 1-1-4: Time mask for TDM with same carrier

| Company | Comments |
| --- | --- |
| Xiaomi | For the 10us time mask, it is agreeable. However, for the 60kHz SCS, we still need to figure out the feasibility of 8us transient period. |
| CATT | Option 1. For the same carrier switching, only transient period should be included in whole switching time. For 60kHz SCS, we support to adopt transient period (8us+8us) that can be covered in one symbol. |
| Huawei | Transient period is not determined by different SCS. 10us is ok, but no reason to have 8us just because to be confined for a symbol for 60k SCS. |
| LGE | Prefer Option 1. For the same carrier switching period for all SCS, the transient period should be included in 1 symbol punctured time to protect the previous symbol data of NR SL. So 8us transient period will be considered for 60kHz SCS. If we consider 10us transient period for 60kHz SCS, then 13th symbol date will be lost by transient period. |
| vivo | This issue can wait after we conclude the issue 1-1-1/2/3. |
| Qualcomm | Option 2: We think that this needs further discussion due to the following reasons: - It is not clear what configuration changes are being made when switching from Uu to SL and vis-versa. The required changes will determine how fast this switching can occur. - Though LO retuning is absent in this case, same carrier switching using a single RF chain may still require RF changes between NR Uu and NR SL, such as PA power, channel bandwidth and RB locations as outlined in our answer to issue 1-1-1. - There may be a timing alignment offset between Uu UL/DL and SL that has to be taken into account depending on if UL/ DL timing is used for SL. - Not known if both Uu and SL are using the same synch reference source which may add additional switching time. Suggested WF - Companies agree on all the possible configuration changes needed when switching from Uu to SL and visa-versa 2 Companies research their capabilities and present results at RAN4#101-e with the goal of seeing if switching can be achieved in 1 symbol or less. 3 From results it will become apparent if these switching masks can be met or not. |
| OPPO | For clarification, where does 8us coming from and the reason is? |

Issue 1-1-5: Time mask for TDM with different carriers

| Company | Comments |
| --- | --- |
| Xiaomi | Sorry for the late response, but in our paper Rev_R4-2112605, we have different time mask as for SL to NR switching. While no time mask is needed for NR to SL switching. This is based on the DL timing alignment. So before the RAN1 reply LS, we think this still need to be considered. We believe an option 3 is needed. Option 3: Switching from SL to NR: No time mask for NR to SL switching. |
| CATT | Need to check whether the scheduling restriction similar to LTE V2X and NR V2X switching defined in Rel-16 RRM session should be reused or not. If similar scheduling restriction applies, the switching time is not expected to cross slot boundary. |
| Huawei | Switching time is determined by UE implementation, which should be separated with guard period, as latter one is related to timing alignment as well as TA adjustment. Time mask requirement similar to Rel-16 can be considered, and interruption could be still decided by RRM session. |
| LGE | Maybe we can wait to RAN1 reply LS, but basic approach is not to impact to cross slot boundary. For the Xiaomi proposal, the 140us switching period and 1symbol (up to 72us) is quite large loss term for the spectral efficiency perspective. So switching period can overlapped the GP. |
| vivo | Agree with CATT. Besides the scheduling restriction, this issue can wait after we conclude the issue 1-1-1/2/3. |
| Qualcomm | Option3: before agreeing on the time mask we should wait for an answer from RAN1 regarding timing reference for SL. This may add a time alignment component to these diagrams. We should also agree that the switching time will be greater than 1 symbol period therefore a RRM scheduling restriction should be adopted similar to LTE SL to NR SL intra-RAT switching |

Issue 1-2-1: In-device coexistence

| Company | Comments |
| --- | --- |
| Xiaomi | Agree with option 1. |
| CATT | Option 1. If con-current reception of SL and Uu transmission for both adjacent and non-adjacent carrier cases in TDD band is not allowed, no in-device coexistence is expected. |
| Huawei | Agree with option 1. |
| LGE | Option 1 |
| vivo | Option 1. In physical layer design, SL Tx/Rx resources are configured in the UL resources in the licensed bands, If the con-current SL Rx and UL are not allowed, this will have an impact RAN1. |
| Qualcomm | Option 2: While con-current SL RX with Uu TX is not allowed for both adjacent and non-adjacent carriers our studies have shown that SL RX with Uu RX may still have degraded REFSENS since 1 carrier can act as a jammer to the other. |
| OPPO | Option 1. |

Issue 1-2-2: Non-adjacent carrier in TDD band

| Company | Comments |
| --- | --- |
| Xiaomi | We see some relationship of this issue 1-2-2 with issue 1-2-1 and they might need to be discussed together. |
| CATT | Option 1. Even for non-contiguous operation, it is difficult for implementation to guarantee the performance of simultaneous UL Tx and SL Rx within a licensed band. Unlike inter-band case, no RF band filter is taken into effect. |
| Huawei | Option 1. We think that may not be feasible for UE supporting simultaneous Rx and Tx in the same band even with frequency separation. |
| LGE | This feature is proposed in Rel-16 from interested operator. So We can hear of operator feedback for this issue. As Rapporteur perspective, we can suggest to treat the simultaneous Rx/Tx capability in intra-band CA UE in NR Uu firstly, then SL can consider this simultaneous Rx/Tx Capability based on the NR Uu study results. So, currently we propose as follow - For the simultaneous Rx/Tx capability in licensed band, it can study the feasibility of the simultaneous Rx/Tx capability in intra-band CA UE in NR Uu firstly, then SLV2X operation will be considered this simultaneous Rx/Tx Capability study results in NR Uu. |
| vivo | Option 1 is OK. We also suggest not to introduce frequency separation. |
| Qualcomm | Option 1: Do not allow simultaneous UL Tx and SL Rx for non-contiguous V2X intra-band con-current operation like contiguous case. |
| OPPO | Option 1. |

Issue 1-2-3: Whether to consider intra-band V2X con-current operation in FDD band

| Company | Comments |
| --- | --- |
| Xiaomi | Agree with moderator to depend on operator request, |
| CATT | Whether to consider intra-band V2X con-current operation for band n14 should be based on operator request. |
| Huawei | FDD bands should be based on request by operator. Also based on agreement in RAN#98e, co-existence study should be considered for the proposed FDD bands if any. |
| LGE | Support moderator proposal |
| vivo | There is only one FDD band discussed, i.e., n14. This issue can be confirmed with the operator. |
| Qualcomm | Discussion on FDD should be reopened only if operators have requested it |
| OPPO | WF is ok, but need to think about how to enable this concurrent operation. |

Issue 1-2-4: Intra-band V2X con-current operation in FDD band

| Company | Comments |
| --- | --- |
| Xiaomi | Option 2. Need more study. |
| CATT | Option 2. |
| Huawei | Option 2. |
| LGE | Option 2 based on operator request |
| vivo | Option 2. |
| Qualcomm | Option 2: More study is needed for con-current SL reception and Uu transmission in FDD band. |
| OPPO | Option 2 is ok, and the concurrent issue in TDD needs also to be considered here. |

Issue 1-2-5: Whether to send LS to RAN1 to indicate RAN4 agreements w.r.t contiguous and non-contiguous V2X con-current operation within a licensed TDD band.

| Company | Comments |
| --- | --- |
| Xiaomi | What is the purpose to send LS to RAN1. Our understanding is that currently, RAN1 doesn’t consider the SL RX and UL TX simultaneously. |
| CATT | Considering potential impact of RAN4 agreements on RAN1, we propose to send LS to indicate RAN4 agreements of contiguous and non-contiguous case. RAN1 should design mechanism on how to avoid simultaneous UL Tx and SL Rx, e.g. based on priority information of two RATs. |
| Huawei | We think that simultaneous Rx and Tx in the same band is not a viable scenario in Rel-17. In our view, no need to send LS to RAN1. |
| LGE | RAN4 still do study anything to support simultaneous UL Tx and SL reception vice versa. So no need to send LS to RAN1. |
| vivo | Option 1. If we decide to not allow the con-current SL Rx and UL, then the LS is needed. |
| Qualcomm | This needs to be discussed further |
| OPPO | Not needed for now. |

CRs/TPs comments collection

Major close-to-finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2112771 (TP on RF requirements for intra-band con-current V2X operation in licensed band) | Qualcomm: We need more time to review section 5.2.4.2.4 configured transmitted power for intra-band con-current V2X operation. do not agree wit the TDM same carrier time masks, figurese 5.2.3.3-1, 5.2.3.3-1 and 5.2.3.3-3. We should defer discussion on TDM operation wit h different carrier time mask until we get a RAN1 reply to the SL timing LS. Do not agree with the n79 REFSENS values in table 5.2.4.3.1-1 because it does not account for the near-far REFSENS degradation. Cannot agree with this TP |
| R4-2111943 (TP on intra-band V2X operation) |  |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Tentative agreements: Candidate options: Recommendations for 2nd round: |
| Issue 1-1-1: Switching time for same carrier and different carrier | In 1st round, 6 companies support option 1 and 1 company supports option 2, i.e. need further discussion. Actually option 1 is a general principle to deal with switching time. Option 1 only indicate separate consideration for cases of same carrier and different carriers rather than determine whether switching period apply to the same carrier case. Interested companies can further discuss switching period based on possible configuration changes for each specific case including same carrier in the next meeting. As moderator, I recommend to agree option 1 based on the majority view. Tentative agreements: Option 1: To consider switching time separately for cases of same carrier and different carriers. Candidate options: NONE Recommendations for 2nd round: No more discussion needed. |
| Issue 1-1-2: Switching time length | In 1st round, 6 companies support option 1 (140us as switching time) and 1 company supports option 2 (210us as switching time). Tentative agreements: NONE Candidate options: - Option 1: 140us switching time can be used as starting point for NR and NR sidelink switching for intra-band concurrent operation with different carriers in TDD band. - Option 2: 210us switching time can be used as starting point for NR and NR sidelink switching for intra-band concurrent operation with different carriers in TDD band. Recommendations for 2nd round: Further discuss switching time in 2nd round. |
| Issue 1-1-3: Switching time position | In 1st round, 6 companies think no impact on RAN1 if decide switching period position based on the priority rule defined in RAN1 (in which 1 company tend to further discuss this). 1 company think the impact on RAN1 should be considered. As moderator, I recommend to agree option 2 based on the majority view. Tentative agreements: - Option 2: No impact on RAN1 in case RAN4 decide the switching period position based on the priority rule defined in RAN1. Candidate options: NONE Recommendations for 2nd round: No more discussion needed. |
| Issue 1-1-4: Time mask for TDM with same carrier | Tentative agreements: In 1st round, 3 companies have concern on 8us+8us transient period for 60kHz SCS. 1 company would like to further discuss time mask based on configuration changes. Candidate options: - Option 1: RAN4 specify ON/OFF Time Mask for TDM operation in same carrier as proposed in paper R4-2112769 and R4-2111945. Recommendations for 2nd round: Further discuss switching time in 2nd round. |
| Issue 1-1-5: Time mask for TDM with different carriers | Tentative agreements: To specify switching time mask between Uu and SL, RAN4 need to wait for RAN1 reply LS and RRM scheduling restriction. Candidate options: - Option 1: To consider the time mask in Figure 4 for SL and Uu switching with different carriers without dual PA capability proposed in paper R4-2112769. Figure 4: Time mask for TDM operation in different carrier from NR SL/NR Uu to NR Uu/NR SL - Option 2: To consider the time mask in Figure 5 and Figure 6 for SL and Uu switching with different carriers without dual PA capability as proposed in paper R4-2111945. Figure 5: Time mask for NR SL (higher priority) switching to NR Uu without dual PA capability Figure 6: Time mask for NR Uu (higher priority) switching to NR SL without dual PA capability - Option 3: Switching from SL to NR: Recommendations for 2nd round: No more discussion needed in 2nd round. Wait for RAN1 reply LS on SL timing and RRM scheduling restrictions. |
| Issue 1-2-1: In-device coexistence | In 1st round, 6 companies support option 1 and 1 company think there is self-interference caused by simultaneous SL Rx and Uu Rx that have degraded REFSENS. Tentative agreements: NONE Candidate options: - Option 1: No in-device coexisting study means that it is not allowed the con-current reception of SL and Uu transmission for both adjacent and non-adjacent carrier cases in TDD band. - Option 2: To guarantee no in-device coexistence problem, simultaneous SL Rx and Uu Rx should be considered. Recommendations for 2nd round: Further check whether simultaneous SL Rx and Uu Rx need to be considered to ensure no in-device coexistence problem. |
| Issue 1-2-2: Non-adjacent carrier in TDD band | In 1st round, 5 companies support option 1, i.e. do not allow simultaneous UL Tx and SL Rx for non-contiguous V2X intra-band con-current operation like contiguous case. 1 company would like to hear the voice of operators and suggest studying the feasibility of simultaneous Rx/Tx capability in intra-band CA UE of NR Uu firstly. Tentative agreements: NONE Candidate options: - Option 1: Do not allow simultaneous UL Tx and SL Rx for non-contiguous V2X intra-band con-current operation like contiguous case. - Option 3: Study the feasibility of simultaneous Rx/Tx capability in intra-band CA UE of NR Uu firstly, and then SLV2X operation will consider simultaneous Rx/Tx Capability study results in NR Uu. Recommendations for 2nd round: Further check whether option 1 is acceptable in Rel-17 timeline. |
| Issue 1-2-3: Whether to consider intra-band V2X con-current operation in FDD band | Tentative agreements: Whether to consider intra-band V2X con-current operation for band n14 should be based on operator request. Candidate options: NONE Recommendations for 2nd round: No more discussion needed. |
| Issue 1-2-4: Intra-band V2X con-current operation in FDD band | Tentative agreements: Option 2. More study is needed for con-current SL reception and Uu transmission in FDD band if introduced. Candidate options: NONE Recommendations for 2nd round: No more discussion needed. |
| Issue 1-2-5: Whether to send LS to RAN1 to indicate RAN4 agreements w.r.t contiguous and non-contiguous V2X con-current operation within a licensed TDD band. | In 1st round, 4 companies think no need to send LS. 2 companies support to send LS and 1 company would like to further discuss this. Tentative agreements: NONECandidate options: - Option 1: Yes - Option 2: No - Option 3: Need further discussion Recommendations for 2nd round: Further discuss whether to send LS to RAN1 to indicate RAN4 agreement. |

Recommendations on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 | WF on intra-band V2X operation | CATT |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |
| R4-2112771 | To be revised. This TP can be revised based on potential agreements in 2nd round. Also, the overlapping parts with CATT TP R4-2111943 should be removed. |
| R4-2111943 | To be approved. |

Discussion on 2nd round (if applicable)

Issue 1-1-2: Switching time length

- Proposals

  - Option 1: 140us switching time can be used as starting point for NR and NR sidelink switching for intra-band concurrent operation with different carriers in TDD band.

  - Option 2: 210us switching time can be used as starting point for NR and NR sidelink switching for intra-band concurrent operation with different carriers in TDD band.

- Recommended WF

  - Further discuss switching time in 2nd round.

Issue 1-1-4: Time mask for same carrier

- Proposals

  - Option 1: RAN4 specify ON/OFF Time Mask for TDM operation in same carrier as proposed in paper R4-2112769 and R4-2111945.

Figure 1: Time mask for NR Uu switching to NR SL

Figure 2: Time mask for NR SL switching to NR Uu (15kHz/30kHz SCS)

Figure 3: Time mask for NR SL switching to NR Uu (60kHz SCS)

- Recommended WF

  - Further discuss switching time in 2nd round.

Issue 1-2-1: In-device coexistence

- Proposals

  - Option 1: No in-device coexisting study means that it is not allowed the con-current reception of SL and Uu transmission for both adjacent and non-adjacent carrier cases in TDD band.

  - Option 2: To guarantee no in-device coexistence problem, simultaneous SL Rx and Uu Rx should be considered.

- Recommended WF

  - Further check whether simultaneous SL Rx and Uu Rx need to be considered to ensure no in-device coexistence problem.

Issue 1-2-2: Non-adjacent carrier in TDD band

- Proposals

  - Option 1: Do not allow simultaneous UL Tx and SL Rx for non-contiguous V2X intra-band con-current operation like contiguous case.

  - Option 3: Study the feasibility of simultaneous Rx/Tx capability in intra-band CA UE of NR Uu firstly, and then SLV2X operation will consider simultaneous Rx/Tx Capability study results in NR Uu.

- Recommended WF

  - Further check whether option 1 is acceptable in Rel-17 timeline.

Issue 1-2-5: Whether to send LS to RAN1 to indicate RAN4 agreements w.r.t contiguous and non-contiguous V2X con-current operation within a licensed TDD band.

- Proposals

  - Option 1: Yes

  - Option 2: No

  - Option 3: Need further discussion

- Recommended WF

  - Further discuss whether to send LS to RAN1 to indicate RAN4 agreement.

Companies views’ collection for 2nd round

Open issues

Issue 1-1-2: Switching time length

| Company | Comments |
| --- | --- |
| LGE | LGE prefer option 1 as mentioned in email discussion. RAN4 already agreed with 150us (120+20+10) between LTE V2X (20us transient period) and NR V2X (10 us transient period) . So, LGE prefer to apply same principle to determine the NR V2X to NR Uu will be considered with 140us (120+10+10) for NR-NR in a licensed band and. |
| Xiaomi | We are ok with the 140us switching time. |
| CATT | Prefer option 1. From RF perspective, it would be better to define a certain switching time given no RRM scheduling availability requirement. RRM session is waiting for RF decision on switching time that will be a basis to define scheduling availability. Actually whether and how to define scheduling should be left to RRM session. If RRM follows the same scheduling availability defined in Rel-16, maybe the switching time will not be tested as what we did in Rel-16. |
| Qualcomm | Opton3: We cannot agree to a switching time spec until we know what event determines the start of switching. Is the advanced scheduling time, like that given in 38.214, section 6.4, required to enable switching a part of the switching time spec? Do we have a SL requirement for how much time is allocated for advanced scheduling? We think that these issues need to be clarified to get an accurate switching time spec. |
| Huawei, HiSilicon | Option 1.The similar issue has been discussed in Rel-16. Based on the agreed value for switching between LTE V2X and NR V2X, we see 140us is a reasonable value. |
| Vivo | Prefer Option 1. In Rel-16 we had a lot of discussion on the switching time, we think 120us can be reused for Uu and SL switching too. |

Issue 1-1-4: Time mask for same carrier

| Company | Comments |
| --- | --- |
| LGE | LGE prefer option 1 as it is since the transient period is 8us can protect the 13th symbol of SL UE for 60kHz SCS. When RAN4 consider the 10us +10 us, the switching time will be impacted to the 13th symbol of SL for 60kHz SCS. |
| Xiaomi | As even in the same carrier, the TDM operation still needs to wait for RAN1 LS about the timing alignment issue. Maybe we can wait for the reply LS from RAN1. |
| CATT | Option 1. We support the time mask for 15kHz/30kHz SCS. For 60kHz SCS, RAN4 can further study the transient period if companies have concern. |
| Qualcomm | Option 2: The fast switching (<1 symbol) time masks being developed for TDM intra-band switching with same carrier should be deemed optional. UE’s that do not support these time masks should be allowed to use the slower switching time masks for TDM intra-band with different carriers, being developed in issue 1-1-5, when performing TDM intra-band same carrier switching. |
| Huawei, HiSilicon | Option 1 is ok for us as starting point, but transient period is determined by UE hardware implementation, which is not relevant to the supported SCS. We don’t think the time mask should be differentiated among different SCS. |

Issue 1-2-1: In-device coexistence

| Company | Comments |
| --- | --- |
| LGE | Currently, we prefer option 1. It also related to the Issue 1-2-2. So the 3rd priority can be defer in REl-18. |
| CATT | Option 1. Based on previous RAN4 agreements, separate RF chain architecture is used for intra-band con-current operation. With separate RF chain, no simultaneous Rx/Rx problem is expected. |
| Qualcomm | Option2: The larger RX signal may act as a jammer for the smaller RX signal. This is the near far problem that exists in Uu concurrent operation. In Uu the PSD difference of the 2 RX signals is limited to 6 dB. However, in Uu/SL coexistence it is not possible to maintain this difference due to the potential non co-location of the Uu and SL sources. Even if 2 separate RX chains are used each will receive both signals and the larger signal will still act as a jammer for the smaller signal. |
| Huawei, HiSilicon | Option 1. The issue is also related to issue 1-2-2, they can be considered together. |
| vivo | Option 1. |

Issue 1-2-2: Non-adjacent carrier in TDD band

| Company | Comments |
| --- | --- |
| LGE | LGE prefer option 3, we are proponent of this option. For FDM operation with non-adjacent carrier in a licensed band, we suggest to treat the simultaneous Rx/Tx capability in intra-band CA UE in NR Uu firstly, then SL can consider this simultaneous Rx/Tx Capability based on the NR Uu study results. |
| CATT | Option 1. Considering 3rd priority and Rel-17 timeline, it would be helpful to not allow simultaneous UL Tx and SL Rx at the current stage. To LGE, I assume LGE is also OK with option 1 for Rel-17 and RAN4 could consider simultaneous Rx/Tx based on NR Uu study results in later release? LGE : Yes, right. In SL WI, option 1 due to lack of time. Then NR Uu can study the simultaneous Rx/Tx capability in licensed band. So RAN4 consider both for the way forward. |
| Qualcomm | Option 1: Do not allow simultaneous UL TX and SL RX for non-contiguous V2X intra-band con-current operation like contiguous case. |
| Huawei, HiSilicon | Option 1. This issue may not be determined by RAN4. Whether to have full duplex study in Rel-18 time frame should be a RAN level discussion. |
| vivo | Option 1. |

Issue 1-2-5: Whether to send LS to RAN1 to indicate RAN4 agreements w.r.t contiguous and non-contiguous V2X con-current operation within a licensed TDD band.

| Company | Comments |
| --- | --- |
| LGE | Option 2 |
| Xiaomi | Not sure what is the RAN4 agreement that needs to be sent to RAN1. |
| CATT | Option 1. OK with option 3. |
| Qualcomm | Option 3 |
| Huawei, HiSilicon | Similar view as Xiaomi. |
| vivo | Option 1. If we decide not to allow con-current Uu Tx and SL Rx, this may restrict SL allocation in the UL resources. In this sense, it is OK to send the LS. |

CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2114982 (WF on intra-band V2X operation) | LGE: It will be updated based on RAN4 consensus. |
|  | CATT: The WF will be updated based on RAN4 agreements. |
|  | Qualcomm: WF will be updated based on RAN4 agreements |
|  | Huawei: WF should be updated also consider the comments in 2nd round discussion. |
| R4-2114984 (revision of R4-2112771, TP on RF requirements for intra-band con-current V2X operation in licensed band ) | LGE: We can update the On/off time mask according to RAN4 consensus. But it is depend on RAN1 reply LS. |
|  | Qualcomm: On/off time masks for same carrier switching should be made optional so that UEs that do not support this feature can use the slower switching time based on different carrier switching to perform same carrier switching. To QC from LGE: it is not clear to me what do you think the switching scenarios? Whtadoes mean the slower switching time. RAN4 can make switching time mask for two cases. 1st one is same carrier TDM operation. And 2nd is the different carrier for TDM operation. It will be distinguished in TS38.101-1. |

Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |
| R4-2114982 (WF on intra-band V2X operation) | Approved |
| R4-2114984 (revision of R4-2112771, TP on RF requirements for intra-band con-current V2X operation in licensed band ) | Approved |

Topic #2: Synchronous operation between SL and Uu

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2114505 | Huawei, HiSilicon | Title: Further consideration on SL timing alignment Observation 1: SL transmission timing is aligned with UL timing is supported for LTE D2D, which could be a reference for NR SL to avoid interference between SL and Uu in licensed bands. Proposal 1: For sidelink transmissions, SL transmission timing should be aligned with Uplink timing when Uu and sidelink is TDMed/FDMed coexistence in the same band, especially for the same carrier scenario. |
| R4-2112769 | LG Electronics France | Title: RF requirements for intra-band con-current V2X operation with NR PC5 and NR Uu in a licensed band Proposal 3: RAN4 can concluded that 15us TA time difference as worst case between NR SL transmission with 13us NTA offset in n79 ISD 500m cell. The all SCS waveform do not have any interference problem in its own device based on current RAN1 & RRM agreement. Proposal 4: For the SL transmission time alignment, RAN4 can keep the current RAN1 & RRM agreements as specified in section 12.2.3 in TS38.133. |
| Rev_R4-2112604 | Xiaomi | Title: Further discussion on FDM intra-band concurrent operation Observation 1: The guard period is designed for sidelink TX and RX switching. Observation 2: The guard period should both cover the transient period and the TTA. Observation 3: For SCS as 15kHz and 30kHz the guard period is long enough, however, for SCS as 60kHz, 2 symbols might be needed . Observation 4: Currently RRM session is considering decreasing the NTA_offset to keep the guard period unchanged as one symbol. Observation 5: For case 2 as sidelink aligned to UL, the guard period is long enough to cover the transient period. Proposal 1: Wait for the RRM session discussion as well as the RAN1 reply LS to finalize the guard period discussion. |

Open issues summary

Based on above contributions, the following sub-topics and issues regarding synchronous operation between SL and Uu are summarized.

- Sub-topic 2-1: SL transmission timing

- Issue 2-1-1: Timing advance

- Issue 2-1-2: SL transmission timing

Sub-topic 2-1: SL transmission timing

Issue 2-1-1: Timing advance

- Proposals

  - Option 1: RAN4 can concluded that 15us TA time difference as worst case between NR SL  transmission with 13us NTA offset in n79 ISD 500m cell. The all SCS waveform do not have any interference problem in its own device based on current RAN1 & RRM agreement.

  - Option 2: 15us TA time difference between NR SL transmission with 13us NTA offset in n79 ISD 500m cell cannot be considered as worst case.

- Recommended WF

  - Need more discussion.

Issue 2-1-2: SL transmission timing

- Proposals

  - Option 1: For sidelink transmissions, SL transmission timing should be aligned with Uplink timing when Uu and sidelink is TDMed/FDMed coexistence in the same band, especially for the same carrier scenario.

  - Option 2: For the SL transmission time alignment, RAN4 can keep the current RAN1 & RRM agreements as specified in section 12.2.3 in TS38.133.

  - Option 3: Wait for RAN1 reply LS to finalize SL transmission timing.

- Recommended WF

  - Need more discussion.

Companies views’ collection for 1st round

Open issues

Issue 2-1-1: Timing advance

| Company | Comments |
| --- | --- |
| Xiaomi | Option 2. The worst case is based on ISD. However, this is a typical simulation characteristics but not the real deployment case. If the cell size is larger the ISD, then the worst case for current ISD is not suitable. |
| CATT | Option 2. Share the similar view with Xiaomi. In the real field, 500m ISD is not a worst case to decide TA. |
| Huawei | Option 2. The guard period is not considered only for 500m ISD case. |
| LGE | Option 1, n79 is only V2X band to support intra-band con-current operation. The n79 cell of above 4.5GHz is quite small coverage compare to 2GHz or 3.8GHz cell range. So this approach is reasonable. What is worst scenarios in n79 for the TA perspective? |
| Qualcomm | Option 2: It is possible that 500m ISD is the worst case to decide TA |

Issue 2-1-2: SL transmission timing

| Company | Comments |
| --- | --- |
| Xiaomi | Option 3. Wait for RAN1 reply LS. For current interference issue, we believe there can be multiple solutions, e.g. UL timing alignment, extending the guard period and scheduling restriction. Either of them has pros and cons. As we are still waiting for RAN1 reply LS, we just want us to keep in mind that other solutions exist. |
| CATT | Option 3. |
| Huawei | Our preference is option 1. RAN1 evaluation is one aspect, but RAN4 can also have separate discussion based on technical analysis. |
| LGE | Original preference is option2. We are OK to wait the RAN1 reply LS to finalize the timing alignment between NR SL and NR Uu. |
| vivo | Option 3. |
| Qualcomm | Option 3 |

CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Tentative agreements: Candidate options:Recommendations for 2nd round: |
| Issue 2-1-1: Timing advance | Tentative agreements: NONE Candidate options: - Option 1: RAN4 can concluded that 15us TA time difference as worst case between NR SL transmission with 13us NTA offset in n79 ISD 500m cell. The all SCS waveform do not have any interference problem in its own device based on current RAN1 & RRM agreement. - Option 2: Ran4 further discuss the NTA,SL values in n79 in real field. And NTA_offset will reuse the NR Uu in n79 for NR SL operation. - Option 3: other options are not precluded Recommendations for 2nd round: RAN4 could continue to discuss the worst case of ISD to determine TA. |
| Issue 2-1-2: SL transmission timing | In 1st round, 5 companies support option 3, i.e. wait for RAN1 reply LS to finalize SL transmission timing, and 1 company support option 1. As moderator, I recommend to agree option 3 based on the majority view. Tentative agreements: - Option 3: Wait for RAN1 reply LS to finalize SL transmission timing. Candidate options: NONE Recommendations for 2nd round: No more discussion needed in 2nd round. |

Suggestion on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

Discussion on 2nd round (if applicable)

Issue 2-1-1: Timing advance

- Proposals

  - Option 1: RAN4 can concluded that 15us TA time difference as worst case between NR SL  transmission with 13us NTA offset in n79 ISD 500m cell. The all SCS waveform do not have any interference problem in its own device based on current RAN1 & RRM agreement.

  - Option 2: RAN4 further discuss the NTA,SL values in n79 in real field. And NTA_offset will reuse the NR Uu in n79 for NR SL operation.

  - Option 3: Other options are not precluded.

- Recommended WF

  - RAN4 continue to discuss the worst case of ISD to determine TA.

Companies views’ collection for 2nd round

Open issues

Issue 2-1-1: Timing advance

| Company | Comments |
| --- | --- |
| LGE | Option1 and NTA_offset will reuse the NR Uu in n79 for NR SL operation based on RAN1 specification. Option 2 is still mis-aligned RAN1 specification for the NTA,SL = 0 in licensed band. To All companies, LGE can agreeable to consider the real level for NTA,SL, But currently RAN1 only considered NTA,SL = 0 for both licensed band and unlicensed bands. Only RAN4 can consider NTA_offset in licensed band. Do you have any concern for the this view? |
| Xiaomi | For NTA, as discussed in first round, the ISD 500m cannot stands for worst case. For NTA_offset, this is the RRM session discussion. Also we don’t see the necessity to discuss the worst case of cell size, NTA should follow the real cell size case. Otherwise, the slot are still not aligned. |
| CATT | Option 2. NTA,SL is determined by cell size. 500m ISD is not the maximum distance in real field. It is not justified to consider 500 ISD as the worst case. |
| Huawei, HiSilicon | Option 2. NTA,SL should not be determined only by typical simulation scenario. |

CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |

Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

Topic #3: Other RF requirements

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2112769 | LG Electronics France | Title: RF requirements for intra-band con-current V2X operation with NR PC5 and NR Uu in a licensed band Proposal 5: Based on Table 4-1, RAN4 define the detailed RF requirements for intra-band con-current V2X operation in TDD band. Proposal 6: Based on Table 4-2, 4-3 and 4-4, RAN4 define the REFSENS requirements for intra-band con-current V2X operation in n79. |
| R4-2112284 | Qualcomm Incorporated | MPR specifications for intra-band con-current V2X operation Proposal 1: MPR development outlined in section 6.2B.2 of 38.101-3 [2] for cellular EN-DC shall be reused without modification for V2X intra-band con-current operation where separate RF chains are used for Uu and SL Proposal 2: The MPR values given in 38.101-3 [2] for cellular EN-DC shall be adopted for intra-band con-current V2X operation where separate RF chains are used for Uu and SL. |
| R4-2112341 | LG Electronics | TP on MPR for NR V2X intra-band con-current operation with Uu |
| R4-2113410 | Huawei, HiSilicon | Discussion on MPR requirements for intra-band con-current V2X operation Observation 1: Bandwidth class is a concept for carrier aggregation and are not applicable for Uu and SL intra-band con-current operation. Observation 2: Considering the worst case, the 40MHz maximum CBW for SL and 100MHz maximum CBW for n79 can be evaluated when RAN4 specify the MPR requirements. Proposal 1: Separate two RF chain architecture should be clarified in the specification for Uu and SL intra-band con-current operation. Observation 3: For Uu and SL intra-band con-current operation, the MPR requirements in clause 6.2.2 apply for NR Uu operation in licensed band, and the MPR requirements in in clause 6.2E.2 apply for NR sidelink operation can be baseline. Proposal 2: To specify 2dB MPR for 23dBm 64QAM, 4dB MPR for 23dBm 256QAM, 4.5dB MPR for 26dBm 64QAM and 7dB MPR for 26dBm 256QAM, no matter what the RB allocation is. Observation 4: it isn’t so urgent to specify the MPR requirements for FDM mode in licensed band. Companies can further study the values. |
| R4-2114589 | Qualcomm Incorporated | MPR specifications for V2X intra-band con-current operation Proposal 1: MPR development outlined in section 6.2B.2 of 38.101-3 [2] for cellular EN-DC shall be reused without modification for V2X intra-band con-current operation where separate RF chains are used for Uu and SL Proposal 2: The MPR values given in 38.101-3 [2] for cellular EN-DC shall be adopted for V2X intra-band con-current operation where separate RF chains are used for Uu and SL. (Same content with R4-2112284) |

Open issues summary

Based on above contributions, the following sub-topics and issues on RF requirements for intra-band con-current operation are summarized.

- Sub-topic 3-1: MPR for intra-band V2X con-current operation

- Issue 3-1-1: MPR

- Issue 3-1-2: Whether to clarify separate two RF chain architecture in the specification for Uu and SL intra-band con-current operation.

- Sub-topic 3-2: Other RF requirements for intra-band con-current V2X operation

- Issue 3-2-1: REFSENS

Sub-topic 3-1: MPR for intra-band V2X con-current operation

Issue 3-1-1: MPR

- Proposals

  - Option 1: Consider MPR requirements for intra-band V2X con-current operation proposed in paper R4-2112341

  - Option 2: Specify 2dB MPR for 23dBm 64QAM, 4dB MPR for 23dBm 256QAM, 4.5dB MPR for 26dBm 64QAM and 7dB MPR for 26dBm 256QAM, no matter what the RB allocation is.

  - Option 3: The MPR values given in 38.101-3 [2] for cellular EN-DC shall be adopted for V2X intra-band con-current operation where separate RF chains are used for Uu and SL.

- Recommended WF

  - Need more discussion.

Issue 3-1-2: Whether to clarify separate two RF chain architecture in the specification for Uu and SL intra-band con-current operation.

- Proposals

  - Option 1: Yes

  - Option 2: No

- Recommended WF

  - Need more discussion.

Sub-topic 3-2: Other RF requirements for intra-band con-current V2X operation

Issue 3-2-1: REFSENS

- Proposals

  - Option 1: To adopt the REFSENS requirements for intra-band con-current V2X operation proposed in paper R4-2112769.

- Recommended WF

  - Need more discussion.

Companies views’ collection for 1st round

Open issues

Issue 3-1-1: MPR

| Company | Comments |
| --- | --- |
| Huawei | Option 2: For 64QAM and 256QAM, the proposed values in WF R4-2103246 are not enough to meet the MPR requirements for single Uu or SL carrier. For 64QAM and 256QAM, the MPR requirements are limited by EVM requirements. So it’s understandable that MPR values for 64QAM and 256QAM are not relative to the RB allocations. |
| LGE | There was any evidence to relax the MPR results in HW paper. The relaxation will be allowed by reasonable simulation data. So still prefer to keep the previous RAN4 agreement as Option 1 |
| Qualcomm | Option 3: We feel that for consistency with current specification the proposed MPRs should not be more stringent than what exists currently in EN-DC. It is reasonable to assume that the same hardware will be used for EN-DC as for V2X con-current operation so the MPR specs should be the same. |

Issue 3-1-2: Whether to clarify separate two RF chain architecture in the specification for Uu and SL intra-band con-current operation.

| Company | Comments |
| --- | --- |
| Xiaomi | Option 1. This has already been used for PC1.5 with a note that dual RF chain to be indicated for requirements. |
| Huawei | Option 1. Current requirements are based on the separate two RF chain assumption. If the assumption is changed, the requirements should be reevaluated. |
| LGE | Same with Huawei, we can capture the detail RF architecture if needed. But TR also will be captured the RF architectures in our TP. |
| vivo | Option 1. |
| Qualcomm | Option 2: If we adopt option 3 for Issue 3-1-1 then no clarification is necessary as it is already defined as 2 PAs |
| OPPO | Option 1. |

Issue 3-2-1: REFSENS

| Company | Comments |
| --- | --- |
| LGE | Support the REFSENS requirements |
| Qualcomm | We think that the n79 REFSENS numbers given in R4-2112769 do not account for the near-far issue where one carrier will act as a jammer or the other RX band. This jamming will degrade the REFSENS numbers over and above what is given in table 4-2. How to account for this degradation needs further discussion |

CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2112341 (TP on MPR for NR V2X intra-band con-current operation with Uu) | Huawei: The MPR requirements for 64QAM and 256QAM are not enough. The proposal 2 in R4-2113410 should be considered. |
|  | LGE: If HW want to revise the MPR requirements, then need to provide MPR simulation results as evidence. Currently, we could not find any evidence. So we prefer to keep the ageed MPR levels in WF at last RAN4 meeting. |
|  | Qualcomm: We think that for consistency we should follow the methodology and the MPRs already established for EN-DC that are in the current spec |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Issue 3-1-1: MPR | Tentative agreements: NONE Candidate options: - Option 1: Consider MPR requirements for intra-band V2X con-current operation proposed in paper R4-2112341 - Option 2: Specify 2dB MPR for 23dBm 64QAM, 4dB MPR for 23dBm 256QAM, 4.5dB MPR for 26dBm 64QAM and 7dB MPR for 26dBm 256QAM, no matter what the RB allocation is. - Option 3: The MPR values given in 38.101-3 [2] for cellular EN-DC shall be adopted for V2X intra-band con-current operation where separate RF chains are used for Uu and SL. Recommendations for 2nd round: Further discuss MPR in 2nd round. |
| Issue 3-1-2: Whether to clarify separate two RF chain architecture in the specification for Uu and SL intra-band con-current operation. | In 1st round, 5 companies support option 1 and 1 companies support option 2. As moderator, I recommend to agree option 1 based on the majority view. Tentative agreements: - Option 1: Yes, to clarify separate two RF chain architecture in the specification for Uu and SL intra-band con-current operation. Candidate options: NONE Recommendations for 2nd round: No more discussed needed. |
| Issue 3-2-1: REFSENS | Tentative agreements: NONE Candidate options: - Option 1: To adopt the REFSENS requirements for intra-band con-current V2X operation proposed in paper R4-2112769. Recommendations for 2nd round: Further discuss REFSENS requirements. |

Suggestion on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 | WF on MPR for intra-band con-current operation | LG Electronics |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |
| R4-2112341 | To be returned. |

Discussion on 2nd round (if applicable)

Issue 3-1-1: MPR

- Proposals

  - Option 1: Consider MPR requirements for intra-band V2X con-current operation proposed in paper R4-2112341

  - Option 2: Specify 2dB MPR for 23dBm 64QAM, 4dB MPR for 23dBm 256QAM, 4.5dB MPR for 26dBm 64QAM and 7dB MPR for 26dBm 256QAM, no matter what the RB allocation is.

  - Option 3: The MPR values given in 38.101-3 [2] for cellular EN-DC shall be adopted for V2X intra-band con-current operation where separate RF chains are used for Uu and SL.

- Recommended WF

  - Further discuss MPR in 2nd round.

Issue 3-2-1: REFSENS

- Proposals

  - Option 1: To adopt the REFSENS requirements for intra-band con-current V2X operation proposed in paper R4-2112769.

- Recommended WF

  - Further discuss REFSENS requirements.

Companies views’ collection for 2nd round

Open issues

Issue 3-1-1: MPR

| Company | Comments |
| --- | --- |
| LGE | Based on the WF contents, the MPR will be revised based on the detail simulation results for intra-band V2X con-current operation. So option 1 is feasible at this stage. For the option 3 is not applicable to intra-band V2X con-current UE since RAN4 shall need to consider the separate specific channel assignment and allocation for S-SSB transmission, PSFCH multiplexing and PSSCH/PSCCH multiplexing mechanism to derive MPR requirements. |
| Qualcomm | Option 3: The MPR values given in 38.101-3 [2] for cellular EN-DC should be adopted for V2X intra-band con-current operation where separate RF chains are used for Uu and SL. For the MPR values in R4-2112341 we find that for PC3 there is no significant MPR difference between QPSK and 256 QAM. This is an issue as the higher order modulations have to meet EVM/ACLR numbers. Even for PC2 the MPR values are generally smaller than for Uu standalone. We feel that while it is true that the bandwidth class is concept for carrier aggregation, from RF perspective Uu and SL intra-band con-current operation still mimic carrier aggregation and the same method used to come up with MPR for carrier aggregation should be applied to Uu and SL intra-band con-current operation. We would like to ask the proponents of option1 and option2 if these results have been verified using actual hardware. |
| Huawei, HiSilicon | Our proposal is option 2. We disagree with that MPR for EN-DC could be reused for SL since the physical channel, RB resource allocation are different from EN-DC. Given the current situation, we are fine to have further study of MPR. |

Issue 3-2-1: REFSENS

| Company | Comments |
| --- | --- |
| LGE | Option 1. For the QC comment, in TDD band, RAN4 already agreed to apply the REFSENS equation in TR38.886 for TDD band. So REFSENS will be derived the equation in n79. And the difference between NR UE and NR SL is quite small. So it can be achievable REFSENS levels. |
| Qualcomm | Option 2: We think that the n79 REFSENS numbers given in R4-2112769 do not account for the near-far issue where one carrier will act as a jammer or the other RX band. This jamming will degrade the REFSENS numbers over and above what is given in table 4-2. How to account for this degradation needs further discussion. Our understanding is that this same issue exists in Uu, but in Uu the PSD difference between the 2 RX signal is limited to 6 dB, but this constraint cannot be enforced as Uu and SL are not co-located. Even if 2 receivers are used the same issue will be present with the larger RX signal jamming the smaller one. To QC from LGE: REFSENS is verified in conducted test mode with UL configuration. So why RAN4 need to consider near-far issue. RAN4 did not consider the jamming problems as you mentioned. It is quite big issue to change the principle. |
| Huawei, HiSilicon | Option 1 is ok for us. For the near-far issue mentioned by QC, we think that should not be considered for the REFSENS requirement. Whether there should be some constraints for the test can be further discussed. |

CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2114983 (WF on MPR for intra-band con-current operation) | Qualcomm: cannot agree with this WF: We believe that the EN-DC MPR values and methodologies used to develop these MPRs should be used. We select option 3 as the proposed WF. |
|  | LGE: QC’s proposal is not feasible to the 5G V2X UE as LGE, HW mentioned. Why QC insist to apply the EN-DC MPR value for V2X con-current UE. These was any reasonable evidence for QC proposal. So, it can be approved and RAN4 can further discuss based on the MPR simulation results based on the agreed RF simulation assumptions. |
| R4-2112341 (TP on MPR for NR V2X intra-band con-current operation with Uu) | Qualcomm: cannot agree with this TP: We believe that the EN-DC MPR values and methodologies used to develop these MPRs should be used. We select option 3 as the proposed WF. |
|  | LGE: same comment to QC. You need to provide simulation results based on the agreed RF simulation assumptions. |

Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |
| R4-2114983 (WF on MPR for intra-band con-current operation) | Approved |
| R4-2112341 (TP on MPR for NR V2X intra-band con-current operation with Uu) | Approved |

Recommendations for Tdocs

1st round

New tdocs

| Title | Source | Comments |
| --- | --- | --- |
| WF on … | YYY |  |
| LS on … | ZZZ | To: RAN_X; Cc: RAN_Y |
| WF on intra-band V2X operation | CATT |  |
| WF on MPR for intra-band con-current operation | LG Electronics |  |

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-210xxxx | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-2112771 | TP on RF requirements for intra-band con-current V2X operation in licensed band | LGE | To be revised | This TP can be revised based on potential agreements in 2nd round. Also, the overlapping parts with CATT TP R4-2111943 should be removed. |
| R4-2111943 | TP on intra-band V2X operation | CATT | To be approved |  |
| R4-2112341 | TP on MPR for NR V2X intra-band con-current operation with Uu | LGE | To be returned |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

2nd round

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-210xxxx | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-2114982 | WF on intra-band V2X operation | CATT | Approved |  |
| R4-2114983 | WF on MPR for intra-band con-current operation | LG Electronics | Approved |  |
| R4-2114984 | TP on RF requirements for intra-band con-current V2X operation in licensed band | LG Electronics | Approved |  |
| R4-2112341 | TP on MPR for NR V2X intra-band con-current operation with Uu | LG Electronics | Approved |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents

- Annex

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| Vivo | Shuai Zhou | shuai.zhou@vivo.com |
| CATT | Yuan Gao | gaoyuan@catt.cn |
| LGE | Suhwan Lim | Suhwan.lim@lge.com |
| Huawei | Ye Liu | leo.liuye@huawei.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)