3GPP TSG-RAN WG4 Meeting # 100-bis-e 												R4-2115208

Electronic Meeting, 16th – 27th August, 2021

Agenda item:			9.9.4.1, 9.9.4.2, 9.9.4.3

Source:	Moderator (Nokia, Nokia Shanghai Bell)

Title:	Email discussion summary for [100-e][218] NR_HST_FR2_RRM_1

Document for:	Information

# Introduction

Briefly introduce background, the scope of this email discussion (e.g. list of treated agenda items) and provide some guidelines for email discussion if necessary.

List of candidate target of email discussion for 1st round and 2nd round

- 1st round: TBA

- 2nd round: TBA

## Background and scope

This T-doc will be used to guide and summarize the email discussion for the topic of Rel-17 NR HST FR2 enhancements RRM core requirements, with the email thread identifier “[100-e][218] NR_HST_FR2_RRM_1”.

In this email thread, the following agenda items will be discussed:

- 9.9.4.1 General

- 9.9.4.2 Number of RX beams

- 9.9.4.3 Idle/Inactive and connected state mobility requirements

In the previous RAN4#99-e meeting, the discussion about RRM requirements for HST operation in FR2 has continued. The discussions are summarized in [R4-2108395], and the ways forward were captured in [R4-2108342].
Between the significant agreements at RAN4#99-e were:

- Use 350kmph as a reference maximum train speed and define RRM requirements to guarantee that.

- Add a flag to enable the UE to identify different/enhanced RRM requirements in HST FR2 deployment.

- RRC Release with Redirection: not applicable for FR2 HST

- Cell reselection requirements in IDLE/INACTIVE mode are enhanced

The important topic with a considerable impact on the RRM requirements is still the number of RX beam. The discussion shall continue in parallel in Deployments and RRM threads.

In general, there are still many open issues regarding the details of RRM requirements. At the same time, with more clarity in the deployment scenarios, the issues are getting more concrete options such as the values of scaling factors in the requirements, the upper bounds of DRX, etc.

As a moderator for FR2 HST enhancements RRM discussion, we would like to suggest the following candidate target of 1st and 2nd round email discussion:

- 1st round: Further discuss of the number of RX beams and their impact on the requirements. Going deeper into the details of CONNECTED/IDLE state mobility requirements. Discussion of network signalling needed to support and enhance HST FR2 requirements.

- 2nd round: Based on the results from the 1st round, identify a few issues that have the potential to achieve agreements and discuss them further. Achieve agreements as much as possible.

## Email discussion guidelines

Unless different guidance is received from the session chairs, the moderator would like to ask companies to adhere to the following guidelines, when taking part in [100-e][218] NR_HST_FR2_RRM_1.

Please also check the “RAN4#100-e meeting arrangements and guidelines”, available on the reflector, for fundamental guidelines and deadlines.

The preferred method of commenting is to add/update your company’s view directly in this email summary document (use change marks if appropriate) and upload it to [100-e][218] NR_HST_FR2_RRM draft folder.

- Draft folder:
[100-e][218] NR_HST_FR2_RRM_1
https://www.3gpp.org/ftp/tsg_ran/WG4_Radio/TSGR4_100-e/Inbox/Drafts/%5B100-e%5D%5B218%5D%20NR_HST_FR2_RRM_1

- It is expected that delegates will download the latest version (including other companies’ versions) of the summary document, insert comments and upload it again.

- To ensure the comments are captured timely and correctly, delegates are encouraged to:

  - Rename the file by adding your company name and changing the file version.
Example: “Summary_218_HST_FR2_RRM_1_1st_round_v05_CATT_Nokia.docx” -> “Summary_218_HST_FR2_RRM_1_1st_round_v06_Nokia_QC.docx”

  - The companies can send an email on the reflector informing that comments are made specifying the updated file name. However, it is not recommended by the latest meting guidelines.

  - Please check for possibly updated base document versions, right before uploading your updates.

- Please do not hesitate to mark your company as supporting a certain option directly in this document.
Please refrain from rewriting existing options and proposed WFs; ask the moderator (in your company’s comment) to modify/add.

- It is encouraged to give a short reasoning for each view expressed (1-2 sentences are recommended).
Please avoid statements like “Option X”, without further explication or reasoning.

- New in this meeting: Please, update your company contact information in the Annex.
It is also recommended to explicitly mention delegate’s name next to company name in the comments if multiple delegates from the same company are commenting.
Another way is to identify different delegates in the Track changes username.

# Topic #1: General

Main technical topic overview. The structure can be done based on sub-agenda basis.

In the previous RAN4#99-e meeting the discussion in this topic mainly focused on signalling of deployment types and UE capabilities.  This meeting, a similar list of sub-topics can be defined based on company contributions. Additionally, a LS on UE capability and network signalling was submitted that require a separate consideration.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2112498 | CMCC | Discussion on general RRM requirements for FR2 HST Observation 1: both connected mode enhancement and IDLE/INACTIVE mode enhancement are considered for FR2 HST Proposal 1: the network flag to enable the UE to identify different/enhanced RRM requirements in HST FR2 deployment is proposed to be a cell specific signaling, similar as the network assistance signaling (highSpeedMeasFlag-r16) introduced in Rel-16 NR HST WI. Proposal 2: whether to introduce network signaling of uni-/bi-directional operation depends on whether same or different RRM enhanced requirements are specified for these two deployment operations, and can be discussed after there is conclusion on the detailed RRM enhanced requirements. Proposal 3: it is not necessary to introduce UE capability to indicate the support of FR2 HST. Proposal 4: it is not necessary to introduce CPE indication of support of uni-/bi-directional operation. Proposal 5: it is not preferred to have restriction on SMTC periodicity for FR2 HST. [Moderator]: Treated in Topic #3. |
| R4-2113213 | ZTE Corporation | General RRM requirements for HST FR2 Proposal 1: Only roof-mounted CPE is considered that should always have a capability to work in HST FR2 scenario in this WI. Proposal 2: Wait for the conclusion of CPE beam number for Scenario B in Scenario session. Proposal 3: Considering multiple differences exist between uni- and bi-directional deployments, different capabilities are needed if different requirements are agreed. |
| R4-2113326 | Ericsson | General requirements impacted for HST FR2 Proposal 1: Support Option 1 or 1a, only roof-mounted CPE is considered that should always have a capability to work in HST FR2 scenario Observation 1: According to current discussions including Te, scaling factor and so on, differentiation of uni-/bi-directional deployment may be beneficious to fast and accurate measurements, signaling and mobility. Observation 2: Another issue we have proposed from last meeting is that occasional direction change may occur in Uni-directional deployment. The phenomenon influents clear definition of Uni- and Bi-directional deployment from measurement point of view in worst case. Proposal 2: Two options of possible RRM requirements definition options are possible to be utilized in Uni-/bi-directional deployments. To answer the question of NW signaling and CPE indication, choice has to be made between the two options firstly. Option1: All RRM requirements are defined uniformly with maximal RX beam sweep number to cover Bi-directional deployments and Uni-directional deployments both. Option2: RRM requirements are defined differently for Bi-directional deployments and Uni-directional deployments after we can conclude how to handle below three issues: - Deployments may change dynamically, it needs operators’ studies and confirmation - Is occasional direction change in Uni-directional deployment treated as a special kind of Bi-direction or a special practical case precluded in discussion? - If deployments change is agreed to be included, the ambiguity of positions where deployment changes happen need to be captured. |
| R4-2114467 | Nokia, Nokia Shanghai Bell | Detailed simulation analysis for FR2 HST [Moderator]: Observations are copied in Topic#3. |
| R4-2112264 | Qualcomm, Inc. | On NR FR2 HST RRM Requirements [Moderator]: Three proposals below are treated in Topic#2. Proposal 1: Deciding number of Rx beams based on Dmin range instead of individual scenarios. Proposal 2: Number of Rx beams in FR2 HST is not fewer than 8. Search and measurement requirement enhancement of reducing Rx sweeping factor based only on number of Rx beam analysis is not feasible. Proposal 3: Network can indicate different SSBs on adjacent RRHs having the same QCL property: signal the mapping between the repeated sets of beams from the adjacent RRHs. Proposal 4: The following additional beam coverage related information can be signaled to UE - Distance between the projections of adjacent beam peaks on the track - Beam peak direction angle relative to track - The 6 dB beam-width projection on track In addition, UE can report speed to the network. [Moderator]: Four proposals below are supposed to be treated in RRM-2 email thread. Proposal 5: The FR2 HST neighboring cell search/measurement requirement is applicable when SSB are TDMed among adjacent RRHs in the neighboring cells (across different BBUs) in. Proposal 6: For FR2 HST neighboring cell search enhancement, removing relaxation factor of 1.5 from the legacy requirement and keep all other scaling factors the same if network can provide assistant information to UE as proposed in section 2.2. Proposal 7: Known TCI state condition can be shorter if network can provide assistant information to UE as proposed in section 2.2. Proposal 8: Keep L1-RSRP measurement requirement as it is if network can provide assistant information to UE as proposed in section 2.2. [Moderator]: Two proposals below are treated in Topic#2. Proposal 9: Network signals uni-direction or bi-direction deployment to UE. In uni-directional deployment, network signals DL beam w.r.t. UE moving direction to UE. Proposal 10: Define UE capability for FR2 HST enhancement support. Proposal 11: No FR2 HST requirement enhancement for long DRx cycles. In FR2 HST, RAN4 only considers SMTC period <= 40ms. [Moderator]: Treated in Topic #3. Proposal 12: Network signals a one-time large TA adjustment after UE switches to the beam from a new RRH in uni-directional model. [Moderator]: Should e treated in RRM-2 email thread. [Moderator]: Two proposals below are treated in Topic #3. Proposal 13: Restrict idle mode enhancement to DRx cycle within 320ms. Proposal 14: Allow 24 DRx cycles detection time when DRx cycle length is 320ms in idle mode enhancement. |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 1-1: Network signalling

Sub-topic description:

This sub-topic is devoted to network signalling needed for the indication of different/enhanced requirements related to the HST FR2 deployments.

Open issues and candidate options before e-meeting:

Following the WF formRAN4#99-e, RHST FR2 network deployment flag:

- Add a flag to enable the UE to identify different/enhanced RRM requirements in HST FR2 deployment.

- Add HST FR2 network deployment flag to the combined LS to RAN2 on all HST FR2 signalling flags.

Following the WF formRAN4#99-e, NW signalling and CPE capabilities for HST FR2 deployments:

- Continue the discussion after the deployment options and presence of non-CPE UEs is clarified:

  - NW signalling of Uni- and/or Bi-directional deployment:

    - Option 1: Network can indicate which high-speed scenario it is.

    - Option 2: Such a flag is not needed.

#### Issue 1-1-1: Identification of different/enhanced RRM requirements

- [Moderator]: It was agreed to add a network flag to enable the UE to identify different/enhanced RRM requirements in HST FR2 deployment. But the details of the network flag, e.g. whether it is a cell specific signaling or it is a UE specific signaling, is not discussed.

- Proposals and/or Observations

  - Proposal 1(CMCC): the network flag to enable the UE to identify different/enhanced RRM requirements in HST FR2 deployment is proposed to be a cell specific signaling, similar as the network assistance signaling (highSpeedMeasFlag-r16) introduced in Rel-16 NR HST WI.

- Recommended WF

  - Continue the discussion in the first round. 
Discussion can also continue regarding the draft of LS on UE capability and network signalling for Rel-17 NR HST RRM, R4-2114568.

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| Nokia | Proposal 1 is OK which is aligned with the draft LS (R4-2114568). |
| Huawei | This issue is related with issue 1-2-1. As in approved deployment WF, RAN4 assume that in HST FR2 Scenario A and B, only high-speed CPEs installed on the roof of the train can be present in the network. Then CPE shall support the enhanced RRM requirements in FR2 HST. So we doubt whether network needs to indicate flag UE. |
| Ericsson | If we understand correctly, the flag isn’t to identify UE’s capacity to support enhanced RRM requirement, instead to indicate UE to use enhanced RRM requirement or not. If so, we agree with suggestion 1. The flag should be cell specific signalling, unless specialized UE behaviour is required. |
| Samsung | Agree with P1. Cell specific signalling is okay and straightforward by following existing method from LTE R14. |
| Intel | Ok with Proposal 1 |
| Apple | The flag should be cell specific, if we agree to add it. However we would like to clarify there seem to be conflicting agreement in RRM WF and deployment WF in 99e. In R4-2108660, it was agreed that “Dedicated network for roof-mounted CPE: - RAN4 assume that in HST FR2 Scenario A and B, only high-speed CPEs installed on the roof of the train can be present in the network. ”. We would like to clarify whether this agreement in deployment WF means there is no need for such flag, therefore a conflicting agreement between deployment and RRM WF. |
| CATT | Agree that the network flag is cell specific signalling. |
| CMCC | In last meeting it was agreed that to add a network flag to enable the UE to identify different/enhanced RRM requirements in HST FR2 deployment. But the details of the network flag, whether it is a cell specific signalling or UE specific signalling is not discussed. That’s the reason we propose proposal 1 to finalize the network flag. |
| ZTE | Agree with Proposal 1. This signaling is similiar as the highSpeedMeasFlag-r16 in Rel-16, both to inform UE to identify the HST deployment, so they can both cell-specific signaling. |

#### Issue 1-1-2: Difference of uni-/bi-directional RRM requirements

- [Moderator]: To continue the discussion of a need for signaling of uni-/bi-directional operation, some companies propose to discuss if those operation cause differences in RRM requirements. Additionally, it proposed to discuss if occasional direction change may occur in Uni-directional deployments.

- Proposals and/or Observations:

  - Option 1 (Ericsson): RRM requirements are defined uniformly with maximal RX beam sweep number to cover Bi-directional deployments and Uni-directional deployments both.

  - Option 2 (Ericsson): RRM requirements are defined differently for Bi-directional deployments and Uni-directional deployments after we can conclude how to handle below three issues:

    - Deployments may change dynamically, it needs operators’ studies and confirmation

    - Is occasional direction change in Uni-directional deployment treated as a special kind of Bi-direction or a special practical case precluded in discussion?

    - If deployments change is agreed to be included, the ambiguity of positions where deployment changes happen need to be captured.

  - Observation 1 (Ericsson): According to current discussions including Te, scaling factor and so on, differentiation of uni-/bi-directional deployment may be beneficious to fast and accurate measurements, signaling and mobility.

- Recommended WF

  - Continue the discussion in the 1st round.

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| QC | We support option 1 to simplify UE implementation. |
| Nokia | Option 1 is Ok to avoid complicating the specification. |
| Huawei | Regarding Option 1, we think if the UE RX beam number is various in different scenario, the UE behaviour of RX beam sweeping is different. Using uniform RX beam sweeping scheme potentially means certain unnecessary cost for some relative good case. Regarding option 2, the issue is interesting. Operator’s inputs would provide valuable information. Just for a question, we’d like to know in which scenario the below deployment would be applied? Regarding observation 1, we are not sure how bi-directional or uni-directional deployment impacts Te. |
| Ericsson | We support Option 2 essentially, i.e. RRM requirements are defined differently for Bi-directional deployments and Uni-directional deployments. The rational is to avoid unnecessary waste. Regarding to Huawei’s question, we listed all possibilities which may need confirms by operators. |
| Samsung | We see the benefit from Option 2, by which different RRM requirement will be introduced if bi- or uni-directional flag is indicate. Different from FR1 HST discussion, which we can have lots of deployment reference from LTE counterpart, the deployment experience is limited and we can’t avoid any kinds of possibilities in this stage. |
| Intel | Support Option 2. Prefer to avoid unnecessary overhead from the RX beam sweeping. |
| Apple | We support option 2. RRM requirements are defined differently for bi-directional deployments and uni-directional deployment to optimize network performance. |
| CATT | Support option 2 which enhance the suitable requirements for different deployments. |
| ZTE | We believe whether using uniform or independent RRM requirements for bi-directional and uni-directional deployment, a main focus is the RX beam number, and which should be analyzed through link budget analysis. Meanwhile, the exact scheme of Scenario B bi-directional deployment is still under discussion in other session, so it’s too early to decide Option 1 or Option 2. |

#### Issue 1-1-3: Signaling of uni-/bi-directional operation

- [Moderator]: In the previous meetings it was discussed if network needs to indicate to the CPE which high-speed scenario it is.
The discussion is still dependent a lot on the detailed agreements on the support of bi-directional and uni-directional HST FR2 scenarios and difference of RRM requirements in those.
In particular, the companies have mentioned the dependency on Rx beam sweep number.

- Proposals and/or Observations

  - Proposal 1 (CMCC, Ericsson): Introduction of the flag depends on whether same or different RRM enhanced requirements are specified for uni- and bi-directional deployments.

  - Proposal 1a (CMCC, ZTE, Ericsson): Can be discussed after there is conclusion on the detailed RRM enhanced requirements and deployment issues

  - Proposal 2 (Qualcomm, Huawei, Apple): Network signals uni-direction or bi-direction deployment to UE.

  - Proposal 2a(Qualcomm): In uni-directional deployment, network signals DL beam w.r.t. UE moving direction to UE.

- Recommended WF

  - Continue the discussion in the 1st round.

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| QC | Flag is still helpful even if the requirements are the same. The information is helpful for faster beam refinement which doesn’t have a specific RRM requirement. |
| Nokia | Proposals 2 and 2a are related to signalling which can be further discussed depending on the outcome of Issue 1-1-4 and sub-topic 2-3 |
| ZZZ | If the UE RX beam number is various in different scenario, the UE behaviour of RX beam sweeping is different. Therefore it is suggested that network indicates the scenario (uni-directional/ bi-directional scenario A/ scenario B) to UE, then UE can judge its beam sweeping behaviour accordingly. |
| Ericsson | Does this mean that RRM criteria for uni-direction and bi-direction deployments should be different if Proposal 2 is followed? If same RRM requirement for uni/ bi-direction deployments, what’s the purpose of signaling of uni-/bi-directional operation from RRM pespective? That’s the concern in Issue 1-1-2 also. Related to Proposal 2a, it needs more studies, e.g. signalling shall be UE-specific and how to deal with occasional direction change in Uni-directional deployment in Issue 1-1-2? Anyhow, UE shall know the direction of DL beam/panel explicitly or implicitly. |
| Samsung | Similar as Ericsson, if the flag is introduced to indicate uni- and bi-directional deployment scenario, but without different requirement, what is the value of the introduced signalling? If we can’t see the benefits from requirement, we have to question the necessity of this signalling. |
| Intel | Support Proposal 1a. Need to wait for final agreement on the HST FR2 deployment scenarios. Highly likely only one type of deployment (uni-or bi-) will be agreed in HST FR2 based on the progress in resolving the issue of propagation delay difference. |
| Apple | Support proposal 2. |
| CATT | It depends on the outcome of other issues. Now support option 2 or option 1a. For the requirements, we think the requirements are different. But it can be decided after clear picture of detailed requirements. |
| CMCC | In our understanding, if the requirements are different between uni-direction and bi-direction, it is necessary to have the network indication to help UE identify which requirements to be applied. But if the requirements are same, we would like to know the benefit to have this indication. |
| ZTE | Support Proposal 1a. Similar analysis as in Issue 1-1-2. |

#### Issue 1-1-4: Signaling of beam coverage information

- [Moderator]:

- Proposals and/or Observations

  - Proposal 1 (Qualcomm): The following additional beam coverage related information can be signaled to UE

    - Distance between the projections of adjacent beam peaks on the track

    - Beam peak direction angle relative to track

    - The 6 dB beam-width projection on track

    - In addition, UE can report speed to the network.

- Recommended WF

  - TBA

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| QC | These items are helpful from UE implementation perspective. We would like to propose and understand network vendors and operators’ perspective. |
| Nokia | This issue is related to Issue 2-3-1 under network assistance. The proposed signalling parameters seem interesting, which can be used to reduce UE RX beam sweeping delay. Considering the signalling overhead is significant (which increases with the number of RRH TX beams), we would like to understand the potential benefits (e.g., the amount of reduction in the number of UE RX sweeping beams). Could the proponent provide further details on the following parameters: • Distance between the projections of adjacent beam peaks on the track • Beam peak direction angle relative to track • The 6 dB beam-width projection on track Does the 6 dB beam width projection on track mean beam coverage? In FR2 HST, the segment length of the track covered by the beam is more relevant than beam coverage area. If L1 measurement reports are not needed, then how to deal with unknown TCI states when TCI state switch occurs? |
| Huawei | This depends on the conclusion of RX beam number in RRM. In addition, the mapping idea based on the assumption that the RRHs are uniformly deployed across the railways and the variation of distances among RRHs and between RRH and the track is small on the same track. In practical deployment, this maybe not easy to be guaranteed. |
| Ericsson | It’s needs more studies, maybe the content cannot be fully agreed because the information essentially relies on the solution effect in practical scenarios. We are open to investigate necessary signalling to improve RRM performance. |
| Samsung | Need more study. The additional information can only help if the UE’s exact relative location w.r.t a certain RRH is known, but how UE know this information? |
| CATT | The proposals need further study. For the idea, we think the signallings are related to several static parameters for network operation. There might be benefit. But whether to add the new signalling, it also needs more evidence to prove there is benefit. One concern is the tight schedule of this WI in R17. |
| QC | Since some of the comments are related to our proposal in issue 2-3-1, we address both there. The proposal discussed in issue 2-3-1 is a relatively simpler signaling with little overhead, only signals the SSB index mapping across adjacent RRHs, while the proposals in this issue has more detailed information. Our response for the comments on the two proposals are listed below: - SSB index mapping across different RRHs: To Nokia: the benefit for neighboring cell detection and L1-RSRP is explained in our contribution. Since we may need to reduce cell identification time, this will be beneficial to speed up PSS/SSS detection. For L1-RSRP, as we explained in our contribution, in HST beam refinement becomes large overhead, and we might need UE to measure the L1-RSRP by fine beam to save beam refinement time after TCI state switch. In order to use fine beam to measure but still satisfy the legacy requirement which assumes rough beam measurement, the network assistant information is necessary. When UE is able to reduce beam refinement time after switching to new TCI states, it can benefit demod performance significantly especially for HST with short time spent on each TCI state. To Huawei: this signaling doesn’t mandate uniform deployment. The purpose is to allow network in uniform deployment cases to signal this useful information to UE. If the deployment is not uniform in some segments on the track route, network can leave this empty or fill just part of the entries that satisfies uniform deployment. To CATT, Ericsson and Samsung: the project and direction signaling probably required more discussion based on current received comment. Overhead is also larger. But the SSB index mapping is a relatively simple and low-overhead signaling, with relevant improvement as we explained in our contribution and above comment. We suggest to prioritize the SSB index mapping proposal. For the rest signaling, we can collect more comments and come up with more concise proposal probably for later release. - Projection and angle information To Nokia: let us explain the usage of each items below: • Distance between the projections of adjacent beam peaks on the track • The 6 dB beam-width projection on track By combining the two above, UE can predict the timing of next TCI state switch and perform beam refinement in advance. These two items are equivalent to the segment length and location of the track covered by each beam as you mentioned in the comment, UE can use any of them to run the prediction. • Beam peak direction angle relative to track This helps UE to decide which rough beams to use for beam discovery and narrow down the preferred subset of fine beams to perform beam refinement in a shorter time. |
| Nokia2 | In response to QC’s comments, we thank for your feedback. We have further comments regarding the following statement: “In order to use fine beam to measure but still satisfy the legacy requirement which assumes rough beam measurement, the network assistant information is necessary.” With the introduction of signalling we expect it can satisfy more stringent requirements given that the signalling parameters (beam peak direction, 6 dB beam width, etc) can provide accurate directional information about RRH TX beams, which helps in finding the most precise fine beam immediately without the need of performing extensive beam search. It defeats the purpose of introducing signalling if it could only satisfy legacy requirements considering overhead. Could you elaborate on why introducing signalling can only satisfy legacy requirements? |

### Sub-topic 1-2: UE capabilities

Sub-topic description

This sub-topic is devoted to the discussion of CPE capabilities the may be needed in HST FR2 deployments.

Open issues and candidate options before e-meeting:

Following the WF formRAN4#99-e, CPE capabilities for HST FR2 deployments:

- Continue the discussion after the deployment options and presence of non-CPE UEs is clarified:

  - UE identification of support for HST FR2 deployment:

    - Option 1: Flag is not needed.

    - Option 1a: Only roof-mounted CPE is considered that should always have a capability to work in HST FR2 scenario in this WI.

    - Option 2: UE capability is needed

  - CPE indication of support of uni-/bi-directional operation

    - Option 1: CPE indication of support of different deployment types is not needed.

    - Option 2: Different capabilities are needed if different requirements are agreed.

#### Issue 1-2-1: CPE support for HST FR2 deployment

- [Moderator]: In the WF on FR2 HST Deployment Scenario Analysis [R4-2108660], it was agreed that

  - Dedicated network for roof-mounted CPE:

    - RAN4 assume that in HST FR2 Scenario A and B, only high-speed CPEs installed on the roof of the train can be present in the network.

    - No need to differentiate roof-mounted CPE from other FR2 UEs in HST FR2 scenario.

- Proposals and/or Observations

  - Option 1(CMCC, ZTE, Ericsson): It is not necessary to introduce UE capability to indicate the support of FR2 HST. Only roof-mounted CPE is considered that should always have a capability to work in HST FR2 scenario.

  - Option 2(Qualcomm): Define UE capability for FR2 HST enhancement support.

- Recommended WF

  - Argumentation related to Option 2 may not consider the deployment scenario agreement.

  - Indicated it Option 1 is agreeable.

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| Nokia | Option 1 is OK if the agreement in the WF on FR2 HST Deployment Scenario Analysis (R4-2108660) is adopted for FR2 HST RRM enhancements. |
| Huawei | Agree with option 1 as only CPE on roof of the train is considered in R17 HST FR2. |
| Ericsson | Agree with Recommended WF, support Option 1. |
| Samsung | By following agreed WF, we don’t need to introduce capability, but the Power Class for HST UE (which is linked to this UE type) will be enough for requirement definition, e.g., the enhanced RRM requirement for FR2 HST is applicable to PC-X (which is corresponding to FR2 UE in TS38.101-2) |
| Intel | How to capture “Only roof-mounted CPE is considered” in the spec? Will the requirements be applied to all UEs? In case of further FR2 HST evolution how to differentiate “roof-mounted CPE” from “enhanced roof-mounted CPE”? We are ok to restrict FR2 HST to specific device type, but the mechanism is unclear. |
| Apple | Support option 1. |
| CATT | Agree with Recommended WF. Support option 1. |
| CMCC | We are OK with option 1 following the agreements in deployment scenario. But considering the forward compatibility, we are also open to discussion. |
| QC | We understand that in this work item, we mainly consider the dedicated network for requirement discussion. However, UE capability only consumes one bit, and it enables the possibilities of future deployment with HST network serving nearby non-HST UEs. It is a much more efficient usage for operator deployment since HST is a relative dense deployment with inter-site distance of 700m. |
| ZTE | Support Option 1. |

#### Issue 1-2-2: CPE support for uni-/bi-directional operation

- Proposals and/or Observations

  - Option 1(CMCC): It is not necessary to introduce CPE indication of support of uni-/bi-directional operation.

  - Option 2(ZTE): Considering multiple differences exist between uni- and bi-directional deployments, different capabilities are needed if different requirements are agreed.

- Recommended WF

  - Continue the discussion in the 1st round.

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| QC | No need to differentiate if the requirements are the same. |
| Nokia | Option 1 is OK which is in line with Issue 1-1-2. |
| Ericsson | The issue relies on the clarification of the issue: ‘CPE support for uni-/bi-directional operation’ means ‘CPE has capacity to support uni-/bi-directional operation’ or ‘CPE can change characteristics, e.g. RX beam sweep number in uni-/bi-directional operation’. If the issue is first one, to our understanding, CPE shall have capacity to support both, especially for occasional direction change in un-direction and uni-/bi-directional deployment changes in different regions. If the issue is latter one, signaling of uni-/bi-directional operation from network shall be enough IF different requirements for uni-/bi-directional operation are agreed. |
| Samsung | No need to support because UE need to support both uni- and bi-directional scenarios. |
| Intel | Same comment as for Issue 1-1-3. Need to approve the deployment scenario first. At the same time, we think that two panels and the ability of operation in both directions (not necessarily in bi-directional mode) for UE should be mandatory. Even in uni-directional mode, we don’t think that CPE will always be installed on the train in alignment of current direction of uni-directional network deployment. |
| Apple | Support option 1. CPE should support both uni-directional and bi-directional deployment |
| CATT | Support option 1. For the capability, CPE should support both. |
| CMCC | Option 1. Our understanding is that both uni-directional and bi-directional deployments are supported by the high-speed CPEs. |
| QC | Based on GTW agreement, different scenarios may have different measurement requirements. If the requirements are different, each UE only has to support one of them depending on the deployment, therefore, RAN4 needs to introduce two additional UE capabilities, and both capabilities are optional. |
| ZTE | Support Option 2. Considering CPE will not always be installed on the train in alignment of current direction, two panels per CPE should be mandatory, but which do not means the support of bi-directional deployment is mandatory. If independent RRM requirements are defined for uni- and bi- directional deployments, the corresponding UE capability is needed. |

### Sub-topic 1-3: LSs

Sub-topic description

Two LSs were received in the agenda item 9.9.4/9.9.4.1. In the sub-topic, the companies can discuss how the work on those can proceed.

Open issues and candidate options before e-meeting:

#### Issue 1-3-1: Responsible for LS on UE capability and network signalling

- [Moderator]:

  - At the previous meeting it was agreed that “Add HST FR2 network deployment flag to the combined LS to RAN2 on all HST FR2 signalling flags.”

  - A version of the LS is submitted as TDoc R4-2114568.

- Options:

  - Option 1: LS Author: Nokia, Nokia Shanghai Bell

  - Other options are not precluded

- Recommended WF

  - Assign LS responsible in the 1st round.

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| Nokia | Nokia can take responsibility. |
| Huawei | When all the answers to sub-topic 1-1 and 1-2 are clear, it is supposed to send LS to RAN2 and option 1 is fine. |
| Ericsson | We suggest to wait agreement on Issue 1-1 and Issue 1-2. |
| Samsung | We appreciate Nokia to take the responsibility of this LS, but the content “The RAN4 has also agreed to introduce UE capability, which is used to indicate that UE is capable of supporting the enhanced NR RRM requirements.” depends on the conclusion from above discussion on Issue 1-2-2. |
| Intel | Need to wait for agreements on sub-topics 1-1 and 1-2 |
| CATT | Wait for outcome of other issues. |
| ZTE | Wait for the conclusions of the related issues. |

#### Issue 1-3-2: LS on Beam Management Enhancement Signaling in FR2 HST R17

- [Moderator]:

  - The LS draft is included as an Appendix in R4-2112264.

  - The related issues are discussed in Issue 1-1-3 and Issue 1-1-4, 2-3-1.

- Recommended WF

  - In the 1st round, companies are invited to disclose their opinions on a need of the LS.

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| Nokia | To be discussed pending the outcome of other issues. |
| Ericsson | Agreements on beam management should be specified before sending the LS. |
| Samsung | Discuss above relevant technical issue firstly. |
| Intel | Need to wait for agreements on the corresponding issues |
| CATT | Need further discussion for the agreements. |
| ZTE | Wait for the conclusions of the related issues. |

#### Issue 1-3-3: Responsible for LS on Beam Management Enhancement Signaling

- [Moderator]:

- Options:

  - Option 1: LS Author: Qualcomm

  - Other options are not precluded

- Recommended WF

  - Assign LS responsible, if there is an agreement to proceed with the LS (previous Issue).

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| QC | We can take the LS if the options are agreed. |
| Ericsson | Agreements on beam management should be specified before sending the LS. |
| Samsung | Discuss above relevant technical issue firstly. |
| Intel | Wait for agreement on issue 1-3-2 |
| ZTE | Wait for the conclusions of the related issues. |

### CRs/TPs comments collection

For close-to-finalize WIs and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| None | Company A |
|  | Company B |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

| Sub-topic | Status summary |
| --- | --- |
| Sub-topic #1-1: Network signalling | Issue 1-1-1: Identification of different/enhanced RRM requirements Background: In the previous meetings two agreements were achieved: - RAN4#99-e, RRM WF[R4-2108342]: Add a flag to enable the UE to identify different/enhanced RRM requirements in HST FR2 deployments. - RAN4#99-e, Deployments WF[R4-2108660]: Agreement on dedicated network for roof-mounted CPE: - RAN4 assume that in HST FR2 Scenario A and B, only high-speed CPEs installed on the roof of the train can be present in the network. - No need to differentiate roof-mounted CPE from other FR2 UEs in HST FR2 scenario. Common understanding is that the flag indicates UE to use enhanced RRM requirements or not. Then, it was necessary to clarify what kind of flag will it be. The majority of companies think that the network flag is cell specific signalling. However, some companies doubt is such a flag might be needed at all because only CPEs capable to support enhanced requirements by default will be used in the network. Tentative agreements: Following the former RRM agreement, the Moderator propose to agree the following rectification to the former agreement: Add a flag in a form of cell-specific signalling to indicate UE to use different/enhanced RRM requirements in HST FR2 deployments. Candidate options: - Option 1 (Nokia, Samsung, Intel, CATT, CMCC, ZTE): Add a flag in a form of cell-specific signalling to indicate UE to use different/enhanced RRM requirements in HST FR2 deployments. - Option 2 (Huawei, Apple): Flag is not needed because only HST FR2 capable CPE are present in the network. Recommendations for 2nd round: Either confirm that tentative agreement is agreeable or discuss further if former agreement needs to be reverted. Issue 1-1-2: Difference of uni-/bi-directional RRM requirements Background: In the email discussion the companies shared different opinions about a need to differentiate the requirements for uni- and bi-direction HST FR2 deployments. However, after the GtW discussion of the number of Rx beams (Issues 2-1-1) that also covered the discussion of different deployments and scenarios (Issues 2-1-2 and Subtopic 2-1) the followgin agreement was achieved: - RX beam number for RRM requirements definition - Define two set of requirements for Scenario A and Scenario B in terms of number of RX beams per UE - Scenario A: [2] RX beams for all scenarios - Scenario B: [6] RX beams for all scenarios - FFS on feasibility and methods to differentiate scenarios from UE perspective - FFS if different UE capabilities shall be used for Scenario A and B support - Note: if there is insignificant difference between Scenario A and B requirements, then further discussion on unified requirements can take place. I.e., it was agreed to define only two sets of RRm requirements: for Scenario A and Scenario B. Therefore, only Optoin 1 is valid following the agreement above. Tentative agreements: RRM requirements are defined uniformly with maximal RX beam sweep number to cover Bi-directional deployments and Uni-directional deployments both. Recommendations for 2nd round: Confirm that the tentative agreement is agreeable. Issue 1-1-3: Signaling of uni-/bi-directional operation Background: Some of the comments on the issue were done before the GtW session discussion. Therefore, it is recommended to re-discuss the issue taking into account the results of GtW session listed above. Regarding Proposal 2a by Qualcomm, the moderator suggests treating it separately from this issue (together with the following issue 1-1-4) in order to make the selection between the other existing options easier in the second round. Tentative agreements: None Candidate options: - Option 1: Network signals uni-direction or bi-direction deployment to UE. - Option 2: Signalling of uni-/bi-directional operation is not needed. Recommendations for 2nd round: Continue the discussion in the second round. Issue 1-1-4(updated): Signalling of network assistance information Background: Originally the issue under the discussion was “Signalling of beam coverage information”. However, also following the comment from the company that came up with the proposal, it seems more logical to the moderator to join the discussion of the issues with Proposal 2a from the previous Issue (1-1-3) and with the Issue (2-3-1) Signalling of SSB index per RRH. As it was commented by QC “The proposal discussed in issue 2-3-1 is a relatively simpler signaling with little overhead, only signals the SSB index mapping across adjacent RRHs, while the proposals in this issue has more detailed information.” The intention of introducing signalling is to reduce the number UE RX sweeping beams. In general, both for Issue 1-1-4 and Issue 2-3-1 the companies agree in the opinion that more study is needed. Therefore, all options are collected together in this Issue for further discussion. Tentative agreements: None Candidate options: FFS a need for signalling of network assistance information in HST FR2 deployments: - Option 1: In uni-directional deployment, network signals DL beam w.r.t. UE moving direction to UE. - Option 2: Enable network signaling of SSB index per RRH - Option 3: Network can indicate different SSBs on adjacent RRHs having the same QCL property: signal the mapping between the repeated sets of beams from the adjacent RRHs - Option 4: Scaling factor based on explicit signalling from network to UE or implicit signalling based on UE’s identification of SSB index/TCI-state, position of UE relative to RRHs and so on - Option 5: The following additional beam coverage related information can be signaled to UE - Distance between the projections of adjacent beam peaks on the track - Beam peak direction angle relative to track - The 6 dB beam-width projection on track - In addition, UE can report speed to the network. - The options are not mutually exclusive and other options are not precluded. Recommendations for 2nd round: - Add options listed above into the WF for the further study. - Keep the discussion open in the second round if companies need to clarify some details further. |
| Sub-topic #1-2: UE capabilities | Issue 1-2-1: CPE support for HST FR2 deployment Background: Even though the majority of the companies do not see a need to introduce UE capability to indicated the support of FR2 HST, some companies has raised a concern on how to differentiate HST FR2 capable CPEs from any other UEs in the specification. One proposed way would be to apply enhanced RRM requirements for FR2 HST based on Power Class corresponding to FR2 UE in TS 38.101-2. Tentative agreements: It is not necessary to introduce UE capability to indicate the support of FR2 HST. Candidate options: - Option 1 (CMCC, ZTE, Ericsson, Huawei, Samsung, CATT): It is not necessary to introduce UE capability to indicate the support of FR2 HST. - Option 2 (Samsung): Apply enhanced RRM requirements for FR2 HST based on Power Class corresponding to FR2 UE in TS 38.101-2. - Option 3(Qualcomm): Define UE capability for FR2 HST enhancement support. - Other options are not precluded. Recommendations for 2nd round: - Confirm if tentative agreement is agreeable. - Continue the discussion on how to differentiate HST FR2 capable CPEs from any other UEs in the specification. Issue 1-1-2: CPE support for uni-/bi-directional operation Background: Based on the results of GtW discussion, the number of RX beams is not differentiated depending on the uni/bi-directional deployment type. Moreover, majority of the companies share an opinion that both uni-directional and bi-directional deployments are supported by the high-speed CPEs. One additional aspect that was mentioned by one of the companies is the difference of understanding between: - CPE has capacity to support uni-/bi-directional operation - CPE can change characteristics, e.g. RX beam sweep number in uni-/bi-directional operation. This seems to need further discussion. Tentative agreements: HST FR2 CPE has a capacity to support both uni- and bi-directional operation. Candidate options: - Option 3: CPE can change characteristics, e.g. RX beam sweep number in uni-/bi-directional operation. - Other options are not precluded. Recommendations for 2nd round: - Confirm that tentative agreement is agreeable. - Continue the discussion of Option 3 in the second round. |
| Sub-topic #1-1: LSs | Issue 1-3-1: Responsible for LS on UE capability and network signalling Background: It seems that that, in general, companies do not object Option 1, however, other Issues are pending agreements before the LS can be finalized. Tentative agreements: None Candidate options: - Option 1: LS Author: Nokia, Nokia Shanghai Bell - Other options are not precluded. Recommendations for 2nd round: None. Continue the work on the LS after the issues in sub-topic 1-1 are clear. Issue 1-3-2: Responsible for LS on UE capability and network signalling Background: The companies share the opinion that the related Issue 1-1-4 shall be discussed, first. Tentative agreements: None Candidate options: - None Recommendations for 2nd round: Continue the discussion in Issue 1-1-4. Issue 1-3-3: Responsible for LS on UE capability and network signalling Background: The companies share the opinion that the technical details in related Issue 1-1-4 shall be discussed, first. Tentative agreements: None Candidate options: - None Recommendations for 2nd round: Continue the discussion in Issue 1-1-4. |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round (if applicable)

TBA

### Sub-topic 1-2: TBA

#### Issue 1-2-2: TBA

Agreements from round 1:

TBA

Candidate options:

- Option 1:

- Option 2:

Recommendations for 2nd round:

TBA

Contributor Comments:

| Company | Comments |
| --- | --- |
| XXX |  |
| YYY |  |
| ZZZ |  |

## Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

# Topic #2: Number of RX beams from RRM perspective

Main technical topic overview. The structure can be done based on sub-agenda basis.

This topic is devoted to the discussion of the feasible number of RX beams at CPE in HST FR2 deployments from the RRM perspective. Deployment scenario study shall be used as a reference.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2111954 | CATT | Discussion on number of RX beams for HST RRM in FR2 Proposal 1: For Scenario-A, uni-directional deployment, the RX beam number can be 1. Proposal 2: For Scenario-A, bi-directional deployment, the RX beam number can be 1 RX beam per panel, 2 in total. Proposal 3: For Scenario-B, uni-directional deployment, the RX beam number can be 1 RX beam per panel. Proposal 4: For Scenario-B, bi-directional deployment, the RX beam number can be larger than 1 RX beam per panel. E.g. [2] RX beams per panel can be used. Proposal 5: In Scenario-A and Scenario-B, the RX beam number is different. It is necessary that network indicates such information. |
| R4-2112091 | Apple | Discussion on number of Rx beam for FR2 HST Proposal 1: In Scenario A, the number of Rx beam is 1 in unidirectional deployment, and 2 in bi-directional deployment. Proposal 2: Enable network signaling of SSB index per RRH and whether this is uni-directional or bi-directional deployment. The signaling can be used to adapt the number of Rx beam to be used in RRM requirement. [Moderator]: Partially treated also in Topic 1. |
| R4-2113214 | ZTE Corporation | Discussion on RX beam number for HST FR2 Proposal 1: Only 1 RX beam per CPE panel is enough for Scenario B uni-directional deployment. Proposal 2: For Scenario B bi-directional deployment, wait for the detailed schemes determined in Channel and Scenario sessions. Once the detailed schemes decided, we can determine RX beam number. Proposal 3: Smaller RX beam number/scaling factor will relax the restriction on DRX cycle. [Moderator]: Three Observations and one Proposal below are more relevant to RRM-2 email discussion thread. Thus, not treated explicitly. Observation 1: The known TCI state condition may not be satisfied especially for Scenario A bi-directional, Scenario B bi-directional RRH deployments. Proposal 4: For bi-directional deployment, all of the known rules are not always met, so Known and unknown TCI state switching should be applied in FR2 HST depends on the deployment. Observation 2: Based on R16 requirements, L1-RSRP measurements delay for unknown case may be too large for HST. Such cases should be further considered. Observation 3: For the assumption of SMTC=40ms and SMTC=160ms, the current requirements for intra-frequency cell identification can not work for HST scenario. Proposal 5: From the point of cell identification, smaller RX beam number can enhance the requirements, so as to satisfy the need of HST scenario. |
| R4-2113272 | OPPO | Discussion on the Rx beams in RRM requirement Proposal 1: Agree to decrease UE RX beam number to reduce measurement delay under proper SNR condition. Proposal 2: A unified requirement set for RX beam is suggested to be defined, considering different deployment and different Dmin. Proposal 3: RAN4 to decide whether to introduce signaling for SSB index per RRH. |
| R4-2113329 | Ericsson | Number of RX beams for HST FR2 Observation 1: Simulation results in scenario session for different scenarios already proved similar results that enough high SNR is kept with 1beam/panel with proper UE panel direction and beam direction. Proposal 1: For Uni-directional deployment, RX beam number =1/panel. Proposal 2: For Bi-directional deployment, RX beam number =1/panel; or 2 totally. Observation 2: For Scenario B + Uni-directional deployment, L1-RSRP measurement delay may cause deep drop of SNR especially when UE moves towards boresight of RRH’s panels. Increasing RX beam number has not help on the issue. Observation 3: For Scenario B + Bi-directional deployment, L1-RSRP measurement delay may cause deep drop of SNR when UE is moving into target RRH’s beam after passing source RRH. Increasing RX beam number has not help on the issue. Observation 4: For Scenario A + Uni-directional deployment, L1-RSRP measurement delay may cause SNR drop when UE moves towards boresight of RRH’s panels. L1-RSRP may be lost when UE is passing source RRH before switching to next beam from target RRH. Increasing RX beam number has not effect on the issue. Observation 5: Based on observations, RRM specific measurement (take L1-RSRP as example) may be impacted by measurement periodicity but increasing RX beam cannot bring benefit effectively. Proposal 3: Given that the network uses L1-RSRP to control UE serving and candidate beams, observed issues in different scenarios might result in deterioration of beam management performance and so on. We proposed mitigation solution in issue of TCI state switch. [Moderator]: Hard to treat the issue/proposal because it seems to belong to the different email thread. Observation 6: it needs to be noted that positions of RRH may be at same side of rail track or both sides of rail track, the phenomenon has unneglectable impact to RRM requirements, e.g., scaling factor. Proposal 4: 2 options for possible number of RX beams or scaling factor definitions can be used to handle different positions of RRH at one/both sides of rail track. - RRM requirements are defined with scaling factor which is double of number of RX sweep number in scenario study. - RRM requirements are defined with adapted scaling factor based on explicit signalling from network to UE or implicit signalling based on UE’s identification of SSB index/TCI-state, position of UE relative to RRHs and so on. Proposal 5: Obstruction between RRH and UE, it may happen in practice indeed, and its impact to RRM, e.g., beam management/RX beam number doesn’t need to be discussed in this session. |
| R4-2113834 | Huawei, HiSilicon | Discussion on number of Rx beam for FR2 HST Proposal 1: Scaling factor used in RRM requirements can wait for further conclusion from deployment scenario study. [Moderator]: The proposal is not treated explicitly in this topic, but similar proposals are present in Topic #3. Proposal 2: To facilitate UE Rx beam sweeping, it is suggested that network can indicate which high speed scenario (uni-directional/ bi-directional scenario A/ scenario B) is. [Moderator]: Treated in Topic 1. |
| R4-2114186 | Intel Corporation | Discussion on the number of RX beams for FR2 HST Observation 1: In uni-directional Scenario-A multiple UE Rx beams do not provide benefit comparing to single Rx beam. Observation 2: In uni-directional Scenario-B multiple UE Rx beams may provide better link budget on some parts of the distance. But single Rx beam can still provide sufficient performance along the whole track [Moderator]: Two proposals below are not treated explicitly because the topic is rather discussed in RRM-2 track: Measurement procedure requirements. Proposal 1: RAN4 to consider RX beam search scaling factor equal to 1 for RRM requirements in case of FR2 HST uni-directional deployment Proposal 2: UE to consider RX beam search scaling factor equal to 2 for RRM requirements in case of bi-directional deployment. |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 2-1: General

Sub-topic description:

In this sub-topic, the general principles behind the reduction of the number of RX beams are discussed.

Open issues and candidate options before e-meeting:

Decreasing RX beam number:

- FFS the feasible number of RX beams from RRM perspective

  - deployment scenario study is used as reference

#### Issue 2-1-1: RX beam number reduction

- Proposals and/or Observations

  - Proposal 1(OPPO): Decrease UE RX beam number to reduce measurement delay under proper SNR condition.

  - Proposal 2 (ZTE): Smaller RX beam number/scaling factor will relax the restriction on DRX cycle.

  - Proposal 3 (ZTE): From the point of cell identification, smaller RX beam number can enhance the requirements, so as to satisfy the need of HST scenario.

  - Proposal 4 (Ericsson): Increasing RX beam number above one per panel has no effect on the issue with SNR drop in multiple scenarios (Scenario B + Uni-directional, Scenario B + Bi-directional, Scenario A + Uni-directional).

  - Proposal 5 (Qualcomm): Number of Rx beams in FR2 HST is not fewer than 8. Search and measurement requirement enhancement of reducing Rx sweeping factor based only on number of Rx beam analysis is not feasible.

- Recommended WF

  - Discuss if it possible to agree on a need to reduce the number of RX beam number in HST FR2.

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| QC | We would like to propose a better methodology to handle the RRM requirement. RAN4 should first look at how short the measurement period is to maintain the connectivity in HST given ISD and speed. Then RAN4 can study how many Rx beam UE can sweep. Note that the deployment scenario analysis shows that more number of beams enhance demod performance. Our analysis shows that 15 beams can achieve much better performance than 1 or 2 beams. But we didn’t propose to use 15 beams in RRM, because the 15 beams are fine beams. Note that once we know a certain measurement period can maintain the connectivity, further reduction on the measurement period has negligible impact on performance. Based on the above observations, we propose to - Analyze the required measurement period to maintain connectivity - Decide the number of Rx sweeping based on 1 Therefore, we can maximize the demod performance gain while maintain connectivity in FR2. Our analysis concludes that 4000ms cell identification time (detection + measurement) is feasible to maintain connectivity in FR2 HST. |
| Nokia | It is recommended to separate the discussion about the number of UE RX beams into • idle/inactive mode requirements • connected mode requirements. For the connected mode requirements, there is a need to achieve high UE throughput while there is no such a need for idle/inactive mode requirements. As such, the number of RX sweeping beams for idle/inactive mode may not be necessarily the same as for connected mode. Based on our system simulation results, the following inequality holds: The number of UE RX beams (idle/inactive mode) <= the number of UE RX beams (connected mode), The number of UE RX beams for idle/inactive and connected modes depends on the outcome of discussions in other issues. |
| Huawei | It is straight forward that decreasing UE RX beam can reduce the L3/L1 measurement delay. The key point is how to guarantee the side condition and proper coverage with reduced RX beam number. |
| Samsung | It is already well demonstrated in many companies’ papers that reduced RX beam number is possible for both uni- and bi-directional RRH deployment cases for both Scenario A and B. From UE implementation perspective, the beambook with smaller number of beams are technical feasible. |
| Intel | Agree with comment from Samsung, both in RRM and deployment session there were many papers showing that the RX beam number can be reduced without significant performance degradation. Quite the opposite, with many TX/RX beams the performance degradation can be observed due to delayed handover and TCI state switching under short beam dwelling time. |
| Apple | Agree in general the number of Rx beams can be reduced, depending on evaluation in deployment scenario discussion. |
| CATT | This issue here is too mixed. Those proposals are not strictly mutually exclusive. The common agreement is the RX beam number can be decreased. We proposed to discuss each requirement directly. |
| ZTE | Support Proposal 1, Proposal 2, Proposal 3, Proposal 4 Agree with Samsung, and our link budget analysis can also show the same point of view. |

#### Issue 2-1-2: Unification of requirements

- Proposals and/or Observations

  - Proposal 1(OPPO): A unified requirement set for RX beam is suggested to be defined, considering different deployment and different Dmin.

  - Proposal 2(Qualcomm): Deciding number of Rx beams based on Dmin range instead of individual scenarios.

  - Proposal 3(CATT): It is necessary that network indicates the RX beam number difference in Scenario-A and Scenario-B

- Recommended WF

  - Continue the discussion in the 1st round.

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| QC | Option 1 and 2 are the same, we suggest to merge them. |
| Nokia | Proposals 1 and 2 seem to be similar. We are open to further discuss about the Dmin range and its relation to the number of UE RX beams. |
| Huawei | Regarding proposal 1, in deployment discussion, the UE RX beam number for scenario A for bi-directional and uni-directional is 1 beam per UE panel. For scenario B, as Dmin is large, the UE Rx beam is supposed to be larger than scenario A. If the RX number in scenario B is far greater than that in scenario A, unified requirements set may be unfair to Scenario A. We are wondering more details in proposal 2 if a Dmin range in [10m,150m] If network indicates UE which scenario (uni-directional/ bi-directional scenario A/ scenario B), UE can judge its beam sweeping behaviour accordingly. |
| Ericsson | We can refer to conclusion from Sub-topic 2-2. |
| Samsung | Discuss Sub-topic 2-2 firstly. |
| Intel | Support Proposal 3 if different RX beam number will be agreed. For Proposal 2: Scenario A and B describe two different approaches for HST deployment: dedicated FR2 HST deployment (Dmin=10m) vs FR1 HST deployment reuse (Dmin=150). They should not be considered as the limits of the Dmin range |
| Apple | Similar comments as Ericsson. Refer conclusion from 2-2. |
| CATT | In deployment discussion, the Dmin range is defined in Scenario-A and Scenario-B. We can use the values for verification. And network indicates UE about the information of scenario, it will be helpful. |
| ZTE | Support Proposal 3 if independent RX beam numbers for different scenarios agreed. |

#### Issue 2-1-3: Impact of RRH position at one/both sides of rail track

- [Moderator]: One of the companies have observed that positions of RRH may be at same side of rail track or both sides of rail track, the phenomenon has unneglectable impact to RRM requirements, e.g., scaling factor.

- Proposals and/or Observations

  - Proposal 1: RRM requirements are defined with scaling factor which is double of number of RX sweep number in scenario study.

  - Proposal 2: RRM requirements are defined with adapted scaling factor based on explicit signalling from network to UE or implicit signalling based on UE’s identification of SSB index/TCI-state, position of UE relative to RRHs and so on.

- Recommended WF

  - Continue discussion in the 1st round.

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| QC | We agree with the motivation of proposal 1, but as we clarified in issue 2-1-1, we should consider number of Rx from scenario analysis after the mobility analysis for HST. |
| Nokia | The proposals seem to be dealing with different deployment scenarios, where RRHs are deployed on one side or both sides of the track. Could the proponent provide further details/analysis on how the different scenarios impact RRM requirements? |
| Huawei | We agree with the observation in proposal 1 of the issue due to RRH position at one side or Z shape at both side. The UE beam sweeping range will enlarge in RRH “Z” shape deployment, then the RX beam number is increased accordingly. For scenario A, maybe the impact is not outstanding, however for scenario B, the issue is unneglectable. Moreover in practical deployment, Z shape deployment can not be completely avoid. How to solve the problem needs further study. |
| Ericsson | First, we'd like to hear your thoughts on the following question: "Does RRH position on one/both sides of rail track impact RRM requirement from the perspective of RX beam sweep number?" One of our concerns is random or ’Z’ shape positions may double RX beam sweep number in worse case. We support Proposal 2 to reduce beam sweep if the response is affirmative. The solution may be explored in further depth with detail and required signaling. |
| Samsung | At least for Scenario-A, no difference since small Dmin means UE can ignore the horizontal plane’s beam sweeping at all. |
| Intel | We want to mention that in RAN4#98-e we had the following agreement: - RAN4 primarily consider HST FR2 deployment with - One train moving over one railway track in one direction; - RRHs are located on one side of the track; Not sure whether we need to strictly follow that agreement. In general, we agree that, for Scenario-B the number of RX beams can be doubled if the RRHs are deployed on both sides of the track. Proposal 2 seems fine for us. |
| Apple | Support proposal 2. For scenario A, whether RRH is in one side or two side of the track might not impact much since Dmin = 10m. However for scenario B, RRH on two side of the track can potentially double the Rx beam number. |
| ZTE | Firstly, if the ‘Z’ shape deployment is not negligible, the agreement listed by Intel should be further discussed. For Scenario A, we agree with Samsung and Apple, whether RRH is in one side or two sides of the track will not impact the number of RX beam. But for Scenario B, maybe RRH on two sides of the track will double the number of RX besm. |

#### Issue 2-1-4: Impact of obstruction between RRH and UE

- [Moderator]: This issue addresses the question raised in last meeting regarding an Obstruction, such as a pole or other piece of equipment, being between the UE and BS.

- Proposals and/or Observations

  - Proposal 1(Ericsson): Impact of obstruction between RRH and UE on RRM e.g., beam management/RX beam number, doesn’t need to be discussed in this session.

- Recommended WF

  - Continue discussion in the 1st round.

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| Nokia | Proposal 1 is Ok since LOS is assumed. |
| Ericsson | It is to answer concerns raised in last meeting. |
| Intel | Ok with the Proposal 1. |
| CATT | Fine with proposal 1. |
| ZTE | Agree with Proposal 1. |

### Sub-topic 2-2: Number of RX beams in HST FR2 deployment scenarios

Sub-topic description

In this sub-topic, the proposals about the number of RX beams in different HST FR2 deployments scenarios are discussed.

Open issues and candidate options before e-meeting:

Network assistance for reducing RX beam number:

- FFS: to be discussed depending on the outcomes of the deployment scenarios

#### Issue 2-2-1: Uni-directional deployment, Scenario-A

- [Moderator]: 1 beam per UE panel already agreed in Deployment scenarios in RAN4#98-Bis-e.

- Proposals and/or Observations

  - Proposal 1(CATT, Apple, Ericsson, Intel): The RX beam number can be 1.

- Recommended WF

  - Agree on 1 RX beam in uni-directional deployment, Scenario-A.

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| QC | We agree the conclusion as an observation for scenario A. But in terms of requirement, as argued in issue 2-1-1, we should first perform mobility analysis. Our analysis concludes that 4000ms cell identification time (detection + measurement) is feasible to maintain connectivity in FR2 HST. |
| Nokia | Further discussions are needed. |
| Ericsson | Agree with Recommended WF |
| Samsung | Agree with Recommended WF |
| Intel | Agree with Recommended WF. For uni-directional, Scenario-A there are no benefits in more beams. |
| Apple | Agree with WF. |
| CATT | Agree with Recommended WF. |
| ZTE | Agree with the recommended WF. |

#### Issue 2-2-2: Bi-directional deployment, Scenario-A

- [Moderator]: Following the agreement in the WF HST FR2 deployments at RAN4#99-e[R4-2108660]:

  - Number of Beam for bi-directional RRH deployment, Scenario-A

    - If bi-directional deployment is confirmed to be used for Scenario-A:

      - 1 beam per RRH panel, two panels in opposite directions

      - 1 beam per UE panel (i.e., 2 beam per UE), already agreed in RAN4#98-Bis-e

- Proposals and/or Observations

  - Proposal 1(CATT, Apple, Ericsson): The RX beam number can be 1 RX beam per panel, 2 in total.

- Recommended WF

  - Agree on 1 RX beam per panel (2RX beams per CPE) in bi-directional deployment, Scenario-A.

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| QC | Same comment as 2-2-1. |
| Ericsson | Agree with Recommended WF if Bi-directional deployment in Scenario-A isn’t precluded from scenario perspective. |
| Samsung | Same comment as Ericsson, the conclusion should be dependent on the necessity of bi-directional deployment in Scenario-A. |
| Intel | Agree with Recommended WF. |
| Apple | Support the WF. |
| CATT | Agree with Recommended WF. |
| ZTE | Agree with Recommended WF. |

#### Issue 2-2-3: Uni-directional, Scenario-B

- Proposals and/or Observations

  - Option 1(CATT, ZTE, Ericsson, Intel): The RX beam number can be 1 per panel.

  - Option 2 (Qualcomm): Number of Rx beams in FR2 HST is not fewer than 8.

- Recommended WF

  - Continue discussion in the 1st round.

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| QC | As we commented in 2-1-1, we believe the mobility analysis should be considered first before using number of Rx beams to decide requirements. |
| Nokia | Further discussions are needed to understand Option 2. |
| Huawei | For option 1, for scenario-B, Dmin is 150ms, 1 beam per panel in UE side means 1 wider beam, then how to guarantee sufficient beam reception gain. In general, we are worrying about the performance degradation by using 1 RX beam per panel in scenario B. We are open to option 2. Not sure whether 8 beams are required. |
| Ericsson | Support Option 1. With more than one beam per panel, the link-budget may display a greater SNR to some extent, but it is not necessarily required for an adequate link-budget. |
| Samsung | Option 1 based on our analysis for deployment scenario. We can compromise to QC’s proposal if the proposal is only applied to Scenario-B. Another option is to remove Scenario-B totally since we don’t see clear operator’s request on Scenario-B, if companies have difficulty to converge on Scenario-B. |
| Intel | Support Option 1. We don’t need more beams for higher SNR at the expense of delayed measurements since 1 RX beam still provides sufficient link budget. |
| Apple | Similar comments as Huawei. Number of Rx beam can be larger in Scenario B compared to scenario A, to ensure Rx beamforming gain. |
| CATT | We prefer option 1. |
| ZTE | Support Option 1. |

#### Issue 2-2-4: Bi-directional, Scenario-B

- Proposals and/or Observations

  - Option 1(CATT): The RX beam number can be larger than 1 RX beam per panel. E.g. [2] RX beams per panel can be used.

  - Option 2(Ericsson): RX beam number =1/panel; or 2 totally.

  - Option 3(Qualcomm): Number of Rx beams in FR2 HST is not fewer than 8

  - Option 4(ZTE): Wait for the detailed schemes determined in Channel and Scenario sessions

- Recommended WF

  - Continue discussion in the 1st round.

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| QC | Same comment as 2-2-3. |
| Nokia | The same comment as Issue 2-2-3. |
| Huawei | similar comments as issue 2-2-3. |
| Ericsson | Support Option 2 and Option 4. To our understanding, RX beam number =1/panel or 2 totally, we can wait for conclusion in scenario session. But Option 1 may be discussed further. We're open to discuss the issue if extra beams(still rather fewer than 8) are needed since we need to examine certain practical difficulties of RX beam direction in reality, such as whether the RX beam can't always correctly direct RRH's panel, e.g. varied Dmin in scenario-B & Bi-directional deployment or change between Bi-directional and uni-directional deployment. Also, Issue2-1-3 may need the use of extra beams. |
| Samsung | We can compromise to QC’s proposal if the proposal is only applied to Scenario-B. Another option is to remove Scenario-B totally since we don’t see clear operator’s request on Scenario-B, if companies have difficulty to converge on Scenario-B. |
| Intel | Support Option 2 Ok with Option 4 |
| Apple | Support option 4 |
| CATT | Support option 1 and fine with option 4. For bi-directional deployment, one wide beam is not enough for coverage. |
| ZTE | Support Option 4. |

### Sub-topic 2-3: Network assistance

Sub-topic description

Some of the companies are proposing that network signaling can be used to adapt the number of RX beam to be used in RRM requirement.

Open issues and candidate options before e-meeting:

Network assistance for reducing RX beam number:

- FFS: to be discussed depending on the outcomes of the deployment scenarios

#### Issue 2-3-1: Signalling of SSB index per RRH

- [Moderator]: Several companies are proposing either to discuss or to introduce signaling for SSB index per RRH. One ot the goals is to adapt UE RX beam scaling factor based on the number of SSB per RRH.
It is also noted that the priori RRH beam direction knowledge can help UE speed up beam discovery and neighboring cell detection by reducing Rx beam sweeping delay.

- Proposals and/or Observations

  - Proposal 1 (Apple): Enable network signaling of SSB index per RRH.

  - Proposal 2 (Qualcomm): Network can indicate different SSBs on adjacent RRHs having the same QCL property: signal the mapping between the repeated sets of beams from the adjacent RRHs

  - Proposal 3 (Ericsson): Scaling factor based on explicit signalling from network to UE or implicit signalling based on UE’s identification of SSB index/TCI-state, position of UE relative to RRHs and so on

- Recommended WF

  - Continue discussion in the 1st round.

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| QC | Need clarification for option 1: how do UE utilize the SSB index signaling? |
| Nokia | For all the 3 listed proposals, the intention of introducing signalling is clear, which is to reduce the number UE RX sweeping beams. However, the rationale beyond each proposal is different. To evaluate the potential benefits versus signalling overhead of each of the proposals, we appreciate if the proponents of each proposal could provide more details. As an example, it is not clear how the reduction in the number of UE RX beams is obtained in Proposal 1 by considering the SSB index per RRH (R4-2112091): • When the number of SSB per RRH is one, and unidirectional deployment is used, number of Rx beam = 1 • When the number of SSB per RRH is 2, and bi-directional deployment is used, number of Rx beam = 2. • When the number of SSB per RRH is 2 and unidirectional deployment is used, number of Rx beam can be 1 or 2 depending on the conclusion in deployment scenario discussion. • When the number of SSB per RRH is 4 and bi-directional deployment is used, number of Rx beam can be 2 or 4 depending on the conclusion in deployment scenario discussion. For Proposal 2, the signalling parameters are discussed in Issue 1-1-4. The amount of reduction in the number of UE RX beams with signalling should be significant, otherwise it defeats the purpose. Thus, we are open to further discuss on the proposed signalling mechanisms. |
| Huawei | Suggest to wait for the conclusion of RX beam number. |
| Ericsson | The sequence of SSB index of RRHs, the location of UE relative to RRHs, and the locations of RRHs on each side of the railway may all be incorporated in network signaling as we discovered. This kind of siganling can help UE to estimation of RX beam sweep more accurately. But, the solution need further studies, we see the different solution behind Option 1, Option 2 and Option3. |
| Intel | Prefer FFS all three proposals |
| Apple | In general, detailed network beam coverage information for each network beam can be helpful, such as Ds, Dmin, each beam coverage angle, beam overlapping etc. More detailed information can help UE to estimate Rx beam sweeping better. Option 1 provided an abstracted information with limited modification to current SSB position signaling. SSB position per RRH indicates how many network beams per RRH, and the sequence of SSB along the track. Combined with uni-directional and bi-directional signaling, this gives a general indication of Scenario A/B uni and bi-directional deployment as shown in the example bullets. |
| CATT | Discuss the signalling later. |
| QC | Same reply as issue 1-1-4, paste the comments below: Since some of the comments are related to our proposal in issue 2-3-1, we address both there. The proposal discussed in issue 2-3-1 is a relatively simpler signaling with little overhead, only signals the SSB index mapping across adjacent RRHs, while the proposals in this issue has more detailed information. Our response for the comments on the two proposals are listed below: - SSB index mapping across different RRHs: To Nokia: the benefit for neighboring cell detection and L1-RSRP is explained in our contribution. Since we may need to reduce cell identification time, this will be beneficial to speed up PSS/SSS detection. For L1-RSRP, as we explained in our contribution, in HST beam refinement becomes large overhead, and we might need UE to measure the L1-RSRP by fine beam to save beam refinement time after TCI state switch. In order to use fine beam to measure but still satisfy the legacy requirement which assumes rough beam measurement, the network assistant information is necessary. When UE is able to reduce beam refinement time after switching to new TCI states, it can benefit demod performance significantly especially for HST with short time spent on each TCI state. To Huawei: this signaling doesn’t mandate uniform deployment. The purpose is to allow network in uniform deployment cases to signal this useful information to UE. If the deployment is not uniform in some segments on the track route, network can leave this empty or fill just part of the entries that satisfies uniform deployment. To CATT, Ericsson and Samsung: the project and direction signaling probably required more discussion based on current received comment. Overhead is also larger. But the SSB index mapping is a relatively simple and low-overhead signaling, with relevant improvement as we explained in our contribution and above comment. We suggest to prioritize the SSB index mapping proposal. For the rest signaling, we can collect more comments and come up with more concise proposal probably for later release. - Projection and angle information To Nokia: let us explain the usage of each items below: • Distance between the projections of adjacent beam peaks on the track • The 6 dB beam-width projection on track By combining the two above, UE can predict the timing of next TCI state switch and perform beam refinement in advance. These two items are equivalent to the segment length and location of the track covered by each beam as you mentioned in the comment, UE can use any of them to run the prediction. • Beam peak direction angle relative to track This helps UE to decide which rough beams to use for beam discovery and narrow down the preferred subset of fine beams to perform beam refinement in a shorter time. |
| Nokia2 | It is the same as the comment in Issue 1-1-4. In response to QC’s comments, we thank for your feedback. We have further comments regarding the following statement: “In order to use fine beam to measure but still satisfy the legacy requirement which assumes rough beam measurement, the network assistant information is necessary.” With the introduction of signalling we expect it can satisfy more stringent requirements given that the signalling parameters (beam peak direction, 6 dB beam width, etc) can provide accurate directional information about RRH TX beams, which helps in finding the most precise fine beam immediately without the need of performing extensive beam search. It defeats the purpose of introducing signalling if it could only satisfy legacy requirements considering overhead. Could you elaborate on why introducing signalling can only satisfy legacy requirements? |

### CRs/TPs comments collection

For close-to-finalize WIs and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| XXX | Company A |
|  | Company B |
| YYY | Company A |
|  | Company B |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

| Sub-topic | Status summary |
| --- | --- |
| Sub-topic #2-1: General | Issue 2-1-1: RX beam number reduction Background: More detailed discussion of the issue took place during the GtW session and the agreements below were achieved. Agreements: - Agreements: - RX beam number for RRM requirements definition - Define two set of requirements for Scenario A and Scenario B in terms of number of RX beams per UE - Scenario A: [2] RX beams for all scenarios - Scenario B: [6] RX beams for all scenarios - FFS on feasibility and methods to differentiate scenarios from UE perspective - FFS if different UE capabilities shall be used for Scenario A and B support - Note: if there is insignificant difference between Scenario A and B requirements, then further discussion on unified requirements can take place. Recommendations for 2nd round: In the second round, the companies are welcomed to share further opinions about the issues marked FFS in the agreement. Issue 2-1-2: Unification of requirements Background: In Moderator’s opinion Proposal 1 and Proposal 2 may be still different because following the contribution [R4-2113272], Proposal 1: “Besides, UE need to consider the entire Dmin range between 10m and 150m. Therefore, when discussing number of Rx beams, the entire range has to be taken into consideration. A unified requirement set is needed for different Dmin.” It can be discussed further whether there is a difference in the second round. During the GtW discussion the following Note was made: “Note: if there is insignificant difference between Scenario A and B requirements, then further discussion on unified requirements can take place”. Tentative agreements: None Candidate options: - Option 1: Define unified requirement set for RX beam, considering different deployment and different Dmin. - Option 2: Decide number of RX beams based on Dmin range instead of scenarios. - Option 3: Network indicates the RX beam number difference in Scenario-A and Scenario-B Recommendations for 2nd round: It is recommended to continue the discission in the second round considering the outcomes of the GtW session. Issue 2-1-3: Impact of RRH position at one/both sides of rail track Background: The companies seem to agree that in Scenario-A whether RRH is in one side or two side of the track might not impact much. However, some companies think that RRM requirements can be impacted in Scenario-B. Tentative agreements: None Candidate options: - Option 1: RRM requirements are defined with scaling factor which is double of number of RX sweep number in scenario study. - Option 2: RRM requirements are defined with adapted scaling factor based on explicit or implicit signalling from network to UE. - Other options are not precluded. Recommendations for 2nd round: It is recommended to re/evaluate the proposals in the second round based on the agreed number of Rx beams from the GtW session. Issue 2-1-4: Impact of obstruction between RRH and UE Background: All comments indicate the support for Proposal 1. Tentative agreements: Impact of obstruction between RRH and UE on RRM e.g., beam management/RX beam number, doesn’t need to be discussed in this session. Recommendations for 2nd round: Confirm that tentative agreement is agreeable. |
| Sub-topic #2-2: Number of RX beams in HST FR2 deployment scenarios | Issue 2-2-1: Uni-directional deployment, Scenario-A Background: Almost all of the companies seem to agree that RX beam number can be 1 in uni-directional deployment, Scenario-A. Tentative agreements: Uni-directional deployment, Scenario-A, the RX beam number can be 1. Candidate options: Recommendations for 2nd round: Follow the GtW agreements, Issues 2-1-1. Issue 2-2-2: Bi-directional, Scenario-A Background: Almost all of the companies seem to agree that RX beam number can be 2 per UE in bi-directional deployment, Scenario-B if such a scenario is not precluded from scenario perspective. Tentative agreements: The RX beam number can be 1 RX beam per panel, 2 in total. Candidate options: Recommendations for 2nd round: Follow the GtW agreements, Issues 2-1-1. Issue 2-2-3: Uni-directional, Scenario-B Background: Many companies agree that the RX beam number can be 1 per panel. However, some companies think that a larger of UE RX beam might be needed in Scenario-B. Tentative agreements: None Candidate options: Recommendations for 2nd round: Follow the GtW agreements, Issues 2-1-1. Issue 2-2-4: Bi-directional, Scenario-B Background: Some of the companies have an opinion that 1 beam per UE panel might not e sufficient. However, there is no agreement on a particular number in the comments to the issue. Therefore, it is recommended to follow the agreements for the GtW. Tentative agreements: None Candidate options: Recommendations for 2nd round: Follow the GtW agreements, Issues 2-1-1. |
| Sub-topic #2-3: Network assistance | Issue 2-3-1: Signalling of SSB index per RRH Background: In general, the companies indicate a need for the further analysis of the proposed options. Due to the closeness of the subjects and duplicated replies, Moderator recommends merging of this issue and Issue 1-1-4(updated): Signalling of network assistance information. Tentative agreements: None Candidate options: - Option 2: Enable network signaling of SSB index per RRH - Option 3: Network can indicate different SSBs on adjacent RRHs having the same QCL property: signal the mapping between the repeated sets of beams from the adjacent RRHs - Option 4: Scaling factor based on explicit signalling from network to UE or implicit signalling based on UE’s identification of SSB index/TCI-state, position of UE relative to RRHs and so on Recommendations for 2nd round: Continue the discussion in the Issue 1-1-4(updated): Signalling of network assistance information. |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round (if applicable)

TBA

### Sub-topic 2-2: TBA

#### Issue 2-2-2: TBA

Agreements from round 1:

TBA

Candidate options:

- Option 1:

- Option 2:

Recommendations for 2nd round:

TBA

Contributor Comments:

| Company | Comments |
| --- | --- |
| XXX |  |
| YYY |  |
| ZZZ |  |

## Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

# Topic #3: RRC Idle/Inactive and connected state mobility requirements

Main technical topic overview. The structure can be done based on sub-agenda basis.

This topic is devoted to the discussion of mobility requirements in RRC_CONNECTED and RRC_IDLE/INACTIVE states, i.e., corresponding to the Sections 4-6 of TS 38.311.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2111955 | CATT | Discussion on RRC Idle/Inactive and connected state mobility requirements for HST RRM in FR2 Proposal 1: The cell selection criterion S is still the baseline. The requirement is still based on number of DRX cycle. Proposal 2: The same upper bound of DRX cycle in Rel-16 HST can be reused while scaling factor N1 = 1. If N1 increased, the upper bound of DRX cycle should be decreased. Requirements for long DRX configurations in CONNECTED state Proposal 3: For long DRX configuration in CONNECTED state, if DRX is larger than the upper bound, option 1 apply existing R16 requirement is reasonable. Handover Proposal 4: For handover, wait for the agreement on RX beam sweep number reduction. |
| R4-2112092 | Apple | Discussion on mobility requirement for FR2 HST Proposal 1: For idle/inactive mode cell reselection enhancement, extend the FR1 HST enhancement with N1 scaling factor, as shown in Table I. Proposal 2: If known cell for FR2 HST is defined, the FR2 handover known cell criterion can reused. Proposal 3: RRC re-establishment requirement can be enhanced as shown in Table II, where N1 depends on the number of Rx beam in general RRM requirement discussion. Serving cell; FR of target NR; Tidentify_intra_NR [ms]. SSB Ês/Iot (dB); cell; Known NR cell; Unknown NR cell. ≥ -8; FR1; MAX (200 ms, 5 x TSMTC); MAX (800 ms, 10 x TSMTC). ≥ -8; FR2; MAX(400ms, 5 x N1 x TSMTC); MAX (1000 ms, 10 x N1 x TSMTC)). < -8; FR1; N/A; 800 Note1. < -8; FR2; N/A; 3520 Note1. Note 1: The UE is not required to successfully identify a cell on any NR frequency layer when TSMTC > 20 ms and serving cell SSB Ês/Iot < -8 dB.. Proposal 4: Handover requirement can be enhanced depends on the number of Rx beam in general RRM requirement discussion. |
| R4-2112499 | CMCC | Discussion on mobility requirements for FR2 HST Proposal 1: for the enhancement of short DRX, the scaling factor M2 introduced for the enhancement of scaling factor 1.5 specified in Rel-16 NR HST can be reused for FR2 HST: M2 = 1.5 if SMTC periodicity > [40] ms, otherwise M2=1. [Moderator]: Two proposals below are related to Measurement Procedures and are recommended for treatment in RRM-2. Proposal 2: it is necessary to perform enhancement/reduction on Mpss/sss and Mmeas_period. Proposal 3: compared with the solution to perform enhancement on the number of samples and RX beam sweeping respectively, it is preferred to have general reduction of Mpss/sss and Mmeas_period, and the details of the number of samples and the RX beam sweeping factor can be left to UE implementation. Proposal 4: for the enhanced solution for FR2 cell-reselection requirements, the enhancement introduced in Rel-16 HST WI, e.g. the number of samples, the enhancement of M2, can be reused. Observation 1: with the enhancement from FR1 HST WI, i.e. M2=1, the number of samples is reduced from 36 to 8 for 0.32 DRX cycle, the FR2 cell re-selection delay still cannot support 350km/h with 700m ISD. Proposal 5: to support the velocity of 350km/h, in addition to reuse the enhancement of Rel-16 FR1 HST, it is necessary to perform enhancement on the scaling factor N1, e.g. adopt small values of N1. |
| R4-2113215 | ZTE Corporation | Discussion on RRC Idle Inactive and Connected state mobility requirements for HST FR2 Proposal 1: Firstly the upper bound of DRX cycle should be determined. Proposal 2: Requirements enhancement should be defined for small DRX cycle ≤ the upper bound. For the DRX cycle > the upper bound, reuse existing Rel-16 requirements, but network should avoid configured long DRX configuration for HSR FR2 enabled case. Observation 1: The scaling factor for handover to unknown cell needs to be aligned with the conclusion from RX beam number discussion. Observation 2：Limits for intra-frequency/inter-frequency SMTC periodicity can be relaxed for the small scaling factor. Proposal 3: Define criteria of known cell for FR2. Proposal 4: Enhance requirements for unknown cell and align with the RX beam sweep number reduction. Proposal 5: Acquiring the HST FR2 requirements based on the HST FR1 enhancement, i.e. introducing scaling factor N1 upon the HST FR1 enhancement in Rel-16. |
| R4-2113273 | OPPO | Discussion on mobility requirements for FR2 HST Proposal 1: Use Rx beam scaling factor N1=2 as a reference for the analysis of RRM requirements. Proposal 2: When SMTC <= 40ms, use the scaling factor M2=M3=M4=1. Proposal 3: The number of samples X should be reduced for FR2 HST: - For cell detection procedure, X should be further reduced to 4. - For measurement procedure, X=1 in FR1 HST could be reused - For evaluation procedure, X=3 in FR1 HST could be reused. Proposal 4: The upper bound of DRX cycle should be discussed after the number of samples X and Rx beam scaling factor N1 are determined. Proposal 5: Support option 1, apply existing R16 requirement for long DRX configuration in RRC CONNECTED state. Proposal 6: If the handover command is allowed before a valid measurement report is sent by the UE, handover requirements when the target cell is unknown should be considered. Proposal 7: If the scenario of unknown target cell is considered, the following enhancements should be considered for handover requirements: - The Rx beam scaling factor could be reduced to 2 - Restrictions on SMTC/SSB periodicity, e.g. [40]ms |
| R4-2113328 | Ericsson | RRC Idle/Inactive and connected state mobility requirements for HST FR2 Proposal 1: The target FR2 cell is known if it has been meeting the relevant cell identification requirement during the last 5 seconds; otherwise, it is unknown. Proposal 2: Relevant content of known FR2 NR cell in Table 6.2.1.2.1-1 is updated as follows: Serving cell; FR of target NR; Tidentify_intra_NR [ms]. SSB Ês/Iot (dB); cell; Known NR cell; Unknown NR cell. ≥ -8; FR1; MAX (200 ms, 5 x TSMTC); MAX (800 ms, 10 x TSMTC). ≥ -8; FR2; MAX (200 ms, 5 x N x TSMTC); MAX (1000 ms, [80] x TSMTC)). < -8; FR1; N/A; 800Note1. < -8; FR2; N/A; 3520Note1. Note 1: The UE is not required to successfully identify a cell on any NR frequency layer when TSMTC > 20 ms and serving cell SSB Ês/Iot < -8 dB. N is scaling factor for FR2, which is enhanced with configured high speed signalling.. Proposal 3: Even while Handover has not specifically requested a reduction in the number of RX beam sweeps, it, like other items, can benefit from the reduction. It appears reasonable to minimise interruptions caused by handover. Proposal 4: Support Option2 but can compromise to Option 1 and Option3.To our understanding the only difference between Option 1 and Option3 is validity of value of upper bound, we suggest [80] ms as upper bound of DRX cycle. Observation 1: For Cell reselection requirements in IDLE/INACTIVE mode, we need to be aware of power class if a new power class is decided for HST FR2 and note is needed to indicate the power class which is compatible with HST FR2. Proposal 5: Enhancements of cell reselection (in idle mode) measurement for intra-frequency NR cells is shown in below table: Table : Tdetect,NR_Intra, Tmeasure,NR_Intra and Tevaluate,NR_Intra for FR2 HST DRX cycle length [s]; Tdetect,NR_Intra [s] (number of DRX cycles); Tmeasure,NR_Intra [s] (number of DRX cycles); Tevaluate,NR_Intra [s] (number of DRX cycles). 0.32; 2.56 x N1 x M2 (8 x N1 x M2); 0.32 x N1 x M3 (1 x N1 x M3); 0.96 x N1 x M4 (3 x N1 x M4). 0.64; 5.12 x N1 (8 x N1); 0.64 x N1 (1 x N1); 1.92 x N1 (3 x N1). 1.28; 8.96 x N1 (7 x N1); 1.28 x N1 (1 x N1); 3.84 x N1 (3 x N1). 2.56; 58.88 x N1 (23 x N1); 2.56 x N1 (1 x N1); 7.68 x N1 (3 x N1). Note 1: when SMTC < = 40 ms, M2 = M3 = M4 = 1; and when SMTC > 40 ms, M2 = 1.5, M3 = M4 = 2 Note 2: This table is applicable for power class= TBD. |
| R4-2113835 | Huawei, HiSilicon | Discussion on RRC Idle/Inactive and connected state mobility requirements for HST in FR2 Proposal 1: Enhance RRM measurements requirements for small DRX (e.g., 320ms) in idle/inactive mode. Proposal 2: The existing FR2 handover delay when target cell is known can be applicable in high speed scenario. RRC connection Re-establishment Proposal 3: The existing definition of known cell for RRC connection re-establishment can be reused for FR2 HST. Proposal 4: The need of enhancement of RRC connection re-establishment delay for FR2 HST scenario is not clear. |
| R4-2114467 | Nokia, Nokia Shanghai Bell | Detailed simulation analysis for FR2 HST Observation 1: For uni-directional Scenarios A and B, handovers due to ping pong are reduced when DRX > 40 ms. Observation 2: For unidirectional Scenarios A and B, inter-cell mobility failure rates are high for DRX ≥ 256 ms when HST travels in the opposite direction as the beams as compared with case where HST travels in the same direction. Observation 3: For unidirectional Scenarios A and B, time-of-outage is high for DRX ≥ 256 ms when HST travels in the opposite direction as the beams as compared with case where HST travels in the same direction. Observation 4: For unidirectional Scenarios A and B, SINR values are low and they can not guarantee performance when DRX > 160 ms. Observation 5: For unidirectional Scenarios A and B, Ds_Offset increases with DRX. Observation 6: For uni-directional Scenario B, increasing the number of beams would reduce Ds_Offset when DRX cycle is short. Observation 7: For bi-directional Scenario B, ping pongs are reduced when DRX > 40 ms. Observation 8: For bi-directional Scenario B, inter-cell mobility failure rates are high for DRX ≥ 160 ms. Observation 9: For bi-directional Scenario B, time-of-outage is high for DRX ≥ 160 ms. |
| R4-2114586 | Nokia, Nokia Shanghai Bell | Discussion on RRM measurement requirements for FR2 HST Proposal 1: For SA intrafrequency measurements, Option 3 - FFS the upper bound of DRX cycle which is determined based on the maximum target CPE speed. Enhancements are defined for small DRX cycle ≤ the upper bound; for DRX cycle > the upper bound, existing Rel-16 FR2 requirements are reused. [Moderator]: This proposal from AI 9.9.4.6 is taken into account here due to it’s direct relevance to the Topic #3. Proposal 2: For FR2 HST L1-RSRP measurement enhancements, K can adopt the same methodology as FR1 HST. Observation 1: For FR2 HST L1-RSRP measurement enhancements, N depends on the number of RX beams under discussions. Observation 2: For cell re-selection with speed up to 350 km/h, it is feasible to apply the same methodology for NR FR1 HST to FR2. Proposal 3: For FR2 HST, the cell reselection requirements are enhanced according to Table 1, where the scaling factor N1 is FFS pending the outcome of the number of RX beam discussions. Table 4: Tdetect,NR_Intra, Tmeasure,NR_Intra and Tevaluate,NR_Intra for FR2 HST DRX cycle length [s]; Tdetect,NR_Intra [s] (number of DRX cycles); Tmeasure,NR_Intra [s] (number of DRX cycles); Tevaluate,NR_Intra [s] (number of DRX cycles). 0.32; 2.56 x N1 x M2 (8 x N1 x M2); 0.32 x N1 x M3 (1 x N1 x M3); 0.96 x N1 x M4 (3 x M4). 0.64; 5.12 x N1 (8 x N1); 0.64 x N1 (1 x N1); 1.92 x N1 (3 x N1). 1.28; 8.96 x N1 (7 x N1); 1.28 x N1 (1 x N1); 3.84 x N1 (3 x N1). 2.56; 58.88 x N1 (23 x N1); 2.56 x N1 (1 x N1); 7.68 x N1 (3 x N1). Note 1: when SMTC < = 40 ms, M2 = M3 = M4 = 1; and when SMTC > 40 ms, M2 = 1.5, M3 = M4 = 2. [Moderator]: This proposal from AI 9.9.4.6 is taken into account here due to it’s direct relevance to the Topic #3. |
| R4-2114188 | Intel | Proposal 2: For FR2 HST consider upper bound of DRX cycle equal to 60ms Proposal 3: For DRX cycle > the upper bound existing Rel-16 FR2 requirements are reused |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 3-1: CONNECTED state mobility

Sub-topic description 
The sub-topic covers the issues related to RRC Connection Mobility Control, specifically on DRX configuration and RRC RRC re-establishment requirements.

Open issues and candidate options before e-meeting:

Way Forward on CONNECTED state mobility after RAN4#98-bis-e meeting [R4-2105794]:

- RRC CONNECTED mode requirements for DRX (based on GtW):

  - Define requirements for the short DRX configurations (≤ [80] ms)

  - FFS whether to define requirements for long DRX configurations (> [80] ms)

    - Option 1: Do not define any requirements

    - Option 2: Apply existing R16 FR2 requirements

    - Option 3: Apply requirements for short DRX configurations

    - Option 4: Define enhanced requirements

- Scaling factor N

  - Option 1: For FR2 HST, the FR2 scaling factor can be reduced as:

    - For uni-directional deployment, N=[1]

    - For bi-direcitonal deployment, N=[2].

  - Option 2: Keep existing RX beam number unchanged

  - Other options are not precluded

  - FFS: whether and what network assisted information is needed to reduce the number of RX beams

- FFS: Connection Mobility Control - RRC Release with Redirection

WF on Requirements for long DRX configurations in CONNECTED state listed in the WF [R4-2108342] after RAN4#99-e:

- Option 1: Apply existing R16 requirements

- Option 2: Apply requirements for short DRX configurations

- Option 3:

  - FFS the upper bound of DRX cycle which is determined based on the maximum target CPE speed.

  - Enhancements are defined for small DRX cycle ≤  the upper bound; for DRX cycle > the upper bound, existing Rel-16 FR2 requirements are reused

WF on Connected state mobility requirements after RAN4#99-e [R4-2108342]:

- Connection Mobility Control

  - RRC Release with Redirection

    - Not applicable to FR2 HST

  - RRC re-establishment

    - FFS: Definition of criteria of known cell for FR2

    - FFS: Enhancement of requirements on Connection Mobility Control - RRC re-establishment.

Restriction on SMTC periodicity in measurement requirements:

- FFS: the upper bound of SMTC periodicity, with [40] ms as a basis

- FFS: impact on the specification if upper bound of SMTC periodicity is agreed.

#### Issue 3-1-1: DRX upper bound

- Proposals and/or Observations

  - Proposal 1(ZTE, Nokia): Firstly the upper bound of DRX cycle should be determined.

  - Proposal 2(Ericsson): [80] ms as upper bound of DRX cycle.

  - Proposal 3(Intel): [60] ms as upper bound of DRX cycle.

- Recommended WF

  - Continue the discussion in the 1st round.

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| QC | We support both option 2 and 3. |
| Nokia | Proposal 1 can be used as a starting point. Our system simulation results show the upper bound of DRX up to 256 ms is feasible. |
| Huawei | Support option 1 and propose [160]ms. In our paper [R4-2113837], we propose [160]ms as upper bound DRX cycle. To our knowledge, the maximum DRX cycle length used in realistic high speed scenario is 160ms. For proposal 3, 60ms is not available for DRX cycle configuration. Is it be [64]ms? |
| Ericsson | we suggest upper limit of DRX cycle = 80ms. Even if a shorter DRX cycle ensures greater performance, it should not be constrained too much to limit application options in the network. On other hands, a longer DRX cycle may work in some cases also depend on other factors. We're willing to talk about the potential of a somewhat longer DRX cycle. |
| Samsung | P2 and P3 are okay to us. Based on the assumption of applying existing non-HST optimized Rel-15/16 requirement for DRX cycle longer than 60ms. Based on all companies’ contribution on deployment scenario analysis, FR2 HST UE could face a distance of more than 100m with great signal strength variance, which means UE’s last measurement occasion can be obsolete if DRX cycle longer than 60 or 80ms. |
| Intel | Support Proposal 3. Ok with Proposal 2. To Huawei: Based on 38.331 60ms is in the list of drx-LongCycleStartOffset |
| CATT | Fine with Proposal 1 and Proposal 2. |
| ZTE | Support Proposal 1 and Proposal 2. |

#### Issue 3-1-2: Requirements for long DRX

- [Moderator]: Long DRX are the DRX above the upper DRX bound.

- Proposals and/or Observations

  - Option 1(CATT, OPPO, Ericsson, Qualcomm, Nokia, Intel): Apply existing R16 requirement

  - Option 1a(ZTE): Apply existing R16 requirement, but network should avoid configured long DRX configuration for HSR FR2 enabled case.

  - Option 2(Ericsson): Apply requirements as for short DRX configurations

- Recommended WF

  - Agree on Option 1 in the 1st round.

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| QC | Support option 1. |
| Nokia | Support Option 1. |
| Huawei | Support option 1. The similar rule as R16 HST. |
| Ericsson | We can compromise to support Option 1. |
| Samsung | Option 1. |
| Intel | Support Option 1. |
| Apple | Support option 1 |
| CATT | Support option 1. |
| ZTE | To push the progress, we can compromise to Option 1. |

#### Issue 3-1-3: Enhancements for short DRX

- [Moderator]: Short DRX are the DRX below the upper DRX bound.

- Proposals and/or Observations

  - Option 1(CMCC): The scaling factor M2 introduced for the enhancement of scaling factor 1.5 specified in Rel-16 NR HST can be reused for FR2 HST: M2 = 1.5 if SMTC periodicity > [40] ms, otherwise M2=1.

  - Option 2(QC): removing relaxation factor of 1.5 from the legacy requirement and keep all other scaling factors the same if network can provide assistant information to UE as proposed in issue 2-3-1 (or 1-1-4).

- Recommended WF

  - Continue the discussion in the 1st round.

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| QC | Our analysis shows that 4000ms cell identification time is sufficiently small to maintain connectivity. This identification time can be achieved by removing the factor of 1.5 or change the scaling factor from 24 to 18. |
| Nokia | Option 1 can be used as a baseline for enhancements. |
| Huawei | Support option 1, reusing M2 can speed up PSS/SSS detection and measurement. |
| Ericsson | Support Option 1. |
| Samsung | Option 1 can be regarded as baseline which help 1.5 times reduction if SMTC is properly configured with smaller period, which is practical configuration in most of cases. |
| Apple | Support option 1 |
| CATT | Option 1 is fine. |
| CMCC | We are OK with option 1. But except reusing M2, whether we also need to have enhancement/reduction on Mpss/sss and Mmeas_period? |
| QC | Given the GTW conclusion, we need to update our proposal. We agree number of Rx = 6 based on the legacy requirement with 1.5 relaxation factor, as we clarified in GTW discussion. Hence our proposal becomes keeping the 1.5 relaxation factor. |
| ZTE | Support Option 1. |

#### Issue 3-1-4: SMTC periodicity

- Proposals and/or Observations

  - Proposal 1(CMCC): It is not preferred to have restriction on SMTC periodicity for FR2 HST.

  - Proposal 2(Qualcomm): Only consider SMTC period <= 40ms.

- Recommended WF

  - Continue the discussion in the 1st round.

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| QC | Our motivation is to limit the SMTC and DRx for deriving the requirement in mobility analysis. Hence proposal 2 can revise as deriving the requirement under the assumption of SMTC period <= 40ms, but not restricting SMTC period in FR2 HST. |
| Nokia | Proposal 1 is Ok. |
| Ericsson | Agree with Proposal 1. Fine with QC’s explanation. |
| Samsung | P2 are okay to us, i.e., no need to define requirement for SMTC period longer than 40ms, which does not conflict with P1. |
| Intel | Agree with QC. We need the limits for requirements derivation. 40ms can be considered |
| Apple | Similar to FR1 HST, enhanced requirement is applied to SMTC <=40ms. This align with 3.2.1.3 option 1 |
| CATT | Prefer proposal 1. And similar enhancement of the requirements of SMTC period 40ms as the boundary. |
| CMCC | Proposal 1 is proposed not to have restriction on the network configuration. But for the requirements derivation, we are OK with proposal 2. |
| ZTEE | Support Proposal 1. |

#### Issue 3-1-5: Connection Re-establishment requirments

- [Moderator]:
For reference, following TS 38.133:
If the UE is not configured with intra-frequency NR carrier for RRC re-establishment then Tidentify_intra_NR=0; otherwise Tidentify_intra_NR shall not exceed the values defined in Table 6.2.1.2.1-1.

- Proposals and/or Observations

  - Proposal 1(Apple): Enhanced as shown in Table II, where N1 depends on the number of Rx beam in general RRM requirement discussion

Table II: Time to identify target NR cell for RRC connection re-establishment to NR intra-frequency cell

| Serving cell | FR of target NR | Tidentify_intra_NR [ms] |  |
| --- | --- | --- | --- |
| SSB Ês/Iot (dB) | cell | Known NR cell | Unknown NR cell |
| ≥ -8 | FR1 | MAX (200 ms, 5 x TSMTC) | MAX (800 ms, 10 x TSMTC) |
| ≥ -8 | FR2 | MAX(400ms, 5 x N1 x TSMTC) | MAX (1000 ms, 10 x N1 x TSMTC)) |
| < -8 | FR1 | N/A | 800 Note1 |
| < -8 | FR2 | N/A | 3520 Note1 |
| Note 1: The UE is not required to successfully identify a cell on any NR frequency layer when TSMTC > 20 ms and serving cell SSB Ês/Iot < -8 dB. |  |  |  |

  - Proposal 2(Ericsson): Relevant content of known FR2 NR cell in Table 6.2.1.2.1-1 is updated as follows:
Table 6.2.1.2.1-1: Time to identify target NR cell for RRC connection re-establishment to NR intra-frequency cell

| Serving cell | FR of target NR | Tidentify_intra_NR [ms] |  |
| --- | --- | --- | --- |
| SSB Ês/Iot (dB) | cell | Known NR cell | Unknown NR cell |
| ≥ -8 | FR1 | MAX (200 ms, 5 x TSMTC) | MAX (800 ms, 10 x TSMTC) |
| ≥ -8 | FR2 | MAX (200 ms, 5 x N x TSMTC) | MAX (1000 ms, [80] x TSMTC)) |
| < -8 | FR1 | N/A | 800Note1 |
| < -8 | FR2 | N/A | 3520Note1 |
| Note 1: The UE is not required to successfully identify a cell on any NR frequency layer when TSMTC > 20 ms and serving cell SSB Ês/Iot < -8 dB. N is scaling factor for FR2, which is enhanced with configured high speed signalling. |  |  |  |

  - Proposal 3 (Huawei): The need of enhancement of RRC connection re-establishment delay for FR2 HST scenario is not clear. The existing definition of known cell for RRC connection re-establishment can be reused for FR2 HST.

- Recommended WF

  - Continue the discussion in the 1st round.

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| QC | We support proposal 3. We don’t see why RAN4 needs to discuss RRC connection re-establishment enhancement. In which circumstances can we have RRC connection re-establishment while UE is moving in 350km/h on the track? RLF or UE disconnection should be rare event for HST. |
| Huawei | Support option 3. Option 3 means it is no need to enhance re-establishment for FR2 except RX beam number (if there is conclusion). FR2 HST scenario deployment is SFN (single frequency network) where multiple RRHs belong to one RRU. If 4 RRHs set up one cell, then according to typical deployment (scenario A or scenario B) the radius of serving cell is 2800m. 350kmph is a reference maximum train speed for FR2 HST. Then a UE will take 28.8s to across a cell. Although the existing RRC connection re-establishment delay is relative long, the cell coverage is large due to SFN deployment. Therefore we don’t observe the need of enhancement of RRC connection re-establishment delay for FR2 HST scenario. |
| Ericsson | For Known NR cell, we can support MAX (400 ms, 5 x N1 x TSMTC) . For unknow NR cell, we suppose that unknown NR cell is rare in HST FR2 and the requirements can be kept. Related to proposal 3, there isn’t available known cell for FR2’s RRC connection re-establishment, the block in current table is blank. Our intention is to close the loophole, not to enhance RRC connection re-establishment. |
| Samsung | Agree with Option 3 and Huawei’s comment above. |
| Apple | Proposal 1 has two aspects: (1) reduce N=8 to N1, where N1 is Rx beam pending discussion. (2) extend to known cell case. These two aspects can be discussed separately. |
| ZTE | We support the aspect (1) in Option 1. For aspect (2), which can be further discussed. |

### Sub-topic 3-2: Handover

Sub-topic description

The sub-topic is devoted to the HO requirements, including HO to known and unknow cells.

Open issues and candidate options before e-meeting:

Way Forward on CONNECTED State Mobility after RAN4#98-e-bis:

- Handover:

  - Existing FR2 requirement should be applicable to the HST FR2 deployments when the target cell is known.

  - FFS: Handover requirements when the target cell is unknown

  - FFS: a need to address the potential change in the scaling factor 8.

- Connection mobility control - RRC re-establishment:

  - FFS: whether the existing requirements can work for FR2 HST.

#### Issue 3-2-1: Criteria of known cell for FR2

- Proposals and/or Observations

  - Proposal 1 (ZTE): Define criteria of known cell for FR2.

  - Proposal 2 (Ericsson): The target FR2 cell is known if it has been meeting the relevant cell identification requirement during the last 5 seconds; otherwise, it is unknown.

- Recommended WF

  - Define the criteria of know cell for FR2 in the 1st round.

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| QC | We can discuss this but the expectation is that this won’t be very relevant. The trajectory is known hence all the cells on the route should be predictable. |
| Ericsson | We suggest to reuse 5 seconds for FR1.There is no need to abbreviate the known condition which is similar to the issue of known TCI-state. |
| Intel | In uni-directional deployment when the UE is moving away from the serving beam it cannot see the next RRH until it passes it. So, when it passes the first RRH of the target cell it is formally in unknown conditions, since none of the target cell SSB was detectable before. At the same time we agree with QC’s comment, the trajectory is known and all the cells on the route should be predictable. Need to further discuss on how to capture it in the spec. |
| ZTE | Maybe Proposal 1 can be removed, we gave such proposal for the issue of RRC re-establishment in our tdoc. Sorry, we should help moderator to further check. We support Proposal 2, which is aligned with FR1. |

#### Issue 3-2-2: Handover requirement

- [Moderator]: At RAN4#98-e-bis it was agreed that
Existing FR2 requirement should be applicable to the HST FR2 deployments when the target cell is known.

- Proposals and/or Observations

  - Proposal 1 (CATT, Apple): Wait for the agreement on RX beam sweep number reduction.

  - Proposal 1a (Ericsson): Minimise interruptions caused by handover by alignment with number of RX beam sweeps.

  - Observation 1 (ZTE): Limits for intra-frequency/inter-frequency SMTC periodicity can be relaxed for the small scaling factor.

  - Proposal 2 (Apple, Huawei): If known cell for FR2 HST is defined, the FR2 handover known cell criterion can be reused.

- Recommended WF

  - Agree that the requirement should be enhanced to align with number of RX beam sweeps.

  - Continue the discussion of other possible enhancements in the 1st round.

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| QC | Support proposal 2. |
| Huawei | Support proposal 2. In high speed scenario, the common case is that target cell is known, as the railway track is fixed and ANR relation is remained in UE. For the known target cell, the handover delay is rough estimated as 102ms (assuming SMTC is 40ms). With 350km/h velocity, the moved distance is about 10m. In realistic network deployment, the handover range can cover such distance. From the coarse evaluation, the existing FR2 handover delay when target cell is known can be applicable in high speed scenario. |
| Ericsson | Agree with Recommended WF. We can reuse number of RX beam sweeps which is decided by other issue. |
| Samsung | Agree with both P1 and P2. For the number of RX beam sweeping, recommended WF is okay. |
| Intel | Agree with Proposals 1 and 1a |
| Apple | Agree with WF. Support proposal 2. |
| CATT | Agree with Recommended WF. |
| ZTE | Agree with the recommended WF. |

#### Issue 3-2-3: Handover to unknown cell

- Proposals and/or Observations

  - Proposal 1(OPPO): If the handover command is allowed before a valid measurement report is sent by the UE, handover requirements when the target cell is unknown should be considered.

- Recommended WF

  - Companies are invited to discuss in the 1st round the relevance of the HO to unknown cell in HST FR2 deployment.

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| QC | How can this happen in HST? The trajectory is deterministic. |
| Huawei | In high speed scenario, the common case is that target cell is known, as the railway track is fixed and ANR relation is remained in UE. We suggest not to define requirements for unknown handover. |
| Samsung | We see no necessity to consider HO to unknown cell. |
| Intel | As we mentioned in our comment for Issue 3-2-1, in uni-directional scenario when the UE moving away from serving beam passes the first RRH of the target cell the criteria of known cell is not fullfiled |
| ZTE | Maybe firstly we need to decide whether considering handover to unknown cell. Several companies belive it will never happen. |

#### Issue 3-2-4: Handover requirement for unknown cell

- Proposals and/or Observations

  - Observation 1(ZTE): The scaling factor for handover to unknown cell needs to be aligned with the conclusion from RX beam number discussion.

  - Proposal 1 (ZTE): Enhance requirements for unknown cell and align with the RX beam sweep number reduction.

  - Proposal 2 (OPPO): the following enhancements should be considered for handover requirements:

    - The Rx beam scaling factor could be reduced to 2

    - Restrictions on SMTC/SSB periodicity, e.g. [40]ms

- Recommended WF

  - Continue the discussion in the 1st round.

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| QC | We don’t think unknown cell handover is a common case in HST. |
| Ericsson | Support Proposal 1. Handover to unknown cell is rare case in HST surely, but here is using same scaling factor as the one in other issues. |
| Samsung | Consider Issue 3-2-3 firstly. If enhanced requirement is confirmed to be introduced, P1 is good enough for the enhancement for this rare case. |
| Intel | Agree with Proposal 1. However, we are not sure whether unknown cell assumption is reasonable in such deterministic scenario as FR2 HST. |
| ZTE | If we decide to consider the handover to unknown cell, we support Proposal 1 and Ovservation 1. |

### Sub-topic 3-3: IDLE state mobility

Sub-topic description:

For reference, in Rel-16 FR1 HST, Table 4.2.2.3-1 is the requirements in legacy FR1, whereas Table 4.2.2.3-2 is the enhanced requirements for FR1 HST in Rel-16.

Table 4.2.2.3-1: Tdetect,NR_Intra, Tmeasure,NR_Intra and Tevaluate,NR_Intra

| DRX cycle length [s] | Scaling Factor (N1) |  | Tdetect,NR_Intra [s] (number of DRX cycles) | Tmeasure,NR_Intra [s] (number of DRX cycles) | Tevaluate,NR_Intra [s] (number of DRX cycles) |
| --- | --- | --- | --- | --- | --- |
|  | FR1 | FR2Note1 |  |  |  |
| 0.32 | 1 | 8 | 11.52 x N1 x M2 (36 x N1 x M2) | 1.28 x N1 x M2 (4 x N1 x M2) | 5.12 x N1 x M2 (16 x N1 x M2) |
| 0.64 |  | 5 | 17.92 x N1 (28 x N1) | 1.28 x N1 (2 x N1) | 5.12 x N1 (8 x N1) |
| 1.28 |  | 4 | 32 x N1 (25 x N1) | 1.28 x N1 (1 x N1) | 6.4 x N1 (5 x N1) |
| 2.56 |  | 3 | 58.88 x N1 (23 x N1) | 2.56 x N1 (1 x N1) | 7.68 x N1 (3 x N1) |
| Note 1: Applies for UE supporting power class 2&3&4. For UE supporting power class 1 or 5, N1 = 8 for all DRX cycle length. Note 2: M2 = 1.5 if SMTC periodicity of measured intra-frequency cell > 20 ms; otherwise M2=1. |  |  |  |  |  |

Table 4.2.2.3-2: Tdetect,NR_Intra, Tmeasure,NR_Intra and Tevaluate,NR_Intra for UE configured with highSpeedMeasFlag-r16 (Frequency range FR1)

| DRX cycle length [s] | Tdetect,NR_Intra [s] (number of DRX cycles) | Tmeasure,NR_Intra [s] (number of DRX cycles) | Tevaluate,NR_Intra [s] (number of DRX cycles) |
| --- | --- | --- | --- |
| 0.32 | 2.56 x M2 (8 x M2) | 0.32 x M3 (1 x M3) | 0.96 x M4 (3 x M4) |
| 0.64 | 5.12 (8) | 0.64 (1) | 1.92 (3) |
| 1.28 | 8.96 (7) | 1.28 (1) | 3.84 (3) |
| 2.56 | 58.88 (23) | 2.56 (1) | 7.68 (3) |
| Note 1: when SMTC < = 40 ms, M2 = M3 = M4 = 1; and when SMTC > 40 ms, M2 = 1.5, M3 = M4 = 2 |  |  |  |

Open issues and candidate options before e-meeting:

Way Forward on IDLE/INACTIVE State Mobility at RAN4#98-e-bis:

- IDLE/INACTIVE mode requirements:

  - The companies are encouraged to provide their views on the following options and share their proposals on possible enhancements:

    - Option 1: Reuse existing Rel-16 requirements

    - Option 2: Study and define enhancements to support FR2 HST condition

    - Option 2a: For FR2 HST, the cell reselection requirements are enhanced according to Table (Tdetect,NR_Intra, Tmeasure,NR_Intra and Tevaluate,NR_Intra for FR2 HST), where N1 ≤ 4:

  - FFS: the way how IDLE/INACTIVE state can be used in HST FR2 deployment

Cell reselection enhancements in IDLE/INACTIVE mode based on RAN4#99-e WF:

- Cell reselection requirements in IDLE/INACTIVE mode are enhanced

- FFS enhancement details

  - upper bound of DRX cycle

  - scaling factor N1

  - others are not precluded

#### Issue 3-3-1: The upper bound of DRX cycle

- Proposals and/or Observations

  - Proposal 1(CATT): The same upper bound of DRX cycle in Rel-16 HST can be reused while scaling factor N1 = 1. If N1 increased, the upper bound of DRX cycle should be decreased.

  - Proposal 2 (OPPO): Should be discussed after the number of samples X and Rx beam scaling factor N1 are determined.

  - Proposal 3 (Huawei, Qualcomm): Enhance RRM measurements requirements for small DRX (e.g., 320ms) in idle/inactive mode.

- Recommended WF

  - Continue the discussion in the 1st round.

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| QC | Support option 3. What’s the use cases for longer iDRx cycles? |
| Nokia | Support proposals 1 and 2 |
| Huawei | Support proposal 3: As FR2 HST CPE is a dedicated equipment, the power consumption is not a bottleneck, the motivation of using long DRX in idle mode is not appealing. Moreover the drawback of long DRX is outstanding, as it would result in mobility performance degradation. To maintain the specification completed, the legacy requirements are reused for long DRX in HST. |
| Ericsson | We can refer to table in Issue 3-3-3. |
| Samsung | P3 with 320ms DRX cycle is used as baseline for following analysis and requirement definition. |
| CATT | In general, the HST focuses on short DRX cycle. These options are not totally mutually exclusive. The common sense is the same. We are fine to determine the upper bound of DRX cycle after the RX beam scaling. |
| ZTE | Support Proposal 1 and Proposal 2. |

#### Issue 3-3-2: Cell selection

- [Moderator]:

  - Note on S criterion presented in contribution R4-2111955: For intra-frequency measurement, if the serving cell fulfills Srxlev > SIntraSearchP and Squal > SIntraSearchQ, the UE may not to perform intra-frequency measurements, otherwise, the UE shall perform intra-frequency measurements.

- Proposals and/or Observations

  - Proposal 1(CATT): The cell selection criterion S is still the baseline. The requirement is still based on number of DRX cycle.

- Recommended WF

  - Continue the discussion in the 1st round.

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| Huawei | In general, proposal 1 is ok. |
| Samsung | Agree with P1, since we have no scope planned to change existing cell selection criterion S. |
| CATT | It is general and basic. Agree to come to the dedicated issue directly. |

#### Issue 3-3-3: Scaling factor N1

- [Moderator]: 
The most of the companies seems to agree that scaling factor N1 shall be introduced in HST FR2 in addition to HST FR1 requirements:
Tdetect,NR_Intra, Tmeasure,NR_Intra and Tevaluate,NR_Intra for FR2 HST

| DRX cycle length [s] | Tdetect,NR_Intra [s] (number of DRX cycles) | Tmeasure,NR_Intra [s] (number of DRX cycles) | Tevaluate,NR_Intra [s] (number of DRX cycles) |
| --- | --- | --- | --- |
| 0.32 | 2.56 x N1 x M2 (8 x N1 x M2) | 0.32 x N1 x M3 (1 x N1 x M3) | 0.96 x N1 x M4 (3 x M4) |
| 0.64 | 5.12 x N1 (8 x N1) | 0.64 x N1 (1 x N1) | 1.92 x N1 (3 x N1) |
| 1.28 | 8.96 x N1 (7 x N1) | 1.28 x N1 (1 x N1) | 3.84 x N1 (3 x N1) |
| 2.56 | 58.88 x N1 (23 x N1) | 2.56 x N1 (1 x N1) | 7.68 x N1 (3 x N1) |
| Note 1: when SMTC < = 40 ms, M2 = M3 = M4 = 1; and when SMTC > 40 ms, M2 = 1.5, M3 = M4 = 2 |  |  |  |

- Proposals and/or Observations

  - Proposal 1 (Apple): Define N1 scaling factor for each DRX cycle (add new column in the table)

  - Proposal 2 (OPPO): Use Rx beam scaling factor N1=2 as a reference for the analysis of RRM requirements.

  - Observation 1 (Ericsson): Note is needed to indicate the power class which is compatible with HST FR2.

  - Proposal 3 (Nokia): Requirements are enhanced according to table above, where the scaling factor N1 is FFS pending the outcome of the number of RX beam discussions.

  - Proposal 4 (QC): specify the total number of DRx cycles without N1 factor.

- Recommended WF

  - Continue the discussion in the 1st round.

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| QC | If the upper bound is 320ms, we don’t see why N1 is relevant as long as T_detect time can maintain connectivity. |
| Nokia | Proposal 3 is preferred since the number of UE RX sweeping beams is still under discussions, which affects the scaling factor N1. As mentioned in Issue 2-1-1, the number of UE RX sweeping beams for idle mode may not be necessarily the same as for connected mode. |
| Huawei | Support proposal 3. UE implementation is uniform to different DRX cycles, it is complicated using different beam sweeping scheme for different DRX cycle. |
| Ericsson | We support proposal 3 slightly, N1 can be aligned with RX beam. Meanwhile, please be aware of power class issue. |
| Samsung | Agree with P3. |
| Apple | Agree N1 depends on Rx beam discussion outcome. |
| CATT | Support Proposal 3. |
| ZTE | Support Proposal 3 |

#### Issue 3-3-4: Scaling factor M2

- [Moderator]:
It is observed by one of the companies that
With the enhancement from FR1 HST WI, i.e. M2=1, the number of samples is reduced from 36 to 8 for 0.32 DRX cycle, the FR2 cell re-selection delay still cannot support 350km/h with 700m ISD.

- Proposals and/or Observations

  - Proposal 1 (CMCC): The enhancement introduced in Rel-16 HST WI, e.g. the number of samples, the enhancement of M2, can be reused.

  - Proposal 2 (OPPO): When SMTC <= 40ms, use the scaling factor M2=M3=M4=1.

- Recommended WF

  - Continue the discussion in the 1st round.

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| QC | Same comment as 3-3-3. |
| Nokia | Proposals 1 and 2 seem similar. |
| Huawei | The difference between proposal 1 and proposal 2 is for SMTC< 40ms. We had a question, do we need to discuss the upper bound of SMTC periodicity for idle mode. |
| Ericsson | To our understanding, proposal 1 and proposal 2 are not controversial, and they are also addressed by note 1 in Issue 3-3-3’s table. |
| Samsung | My understanding is P1 include both the reduced “number of samples (36->8)” and the reduced value of M2 (M=1 if SMTC<=40ms). We can use Rel-16 FR1 HST enhancement as baseline to discuss for idle mode requirement enhancement. |
| CATT | P1 has the proposal to use the same number of samples as that in FR1 while proposal 2 only mentioned the M2 scaling factor. we agree that Proposal 1 can be the start point. But we need to verify whether the same values can be used in FR2 or not. The scenarios and channel condition are different from FR1. The numbers of samples may not be reused directly. Need simulation verification. |
| CMCC | We support proposal 1. For the M2 scaling factor, proposal 1 and proposal 2 are the same. Except M2 scaling factor, proposal 1 also suggest to reuse the reduced number of samples. In our view, both M2 (M=1 if SMTC<=40ms) and the reduced number of samples in Rel-16 FR1 HST can be reused for FR2 HST. |

#### Issue 3-3-5: Number of measurement samples

- [Moderator]:
It is observed by one of the companies that
the number of samples in FR1 HST is quite small and could apply to FR2 HST. However, if 8 samples in FR1 HST are reused, the delay for cell detection procedure will be too long. For example, when DRX cycle=640ms, N1=2, then the detection time will be 5.12*2=10.24s, which corresponds to about 996m moving distance. It is necessary to further reduce the number of samples for cell detection procedure especially for long DRX cycle.
Another company notes that with 320ms DRx cycle length, UE travels about 700m in 24 DRx cycles.

- Proposals and/or Observations

  - Proposal 1 (OPPO): The number of samples X should be reduced for FR2 HST:

    - For cell detection procedure, X should be further reduced to 4.

    - For measurement procedure, X=1 in FR1 HST could be reused

    - For evaluation procedure, X=3 in FR1 HST could be reused.

  - Proposal 2 (Qualcomm): Allow 24 DRx cycles detection time when DRx cycle length is 320ms in idle mode enhancement regardless of number of measurement samples.

- Recommended WF

  - Continue the discussion in the 1st round.

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| QC | Support proposal 2. Same comment as 3-3-3. |
| Huawei | Firstly we suggest to limit DRX cycles in FR2 HST scenario. Secondly the RX beam number for idle mode shall align with the conclusion of topic #2. The discussion on whether the sample number need to be reduced is suggested to be carried out after the above issues has conclusions. |
| Ericsson | We suggest to use Issue 3-3-3’s table with advantage from N1. |
| Samsung | Discuss 320ms DRX cycle length as upper bound for FR2 HST requirement firstly. |
| CATT | Similar comment as in issue 3-3-4 |

### CRs/TPs comments collection

For close-to-finalize Wis and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing Wis, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| XXX | Company A |
|  | Company B |
| YYY | Company A |
|  | Company B |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

| Sub-topic | Status summary |
| --- | --- |
| Sub-topic #3-1: CONNECTED state mobility | Issue 3-1-1: DRX upper bound Background: Companies seems to agree with the approach that firstly the upper bound of DRX cycle should be determined. However, there are different proposals about the values of the upper bound. The mos popular option is 80 ms. Tentative agreements: None Candidate options: - Option 1[Nokia]: Up to [256] ms - Option 2[Huawei]: [160] ms - Option 3[Ericsson, Samsung, Intel, CATT, ZTE]: [80] ms - Option 3[Intel, Samsung]: [60] ms Recommendations for 2nd round: Continue the discussion in the second round. Issue 3-1-2: Requirements for long DRX Background: All of the companies agreed on Option 1. Tentative agreements: Apply existing R16 requirements for long DRX cycles, i.e. above the upper bound of DRX cycle. Candidate options: Recommendations for 2nd round: Tentative agreement is ageable. Issue 3-1-3: Enhancements for short DRX Background: The majority of the companies seems to agree on the enhancement of scaling factor 1.5 for short SMTC periodicity. However, one company still have an opinion that M2=1.5 shall be kept. Tentative agreements: None Candidate options: - Option 1[Nokia, Huawei, Ericsson, Samsung, Apple, CATT, CMCC, ZTE]: M2 = 1.5 if SMTC periodicity > [40] ms, otherwise M2=1. - Option 2[QC]: Keep the 1.5 relaxation factor. Recommendations for 2nd round: Continue the discussion in the second round. Issue 3-1-4: SMTC periodicity Background: It looks like companies agree in the opinion that the requirements should be enhanced for SMTC periodicities below 40 ms, even though the SMTC periodicity shall not be restricted. Tentative agreements: HST FR2 enhanced requirement is applied to SMTC <=40ms. SMTC periodicity is not restricted. Candidate options: Recommendations for 2nd round: Confirm that tentative agreement is ageable. Issue 3-1-5: Connection Re-establishment requirements Background: There is no common agreement between the companies on the possible options. However, the common approach between the companies’ proposing enhancements can be observed. Tentative agreements: None Candidate options: - Known NR cell enhancements for HST FR2 - Option 1 [Huawei, Apple, Samsung, ZTE]: Do not introduce enhancements, except for scaling factor (N1) corresponding the number of RX beams. MAX (200 ms, 5 x N1 x TSMTC). - Option 2 [Ericsson]: MAX (400 ms, 5 x N1 x TSMTC). - Option 3 [QC]: Do not introduce any enhancements. - Other options are not precluded. - FFS the value of N1. - Unknow NR cell enhancements for HST FR2: - Option 1: Keep current requirements with N1 scaling factor corresponding the number of RX beams: MAX (1000 ms, 10 x N1 x TSMTC) - Option 2: Do not introduce any enhancements. - Other options are not precluded - FFS the value of N1. Recommendations for 2nd round: Continue the discussion in the second round. |
| Sub-topic #3-2: TBA | Issue 3-2-1: Criteria of known cell for FR2 Background: It looks like the company does not see an issue with the definition of the criteria of known known cell as such. However, it was proposed to re-confirm that 4 second condition from FR1. Tentative agreements: The target FR2 cell is known if it has been meeting the relevant cell identification requirement during the last 5 seconds; otherwise, it is unknown. Candidate options: Recommendations for 2nd round: Agree on a tentative agreement in the second round. Issue 3-2-2: Handover requirement Background: The companies seem to agree that the HO requirement to known cell is applicable, and the only potential enhancements needed is taking into account the number of RX beam sweeps. Therefore, the former agreement from RAN4#98-e-bis can be still followed. Tentative agreements: None Candidate options: Recommendations for 2nd round: Add FFS to the WF on the scaling factor corresponding the number of RX beam sweep. Issue 3-2-3: Handover to unknown cell Background: Some companies raise concerns if a HO to unknow cell is possible in HST FR2 deployments. Tentative agreements: None Candidate options: Recommendations for 2nd round: Add to the WF, FFS: A feasibility of HO to unknow cell in HST FR2 deployments. Issue 3-2-4: Handover to unknown cell Background: The companies, firstly, see a need to agree on feasibility of HO to unknown cell (previous issue). Tentative agreements: None Candidate options: Recommendations for 2nd round: None |
| Sub-topic #3-3: IDLE state mobility | Issue 3-3-1: The upper bound of DRX cycle Background: In general, it looks like the companies need more time to discuss this Issue. However, there is Tentative agreements: None Candidate options: - Option 1: Use 320 ms DRX cycle as baseline for following analysis and requirement definition - Other options are not precluded. Recommendations for 2nd round: Add the upper bound of DRX cycle FFS in the WF. Issue 3-3-2: Cell selection Background: Does not looks like companies are eager to update the principles how cell selection requirement is formulated. Tentative agreements: Use cell selection criterion S and number of DRX cycles as a baseline for cell selection requirement. Candidate options: Recommendations for 2nd round: Confirm that the tentative agreement is agreeable in the second round. Issue 3-3-3: Scaling factor N1 Background: Most of the companies except one agree that the proposed table format shall be used as a basis for the HST FR2 requirements. The values of the scaling factors and parameters in the table require further discussion. Tentative agreements: Use table below as a background for further discussion of HST FR2 IDLE mode requirement: Tdetect,NR_Intra, Tmeasure,NR_Intra and Tevaluate,NR_Intra for FR2 HST DRX cycle length [s]; Tdetect,NR_Intra [s] (number of DRX cycles); Tmeasure,NR_Intra [s] (number of DRX cycles); Tevaluate,NR_Intra [s] (number of DRX cycles). 0.32; 2.56 x N1 x M2 (8 x N1 x M2); 0.32 x N1 x M3 (1 x N1 x M3); 0.96 x N1 x M4 (3 x M4). 0.64; 5.12 x N1 (8 x N1); 0.64 x N1 (1 x N1); 1.92 x N1 (3 x N1). 1.28; 8.96 x N1 (7 x N1); 1.28 x N1 (1 x N1); 3.84 x N1 (3 x N1). 2.56; 58.88 x N1 (23 x N1); 2.56 x N1 (1 x N1); 7.68 x N1 (3 x N1). Note 1: when SMTC < = 40 ms, M2 = M3 = M4 = 1; and when SMTC > 40 ms, M2 = 1.5, M3 = M4 = 2. Candidate options: Recommendations for 2nd round: - Confirm that the tentative agreement is agreeable in the second round. - Include the values of scaling factors: N1, M2, SMTC FFS in the WF. Issue 3-3-4: Scaling factor M2 Background: It seems that more time is still needed for the discussion of the issue., however the principles are formulated in Issues 3-3-3. Tentative agreements: None Candidate options: Recommendations for 2nd round: See Issue 3-3-3. Issue 3-3-5: Number of measurement samples Background: It seems that more time is still needed for the discussion of the issue., however the principles are formulated in Issues 3-3-3. Tentative agreements: None Candidate options: Recommendations for 2nd round: See Issue 3-3-3. |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round (if applicable)

TBA

### Sub-topic 2-2: TBA

#### Issue 2-2-2: TBA

Agreements from round 1:

TBA

Candidate options:

- Option 1:

- Option 2:

Recommendations for 2nd round:

TBA

Contributor Comments:

| Company | Comments |
| --- | --- |
| XXX |  |
| YYY |  |
| ZZZ |  |

## Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

# Recommendations for Tdocs

## 1st round

New tdocs

| Title | Source | Comments |
| --- | --- | --- |
| WF on … | YYY |  |
| LS on … | ZZZ | To: RAN_X; Cc: RAN_Y |

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-210xxxx | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-2114568 | LS on UE capability and network signalling | Nokia, Nokia Shaghai Bell | Not Pursued | The preparation of LS is pending on the outcomes of ongoing discussions. |
| R4-2112264 | LS on Beam Management Enhancement Signaling | Qualcomm | Not Pursued | Need to wait agreement on the corresponding issues. |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

## 2nd round

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-210xxxx | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-210xxxx | WF on … | YYY | Agreeable, Revised, Noted |  |
| R4-210xxxx | LS on … | ZZZ | Agreeable, Revised, Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents

# Annex

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| Moderator (Nokia, Nokia Shanghai Bell) | Dmitry Petrov | dmitry.a.petrov@nokia-bell-labs.com |
| Ericsson | Ming Li | Ming.l.li@ericsson.com |
| Nokia, Nokia Shanghai Bell | Anthony Lo | nthony.lo@nokia.com |
| Samsung | He Wang (Jackson) | h0809.wang@samsung.com |
| Intel | Ilya Bolotin | ilya.bolotin@intel.com |
| CATT | Yanze Fu | fuyanze@catt.cn |
| CMCC | Jingjing Chen | chenjingjing@chinamobile.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)