3GPP TSG-RAN WG4 meeting #100-e	R4-2115224

Electronic Meeting, 16 – 27 August 2021

Agenda item:			9.20.3.1, 9.20.3.2

Source:	Moderator (Ericsson)

Title:	Email discussion summary for [100-e][234] NR_redcap_RRM_1

Document for:	Information

# Introduction

The discussion covers NR RedCap RRM AIs within 9.20.3.1, 9.20.3.2.

When updating this document, please remember to:

- use track changes while adding your comments in this document (only updates marked with change marks will be taken into the next version),

- change the file name, adding your company name, according to the instructions from RAN4 chair:

- Length of file names shall be reduced, e.g.

  - At the beginning of first round, moderators share / ftp / tsg_ran / WG4_Radio / TSGR4_98_e / Inbox / Drafts / [98e][101] NR_NewRAT_SysParameters\Summary_101_1st round_v01.docx

  - After update by company A: Summary_101_1st round_v02_companyA

  - After update by company B: Summary_101_1st round_v03_companyA_companyB

  - After update by company C: Summary_101_1st round_v04_companyB_companyC

## 1st round

The following list of open issues was identified, based on the contributions, for the 1st round.

The following colour marking is used below:

- A topic/issue proposed for discussion in: GTW session 1

- No discussion in the 1st round

- Topic #1: General and RRM requirements impacts

- Topic #2: UE complexity reduction

- Topic #3: Simulation Assumptions

- Topic #4: Specification Impact

Sub-topic 4-1: RRM requirements in TS 38.133

Sub-topic 4-2: RRM requirements in TS 36.133

## 2nd round

TBD

# Topic #1: General and RRM requirements impacts

Contributions from AI 9.20.3.1 are discussed here.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2112129 | Apple | Proposal 1: RAN4 defines inter-frequency RRM requirements for RedCap UE in Rel-17. Proposal 2: • Do not define inter-RAT RRM requirements on 2G/3G for RedCap UE in Rel-17 • RAN4 defines inter-RAT RRM requirements on LTE for RedCap UE in Rel-17 Proposal 3: agree on the table 1 for RRM impacts on TS38.133 for RedCap UE. Proposal 4: Whether LTE cat-M1 or NB-IoT shall be considered for RedCap requirement in TS36.133 is discussed in case by case manner after sufficient progress is made in the WI. Proposal 5: agree on the table 2 for RRM impacts on TS36.133 for RedCap UE. |
| R4-2112191 | CMCC | Proposal 1: It is proposed to discuss eDRX enhancements in RAN4 after the final conclusions on eDRX configurations are made in RAN2. Proposal 2: Rel-16 RRM relaxation requirements in idle/inactive mode can be considered as the baseline for Rel-17 RedCap RRM relaxation in idle/inactive mode. Proposal 3: New RRM relaxation requirements in connected mode need to be defined for Rel-17 RedCap RRM relaxation. Proposal 4: Define inter-frequency RRM requirements for RedCap UE in Rel-17 as low priority. Proposal 5: Do not define inter-RAT RRM requirements for RedCap UE in Rel-17. |
| R4-2112643 | Vivo | Proposal 1: The inter-frequency carriers for a Redcap terminal to be capable of monitoring is at least 3. Similar rule could be applied for LTE FDD/TDD if necessary. Proposal 2: consider inter-frequency measurements (idle/inactivate/connected state) as the same priority as that of intra-frequency measurement (idle/inactivate/connected state). Proposal 3: There is no impact on the requirement for maximum interruption in paging reception. Proposal 4: The requirements of Handover, RRC re-establishment, RRC Connection Release with Redirection for Redcap will be impacted. Proposal 5: For the RLM, the maximum number of RLM-RS resources, the PDCCH parameters for out-of-sync and in-sync evaluation and the evaluation period could be impacted. The interruption requirements defined at 8.2 are not applicable (no impact) for Redcap. Proposal 6: Do not discuss V2X requirements for Redcap at Rel-17 time frame. |
| R4-2113148 | Intel Corporation | Reduced number of Rx branches: New test setups: antenna configurations Half-duplex FDD: General contention resolution aspect: scheduling restrictions Dedicated RRM measurements: cell detection, measurements, radio link monitoring, beam management Test applicability for HD-FDD New test setups: new RMC-s for HD-FDD Impact due to eDRX up to 10.24s eDRX cycle: Cell reselection requirements: serving cell evaluations, cell identification and measurement, reselection criteria and paging interruption Test applicability for eDRX New test setups: new eDRX configurations Impact due to eDRX up to Up to 10000~s: New designs are pending discussions Impact due to relaxed RRM neighbour cell measurements in IDLE/INACTIVE modes: Relaxation of serving cell evaluation Relaxed monitoring of neighbour cells Impact due to relaxed RRM neighbour cell measurements in CONNECTED mode: Relaxed RLM Relaxed neighbour cell measurement |
| R4-2113284 | OPPO | Proposal 1: RAN4 to define inter-frequency RRM requirements for RedCap UE in Rel-17. Proposal 2: Do not define inter-RAT RRM requirements on 2G/3G for RedCap UE in Rel-17. Proposal 3: Considering the demand of dual mode terminals, RAN4 to define inter-RAT RRM requirements on LTE for RedCap UE in Rel-17. |
| R4-2113845 | Huawei, HiSilicon | Proposal 1: Inter-frequency RRM requirements are supposed to be defined for RedCap UE in Rel-17. Proposal 2: Inter-RAT RRM requirements on LTE are supposed to be defined for RedCap UE in Rel-17. Proposal 3: The existing UE transmit timing requirements (including Te and Tq) can be applied for RedCap UE. Proposal 4: The existing core requirements of RLM, BFD and CBD can be applied for RedCap UE. The threshold in test cases needs further study. |
| R4-2113865 | ZTE Corporation | Proposal 1: Both 2-step and 4-step RA requirements shall be specified for reduced capability UEs. Proposal 2: Define inter-RAT RRM requirements for RedCap UE in Rel-17. Proposal 3: When discussing possible combinations of Rel-16 features and RedCap, we should by default assume that the features are not applicable and then identify which features can be combined with RedCap. |
| R4-2113947 | MediaTek Inc. | Proposal 1: Support further study the bandwidth parameters for CSI-RS based RLM to accommodate the reduced maximum bandwidth in RedCap for FR1. Proposal 2: Support modification on PDCCH transmission parameters for RLM requirements with the single antenna port in RedCap devices, including the ratio of hypothetical PDCCH RE energy to average SSS/CSI-RS RE energy for out-of-sync and in-sync with single antenna port to be with [3] dB higher compared to the existing two antenna requirements. Proposal 3: Support adding the following condition for the evaluation period of 5G NR RLM requirement for HD-FDD: At least 1 RLM-RS must fall with DL occasion within an indication period. The UE determines the indication period as the maximum between the shortest periodicity for radio link monitoring resources and 10 msec. Proposal 4: Support extending the lower bound of the evaluation period of 5G NR RedCap RLM requirement by two compared to the existing general 5G NR RLM requirements. Proposal 5: Support introducing scheduling availability restriction on 5G NR RedCap UEs performing measurements in HD-FDD bands. Proposal 6: Support inter-frequency mobility for 5G NR RedCap devices in Rel-17. Proposal 7: Support LTE inter-RAT mobility for 5G NR RedCap devices in Rel-17 at least in IDLE mode. FFS CONNECTED mode. |
| R4-2114068 | Nokia, Nokia Shanghai Bell | Proposal 1: RRM core requirements for RedCap device are based on single CC. Proposal 2: No new RRM core requirements for HD-FDD type A RedCap device compared to legacy NR device need to be specified. Proposal 3: No SUL support is considered for RedCap device. Proposal 4: Inter-frequency RRM measurement requirements for RedCap device are needed for Rel-17. Proposal 5: Inter-RAT measurement requirements for RedCap device are limited to LTE. |
| R4-2114084 | Ericsson | Proposal #1: RAN4 to prioritize the work to develop to the intra-frequency requirements in release 17 RedCap WI. Inter-frequency requirements are developed if time allows in release 17. Proposal #2: RAN4 to not define inter-RAT RRM requirements for RedCap UE in Rel-17. Proposal #3: RAN4 to study the cell detection performance for RedCap UE assuming the simulation assumptions in Annex A. Proposal #4: RAN4 to study the SSB based RRM measurement performance for RedCap UE assuming the simulation assumptions in Annex B. Proposal #5: RAN4 to study the RLM and BFD performance for RedCap UE assuming the simulation assumptions in Annex C. Proposal #6: RAN4 to study the L1 RSRP measurement performance for RedCap UE assuming the simulation assumptions in Annex C. Proposal #7: RAN4 to study the SI reading performance for RedCap UE assuming the simulation assumptions in Annex D. |
| R4-2112415 | Xiaomi | Proposal 7: RAN4 to specify the RRM requirements for both intra-frequency measurement and inter-frequency measurement. Proposal 8: RAN4 to deprioritize the discussion on the mobility requirements for inter-RAT measurements. |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 1-1: Inter-frequency support

Issue 1-1-1: Inter-frequency support

Proposal 1 (Apple, Vivo, Oppo, Huawei, ZTE, MTK, Nokia, Xiaomi): RAN4 defines inter-frequency RRM requirements for RedCap UE in Rel-17.

Proposal 2 (CMCC, Ericsson): RAN4 prioritizes the work on intra-frequency RRM requirements, inter-frequency requirements are developed if time allows in the WI.

Recommended WF

Discuss the proposals.

### Sub-topic 1-2: Inter-RAT 2G/3G in IDLE/INACTIVE states

Issue 1-2-1: Inter-RAT 2G/3G in IDLE/INACTIVE states

Proposal 1 (Apple, Oppo, Huawei, ZTE, Nokia, MTK): RAN4 to define inter-RAT RRM requirements on LTE for RedCap in Rel-17

Proposal 1a (Apple, Oppo, Huawei, ZTE, Nokia):

• Do not define inter-RAT RRM requirements on 2G/3G for RedCap UE in Rel-17

• RAN4 defines inter-RAT RRM requirements on LTE for RedCap UE in Rel-17

Proposal 1b (MTK):

• Support LTE inter-RAT mobility for 5G NR RedCap devices in Rel-17 at least in IDLE mode. FFS CONNECTED mode.

Proposal 2 (Ericsson, CMCC, Xiaomi):

• Do not define inter-RAT RRM requirements for RedCap UE in Rel-17.

Recommended WF

Discuss the proposals.

### Sub-topic 1-3: Inter-RAT 2G/3G in CONNECTD state

Issue 1-3-1: Inter-RAT 2G/3G in CONNECTED state

Proposal 1 (Apple, Oppo, Huawei, ZTE, Nokia, MTK): RAN4 to define inter-RAT RRM requirements on LTE for RedCap in Rel-17

Proposal 1a (Apple, Oppo, Huawei, ZTE, Nokia):

• Do not define inter-RAT RRM requirements on 2G/3G for RedCap UE in Rel-17

• RAN4 defines inter-RAT RRM requirements on LTE for RedCap UE in Rel-17

Proposal 1b (MTK):

• Support LTE inter-RAT mobility for 5G NR RedCap devices in Rel-17 at least in IDLE mode. FFS CONNECTED mode.

Proposal 2 (Ericsson, CMCC, Xiaomi):

• Do not define inter-RAT RRM requirements for RedCap UE in Rel-17.

Recommended WF

Discuss the proposals.

### Sub-topic 1-4: Inter-RAT LTE in IDLE/INACTIVE states

Issue 1-4-1: Inter-RAT LTE in IDLE/INACTIVE states

Proposal 1 (Apple, Oppo, Huawei, ZTE, Nokia, MTK): RAN4 to define inter-RAT RRM requirements on LTE for RedCap in Rel-17

Proposal 1a (Apple, Oppo, Huawei, ZTE, Nokia):

• Do not define inter-RAT RRM requirements on 2G/3G for RedCap UE in Rel-17

• RAN4 defines inter-RAT RRM requirements on LTE for RedCap UE in Rel-17

Proposal 1b (MTK):

• Support LTE inter-RAT mobility for 5G NR RedCap devices in Rel-17 at least in IDLE mode. FFS CONNECTED mode.

Proposal 2 (Ericsson, CMCC, Xiaomi):

• Do not define inter-RAT RRM requirements for RedCap UE in Rel-17.

Recommended WF

Discuss the proposals.

### Sub-topic 1-5: Inter-RAT LTE in CONNECTD state

Issue 1-5-1: Inter-RAT LTE in CONNECTED state

Proposal 1 (Apple, Oppo, Huawei, ZTE, Nokia, MTK): RAN4 to define inter-RAT RRM requirements on LTE for RedCap in Rel-17

Proposal 1a (Apple, Oppo, Huawei, ZTE, Nokia):

• Do not define inter-RAT RRM requirements on 2G/3G for RedCap UE in Rel-17

• RAN4 defines inter-RAT RRM requirements on LTE for RedCap UE in Rel-17

Proposal 1b (MTK):

• Support LTE inter-RAT mobility for 5G NR RedCap devices in Rel-17 at least in IDLE mode. FFS CONNECTED mode.

Proposal 2 (Ericsson, CMCC, Xiaomi):

• Do not define inter-RAT RRM requirements for RedCap UE in Rel-17.

Recommended WF

Discuss the proposals.

### Sub-topic 1-6: SUL support

Issue 1-6-1: SUL support

Proposal 1 (Nokia): No SUL support is considered for RedCap device.

Recommended WF

Try to agree on proposal 1 under the assumption there is no RF support for SUL.

### Sub-topic 1-7: Consideration of LTE cat-M1 or NB-IoT for RedCap in 36.133

Issue 1-7-1: Consideration of LTE cat-M1 or NB-IoT for RedCap in TS 36.133

Proposal 1 (Apple): Whether LTE cat-M1 or NB-IoT shall be considered for RedCap requirement in TS36.133 is discussed in case by case manner after sufficient progress is made in the WI.

Recommended WF

To be further discussed.

### Sub-topic 1-8: V2X requirements for Rel-17 RedCap

Issue 1-8-1: V2X requirements for Rel-17 RedCap

Proposal 1 (Vivo): Do not discuss V2X requirements for Redcap at Rel-17 time frame.

Recommended WF

It is observed that V2X is not within the scope of RedCap WID. Thus no discussions are needed on this topic.

### Sub-topic 1-9: Combination of features

Issue 1-9-1: Combination of features

Proposal 1 (ZTE): When discussing possible combinations of Rel-16 features and RedCap, we should by default assume that the features are not applicable and then identify which features can be combined with RedCap.

Proposal 2(Apple): For R16 CSI-RS based L3 measurement, L1-SINR measurement, SFTD measurement, CGI reading, and PL-RS change, defer the discussion until sufficient progress is made in the WI.

Recommended WF

Need further discussions.

### Sub-topic 1-10: Assumptions for defining UE requirements

Issue 1-10-1: Assumptions on UE

Proposal 1 (Ericsson): When RAN4 defines the RRM requirement, it shall further consider the following factors for UE complexity reduction:

•	Single RF path is expected

•	Single searcher is expected

Recommended WF

Discuss the proposal.

## Companies views’ collection for 1st round

### Open issues

| Company |  | Comments |  |
| --- | --- | --- | --- |
| Company A |  | ### Sub-topic 1-1: Inter-frequency support Issue 1-1-1: Inter-frequency support ### Sub-topic 1-2: Inter-RAT 2G/3G in IDLE/INACTIVE states Issue 1-2-1: Inter-RAT 2G/3G in IDLE/INACTIVE states ### Sub-topic 1-3: Inter-RAT 2G/3G in CONNECTD state Issue 1-3-1: Inter-RAT 2G/3G in CONNECTED state ### Sub-topic 1-4: Inter-RAT LTE in IDLE/INACTIVE states Issue 1-4-1: Inter-RAT LTE in IDLE/INACTIVE states ### Sub-topic 1-5: Inter-RAT LTE in CONNECTD state Issue 1-5-1: Inter-RAT LTE in CONNECTED state ### Sub-topic 1-6: SUL support Issue 1-6-1: SUL support ### Sub-topic 1-7: Consideration of LTE cat-M1 or NB-IoT for RedCap in 36.133 Issue 1-7-1: Consideration of LTE cat-M1 or NB-IoT for RedCap in TS 36.133 ### Sub-topic 1-8: V2X requirements for Rel-17 RedCap Issue 1-8-1: V2X requirements for Rel-17 RedCap ### Sub-topic 1-9: Combination of features Issue 1-9-1: Combination of features ### Sub-topic 1-10: Assumptions for defining UE requirements Issue 1-10-1: Assumptions on UE |  |
| ZTE |  | ### Sub-topic 1-1: Inter-frequency support Issue 1-1-1: Inter-frequency support Support defining inter-RAT, doesn’t see it as a problem. ### Sub-topic 1-2: Inter-RAT 2G/3G in IDLE/INACTIVE states Issue 1-2-1: Inter-RAT 2G/3G in IDLE/INACTIVE states Inter-RAT for 2G/3G can be skipped. ### Sub-topic 1-6: SUL support Issue 1-6-1: SUL support Support Option 1. ### Sub-topic 1-8: V2X requirements for Rel-17 RedCap Issue 1-8-1: V2X requirements for Rel-17 RedCap Support Option 1 and the recommended WF. ### Sub-topic 1-9: Combination of features Issue 1-9-1: Combination of features Support Option 1. The intention is to focus on what’s explicitly included in the WID. |  |
| Huawei |  | ### Sub-topic 1-1: Inter-frequency support Issue 1-1-1: Inter-frequency support Support option 1. Both intra-frequency and inter-frequency mobility are crucial functionalities for UE. Moreover if there are no requirements of inter-frequency mobility, it means there is no limitation of UE performing inter-frequency mobility. As a result, the inter-frequency mobility performance is hard to be guaranteed. ### Sub-topic 1-2: Inter-RAT 2G/3G in IDLE/INACTIVE states Issue 1-2-1: Inter-RAT 2G/3G in IDLE/INACTIVE states Support option 1a. In current practical network, full NR coverage is not well guaranteed. At least LTE could be used as a coverage hole supplementary for NR. In order to ensure Redcap UE can have good connection, inter-RAT LTE mobility is supposed to be supported. ### Sub-topic 1-3: Inter-RAT 2G/3G in CONNECTD state Issue 1-3-1: Inter-RAT 2G/3G in CONNECTED state Support option 1a. ### Sub-topic 1-4: Inter-RAT LTE in IDLE/INACTIVE states Issue 1-4-1: Inter-RAT LTE in IDLE/INACTIVE states Proposal 1a. Same reason as issue 1-2-1. ### Sub-topic 1-5: Inter-RAT LTE in CONNECTD state Issue 1-5-1: Inter-RAT LTE in CONNECTED state Proposal 1a. Same reason as issue 1-2-1. ### Sub-topic 1-6: SUL support Issue 1-6-1: SUL support Disagree with proposal 1. RF session is discussing the issue in parallel. ### Sub-topic 1-7: Consideration of LTE cat-M1 or NB-IoT for RedCap in 36.133 Issue 1-7-1: Consideration of LTE cat-M1 or NB-IoT for RedCap in TS 36.133 Agree with proposal 1, it is supposed to be discussed in case by case manner for Redcap. ### Sub-topic 1-8: V2X requirements for Rel-17 RedCap Issue 1-8-1: V2X requirements for Rel-17 RedCap Agree with the recommended WF. ### Sub-topic 1-9: Combination of features Issue 1-9-1: Combination of features Proposal 1 and proposal 2 are fine. ### Sub-topic 1-10: Assumptions for defining UE requirements Issue 1-10-1: Assumptions on UE Agree with observation with single searcher as CA is not supported for Redcap. |  |
| Apple |  | Issue 1-1-1: Inter-frequency support Support option 1. Issue 1-2-1: Inter-RAT 2G/3G in IDLE/INACTIVE states Support option 1a. Issue 1-3-1: Inter-RAT 2G/3G in CONNECTED state Support option 1a. Issue 1-4-1: Inter-RAT LTE in IDLE/INACTIVE states Support option 1a. Issue 1-5-1: Inter-RAT LTE in CONNECTED state Support option 1a. Issue 1-6-1: SUL support Fine with proposal 1 unless RF session has conclusion to introduce it in RedCap Issue 1-7-1: Consideration of LTE cat-M1 or NB-IoT for RedCap in TS 36.133 Support proposal 1. Issue 1-8-1: V2X requirements for Rel-17 RedCap Support proposal 1. Issue 1-9-1: Combination of features Support both proposal 1 and proposal 2. Issue 1-10-1: Assumptions on UE Fine with proposal 1. |  |
| Intel |  | ### Sub-topic 1-1: Inter-frequency support Issue 1-1-1: Inter-frequency support Support specifying Inter-frequency RRM requirements. ### Sub-topic 1-2: Inter-RAT 2G/3G in IDLE/INACTIVE states Issue 1-2-1: Inter-RAT 2G/3G in IDLE/INACTIVE states No need to consider 2G/3G in general. ### Sub-topic 1-3: Inter-RAT 2G/3G in CONNECTD state Issue 1-3-1: Inter-RAT 2G/3G in CONNECTED state No need to consider 2G/3G in general. ### Sub-topic 1-4: Inter-RAT LTE in IDLE/INACTIVE states Issue 1-4-1: Inter-RAT LTE in IDLE/INACTIVE states This is important. Support specify IDLE RRM requirements towards LTE. ### Sub-topic 1-5: Inter-RAT LTE in CONNECTD state Issue 1-5-1: Inter-RAT LTE in CONNECTED state Deprioritized. ### Sub-topic 1-6: SUL support Issue 1-6-1: SUL support No restriction is seen. ### Sub-topic 1-7: Consideration of LTE cat-M1 or NB-IoT for RedCap in 36.133 Issue 1-7-1: Consideration of LTE cat-M1 or NB-IoT for RedCap in TS 36.133 Important discussion. We are open currently. Let’s not exclude cat-1bis. ### Sub-topic 1-8: V2X requirements for Rel-17 RedCap Issue 1-8-1: V2X requirements for Rel-17 RedCap ### Sub-topic 1-9: Combination of features Issue 1-9-1: Combination of features R15 should serve as the baseline. Besides case-by-case discussion on specific R16 features. ### Sub-topic 1-10: Assumptions for defining UE requirements Issue 1-10-1: Assumptions on UE Further discussion is needed. |  |
| CMCC |  | ### Sub-topic 1-1: Inter-frequency support Issue 1-1-1: Inter-frequency support Our consideration to deprioritize inter-frequency RRM requirements is to consider the limited time left in Rel-17. We prefer to focus on intra-frequency first, if time allows, we can also define inter-frequency requirements. ### Sub-topic 1-2: Inter-RAT 2G/3G in IDLE/INACTIVE states Issue 1-2-1: Inter-RAT 2G/3G in IDLE/INACTIVE states ### Sub-topic 1-3: Inter-RAT 2G/3G in CONNECTD state Issue 1-3-1: Inter-RAT 2G/3G in CONNECTED state ### Sub-topic 1-4: Inter-RAT LTE in IDLE/INACTIVE states Issue 1-4-1: Inter-RAT LTE in IDLE/INACTIVE states ### Sub-topic 1-5: Inter-RAT LTE in CONNECTD state Issue 1-5-1: Inter-RAT LTE in CONNECTED state For all the inter-RAT related discussion, our proposal is to not define inter-RAT requirements for RedCap UE in Rel-17 considering the limited time left. For inter-RAT 2G/3G requirements, all companies agree to not define RRM requirements. For inter-RAT LTE requirements, some companies think inter-RAT LTE measurement is very important for wearable RedCap UE. One question is that what UE category we consider for LTE when considering the inter-RAT mobility between LTE and NR. And for proposal 1b (support LTE inter-RAT mobility for 5G NR RedCap devices at least in IDLE mode, FFS CONNECTED mode), we have different view on this point. If companies think inter-RAT mobility between LTE and NR are very important, connected mode requirements should be prioritied, instead of idle mode. And if companies believe inter-RAT LTE mobility is essential, we prefer to focus on intra-frequency first, if time allows, we can also define inter-RAT LTE mobility requirements. ### Sub-topic 1-6: SUL support Issue 1-6-1: SUL support OK with the recommended WF, follow RF conclusion on the support of SUL. ### Sub-topic 1-7: Consideration of LTE cat-M1 or NB-IoT for RedCap in 36.133 Issue 1-7-1: Consideration of LTE cat-M1 or NB-IoT for RedCap in TS 36.133 OK with the recommended WF, follow RF conclusion on the support of SUL. As we commented in issue 1-5-1, which LTE UE category is considered for inter-RAT mobility is an important aspect we need to consider. This issue is mainly related to the UE types and use cases, better to have a more focused scope in Rel-17 than involving all possible cases. ### Sub-topic 1-8: V2X requirements for Rel-17 RedCap Issue 1-8-1: V2X requirements for Rel-17 RedCap Agree with the recommended WF. ### Sub-topic 1-9: Combination of features Issue 1-9-1: Combination of features Difference between proposal 1 and 2 is that proposal 2 list some explicit features, but proposal 1 is more general. In general, we are OK with the principle in proposal 1. ### Sub-topic 1-10: Assumptions for defining UE requirements Issue 1-10-1: Assumptions on UE ### Whether single RF path is expected should depend on RF session discussion. For the single searcher assumption, does it mean that RedCap UE will not support CA at all? More discussion is needed before agree on the assumptions. |  |
| Xiaomi |  | ### Sub-topic 1-1: Inter-frequency support Issue 1-1-1: Inter-frequency support Support Option 1. ### Sub-topic 1-2: Inter-RAT 2G/3G in IDLE/INACTIVE states Issue 1-2-1: Inter-RAT 2G/3G in IDLE/INACTIVE states We propose to deprioritize the inter-RAT discussion for the limited time reason. If most companies support to define the inter-RAT requirement in R17, we can go with Option 1a. ### Sub-topic 1-3: Inter-RAT 2G/3G in CONNECTD state Issue 1-3-1: Inter-RAT 2G/3G in CONNECTED state Same comment as Issue 1-2-1. ### Sub-topic 1-4: Inter-RAT LTE in IDLE/INACTIVE states Issue 1-4-1: Inter-RAT LTE in IDLE/INACTIVE states Same comment as Issue 1-2-1. ### Sub-topic 1-5: Inter-RAT LTE in CONNECTD state Issue 1-5-1: Inter-RAT LTE in CONNECTED state Same comment as Issue 1-2-1. ### Sub-topic 1-6: SUL support Issue 1-6-1: SUL support Support Option 1. ### Sub-topic 1-7: Consideration of LTE cat-M1 or NB-IoT for RedCap in 36.133 Issue 1-7-1: Consideration of LTE cat-M1 or NB-IoT for RedCap in TS 36.133 Support the proposal, and conclusions from LTE Cat-1bis UE requirements could be considered as well. ### Sub-topic 1-8: V2X requirements for Rel-17 RedCap Issue 1-8-1: V2X requirements for Rel-17 RedCap Support the recommended WF. ### Sub-topic 1-9: Combination of features Issue 1-9-1: Combination of features Both proposals are fine |  |
| OPPO |  | ### Sub-topic 1-1: Inter-frequency support Issue 1-1-1: Inter-frequency support Support proposal 1. ### Sub-topic 1-2: Inter-RAT 2G/3G in IDLE/INACTIVE states Issue 1-2-1: Inter-RAT 2G/3G in IDLE/INACTIVE states Option 1a. ### Sub-topic 1-3: Inter-RAT 2G/3G in CONNECTD state Issue 1-3-1: Inter-RAT 2G/3G in CONNECTED state Same comment as Issue 1-2-1. ### Sub-topic 1-4: Inter-RAT LTE in IDLE/INACTIVE states Issue 1-4-1: Inter-RAT LTE in IDLE/INACTIVE states Same comment as Issue 1-2-1. ### Sub-topic 1-5: Inter-RAT LTE in CONNECTD state Issue 1-5-1: Inter-RAT LTE in CONNECTED state Same comment as Issue 1-2-1. ### Sub-topic 1-7: Consideration of LTE cat-M1 or NB-IoT for RedCap in 36.133 Issue 1-7-1: Consideration of LTE cat-M1 or NB-IoT for RedCap in TS 36.133 Support the proposal, and conclusions from LTE Cat-1bis UE requirements could be considered as well. ### Sub-topic 1-8: V2X requirements for Rel-17 RedCap Issue 1-8-1: V2X requirements for Rel-17 RedCap Support the recommended WF. ### Sub-topic 1-9: Combination of features Issue 1-9-1: Combination of features Both proposals are fine |  |
| vivo |  | ### Sub-topic 1-1: Inter-frequency support Issue 1-1-1: Inter-frequency support Support option 1. ### Sub-topic 1-2: Inter-RAT 2G/3G in IDLE/INACTIVE states Issue 1-2-1: Inter-RAT 2G/3G in IDLE/INACTIVE states Support option 1a. From deployment point of view, supporting LTE technology is crucial from coverage and providing ubiquitous user experience for Redcap UE point of view. ### Sub-topic 1-3: Inter-RAT 2G/3G in CONNECTD state Issue 1-3-1: Inter-RAT 2G/3G in CONNECTED state Support option 1a. ### Sub-topic 1-4: Inter-RAT LTE in IDLE/INACTIVE states Issue 1-4-1: Inter-RAT LTE in IDLE/INACTIVE states Proposal 1a. ### Sub-topic 1-5: Inter-RAT LTE in CONNECTD state Issue 1-5-1: Inter-RAT LTE in CONNECTED state Proposal 1a. Same reason as issue 1-2-1. ### Sub-topic 1-6: SUL support Issue 1-6-1: SUL support OK with proposal 1. ### Sub-topic 1-7: Consideration of LTE cat-M1 or NB-IoT for RedCap in 36.133 Issue 1-7-1: Consideration of LTE cat-M1 or NB-IoT for RedCap in TS 36.133 FIne with proposal 1 ### Sub-topic 1-8: V2X requirements for Rel-17 RedCap Issue 1-8-1: V2X requirements for Rel-17 RedCap Agree with the recommended WF. ### Sub-topic 1-9: Combination of features Issue 1-9-1: Combination of features Support Proposal 1 as a general principle. Proposal 2 is fine. ### Sub-topic 1-10: Assumptions for defining UE requirements Issue 1-10-1: Assumptions on UE ### OK with the proposal. |  |
| Ericsson |  | ### Sub-topic 1-1: Inter-frequency support Issue 1-1-1: Inter-frequency support Our preference is to develop the RedCap requirements following a step-by-step approach where the intra-frequency requirements are developed first, followed by inter-frequency requirements. However, due to large interest from companies to develop both intra- and inter-frequency requirements at the same time, we can also accept option 1. ### Sub-topic 1-2: Inter-RAT 2G/3G in IDLE/INACTIVE states Issue 1-2-1: Inter-RAT 2G/3G in IDLE/INACTIVE states We are fine to not define any inter-RAT RRM requirements on 2G/3G in IDLE/INACTIVE state. On inter-RAT LTE see our comments under issues 1-4-1 and 1-5-1. ### Sub-topic 1-3: Inter-RAT 2G/3G in CONNECTD state Issue 1-3-1: Inter-RAT 2G/3G in CONNECTED state We are fine to not define any inter-RAT RRM requirements on 2G/3G in CONNECTED state. ### Sub-topic 1-4: Inter-RAT LTE in IDLE/INACTIVE states Issue 1-4-1: Inter-RAT LTE in IDLE/INACTIVE states Our preference is to develop the RedCap requirements following a step-by-step approach where the NR intra-frequency (and inter-frequency) requirements are developed first, followed by inter-RAT LTE requirements. To reduce workload we propose not to develop any inter-RAT requirements in LTE in release 17. In any case no new LTE requirements should be introduced. Therefore, if inter-RAT LTE requirements are specified then the existing inter-RAT LTE requirements (e.g. LTE inter-RAT in 38.133) shall be reused for redcap. ### Sub-topic 1-5: Inter-RAT LTE in CONNECTD state Issue 1-5-1: Inter-RAT LTE in CONNECTED state Same view as for Inter-RAT LTE in IDLE/INACTIVE states (issue 1-4-1). Our preference is to develop the RedCap requirements following a step-by-step approach where the NR requirements are developed first, followed by inter-RAT LTE requirements. Therefore we propose not to develop any inter-RAT requirements in LTE in release 17. In any case no new LTE requirements should be introduced. Therefore, if inter-RAT LTE requirements are specified then the existing inter-RAT LTE requirements (e.g. LTE inter-RAT in 38.133) shall be reused for redcap. ### Sub-topic 1-6: SUL support Issue 1-6-1: SUL support We are fine with recommended WF. ### Sub-topic 1-7: Consideration of LTE cat-M1 or NB-IoT for RedCap in 36.133 Issue 1-7-1: Consideration of LTE cat-M1 or NB-IoT for RedCap in TS 36.133 As commented in issue 1-4-1/1-5-1, our view is to not develop LTE inter-RAT requirements in release 17 given the large amount of work this would require and this work cannot be done within the current time plan of release 17 RedCap WI. ### Sub-topic 1-8: V2X requirements for Rel-17 RedCap Issue 1-8-1: V2X requirements for Rel-17 RedCap ### We support recommended WF ### Sub-topic 1-9: Combination of features Issue 1-9-1: Combination of features Both proposals are fine and combination of features shall be discussed later when more progress is reached in the WI. ### Sub-topic 1-10: Assumptions for defining UE requirements Issue 1-10-1: Assumptions on UE Proposal 1 is agreeable since only single carrier is assumed for RedCap in release 17. The WI includes RedCap UEs with both 1 receive branch and 2 receive branches. For RedCap UEs with 2 receive branches, we propose to also agree on following: “For RedCap UE with 2 receive branches, the release 15 NR UE measurement requirements are reused that are not affected by the reduced BW.” |  |
| MediaTek |  | ### Sub-topic 1-1: Inter-frequency support Issue 1-1-1: Inter-frequency support Support Inter-frequency mobility in RedCap UEs in Rel-17. ### Sub-topic 1-2: Inter-RAT 2G/3G in IDLE/INACTIVE states Issue 1-2-1: Inter-RAT 2G/3G in IDLE/INACTIVE states Don’t support Inter-RAT mobility 2G/3G in IDLE/INACTIVE states for RedCap UEs in Rel-17. ### Sub-topic 1-3: Inter-RAT 2G/3G in CONNECTD state Issue 1-3-1: Inter-RAT 2G/3G in CONNECTED state Don’t support Inter-RAT mobility 2G/3G in CONNECTED state for RedCap UEs in Rel-17. ### Sub-topic 1-4: Inter-RAT LTE in IDLE/INACTIVE states Issue 1-4-1: Inter-RAT LTE in IDLE/INACTIVE states Support LTE inter-RAT mobility for 5G NR RedCap devices in Rel-17 in IDLE/Inactive mode. ### Sub-topic 1-5: Inter-RAT LTE in CONNECTD state Issue 1-5-1: Inter-RAT LTE in CONNECTED state The LTE inter-RAT mobility for 5G NR RedCap devices in Rel-17 in CONNECTED mode are FFS. Support LTE inter-RAT mobility for 5G NR RedCap devices in Rel-17 in CONNECTED mode. Given the majority of the companies are tend to be supportive to Inter-RAT LTE in IDLE/INACTIVE and in CONNECTED modes, hence we are also fine to support inter-RAT LTE in CONNECTED mode. ### Sub-topic 1-6: SUL support Issue 1-6-1: SUL support Support SUL for RedCap UEs. However, we think this agenda should be discussed in RAN4 RF requirement. ### Sub-topic 1-7: Consideration of LTE cat-M1 or NB-IoT for RedCap in 36.133 Issue 1-7-1: Consideration of LTE cat-M1 or NB-IoT for RedCap in TS 36.133 Agree with proposal 1. ### Sub-topic 1-8: V2X requirements for Rel-17 RedCap Issue 1-8-1: V2X requirements for Rel-17 RedCap Agree with recommended WF. ### Sub-topic 1-9: Combination of features Issue 1-9-1: Combination of features In general, we agree with proposal 1 but also we are fine with recommended WF. ### Sub-topic 1-10: Assumptions for defining UE requirements Issue 1-10-1: Assumptions on UE ### Define requirement by assuming single RF path and single searcher is reasonable. |  |
| Qualcomm |  | Issue 1-1-1: Inter-frequency support Support proposal 1. Inter-frequency requirements are needed for Redcap Issue 1-2-1: Inter-RAT 2G/3G in IDLE/INACTIVE states Support proposal 1. Given limited NR coverage, it is desirable to provide support for LTE inter-RAT mobility Issue 1-3-1: Inter-RAT 2G/3G in CONNECTED state Given limited NR coverage, it is desirable to provide support for LTE inter-RAT mobility in Connected mode as well. Issue 1-6-1: SUL support Agree with proposal 1. Issue 1-7-1: Consideration of LTE cat-M1 or NB-IoT for RedCap in TS 36.133 Agree with proposal 1 Issue 1-8-1: V2X requirements for Rel-17 RedCap Agree with proposal 1 Issue 1-9-1: Combination of features Fine with proposal 1 Issue 1-10-1: Assumptions on UE ### Agree with proposal 1. |  |
| Nokia |  | ### Sub-topic 1-1: Inter-frequency support Issue 1-1-1: Inter-frequency support Support proposal 1. Inter-frequency support is required due to mobility of RedCap devices. It should be handled with equal priority to intra-frequency carrier support. ### Sub-topic 1-2: Inter-RAT 2G/3G in IDLE/INACTIVE states Issue 1-2-1: Inter-RAT 2G/3G in IDLE/INACTIVE states Support proposal 1a. Mobility towards 2G/3G is not needed for RedCap devices in idle state. ### Sub-topic 1-3: Inter-RAT 2G/3G in CONNECTD state Issue 1-3-1: Inter-RAT 2G/3G in CONNECTED state Support proposal 1a. Mobility towards 2G/3G is not needed for RedCap devices in connected state. ### Sub-topic 1-4: Inter-RAT LTE in IDLE/INACTIVE states Issue 1-4-1: Inter-RAT LTE in IDLE/INACTIVE states Support proposal 1a. Underlying LTE coverage should be usable for RedCap devices in idle state. ### Sub-topic 1-5: Inter-RAT LTE in CONNECTD state Issue 1-5-1: Inter-RAT LTE in CONNECTED state Support proposal 1a. Underlying LTE coverage should be usable for RedCap devices in connected state. ### Sub-topic 1-6: SUL support Issue 1-6-1: SUL support Proposal 1. We agree to the recommended WF. ### Sub-topic 1-7: Consideration of LTE cat-M1 or NB-IoT for RedCap in 36.133 Issue 1-7-1: Consideration of LTE cat-M1 or NB-IoT for RedCap in TS 36.133 Support proposal 1. ### Sub-topic 1-8: V2X requirements for Rel-17 RedCap Issue 1-8-1: V2X requirements for Rel-17 RedCap Fine with the recommended WF. ### Sub-topic 1-9: Combination of features Issue 1-9-1: Combination of features We support proposal 1. Each feature being combined with RedCap shall have sufficient justification as complexity reduction is a major target for RedCap device. ### Sub-topic 1-10: Assumptions for defining UE requirements Issue 1-10-1: Assumptions on UE We support proposal 1. |  |

### CRs/TPs comments collection

For close-to-finalize WIs and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
|  | Company A |
|  | Company B |
|  | Company A |
|  | Company B |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic 1-1, | ### Sub-topic 1-1: Inter-frequency support Issue 1-1-1: Inter-frequency support Tentative agreement: RAN4 to develop intra-frequency and inter-frequency requirements for release 17 RedCap with equal priority. ### Sub-topic 1-2: Inter-RAT 2G/3G in IDLE/INACTIVE states Issue 1-2-1: Inter-RAT 2G/3G in IDLE/INACTIVE states Tentative agreement: Do not define inter-RAT RRM requirements on 2G/3G for RedCap UE in Rel-17 ### Sub-topic 1-3: Inter-RAT 2G/3G in CONNECTD state Issue 1-3-1: Inter-RAT 2G/3G in CONNECTED state Tentative agreement: Do not define inter-RAT RRM requirements on 2G/3G for RedCap UE in Rel-17 ### Sub-topic 1-4: Inter-RAT LTE in IDLE/INACTIVE states Issue 1-4-1: Inter-RAT LTE in IDLE/INACTIVE states To be discussed in GTW session 1 Candidate options: - Option 1 (Huawei, Apple, Intel, Oppo, Vivo, MTK, Nokia): RAN4 defines inter-RAT RRM requirements on LTE for RedCap UE in Rel-17 - Option 2 (CMCC; Ericsson, Xiaomi): Do not define inter-RAT RRM requirements for RedCap UE in Rel-17. ### Sub-topic 1-5: Inter-RAT LTE in CONNECTD state Issue 1-5-1: Inter-RAT LTE in CONNECTED state To be discussed in GTW session 1 Candidate options: - Option 1 (Huawei, Apple, Intel, Oppo, Vivo, MTK, Nokia): RAN4 defines inter-RAT RRM requirements on LTE for RedCap UE in Rel-17 - Option 2 (CMCC; Ericsson, Xiaomi): Do not define inter-RAT RRM requirements for RedCap UE in Rel-17. ### Sub-topic 1-6: SUL support Issue 1-6-1: SUL support - Proposal 1: No SUL support is considered for RedCap device. - Proposal 2 (Moderator): No SUL support is considered for RedCap device under the assumption there is no RF support for SUL. - Supporting companies for proposal 1: Nokia, ZTE, Apple, Xiaomi, Vivo, Qualcomm, (Intel?) - Objecting companies for proposal 1: Huawei - Supporting companies for proposal 2: CMCC, Ericsson, MTK, Nokia Tentative agreement: Whether to support SUL for RedCap in RRM depends on RF agreement. ### Sub-topic 1-7: Consideration of LTE cat-M1 or NB-IoT for RedCap in 36.133 Issue 1-7-1: Consideration of LTE cat-M1 or NB-IoT for RedCap in TS 36.133 Candidate options: - Option 1: Whether LTE cat-M1 or NB-IoT shall be considered for RedCap requirement in TS36.133 is discussed in case by case manner after sufficient progress is made in the WI. Tentative agreement: Consideration of LTE cat-M1 or NB-IoT for RedCap is related to LTE inter-RAT support under sub-topic 1-4 and 1-5. This issue shall be discussed after agreement is reached on whether to support LTE inter-RAT support for RedCap. ### Sub-topic 1-8: V2X requirements for Rel-17 RedCap Issue 1-8-1: V2X requirements for Rel-17 RedCap Tentative agreement: V2X is not within the scope of RedCap WID. Thus no more discussions are needed on this topic. ### Sub-topic 1-9: Combination of features Issue 1-9-1: Combination of features Tentative agreement: - When discussing possible combinations of Rel-16 features and RedCap, we should by default assume that the features are not applicable and then identify which features (such as R16 CSI-RS based L3 measurement, L1-SINR measurement, SFTD measurement, CGI reading, and PL-RS change, etc.) can be combined with RedCap case by case based on justification. ### Sub-topic 1-10: Assumptions for defining UE requirements Issue 1-10-1: Assumptions on UE Tentative agreements: Candidate options: Recommendations for 2nd round: Discuss whether following revised proposal 1 can be agreed: - Proposal 1: When RAN4 defines the RRM requirement, it shall further consider the following factors for UE complexity reduction: - Single RF path is expected based on RF agreement - Single searcher is expected given that there is no CA support as agreed in R4-2108359. Also discuss whether proposal 2 can be agreed: - Proposal 2: “For RedCap UE with 2 receive branches, the release 15 NR UE measurement requirements are reused that are not affected by the reduced BW.” |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round (if applicable)

# Topic #2: UE complexity reduction

Proposals based on contributions from AI 9.20.3.2 and 9.20.3.2 are discussed here.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2112130 | Apple | Proposal 1: Regarding the reduced maximum UE bandwidth for RedCap, - for CSI-RS based RLM/BFD, the bandwidth in hypothetical PDCCH transmission parameter tables shall be changed to 24PRBs for 60kHz SCS case in FR1. - for CSI-RS based L1-RSRP, CSI-RS BW for L1-RSRP measurement accuracy requirement with 60kHz SCS in FR1 could be changed to 24PRBs and the corresponding accuracy requirement shall be re-evaluated. No change is needed for CSI-RS based L1-RSRP measurement delay requirement. - for CSI-RS based L3 measurement and CSI-RS based L1-SINR measurement, defer the discussion until sufficient progress is made in the WI. Proposal 2: Regarding the reduced minimum number of Rx branches for RedCap, - for IDLE/Inactive mobility, - Cell re-selection (cell evaluation, measurement, detection) needs to be revisited due to 1Rx - Ranking margin for re-selection (up to measurement accuracy) needs to be revisited 1Rx - for Connected mobility, - SSB based Cell identification/measurement delay needs to be revisited due to 1Rx - SSB based L3 measurement accuracy requirement needs to be revisited due to 1Rx. - for HO/Reestablishment/redirection/active TCI switching, delay requirement needs to be revisited due to 1Rx - for SSB/CSI-RS based RLM/BFD, - Hypothetical PDCCH transmission parameter needs to be revisited due to 1Rx. - The SNR threshold setup in TCs need to be revisited due to 1Rx. - for SSB/CSI-RS based CBD, - No need to revisit evaluation period due to 1Rx. - The margin of L1-RSRP setup in TCs need to be revisited due to 1Rx - for SSB/CSI-RS based L1-RSRP - No need to revisit SSB/CSI-RS based L1-RSRP measurement delay requirement due to 1 Rx - SSB/CSI-RS based L1-RSRP accuracy needs to be revisited due to 1Rx. - For CSI-RS based L3 measurement, L1-SINR measurement, SFTD measurement, CGI reading, and PL-RS change, defer the discussion until sufficient progress is made in the WI. Proposal 3: RAN4 to discuss maximum number of DL MIMO layers and relaxed maximum modulation order in performance phase for demod. Proposal 4: RRM DL measurement is prioritized over the UL transmission of HD-FDD for RedCap UE. Proposal 5: Measurement gap is prioritized over the UL transmission of HD-FDD for RedCap UE, i.e., no UL transmission due to HD-FDD is allowed during MG duration. |
| R4-2112415 | Xiaomi | Proposal 1: The RRM measurement requirements would not be impacted by the UE bandwidth reduction. Proposal 2: RAN4 to further check if the exist BWP switching requirements could be reused for RedCap UE once RAN1 reached an agreement. Proposal 3: The current measurement requirements would not be impacted by HD-FDD. Proposal 4: The neighbour cell dfetection requirements need to be revaluated for 1Rx RedCap UE. Proposal 5: The cell identification delay would need to be extended for 1Rx RedCap UE. Proposal 6: RAN4 to evaluate UE measurement capability for 1Rx RedCap device, and the corresponding side conditions and accuracy requirements. |
| R4-2112644 | Vivo | Proposal 1: The baseline simulation assumptions of above areas listed at appendix are agreed in this meeting and captured in the related WF. Proposal 2: Compared with the legacy requirements, considering more samples when defining PSS/SSS detection requirement for RedCap. Proposal 3: Compared with the legacy requirements, considering more samples when defining SSB index detection delay requirement for RedCap. Proposal 4: Compared with the legacy requirements, considering more samples when defining SS-RSRP measurement requirements for RedCap. Proposal 5: Under the same BLER value for IS and OOS, increasing the number of CCEs or power boosting could be considered to ensure Redcap UE has compatible coverage performance as legacy UE. |
| R4-2113285 | OPPO | Proposal 1: RAN4 to evaluate the fast BWP switching delay if only the central frequency is changed among the BWP for RedCap UE. |
| R4-2113846 | Huawei, HiSilicon | Proposal 1: RRM Mobility measurement requirements for redcap UE can wait for the RAN1 progress, e.g., RF retuning related conclusion. Proposal 2: RAN4 may need to specify the delay and/or interruption requirements of redcap UE RF retuning when RedCap UE hops to transmit/receive outside 20MHz. Proposal 3: Measurement accuracy shall be evaluated for RedCap UE with 1 Rx. Proposal 4: The threshold Qout/Qin in RLM test shall be re-evaluated for RedCap UE with 1 Rx. Proposal 5: It is suggested to further clarify the priority of SMTC and uplink transmission for RedCap UE with HD-FDD. Proposal 6: No RRM impact due to Maximum number of DL MIMO layers and Relaxed maximum modulation order. |
| R4-2113847 | Huawei, HiSilicon | Simulation assumption for measurement accuracy for RedCap UE with 1 RX |
| R4-2113866 | ZTE Corporation | Proposal 1: RRM requirements related to PDSCH processing time, PUSCH preparation time, HARQ-ACK transmission delay and CSI reporting shall be modified. Proposal 2: For some RRM requirements for instance the random access, there may need to be at least two sets of requirements for RedCap UEs depending on whether early indication of the UR type is supported / successful. Proposal 3: RRM requirements related to interruptions shall be modified for redCap UEs |
| R4-2113955 | MediaTek Inc. | Proposal 1: Support further study the bandwidth parameters for CSI-RS based RLM to accommodate the reduced maximum bandwidth in RedCap for FR1. Proposal 2: Support modification on PDCCH transmission parameters for RLM requirements with the single antenna port in RedCap devices, including the ratio of hypothetical PDCCH RE energy to average SSS/CSI-RS RE energy for out-of-sync and in-sync with single antenna port to be with [3] dB higher compared to the existing two antenna requirements. Proposal 3: Support adding the following condition for the evaluation period of 5G NR RLM requirement for HD-FDD: At least 1 RLM-RS must fall with DL occasion within an indication period. The UE determines the indication period as the maximum between the shortest periodicity for radio link monitoring resources and 10 msec. Proposal 4: Support extending the lower bound of the evaluation period of 5G NR RedCap RLM requirement by two compared to the existing general 5G NR RLM requirements. Proposal 5: Support introducing scheduling availability restriction on 5G NR RedCap UEs performing measurements in HD-FDD bands. Proposal 6: Support the study and discussion of reducing the number cells/layers/frequencies/SSBs to be monitored/measured by RedCap UEs. Proposal 7: Support using SLS to determine the reduction in the number cells/layers/frequencies/SSBs in RedCap NR UEs. |
| R4-2114083 | Ericsson | Proposal 1: When RAN4 defines the RRM requirement, it shall further consider the following factors for UE complexity reduction: - Single RF path is expected - Single searcher is expected Proposal 2: CSSFoutside_gap,i = 1 for RedCap UE measurement outside gap. Proposal 3: RAN4 needs to revisit the design for CSSF within gap to guarantee PCell’s mobility. Proposal 4: To simplify UE complexity, RedCap UE won’t support ‘NeedForGap’ and ‘NCSG’ measurement capabilities. Proposal 5: RedCap UE won’t support ‘Inter-frequency without gap’ measurement capability. Proposal 6: The maximum interruption time during paging reception shall be studied for RedCap UEs. Proposal 7: New handover requirements are needed for RedCap UEs. Proposal 8: RAN4 to define new requirements for RRC re-establishment procedure RedCap. Proposal 9: RAN4 to define new requirements for RRC Connection release with redirection procedure for RedCap. Proposal 10: Whether new RA requirements are introduced for RedCap depends on RAN1 PRACH design. Proposal 11: RAN4 to discuss whether Rel-15 NR UE transmit timing requirements can be reused for RedCap. Proposal 12: New RLM and BFD requirements are defined for RedCap based on the simulation study. Proposal 13: New RRM measurement requirements are defined for RedCap based on the simulation study. |
| R4-2114575 | Qualcomm Incorporated | Proposal 1: A RedCap UE shall meet the Te requirements for an initial transmission provided that at-least one SSB is available in the active DL BWP of the UE during the last 160ms. Proposal 2: When a RedCap UE is operating in a RRC configured DL BWP without CORESET0 or SSB (FG 6-1a), CSI-RS/TRS may be used to acquire the reference cell timing. Proposal 3: RAN4 to discuss the impact of Rx branch reduction on the following topics - Cell reselection - RLM/LR - Measurement procedures - Measurement accuracy Proposal 4: Discuss if, and what, conclusions from LTE Cat 1bis UE requirements w.r.t. Rx antenna reduction can be used for NR RedCap UE as well. |
| R4-2114490 | Huawei, HiSilicon | Proposal 1: RedCap UE should support center frequency change/RF retuning across a bandwidth larger than its maximum UE bandwidth. Proposal 2: BWP framework can be utilized for Redcap UE for the scenario that UE only performs RF retuning across a bandwidth wider than UE maximum bandwidth without change BWP specific parameters. Proposal 3: Introduce new BWP retuning delay requirement for Redcap UE applying for cases that only BWP center frequency is changed, the other BWP parameters does not change. The new BWP retuning schedule delay follows current BWP switching delay defined in TS 38.133, i.e. type1 or type2 is up to UE capability reporting. The UE is not required to transmit or receive signals with a time duration of 140us. Proposal 4: Send LS to RAN1 to inform them the new BWP retuning delay requirement for Redcap UE, the specific use case applied with the new requirement is up to RAN1. |
| R4-2113868 | ZTE Corporation | Proposal 1: RRM requirements related to PDSCH processing time, PUSCH preparation time, HARQ-ACK transmission delay and CSI reporting shall be modified. Proposal 2: When the delay is proportional to the number of Rx chains, relax the delay by multiplying a constant parameter. Proposal 3: Accuracy requirements shall be studied after core requirements are specified. Proposal 4: For some RRM requirements for instance the random access, there may need to be at least two sets of requirements for RedCap UEs depending on whether early indication of the UR type is supported / successful. |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 2-1: RLM

Issue 2-1-1: Impact on RLM-RS resources

Proposal 1 (Vivo): The maximum number of RLM-RS resources could be impacted.

Recommended WF

Discuss the proposal.

Issue 2-1-2: Impact on BW used in RLM

Proposal 1 (MTK): Support further study the bandwidth parameters for CSI-RS based RLM to accommodate the reduced maximum bandwidth in RedCap for FR1.

Proposal 2 (Apple, Ericsson): For CSI-RS based RLM/BFD, the bandwidth in hypothetical PDCCH transmission parameter tables shall be changed to 24PRBs for 60kHz SCS case in FR1.

Recommended WF

Discuss the proposals.

Issue 2-1-3: Impact on RLM evaluation period

Proposal 1 (Vivo, Ericsson): The evaluation period could be impacted.

Proposal 2 (Huawei): The existing core requirements of RLM can be applied for RedCap UE. The threshold in test cases needs further study.

Recommended WF

Discuss the proposals.

Issue 2-1-4: How to derive RLM evaluation period if impacted

Proposal 1 (MTK): Support extending the lower bound of the evaluation period of 5G NR RedCap RLM requirement by two compared to the existing general 5G NR RLM requirements.

Recommended WF

Discuss the proposal.

Issue 2-1-5: Impact on parameters for IN/OOS

Proposal 1 (Vivo, Apple, Ericsson): The PDCCH parameters for out-of-sync and in-sync evaluation could be impacted.

Proposal 2 (Huawei): Existing requirements apply.

Recommended WF

Companies to discuss the proposals.

Issue 2-1-6: If impact on IN/OOS, list of impacted parameters

Proposal 1 (Vivo): number of CCEs, power boosting

Proposal 2 (MTK): the ratio of hypothetical PDCCH RE energy to average SSS/CSI-RS RE energy for out-of-sync and in-sync with single antenna port to be with [3] dB higher compared to the existing two antenna requirements.

Recommended WF

Companies to discuss the proposals.

Issue 2-1-7: Impact on in-sync/out-of-sync thresholds

Proposal 1 (Vivo): The PDCCH parameters for out-of-sync and in-sync evaluation could be impacted.

Proposal 1a (Huawei, Ericsson, Apple): The threshold Qout/Qin in RLM test shall be re-evaluated for RedCap UE with 1 Rx.

Proposal 1b (Apple,): For SSB/CSI-RS based RLM,

  - The SNR threshold setup in TCs need to be revisited due to 1Rx.

Recommended WF

The detailed SINR threshold to be discussed under performance part.

### Sub-topic 2-2: Link recovery requirements

Issue 2-2-1: Impact on BW used in BFD

Proposal 1 (Apple, Ericsson): For CSI-RS based BFD, the bandwidth in hypothetical PDCCH transmission parameter tables shall be changed to 24PRBs for 60kHz SCS case in FR1.

Recommended WF

Discuss the proposal.

Issue 2-2-2: Impact on SINR thresholds used in BFD

Proposal 1 (Apple, Ericsson): For SSB/CSI-RS based BFD, The SNR threshold setup in TCs need to be revisited due to 1Rx.

Proposal 2 (Huawei): The threshold in test cases needs further study.

Recommended WF

The SNR threshold setup in test cases can be discussed in the performance part of the WI at later stage. Thus no need to discuss the proposals.

Issue 2-2-3: Other impact on BFD hypothetical transmission parameters

Proposal11 (Apple, Ericsson): For SSB/CSI-RS based BFD,

  - Hypothetical PDCCH transmission parameter needs to be revisited due to 1Rx.

Proposal 2 (Huawei): The existing core requirements of BFD can be applied for RedCap UE.

- Recommended WF

Discuss the proposals.

Issue 2-2-4: Impact on evaluation period used in CBD

Proposal1 1 (Apple): For SSB/CSI-RS based CBD, no impact on evaluation period due to 1 rx. The margin of L1-RSRP setup in TCs need to be revisited due to 1Rx

Proposal 2 (Huawei): The existing core requirements of CBD can be applied for RedCap UE. The threshold in test cases needs further study.

- Recommended WF

Discuss the proposals.

Issue 2-2-5: Impact on BW used in CSI-RS based measurement

Proposal1 1 (Apple): For CSI-RS based L1-RSRP, CSI-RS BW for L1-RSRP measurement accuracy requirement with 60kHz SCS in FR1 could be changed to 24PRBs and the corresponding accuracy requirement shall be re-evaluated. No change is needed for CSI-RS based L1-RSRP measurement delay requirement.

- Recommended WF

Discuss the proposals.

Issue 2-2-6: Impact on L1 RSRP measurement delay requirement

Proposal1 1 (Apple, Huawei):

  - for SSB/CSI-RS based CBD,

    - No need to revisit evaluation period due to 1Rx.

  - for SSB/CSI-RS based L1-RSRP

    - No need to revisit SSB/CSI-RS based L1-RSRP measurement delay requirement due to 1 Rx

Proposal1 2 (Ericsson): L1 RSRP measurement requirements used in CBD are impacted. Simulation assumptions are proposed to study the impact.

- Recommended WF

L1-RSRP measurement accuracy levels and setup in test cases belong to performance part. However, companies to provide their view on the core requirements (number of samples, evaluation period) for the L1-RSRP measurement requirements due to 1Rx.

Issue 2-2-7: Impact on L1 RSRP measurement accuracy requirement

Proposal1 1 (Apple, Huawei):

  - for SSB/CSI-RS based CBD,

    - The margin/threshold of L1-RSRP setup in TCs need to be revisited due to 1Rx

  - for SSB/CSI-RS based L1-RSRP

    - SSB/CSI-RS based L1-RSRP accuracy needs to be revisited due to 1Rx.

Proposal1 2 (Ericsson): L1 RSRP measurement requirements used in CBD are impacted. Simulation assumptions are proposed to study the impact.

- Recommended WF

Discuss the proposals.

Issue 2-2-8: Impact on CSI-RS based L3 measurement and CSI-RS based L1-SINR measurement

Proposal1 1 (Apple): For CSI-RS based L3 measurement and CSI-RS based L1-SINR measurement, defer the discussion until sufficient progress is made in the WI.

- Recommended WF

Try to agree on proposal 1.

### Sub-topic 2-3: HD-FDD UE

Issue 2-3-1: Impact on HD-FDD UE requirements

Proposal 1 (Nokia, Xiaomi): No new RRM core requirements for HD-FDD type A RedCap device compared to legacy NR device need to be specified.

Proposal 2 (Ericsson, Intel, MTK, Apple): New RRM core requirements are needed for HD-FDD type A RedCap UEs.

Recommended WF

Discuss the proposals.

Issue 2-3-2: Whether UE measurements are always prioritized over UL for HD-FDD

Proposal 1 (MTK): Support introducing scheduling availability restriction on 5G NR RedCap UEs performing measurements in HD-FDD bands.

Proposal 1a (Apple): RRM DL measurement is prioritized over the UL transmission of HD-FDD for RedCap UE.

Proposal 1b (Apple): Measurement gap is prioritized over the UL transmission of HD-FDD for RedCap UE, i.e., no UL transmission due to HD-FDD is allowed during MG duration.

Proposal 2 (Huawei): It is suggested to further clarify the priority of SMTC and uplink transmission for RedCap UE with HD-FDD.

Recommended WF

Companies to provide their view on whether the UE measurements are always prioritized over UL transmissions.

Issue 2-3-3: If measurements are prioritized, the scenarios for measurements for HD-FDD

Recommended WF

If UE measurements are prioritzed over UL transmission for HD-FDD (see previous issue), then companies to provide their views on the measurements scenarios, e.g. whether all DL measurements are prioritized or all gaps are prioritized.

Issue 2-3-4: Conditions for performing RLM for HD-FDD UE

Proposal 1 (MTK): Support adding the following condition for the evaluation period of 5G NR RLM requirement for HD-FDD: At least 1 RLM-RS must fall with DL occasion within an indication period. The UE determines the indication period as the maximum between the shortest periodicity for radio link monitoring resources and 10 msec.

Recommended WF

Discuss the proposals.

### Sub-topic 2-4: Measurement capability

Issue 2-4-1: Whether measurement capability may have to be reduced

Proposal 1 (Vivo): RedCap measurement capability is reduced.

Proposal 2 (MTK): Support the study and discussion of reducing the number cells/layers/frequencies/SSBs to be monitored/measured by RedCap UEs.

Recommended WF

Discuss the proposals.

Issue 2-4-2: If measurement capability is reduced, how to determine the measurement capability for RedCap

Proposal 1 (Vivo): The inter-frequency carriers for a Redcap terminal to be capable of monitoring is at least 3. Similar rule could be applied for LTE FDD/TDD if necessary.

Proposal 2 (MTK): Support using system level simulation (SLS) to determine the reduction in the number cells/layers/frequencies/SSBs in RedCap NR UEs.

Recommended WF

Discuss the proposals.

Issue 2-4-3: Multicarrier operation

Proposal 1 (Nokia): RRM core requirements for RedCap device are based on single CC.

Recommended WF

Following agreement was reached at previous meeting [R4-2108359]: “RAN4 should focus on RRM requirements that are relevant to PCell operation, i.e. no DC or CA is considered for RedCap in release 17.“. Therefore no need to discuss proposal 1 further.

### Sub-topic 2-5: UE transmit timing

Issue 2-5-1: Impact on UE transmit timing requirements

Proposal 1 (Apple, Huawei): The existing UE transmit timing requirements (including Te and Tq) can be applied for RedCap UE.

Proposal 2 (Qualcomm Incorporated): A RedCap UE shall meet the Te requirements for an initial transmission provided that at-least one SSB is available in the active DL BWP of the UE during the last 160ms.

Proposal 3 (Ericsson): RAN4 to discuss whether Rel-15 NR UE transmit timing requirements can be reused for RedCap.

Recommended WF

Discuss the proposals

### Sub-topic 2-6: Interruptions

Issue 2-6-1: Impact on interruption requirements in IDLE/INACTIVE states

Proposal 1 (Ericsson, Intel): The maximum interruption time during paging reception shall be studied for RedCap UEs. Simulation assumptions proposed.

Proposal 2 (Vivo, Apple): There is no impact on the requirement for maximum interruption in paging reception.

Recommended WF

Discuss the proposals.

Issue 2-6-2: Impact on interruption requirements in CONNECTED state

Proposal 1 (ZTE): RRM requirements related to interruptions shall be modified for redCap UEs.

Proposal 2 (Vivo, Apple): The interruption requirements defined at 8.2 are not applicable (no impact) for Redcap.

Recommended WF

Discuss the proposals.

### Sub-topic 2-7: Random Access

Issue 2-7-1: Type of RAs

Proposal 1 (ZTE): Both 2-step and 4-step RA requirements shall be specified for reduced capability UEs.

Proposal 2 (Ericsson): Whether new RA requirements are introduced for RedCap depends on RAN1 PRACH design.

Recommended WF

Companies to provide their view on the proposal.

Issue 2-7-2: RA requirements based on UE type

Proposal 1 (ZTE): For some RRM requirements for instance the random access, there may need to be at least two sets of requirements for RedCap UEs depending on whether early indication of the UR type is supported / successful.

Proposal 2 (Ericsson): Whether new RA requirements are introduced for RedCap depends on RAN1 PRACH design.

Recommended WF

Companies to provide their view on the proposal.

### Sub-topic 2-8: BWP switching

Issue 2-8-1: RRM impact due to BWP switching

Proposal 1 (Xiaomi): RAN4 to further check if the exist BWP switching requirements could be reused for RedCap UE once RAN1 reached an agreement.

Proposal 2 (Huawei, Oppo): RAN4 to evaluate the fast BWP switching delay if only the central frequency is changed among the BWP for RedCap UE.

Proposal 2a (Huawei):

      - RedCap UE should support center frequency change/RF retuning across a bandwidth larger than its maximum UE bandwidth.

      - BWP framework can be utilized for Redcap UE for the scenario that UE only performs RF retuning across a bandwidth wider than UE maximum bandwidth without change BWP specific parameters.

      - Introduce new BWP retuning delay requirement for Redcap UE applying for cases that only BWP center frequency is changed, the other BWP parameters does not change. The new BWP retuning schedule delay follows current BWP switching delay defined in TS 38.133, i.e. type1 or type2 is up to UE capability reporting. The UE is not required to transmit or receive signals with a time duration of 140us.

Recommended WF

Companies to provide their views on the proposals.

Issue 2-8-2: If new BWP retuning delay is agreed in RAN4, whether to send LS to RAN1 on new BWP retuning delay

Proposal 1 (Huawei): Send LS to RAN1 to inform them the new BWP retuning delay requirement for Redcap UE, the specific use case applied with the new requirement is up to RAN1.

Recommended WF

If new BWP retuning delay is agreed for RedCap, discuss whether to send LS to RAN1.

### Sub-topic 2-9: RRM impact of UE transmitting/receiving outside 20 MHz

Issue 2-9-1: RRM impact of UE transmit/receive outside 20MHz

Proposal 1 (Huawei): RAN4 may need to specify the delay and/or interruption requirements of redcap UE RF retuning when RedCap UE hops to transmit/receive outside 20MHz.

Recommended WF

Companies to provide their view on the proposal.

Issue 2-9-2: Reference cell timing in RedCap BWP

Proposal 1 (Qualcomm Incorporated): When a RedCap UE is operating in a RRC configured DL BWP without CORESET0 or SSB (FG 6-1a), CSI-RS/TRS may be used to acquire the reference cell timing.

Recommended WF

Companies to provide their view on the proposal.

### Sub-topic 2-10: CONNECTED mode measurements

Issue 2-10-1: CSSF design for RedCap UE measurements

Proposal 1 (Ericsson):

For measurement outside gap:  CSSFoutside_gap,i = 1

For measurement within gap: RAN4 needs to revisit the design for CSSF within gap to guarantee PCell’s mobility.

Recommended WF

Discuss the proposal.

Issue 2-10-2: Gapless measurement capabilities for RedCap UE: ‘NeedForGap’ and ‘NCSG’

Proposal 1 (Ericsson): To simplify UE complexity, RedCap UE won’t support ‘NeedForGap’ and ‘NCSG’ measurement capabilities.

Recommended WF

Discuss the proposal.

Issue 2-10-3: Gapless measurement capabilities for RedCap UE: ‘inter-frequency without gap’

Proposal 2 (Ericsson): RedCap UE won’t support ‘Inter-frequency without gap’ measurement capability.

Recommended WF

Discuss the proposal.

Issue 2-10-4: Measurement period for cell detection

Proposal 1 (vivo): The cell detection delay needs to be extended due to 1Rx.

Recommended WF

Discuss the proposal.

Issue 2-10-5: Measurement period for SS-RSRP measurement

Proposal 1 (vivo): The SS-RSRP measurement delay needs to be extended due to 1Rx.

Proposal 2 (ZTE): When the delay is proportional to the number of Rx chains, relax the delay by multiplying a constant parameter.

Recommended WF

Discuss the proposals.

Issue 2-10-6: Measurement period for SSB index detection

Proposal 1 (vivo): The SSB index detection delay needs to be extended due to 1Rx.

Recommended WF

Discuss the proposal.

### Sub-topic 2-11: Others

Issue 2-11-1: RRM impact due to maximum number of DL MIMO layers

Proposal 1 (Apple): RAN4 to discuss maximum number of DL MIMO layers in performance phase for demod.

Proposal 2 (Huawei, HiSilicon): No RRM impact due to Maximum number of DL MIMO layers and Relaxed maximum modulation order.

Recommended WF

Discuss the proposals.

Issue 2-11-2: RRM impact related to PDSCH processing time, PUSCH preparation time, HARQ-ACK transmission delay and CSI reporting

Proposal 1 (ZTE): RRM requirements related to PDSCH processing time, PUSCH preparation time, HARQ-ACK transmission delay and CSI reporting shall be modified.

Recommended WF

Companies to provide their view on whether there is any RRM impact due to PDSCH processing time, PUSCH preparation time, HARQ-ACK transmission delay and CSI reporting

Issue 2-11-3: RRM impact due to relaxed maximum modulation order

Proposal 1 (Apple): RAN4 to discuss relaxed maximum modulation order in performance phase for demod.

Proposal 2 (Huawei): No RRM impact due to relaxed maximum modulation order.

Recommended WF

Companies to provide their view on whether there is any RRM impact due to relaxed maximum modulation order.

Issue 2-11-4: Relation to LTE cat-1bis requirements

Proposal 1 (Qualcomm Incorporated): Discuss if, and what, conclusions from LTE Cat 1bis UE requirements w.r.t. Rx antenna reduction can be used for NR RedCap UE as well.

Recommended WF

Discuss the proposal

## Companies views’ collection for 1st round

### Open issues

| Company | Comments |
| --- | --- |
| Company A | ### Sub-topic 2-1: RLM Issue 2-1-1: Impact on RLM-RS resources Issue 2-1-2: Impact on BW used in RLM Issue 2-1-3: Impact on RLM evaluation period Issue 2-1-4: How to derive RLM evaluation period if impacted Issue 2-1-5: Impact on parameters for IN/OOS Issue 2-1-6: If impact on IN/OOS, list of impacted parameters Issue 2-1-7: Impact on in-sync/out-of-sync thresholds ### Sub-topic 2-2: Link recovery requirements Issue 2-2-1: Impact on BW used in BFD Issue 2-2-2: Impact on SINR thresholds used in BFD Issue 2-2-3: Other impact on BFD hypothetical transmission parameters Issue 2-2-4: Impact on evaluation period used in CBD Issue 2-2-5: Impact on BW used in CSI-RS based measurement Issue 2-2-6: Impact on L1 RSRP measurement delay requirement Issue 2-2-7: Impact on L1 RSRP measurement accuracy requirement Issue 2-2-8: Impact on CSI-RS based L3 measurement and CSI-RS based L1-SINR measurement ### Sub-topic 2-3: HD-FDD UE Issue 2-3-1: Impact on HD-FDD UE requirements Issue 2-3-2: Whether UE measurements are always prioritized over UL for HD-FDD Issue 2-3-3: If measurements are prioritized, the scenarios for measurements for HD-FDD Issue 2-3-4: Conditions for performing RLM for HD-FDD UE ### Sub-topic 2-4: Measurement capability Issue 2-4-1: Whether measurement capability may have to be reduced Issue 2-4-2: If measurement capability is reduced, how to determine the measurement capability for RedCap Issue 2-4-3: Multicarrier operation ### Sub-topic 2-5: UE transmit timing Issue 2-5-1: Impact on UE transmit timing requirements ### Sub-topic 2-6: Interruptions Issue 2-6-1: Impact on interruption requirements in IDLE/INACTIVE states Issue 2-6-2: Impact on interruption requirements in CONNECTED state ### Sub-topic 2-7: Random Access Issue 2-7-1: Type of RAs Issue 2-7-2: RA requirements based on UE type ### Sub-topic 2-8: BWP switching Issue 2-8-1: RRM impact due to BWP switching Issue 2-8-2: If new BWP retuning delay is agreed in RAN4, whether to send LS to RAN1 on new BWP retuning delay ### Sub-topic 2-9: RRM impact of UE transmitting/receiving outside 20 MHz Issue 2-9-1: RRM impact of UE transmit/receive outside 20MHz Issue 2-9-2: Reference cell timing in RedCap BWP ### Sub-topic 2-10: CONNECTED mode measurements Issue 2-10-1: CSSF design for RedCap UE measurements Issue 2-10-2: Gapless measurement capabilities for RedCap UE: ‘NeedForGap’ and ‘NCSG’ Issue 2-10-3: Gapless measurement capabilities for RedCap UE: ‘inter-frequency without gap’ Issue 2-10-4: Measurement period for cell detection Issue 2-10-5: Measurement period for SS-RSRP measurement Issue 2-10-6: Measurement period for SSB index detection ### Sub-topic 2-11: Others Issue 2-11-1: RRM impact due to maximum number of DL MIMO layers Issue 2-11-2: RRM impact related to PDSCH processing time, PUSCH preparation time, HARQ-ACK transmission delay and CSI reporting Issue 2-11-3: RRM impact due to relaxed maximum modulation order Issue 2-11-4: Relation to LTE cat-1bis requirements |
| ZTE | Issue 2-7-1: Type of RAs Support Option 1. From our perspective, there is no objection in RAN1 saying that 2-step RACH is not supported for RedCap. 2-step RACH is defined in a generic way such that it applies to almost all scenarios when random access is involved, so from our point of view 2-step RA is supported in RAN1. If companies have different understandings on this, we can draft a LS to RAN1 asking them whether 2-step RA is supported (although we won’t feel necessary to do so but if companies prefer to ask we can ask). Issue 2-10-5: Measurement period for SS-RSRP measurement Support both options. Actually what we were proposing is how to extend if the delay needs to be extended. Issue 2-11-2: RRM impact related to PDSCH processing time, PUSCH preparation time, HARQ-ACK transmission delay and CSI reporting We think that proposal 1 can be used as a baseline. |
| Huawei | ### Sub-topic 2-1: RLM Issue 2-1-1: Impact on RLM-RS resources Don’t observe the impact on SSB based RLM resources. For CSI-RS based RLM with 60Hz SCS, the current RLM-RS resource would be impacted. Issue 2-1-2: Impact on BW used in RLM Support proposal 1. Issue 2-1-3: Impact on RLM evaluation period Before discussing the details, we’d like to know which complexity reduction feature we are discussing for this issue? For bandwidth reduction, 1 Rx or HD-FDD? If we are talking about bandwidth reduction, we support option 2. As discussed in issue 2-1-1, PDCCH transmission parameters table may be impacted due to bandwidth reduction. However we don’t observe the clear justification why RLM evaluation period need to be relaxed. Issue 2-1-4: How to derive RLM evaluation period if impacted Before discussing the details, we’d like to know which complexity reduction feature we are discussing for this issue? For bandwidth reduction, or 1 Rx or HD-FDD? For bandwidth reduction, we don’t observe the justification of extending the lower bound of the RLM evaluation period. If we are talking about the impact due to HD-FDD, we need further consideration. Issue 2-1-5: Impact on parameters for IN/OOS If this is talking about the bandwidth reduction, for CSI-RS based RLM, proposal 1 is ok. Issue 2-1-6: If impact on IN/OOS, list of impacted parameters Disagree with power boosting. In existing RLM PDCCH transmission parameters table, Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy=4db, it already reached the upper bound of boosting range. We doubt the feasibility of power boosting for RLM. Issue 2-1-7: Impact on in-sync/out-of-sync thresholds Agree with proposal 1a and 1b. ### Sub-topic 2-2: Link recovery requirements Issue 2-2-1: Impact on BW used in BFD Agree with proposal 1 Issue 2-2-2: Impact on SINR thresholds used in BFD No big difference between proposal 1 and proposal 2. Issue 2-2-3: Other impact on BFD hypothetical transmission parameters Regarding proposal 1, as we discussed in RLM, increasing hypothetical boosting is not feasible as 4dB is already the upper bound of dynamic power control. BFD is for beam level link quality monitoring. If there is no change on cell level RLM, increasing power boosting for BFD is very curious. It means the beam failure of RedCap UE with 1RX can be detected in the center of the cell, however the cell level link failure still happens at the same position as 2RX. We need further consideration on the case. Issue 2-2-4: Impact on evaluation period used in CBD No big difference between proposal 1 and proposal 2. Proposal 2 focus on evaluation period as well. Issue 2-2-5: Impact on BW used in CSI-RS based measurement We agree the issue need to be discussed, however whether reduction to 24RB needs further discussion as the accuracy degradation would seriously impact system performance. Issue 2-2-6: Impact on L1 RSRP measurement delay requirement Support option 1. Issue 2-2-7: Impact on L1 RSRP measurement accuracy requirement Support option 1. Issue 2-2-8: Impact on CSI-RS based L3 measurement and CSI-RS based L1-SINR measurement Agree with option 1 and recommended WF. ### Sub-topic 2-3: HD-FDD UE Issue 2-3-1: Impact on HD-FDD UE requirements We think at least the clarification the priority of SMTC and uplink transmission for RedCap UE with HD-FDD is needed. Issue 2-3-2: Whether UE measurements are always prioritized over UL for HD-FDD Support option 2. And maybe option 1a for prioritize DL measurement is feasible. Issue 2-3-3: If measurements are prioritized, the scenarios for measurements for HD-FDD Needs further discussion Issue 2-3-4: Conditions for performing RLM for HD-FDD UE In LTE, there is a similar condition. Thus the condition of “At least 1 RLM-RS must fall with DL occasion within an indication period.” may be reasonable. ### Sub-topic 2-4: Measurement capability Issue 2-4-1: Whether measurement capability may have to be reduced Needs further discussion. Issue 2-4-2: If measurement capability is reduced, how to determine the measurement capability for RedCap Needs further discussion. Issue 2-4-3: Multicarrier operation Agree with the recommended WF. ### Sub-topic 2-5: UE transmit timing Issue 2-5-1: Impact on UE transmit timing requirements Support proposal 1. Regarding proposal 2, in R15 the case SSB is not in active BWP is available as well. However there is no additional restriction to apply Te requirements in active BWP in legacy requirements as well. So we are not sure the necessity of proposal 2. ### Sub-topic 2-6: Interruptions Issue 2-6-1: Impact on interruption requirements in IDLE/INACTIVE states Prefer proposal 2. In existing paging reception interruption, a general description is made as TSI-NR + 2*Ttarget_cell_SMTC_period ms. No concrete value for each items. Therefore we think this part can be unchanged for redcap UE. Issue 2-6-2: Impact on interruption requirements in CONNECTED state Prefer option 1. BWP switching time may be reduced, then the interruption accordingly would be modified. ### Sub-topic 2-7: Random Access Issue 2-7-1: Type of RAs Prefer proposal 2. Issue 2-7-2: RA requirements based on UE type Depends on the conclusion of issue 2-7-2. ### Sub-topic 2-8: BWP switching Issue 2-8-1: RRM impact due to BWP switching Option 2. The max UE CBW for redcap is 20MHz, while the system CBW is up to 100MHz, thus if redcap UE is configured in the middle of system CBW, it will introduce severe PUSCH fragment, and lead to UL performance impact cause only part of the CBW can be used for other non-redcap UEs. To solve the coexistence between redcap UE and legacy non-redcap UE, we think it is necessary to define the fast BWP switching that only center frequency change across CBW larger than its max UE CBW. Our proposal is to reuse the delay definition as type 1 and type 2 in TS 38.133, but the time period that UE is not required to transmit or receive is 140us. Issue 2-8-2: If new BWP retuning delay is agreed in RAN4, whether to send LS to RAN1 on new BWP retuning delay Considering RAN1 is discussing on many specific use cases that would benefit from the fast BWP switching requirement, we should send LS to RAN1 to inform them about the agreement. Additionally, if RAN4 cannot decide on the specific value of the fast BWP switching requirement, we still need to send LS to RAN1 on the status that RAN4 is evaluating on the fast BWP switching requirement. We provide a draft LS to trigger the discussion on fast BWP switching for Redcap: LS on fast BWP switching delay requirement for Redcap RAN4 has discussed on the BWP switching time for Redcap UE, and RAN4 sees the necessity to introduce fast BWP switching delay requirement for Redcap that only BWP center frequency is changed, the other BWP parameters does not change. The new BWP switching delay follows current BWP switching delay defined in TS 38.133, i.e. type1 or type2 is up to UE capability reporting. The UE is not required to transmit or receive signals with a time duration of 140us. The specific use case applied with the new requirement is up to RAN1. ### Sub-topic 2-9: RRM impact of UE transmitting/receiving outside 20 MHz Issue 2-9-1: RRM impact of UE transmit/receive outside 20MHz Proposal 1. For the purpose of improving RedCap UE data rate experience, network may need to schedule a piece of 20MHz resource for RedCap which has lower overhead with transmitting less always-on signals (e.g. SSB and system information in the downlink). In other words, network may allocate redcap UE to a 20MHz bandwidth which does not contain SSB. In addition, from neighbour cell RRM measurement point of view, the target to-be-measured SSB may be not in the active BWP as well. Therefore in this case RedCap UE needs to retune RF to perform RRM measurement. RAN1 is discussing the RF retuning time and BWP operation scheme for Redcap UE. Certain enhancement on reduced RF retuning time was proposed. RAN4 can wait for the progress from RAN1. Issue 2-9-2: Reference cell timing in RedCap BWP It depends on RAN1 design to acquire the serving cell timing. It shall be noted that for L3 mobility measurement, it focus on neighbour cell measurement, whether CSI-RS based L3 measurement is needed depends on the conclusion of issue 2-2-8. ### Sub-topic 2-10: CONNECTED mode measurements Issue 2-10-1: CSSF design for RedCap UE measurements We think the searcher for redcap UE is 1. However for measurement outsidegap, if there are inter-frequency without gap layer, the CSSFoursidegap is larger than 1. For measurement within gap, we are wondering whether per FR gap is needed or not. Issue 2-10-2: Gapless measurement capabilities for RedCap UE: ‘NeedForGap’ and ‘NCSG’ Proposal 1 seems reasonable. Issue 2-10-3: Gapless measurement capabilities for RedCap UE: ‘inter-frequency without gap’ Depends on the network deployment, operator’s input is encouraged. Issue 2-10-4: Measurement period for cell detection It is early to draw conclusion whether 1 RX would extend cell detection delay, simulation are suggested. Issue 2-10-5: Measurement period for SS-RSRP measurement 1RX would impact measurement accuracy rather than measurement period. Issue 2-10-6: Measurement period for SSB index detection It is early to draw conclusion whether 1 RX would extend SSB index detection, simulation are suggested. ### Sub-topic 2-11: Others Issue 2-11-1: RRM impact due to maximum number of DL MIMO layers Proposal 1 and 2 are supposed be the same. Issue 2-11-2: RRM impact related to PDSCH processing time, PUSCH preparation time, HARQ-ACK transmission delay and CSI reporting It depends on RAN1 conclusion whether the processing time, HARQ feedback etc. would be modified. Issue 2-11-3: RRM impact due to relaxed maximum modulation order Proposal 1 and 2 are supposed be the same. Issue 2-11-4: Relation to LTE cat-1bis requirements Suggest to discuss case by case. |
| Apple | ### Sub-topic 2-1: RLM Issue 2-1-1: Impact on RLM-RS resources The number of RLM-RS is determined by RAN1 in TS38.213. RAN4 could follow the RAN1 conclusion if have. Issue 2-1-2: Impact on BW used in RLM Support option 2. The most possible BW of CSI-RS for RLM/BFD is 24PRBs with 60kHz in FR1 for 20MHz RedCap reduced BW. Issue 2-1-3: Impact on RLM evaluation period We don’t think the evaluation period shall be revised due to RedCap, like in LTE we have the same evaluation period for cat-0 UE as the legacy UE. The RLM requirement/test might be impacted from the following three aspects: - Hypothetical PDCCH transmission parameter may need change due to 1Rx and reduced BW (Current hypothetical BW for CSI-RS is larger than 20MHz in 60kHz). - RLM requirement applicability due to HD-FDD. - The SNR threshold setup in TCs may need change due to 1Rx Thus, we propose option 3 for this issue: Proposal 3: The existing OOS/IS evaluation period in core requirements of RLM can be applied for RedCap UE. The hypothetical PDCCH transmission parameter, requirement applicability due to HD-FDD, and SNR1/2/3/4/5 threshold in test cases needs further study. Issue 2-1-4: How to derive RLM evaluation period if impacted As discussed in issue 2-1-3, we don’t think the evaluation period needs to be changed for RedCap. Issue 2-1-5: Impact on parameters for IN/OOS Support proposal 1 due to reduced BW and/or 1Rx. Hypothetical PDCCH transmission parameter may need change due to 1Rx (power boosting for PDCCH for IS) and reduced BW (Current hypothetical BW for CSI-RS is larger than 20MHz in 60kHz). Issue 2-1-6: If impact on IN/OOS, list of impacted parameters The further power boosting shall not be considered in OOS hypothetical parameter as commented by Huawei, however, it’s still FFS for further power boosting in IS hypothetical parameter (the current assumption is 0dB as duplicated below). We don’t think CCE needs to be changed for 1Rx. We have proposal for this issue 2-1-6, Proposal 3: Due to 1Rx, RAN4 to discuss on whether and how to update the ratio of hypothetical PDCCH RE energy to average SSS/CSI-RS RE energy in IS evaluation. Reuse the existing CCE number of OOS/IS and existing ratio of hypothetical PDCCH RE energy to average SSS/CSI-RS RE energy in OOS evaluation for 1Rx RedCap. Issue 2-1-7: Impact on in-sync/out-of-sync thresholds Support proposal 1b and 1a, and we would like to clarify proposal 1a doesn’t mean the BLER threshold need to change but means the SNR threshold for Qout an Qin needs re-evaluation. ### Sub-topic 2-2: Link recovery requirements Issue 2-2-1: Impact on BW used in BFD Support proposal 1. The most possible BW of CSI-RS for RLM/BFD is 24PRBs with 60kHz in FR1 for 20MHz RedCap reduced BW. Issue 2-2-2: Impact on SINR thresholds used in BFD Support proposal 1 and 2. Issue 2-2-3: Other impact on BFD hypothetical transmission parameters The existing hypothetical power boosting parameter for BFD is 0dB, so technically current BFD hypothetical assumption allows further power boosting. But agree with Huawei’s observation , we need FFS on whether or how to do such further power boosting. Issue 2-2-4: Impact on evaluation period used in CBD Support both proposal 1 and proposal 2, they are technically same. Issue 2-2-5: Impact on BW used in CSI-RS based measurement Support proposal 1 since 24PRBs is the max BW we could use for CSI-RS based measurement with 60kHz in FR1 for RedCap, and the accuracy requirement needs re-evaluation. Issue 2-2-6: Impact on L1 RSRP measurement delay requirement Support proposal 1. Issue 2-2-7: Impact on L1 RSRP measurement accuracy requirement Support proposal 1. Issue 2-2-8: Impact on CSI-RS based L3 measurement and CSI-RS based L1-SINR measurement Support proposal 1. ### Sub-topic 2-3: HD-FDD UE Issue 2-3-1: Impact on HD-FDD UE requirements Support option 2. The prioritization between DL measurement and UL scheduling shall be defined. Issue 2-3-2: Whether UE measurements are always prioritized over UL for HD-FDD Support proposal 1a and 1b. Issue 2-3-3: If measurements are prioritized, the scenarios for measurements for HD-FDD Can be further discussed after concluded in issue 2-3-2. Issue 2-3-4: Conditions for performing RLM for HD-FDD UE Need FFS. Also depends on if RLM evaluation is prioritized over UL scheduling or not. Since in NR UE may have multiple RLM-RSs, the condition might be further clarified: “For evaluation of each RLM-RS configured by network, at least 1 RLM-RS sample must fall with DL occasion within an indication period” ### Sub-topic 2-4: Measurement capability Issue 2-4-1: Whether measurement capability may have to be reduced Support proposal 2. Issue 2-4-2: If measurement capability is reduced, how to determine the measurement capability for RedCap Need FFS on the methodology to determine the capability reduction. Issue 2-4-3: Multicarrier operation Agree with the recommended WF. ### Sub-topic 2-5: UE transmit timing Issue 2-5-1: Impact on UE transmit timing requirements Support proposal 1. ### Sub-topic 2-6: Interruptions Issue 2-6-1: Impact on interruption requirements in IDLE/INACTIVE states Same understanding as Huawei. Support proposal 2. Issue 2-6-2: Impact on interruption requirements in CONNECTED state Support proposal 2a. The wording of proposal 2 might need further rephrased, Proposal 2a: RedCap features would not cause impact to the existing interruption requirements defined at 8.2. ### Sub-topic 2-7: Random Access Issue 2-7-1: Type of RAs Fine with Proposal 2 Issue 2-7-2: RA requirements based on UE type Fine with Proposal 2 ### Sub-topic 2-8: BWP switching Issue 2-8-1: RRM impact due to BWP switching For instance, in DCI based BWP switching, the PDCCH parsing time and BB/RF preparation time is up to UE implementation and we think RedCap UE cannot have shorter processing time for those components. We have proposal 3 for BWP switching in RedCap: Proposal 3: the exist BWP switching requirements could be reused for RedCap UE. Issue 2-8-2: If new BWP retuning delay is agreed in RAN4, whether to send LS to RAN1 on new BWP retuning delay Up to discussion on issue 2-8-1. ### Sub-topic 2-9: RRM impact of UE transmitting/receiving outside 20 MHz Issue 2-9-1: RRM impact of UE transmit/receive outside 20MHz Generally fine with proposal 1. But need to wait for conclusions from RAN1. Issue 2-9-2: Reference cell timing in RedCap BWP Need more conclusions from RAN1. For timing tracking, it makes sense to use proposal 1, but for mobility based measurement, gap based or interruption based measurement could be considered on the SSB outside active BWP. ### Sub-topic 2-10: CONNECTED mode measurements Issue 2-10-1: CSSF design for RedCap UE measurements Fine with 1st bullet in proposal 1, and the inter-frequency without MG shall not be considered at this stage as that’s R16 feature. For the 2nd bullet in proposal 1, we don’t understand the rationale. In our understanding, the existing MG sharing could be used to prioritize the PCell measurement if needed. Issue 2-10-2: Gapless measurement capabilities for RedCap UE: ‘NeedForGap’ and ‘NCSG’ Support proposal 1. Issue 2-10-3: Gapless measurement capabilities for RedCap UE: ‘inter-frequency without gap’ Support proposal 1. Issue 2-10-4: Measurement period for cell detection Need simulation evaluation first. Issue 2-10-5: Measurement period for SS-RSRP measurement Need simulation evaluation first. Consider this issue together with accuracy relaxation. Issue 2-10-6: Measurement period for SSB index detection Need simulation evaluation first. ### Sub-topic 2-11: Others Issue 2-11-1: RRM impact due to maximum number of DL MIMO layers Support both proposal 1 and 2. Issue 2-11-2: RRM impact related to PDSCH processing time, PUSCH preparation time, HARQ-ACK transmission delay and CSI reporting Could proponents explicitly indicate which RRM requirement would be impacted due to PDSCH processing time, PUSCH preparation time, HARQ-ACK transmission delay and CSI reporting change? For instance, we are not sure if PUSCH preparation time would impact any RRM requirement. More conclusions from RAN1 is needed also. Issue 2-11-3: RRM impact due to relaxed maximum modulation order Support both proposal 1 and 2. Issue 2-11-4: Relation to LTE cat-1bis requirements Same view as HW. Need to check the relation in a case by case manner. |
| CMCC | ### Sub-topic 2-1: RLM Issue 2-1-1: Impact on RLM-RS resources Further discuss is needed considering RAN1 conclusion. Issue 2-1-2: Impact on BW used in RLM Both proposal 1 and proposal 2. Issue 2-1-3: Impact on RLM evaluation period In principle, RLM evaluation period should not be relaxed for RedCap UE. More discussion is needed before agree on this. Issue 2-1-4: How to derive RLM evaluation period if impacted More discussion is needed before agree on this. Issue 2-1-5: Impact on parameters for IN/OOS IN/OSS may be impacted due to reduced BW and Rx. Further study is needed. Issue 2-1-6: If impact on IN/OOS, list of impacted parameters Issue 2-1-7: Impact on in-sync/out-of-sync thresholds OK with the recommended WF ### Sub-topic 2-2: Link recovery requirements Issue 2-2-1: Impact on BW used in BFD OK with proposal 1 Issue 2-2-2: Impact on SINR thresholds used in BFD OK with recommended WF Issue 2-2-3: Other impact on BFD hypothetical transmission parameters More discussion is needed. Issue 2-2-4: Impact on evaluation period used in CBD Proposal 1 and Proposal 2. Core requirements on evalution period can be applied for RedCap UE. The parameters in TCs can be discussed later in the performance part. Issue 2-2-5: Impact on BW used in CSI-RS based measurement OK with proposal 1. Meanwhile, we should make agreement that no changes are needed for 15KHz and 30KHz. Issue 2-2-6: Impact on L1 RSRP measurement delay requirement Proposal 1 Issue 2-2-7: Impact on L1 RSRP measurement accuracy requirement Proposal 1 Issue 2-2-8: Impact on CSI-RS based L3 measurement and CSI-RS based L1-SINR measurement OK with proposal 1 ### Sub-topic 2-3: HD-FDD UE Issue 2-3-1: Impact on HD-FDD UE requirements The delay requirements can be reused for HD-FDD. But some core requirements like scheduling restrictions may need to be revisited for HD-FDD. Issue 2-3-2: Whether UE measurements are always prioritized over UL for HD-FDD Further study is needed. Not sure that SMTC of MG should always be prioritized over UL transmission. For example, in some cases, mobility is not very critical for RedCap UE. Issue 2-3-3: If measurements are prioritized, the scenarios for measurements for HD-FDD Issue 2-3-4: Conditions for performing RLM for HD-FDD UE ### Sub-topic 2-4: Measurement capability Issue 2-4-1: Whether measurement capability may have to be reduced No. Many companies propose to define the inter-frequency and inter-RAT measurement requirements and think the mobility is essential for RedCap UE. Then we don’t understand the motivation to reduce the number of cells/layers to be monitored. Issue 2-4-2: If measurement capability is reduced, how to determine the measurement capability for RedCap Do not agree with proposal 1. For proposal 2, need to understand better how to use system level simulation to determine the reduction since the measurement capability is related to the spectrum resources operators hold. Issue 2-4-3: Multicarrier operation Agree with the recommended WF. ### Sub-topic 2-5: UE transmit timing Issue 2-5-1: Impact on UE transmit timing requirements Proposal 1: OK Proposal 2: Not sure this is needed, legacy UE also has the similar issue if the SSB is not available in the active DL BWP ### Sub-topic 2-6: Interruptions Issue 2-6-1: Impact on interruption requirements in IDLE/INACTIVE states Proposal 2 Issue 2-6-2: Impact on interruption requirements in CONNECTED state Proposal 2a ### Sub-topic 2-7: Random Access Issue 2-7-1: Type of RAs Proposal 2 Issue 2-7-2: RA requirements based on UE type Proposal 2 ### Sub-topic 2-8: BWP switching Issue 2-8-1: RRM impact due to BWP switching Proposal 2. We think it is feasible to reduce the BWP switching delay if only the central frequency is changed Issue 2-8-2: If new BWP retuning delay is agreed in RAN4, whether to send LS to RAN1 on new BWP retuning delay OK with proposal 1. ### Sub-topic 2-9: RRM impact of UE transmitting/receiving outside 20 MHz Issue 2-9-1: RRM impact of UE transmit/receive outside 20MHz Interruption requirements can be studied for RedCap UE RF retuning. Issue 2-9-2: Reference cell timing in RedCap BWP Agree with proposal 1. ### Sub-topic 2-10: CONNECTED mode measurements Issue 2-10-1: CSSF design for RedCap UE measurements This is related to the searcher assumption. Can be discussed later when the searcher assumption was agreed. Issue 2-10-2: Gapless measurement capabilities for RedCap UE: ‘NeedForGap’ and ‘NCSG’ There is no NCSG and NeedForGap requirements so far. OK to not consider in Rel-17. Issue 2-10-3: Gapless measurement capabilities for RedCap UE: ‘inter-frequency without gap’ Do not agree with the proposal. In Ericsson’s paper, pervious agreements on inter without gap in WF R4-2005348 was copied. But the agreements are copied without the pre-condition. The previous agreement is that : In the case that inter-frequency measurement without MG partially overlapped with MG, it was agreed to define requirements based on the assumption that UE perform measurement within gaps. So in other cases, UE can still perform measurement outside the gap, even though UE is non-CA capable UE. Issue 2-10-4: Measurement period for cell detection Further study is needed. Issue 2-10-5: Measurement period for SS-RSRP measurement Further study is needed. Issue 2-10-6: Measurement period for SSB index detection Further study is needed. ### Sub-topic 2-11: Others Issue 2-11-1: RRM impact due to maximum number of DL MIMO layers Both proposal 1 and proposal 2. Issue 2-11-2: RRM impact related to PDSCH processing time, PUSCH preparation time, HARQ-ACK transmission delay and CSI reporting Further study on the potential impacts on these aspects is needed. Issue 2-11-3: RRM impact due to relaxed maximum modulation order Both proposal 1 and proposal 2. Issue 2-11-4: Relation to LTE cat-1bis requirements Further study is needed. |
| Xiaomi | ### Sub-topic 2-1: RLM Issue 2-1-1: Impact on RLM-RS resources The issue of RLM-RS resources should be decided in RAN1. Issue 2-1-2: Impact on BW used in RLM Support Option 1. We can further study. Issue 2-1-3: Impact on RLM evaluation period Further study is needed. Issue 2-1-4: How to derive RLM evaluation period if impacted Further study is needed. Issue 2-1-5: Impact on parameters for IN/OOS Support Option1. ### Sub-topic 2-2: Link recovery requirements Issue 2-2-1: Impact on BW used in BFD Support Option 1. Issue 2-2-2: Impact on SINR thresholds used in BFD Support the recommended WF. Issue 2-2-3: Other impact on BFD hypothetical transmission parameters Further study is needed. Issue 2-2-4: Impact on evaluation period used in CBD Further study is needed. ### Sub-topic 2-3: HD-FDD UE Issue 2-3-1: Impact on HD-FDD UE requirements We can support Option 2 considering the potential colliding between RRM DL measurement and UL transmission due to HD-FDD. Issue 2-3-2: Whether UE measurements are always prioritized over UL for HD-FDD We prefer to follow the same principle in R15 to prioritize UE measurements over UL transmissions. ### Sub-topic 2-8: BWP switching Issue 2-8-1: RRM impact due to BWP switching Support Option 1. In our knowledge, the issue of fast BWP switching for redcap UE is under discussion in RAN1, so we support to wait for RAN1 decision. Issue 2-8-2: If new BWP retuning delay is agreed in RAN4, whether to send LS to RAN1 on new BWP retuning delay We prefer to wait for RAN1 decision. ### Sub-topic 2-10: CONNECTED mode measurements Issue 2-10-1~ Issue 2-10-3: We can further study. Issue 2-10-4: Measurement period for cell detection Support the proposal. Issue 2-10-5: Measurement period for SS-RSRP measurement Support both proposals. Issue 2-10-6: Measurement period for SSB index detection Support the proposal. |
| OPPO | ### Sub-topic 2-1: RLM Issue 2-1-1: Impact on RLM-RS resources Depending on RAN1. Issue 2-1-2: Impact on BW used in RLM Support Option 1. Issue 2-1-3: Impact on RLM evaluation period FFS. Issue 2-1-4: How to derive RLM evaluation period if impacted FFS. Issue 2-1-5: Impact on parameters for IN/OOS Support Option1. FFS the details based on RAN1’s progress. ### Sub-topic 2-2: Link recovery requirements Issue 2-2-1: Impact on BW used in BFD Support Option 1. Issue 2-2-2: Impact on SINR thresholds used in BFD Support the recommended WF. ### Sub-topic 2-4: Measurement capability Issue 2-4-1: Whether measurement capability may have to be reduced Proposal 2 is fine. Issue 2-4-3: Multicarrier operation Agree with the recommended WF. ### Sub-topic 2-5: UE transmit timing Issue 2-5-1: Impact on UE transmit timing requirements Proposal 1 is OK. Not sure about the case that SSB is always within active DL BWP. ### Sub-topic 2-6: Interruptions Issue 2-6-1: Impact on interruption requirements in IDLE/INACTIVE states Proposal 2 Issue 2-6-2: Impact on interruption requirements in CONNECTED state Proposal 2 ### Sub-topic 2-8: BWP switching Issue 2-8-1: RRM impact due to BWP switching Support Proposal 2 as baseline. RAN1 identified the issue of fast BWP switching for redcap UE, which also has impact on RAN4 requirements. SO RAN4 can evaluate the case of fast BWP switching. Issue 2-8-2: If new BWP retuning delay is agreed in RAN4, whether to send LS to RAN1 on new BWP retuning delay Proposal 1 is ok. The details of LS can be discussed. ### Sub-topic 2-10: CONNECTED mode measurements Issue 2-10-1~ Issue 2-10-3: FFS. Issue 2-10-4: Measurement period for cell detection Support the proposal. Issue 2-10-5: Measurement period for SS-RSRP measurement Support both proposals. Issue 2-10-6: Measurement period for SSB index detection Support the proposal. |
| vivo | ### Sub-topic 2-1: RLM Issue 2-1-1: Impact on RLM-RS resources Issue 2-1-2: Impact on BW used in RLM We agree with the observation from proposal 2 that for 60kHz there is insufficient bandwidth to accommodate all CSI-RS. Whether change it to 24PRBs could be for FFS Issue 2-1-3: Impact on RLM evaluation period To Huawei, we provide our simulation results based on 1Rx, i.e., the complexity reduction feature to derive this conclusion is “reducing number of Rx”. Issue 2-1-4: How to derive RLM evaluation period if impacted FFS Issue 2-1-5: Impact on parameters for IN/OOS To our understanding, a few parameters will jointly impact the coverage issue of Redcap UE, if we use the same IN/OOS value as that used for legacy UE. Other techniques such as increasing aggregation level etc. may be needed to ensure Redcap UE obtains same coverage compared with a legacy UE. Issue 2-1-6: If impact on IN/OOS, list of impacted parameters Based on issue 2-1-5, we provide possible solutions for considerations, i.e., increasing aggregation level, power boosting, or use a different IN/OOS pair. Issue 2-1-7: Impact on in-sync/out-of-sync thresholds OK with proposal 1a. ### Sub-topic 2-2: Link recovery requirements Issue 2-2-1: Impact on BW used in BFD Agree with the observation. Concrete solution, number could be FFS Issue 2-2-2: Impact on SINR thresholds used in BFD Ok with both proposal 1. Seems no big difference between proposal 1 and proposal 2.. Issue 2-2-3: Other impact on BFD hypothetical transmission parameters OK with proposal 1. Issue 2-2-4: Impact on evaluation period used in CBD Issue 2-2-5: Impact on BW used in CSI-RS based measurement Agree the issue should be discussed however the number could be FFS Issue 2-2-6: Impact on L1 RSRP measurement delay requirement Issue 2-2-7: Impact on L1 RSRP measurement accuracy requirement We are ok to study the impact on L1-RSRP measurement Issue 2-2-8: Impact on CSI-RS based L3 measurement and CSI-RS based L1-SINR measurement Agree with the recommended WF. ### Sub-topic 2-3: HD-FDD UE Issue 2-3-1: Impact on HD-FDD UE requirements To our understanding, the issue of 2-3-2 will have impact on core requirements. Issue 2-3-2: Whether UE measurements are always prioritized over UL for HD-FDD Ok with option 2. Issue 2-3-3: If measurements are prioritized, the scenarios for measurements for HD-FDD FFS Issue 2-3-4: Conditions for performing RLM for HD-FDD UE Proposal 1 could be feasible however better for FFS ### Sub-topic 2-4: Measurement capability Issue 2-4-1: Whether measurement capability may have to be reduced We do not see big difference with proposal 1 and 2. OK with proposal 1 and 2. Issue 2-4-2: If measurement capability is reduced, how to determine the measurement capability for RedCap Ok with proposal 1. Issue 2-4-3: Multicarrier operation Agree with the recommended WF. ### Sub-topic 2-5: UE transmit timing Issue 2-5-1: Impact on UE transmit timing requirements Support proposal 1. ### Sub-topic 2-6: Interruptions Issue 2-6-1: Impact on interruption requirements in IDLE/INACTIVE states Support proposal 2. Issue 2-6-2: Impact on interruption requirements in CONNECTED state Support option 2. ### Sub-topic 2-7: Random Access Issue 2-7-1: Type of RAs OK with proposal 2. Issue 2-7-2: RA requirements based on UE type Ok with proposal 2. ### Sub-topic 2-8: BWP switching Issue 2-8-1: RRM impact due to BWP switching Fine with proposal 1 Issue 2-8-2: If new BWP retuning delay is agreed in RAN4, whether to send LS to RAN1 on new BWP retuning delay ### Wait the outcome of issue 2-8-1 ### Sub-topic 2-9: RRM impact of UE transmitting/receiving outside 20 MHz Issue 2-9-1: RRM impact of UE transmit/receive outside 20MHz We think this feature should be discussed at RAN1 and RAN4 should wait for RAN1 dec Issue 2-9-2: Reference cell timing in RedCap BWP Up to RAN1 ### Sub-topic 2-10: CONNECTED mode measurements Issue 2-10-1: CSSF design for RedCap UE measurements Issue 2-10-2: Gapless measurement capabilities for RedCap UE: ‘NeedForGap’ and ‘NCSG’ OK with the proposal Issue 2-10-3: Gapless measurement capabilities for RedCap UE: ‘inter-frequency without gap’ OK with the proposal. Issue 2-10-4: Measurement period for cell detection We get this conclusion based on our simulation results for 1Rx and provide our simulation assumptions and we are open for discussion on simulation assumptions. Issue 2-10-5: Measurement period for SS-RSRP measurement For 1rx, if the accuracy is not changed, the measurement delay should be extended. Issue 2-10-6: Measurement period for SSB index detection Same as 2-10-4 ### Sub-topic 2-11: Others Issue 2-11-1: RRM impact due to maximum number of DL MIMO layers OK with proposal 2. Issue 2-11-2: RRM impact related to PDSCH processing time, PUSCH preparation time, HARQ-ACK transmission delay and CSI reporting FFS Issue 2-11-3: RRM impact due to relaxed maximum modulation order OK with proposal 2. Issue 2-11-4: Relation to LTE cat-1bis requirements ### Suggest to discuss case by case. |
| Ericsson | ### Sub-topic 2-1: RLM Issue 2-1-1: Impact on RLM-RS resources The number of RLM-RS resources that can be supported by a RedCap UE is RAN1 related discussion since it is defined in TS 38.213 Clause 5. We expect RAN1 add new table if the number of resources are different from Rel-15 NR UE. Thus, no need to discussion this issue in RAN4. Issue 2-1-2: Impact on BW used in RLM We agree that the CSI-RS BW in the CSI-RS based RLM need to be reduced for 60 kHz SCS in FR1. Alternatively 60 kHz SCS case should be excluded. We prefer the first option of reducing the BW to 24 PRBs. Thus proposal 2 is agreeable. Issue 2-1-3: Impact on RLM evaluation period At least for 1 Rx UE, RAN4 needs to study whether the release 15 NR RLM evaluation period can be reused. For example, both cat-M and NB-IoT evaluation period were extended in LTE due to 1 Rx operation. We are open to study the performance and whether or not evaluation period is extended can be decided based on the simulation study. For this purpose, we have proposed simulation assumptions in this meeting. We therefore propose a third proposal: - Proposal 3: RAN4 to decide whether to extend the RLM evaluation period for 1 Rx UE (i.e., increase the number of measurement samples) based on simulation study. The evaluation period of 2 Rx UE can be same as Rel-15 NR UE. Issue 2-1-4: How to derive RLM evaluation period if impacted How to derive the RLM evaluation period if it is impacted can be discussed based on how much performance is degraded (if degraded). Therefore we propose to discuss how to derive the extended RLM evaluation period based on the RLM simulation study. Issue 2-1-5: Impact on parameters for IN/OOS We support proposal 1 since at least the measured BW of the RLM RS in the OOS/IS evaluation is going to be different for CSI-RS based RLM evaluations for 60 kHz SCS. Other parameters can remain the same. Issue 2-1-6: If impact on IN/OOS, list of impacted parameters The first step is to evaluate the IN/OOS performance assuming the Rel-15 PDCCH parameters (except BW as commented in previous issue). We don’t see any need to consider different number of CCE, power boosting for 1 Rx at the moment. The need for assuming different number of CCEs and/or power boosting can take place based on the initial PDCCH simulation results if performance degradation is observed. Issue 2-1-7: Impact on in-sync/out-of-sync thresholds We agree that the impact on RLM SINR thresholds could be discussed under performance part. ### Sub-topic 2-2: Link recovery requirements Issue 2-2-1: Impact on BW used in BFD We agree that the CSI-RS BW in the CSI-RS based BFD need to be reduced for 60 kHz SCS in FR1. Alternatively 60 kHz SCS case should be excluded. We prefer the first option of reducing the BW to 24 PRBs. Thus proposal 2 is agreeable. Issue 2-2-2: Impact on SINR thresholds used in BFD We agree with the recommended WF that the SNR threshold in the test setup can be discussed in the performance part of the WI. Issue 2-2-3: Other impact on BFD hypothetical transmission parameters Except the BW for CSI-RS based BFD under 60 kHz SCS, we don’t see any need to consider different PDCCH transmission parameters compared to corresponding parameters in Rel-15 NR. Whether any new PDCCH parameters needs to be considered can be discussed later based on the initial simulation results if performance degradation is observed. Issue 2-2-4: Impact on evaluation period used in CBD Similar comment as for RLM evaluation period in issue 2-1-3. At least for 1 Rx UE, RAN4 needs to study whether the release 15 NR CBD evaluation period (i.e., L1-RSRP measurement period) can be reused. For example, both cat-M and NB-IoT evaluation period were extended in LTE due to 1 Rx operation. We are open to study the performance and whether or not evaluation period is extended can be decided based on the simulation study. For this purpose, we have proposed simulation assumptions in this meeting. We therefore propose a third proposal: - Proposal 3: RAN4 to decide whether to extend the CBD evaluation period for 1 Rx UE based on simulation study. The CBD evaluation period of 2 Rx UE can be same as Rel-15 NR UE. Issue 2-2-5: Impact on BW used in CSI-RS based measurement We agree that the CSI-RS BW in the CSI-RS based L1-RSRP need to be reduced for 60 kHz SCS in FR1. However, we are not sure whether the 2 Rx delay requirements can be reused for 1 Rx. Thus we propose following alternative proposal: - Proposal 2: BW in CSI-RS based L1 RSRP is changed to 24 PRBs for the case 60 kHz SCS in FR1. Whether the L1 RSRP measurement delay requirements from NR can be reused for RedCap UE with 1 Rx is FFS. Issue 2-2-6: Impact on L1 RSRP measurement delay requirement We propose to study the L1-RSRP measurement performance based on simulation study to determine whether the NR L1-RSRP delay requirements can be reused for RedCap 1 Rx UE. Issue 2-2-7: Impact on L1 RSRP measurement accuracy requirement We propose to discuss the accuracy requirements later in performance part of the WI. It might be possible to reuse the same simulations derive the accuracy requirements. Issue 2-2-8: Impact on CSI-RS based L3 measurement and CSI-RS based L1-SINR measurement Proposal 1 is agreeable. ### Sub-topic 2-3: HD-FDD UE Issue 2-3-1: Impact on HD-FDD UE requirements We support proposal 2. Since the HD-FDD UE cannot transmit/receive simultaneously, RAN4 may have to define separate requirements. For example, in eMTC HD-FDD in LTE, the requirements applied provided a certain number of DL subframes available for measurements. Similar conditions are needed for RedCap HD-FDD. Issue 2-3-2: Whether UE measurements are always prioritized over UL for HD-FDD RAN1 is currently discussing different collision scenarios between uplink and downlink for HD-FDD UE. RAN4 should therefore not discuss the UL/DL (measurements) collisions for HD-FDD in parallel, instead it is proposed that prioritization of UE measurements are discussed after RAN1 progress on this matter. Issue 2-3-3: If measurements are prioritized, the scenarios for measurements for HD-FDD Same comment as for issue 2-3-2. RAN1 is currently discussing different collision scenarios between uplink and downlink for HD-FDD UE. RAN4 should therefore not discuss the UL/DL (measurements) collisions for HD-FDD in parallel, instead it is proposed that prioritization of UE measurements are discussed after RAN1 progress on this matter. Issue 2-3-4: Conditions for performing RLM for HD-FDD UE We understand the motivation, but we would like to keep the number of subframes for further study. ### Sub-topic 2-4: Measurement capability Issue 2-4-1: Whether measurement capability may have to be reduced Since single carrier is considered in release 17 RedCap, it is reasonable to assume that the measurement capability in terms of number of number cells/layers/frequencies/SSBs to be measured/monitored is reduced compared to legacy NR UE that supports multiple carriers. Thus we support proposal 2. Issue 2-4-2: If measurement capability is reduced, how to determine the measurement capability for RedCap More discussions are needed on the exact numbers. In this meeting, we propose to reach a conclusion on previous issue related to whether to assume reduced measurement capability. The numbers can be discussed afterwards. Issue 2-4-3: Multicarrier operation ### Sub-topic 2-5: UE transmit timing Issue 2-5-1: Impact on UE transmit timing requirements Proposal 1 is agreeable. ### Sub-topic 2-6: Interruptions Issue 2-6-1: Impact on interruption requirements in IDLE/INACTIVE states We can agree to proposal 2. Issue 2-6-2: Impact on interruption requirements in CONNECTED state There might be impact on the CONNECTED mode interruption requirements since the UE may have to perform RF retuning when changing the RedCap BWP. This is related to discussions in sub-topic 2-8. Thus we share similar view as ZTE (proposal 1). ### Sub-topic 2-7: Random Access Issue 2-7-1: Type of RAs Both proposal 1 and 2 are related. We share the same view as ZTE that both 2-step and 4-step RA requirements can be supported for RedCap. However, we would like to point out that 2-step RA is a release 16 feature and combination of features (i.e. which release 16 features are to be supported for RedCap) are to be discussed at later stage when more progress is reached in the WI). Currently, the basic assumption should be the release 15 requirements. Issue 2-7-2: RA requirements based on UE type In general, the RRM requirements are minimum UE requirements and RAN4 should avoid defining two set of requirements for the same type of UE for the same procedure. Currently, we do not see any need to define two set of requirements. However, we are open to study the impact on RRM requirements due to whether early indication of UE type should be FFS. ### Sub-topic 2-8: BWP switching Issue 2-8-1: RRM impact due to BWP switching We share similar view as Xiaomi, Huawei and Oppo that impact on existing BWP switching requirements shall be evaluated based on latest RAN1 agreements on BWPs. It is proposed to capture this as an issue to be studied more in detail till November meeting. This study may include whether only center frequency is changed while other (e.g. baseband) parameters are kept unchanged. Issue 2-8-2: If new BWP retuning delay is agreed in RAN4, whether to send LS to RAN1 on new BWP retuning delay It is too early to discuss any LS. RAN4 needs to first study the and identify the impact. Thus proposal 1 is not agreeable at the moment. ### Sub-topic 2-9: RRM impact of UE transmitting/receiving outside 20 MHz Issue 2-9-1: RRM impact of UE transmit/receive outside 20MHz The RedCap UE can be configured to operate in a RedCap BWP which does not necessarily have SSBs. Therefore the UE may need to perform RF retuning to other BWPs that have SSBs transmitted (e.g. initial BWP) to be able to perform e.g. time/frequency tracking. RAN4 needs to discuss the requirements for the case when the UE is performing RF retuning to other BWPs for SSB reception. This may for example depend on DRX cycle or SSB periodicity. Issue 2-9-2: Reference cell timing in RedCap BWP It is better to wait for RAN1 agreements on RedCap BWP before making any agreements on using CSI-RS/TRS for acquiring the reference cell timing. ### Sub-topic 2-10: CONNECTED mode measurements Issue 2-10-1: CSSF design for RedCap UE measurements Proposal 1 is agreeable. Issue 2-10-2: Gapless measurement capabilities for RedCap UE: ‘NeedForGap’ and ‘NCSG’ Proposal 1 is agreeable. Issue 2-10-3: Gapless measurement capabilities for RedCap UE: ‘inter-frequency without gap’ Proposal 1 is agreeable. Issue 2-10-4: Measurement period for cell detection We propose to study the cell detection delay based on 1 Rx and whether to extend the delay can be discussed based on the simulation outcome. For the 2 Rx, we propose to agree that the release 15 NR UE requirements with 2 Rx can be reused as proposed in issue 1-10-1. Issue 2-10-5: Measurement period for SS-RSRP measurement We propose to study the measurement period based on 1 Rx and whether to extend the delay can be discussed based on the simulation outcome. For the 2 Rx, we propose to agree that the release 15 NR UE requirements with 2 Rx can be reused as proposed in issue 1-10-1. Issue 2-10-6: Measurement period for SSB index detection We propose to study the measurement period for SSB index detection based on 1 Rx and whether to extend the delay can be discussed based on the simulation outcome. For the 2 Rx, we propose to agree that the release 15 NR UE requirements with 2 Rx can be reused as proposed in issue 1-10-1. ### Sub-topic 2-11: Others Issue 2-11-1: RRM impact due to maximum number of DL MIMO layers We share the similar view as Apple that impact on maximum number of DL MIMO layers should be discussed in the performance phase of the WI in demodulation section. Thus we support proposal 1. It has no impact on RRM requirements (support both options 1 and 2). Issue 2-11-2: RRM impact related to PDSCH processing time, PUSCH preparation time, HARQ-ACK transmission delay and CSI reporting We don’t see any impact on RRM due to PDSCH processing time, PUSCH preparation time, HARQ-ACK transmission delay and CSI reporting. In our view, it is RAN1 issue, see reference Ts38.214 5.3 UE PDSCH processing procedure time. Issue 2-11-3: RRM impact due to relaxed maximum modulation order Proposal 1 is agreeable. Issue 2-11-4: Relation to LTE cat-1bis requirements Since the reference signals used in the LTE cat-1bis requirements and NR reference signals are different (e.g., CRS for Cat-1bis, but SSB or CSI-RS for RedCap), we don’t think the LTE cat-1bis requirements can be reused for NR RedCap UE with 1 Rx. |
| MediaTek | ### Sub-topic 2-1: RLM Issue 2-1-1: Impact on RLM-RS resources From the proposal, it is not clear what the impact is. In our contribution, we highlight that using RLM based CSI-RS with SCS = 60 KHz will have a total BW higher than the specified BW in the WID (i.e. BW = 20 MHz). Issue 2-1-2: Impact on BW used in RLM Support proposal 1. Reducing the number of PRBs down to 24 may affect the performance and hence we need to discuss this first before agreeing on a number. Issue 2-1-3: Impact on RLM evaluation period Support proposal 1. From the LTE specs, the lower bound of the evaluation period for RLM requirements was increased at lease by two in LTE Cat-M RLM requirements compared to the general LTE RLM requirements. Given that the LTE Cat-M is for services that costs less, long battery life and cheaper devices, therefore, the lower bound in the evaluation period of the 5G NR RedCap RLM requirements shall be doubled. Issue 2-1-4: How to derive RLM evaluation period if impacted Support proposal 1. Same comment as in the above Issue 2-1-3. Issue 2-1-5: Impact on parameters for IN/OOS Support proposal 1. Issue 2-1-6: If impact on IN/OOS, list of impacted parameters Support proposal 2. The existing requirements for hypothetical PDCCH parameters of 5G NR RLM are specified with the minimum number of antennas equal to two. Besides, reducing the number of antenna to one antenna can result in reducing the received power down to half compared to using two antennas. Therefore, the hypothetical PDCCH parameters shall be extended to cover the case of using a single antenna with a 3 dB difference compared to the case of using two antennas. Issue 2-1-7: Impact on in-sync/out-of-sync thresholds Support recommended WF. ### Sub-topic 2-2: Link recovery requirements Issue 2-2-1: Impact on BW used in BFD Support recommended WF. Issue 2-2-2: Impact on SINR thresholds used in BFD Support recommended WF. Issue 2-2-3: Other impact on BFD hypothetical transmission parameters Support proposal 1. In general, the impact observed in RLM requirements should be considered in BFD too. Issue 2-2-4: Impact on evaluation period used in CBD Support recommended WF. Issue 2-2-5: Impact on BW used in CSI-RS based measurement Support recommended WF. Issue 2-2-6: Impact on L1 RSRP measurement delay requirement Support recommended WF. Issue 2-2-7: Impact on L1 RSRP measurement accuracy requirement L1-RSRP measurement accuracy levels and setup in test cases should belong to the performance part. Issue 2-2-8: Impact on CSI-RS based L3 measurement and CSI-RS based L1-SINR measurement Support proposal 1. ### Sub-topic 2-3: HD-FDD UE Issue 2-3-1: Impact on HD-FDD UE requirements Support proposal 2. Issue 2-3-2: Whether UE measurements are always prioritized over UL for HD-FDD Support introducing scheduling availability restriction due to the RRM DL measurement is the current practice. Issue 2-3-3: If measurements are prioritized, the scenarios for measurements for HD-FDD We can assume gap > DL RRM meas > UL, unless specific issues are identified. Issue 2-3-4: Conditions for performing RLM for HD-FDD UE Support proposal 1. ### Sub-topic 2-4: Measurement capability Issue 2-4-1: Whether measurement capability may have to be reduced Support proposal 1. Issue 2-4-2: If measurement capability is reduced, how to determine the measurement capability for RedCap Support proposal 2. Issue 2-4-3: Multicarrier operation Support recommended WF. ### Sub-topic 2-5: UE transmit timing Issue 2-5-1: Impact on UE transmit timing requirements Support recommended WF. ### Sub-topic 2-6: Interruptions Issue 2-6-1: Impact on interruption requirements in IDLE/INACTIVE states Our preference is proposal 1. Using single antenna, the SIB decoding rate will decrease, thus it will need longer time to achieve target detection rate. Simulation will be needed. Issue 2-6-2: Impact on interruption requirements in CONNECTED state Interruption requirements will be also needed for Redcap. Proposal 2 is not clear to us, does that mean no requirement for Redcap or it actually means it needs to introduce different requirement for Redcap? ### Sub-topic 2-7: Random Access Issue 2-7-1: Type of RAs To our understanding, RAN1 is not aiming to re-design the PRACH, however, this can be FFS in case a new design is introduced. Issue 2-7-2: RA requirements based on UE type To our understanding, RAN1 is not aiming to re-design the PRACH, however, this can be FFS in case a new design is introduced. ### Sub-topic 2-8: BWP switching Issue 2-8-1: RRM impact due to BWP switching We prefer to follow the legacy 600us/2000us requirement and further check if it is applicable for Redcap. Regarding proposal 2, it Issue 2-8-2: If new BWP retuning delay is agreed in RAN4, whether to send LS to RAN1 on new BWP retuning delay At this stage, it is ttill early to consider sending LS. ### Sub-topic 2-9: RRM impact of UE transmitting/receiving outside 20 MHz Issue 2-9-1: RRM impact of UE transmit/receive outside 20MHz FFS Issue 2-9-2: Reference cell timing in RedCap BWP FFS ### Sub-topic 2-10: CONNECTED mode measurements Issue 2-10-1: CSSF design for RedCap UE measurements Proposal 1 makes sense for 1 searcher. Issue 2-10-2: Gapless measurement capabilities for RedCap UE: ‘NeedForGap’ and ‘NCSG’ No support makes sense too but it depends on the outcome of Issue 1-10-1. Hence, FFS. Issue 2-10-3: Gapless measurement capabilities for RedCap UE: ‘inter-frequency without gap’ No support is reasonable for 1 searcher, but it depends the outcome of Issue 1-10-1. Hence, FFS. Issue 2-10-4: Measurement period for cell detection Using more samples is not the only way. It could allow more inaccuracy or assume higher SNR side condition. RAN4 need to further investigate that. Issue 2-10-5: Measurement period for SS-RSRP measurement Using more samples is not the only way. It could allow more inaccuracy or assume higher SNR side condition. RAN4 need to further investigate that. Issue 2-10-6: Measurement period for SSB index detection Using more samples is not the only way. It could allow more inaccuracy or assume higher SNR side condition. RAN4 need to further investigate that. ### Sub-topic 2-11: Others Issue 2-11-1: RRM impact due to maximum number of DL MIMO layers This can be discussed in Demod. We don’t see what could be the RRM core impact. Issue 2-11-2: RRM impact related to PDSCH processing time, PUSCH preparation time, HARQ-ACK transmission delay and CSI reporting MAC-CE based TCI state switch should be impacted by the HARQ-ACK transmission delay. Issue 2-11-3: RRM impact due to relaxed maximum modulation order This can be discussed in Demod. We don’t see what could be the RRM core impact. Issue 2-11-4: Relation to LTE cat-1bis requirements ### FFS. |
| Qualcomm | Issue 2-1-1: Impact on RLM-RS resources Issue 2-1-2: Impact on BW used in RLM Fine with both the proposals. The number of CSI-RS resources for RLM with SCS of 60kHz is an issue and needs to be addressed. Issue 2-1-3: Impact on RLM evaluation period Agree with proposal 2. Only thresholds in TCs need to be modified Issue 2-1-4: How to derive RLM evaluation period if impacted May not be needed, depending on Issue 2-1-3. Issue 2-1-5: Impact on parameters for IN/OOS Agree with proposal 1, hypothetical PDCCH parameters needs to be updated because of reduced BW and Rx antenna Issue 2-1-6: If impact on IN/OOS, list of impacted parameters To be further discussed Issue 2-1-7: Impact on in-sync/out-of-sync thresholds Agree with the WF Issue 2-2-1: Impact on BW used in BFD Agree with the proposal Issue 2-2-2: Impact on SINR thresholds used in BFD Agree with the recommended WF Issue 2-2-3: Other impact on BFD hypothetical transmission parameters Agree with proposal 1. Agreements from RLM could be used as baseline for BFD Issue 2-2-4: Impact on evaluation period used in CBD Agree with both the proposals Issue 2-2-5: Impact on BW used in CSI-RS based measurement Needs further study Issue 2-2-6: Impact on L1 RSRP measurement delay requirement No need to revisit evaluation periods. Existing core requirements may apply. Issue 2-2-7: Impact on L1 RSRP measurement accuracy requirement Agree with proposal 1 Issue 2-2-8: Impact on CSI-RS based L3 measurement and CSI-RS based L1-SINR measurement Fine with the proposal, but CSI-RS based L3 and L1-SINR may need to be considered since the SSB resources may lie outside the active BWP Issue 2-3-1: Impact on HD-FDD UE requirements New requirements on scheduling restriction are needed. Issue 2-3-2: Whether UE measurements are always prioritized over UL for HD-FDD Can support proposal 1 Issue 2-3-3: If measurements are prioritized, the scenarios for measurements for HD-FDD Exact scenarios could be further discussed. Issue 2-3-4: Conditions for performing RLM for HD-FDD Support the proposal Issue 2-4-1: Whether measurement capability may have to be reduced Fine with proposal 2 Issue 2-4-2: If measurement capability is reduced, how to determine the measurement capability for RedCap Needs further study Issue 2-5-1: Impact on UE transmit timing requirements Support proposal 1&2. It’s important to specify that UE measures SSB only within the active BWP. Current specification says “The UE shall meet the Te requirement for an initial transmission provided that at least one SSB is available at the UE during the last 160 ms” We need some clarification on the term “availability” here. Does that mean that SSB can be outside the active BWP as well? If yes, that would require a RedCap UE to perform RF re-tuning 160ms before the UL to measure the SSB outside the active BWP adding to UE complexity which is not desired for a RedCap UE. This was not much of an issue for R15 as the chances of this happening were pretty slim because of wider UE BW and availability of SSBs on other CCs as well. Issue 2-6-1: Impact on interruption requirements in IDLE/INACTIVE states Support proposal 2 Issue 2-6-2: Impact on interruption requirements in CONNECTED state For proposal 1, what impact is foreseen on the interruption requirements? For proposal 2, interruptions would still be needed. Issue 2-8-1: RRM impact due to BWP switching We think fast BWP switching delay is not required for RedCap UE. Propose to use existing requirements. Issue 2-9-1: RRM impact of UE transmit/receive outside 20MHz Hopping outside 20MHz is being discussed in RAN1. RAN4 should wait for agreements from RAN1 Issue 2-9-2: Reference cell timing in RedCap BWP Support proposal 1. |
| Nokia | ### Sub-topic 2-1: RLM Issue 2-1-1: Impact on RLM-RS resources Support proposal 1. For CSI-RS based RLM with 60 kHz SCS, the number of RLM-RS resources is impacted for RedCap UEs. Issue 2-1-2: Impact on BW used in RLM Support proposal 1. For CSI-RS based RLM with 60 kHz SCS, 48 PRBs cannot be used, so either 24 PRBs will be specified for the RLM requirement or SCS=60 KHz is excluded from CSI-RS based RedCap RLM requirements, assuming that the requirement for the RedCap device applies for lower SCS in FR1. Issue 2-1-3: Impact on RLM evaluation period Support proposal 1. Whether RLM evaluation periods for OOS and IS shall be adjusted due to 1 Rx needs to be investigated. Issue 2-1-4: How to derive RLM evaluation period if impacted Proposal 1 can be a baseline but should be confirmed by simulation. Issue 2-1-5: Impact on parameters for IN/OOS Support proposal 1. Issue 2-1-6: If impact on IN/OOS, list of impacted parameters Proposal 1 is generic and can serve as baseline. Issue 2-1-7: Impact on in-sync/out-of-sync thresholds We agree to the recommended WF. ### Sub-topic 2-2: Link recovery requirements Issue 2-2-1: Impact on BW used in BFD As for issue 2-1-2, either bandwidth for 60 kHz SCS is changed to 24 PRBs as per proposal 1 or the requirement is removed for 60 kHz SCS, assuming that the requirement for the RedCap device applies for lower SCS in FR1. Issue 2-2-2: Impact on SINR thresholds used in BFD We agree to the recommended WF. Issue 2-2-3: Other impact on BFD hypothetical transmission parameters Support proposal 1. Issue 2-2-4: Impact on evaluation period used in CBD Support proposal 1. Proposal 2 in fact looks identical. Issue 2-2-5: Impact on BW used in CSI-RS based measurement As for issues 2-1-2 and 2-2-1, either bandwidth for 60 kHz SCS is changed to 24 PRBs as per proposal 1 or the requirement is removed for 60 kHz SCS, assuming that the requirement for the RedCap device applies for lower SCS in FR1. We propose to remove last sentence of proposal 1, since not related to BW but delay requirement. Issue 2-2-6: Impact on L1 RSRP measurement delay requirement In our view, this issue is connected to issue 2-2-4 (evaluation period as core requirement). Issue 2-2-7: Impact on L1 RSRP measurement accuracy requirement If the evaluation period is kept same, then L1 RSRP measurement accuracy can be evaluated in the performance part. Issue 2-2-8: Impact on CSI-RS based L3 measurement and CSI-RS based L1-SINR measurement Agree with the recommended WF. ### Sub-topic 2-3: HD-FDD UE Issue 2-3-1: Impact on HD-FDD UE requirements We agree that the priority of DL measurements against UL transmission is a particular matter for HD-FDD type A which affects the scheduling availability and can be seen as new requirement. Hence, we support proposal 2 (remove support for proposal 1). Issue 2-3-2: Whether UE measurements are always prioritized over UL for HD-FDD Support proposals 1 and 1b. For proposal 1a more study is needed, whether to have further differentiation. Issue 2-3-3: If measurements are prioritized, the scenarios for measurements for HD-FDD Related to issue 2-3-2. Measurements within gaps (known to NW) should be prioritized over UL transmissions, FFS for measurements outside gaps. Issue 2-3-4: Conditions for performing RLM for HD-FDD UE Can be left FFS. ### Sub-topic 2-4: Measurement capability Issue 2-4-1: Whether measurement capability may have to be reduced Support both proposals. Issue 2-4-2: If measurement capability is reduced, how to determine the measurement capability for RedCap Support proposal 2: We agree that system simulations are needed to derive the possible level of reduction of measurements. Issue 2-4-3: Multicarrier operation Support proposal 1. ### Sub-topic 2-5: UE transmit timing Issue 2-5-1: Impact on UE transmit timing requirements Support proposal 1. Rel-15 transmit timing requirements should be targeted for RedCap UE. We note that this issue is dependent on the ongoing discussion in the RedCap RF session on Tx/Rx switching requirements for RedCap UE. ### Sub-topic 2-6: Interruptions Issue 2-6-1: Impact on interruption requirements in IDLE/INACTIVE states Support proposal 1. Impact due to 1 Rx should be studied. Issue 2-6-2: Impact on interruption requirements in CONNECTED state Support proposal 1. Interruption requirements for RRM procedures such as handover, RRC re-establishment and re-connection, and due to the aspect of prioritized DL measurements (sub-topic 2.3) are expected to be impacted for RedCap devices. ### Sub-topic 2-7: Random Access Issue 2-7-1: Type of RAs Support proposal 2. Should be kept open until RAN1’s conclusion on the PRACH design. Issue 2-7-2: RA requirements based on UE type Support proposal 2. Should be kept open until RAN1’s conclusion on the PRACH design. ### Sub-topic 2-8: BWP switching Issue 2-8-1: RRM impact due to BWP switching Support proposal 1. RAN4 to await outcome of further RAN1 discussion. In our view, RAN1 needs to take the decision in view of UE complexity reduction, thereafter RAN4 should evaluate impact on interruption requirements. Issue 2-8-2: If new BWP retuning delay is agreed in RAN4, whether to send LS to RAN1 on new BWP retuning delay In general, we agree with the recommended WF, however as RAN1 is still discussing this matter, RAN4 should refrain from sending LS to RAN1. ### Sub-topic 2-9: RRM impact of UE transmitting/receiving outside 20 MHz Issue 2-9-1: RRM impact of UE transmit/receive outside 20MHz Related to sub-topic 2.8. Can be left open. Issue 2-9-2: Reference cell timing in RedCap BWP No issue to be addressed by RAN4. In our understanding, RAN1 is responsible for the reference signal design and applicability. ### Sub-topic 2-10: CONNECTED mode measurements Issue 2-10-1: CSSF design for RedCap UE measurements Support proposal 1. Issue 2-10-2: Gapless measurement capabilities for RedCap UE: ‘NeedForGap’ and ‘NCSG’ Support proposal 1. Issue 2-10-3: Gapless measurement capabilities for RedCap UE: ‘inter-frequency without gap’ Support the proposal (marked as proposal2). Issue 2-10-4: Measurement period for cell detection Support proposal 1. Issue 2-10-5: Measurement period for SS-RSRP measurement Support proposal 1. Issue 2-10-6: Measurement period for SSB index detection Support proposal 1. ### Sub-topic 2-11: Others Issue 2-11-1: RRM impact due to maximum number of DL MIMO layers We agree to both proposals. Issue 2-11-2: RRM impact related to PDSCH processing time, PUSCH preparation time, HARQ-ACK transmission delay and CSI reporting We agree to proposal 1. RAN4 needs to study the RRM impact once RAN1/2 design has advanced. Issue 2-11-3: RRM impact due to relaxed maximum modulation order We agree to both proposals. Issue 2-11-4: Relation to LTE cat-1bis requirements We are open on proposal 1. The scope should be RRM requirements for UE. |

### CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
|  | Company A |
|  | Company B |
|  | Company A |
|  | Company B |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic 2-1, issue 2-1-1: | ### Sub-topic 2-1: RLM Issue 2-1-1: Impact on RLM-RS resources Tentative agreement: The number of RLM-RS is determined by RAN1 in TS38.213. RAN4 shall follow the RAN1 conclusion on the number of RLM -RS. Issue 2-1-2: Impact on BW used in RLM Tentative agreement: Impact on bandwidth of CSI-RS for RLM with 60 kHz SCS in FR1 is identified. How to address that impact is FFS. Following options are discussed: - Option 1: Set BW of CSI-RS to 24 PRBs for 60 kHz SCS in FR1 - Option 2: Exclude 60 kHz SCS in FR1 - Option 3: other options are not precluded. Issue 2-1-3: Impact on RLM evaluation period Recommendations for 2nd round: Companies to discuss whether following alternative proposal can be agreed: - Proposal 1: RAN4 to perform simulation study to decide whether to extend the RLM evaluation period due to: - 1 Rx - BW reduction - HD-FDD operation Issue 2-1-4: How to derive RLM evaluation period if impacted Recommendations for 2nd round: To be discussed based on proposal in issue 2-1-3. Issue 2-1-5: Impact on parameters for IN/OOS Tentative agreement: Impact on parameters for IN/OOS due to BW reduction and 1 Rx is expected. Recommendations for 2nd round: Continue the discussions on the details. Issue 2-1-6: If impact on IN/OOS, list of impacted parameters Recommendations for 2nd round: Continue the discussions on the details of impacted parameters. Issue 2-1-7: Impact on in-sync/out-of-sync thresholds Tentative agreement: SNR thresholds for OOS/IS impacted due to 1 rx. The detailed SINR threshold to be discussed under performance part. ### Sub-topic 2-2: Link recovery requirements Issue 2-2-1: Impact on BW used in BFD Tentative agreement: Impact on bandwidth of CSI-RS for BFD with 60 kHz SCS in FR1 is identified. How to address that impact is FFS. Following options are discussed: - Option 1: Set BW of CSI-RS to 24 PRBs for 60 kHz SCS in FR1 - Option 2: Exclude 60 kHz SCS in FR1 - Option 3: other options are not precluded. Issue 2-2-2: Impact on SINR thresholds used in BFD Tentative agreement: Impact on SINR thresholds setup in test cases can be discussed in the performance part of the WI at later stage. Issue 2-2-3: Other impact on BFD hypothetical transmission parameters Recommendations for 2nd round: Continue the discussions. Issue 2-2-4: Impact on evaluation period used in CBD Recommendations for 2nd round: Follow the conclusion from RLM. Issue 2-2-5: Impact on BW used in CSI-RS based measurement Recommendations for 2nd round: Continue the discussions. Issue 2-2-6: Impact on L1 RSRP measurement delay requirement Recommendations for 2nd round: Companies to discuss following options based on 1st round comments: - Option 1: Existing core requirements apply. - Option 2: Study the impact using the simulation assumptions to decide whether existing requirements can be reused. Issue 2-2-7: Impact on L1 RSRP measurement accuracy requirement Recommendations for 2nd round: L1 RSRP measurement accuracy is to be discussed under performance part of the WI. Issue 2-2-8: Impact on CSI-RS based L3 measurement and CSI-RS based L1-SINR measurement Tentative agreement: Follow the agreement from issue 1-9-1. ### Sub-topic 2-3: HD-FDD UE Issue 2-3-1: Impact on HD-FDD UE requirements Tentative agreement: New RRM core requirements or clarifications are needed for HD-FDD type A RedCap UEs. Issue 2-3-2: Whether UE measurements are always prioritized over UL for HD-FDD Recommendations for 2nd round: Further study needed. Issue 2-3-3: If measurements are prioritized, the scenarios for measurements for HD-FDD Recommendations for 2nd round: Further study needed. Issue 2-3-4: Conditions for performing RLM for HD-FDD UE Recommendations for 2nd round: Further study needed. ### Sub-topic 2-4: Measurement capability Issue 2-4-1: Whether measurement capability may have to be reduced Tentative agreement: Support the study and discussion of reducing the number cells/layers/frequencies/SSBs to be monitored/measured by RedCap UEs. Issue 2-4-2: If measurement capability is reduced, how to determine the measurement capability for RedCap Recommendations for 2nd round: Further discussions needed. Issue 2-4-3: Multicarrier operation ### Sub-topic 2-5: UE transmit timing Issue 2-5-1: Impact on UE transmit timing requirements Recommendations for 2nd round: Further discussions needed. ### Sub-topic 2-6: Interruptions Issue 2-6-1: Impact on interruption requirements in IDLE/INACTIVE states Recommendations for 2nd round: Companies to provide view on following options: - Option 1: Study the impact due to 1 rx using simulations. - Option 2: There is no impact on the requirements for maximum interruption in paging reception. Issue 2-6-2: Impact on interruption requirements in CONNECTED state Recommendations for 2nd round: More discussions needed. ### Sub-topic 2-7: Random Access Issue 2-7-1: Type of RAs Moderator: Since 2-step RA is a release 16 feature and combination of features is being discussed in sub-topic 1-9, issue -19-1. Follow the conclusion from that issue. For now release 15 NR requirements are used as reference. Therefore no more discussions are needed in the 2nd round on this issue. Issue 2-7-2: RA requirements based on UE type Recommendations for 2nd round: More discussions needed. ### Sub-topic 2-8: BWP switching Issue 2-8-1: RRM impact due to BWP switching Recommendations for 2nd round: Following options are being discussed: - Option 1: RAN4 to further check if the exist BWP switching requirements could be reused for RedCap UE once RAN1 reached an agreement. Take into account following aspects: - Only center frequency change/RF retuning across a bandwidth larger than its maximum UE bandwidth - Option 2: the existing BWP switching requirements could be reused for RedCap UE. Issue 2-8-2: If new BWP retuning delay is agreed in RAN4, whether to send LS to RAN1 on new BWP retuning delay Recommendations for 2nd round: It depends on conclusion of issue 2-8-1. ### Sub-topic 2-9: RRM impact of UE transmitting/receiving outside 20 MHz Issue 2-9-1: RRM impact of UE transmit/receive outside 20MHz Recommendations for 2nd round: Further study needed and pending RAN1 progress. Issue 2-9-2: Reference cell timing in RedCap BWP Recommendations for 2nd round: Following options are being discussed: - Option 1: Need more conclusion from RAN1. - FFS on mobility based measurement, gap based or interruption based measurement to be considered on the SSB outside active BWP. ### Sub-topic 2-10: CONNECTED mode measurements Issue 2-10-1: CSSF design for RedCap UE measurements Recommendations for 2nd round: More discussions needed. Issue 2-10-2: Gapless measurement capabilities for RedCap UE: ‘NeedForGap’ and ‘NCSG’ Recommendations for 2nd round: Companies to discuss if following proposal can be agreed: - Proposal 1: To simplify UE complexity, RedCap UE won’t support ‘NeedForGap’ and ‘NCSG’ measurement capabilities if single path and single searcher is assumed for RedCap. Issue 2-10-3: Gapless measurement capabilities for RedCap UE: ‘inter-frequency without gap’ Recommendations for 2nd round: More discussions needed. Issue 2-10-4: Measurement period for cell detection Recommendations for 2nd round: More study is needed. Continue to discuss the simulation assumptions for cell detection presented in topic #3. Issue 2-10-5: Measurement period for SS-RSRP measurement Recommendations for 2nd round: More study is needed. Continue to discuss the simulation assumptions for SS-RSRP measurement presented in topic #3. Issue 2-10-6: Measurement period for SSB index detection Recommendations for 2nd round: More study is needed. Continue to discuss the simulation assumptions for SSB index detection presented in topic #3. ### Sub-topic 2-11: Others Issue 2-11-1: RRM impact due to maximum number of DL MIMO layers Tentative agreement: - No RRM impact due to Maximum number of DL MIMO layers and Relaxed maximum modulation order. Issue 2-11-2: RRM impact related to PDSCH processing time, PUSCH preparation time, HARQ-ACK transmission delay and CSI reporting Recommendations for 2nd round: More discussions needed. Issue 2-11-3: RRM impact due to relaxed maximum modulation order Recommendations for 2nd round: The conclusion in issue 2-11-1 applies. No more discussions needed. Issue 2-11-4: Relation to LTE cat-1bis requirements Recommendations for 2nd round: More discussions needed. |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

# Topic #3: Simulation Assumptions

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2112644 | Vivo | Proposal 1: The baseline simulation assumptions of above areas listed at appendix are agreed in this meeting and captured in the related WF. |
| R4-2114084 | Ericsson | Proposal #3: RAN4 to study the cell detection performance for RedCap UE assuming the simulation assumptions in Annex A. Proposal #4: RAN4 to study the SSB based RRM measurement performance for RedCap UE assuming the simulation assumptions in Annex B. Proposal #5: RAN4 to study the RLM and BFD performance for RedCap UE assuming the simulation assumptions in Annex C. Proposal #6: RAN4 to study the L1 RSRP measurement performance for RedCap UE assuming the simulation assumptions in Annex C. - Proposal #7: RAN4 to study the SI reading performance for RedCap UE assuming the simulation assumptions in Annex D. |
| R4-2113847 | Huawei, HiSilicon | - Simulation assumption for measurement accuracy for RedCap UE with 1 RX |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 3-1: Simulation Assumptions for RedCap

Issue 3-1-1: Simulation assumptions to study cell detection performance

Proposals 1 (Vivo): Simulation assumptions for PSS/SSS detection provided in R4-2112644.

Proposals 2 (Ericsson): Simulation assumptions for to study the cell detection performance is provided in Annex A in R4-2114084.

Recommended WF

Companies to discuss work split for driving the simulation assumptions for cell detection performance.

Issue 3-1-2: Simulation assumptions to study SSB based RRM measurement performance

Proposals 1 (Vivo): Simulation assumptions for SS block measurement performance provided in R4-2112644.

Proposals 2 (Ericsson): Simulation assumptions to study the SSB based RRM measurement performance

is provided in Annex B in R4-2114084.

Proposals 3 (Huawei): Simulation assumption for measurement accuracy for RedCap UE with 1 RX is provided in R4-2113847.

Proposal 4 (ZTE): Accuracy requirements shall be studied after core requirements are specified.

Recommended WF

Companies to discuss work split for driving the simulation assumptions for SSB based RRM measurement performance. Regarding proposal 4, accuracy requirements belong to performance part already.

Issue 3-1-3: Simulation assumptions to study RLM and BFD performance

Proposals 1 (Vivo): Simulation assumptions to study RLM are provided in R4-2112644.

Proposals 2 (Ericsson): Simulation assumptions for to study RLM and BFD performance is provided in Annex C in R4-2114084.

Recommended WF

Companies to discuss work split for driving the simulation assumptions for RLM and BFD performance.

Issue 3-1-4: Simulation assumptions to study L1 RSRP measurement performance

Proposals 1 (Ericsson): Simulation assumptions for to study L1 RSRP measurement performance are provided in Annex C in R4-2114084.

Recommended WF

Discuss the proposal.

Issue 3-1-5: Simulation assumptions to study PBCH/SI reading detection performance

Proposals 1 (Vivo): Simulation assumptions to study PBCH detection performance are provided in R4-2112644.

Proposals 1 (Ericsson): Simulation assumptions to study SI reading performance are provided in Annex D in R4-2114084.

Recommended WF

Discuss the proposal.

## Companies views’ collection for 1st round

### Open issues

| Company | Comments |
| --- | --- |
| Huawei | Issue 3-1-2: Simulation assumptions to study SSB based RRM measurement performance Huawei would like to volunteer this work. |
| Apple | Issue 3-1-3: Simulation assumptions to study RLM and BFD performance This simulation shall be deprioritized until the performance stage, since the SNR threshold would be only discussed in the testing setup and we don’t think evaluation period would be changed for RLM/BFD. Issue 3-1-5: Simulation assumptions to study PBCH/SI reading detection performance This simulation shall be deprioritized until the performance stage, since the SI reading delay would be only set in testing (e.g., for RRC re-establishment) and also we don’t think RAN4 needs to consider R16 CGI reading feature for RedCap UE at this stage. |
| Intel | Issue 3-1-5: Simulation assumptions to study PBCH/SI reading detection performance We may need to separately consider PBCH and SI detections. PBCH detection evaluation is needed for SSB index acquisition. |
| vivo | Issue 3-1-1: Propose to align simulation assumptions for PSS/SSS detection based on R4-2112644 In our understanding, this simulation is useful to evaluate the time period used in PSS/SSS detection for RedCap. vivo would like to volunteer this work. Issue 3-1-2: Propose to align simulation assumptions based on R4-2112644 This simulation may be useful for evaluate SSB measurement period and measurement accuracy. We understand when the number of receiving antennas becomes 1, to guarantee the same accuracy as the legacy UE, the numbers of sample would need to increase. So this needs to consider the trade-off between the two aspects. In our understanding, there are two directions to define SSB measurement requirements: Alt 1. Keep the measurement accuracy the same as legacy UE and extend the measurement period by increasing the number of samples. Alt 2. Keep the measurement accuracy the same as legacy UE and relax the measurement accuracy. Therefore, in our opinion, RNA4 may need to consider how to balance the RRM measurement period and measurement accuracy. Issue 3-1-3：Simulation assumptions to study RLM and BFD performance We understand this simulation is useful to determine the evaluation period, PDCCH transmission parameters and the threshold Qout/Qin for RLM and BFD. Issue 3-1-4：Simulation assumptions to study L1 RSRP measurement performance In our understanding, this simulation is useful to evaluate the L1-RSRP measurement accuracy for RedCap. Issue 3-1-5: Simulation assumptions to study PBCH/SI reading detection performance Propose to align simulation assumptions on PBCH detection performance based on R4-2112644 We think this simulation is useful to evaluate the time period used to acquire the index of the SSB for RedCap. vivo would like to volunteer this work. |
| Ericsson | Issue 3-1-1: Simulation assumptions to study cell detection performance Issue 3-1-2: Simulation assumptions to study SSB based RRM measurement performance Issue 3-1-3: Simulation assumptions to study RLM and BFD performance As discussed earlier, RAN4 should study the RLM and BFD performance using 1 Rx to understand whether existing requirements (that belong to core part) can be reused. We agree with Apple that the impact on SNR thresholds belong to the performance part. Ericsson can volunteer to prepare the simulation assumptions for RLM and BFD. Issue 3-1-4: Simulation assumptions to study L1 RSRP measurement performance Same comment as for issue 3-1-3. Ericsson can also volunteer to drive the simulation assumption work for L1 RSRP measurement performance. Issue 3-1-5: Simulation assumptions to study PBCH/SI reading detection performance We are fine to have separate assumptions for PBCH and SI reading as suggested by Intel. |

### CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, it is suggested to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
|  | Company A |
|  | Company B |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic x-x | Tentative agreement: Based on the comments, following new work split is proposed: Title; Source. Simulation assumptions for cell detection performance; Vivo. Simulation assumptions for SSB based RRM measurement performance; Huawei, HiSilicon. Simulation assumptions for RLM and BFD performance; Ericsson. Simulation assumptions for L1 RSRP measurement performance; Ericsson. Simulation assumptions for PBCH detection; Vivo. Recommendation for the 2nd round: The companies to send out the simulation assumptions as per work split above and companies to discuss the simulation assumptions. |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section with the title ”Recommendations for Tdocs”.

# Topic #4: Specification impact

## Companies’ contributions summary

Contributions from AI 9.20.3.1 and 9.20.3.2 are discussed here.

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 4-1: RRM requirements in TS 38.133

Background:

Based on the agreed WF in R4-2108359, RRM impact on some of the requirements are FFS which are highlighted below. Companies are to provide their updated view on those.

Issue 4-1-1: Impact on TS 38.133

- Recommended WF

  - Companies are encouraged to update their view in Table 1 below for the highlighted requirements based on agreement in R4-2108359.

Table 1 Impact on TS 38.133

| Section | Type of requirements | Example | Apple | Ericsson | vivo | Company D | Company E | Company F |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3. Definitions, symbols and abbreviations | 3.1 Definitions, 3.2 Symbols, 3.3 Abbreviations | Already agreed. |  |  |  |  |  |  |
|  | 3.5 Frequency bands grouping |  |  |  |  |  |  |  |
|  | 3.6 Applicability of requirements in this specification version |  |  |  |  |  |  |  |
| 4. IDLE state mobility | 4.2.2.1 UE measurement capability | Already agreed. |  |  |  |  |  |  |
|  | 4.2.2.2 Measurement and evaluation of serving cell |  |  |  |  |  |  |  |
|  | 4.2.2.3 Measurements of intra-frequency NR cells |  |  |  |  |  |  |  |
|  | 4.2.2.4 Measurements of inter-frequency NR cells |  |  |  |  |  |  |  |
|  | 4.2.2.5 Measurements of inter-RAT E-UTRAN cells |  |  |  |  |  |  |  |
|  | 4.2.2.6 Maximum interruption in paging reception | Topic being discussed in sub-topic 2-6. |  |  |  |  |  |  |
| 5. INACTIVE state mobility | 5.1.2.1 UE measurement capability | Already agreed. |  |  |  |  |  |  |
|  | 5.1.2.2 Measurement and evaluation of serving cell |  |  |  |  |  |  |  |
|  | 5.1.2.3 Measurements of intra-frequency NR cells |  |  |  |  |  |  |  |
|  | 5.1.2.4 Measurements of inter-frequency NR cells |  |  |  |  |  |  |  |
|  | #### 5.1.2.5 Measurements of inter-RAT E-UTRAN cells |  |  |  |  |  |  |  |
|  | #### 5.1.2.6 Maximum interruption in paging reception | #### Topic being discussed in sub-topic 2-6. |  |  |  |  |  |  |
| 6. CONNECTED state mobility | #### 6.1 Handover | #### Yes/No/FFS | #### YES | #### Yes | #### Yes |  |  |  |
|  | #### 6.2.1 RRC re-establishment | #### Yes/No/FFS | #### YES | #### Yes | #### Yes |  |  |  |
|  | #### 6.2.2 Random Access | #### Topic being discussed in sub-topic 2-7. |  |  |  |  |  |  |
|  | #### 6.2.3 RRC Connection Release with Redirection | #### Yes/No/FFS | #### YES | #### Yes | #### Yes |  |  |  |
| 7. Timing | #### 7.1 UE transmit timing | #### Topic under discussion in sub-topic 2-5. |  |  |  |  |  |  |
|  | #### 7.2 UE timer accuracy | #### Already agreed. |  |  |  |  |  |  |
|  | #### 7.3 Timing Advance | #### Yes/No/FFS | #### NO | #### FFS | #### FFS |  |  |  |
|  | #### 7.4 Cell phase synchronization accuracy | #### Already agreed. |  |  |  |  |  |  |
|  | #### 7.5 Maximum Transmission Timing Difference |  |  |  |  |  |  |  |
|  | #### 7.6 Maximum Receive Timing Difference |  |  |  |  |  |  |  |
|  | #### 7.7 deriveSSB-IndexFromCell tolerance |  | #### NO | #### FFS |  |  |  |  |
| 8. Signalling characteristics | #### 8.1 Radio Link Monitoring | #### Topic under discussion in sub-topic 2-1. |  |  |  |  |  |  |
|  | #### 8.2 Interruption | #### Topic under discussion in sub-topic 2-6. |  |  |  |  |  |  |
|  | #### 8.3 SCell Activation and Deactivation Delay | #### Already agreed. |  |  |  |  |  |  |
|  | #### 8.4 UE UL carrier RRC reconfiguration delay | #### Yes/No/FFS | #### Not applicable for Redcap in release 17 | #### N/A | #### N/A |  |  |  |
|  | #### 8.5 Link Recovery Procedures | #### Topic under discussion in sub-topic 2-2. |  |  |  |  |  |  |
|  | #### 8.6 Active BWP switch delay | #### Topic under discussion in sub-topic 2-8. |  |  |  |  |  |  |
|  | ## 8.8 NE-DC: E-UTRAN PSCell Addition and Release Delay | ## Already agreed. |  |  |  |  |  |  |
|  | ## 8.9 NR-DC: PSCell Addition and Release Delay |  |  |  |  |  |  |  |
|  | ## 8.10 Active TCI state switching delay | ## Yes/No/FFS | ## NO | ## FFS | ## FFS |  |  |  |
|  | ## 8.11 PSCell Change | ## Already agreed. |  |  |  |  |  |  |
|  | ## 8.12 Uplink spatial relation switch delay | ## Yes/No/FFS | ## FFS until sufficient progress of WI | ## FFS | ## FFS |  |  |  |
|  | ## 8.13 UE-specific CBW change | ## Yes/No/FFSs | ## FFS until sufficient progress of WI | ## FFS | ## FFS Should we focus on Rel-15 right now? |  |  |  |
| 9. Measurement Procedure | ## 9.1 General measurement requirement | ## Already agreed. |  |  |  |  |  |  |
|  | ## 9.2 NR intra-frequency measurements |  |  |  |  |  |  |  |
|  | ## 9.3 NR inter-frequency measurements |  |  |  |  |  |  |  |
|  | ## 9.4 Inter-RAT measurements |  |  |  |  |  |  |  |
|  | ## 9.5 L1-RSRP measurements for Reporting | ## Topic under discussion in sub-topic 2-2. |  |  |  |  |  |  |
|  | ## 9.6 NE-DC: Measurements | ## Already agreed. |  |  |  |  |  |  |
|  | ## 9.7 Cross Link Interference measurements | ## Yes/No/FFS | ## FFS until sufficient progress of WI | ## FFS | ## FFS |  |  |  |
|  | ## 9.8 L1-SINR measurements for Reporting | ## Topic under discussion in sub-topic 2-2. |  |  |  |  |  |  |
|  | ## 9.9 NR measurements for positioning | ## Yes/No/FFS | ## FFS until sufficient progress of WI | ## N/A | ## FFS |  |  |  |
|  | ## 9.10 CSI-RS based L3 measurements | ## Topic under discussion in sub-topic 2-2. |  |  |  |  |  |  |
|  | ## 9.11 NR measurements with autonomous gaps | ## Topic under discussion in issue 3-1-5. |  |  |  |  |  |  |
| 10. Measurement Performance requirements | ## 10.1 NR measurements | ## Already agreed. |  |  |  |  |  |  |
| 12. V2X Requirements | ## V2X Requirements | ## Topic under discussion in sub-issue 1-8. |  |  |  |  |  |  |

Issue 4-1-2: Impact on 38.133 in TS 38.133

- Recommended WF

  - If needed, companies can provide comments using this issue to for the highlighted requirements in 38.133 in Table 1.

### Sub-topic 4-2: RRM requirements in TS 36.133

Background:

Based on the agreed WF in R4-2108359, RRM impact on some of the requirements are FFS which are highlighted below. Companies are to provide their updated view on those.

Issue 4-2-1: Impact on TS 36.133

- Recommended WF

If needed, companies can provide comments using this issue to for the highlighted requirements in 36.133 in Table 1.

Table 2 Impact on TS 36.133

| Section | Type of requirements | Example | Apple | Company B | Company C |
| --- | --- | --- | --- | --- | --- |
| 3. Definitions, symbols and abbreviations | 3.1 Definitions, 3.2 Symbols, 3.3 Abbreviations | Already agreed. |  |  |  |
| 4. E-UTRAN RRC_IDLE state mobility | 4.1 Cell Selection |  | No |  |  |
|  | 4.2 Cell Re-selection | Already agreed. |  |  |  |
|  | 4.3 Minimization of Drive Tests (MDT) |  |  |  |  |
|  | 4.4 MBSFN Measurements |  |  |  |  |
|  | 4.5 Proximity-based Services |  |  |  |  |
|  | 4.6 Cell Selection and Re-selection Requirements for UE category NB1 |  | FFS until sufficient progress of WI |  |  |
|  | 4.7 Cell Selection and Re-selection Requirements for UE category M1 |  | FFS until sufficient progress of WI |  |  |
|  | 4.8 Idle State Positioning Measurement Requirements for UE category NB1 | Already agreed. |  |  |  |
| 5. E-UTRAN RRC_CONNECTED state mobility | 5.1 E-UTRAN Handover | Already agreed. |  |  |  |
|  | 5.3 Handover to other RATs |  |  |  |  |
|  | 5.4 Handover to Non-3GPP RATs |  |  |  |  |
|  | ## 5.5 E-UTRAN Handover for Cat-M1 UEs |  | FFS until sufficient progress of WI |  |  |
|  | ## 5.7 E-UTRAN DAPS Handover | Already agreed. |  |  |  |
| 6. RRC Connection Mobility Control | ## 6.1 RRC Re-establishment | Already agreed. |  |  |  |
|  | ## 6.2 Random Access |  |  |  |  |
|  | ## 6.3 RRC Connection Release with Redirection |  | YES |  |  |
|  | ## 6.4 CSG Proximity Indication for E-UTRAN and UTRAN | Already agreed. |  |  |  |
|  | ## 6.5 RRC Re-establishment for NB-IoT UEs |  |  |  |  |
|  | ## 6.6 Random Access for UE category NB1 |  |  |  |  |
|  | ## 6.7 RRC Re-establishment for Cat-M1 UEs |  |  |  |  |
|  | ## 6.8 RRC Connection Release with Redirection for Cat-M1 UEs |  | FFS until sufficient progress of WI |  |  |
|  | ## 6.9 RRC Connection Redirection to Non-anchor Carrier in NB-IoT |  | FFS until sufficient progress of WI |  |  |
| 7. Timing and signalling characteristics |  | Already agreed. |  |  |  |
| 8. UE Measurements Procedures in RRC_CONNECTED State |  |  |  |  |  |
| 9. Measurements performance requirements for UE |  |  |  |  |  |
| 10. Measurements Performance Requirements for E-UTRAN |  | Already agreed. |  |  |  |
| 11. ProSe Requirements in Any Cell Selection state |  |  |  |  |  |
| 12. V2V Sidelink Communication Requirements for V2V Operation on Dedicated V2V Carrier |  |  |  |  |  |
| 13. V2X Requirements |  |  |  |  |  |

## Companies views’ collection for 1st round

### Open issues

| Company | Comments |
| --- | --- |
| Ericsson | Issue 4-2-1: Impact on TS 36.133 It depends on the conclusion on whether to support inter-RAT requirements on LTE as being discussed in topic #1. |

### CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
|  | Company A |
|  | Company B |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section with the title ”Recommendations for Tdocs”.

# Recommendations for Tdocs

## 1st round

New tdocs

| Title | Source | Comments |
| --- | --- | --- |
| WF on … | YYY |  |
| LS on … | ZZZ | To: RAN_X; Cc: RAN_Y |
| WF on RedCap RRM requirements | Ericsson | To capture the agreements from e-mail thread [100-e][234] NR_redcap_RRM_1 |
| Simulation assumptions for cell detection performance | Vivo |  |
| Simulation assumptions for SSB based RRM measurement performance | Huawei, HiSilicon |  |
| Simulation assumptions for RLM and BFD performance | Ericsson |  |
| Simulation assumptions for L1 RSRP measurement performance | Ericsson |  |
| Simulation assumptions for PBCH detection | Vivo |  |

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-210xxxx | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

## 2nd round

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-210xxxx | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-210xxxx | WF on … | YYY | Agreeable, Revised, Noted |  |
| R4-210xxxx | LS on … | ZZZ | Agreeable, Revised, Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents

# Annex

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| Huawei | Jing Han | hw.hanjing@huawei.com |
| CMCC | Xiaoran ZHANG | zhangxiaoran@chinamobile.com |
| Ericsson | Kazuyoshi Uesaka | kazuyoshi.uesaka@ericsson.com |
| MediaTek | Waseem Ozan | Waseem.ozan@mediatek.com |
| Nokia | Juergen Hofmann | juergen.hofmann@nokia.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)