3GPP TSG-RAN WG4 Meeting # 100-e 	    										            R4-2115275

Electronic Meeting, August 16-27, 2021.

Agenda item:			6.1.1.5, 6.1.1.6.3.12-20

Source:	Ericsson

Title:	WF on NR-U RRM Requirements

Document for:	Approval

# Introduction

This way forward contains all agreements in RAN4#100-e in email thread: [100-e][206] NR_unlic_RRM_1.

Agreements from the 1st GTW session

Agreements from the 1st round

Agreements from the 2nd round

# Agreements on core requirements

## Availability of SSB occasions for RLM In-sync

How frequent the UE shall determine the availability of SSB occasions for RLM In-sync

- For RLM, the UE is not required to determine the availability of SSB occasions more frequent than:

- Once per max(10ms, TSSB* P) if DRX is not used

- Once per max(10ms, ceil(1.5 * P) * TDRX, ceil(1.5 * P) * TSSB) if TDRX ≤ 320ms

- Once per TDRX * P if TDRX > 320ms.

## Availability of SSB occasions for RLM Out-of-sync

Whether to specify the availability of SSB occasions for RLM Out-of-sync.

- No need to specify how frequent the UE would determine the availability of SSB occasions.

## Availability of SSB occasions for BFR

Whether to specify the availability of SSB occasions for BFR

- No need to specify how frequent the UE would determine the availability of SSB occasions.

## Availability of SSB occasions for L1-RSRP

How frequent the UE shall determine the availability of SSB occasions for L1-RSRP

- For L1-RSRP, the UE is not required to determine the availability of SSB occasions more frequent than:

- Once per max(TReport, TSSB * P) if DRX is not used

- Once per max(TReport, ceil(1.5*P)*max(TDRX,TSSB)) if TDRX ≤ 320ms

- Once per TDRX *P if TDRX > 320ms

# Agreements on performance requirements

## CCA parameters for RLM in-sync test cases in non-DRX

- Configure CCA model with LCCA_DL=Lin,max and WCCA_DL=Tidentify-NR_CCA for the test cases of RLM in-sync test cases in non-DRX mode.

## CCA parameters for RLM out-of-sync test cases

- Do not configure the CCA parameters LCCA_DL and WCCA_DL for out-of-sync RLM test cases with CCA.

## CCA parameters for BFD and link recovery test cases

- For the beam failure detection and link recovery test cases , configure LCCA_DL=7 for TCs without DRX, LCCA_DL=3 for TCs with DRX and WCCA_DL= TEvaluate_CBD_SSB_CCA .

## Whether to remove test 2 in current BFD and CBD test cases

- The PDCCH configuration shall be same as the hypothetical PDCCH in the spec and test 2 is not needed.

## Test to verify delay in HARQ feedback transmission

- RAN4 agrees to not define “MAC CE based TCI state switch delay” test case with UL CCA failure.