3GPP TSG-RAN WG4 Meeting # 100-e 	    R4-2115318

Electronic Meeting 16th August – 27th August 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | DraftCR | rev | 1 | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to gNB Rx-Tx measurement in 38.133 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos-Perf |  |  |  |  |  | Date: |  |  | 2021-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | gNB Rx-Tx measurement accuracy requirements updated based on current simulation results |  |  |  |  |  |  |  |  |
| Summary of change: |  | gNB Rx-Tx measurement accuracy requirements updated based on current simulation results |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect gNB Rx-Tx measurement accuracy requirements |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 13.2.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

----------------------START OF CHANGES----------------------------

### Measurement Accuracy Requirements

#### 13.2.2.1	Introduction

This clause defines accuracy requirements for gNB Rx-Tx time difference measurement in FR1 and FR2. The requirements are applicable for gNB supporting gNB Rx-Tx time difference measurement. The gNB, which declares the support for gNB Rx-Tx time difference measurement also declares that it meets gNB Rx-Tx time difference accuracy requirements at least for one side condition Ês/Iot ≥ +3 dB or Ês/Iot ≥ -13 dB.

#### 13.2.2.2	Requirements

The accuracy requirements for gNB Rx-Tx time difference measurement shall be within ±(X+Y) Tc under the following conditions:

-	AWGN propagation conditions.

-	The measured signals are in the directions covered by RoAoA of OTA reference sensitivity requirements for gNB type 1-O and 2-O BS

where

-	X is defined in Table 13.2.2.2-1 for gNB types 1-C, 1-H and 1-O and in Table 13.2.2.2-2 for gNB type 2-O.

-	Y is declared by manufacturer and can be different for different gNB types 1-C, 1-H, 1-O and 2-O.

Note: The measurement accuracy requirements in Table 13.2.2.2-1 and Table 13.2.2.2-2 are defined under an assumption that gNB is not mandated to perform receive beam sweeping.

Table 13.2.2.2-1: gNB Rx-Tx time difference absolute accuracy in FR1 for gNB type 1-C, 1-H and 1-O

| Accuracy | SRS Ês/Iot | SCS | SRS bandwidth range |
| --- | --- | --- | --- |
| Unit: Tc | Unit: dB | Unit: kHz | Unit: RB |
| 123 | ≥ -13 | 15 | 44 ≤ BW ≤ 84 |
| 48 |  |  | 88 ≤ BW ≤ 168 |
| 17 |  |  | 176 ≤ BW |
| 122 | ≥ +3 |  | 24 ≤ BW ≤ 40 |
| 62 |  |  | 44 ≤ BW ≤ 84 |
| 32 |  |  | 88 ≤ BW ≤ 168 |
| 16 |  |  | 176 ≤ BW |
| 42 | ≥ -13 | 30 | 48 ≤ BW ≤ 84 |
| 24 |  |  | 88 ≤ BW ≤ 168 |
| 8 |  |  | 176 ≤ BW |
| 32 | ≥ +3 |  | 48 ≤ BW ≤ 84 |
| 17 |  |  | 88 ≤ BW ≤ 168 |
| 9 |  |  | 176 ≤ BW |
| 21 | ≥ -13 | 60 | 48 ≤ BW ≤ 84 |
| 12 |  |  | 88 ≤ BW |
| 16 | ≥ +3 |  | 48 ≤ BW ≤ 84 |
| 9 |  |  | 88 ≤ BW |

Table 13.2.2.2-2: gNB Rx-Tx time difference absolute accuracy in FR2 for gNB type 2-O

| Accuracy | SRS Ês/Iot | SCS | SRS bandwidth range |
| --- | --- | --- | --- |
| Unit: Tc | Unit: dB | Unit: kHz | Unit: RB |
| 9 | ≥ -13 | 60 | 132 ≤ BW ≤ 168 |
| 8 |  |  | 176 ≤ BW |
| 9 | ≥ +3 |  | 132 ≤ BW ≤ 168 |
| 8 |  |  | 176 ≤ BW |
| 22 | ≥ -13 | 120 | 32 ≤ BW ≤ 40 |
| 15 |  |  | 44 ≤ BW ≤ 84 |
| 8 |  |  | 88 ≤ BW |
| 16 | ≥ +3 |  | 32 ≤ BW ≤ 40 |
| 9 |  |  | 44 ≤ BW ≤ 84 |
| 8 |  |  | 88 ≤ BW |

----------------------END OF CHANGES----------------------------