3GPP TSG-RAN WG4 Meeting # 100-e 						R4-2115335

Electronic Meeting, 16th – 27th Aug, 2021

Agenda item:			9.9.4

Source:	Samsung

Title:	WF for FR2 HST RRM part 2

Document for:	Approval

Introduction

This contribution is to capture the agreements for the email discussion for the topic of Rel-17 NR HST FR2 enhancements RRM core requirements (AI 9.9.4) in RAN4 #100, with the email thread identifier “[100][219] NR_HST_FR2_RRM_2”.

This e-mail thread will capture the e-mail discussions for the following sub-agenda items for FR2 HST RRM

- AI 9.9.4.4	Timing requirements

- AI 9.9.4.5	Signalling characteristics requirements

- AI 9.8.4.6	Measurement procedure requirements

The detailed discussions on these agreements can be found in the e-mail discussion summary.

Way-forward

Downlink timing

- In FR2 HST scenario, PSS/SSS detection is robust enough to handle the ISI and time difference

- RAN4 will not introduce the SSB index allocation limitation in the specifications for FR2 HST scenarios

Uplink timing

- RAN4 will further study the below options to address uplink timing issues

  - Option 1: One shot UE autonomous large uplink timing adjustment

  - Option 2: Other implementation/deployment based solution

Scheduling restriction

- Study the impact of timing different between beams on measurement and demod to decide whether scheduling restrictions shall apply for one symbol before and one symbol after resources (SSB, CSI-RS etc) used for L1-RSRP measurements.

RLM/BFD evaluation period

- The sharing factor P and PCBD, current specification can be reused for FR2 HST

- RAN4 will further study the time instances for measurement for RLM/CBD evaluation period for short DRX case

TCI switching delay requirements

- RAN4 will further study 1280ms duration for known condition

- RAN4 will further study the TCI switching delay requirements

  - Option 1: Reuse the existing TCI switching delay requirements for known condition

  - Option 2: NW triggered TCI switching to avoid sharp SNR drop

- Further enhancement on TCI switching delay based on Rel-17 TCI design can be discussed in FeMIMO WI.

PSS/SSS detection & intra-frequency measurement

- FFS whether to reuse the Rel-16 FR1 HST scaling factor M2 for FR2 HST or keep the 1.5 factor

- Mpss/sss_sync_w/o_gaps and Mmeas_period_w/o_gaps is proportional to the number of samples (S) and of receiver sweeping beams (N)

L1-RSRP measurement

- FFS whether to reuse the Rel-16 FR1 HST scaling factor K for FR2 HST L1-RSRP measurement requirement, with the same SMTC periodicity bound of 40ms or keep the 1.5 factor

- Further enhancement can be considered if network assistant information is introduced.

Non-overlapping SSB location

- RRM requirements specified in RAN4 will NOT consider SSB overlapping case.