3GPP TSG-RAN WG4 Meeting # 100-e	                                             R4-2115350

# Electronic Meeting, Aug. 16-27, 2021

# WF on NR SL enhancements RRM requirements

# LG Electronics

# Related to new operating scenario (intra-band con-current V2X operation)

- Issue 1-1-1 : NTA_offset when NR Cell is configured as synchronization reference source

  - Postpone until RF decision or RAN1 feedback on whether SL transmit timing is aligned with Uu uplink timing

- Issue 1-1-2 : NTA,SL when NR Cell is configured as synchronization reference

  - Postpone until RF decision or RAN1 feedback on whether SL transmit timing is aligned with Uu uplink timing

- Issue 1-1-3 : Te when NR Cell is configured as synchronization reference source

  - Reuse Rel-16 requirement

- Issue 1-1-4 : Scheduling availability requirements when switching TDM based intra-band con-current SL operation

  - Specify the scheduling availability requirements considering RF inputs on the switching time and the reference timing of SL

- Issue 1-1-6 : RRM requirements for FDM based intra-band con-current SL operation

  - Study the interruptions on SL due to Uu BWP switch

  - FFS whether to specify interruption requirements on SL due to Uu BWP switch.

# Related to SL-DRX

- Issue 1-2-1 : Initiation/cease of SLSS transmissions due to SL-DRX when GNSS/NR Cell /EUTRAN Cell is synchronization reference source

  - Option 1 : Consider Rel-16 evaluation period requirement as starting point if it does not depend on the progress of SL DRX

  - Option 2 : Consider the progress of SL DRX, e.g., impact of alignment between SL DRX and Uu DRX

- Issue 1-2-2 : Initiation/cease of SLSS transmissions due to SL-DRX when SyncRef UE is synchronization reference source

  - Option 1 : Consider Tevaluate,SLSS =  max(4 S-SSB periods, 4 SL-DRX cycles) as starting point

    - For multiple SL-DRX cycles, FFS which one would be applied

    - Other options based on the assumption of UE behavior different than one SLSS measurement per SL-DRX cycle are not precluded

  - Option 2 : Consider the progress of SL DRX, e.g., impact of alignment between SL DRX and Uu DRX

- Issue 1-2-3 : SyncRef UE detection time of Selection/reselection of V2X Synchronization Reference Source

  - Define the requirements separately for synchronous case and asynchronous case

  - FFS

    - How to select SL-DRX cycle length for multiple active SL-DRX configurations

    - Whether to consider measuring SLSS from multiple SLSS periods in a SL-DRX cycle, or measuring only SLSS from one SLSS period per SL-DRX cycle, if SL-DRX cycle length > SLSS period

- Issue 1-2-4 : PSBCH-RSRP measurement period of Selection/reselection of V2X Synchronization Reference Source

  - Option 1 : max(320ms, 2 SL-DRX cycles) as starting point

    - For multiple SL-DRX cycles, FFS which one would be applied

    - Other options based on the assumption of UE behavior different than one SLSS measurement per SL-DRX cycle are not precluded

  - Option 2 : FFS whether or not to depend on the progress of SL DRX

- Issue 1-2-5 : UE dropping requirements of Selection/reselection of V2X Synchronization Reference Source

  - Selection/reselection of V2X Synchronization Reference Source with SL-DRX

    - Asynchronous case: UE is allowed to drop V2X reception for the purpose selection/reselection of V2X Synchronization Reference Source

    - FFS if TX dropping requirement shall be defined and how to take into account SL-DRX

- Issue 1-2-6 : Interruption to WAN due to SL-DRX

  - Option 1 : Consider Rel-16 EN-DC interruption requirement as starting point

    - interruptions can occur due to tuning ON/OFF SL RF

      - at transitions between active and non-active during SL-DRX

      - at transitions from non-SL-DRX to SL-DRX

    - Consider to avoid interruptions during certain occasions

  - Option 2 : Consider interruption requirements during LTE ProSe as reference

    - Consider to avoid interruptions during certain occasions such as while onDurationTimer is running, during paging reception.

- Issue 1-2-7 : Interruption to SL due to Uu DRX

  - FFS for specific scenarios for interruption to SL due to Uu DRX

  - Consider Rel-16 EN-DC interruption requirement as starting point

# Annex : Discussion in 2nd round

Sub-topic 1-1 : Related to new operating scenario (intra-band con-current V2X operation)

Issue 1-1-1: NTA_offset when NR Cell is configured as synchronization reference source

| Company | Comments |
| --- | --- |
| Ericsson | Proposed WF is fine. |
| LG Electronics | Fine with the proposed WF |

Issue 1-1-2: NTA,SL when NR Cell is configured as synchronization reference

| Company | Comments |
| --- | --- |
| Ericsson | Proposed WF is fine. |
| LG Electronics | Fine with the proposed WF |

Issue 1-1-6: RRM requirements for FDM based intra-band con-current SL operation

| Company | Comments |
| --- | --- |
| Ericsson | We are fine to keep both options and study the impact until next meeting. |
| Huawei | It is not a new issue for due to new operating scenario. In R16, the issue for interruption on SL due to Uu BWP switch was also discussed. However, RAN4 agreed not to specify the requirement due to that the interruption on SL cannot be verified. |
| LG Electronics | Fine with update by Huawei. Further discuss in next meeting. We think Ericsson’s comment is for issue 1-2-6 not issue 1-1-6. |

Sub-topic 1-2 : Related to SL-DRX

Issue 1-2-1: Initiation/cease of SLSS transmissions due to SL-DRX when GNSS/NR Cell /EUTRAN Cell is synchronization reference source

| Company | Comments |
| --- | --- |
| Ericsson | We are fine to keep both options for FFS. |

Issue 1-2-2: Initiation/cease of SLSS transmissions due to SL-DRX when SyncRef UE is synchronization reference source

| Company | Comments |
| --- | --- |

Issue 1-2-3: SyncRef UE detection tim of Selection/reselection of V2X Synchronization Reference Source

| Company | Comments |
| --- | --- |

Issue 1-2-4: PSBCH-RSRP measurement period of Selection/reselection of V2X Synchronization Reference Source

| Company | Comments |
| --- | --- |

Issue 1-2-5: UE dropping requirements of Selection/reselection of V2X Synchronization Reference Source

| Company | Comments |
| --- | --- |
| Ericsson | Keep the GTW agreement. |

Issue 1-2-6: Interruption to WAN due to SL-DRX

| Company | Comments |
| --- | --- |
| Ericsson | We are fine to keep both options and study the impact until next meeting based on these options. |

Issue 1-2-7: Interruption to SL due to DRX

| Company | Comments |
| --- | --- |
| Ericsson | Keep the GTW agreement. |
| Huawei | Same comments as for issue 1-6. The interruption on SL may not be verified. Whether to specify the interruption requirements on SL is FFS. |
| LG Electronics | Keep the GTW agreement. Because, there is no agreement for 3rd bullet. We think 3rd bullet(Huawei’s suggestion) can be discussed in issue 1-1-6. And, conclusion of Issue 1-1-6 can be considered in issue 1-2-7 in next meeting. |