3GPP TSG-RAN WG4 Meeting #100-e		R4-2115352

E-meeting, 16 Aug – 27 Aug, 2021

Agenda item:	9.16.7

Source:	Intel Corporation

Title:	WF on NR extension to 71 GHz – RRM - Part 2

Document for:	Approval

# Introduction

In this contribution we capture the agreements achieved during the email discussion on the topic Rel-17 NR ext. to 71GHz RRM core requirements (AI 9.16.7) in RAN4#100-e under email thread [100-e][231] NR_ext_to_71GHz_RRM_2. This email thread covers following sub-agenda items:

- AI 9.16.7.4	Active BWP switching delay requirements

- AI 9.16.7.5	Measurement gap interruption requirements

The details on the discussion can be found at the [231] thread discussion summary [1].

# Way forward

BWP switching delay for FR2-2

- DCI and timer based BWP switch delay on a single CC

  - As baseline, follow 600us and 2000us switching delay for Type 1 and Type 2 respectively

  - FFS: if BWP delay reduction is possible for the operation on 480 and 960 kHz SCS

  - RAN4 to change Table 8.6.2-1 as follows

Table 8.6.2-1: BWP switch delay

|  | NR Slot length | BWP switch delay TBWPswitchDelay (slots) |  |
| --- | --- | --- | --- |
|  | (ms) | Type 1Note 1 | Type 2Note 1 |
| 0 | 1 | 1 | 3 |
| 1 | 0.5 | 2 | 5 |
| 2 | 0.25 | 3 | 9 |
| 3 | 0.125 | 6 | 18 |
| 4 | 0.03125 | [20] | [65] |
| 5 | 0.015625 | [39] | [129] |
| Note 1: Depends on UE capability. Note 2: If the BWP switch involves changing of SCS, the BWP switch delay is determined by the smaller SCS between the SCS before BWP switch and the SCS after BWP switch. |  |  |  |

Note: TBWPswitchDelay is the BWP switching delay on single CC

- RRC-based BWP switch

  - Reuse the existing requirements for RRC-based BWP switch

- BWP Switching on Multiple CCs

  - Postpone the discussion for the later stage after requirements for one BWP will be agreed

Measurement gaps

- Measurement gaps applicability for FR2-2

  - The existing gaps are also feasible for higher SCS

  - New gap patterns with smaller MGL, if justified, can be considered

- Per-FR measurement gaps for FR2-1 and FR2-2

  - FFS if per-FR measurement gaps for FR2-1 and FR2-2 should be defined independently

- Effective MGRP used to determine requirements

  - For per-FR measurement gap capable UE in NR standalone operation (with single carrier, NR CA and NR-DC configuration), for per-FR gap-based measurement, when there is no serving cell in a particular FR, where measurement objects are configured, regardless if explicit per-FR measurement gap is configured in this FR, the effective MGRP in this FR is used to determine requirements:

    - 20 ms for FR2 NR measurements also is applicable for 52.6 – 71 GHz

Measurement gap interruption requirements

- MG timing advance for FR2-2

  - RAN4 should reuse 0.25ms as MG timing advance for 52.6-71GHz

- High SCS in the case with per-UE gap or per-FR gap for FR1

  - RAN4 should modify NOTE 2 in Table 9.1.2-4 and Table 9.1.2-4a to capture that 480 kHz and 960 kHz are only applicable to the case with per-UE measurement gap

- MRTD impact on gap interruptions

  - FFS: The impact of MRTD when defining the requirements for gap interruptions.

- Measurement gap interruption requirements

  - Table 9.1.2-4, Table 9.1.2-4a and Table 9.1.2-4b to include higher SCS’s for 52.6 – 71 GHz as shown below

Table 9.1.2-4: Total number of interrupted slots on all serving cells during MGL for Synchronous EN-DC, NR standalone operation (with single carrier, NR CA and synchronous NR-DC configuration) and NE-DC, and on all serving cells in MCG for NR standalone operation (with asynchronous NR-DC configuration) with per-UE measurement gap or per-FR measurement gap for FR1

| NR | Total number of interrupted slots on serving cells |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SCS | When MG timing advance of 0ms is applied |  |  |  |  | When MG timing advance of 0.5ms is applied |  |  |  |  |
| (kHz) | MGL=20ms | MGL=10ms | MGL=6ms | MGL=4ms | MGL=3ms | MGL=20ms | MGL=10ms | MGL=6ms | MGL=4ms | MGL=3ms |
| 15 | 20 | 10 | 6 | 4 | 3 | 21Note3 | 11Note3 | 7Note3 | 5Note3 | 4Note3 |
| 30 | 40 | 20 | 12 | 8 | 6 | 40 | 20 | 12 | 8 | 6 |
| 60 | 80 | 40 | 24 | 16 | 12 | 80 | 40 | 24 | 16 | 12 |
| 120 | 160 | 80 | 48 | 32 | 24 | 160 | 80 | 48 | 32 | 24 |
| 480 | 640 | 320 | 192 | 128 | 96 | 640 | 320 | 192 | 128 | 96 |
| 960 | 1280 | 640 | 384 | 256 | 192 | 1280 | 640 | 384 | 256 | 192 |
| NOTE 1: For Gap Pattern ID 0, 1, 2 and 3, total number of interrupted subframes on MCG is MGL subframes when MG timing advance of 0ms is applied, and (MGL+1) subframes when MG timing advance of 0.5ms is applied. NOTE 2: NR SCS of 120 kHz ,480 kHz and 960 kHz is only applicable to the case with per-UE measurement gap. NOTE 3: Non-overlapped half-slots occur before and after the measurement gap. Whether a Rel-15 UE can receive and/or transmit in those half-slots is up to UE implementation. |  |  |  |  |  |  |  |  |  |  |

Table 9.1.2-4a: Total number of interrupted slots on serving cells during MGL for Asynchronous EN-DC, and on all serving cells in SCG for NR standalone operation (with asynchronous NR-DC configuration) with per-UE measurement gap or per-FR measurement gap for FR1

| NR | Total number of interrupted slots on serving cells |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SCS | When MG timing advance of 0ms is applied |  |  |  |  | When MG timing advance of 0.5ms is applied |  |  |  |  |
| (kHz) | MGL=20ms | MGL=10ms | MGL=6ms | MGL=4ms | MGL=3ms | MGL=20ms | MGL=10ms | MGL=6ms | MGL=4ms | MGL=3ms |
| 15 | 21 | 11 | 7 | 5 | 4 | 21 | 11 | 7 | 5 | 4 |
| 30 | 41 | 21 | 13 | 9 | 7 | 41 | 21 | 13 | 9 | 7 |
| 60 | 81 | 41 | 25 | 17 | 13 | 81 | 41 | 25 | 17 | 13 |
| 120 | 161 | 81 | 49 | 33 | 25 | 161 | 81 | 49 | 33 | 25 |
| 480 | 641 | 321 | 193 | 129 | 97 | 641 | 321 | 193 | 129 | 97 |
| 960 | 1281 | 641 | 385 | 257 | 193 | 1281 | 641 | 385 | 257 | 193 |
| NOTE 1: For Gap Pattern ID 0, 1, 2 and 3, total number of interrupted subframes on MCG is MGL subframes when MG timing advance of 0ms is applied, and (MGL+1) subframes when MG timing advance of 0.5ms is applied. NOTE 2: NR SCS of 120 kHz ,480 kHz and 960 kHz is only applicable to the case with per-UE measurement gap. |  |  |  |  |  |  |  |  |  |  |

Table 9.1.2-4b: Total number of interrupted slots on FR2 serving cells during MGL for EN-DC, NR standalone operation (with single carrier, NR CA and NR-DC configuration) and NE-DC with per-UE measurement gap or per-FR measurement gap for FR2

| NR | Total number of interrupted slots on FR2 serving cells |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SCS | When MG timing advance of 0ms is applied |  |  |  |  | When MG timing advance of 0.25ms is applied |  |  |  |  |
| (kHz) | MGL= 20ms | MGL= 10ms | MGL= 5.5ms | MGL= 3.5ms | MGL= 1.5ms | MGL= 20ms | MGL= 10ms | MGL= 5.5ms | MGL= 3.5ms | MGL= 1.5ms |
| 60 | 80 | 40 | 22 | 14 | 6 | 80 | 40 | 22 | 14 | 6 |
| 120 | 160 | 80 | 44 | 28 | 12 | 160 | 80 | 44 | 28 | 12 |
| 480 | 640 | 320 | 176 | 112 | 48 | 640 | 320 | 176 | 112 | 48 |
| 960 | 1280 | 640 | 352 | 224 | 96 | 1280 | 640 | 352 | 224 | 96 |
| NOTE 1: The total number of interrupted slots is based on that SFN and subframe reference for per-FR gap in FR2 indicated by high layer parameter refServCellIndicator is an FR2 serving cell. NOTE 2: Slot occurs before or after the measurement gap may be interrupted additionally if SFN and subframe reference for per-FR gap in FR2 indicated by high layer parameter refServCellIndicator is an FR1 serving cell. |  |  |  |  |  |  |  |  |  |  |

# References

- R4-2115406, Email discussion summary for [100-e][231] NR_ext_to_71GHz_RRM_2, Intel, RAN4 #100e, 16th – 27th August 2021