3GPP TSG-RAN WG4 Meeting #100-e	R4-2115362

Electronic Meeting, August 16-27, 2021

Title: 	Simulation assumptions for RedCap L1 RSRP measurement

Source: 	Ericsson

Agenda item:	9.20.3.2

Document for:	Approval

# Introduction

The work item on NR support of reduced capability NR devices has been approved [1]. This work item contains an objective on UE complexity reduction as shown below [1].

- Specify support for the following UE complexity reduction features [RAN1, RAN2, RAN4]: - Reduced maximum UE bandwidth: - Maximum bandwidth of an FR1 RedCap UE during and after initial access is 20 MHz. - Maximum bandwidth of an FR2 RedCap UE during and after initial access is 100 MHz. - Reduced minimum number of Rx branches: - For frequency bands where a legacy NR UE is required to be equipped with a minimum of 2 Rx antenna ports, the minimum number of Rx branches supported by specification for a RedCap UE is 1. The specification also supports 2 Rx branches for a RedCap UE in these bands. - For frequency bands where a legacy NR UE (other than 2-Rx vehicular UE) is required to be equipped with a minimum of 4 Rx antenna ports, the minimum number of Rx branches supported by specification for a RedCap UE is 1. The specification also supports 2 Rx branches for a RedCap UE in these bands. - A means shall be specified by which the gNB can know the number of Rx branches of the UE. - ….. - Duplex operation: - HD-FDD type A with the minimum specification impact (Note that FD-FDD and TDD are also supported.)

This contribution contains simulation assumptions to evaluate the L1 RSRP measurement performance for release 17 RedCap UE with 1 Rx.

# Simulation Assumption for L1 RSRP measurement evaluation

## SSB based L1-RSRP measurements

Table 3: Parameters for SSB based L1-RSRP Measurements

| Parameters | Value |  |
| --- | --- | --- |
|  | FR1 | FR2 |
| Carrier frequency | 4GHz | 30GHz |
| Subcarrier Spacing for SSB | 15kHz, 30kHz | 120kHz |
| Data and control PSD relative to NR-PSS and NR-SSS | 0 dB |  |
| Transmitted SSB per SSB Periodicity | SSB #0 |  |
| SSB Periodicity | 20ms |  |
| Number of samples in L1 averaging | 3 samples (Same as Rel-15 L1-RSRP measurement) 5 samples Others are not precluded. |  |
| Transmit antennas | 1 |  |
| Receive antennas | 1 |  |
| Propagation conditions | AWGN TDL-A 30ns TDL-B 100ns TDL-C 300ns | AWGN TDL-A 30ns |
| UE speed | FR1: 30 km/h FR2: 3 km/h |  |
| CP type | Normal CP |  |
| SSB Es/Noc | [-3, -2, -1, 0] dB |  |

## CSI-RS based L1-RSRP measurements

Table 4: Parameters for CSI-RS based L1-RSRP Measurements

| Parameters | Value |  |
| --- | --- | --- |
|  | FR1 | FR2 |
| Carrier frequency | 4GHz | 30GHz |
| Subcarrier Spacing for CSI-RS | 15kHz, 30kHz | 120lHz |
| CSI-RS bandwidth | 24RBs |  |
| Data and control PSD relative to CSI-RS | 0 dB |  |
| CDM Type | ‘No CDM’ |  |
| Number of CSI-RS ports | 1 |  |
| CSI-RS Density | 3 |  |
| CSI-RS Periodicity | 20 slots for 15 kHz 40 slots for 30 kHz 80 slots for 120 kHz |  |
| Number of samples in L1 averaging | 3 samples (Same as Rel-15 L1-RSRP measurement) 5 samples Others are not precluded. |  |
| Transmit antennas | 1 |  |
| Receive antennas | 1 |  |
| Propagation conditions | FR1: AWGN TDL-A 30ns TDL-B 100ns TDL-C 300ns FR2: AWGN TDL-A 30ns |  |
| UE speed | FR1: 30 km/h FR2: 3 km/h |  |
| CP type | Normal CP |  |
| Es/Noc | [-3, -2, -1, 0] dB |  |

# Performance Metric

The following performance metrics can be considered

- The CDF curves (including the 95% and 5% CDF points) for delta L1-RSRP.

Delta L1-RSRP and delta CSI-RSRP is defined as follows:

Delta L1-RSRP = Measured L1-RSRP – Ideal L1-RSRP.

Ideal L1-RSRP is defined as the true value assuming perfect channel estimation using the same sampling rate and sampling occasions as used in the measured L1-RSRP evaluation.

Note1: The definition of SSB based L1-RSRP is the same as SS-RSRP defined in TS38.215.

Note 2: The definition of CSI-RS based L1-RSRP is the same as CSI-RSRP defined in TS38.215.

Interest companies are encouraged to evaluate the L1-RSRP measurement accuracy impact due to 1Rx.

# Reference

- RP-211038, Revised WID on support of reduced capability NR devices, Ericsson