3GPP TSG-RAN WG4 Meeting # 100-e 												         R4-2115421

Electronic Meeting, August 16-27, 2021

Agenda item:			6.1.7

Source:	Intel

Title:	WF on R16 NR RRM Enhancements

Document for:	Approval

### Sub-topic 1-1 Mandatory gap pattern signaling

Issue 1-1: Whether to keep 1 bit signalling for NR only measurement gap patten in LTE SA, EN-DC and NE-DC

- Option 1: Yes (Huawei, Ericsson, MTK, Intel)

- Option 2: No (Qualcomm):

  - Proposal 1: Add new bitmaps for signaling the support of NR-only measurement GPs in LTE-SA following NR-SA and NR-DC with the capability being optional for all GPs including GP 2,3,11.

  - Proposal 2: The bitmap for NR-only measurement gap patterns support in LTE-SA needs to be consistent with the support of NR-only measurement gap patterns in NR-SA and NR-DC. Otherwise, they should be all zero.

  - Proposal 3: In EN-DC, since UE signals the bitmap for LTE-SA, UE can use one bit to indicate the support for gap patterns as signalled by the bitmaps for LTE-SA. Similarly, in NE-DC, since UE signals the bitmap for NR-SA/DC, UE can use one bit to indicate the support for gap patterns as signalled by the bitmaps for NR-SA/DC.

  - Proposal 4: RAN4 takes one of the following action:

    - (1) Update note for UE feature list item 9-3 as:

Note: Agreements are provided in [TBD]. According to RAN4 agreement, a bitmap should be introduced for LTE-SA and a signle bit should be introduced for EN-DC and NE-DC.

    - (2) Send a separate LS to RAN2 to ask for new UE capability signaling.

### Sub-topic 1-2 Test case design of mandatory gap pattern

Issue 2-1: Test case design of mandatory gap pattern for Rel-16

Agreement in GTW (26th, Aug.):

- Session chair: No consensus to agree on Option 2. By default Option 1 applies. No further discussion expected.

- Option 1(Nokia, Ericsson):

  - For both FR1 and FR2, R15 test cases on mandatory gap patterns shall be inherited completely to R16 specifications, and R16 UEs shall pass all test cases.

- Option 2 (Apple, Qualcomm, OPPO):

  - For FR1 test:

    - UE capable of per-FR gap and GP#4 needs to pass both A.6.6.2.1 and A.6.6.2.9.

    - UE not capable of either per-FR gap or GP#4 needs to pass A.6.6.2.9 and is allowed to skip A.6.6.2.1

  - For FR2 test:

    - If the UE can pass A.7.6.2.9, it is allowed to skip A.7.6.2.1.