3GPP TSG-RAN WG4 Meeting#100-e	                                                                      R4-2115615

Online, 16th Aug – 27th Aug, 2021

Agenda item:			9.8.3

Source:	Moderator (CMCC)

Title:	Email discussion summary for [100-e][324] NR_HST_FR1_Demod

Document for:	Information

- Introduction

This email discussion focuses on UE demodulation for Rel-17 NR HST, including agenda 9.8.3.1~9.8.3.2. Two topics are included in total, including PDSCH requirements for CA scenarios and enhanced transmission schemes. The agreed way forward in previous meeting is in R4-2108635.

The targets of email discussion for 1st round and 2nd round are:

- 1st round: discuss the open issues and strive to minimize the open issues

- 2nd round: according to 1st round discussion, discuss left open issues for 2nd round, and strive to minimize the open issues, and strive to approve the WF.

Topic #1 PDSCH requirements for CA scenarios

Agenda  9.8.3.2

Companies’ contributions summary

| TDoc | Title | Source | Proposals/ Observations |
| --- | --- | --- | --- |
| R4-2113455 | Summary for FR1 HST demodulation results | Ericsson |  |
| R4-2112103 | Discussion on PDSCH CA Requirements in HST | Apple | Proposal #1: Define HST-CA requirements for FDD 15+FDD15, FDD 15+TDD 30, TDD 30+TDD 30, with same test applicability rule as CA CQI requirements. Proposal #2: If UE supports demodulationEnhancement-r16, for PDSCH CA requirements only HST-SFN JT requirements shall apply, otherwise HST-DPS requirements shall apply. Proposal #3: Define applicability rule between CA and single carrier requirements as: If UE supports demodulationEnhancement-r16, then HST-SFN requirements for CA shall apply. – If UE passes HST-SFN JT requirements for CA, UE can skip single carrier HST-SFN requirements If UE doesn’t support demodulationEnhancement-r16, then HST-DPS requirements for CA shall apply. – If UE passes HST-DPS JT requirements for CA, UE can skip single carrier HST-DPS requirements Proposal #4: Discuss release independent requirements based on conclusion of UE capability for HST CA requirements and whether RRM requirements for CA are defined as release independent. |
| R4-2112504 | Discussion on FR1 HST UE demodulation for CA scenario | CMCC | Proposal 1: it is not necessary to introduce new UE capabilities for HST-DPS CA and HST-SFN CA • for HST-DPS CA, no need to introduce UE capability • for HST-SFN CA, the existing UE capability introduced in Rel-16 NR HST can be reused Proposal 2: considering that advanced receiver is required for HST-SFN, we are OK with the applicability rule that if UE supports demodulationEnhancement-r16, only HST-SFN JT requirements shall apply for CA, otherwise HST-DPS requirements shall apply for CA. Proposal 3: for SCS configuration for CA scenario to be considered and applicability rule for SCS configuration, we are OK with option3. Observation 1: for single carrier scenario, there is applicability rule between HST-SFN and single tap. However, for CA scenario, only HST-SFN is considered. Proposal 4: for the applicability rule that if UE pass the HST-SFN JT requirements for CA, UE can skip HST-SFN JT requirements for single carrier defined in Rel-16, it is suggested for companies to consider how to handle single tap requirements for single carrier defined in Rel-16, does it mean that the single tap requirements for single carrier can also be skipped? Since there is applicability rule between HST-SFN for single carrier and single tap for single carrier defined in Rel-16. Proposal 5: it is not preferred to introduce applicability rule between single carrier and CA, since it has impact on requirements of other channel model, e.g. HST single tap, defined in Rel-16. Proposal 6: it is proposed that Rel-17 FR1 HST PDSCH CA requirements are release independent from Rel-15. |
| R4-2112511 | Simulation results for HST-DPS for CA scenario | CMCC |  |
| R4-2113131 | Views on HST CA PDSCH performance requirements | Intel Corporation | Proposal #1: Consider Option 4 for HST CA requirements definition. Proposal #2: Do not define UE capabilities for HST-DPS and HST-SFN CA. Proposal #3: Consider Option 1: If UE supports demodulationEnhancement-r16, only HST-SFN JT requirements shall apply, otherwise HST-DPS requirements shall apply for CA. Proposal #4: UE skip single carrier test case if it explicitly passes corresponding CA test case. Proposal #5: Define HST CA demodulation requirements in release independent manner only if corresponding RRM requirements will be release independent. Align exact release number with RRM requirements if such approach will be agreed. |
| R4-2113192 | Discussion on PDSCH requirements for CA scenarios | ZTE Corporation | Proposal 1: Option 1 is preferred for applicability rule for SFN and DPS. Proposal 2: If one UE passes Rel-17 DPS CA tests the Rel-16 DPS single carrier tests can be skipped. If one UE passes Rel-17 SFN CA tests the Rel-16 SFN single carrier tests can be skipped. Proposal 3: To wait for the outcome of RRM requirement to determine whether to support release independent from Rel-15 for HST PDSCH CA requirement. |
| R4-2113219 | Views on HST CA tests for FR1 | NTT DOCOMO, INC. | Proposal 1: For SCS configuration and applicability rules for SCS configuration, we prefer option 3. Proposal 2: For applicability rule between HST-SFN CA and DPS CA, we prefer to define Option 1 (i.e. If UE supports demodulationEnhancement-r16, only HST-SFN JT requirements shall apply, otherwise HST-DPS requirements shall apply for CA). Proposal 3: For the discussion about applicability rule between single carrier and CA, we prefer to clarify first the relationship between the Rel.16 applicability rule and the Rel.17 applicability rule before we discuss whether or not to introduce this applicability rule. Observation 1: In our understanding, if RAN4 agrees to introduce the applicability rule between HST-SFN/DPS single carrier requirements and HST-SFN/DPS CA requirements and UE pass the HST-SFN/DPS CA requirements, HST-SFN/DPS single carrier requirements and both Rel.15 and Rel.16 Single-tap requirements except for Rel.16 FDD Single-tap requirement can be skipped. |
| R4-2113454 | Update of simulation results for CA PDSCH with HST | Ericsson |  |
| R4-2113456 | PDSCH demodulation requirements for CA with HST-SFN scenario | Ericsson | Proposal 1: For PDSCH CA demodulation in HST (both SFN JT and DPS), RAN4 defines the following combinations: • FDD SCS=15kHz + FDD SCS=15kHz, TDD SCS=30kHz + TDD SCS=30kHz, and FDD SCS=15kHz + TDD SCS=30kHz. RAN4 don’t define PDSH CA requirements in HST (both SFN JT and DPS) for FDD 15KHz + TDD 15KHz CA and TDD 15 kHz + TDD 30 kHz CA. Proposal 2: If UE supports demodulationEnhancement-r16, only HST-SFN JT requirements for CA shall apply, otherwise HST-DPS requirements for CA shall apply. Proposal 3: Define the following applicability rules for HST-SFN JT and HST-DPS requirements for CA. • If UE pass the HST-SFN JT requirements for CA, UE can skip HST-SFN JT requirements for single carrier defined in Rel-16. o UE need to pass Rel-16 HST-DPS single carrier test according to the capability of active TCI state handling. o UE can skip Rel-15 HST single tap test. • If UE pass the HST-DPS requirements for CA, UE can skip HST-DPS requirements for single carrier defined in Rel-16. o UE don’t need to pass Rel-16 HST-SFN JT single carrier test if UE does not have the capability demodulationEnhancement-r16. o UE can skip Rel-15 HST single tap tests. |
| R4-2113790 | Discussion on PDSCH CA scenarios for NR UE HST FR1 performance requirements | Huawei,HiSilicon | Observation 1: Considering the RAN1 enhancement for HST-SFN and RAN4 HST-SFN CA are both Rel-17 features, the application scenario for normal HST-SFN is very limited. Proposal 1: Only define FDD 15kHz + FDD 1 kHz, TDD 30kHz + TDD30 kHz, FDD 15kHz +TDD30kHz requirements − Test #1: FDD 15 kHz + TDD 30 kHz > FDD 15 kHz + FDD 15 kHz − Test #2: TDD 30 kHz + TDD 30 kHz Proposal 2: For the applicability rule for HST-SFN joint transmission scheme and DPS transmission scheme, - Define applicability rule that UE has passed DPS CA requirements can skip SFN CA requirements, or - Define two UE capabilities for HST-DPS CA and HST-SFN CA, UE perform the test only when UE supports it. if UE supports both, test one or two scheme: − If UE has passed HST-DPS CA tests, HST-SFN CA tests can be skipped − If UE has passed HST-SFN CA tests, HST-DPS CA tests can be skipped Proposal 3: Define applicability rule between single carrier and CA, i.e. UE has passed HST CA requirements can skip corresponding single carrier requirements. Proposal 4: Do not define release independent from Rel-15 for HST PDSCH CA requirements. |
| R4-2113791 | Simulation results on PDSCH CA scenarios for NR UE HST FR1 performance requirements | Huawei, HiSilicon |  |
| R4-2114536 | Views on FR1 HST PDSCH CA Tests | Qualcomm Incorporated | Proposal 1: If UE passes the FR1 HST PDSCH CA tests for HST-SFN scheme, tests with HST-DPS scheme can be skipped. Proposal 2: Do not define FR1 HST PDSCH CA tests for Duplex/SCS combinations involving TDD 15kHz. Proposal 3: Reuse CA CQI applicability rule on CA duplex modes for testing: If UE supports both FDD 15 kHz + TDD 30 kHz and FDD 15 kHz + FDD 15 kHz CA duplex modes, FDD 15kHz + FDD 15kHz CA tests can be skipped. |

Open issues summary

Applicabiliy rule

Issue 1-1: SCS configurations and applicability rules for SCS configuration

- Agreements in RAN4#99-e meeting:

- Proposals in RAN4#100-e meeting:

|  | Number of CA Duplex modes | Number of required tests if UE supports all duplex modes |
| --- | --- | --- |
| Option 1 (Apple, ZTE, Ericsson, Huawei, Qualcomm) | 3 (FDD 15+FDD 15, FDD 15+TDD 30, TDD 30+TDD 30) | 2 (FDD 15 + TDD 30; TDD 30 + TDD 30) If UE supports both FDD 15 kHz + TDD 30 kHz and FDD 15 kHz + FDD 15 kHz CA duplex modes, apply requirements only to the first on |
| Option 3 (CMCC, DOCOMO, Ericsson) | 3 (FDD 15+FDD 15, FDD 15+TDD 30, TDD 30+TDD 30) | 3 (FDD 15 + FDD 15; FDD 15 + TDD 30; TDD 30 + TDD 30) |
| Option 4 (Intel) | 5 (FDD 15+FDD 15, FDD 15+TDD 15; FDD 15 + TDD 30; TDD 30 + TDD 30, TDD 15+TDD 30) | 2 (FDD 15 + TDD 30; TDD 30 + TDD 30) If UE supports both FDD 15 kHz + TDD 30 kHz and FDD 15 kHz + TDD 15 kHz CA duplex modes, apply requirements only to the first on If UE supports both FDD 15 kHz + TDD 30 kHz and FDD 15 kHz + FDD 15 kHz CA duplex modes, apply requirements only to the first on If UE supports both TDD 15 kHz + TDD 30 kHz and TDD 30 kHz + TDD 30 kHz CA duplex modes, apply requirements only to the first on |

- Recommended WF

  - More discussion is needed.

Issue 1-2: Applicability rule for HST-SFN joint transmission scheme and DPS transmission scheme

- Agreements in RAN4#99-e meeting:

  - Option 1: If UE supports demodulationEnhancement-r16, only HST-SFN JT requirements shall apply, otherwise HST-DPS requirements shall apply for CA.

  - Option 2: Define applicability rule that UE has passed DPS CA requirements can skip SFN CA requirements

  - Option 3: Define two UE capabilities for HST-DPS CA and HST-SFN CA, UE perform the test only when UE supports it. if UE supports both, test one scheme:

    - If UE has passed HST-DPS CA tests, HST-SFN CA tests can be skipped

    - If UE has passed HST-SFN CA tests, HST-DPS CA tests can be skipped

- Proposals in RAN4#100-e meeting:

  - Option 1 (Apple, CMCC, Intel, DOCOMO, Ericsson, Qualcomm): If UE supports demodulationEnhancement-r16, only HST-SFN JT requirements shall apply, otherwise HST-DPS requirements shall apply for CA.

  - Option 2 (Huawei):

    - Option 2a: Define applicability rule that UE has passed DPS CA requirements can skip SFN CA requirements, or

    - Option 2b: Define two UE capabilities for HST-DPS CA and HST-SFN CA, UE perform the test only when UE supports it. if UE supports both, test one or two scheme:

      - If UE has passed HST-DPS CA tests, HST-SFN CA tests can be skipped

      - If UE has passed HST-SFN CA tests, HST-DPS CA tests can be skipped

- Recommended WF

  - Option 1

Issue 1-3: Applicability rule between single carrier and CA

- Agreements in RAN4#99-e meeting:

  - FFS applicability rule between single carrier and CA

- Proposals in RAN4#100-e meeting:

  - Option 1 (Intel, ZTE, Huawei): UE skip single carrier test case if it explicitly passes corresponding CA test case

  - Option 1b (Ericsson): UE skip single carrier test case if it explicitly passes corresponding CA test case

    - If UE pass the HST-SFN JT requirements for CA, UE can skip HST-SFN JT requirements for single carrier defined in Rel-16.

      - UE need to pass Rel-16 HST-DPS single carrier test according to the capability of active TCI state handling.

      - UE can skip Rel-15 HST single tap test.

    - If UE pass the HST-DPS requirements for CA, UE can skip HST-DPS requirements for single carrier defined in Rel-16.

      - UE don’t need to pass Rel-16 HST-SFN JT single carrier test if UE does not have the capability demodulationEnhancement-r16.

      - UE can skip Rel-15 HST single tap tests.

  - Option 3 (DOCOMO): For the discussion about applicability rule between single carrier and CA, we prefer to clarify first the relationship between the Rel.16 applicability rule and the Rel.17 applicability rule before we discuss whether or not to introduce this applicability rule.

    - if RAN4 agrees to introduce the applicability rule between HST-SFN/DPS single carrier requirements and HST-SFN/DPS CA requirements and UE pass the HST-SFN/DPS CA requirements, HST-SFN/DPS single carrier requirements and both Rel.15 and Rel.16 Single-tap requirements except for Rel.16 FDD Single-tap requirement can be skipped.

  - Option 4 (CMCC): it is not preferred to introduce applicability rule between single carrier and CA, since it has impact on requirements of other channel model, e.g. HST single tap, defined in Rel-16.

- Recommended WF

  - 2 companies think the relation between Rel-16 and Rel-17 applicability rule need to be further clarified before we agree on introducing applicability rule between single carrier and CA.

  - Option 1b provides detailed description on which tests should be passed by UE under different conditions. Companies please provide your comments on whether option 1b is agreeable.

Issue 1-4: Release independent

- Agreements in RAN4#99-e meeting:

  - FFS whether HST PDSCH CA requirements can be release independent from Rel-15

- Proposals in RAN4#100-e meeting:

  - Option 1a (Apple): Discuss release independent requirements based on conclusion of UE capability for HST CA requirements and whether RRM requirements for CA are defined as release independent.

  - Option 1b (Intel, ZTE): Define HST CA demodulation requirements in release independent manner only if corresponding RRM requirements will be release independent. Align exact release number with RRM requirements if such approach will be agreed.

  - Option 2 (CMCC): it is proposed that Rel-17 FR1 HST PDSCH CA requirements are release independent from Rel-15.

  - Option 3 (Huawei): Do not define release independent from Rel-15 for HST PDSCH CA requirements

- Recommended WF

  - 3 companies think the release independent of demodulation should follow the conclusion of RRM. It seems that there are similar comments to suggest RRM follow demodulation conclusion. To avoid the back and force discussion, can we have the following agreement?

    - The release independent of Rel-17 CA HST should be first discussed in RRM session, and the same conclusion of release independent of RRM for HST CA will be applied to HST CA demodulation.

Companies views’ collection for 1st round

Open issues

| Company | Comments |
| --- | --- |
| Ericsson | Issue 1-1: We prefer option 1 or 3. As we explained in our paper, we think TDD SCS=15kHz performance in HST-SFN/DPS scenario can be ensured even with CA scenario by defining the following test cases: - Single carrier PDSCH with HST-SFN/DPS with FDD SCS=15kHz (defined in Rel-16) - CA PDSCH with HST-SFN/DPS with FDD SCS=15kHz (to be defined in Rel-17) - CA PDSCH without HST-SFN/DPS with TDD SCS=15kHz (defined in Rel-16) HST-SFN/DPS performance should be same for both FDD and TDD as far as SCS and TRS configurations are same. PDSCH performance should be same regardless of CA or single carrier. We believe the test cases above ensure the performance with ’CA PDSCH with HST-SFN/DPS with TDD SCS=15kHz’. Issue 1-2: Option 1. Since the advanced HST-SFN receiver requires more baseband processing compared with DPS, some UE may only support single carrier case. In this case, UE does not need to declare the capability demodulationEnhancement-r16 for CA tests. [20210818] If our understanding is correct, companies agree to test either HST-SFN CA or HST-DPS CA, according to RAN#91e, as pointed by Apple. Then we are discussing how to choose test case. Option 1 is based on UE capability signaling demodulationEnhancement and option 2b is based on additional capabilities for CA. However option 2a is not clear for us how UE choose test. In our understanding UE receiver capability does not depend on the number of carriers or CBW. For example, Enhanced Receiver Type 1 should be applicable for CA case also, although RAN4 has no requirements. Since most companies do not want to introduce additional capabilities in Rel-17 FR1 HST WI, we prefer to use the UE capability demodulationEnhancement to choose tests. We would like to clarify our proposal more. We believe the second bullet captures option 2a. - When UE declares the capability demodulationEnhancement-r16, UE need to pass HST-SFN JT for CA. UE can skip HST-DPS for CA - When UE does not declare the capability demodulationEnhancement-r16, UE need to pass HST-DPS for CA. UE can skip HST-SFN for CA. Issue 1-3: Considering the concern by Docomo, we propose to modify option 1b as follows: - UE skip single carrier test case if it explicitly passes corresponding CA test case - If UE pass the HST-SFN JT requirements for CA, UE can skip HST-SFN JT requirements for single carrier defined in Rel-16. - UE need to pass Rel-16 HST-DPS single carrier test according to the capability of active TCI state handling. - UE need to pass Rel-16 FDD HST single tap test. - UE can skip Rel-15 HST single tap test. - If UE pass the HST-DPS requirements for CA, UE can skip HST-DPS requirements for single carrier defined in Rel-16. - UE don’t need to pass Rel-16 HST-SFN JT single carrier test if UE does not have the capability demodulationEnhancement-r16. - UE need to pass Rel-16 FDD HST single tap test. - UE can skip Rel-15 HST single tap tests. Issue 1-4: Support the recommended WF. |
| Intel | Issue 1-1: SCS configurations and applicability rules for SCS configuration RAN4 should find a balance between test coverage and test efforts considering that UE has to pass normal CA requirements and CA CQI requirements. Testing of FDD15+FDD15 combination is redundant taking into account that UE needs to pass FDD15 + TDD30 combination in HST CA and FDD15+FDD15 in Normal CA requirements. In this case either Option 1 or Option 4 is fine for us but Option 3 leads to unnecessary additional test efforts. Issue 1-2: Applicability rule for HST-SFN joint transmission scheme and DPS transmission scheme In general, neither HST-SFN cannot cover HST-DPS nor HST-DPS cannot cover HST-SFN. From demodulation performance and receive processing these scenarios are completely different. From UE complexity perspective it is better to perform testing of HST-SFN in case UE supports both schemes. Same time since HST-SFN feature was introduced from Rel-13 LTE we can expect that most of the UEs will support HST-SFN processing and HST-DPS CA testing will not be performed at all. However, HST-DPS scheme is more important for NR deployments because it provides higher performance than HST-SFN. In this case we would like to propose new option for further discussion: No applicability rule between HST-SFN CA and HST-DPS. In our understanding it is the most optimal way forward considering diverse views on this issue. Issue 1-3: Applicability rule between single carrier and CA We think clarified option proposed by Ericsson is a reasonable solution to reduce UE test efforts. Issue 1-4: Release independent At least one UE vendor raised the issue that joint HST-SFN processing + support of CA might be too complex implementation from baseband perspective for Rel-15 UEs. In this case it is better to ask other UE vendors to provide their view on it. At least from specification perspective it is possible since both HST-SFN and normal CA processing are supported from Rel-15. |
| Huawei | Issue 1-1: SCS configurations and applicability rules for SCS configuration Option 1. We do not think that TDD 15kHz SCS is typical for NR TDD deployment. Also support of different SCS configurations has been covered in CA test, it is not necessary to duplicate the SCS configuration testing by HST CA requirements, like did for CA CQI without including TDD 15 kHz SCS. So we prefer to reuse the same methodology for CA CQI test. Issue 1-2: Applicability rule for HST-SFN joint transmission scheme and DPS transmission scheme We don’t agree some companies’ argument that “it is very unlikely that UE supporting HST-SFN will fail HST-DPS test since HST-SFN demodulation algorithm is much more advanced”. In our view, very different algorithms for these two transmission schemes are used, it is unreasonable to compare these two totally different transmission schemes. As per WID RP-210833, the purpose for defining applicability rule is to minimize the testing burden. However, considering the applicability rule between single carrier and CA at the same time, the maximum number of cases is 2 for all cases as shown below. Case 1; Case 2; Case 3. HST-SFN CA; o; x; o. HST-DPS CA; x; o; o. HST-SFN; x; o; x. HST-DPS; o; x; x. Number of cases; 2; 2; 2. As per the analysis shown in above table, introduction of test applicability rule between HST-SFN CA and HST-DPS CA will not minimize the testing at all. Considering HST-DPS 1a is mandatory, it is unreasonable to skip HST-DPS CA if UE supports demodulationEnhancement-r16 for optional HST-SFN, this will allow UE to skip a mandatory feature testing with support of an optional feature testing, so Option 1 is unreasonable and unacceptable for us. That is the logic that we propose Option 2a that UE has passed HST-DPS CA requirements can skip HST-SFN CA requirements considering HST-DPS 1a is mandatory to support, but we are also ok with Option 2b as a compromise way to define UE capability for HST-DPS 1b CA and HST-SFN CA to move forward, it is leave up to UE for real testing selection. Also considering a UE support HST-SFN single carrier by reporting demodulationEnhancement-r16 but not support normal HST-SFN CA and support HST-SFN with DL frequency offset pre-compensation CA, if we still use the same capability signalling demodulationEnhancement-r16 for both HST-SFN single carrier and HST-SFN CA, it is very confusing. [2021-08-19] As we stated, if we check the WID RP-210833, the following is captured: We cannot derive the conclusion that test applicability rule should be defined between HST-SFN CA and HST-DPS CA transmission schemes. It just states that test applicability should be discussed in order to minimize the testing. Whether to define the test applicability or how to define the test applicability depend on the discussion, but the ultimate purpose is to minimize the testing burden. As we analysed, there are three possible test applicability rules that can achieve the same testing burden reduction purpose. Especially now we are also discussing the test applicability between CA and single carrier. Also we disagree that companies draw conclusion just based on their own theory analysis and ignore the real product implementation that comprehensively consider and balance the complexity to implement more features published in one release. It is unreasonable to do the simple addition operation to implement more than one features, i.e. HST-SFN + CA ≠ HST-SFN CA Issue 1-3: Applicability rule between single carrier and CA Considering that CA requirements are defined for each CC and there is no any difference between single carrier and CA requirements from the perspective of requirements definition, also the performance is checked per CC, but higher requirements for CA with all supported carriers are activated during the test. Therefore, we prefer to define applicability rule between single carrier and CA, i.e. UE has passed HST CA requirements can skip corresponding single carrier requirements. For the applicability rule between HST-SFN CA/HST-DPS CA and single-tap, we think the single-tap case can be skipped if HST-SFN CA/HST-DPS CA is passed. Issue 1-4: Release independent Currently in Rel-17 NR HST WI, whether RRM measurement enhancement for SCell to be specified is still under discussion, maybe stricter RRM measurement requirements will be specified that is higher challenge to UE implementation. From the demodulation part we also need to further discuss whether additional capability signalling is required for demodulation of HST-SFN JT CA considering the complicated processing for HST-SFN CA, it can’t be ensured that UE supporting HST-SFN and CA features can work in HST-SFN CA. Especially a Rel-16 UE can declare to support demodulationEnhancement-r16 for HST-SFN, but a Rel-17 UE does not support normal HST-SFN, Rel-17 UE can only supports HST-SFN with DL frequency offset pre-compensation without support demodulationEnhancement-r16 for HST-SFN single carrier and HST-SFN CA as a Rel-16 UE does, this is different from our previous logic that Rel-17 UE should support all features which a Rel-16 UE supports, but this is reasonable considering HST-SFN enhancement for Release 17. Different release UEs have different implementation logic, we cannot mandate a Release 15 UE to meet the requirements to defined in later Release 17, also Release 15 chipset has been launched several years ago, it is hard to do all upgrades, but new published Release 17 UE with Release 15 chipset inside will have to be tested against on Release 17 requirements. So it is unreasonable to impose release 17 requirements to be release independent from Rel-15, i.e. we prefer to not define release independence for HST CA requirements. |
| Apple | Issue 1-1: SCS configurations and applicability rules for SCS configuration We support option 1. We don’t think its reasonable to define HST CA requirements for all Duplex SCS combinations like we had for normal PDSCH. Hence we support to use the same methodology as CQI reporting in CA. Issue 1-2: Applicability rule for HST-SFN joint transmission scheme and DPS transmission scheme We support option 1 for the reasons provided in our paper. In RAN#91e it was agreed that applicability rule shall be defined between HST SFN-JT and HST-DPS schemes for CA scenarios to reduce test burden. Also, if HST-DPS single carrier will still be tested even if UE supports HST-SFN. We don’t see any requirement coverage hole. Issue 1-3: Applicability rule between single carrier and CA We support option 1b with additional clarification from Ericsson above. Issue 1-4: Release independent We are fine with recommended WF. Also, the exact release number should be aligned between RRM and demod if any release independent requirements agreements are made. |
| Qualcomm | Issue 1-1: SCS configurations and applicability rules for SCS configuration Prefer option 1. We don’t see how it helps testing both FDD15+FDD15 and FDD15+TDD30 since requirements are defined per carrier and FDD15 requirements will already get tested with FDD15+TDD30, if UE supports both. Also, TDD15 is not a typical scenario and it wasn’t considered for single carrier requirements. So, we don’t see the need to define those tests for CA. Issue 1-2: Applicability rule for HST-SFN joint transmission scheme and DPS transmission scheme Ok with recommended WF. Issue 1-3: Applicability rule between single carrier and CA Ok with Ericsson’s modified Option 1b. Issue 1-4: Release independent Ok with recommended WF. |
| ZTE | Issue 1-1: SCS configurations and applicability rules for SCS configuration We support option 1. TDD 15kHz is not a typical deployment. Comparing option 1 and option 3, option 1 is preferred to reduce test cases. Issue 1-2: Applicability rule for HST-SFN joint transmission scheme and DPS transmission scheme HST-SFN and DPS belong to different transmission scheme, and the receiving algorithm and performance will be different. If the applicability rule needs to be introduced between SFN and DPS, we prefer option 1. Issue 1-3: Applicability rule between single carrier and CA Option 1 is preferred. When the transmission scheme for CA is tested the transmission for single carrier can be skipped and single tap can be skipped. Issue 1-4: Release independent Ok with recommended WF. |
| CMCC | Issue 1-1: SCS configurations and applicability rules for SCS configuration Option 3. We can compromise not to specify test case for TDD 15KHz. But for FDD 15+FDD 15, FDD 15+TDD 30, TDD 30+TDD 30, we prefer not to introduce test applicability rule as CA CQI requirements. In normal PDSCH CA requirements, there is no test applicability rule for these three test cases, we prefer to follow the same way for HST PDSCH CA requirements. Issue 1-2: Applicability rule for HST-SFN joint transmission scheme and DPS transmission scheme We can compromise to option 1. As we mentioned in previous meetings, we do not prefer to introduce any applicability rule between HST-SFN joint transmission and HST DPS transmission, since they are different transmission schemes, that is the reason we do not have applicability rule between these two schemes for single carrier case. For CA case, if companies have strong preference to have applicability rule for CA case, we can compromise to option 1, considering that both HST-SFN joint transmission scheme and DPS scheme are verified in single carrier cases. Issue 1-3: Applicability rule between single carrier and CA Considering that there is applicability rule between different channel model for single carrier cases, for example, the applicability rule between HST-SFN/DPS and single tap, we do not prefer to introduce applicability rule between single carrier and CA. However, if companies have strong preference to have the applicability rule between single carrier and CA, as a compromise, we can consider the applicability rule but it is necessary to clear the impact to the applicability to the existing Rel-15/16 requirements. Another consideration is that if UE do not explicitly pass corresponding CA test case, the single carrier test cases cannot be skipped. Taking above consideration into account, we can compromise to option 1b or the modified 1b proposed by Ericsson. Issue 1-4: Release independent We are OK with the recommended WF. |
| DCM | Issue 1-1: SCS configurations and applicability rules for SCS configuration Our preference is Option 3. We have a similar view as CMCC. We don’t see the necessity to apply CA CQI applicability rule to CA PDSCH test. Issue 1-2: Applicability rule for HST-SFN joint transmission scheme and DPS transmission scheme OK with the recommended WF. Issue 1-3: Applicability rule between single carrier and CA In our understanding, the modified 1b proposed by Ericsson reflect the existing Rel.15/16 single carrier applicability rulers. On the other hand, some companies show the different understanding. Thus, we prefer to clarify first the relationship between the existing Rel.15/16 and the new applicability rule. Also, we don’t have a strong motivation to introduce the application rules between single carrier and CA. Issue 1-4: Release independent OK with the recommended WF |

CRs/TPs comments collection

| CR tdoc number | Comments collection |
| --- | --- |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Issue 1-1: SCS configurations and applicability rules for SCS configuration | Option 1 (Apple, ZTE, Ericsson, Huawei, Qualcomm, Intel) Option 3 (CMCC, DOCOMO, Ericsson) Option 4 (Intel) 6 companies support option1, 3 companies support option 3, while only 1 company support option4. So moderator recommends to remove option4. Recommended WF for 2nd round: Continue to discuss option1 and option3 in 2nd round. Compromise is highly appreciated. |
| Issue 1-2: Applicability rule for HST-SFN joint transmission scheme and DPS transmission scheme | Option1 (Ericsson, Apple, Qualcomm, ZTE, CMCC,DOCOMO) Option 2 (Huawei) 6 companies support option1, 1 company insist on option2, while 1 company propose to not define any applicability rule between HST-SFN CA and HST-DPS. Recommended WF for 2nd round: Continue to discuss the following proposal in 2nd round (modified option1 proposed by Ericsson in 1st round). Compromise is highly appreciated. • When UE declares the capability demodulationEnhancement-r16, UE need to pass HST-SFN JT for CA. UE can skip HST-DPS for CA • When UE does not declare the capability demodulationEnhancement-r16, UE need to pass HST-DPS for CA. UE can skip HST-SFN for CA. |
| Issue 1-3: Applicability rule between single carrier and CA | Majority companies agree with the modified option 1b from Ericsson in 1st round discussion. While some company still prefer to further clarify the relationship between existing R15/R16 and the new applicability rule. Tentative agreements: - UE skip single carrier test case if it explicitly passes corresponding CA test case - If UE pass the HST-SFN JT requirements for CA, UE can skip HST-SFN JT requirements for single carrier defined in Rel-16. - UE need to pass Rel-16 HST-DPS single carrier test according to the capability of active TCI state handling. - UE need to pass Rel-16 FDD HST single tap test. - UE can skip Rel-15 HST single tap test. - If UE pass the HST-DPS requirements for CA, UE can skip HST-DPS requirements for single carrier defined in Rel-16. - UE don’t need to pass Rel-16 HST-SFN JT single carrier test if UE does not have the capability demodulationEnhancement-r16. - UE need to pass Rel-16 FDD HST single tap test. - UE can skip Rel-15 HST single tap tests. Recommended WF for 2nd round: Check and approve the above tentative agreement. |
| Issue 1-4: Release independent | 5 companies support the recommended WF, while 2 companies think release independent of demodulation also needs to be discussed. Recommended WF for 2nd round: Focus on the feasibility and technical discussion on release independent of demodulation requirements in this email thread. Note: The formal agreements of release independent will consider the conclusion of RRM session. |

Recommendations on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |

Discussion on 2nd round (if applicable)

Open issues summary

Open issues

| Company | Comments |
| --- | --- |

Summary on 2nd round (if applicable)

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |

Recommendations for Tdocs

1st round

New tdocs

| Title | Source | Comments |
| --- | --- | --- |
| WF on FR1 HST demodulation | CMCC |  |

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2113455 | Summary for FR1 HST demodulation results | Ericsson | Noted |  |
| R4-2112103 | Discussion on PDSCH CA Requirements in HST | Apple | Noted |  |
| R4-2112504 | Discussion on FR1 HST UE demodulation for CA scenario | CMCC | Noted |  |
| R4-2112511 | Simulation results for HST-DPS for CA scenario | CMCC | Noted |  |
| R4-2113131 | Views on HST CA PDSCH performance requirements | Intel Corporation | Noted |  |
| R4-2113192 | Discussion on PDSCH requirements for CA scenarios | ZTE Corporation | Noted |  |
| R4-2113219 | Views on HST CA tests for FR1 | NTT DOCOMO, INC. | Noted |  |
| R4-2113454 | Update of simulation results for CA PDSCH with HST | Ericsson | Noted |  |
| R4-2113456 | PDSCH demodulation requirements for CA with HST-SFN scenario | Ericsson | Noted |  |
| R4-2113790 | Discussion on PDSCH CA scenarios for NR UE HST FR1 performance requirements | Huawei,HiSilicon | Noted |  |
| R4-2113791 | Simulation results on PDSCH CA scenarios for NR UE HST FR1 performance requirements | Huawei, HiSilicon | Noted |  |
| R4-2114536 | Views on FR1 HST PDSCH CA Tests | Qualcomm Incorporated | Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

2nd round

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents

- Annex

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| Ericsson | Kazuyoshi Uesaka | kazuyoshi.uesaka@ericsson.com |
| Intel | Putilin Artyom | artyom.putilin@intel.com |
| Apple | Manasa Raghavan | Manasa.raghavan@apple.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)