3GPP TSG-RAN4 Meeting #100-e	R4-2115808

# Online, , 16th August 2021 - 27th August 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | 0269 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR: TS 38.101-4: n262 demodulation requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_47GHz_band-Perf |  |  |  |  |  | Date: |  |  | 2021-08-26 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Existing radiating requirements support up to 40000MHz and it does not cover the new NR 47GHz band (n262). In addition, Noc values are not specified for n262. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Frequency range for performance requirements extended from 40000 to 48200 MHz. TS38.101-2 V17.2.0 Clauses 7.3.2.x specifie REFSENS of n262 with CBW=50MHz as -92.5/-86.8/-82.8/-91.0 dBm for PC3. Noc values are derived according TS38.101-4 4.5.3.3: Noc(PC_P, Band_X) = REFSENSPC_P, Band_X, 50MHz – 10log10(12 x 120kHz x PRBREFSENS) – SNRREFSENS + Δthermal = REFSENSPC_P, Band_X, 50MHz - 69.8 * Noc(PC1, Band_n262) = -92.5 - 69.8 = -162.3 * Noc(PC2, Band_n262) = -86.8 - 69.8 = -156.6 * Noc(PC3, Band_n262) = -82.8 - 69.8 = -152.6 * Noc(PC4, Band_n262) = -91.0 - 69.8 = -160.8 Introducing the applicability rule of FR2 UE demodulation requirements for UE supporting band n262. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Performance requirement definition for n262 is not complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.5.3.2, 7.1.1.1, 7.1.1.6 (new), 8.1.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-4 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

----------------------------------------------------- Beginning of Change ------------------------------------------------------------

#### 4.5.3.2	Noc for NR operating bands in FR2

Values for Noc according to operating band and power class for single carrier requirements are specified in Table 4.5.3.2-1 for ∆BB =1dB.

Table 4.5.3.2-1: Noc power level for different UE power classes and frequency bands

| Operating band | UE Power class |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 |
| n257 | -167.3 | -161.8 | -158.1 | -166.8 | -161.9 |
| n258 | -167.3 | -161.8 | -158.1 | -166.8 | -162.1 |
| n259 |  |  | -154.5 |  |  |
| n260 | -164.3 |  | -155.5 | -164.8 |  |
| n261 | -167.3 | -161.8 | -158.1 | -166.8 |  |
| n262 | -162.3 | -156.6 | -152.6 | -160.8 |  |
| Note 1: Noc levels are specified in dBm/Hz |  |  |  |  |  |

For PC3 multi-band devices, the Noc power level (NocMB) shall increase by multi-band relaxation defined in Table 6.2.1.3-4 of TS 38.101-2 [7]:

NocMB = NocSB + ∆MBP,n

-	NocSB is the Noc defined in Table 4.5.3.2-1

-	∆MBP,n values are specified in TS 38.101-2 [7].

For CA case, the Noc power level (NocCA) shall increase by a relaxation factor defined in TS 38.101-2 [7] Table 7.3A.2.1-1:

NocCA = NocSC + ΔRIB

-	NocSC is derived by assuming UE supports single carrier.

-	ΔRIB values are specified in TS 38.101-2 [7].

------------------------------------------------ Unchanged sections omitted --------------------------------------------------------

## 7.1	General

### 7.1.1	Applicability of requirements

#### 7.1.1.1	General

The minimum performance requirements are applicable to the FR2 operating bands defined in TS 38.101-2 [7] with FDL_high not exceeding 48200 MHz. Additional applicability rules for certain operating bands are specified in Clause 7.1.1.6.

The minimum performance requirements in Clause 7 are mandatary for UE supporting NR operation, except test cases listed in Clause 7.1.1.3, 7.1.1.4, 7.1.1.5.

If same test is listed for different UE features/capabilities in Clauses 7.1.1.3 and 7.1.1.4, then this test shall apply for UEs which support all corresponding UE features/capabilities.

------------------------------------------------ Unchanged sections omitted --------------------------------------------------------

#### 7.1.1.6	Applicability of requirements for operating bands

The applicability rules for FR2 operating bands are specified in Table 7.1.1.6-1.

Table 7.1.1.6-1: Requirements applicability for operating bands

| Test type |  | Test list | Applicability notes |
| --- | --- | --- | --- |
| FR2 TDD | PDSCH | Clause 7.2.2.2.1 (Test 1-4) | The requirements are applicable for bands with FDL_high higher than 40000 MHz and lower than 48200 MHz with additional margin as TBD dB. |
| FR2 TDD | PDSCH | Clause 7.2.2.2.1 (Test 2-6) Clause 7.2.2.2.1 (Test 3-1) | The requirements are applicable for bands with FDL_high higher than 40000 MHz and lower than 48200 MHz with additional margin as 0.5 dB. |

------------------------------------------------ Unchanged sections omitted --------------------------------------------------------

#### 8.1.1.1	General

The minimum performance requirements are applicable to the FR2 operating bands defined in TS 38.101-2 [7] with FDL_high not exceeding 48200 MHz.

The minimum performance requirements in Clause 8 are mandatory for UE supporting NR operation, except test cases listed in Clause 8.1.1.3, 8.1.1.4, 8.1.1.5.

If same test is listed for different UE features/capabilities in Clauses 8.1.1.3 and 8.1.1.4, then this test shall apply for UEs which support all corresponding UE features/capabilities.

------------------------------------------------------------- End of change ------------------------------------------------------------