3GPP TSG-RAN WG4 Meeting #100-e	R4-21xxxxx

Electronic Meeting, Aug. 16 – Aug. 27, 2021

Agenda Item:	9.4.3 and 9.4.6.3

Source:	Apple

Title:	WF on FR2 enhancement part 2: UL gaps

Document for:	Discussion

# 1	Tx power management: RF aspect

Summary of 1st round email discussion is captured in [1]. Some observations are captured below:

- On phantom or blocking be introduced in UL gap testing.

  - No feasible in R17: oppo, Apple, HW, Sony, vivo, Intel.

  - Phantom should be introduced: QC

- Metric:

  - Option 1: Delta P-MPR: Apple, ZTE, Sony, vivo, Intel, DCM, Ericsson.

  - Option 2: Peak EIRP: Oppo,

  - Option 1 and option 2 without phantom: Nokia, Ericsson

  - Option 1 and option 2 with phantom: QC

Proposed agreement:

- No phantom is introduced in R17 UL gap testing.

Proposed agreement:

- Option 1: “P-MPR report without phantom” based, X dB P-MPR enhancement should be achieved.

  - X is defined as one of the options below

    - 6dB

    - A value between 6dB and 3dB, which is typical in the field

    - A value below 3dB

  - FFS on the impact of P-MPR report granularity

  - FFS on the implementation margin

- Option 2: “P-MPR report+peak EIRP without phantom”, X dB EIRP gain and P-MPR<YdB should be achieved.

  - X is defined as one of the options below

    - 6dB

    - A value between 6dB and 3dB, which is typical in the field

    - A value below 3dB

  - Y is defined as

    - A value below 3dB

    - 0dB

  - FFS on the implementation margin

# 2	UL Tx power management: RRM aspect

Summary of 1st round email discussion is captured in [1]. Some observations are captured below:

- On UL gap activation:

  - Option 1: implicitly activated by P-MPR reporting from UE. The activation criteria is determined and signaled by the NW (apple)

  - Option 2:  explicitly activated by NW (apple, Huawei, ZTE, qualcomm, sony, nokia, DCM, vivo, Ericsson, intel)

- On UL gap deactivation

  - Option 1: implicitly deactivated by [TBD] reporting from UE. The deactivation criteria is determined and signaled by the NW (apple)

  - Option 2:  explicitly deactivated by NW (apple, Huawei, ZTE, qualcomm, sony, nokia, DCM, vivo, Ericsson, intel)

- In case of explicit activation/deactivation, UL gap should be

  - Option 1: MAC CE (Apple, Huawei, ZTE, Qualcomm, Sony, DCM, vivo, Ericsson, Intel )

  - Option 2: DCI (DCM)

  - Option 3: RRC (Nokia, Ericsson)

- Candidate gap configurations: UGL (UL gap length), UGRP (UL gap repetition periodicity)

  - UGL: 0.5ms, UGRP: 20ms (Huawei)

  - UGL: 1ms, UGRP:20ms (Huawei, apple)

  - UGL: 1.25ms, UGRP: 20ms (apple)

  - UGL: 0.5ms, UGRP:40ms (Huawei)

  - UGL: 1ms, UGRP:40ms (Huawei)

  - UGL: 0.125ms, UGRP:5ms (Qualcomm)

  - UGL: 0.125ms, UGRP:10ms (Qualcomm)

  - UGL: 0.125ms, UGRP:20ms (Qualcomm)

  - UGRP: 160ms (Sony, vivo, Ericsson, intel)

  - UGRP: 320ms (Sony, vivo, Ericsson, intel)

Proposed agreement:

- UL gap should be explicitly activated by NW via signaling

  - How can UE indicate the NW UL gap activation is needed?

    - Option 1: UE explicitly indicates to NW by signaling

    - Option 2: UE implicitly indicate to NW by P-MPR reporting. The exact P-MPR value is FFS.

- UL gap should be explicitly deactivated by NW via signaling

  - How can UE indicate the NW UL gap deactivation is needed?

    - Option 1: UE explicitly indicates to NW by signaling

    - Option 2: UE implicitly indicate to NW by [TBD] reporting.

- UL gap should be explicitly activated and deactivated by MAC CE

- Send LS to RAN2 for above agreement.

- Further discussion on down-selection of gap configurations.

# 3	UL coherent MIMO

Summary of 1st round email discussion is captured in [1].

Proposed WF on performance gain evaluation:

- The gain of UL MIMO with UL gap configured has been shown.

- Observations:

  - In R4-2111383, it shows 20.3% mean throughput gain and maximum 40.7% throughput gain with 40 degree phase error.

  - In R4-2114492, it shows further 8% throughput gain can be reached by further improve relative phase error requirement.

- The metric of the performance gain for further discussion:

    - Option 1: By removing side conditions of coherent UL MIMO requirement, the requirement for relative phase/power keep the same, i.e. 40 degree/4dB

    - Option 2: By improving the requirement for relative phase/power, e.g. 30 degree for relative phase

- New requirement if identified can be discussed in phase II

- Companies are encouraged to provide analysis based on gap pattern examples

  - Other gap pattern is not precluded

# References

[1] R4-2114730, “Email discussion summary for [100-e][130] NR_RF_FR2_req_enh2_Part_2”, Apple