3GPP TSG-RAN4 Meeting #100-e	R4-2115138

# Electronic Meeting, August 16, 2021 - August 27, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.716-21-21 | CR | 0001 | rev |  | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR 37.716-21-21: Addition of missing lower order fallbacks (Rel-16) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | DC_R16_xBLTE_2BNR_yDL2UL-Core |  |  |  |  |  | Date: |  |  | 2021-08-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | These configurations have relating higher order configurations already in REL16 specs. This CR captures necessary analysis into the TR. DC_3A_n7A-n28A DC_3C_n7A-n28A DC_3A_n7A-n78(2A) DC_3C_n7A-n78(2A) |  |  |  |  |  |  |  |  |
| Summary of change: |  | Missing lower order fallbacks are added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lower order fallbacks are missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.67 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***************************** Start of Changes ******************************

## 6.67	DC_3_n7-n28

### 6.67.1	Operating bands for DC

Table 6.67.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3_n7-n28 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | TDD |

### 6.67.2	Channel bandwidths per operating band for DC

Table 6.67.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_3A_n7A-n28A | DC_3A_n7A DC_3A_n28A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 90 |
|  |  | n7 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3C_n7A-n28A | DC_3A_n7A DC_3A_n28A DC_3C_n7A DC_3C_n28A | 3 | 15 | See CA_3C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 110 |
|  |  | n7 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.67.3	Co-existence studies

The harmonics and intermodulation products analysis for 2UL bands 3 and 7 with 3DL bands 3, 7 and 28 is presented in clause 6.11.1.2 in TR 36.879-13.

The harmonics and intermodulation products analysis for 2UL bands 3 and 28 with 3DL bands 3, 7 and 28 is presented in clause 6.21.1.2 in TR 36.715-00-02.

Based on the above studies,

-	2nd order IMD generated by dual uplink of Band 3 + Band n7 may fall into own Rx of band n28

-	3rd order IMD generated by dual uplink of Band 3 + Band n28 may fall into own Rx of band n7

### 6.67.4	∆TIB and ∆RIB values

For DC_3_n7-n28, the TIB,c and RIB values can be based on E-UTRA CA_3-7-28 given in the tables below.

Table 6.67.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3_n7-n28 | 3 | 0.5 |
|  | n7 | 0.5 |
|  | n28 | 0.3 |

Table 6.67.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3_n7-n28 | 3 | 0 |
|  | n7 | 0 |
|  | n28 | 0 |

### 6.67.5	MSD

Table 6.67.5-1 lists the MSD for DC_3A_n7A-n28A which is reused from E-UTRA 3DL/2UL CA_3A_7A-28A in TS 36.101. The MSD values were previously derived in clause 6.11.1.4 in TR 36.879-13 and clause 6.21.1.4 in TR 36.715-00-02.

Table 6.67.5-1: MSD for the DC configuration

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA | EUTRA CA | EUTRA band | UL Fc | UL BW | UL | DL Fc | DL BW | MSD | Duplex mode |  |
| DL Configuration | UL Configuration |  | (MHz) | (MHz) | CLRB | (MHz) | (MHz) | (dB) |  |  |
| DC_3A_n7A-n28A DC_3C_n7A-n28A | DC_3A_n7A DC_3C_n7A | 3 | 1747 | 5 | 25 | 1842 | 5 | N/A | FDD | N/A |
|  |  | n7 | 2543 | 5 | 25 | 2663 | 5 | N/A |  | N/A |
|  |  | n28 | 741 | 5 | 25 | 796.0 | 5 | 20.0 |  | IMD2 |
|  | DC_3A-n28A DC_3C-n28A | 3 | 1712.5 | 5 | 25 | 1807.5 | 5 | N/A | FDD | N/A |
|  |  | n7 | 2562 | 5 | 25 | 2682 | 5 | 17.0 |  | IMD3 |
|  |  | n28 | 743 | 5 | 25 | 798 | 5 | N/A |  | N/A |

## 6.x	DC_3A_n7A-n78(2A) and DC_3C_n7A-n78(2A)

### 6.x.1	Operating bands for DC

Table 6.10.1-1: DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3A_n7A-n78(2A) DC_3C_n7A-n78(2A) | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 6.x.2	Channel bandwidths per operating band for DC

Table 6.x.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_3A_n7A-n78(2A) | DC_3A_n7A DC_3A_n78A | 3 | See CA_3A in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  | 260 |
|  |  | n7 | See CA_n7A-n78(2A) in Table 5.5A.3.1-1 TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3C_n7A-n78(2A) | DC_3C_n7A DC_3A_n7A DC_3C_n78A DC_3A_n78A | 3 | See CA_3C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  | 290 |
|  |  | n7 | See CA_n7A-n78(2A) in Table 5.5A.3.1-1 TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.x.3	Co-existence studies

Co-existence studies of Band 3 + Band n7 + Band n78 already have been captured in current specification for DC_3A_n7A-n78A.

### 6.x.4	∆TIB and ∆RIB values

The TIB,c and RIB values can reuse the same requirement for DC_3A_n7A-n78A.

### 6.x.5	MSD

The MSD requirement can reuse the same requirement for DC_3A_n7A-n78A.

***************************** End of Changes ********************************