3GPP RAN WG4 Meeting #101bis-e		R4-2200444

Online, January 17th – 25th, 2022

Agenda item:	5.36.2.2

Source:	Apple Inc.

Title:	HPUE A-MPR Proposal for NS_05

Release:	Rel-17

Document for:	Approval

# 1	Introduction

During last RAN4 meeting, RAN4#101-e, a new way forward on Tx requirements was agreed [1]. The way forward asks interested companies to provide PC2 A-MPR proposals for certain network signalling. This contribution provides proposals for NS_05 and NS_05U.

# 2  Discussion

###### 2.1 Simulation assumptions

The following assumptions for the simulations were used:

- Power Class 2

- Calibration: 1dB MPR: DFT-s-OFDM QPSK 20MHz, 100RB0

- Carrier Leakage: 28dBc

- Image: 28dBc

- CIM3: 60dBc

- CIM5: 70dBc

- Filter rejection at B3 Rx: 50dB

- Filter rejection at B34 Rx: 11dB

The simulations were done for NS_05 and NS_05U. The spurious co-existence was according to table 1.

Table1: Co-existence for n1

| NR Band | Spurious emission for UE co-existence |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| n1, n84 | E-UTRA Band 1, 5, 7, 8, 11, 18, 19, 20, 21, 22, 26, 27, 28, 31, 32, 38, 40, 41, 42, 43, 44, 45, 50, 51, 52, 65, 67, 68, 69, 72, 73, 74, 75, 76, NR Band n78, n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 3, 34 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 15, 27 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 15, 26, 27 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 15, 26, 27 |

###### 2.2 Simulation results

Simulations have been conducted for all channel bandwidth from 5MHz to 20MHz. The PC3 A-MPR regions were used as basis and adopted to suit the needs of PC2. Selected results can be found in section 3. DFT-s-OFDM and CP-OFDM for QPSK are shown and compared to PC3. The scaling for PC3 and PC2 is same for easy comparison. It can be observed that the power backoff need increases considerable with PC2. The A-MPR RB regions need to be extended accordingly to meet the increased power backoff demands for PC2. Table 2 contains the PC2 RB regions. Differences to PC3 are marked in red. Table 3,4 and 5 provide the A-MPR values for the individual regions.

Table 2: A-MPR regions for NS_05 and NS_05U (Power Class 2)

| Channel Bandwidth (MHz) | Carrier Centre Frequency, Fc (MHz) | Region A |  |  | Region B |  |  | Region C |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | RBstart | LCRB | A-MPR | RBstart | LCRB | A-MPR | RBstart | LCRB | A-MPR |
| 5 | 1922.5 ≤ FC < 1927.5 | < 1.98 MHz/12/SCS | > 1.44 MHz/12/SCS | A3 | < 0.72 MHz/12/SCS | ≤ 1.44 MHz/12/SCS | A4 |  |  |  |
| 10 | 1925 ≤ FC < 1935 | < 1.98 MHz/12/SCS | > 0 | A1 | ≥ 1.98 MHz/12/SCS | > max(0, RBstart-1.08 MHz/12/SCS) | A7 | ≥ 7.2 MHz/12/SCS | ≤ 1.08 MHz/12/SCS | A2 |
| 10 | 1935 ≤ FC < 1945 |  | > 4.5 MHz/12/SCS | A4 |  |  |  |  |  |  |
| 15 | 1927.5 ≤ FC < 1932.5 | < 3.6 MHz/12/SCS | > 0 | A1 | ≥ 3.6 MHz/12/SCS | > max(0, RBstart-1.8 MHz/12/SCS) | A7 | ≥ 10.08 MHz/12/SCS | ≤ 1.08 MHz/12/SCS | A2 |
| 15 | 1932.5 ≤ FC < 1942.5 | < 1.98 MHz/12/SCS | > 0 | A1 | ≥ 1.98 MHz/12/SCS | > max(0, RBstart+1.08 MHz/12/SCS) | A7 | ≥ 12.24 MHz/12/SCS | ≤ 1.08 MHz/12/SCS | A2 |
| 15 | 1942.5 ≤ FC < 1947.5 |  | > 7.2 MHz/12/SCS | A5 |  |  |  |  |  |  |
| 20 | 1930 ≤ FC < 1950 | < 5.04 MHz/12/SCS | > 0 | A1 | ≥ 5.04 MHz/12/SCS | > max(0, RBstart-3.6 MHz/12/SCS) | A7 | ≥ 13.68 MHz/12/SCS | ≤ 1.08 MHz/12/SCS | A2 |
| 20 | 1950 ≤ FC < 1960 |  | > 9.0 MHz/12/SCS | A6 |  |  |  |  |  |  |
| NOTE 1: The A-MPR values are specified in Table 6.2.3.4-b, 6.2.3.4-c and 6.2.3.4-d. NOTE 2: Void |  |  |  |  |  |  |  |  |  |  |

Table 3: A-MPR for NS_05 and NS_05U (Power Class 2)

| Modulation/Waveform |  | A1 (dB) | A2 (dB) | A3 (dB) |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner | Outer | Inner |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 11.5 | ≤ 6.0 | ≤ 5.5 | ≤ 2.0 |
|  | QPSK | ≤ 11.5 | ≤ 6.0 | ≤ 6.0 | ≤ 2.0 |
|  | 16 QAM | ≤ 11.5 | ≤ 6.0 | ≤ 7.0 | ≤ 2.0 |
|  | 64 QAM | ≤ 12.0 | ≤ 6.0 | ≤ 7.0 | ≤ 2.0 |
|  | 256 QAM | ≤ 13.0 | ≤ 6.0 | ≤ 7.0 |  |
| CP-OFDM | QPSK | ≤ 11.5 | ≤ 6.0 | ≤ 8.0 | ≤ 4.0 |
|  | 16 QAM | ≤ 11.5 | ≤ 6.0 | ≤ 8.0 | ≤ 4.0 |
|  | 64 QAM | ≤ 11.5 | ≤ 6.0 | ≤ 8.5 | ≤ 4.0 |
|  | 256 QAM | ≤ 13.0 |  | ≤ 10.0 |  |
| NOTE 1: Void NOTE 2: Void |  |  |  |  |  |

Table 4: A-MPR for NS_05 (Power Class 2)

| Modulation/Waveform |  | A4 (dB) |  | A5 (dB) |  | A6 (dB) |  | A7 (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer | Inner | Outer | Inner | Outer | Inner | Outer/Inner |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 2.5 | N/A | ≤ 2.5 |  | ≤ 2.5 | N/A | ≤ 7.0 |
|  | QPSK | ≤ 2.5 |  | ≤ 2.5 |  | ≤ 2.5 |  | ≤ 7.0 |
|  | 16 QAM | ≤ 3.0 |  | ≤ 2.5 |  | ≤ 2.5 |  | ≤ 7.0 |
|  | 64 QAM | ≤ 3.0 |  |  |  |  |  | ≤ 7.0 |
|  | 256 QAM |  |  |  |  |  |  | ≤ 7.0 |
| CP-OFDM | QPSK | ≤ 4.5 |  | ≤ 4.5 |  | ≤ 4.5 |  | ≤ 7.5 |
|  | 16 QAM | ≤ 4.5 |  | ≤ 4.5 |  | ≤ 4.5 |  | ≤ 7.5 |
|  | 64 QAM | ≤ 5.0 |  | ≤ 5.0 | ≤ 5.0 | ≤ 4.5 |  | ≤ 7.5 |
|  | 256 QAM |  |  |  |  |  |  | ≤ 7.5 |
| NOTE 1: Void NOTE 2: Void |  |  |  |  |  |  |  |  |

Table 6.2.3.4-4 - Table 6.2.3.4-9: Void

Table 5: A-MPR for modulation and waveform type for NS_05U (Power Class 2)

| Modulation/Waveform |  | A5 (dB) |  | A6 (dB) |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Outer | Inner | Outer | Inner |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 2.5 |  | ≤ 2.5 | N/A |
|  | QPSK | ≤ 2.5 |  | ≤ 2.5 |  |
|  | 16 QAM | ≤ 2.5 |  | ≤ 2.5 |  |
|  | 64 QAM | ≤ 3.0 |  | ≤ 3.0 |  |
|  | 256 QAM | ≤ 4.5 |  | ≤ 4.5 |  |
| CP-OFDM | QPSK | ≤ 4.5 |  | ≤ 4.5 |  |
|  | 16 QAM | ≤ 4.5 |  | ≤ 4.5 |  |
|  | 64 QAM | ≤ 5.0 | ≤ 4 | ≤ 4.5 |  |
|  | 256 QAM | ≤ 6.5 |  | ≤ 6.5 |  |
| NOTE 1: Void NOTE 2: Void |  |  |  |  |  |

Proposal: Use regions found in Table 2 for NS_48 and regions found in Table 3 for NS_49

# 3  Simulations

###### 3.1 NS_05 DFT-s-OFDM

| PC3 | PC2 |
| --- | --- |

###### 3.2 NS_05 CP-OFDM

| PC3 | PC2 |
| --- | --- |

# 4	Conclusions

Following the way forward on Tx requirements [1] this contribution provides A-MPR simulation results and proposals for NS_05 and NS_05U with PC2. The following proposal is made:

Proposal: Use regions found in Table 2 for NS_05 and regions found in Table 3,4 and 5.

# 4	References

- R4-2119946, “WF on NR FDD PC2 HPUE“, China Unicom, 3GPP TSG-RAN WG4 Meeting #101-e