3GPP TSG-RAN WG4 Meeting #101-bis-e	R4-2200763

Online Meeting, Jan 17 – 25, 2022

Agenda item:	8.9.6.2.1

Source: 	Qualcomm Incorporated

Title: 	On UE demodulation requirements for 16-QAM NB-IoT

Document for:	Discussion

# Introduction

The Rel-17 WI for Additional enhancements to NB-IoT and LTE-MTC includes the following objectives regarding the introduction of 16 QAM for NB-IoT [1].

- Specify 16-QAM for unicast in UL and DL, including necessary changes to DL power allocation for NPDSCH and DL TBS. This is to be specified without a new NB-IoT UE category. For DL, increase in maximum TBS of e.g. 2x the Rel-16 maximum, and soft buffer size will be specified by modifying at least existing Category NB2. For UL, the maximum TBS is not increased. [NB-IoT] [RAN1, RAN4]

  - Extend the NB-IoT channel quality reporting based on the framework of Rel-14—16, to support 16-QAM in DL. [NB-IoT] [RAN2, RAN1, RAN4]

A work plan for RAN4 was approved in RAN4#99-e [2]. Discussion of demodulation requirements for 16-QAM in NB-IoT is expected to begin in this meeting. In this brief contribution we include proposals regarding simulation assumptions and new tests of NPDSCH demodulation performance.

# Discussion

We have the following proposals regarding the definition and testing of demodulation requirements for 16-QAM in NB-IoT.

Proposal 1: Define minimum requirements for NPDSCH demodulation with 16-QAM for UE category NB2, Half-duplex FDD and TDD. Minimum requirements are defined in a similar way as the existing requirements for QPSK: SNR reference value at 70% of maximum throughput.

Proposal 2: Introduce new tests of NPDSCH demodulation of 16-QAM for UE category NB2, Half-duplex FDD and TDD, in Standalone mode.

Proposal 3: For testing NPDSCH demodulation requirements take the existing test configurations for UE category NB2 in Standalone mode as the baseline for the new 16-QAM demodulation tests

- Use reference channels R.NB.7 FDD and R.NB.7 TDD as the baseline for HD-FDD and TDD, respectively. Change the modulation order to 16-QAM and adjust the coding rate and TBS.

- Reuse test configuration from TS 36.101 Table 8.12.1.1.3-2/8.12.1.2.3-2, as shown below

| Test number | Bandwidth | Carrer Type | Reference Channel | Repetition number | Propagation condition | Number of NRS ports | Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 200kHz | Non-anchor | TBD | 1 | EPA5 | 1 | 1x1 | TBD | TBD | NB2 |

# Conclusions

Proposal 1: Define minimum requirements for NPDSCH demodulation with 16-QAM for UE category NB2, Half-duplex FDD and TDD. Minimum requirements are defined in a similar way as the existing requirements for QPSK: SNR reference value at 70% of maximum throughput.

Proposal 2: Introduce new tests of NPDSCH demodulation of 16-QAM for UE category NB2, Half-duplex FDD and TDD, in Standalone mode.

Proposal 3: For testing NPDSCH demodulation requirements take the existing test configurations for UE category NB2 in Standalone mode as the baseline for the new 16-QAM demodulation tests

- Use reference channels R.NB.7 FDD and R.NB.7 TDD as the baseline for HD-FDD and TDD, respectively. Change the modulation order to 16-QAM and adjust the coding rate and TBS.

- Reuse test configuration from TS 36.101 Table 8.12.1.1.3-2/8.12.1.2.3-2, as shown below

| Test number | Bandwidth | Carrer Type | Reference Channel | Repetition number | Propagation condition | Number of NRS ports | Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 200kHz | Non-anchor | TBD | 1 | EPA5 | 1 | 1x1 | TBD | TBD | NB2 |

# References

[1] RP-211340, WID revision: Additional enhancements for NB-IoT and LTE-MTC, RAN#92-e

[2] R4-2107255, Work plan of Rel-17 enhancements for NB-IoT and LTE-MTC, RAN4#99-e

[3] R1-2112971, LS on channel quality reporting for NB-IoT, RAN1#107-e