3GPP TSG-RAN WG4 Meeting # 101-bis-e 	R4-2200781

# Electronic Meeting, January. 17-25, 2022

Agenda Item:	6.13.2.1

Source: 		Qualcomm Incorporated

Title: 	Coexistence simulation results for TN-NTN

Document for:	Discussion

# Introduction

In RAN4#101-e meeting, NTN co-existence simulation assumptions were discussed and RAN4 agreed to consider the simulation assumptions in [1].  In this paper, we provide our simulation results for TN with NTN and NTN with TN system in all the cases captured in [1] and depicted in Table 1.

Table 1. List of study cases

| No. | Aggressor | Victim |
| --- | --- | --- |
| 1 | TN DL | NTN DL |
| 2 | TN UL | NTN UL |
| 3 | NTN DL | TN DL |
| 4 | NTN UL | TN UL |
| 5 | NTN UL | TN DL |
| 6 | TN DL | NTN UL |

# 2. Discussion

In this document we present our simulation results between TN and NTN for the 6 cases captured in Annex 2 in [1]. Based on the simulation results which is attached to the Annex of this document and summarized in Table 2, we have some observations as following:

Table 2: Summary of required ACIR for all simulation cases

| No. |  | Average T-Loss ACIR [dB] |  |  |  |  |  | Cell edge T-Loss ACIR [dB] |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Environment / Satellite |  | Rural |  |  | Urban |  |  | Rural |  |  | Urban |  |  |
|  |  | GEO @ EL 90 | LEO 600 | LEO 1200 | GEO @ EL 90 | LEO 600 | LEO 1200 | GEO @ EL 90 | LEO 600 | LEO 1200 | GEO @ EL 90 | LEO 600 | LEO 1200 |
| 1 | TN DL to NTN DL Option 1 | 10 | 0 | 0 | 38 | 30 | 30 | 14 | 4 | 4 | 46 | 36 | 36 |
|  | Option 2 |  |  |  | 20 | 12 | 12 |  |  |  | 28 | 16 | 16 |
| 2 | TN UL to NTN UL | 20 | 18 | 20 | 28 | 24 | 28 | 20 | 16 | 18 | NAN | 20 | NAN |
| 3 | NTN DL to TN DL | 10 | 20 | 20 | 0 | 0 | 0 | 16 | 26 | 26 | 0 | 0 | 0 |
| 4 | NTN UL to TN UL | 0 | 0 | 0 | 20 | 20 | 20 | 8 | 8 | 8 | 34 | 34 | 34 |
| 5 | NTN UL to TN DL | 16 | 16 | 16 | 12 | 12 | 12 | 26 | 26 | 26 | 20 | 20 | 20 |
| 6 | TN DL to NTN UL | 26 | 20 | 22 | 40 | 34 | 38 | 26 | 18 | 20 | NAN | NAN | NAN |

Observation 1: Coexistence in case 1, i.e., TN DL to NTN DL, is challenging in urban scenario. This is due to the NTN DL being much weaker than TN DL. In the area outside of the TN clusters where UE could receive TN signals, that UE supporting both TN and NTN functionalities shall access TN rather than NTN Therefore, a minimum isolation distance as simulation assumption is required to evaluate required ACIR in urban scenario.

Observation 2: Based on the simulation results from Case 1, the current BS ACLR requirement will be sufficient to meet the coexistence criteria. The NTN UE ACS requirement can be the same as current TN UE requirement.

Observation 3: In case 2, TN UL and NTN UL can coexist. However, in this scenario, for the GEO and LEO 1200 cases, we observed that cell edge UEs are out of coverage since the UL SINR is less than -10 dB at 5% CDF.

Observation 4: Based on the simulation results from Case 2, the current TN UE ACLR requirement will allow to meet the coexistence criteria.

Observation 5: The interference from NTN to TN in DL is negligible since TN has much stronger link compared to the interference from NTN. Therefore, the NTN and TN in case 3 can coexist.

Observation 6: Based on the simulation results from Case 3, the current TN UE ACS requirement will allow to meet the coexistence criteria. The NTN BS ACLR requirement should be around 26dB.

Observation 7: In case 4, all the NTN UEs use full power so we can see similar performance from the NTN UEs in UL direction.

Observation 8: Based on the simulation results from Case 4 and given that NTN UE should be placed with a minimum isolation distance in urban scenario, the current BS ACS requirement will allow to meet the coexistence criteria. The NTN UE ACLR requirement can be the same as current TN UE requirement.

Observation 9: Based on the simulation results from Case 5, the current TN UE ACS requirement will allow to meet the coexistence criteria. Furthermore, the NTN UE ACLR requirement can be the same as current TN UE requirement.

Observation 10: In case 6, NTN cell edge UEs are out of coverage for the urban case since the UL SINR is less than -10 dB at 5% CDF.

Observation 11: Based on the simulation results from case 6, lower elevation angles for GEO - e.g., 45 degrees - will require a tighter ACIR of about 6 to 7dB. Based on that, the NTN BS ACS requirement should be around 46 dB same as TN BS ACS requirement.

# 3.	Conclusion

In this paper, we provided simulation results between TN and NTN for 6 coexistences scenarios. Based on the results obtained so far, the following proposals can be made:

Proposal 1: To use as simulation assumption the minimum isolation distance in urban scenario in Case 1 and Case 4.

Proposal 2: To use free space path loss propagation model in Case 5 between NTN UE and TN UE because the BS antenna height range in 38.901 RMa or UMa model exceeds the NTN UE antenna heigh which is 1.5 m.

Proposal 3: To not change current requirements for TN BS and TN UE.

Proposal 4: To reuse same requirement of TN UE with NTN UE.

Proposal 5: To consider the lower elevation angles for Case 6 since this represents the worst-case scenario for NTN BS ACS.

Proposal 6: Suggest using 26 dB as NTN BS ACLR.

Proposal 7: Suggest using 46 dB as NTN BS ACS when low EL. i.e., 45 deg, is considered for case 6.

Proposal 8: In summary, the following ACLR and ACS are proposed for NTN BS and UE:

Table 3 Summary of NTN BS and UE ACLR/ACS

| BS ACLR (dB) | 26 |
| --- | --- |
| BS ACS (dB) | 46 |
| UE ACLR (dB) | 30 |
| UE ACS (dB) | 33 (5/10MHz channel BW) |

# 4.	References

- R4-2120671, Simulation assumptions for NTN co-existence study, Samsung, and CATT

# 5.	Annex

Simulation results