3GPP TSG-RAN WG4 Meeting # 101-bis-e                	R4-2200917

Electronic Meeting, 17th - 25th Juanuary 2022

Source: 	Nokia, Nokia Shanghai Bell

Title: 	Discussion on interruption requirements for FR2-2

Agenda item: 	6.16.7.3

Document for: 	Discussion

Introduction

This paper presents Nokia’s view on RRM aspects related to the operation between 52.6 GHz and 71 GHz. This paper discusses the impact of operation above 52.6 GHz on interruption.

Discussion

During the last RAN4 meetings, some agreements were reached for the definition of interruption requirements for FR2-2, but an open point is left regarding extending NR-DC agreements from RAN4#100-e to intra-band CA [6]:

- RAN4 to further discuss whether to extend to NR-DC case the agreements from RAN4 #100-e regarding interruptions at SCell addition/release for intra-band CA, interruptions at Scell activation/deactivation for intra-band CA, interruptions during measurements on deactivated SCC, interruptions due to Active BWP switching, interruptions at NR SRS carrier based switching.

The agreements from RAN4#100 that were not including NR-DC are [3]:

Issue 3-3-2: Interruptions at SCell addition/release (intra-band CA) Agreements: - Update Table 8.2.2.2.1-2 in TS 38.133 with 480/960 kHz subcarrier spacing as below: Interruption duration for SCell addition/release for intra-band CA NR Slot length (ms); Interruption length (slots). 0; 1; 1 + TSMTC_duration * Nslotsubframe,μ. 1; 0.5; 2 + TSMTC_duration * Nslotsubframe,μ. 2; 0.25; 4 + TSMTC_duration * Nslotsubframe,μ. 3; 0.125; 8 + TSMTC_duration * Nslotsubframe,μ. 5; 0.03125; 32 + TSMTC_duration * Nslotsubframe,μ. 6; 0.015625; 64 + TSMTC_duration * Nslotsubframe,μ. NOTE 1: TSMTC_duration measured in subframes is - the longest SMTC duration among all above active serving cells and the SCell being added when one SCell is added; - the longest SMTC duration among all active serving cells in the same band when one SCell is released. NOTE 2: Nslotsubframe,μ is as defined in TS 38.211 [6].. Issue 3-3-3: Interruptions at SCell activation/deactivation (inter-band CA) Agreements: - Update Table 8.2.2.2.2-1 in TS 38.133 with 480/960 kHz subcarrier spacing as below: Interruption length X2 for SCell activation/deactivation for inter-band CA NR Slot length (ms) of victim cell; Interruption length X2 (slots). 0; 1; 1. 1; 0.5; 1. 2; 0.25; Both aggressor cell and victim cell are on FR2; 2. Either aggressor cell or victim cell is on FR1; 3. 3; 0.125; Aggressor cell is on FR2; 4. Aggressor cell is on FR1; 5. 5; 0.03125; Aggressor cell is on FR2-2; 16. Aggressor cell is on FR2-1; FFS. Aggressor cell is on FR1; FFS. 6; 0.015625; Aggressor cell is on FR2-2; 32. Aggressor cell is on FR2-1; FFS. Aggressor cell is on FR1; FFS. Issue 3-3-4: Interruptions at SCell activation/deactivation (intra-band CA) Agreements: - Update Table 8.2.2.2.2-2 in TS 38.133 with 480/960 kHz subcarrier spacing as below: Interruption duration for SCell activation/deactivation for intra-band CA NR Slot length (ms); Interruption length (slots). 0; 1; 1 + TSMTC_duration * Nslotsubframe,μ. 1; 0.5; 1 + TSMTC_duration * Nslotsubframe,μ. 2; 0.25; 2 + TSMTC_duration * Nslotsubframe,μ. 3; 0.125; 4 + TSMTC_duration * Nslotsubframe,μ. 5; 0.03125; 16 + TSMTC_duration * Nslotsubframe,μ. 6; 0.015625; 32 + TSMTC_duration * Nslotsubframe,μ. NOTE 1: TSMTC_duration measured in subframes is - the longest SMTC duration among all above active serving cells and the SCell being added when one SCell is added; - the longest SMTC duration among all active serving cells in the same band when one SCell is released. NOTE 2: Nslotsubframe,μ is as defined in TS 38.211 [6].. Issue 3-3-7: Interruptions due to UE-specific CBW change Agreements: - Update Table 8.2.2.2.1-1 in TS 38.133 with 480/960 kHz subcarrier spacing as below: Interruption length X NR Slot; Interruption length X (slots). length (ms). 0; 1; 1. 1; 0.5; 1. 2; 0.25; 3. 3; 0.125; 5. 5; 0.03125; 17. 6; 0.015625; 33. Issue 3-3-8: Interruptions at NR SRS carrier based switching Agreements: - For Standalone NR Carrier Aggregation, when SRS carrier based switching is performed between carriers, the interruption requirements on any active serving cell can be shown in Table 1 and Table 2: Table 1: Interruption length X1 (slot) NR Slot length; SRS carrier; Interruption length X1 (slots). (ms) of victim cell; switching time (us)Note 1; Sub carrier spacing for agressor cell (kHz). 15; 30. 0; 1; ≤ 200; 2; 2. 300, 500; 2; 2. 900; 2; 2. 1; 0.5; ≤ 200; 3; 2. 300, 500; 3; 3. 900; 4; 4. 2; 0.25; ≤ 200; 4; 3. 300, 500; 5; 4. 900; 7; 6. 3; 0.125; ≤ 200; 7; 5. 300, 500; 9; 7. 900; 12; 10. 5; 0.03125; ≤ 200; 22; 15. 300, 500; 31; 24. 900; 44; 37. 6; 0.015625; ≤ 200; 42; 28. 300, 500; 61; 47. 900; 87; 73. Note1: NR SRS carrier switching time is UE capability indicated by higher layer parameter SRS-SwitchingTimeNR.. Table 2: Interruption length X2 (slot) NR Slot; SRS carrier; Interruption length X2 (slots). length (ms) of victim cell; switching time (us) Note 1; Sub carrier spacing for agressor cell (kHz). 60; 120; 480; 960. 0; 1; ≤ 200; 2; 2; 2; 2. 1; 0.5; ≤ 200; 2; 2; 2; 2. 2; 0.25; ≤ 200; 3; 3; 2; 2. 3; 0.125; ≤ 200; 4; 4; 3; 3. 5; 0.03125; ≤ 200; 11; 10; 8; 8. 6; 0.015625; ≤ 200; 21; 18; 15; 15. Note1: NR SRS carrier switching time is UE capability indicated by higher layer parameter SRS-SwitchingTimeNR..

Which were updated in the last RAN4 #101-e meeting with the following agreements [6]:

- For interruptions at SCell addition/release and activation/deactivation for inter-band CA update Table 8.2.2.2.1-1 and Table 8.2.2.2.2-1 in TS 38.133 with 480/960 kHz subcarrier spacing as below: Table 8.2.2.2.1-1: Interruption length X1 for Scell addition/release for inter-band CA NR Slot length (ms) of victim cell; Interruption length X1 (slots). 0; 1; 1. 1; 0.5; 2. 2; 0.25; Both aggressor cell and victim cell are on FR2; 4. Either aggressor cell or victim cell is on FR1; 5. 3; 0.125; Aggressor cell is on FR2; 8. Aggressor cell is on FR1; 9. 5; 0.03125; Aggressor cell is on FR1; 33. 6; 0.015625; Aggressor cell is on FR1; 65. Table 8.2.2.2.2-1: Interruption length X2 for Scell activation/deactivation for inter-band CA NR Slot length (ms) of victim cell; Interruption length X2 (slots). 0; 1; 1. 1; 0.5; 1. 2; 0.25; Both aggressor cell and victim cell are on FR2; 2. Either aggressor cell or victim cell is on FR1; 3. 3; 0.125; Aggressor cell is on FR2; 4. Aggressor cell is on FR1; 5. 5; 0.03125; Aggressor cell is on FR1; 17. 6; 0.015625; Aggressor cell is on FR1; 33.

Considering that the agreements were reached for CA  requirements, they are incomplete so far, since NR-DC scenario is not covered. For most of the interruption requirements, the same interruption length requirements apply for both NR-DC and CA scenarios. Therefore we propose to reuse the values and organize the points where reusing is possible in the following proposals.

Interruption requirements from RAN4 #100-e did not include NR-DC but mostly CA.

For most of the CA requirements agreed for interruption, the same logic can be reused for NR-DC.

Update interruption duration at NR-DC PSCell/SCell addition/release on Table 8.2.4.2.1-2 to include 480 and 960 SCS as

Table 8.2.4.2.1-2: Interruption duration for SCell addition/release for intra-band DC/CA

|  | NR Slot length (ms) | Interruption length (slots) |
| --- | --- | --- |
| 0 | 1 | 1 + TSMTC_duration * Nslotsubframe,μ |
| 1 | 0.5 | 2 + TSMTC_duration * Nslotsubframe,μ |
| 2 | 0.25 | 4 + TSMTC_duration * Nslotsubframe,μ |
| 3 | 0.125 | 8 + TSMTC_duration * Nslotsubframe,μ |
| 5 | 0.03125 | 32 + TSMTC_duration * Nslotsubframe,μ |
| 6 | 0.015625 | 64 + TSMTC_duration * Nslotsubframe,μ |
| NOTE 1: TSMTC_duration measured in subframes is - the longest SMTC duration among all above activeserving cells and the SCell being added when one SCell is added; - the longest SMTC duration among all active serving cells in the same band when one SCell is released. NOTE 2: Nslotsubframe,μ is as defined in TS 38.211 [6] |  |  |

Update Table 8.2.4.2.2-1 for interband duration in NR-DC for NR-DC inter-band SCell activation/deactivation as

Table 8.2.4.2.2-1: Interruption duration for SCell activation/deactivation for inter-band DC/CA

|  | NR Slot length | Interruption length (slots) |  |  |
| --- | --- | --- | --- | --- |
|  | (ms) of victim cell | Sync |  | Async |
| 0 | 1 | 1 |  | 2 |
| 1 | 0.5 | 1 |  | 2 |
| 2 | 0.25 | Both aggressor cell and victim cell are on FR2 | 2 | 3 |
|  |  | Either aggressor cell or victim cell is on FR1 | 3 |  |
| 3 | 0.125 | Aggressor cell is on FR2 | 4 | 5 |
|  |  | Aggressor cell is on FR1 | 5 |  |
| 5 | 0.03125 | Aggressor cell is on FR1 | 17 | 17 |
| 6 | 0.015625 | Aggressor cell is on FR1 | 33 | 33 |

Update Table 8.2.4.2.2-1 for SCell activation/deactivation interband duration in NR-DC for inter-band SCell activation/deactivation as

Table 8.2.4.2.2-2: Interruption duration for SCell activation/deactivation for intra-band DC/CA

|  | NR Slot length (ms) | Interruption length (slots) |
| --- | --- | --- |
| 0 | 1 | 1 + TSMTC_duration * Nslotsubframe,μ |
| 1 | 0.5 | 1 + TSMTC_duration * Nslotsubframe,μ |
| 2 | 0.25 | 2 + TSMTC_duration * Nslotsubframe,μ |
| 3 | 0.125 | 4 + TSMTC_duration * Nslotsubframe,μ |
| 5 | 0.03125 | 16 + TSMTC_duration * Nslotsubframe,μ |
| 6 | 0.015625 | 32 + TSMTC_duration * Nslotsubframe,μ |
| NOTE 1: TSMTC_duration measured in subframes is - the longest SMTC duration among all above active serving cells and the SCell being activated when one SCell is activated; - the longest SMTC duration among all active serving cells in the same band when one SCell is deactivated. NOTE 2: Nslotsubframe,μ is as defined in TS 38.211 [6]. |  |  |

Update interruption SRS carrier based switching requirements for NR-DC in Table 8.2.4.2.9-1 as

Table 8.2.4.2.9-1: Interruption length X1 (slot)

|  | NR Slot length | SRS carrier | Interruption length X1 (slots) |  |
| --- | --- | --- | --- | --- |
|  | (ms) of victim cell | switching time (us)Note 1 | Sub carrier spacing for agressor cell (kHz) |  |
|  |  |  | 15 | 30 |
| 0 | 1 | ≤ 200 | 2 | 2 |
|  |  | 300, 500 | 2 | 2 |
|  |  | 900 | 3 | 3 |
| 1 | 0.5 | ≤ 200 | 3 | 2 |
|  |  | 300, 500 | 3 | 3 |
|  |  | 900 | 4 | 4 |
| 2 | 0.25 | ≤ 200 | 4 | 3 |
|  |  | 300, 500 | 5 | 4 |
|  |  | 900 | 7 | 6 |
| 3 | 0.125 | ≤ 200 | 7 | 5 |
|  |  | 300, 500 | 9 | 7 |
|  |  | 900 | 12 | 10 |
| 5 | 0.03125 | ≤ 200 | 22 | 15 |
|  |  | 300, 500 | 31 | 24 |
|  |  | 900 | 44 | 37 |
| 6 | 0.015625 | ≤ 200 | 42 | 28 |
|  |  | 300, 500 | 61 | 47 |
|  |  | 900 | 87 | 73 |
| Note1: NR SRS carrier switching time is UE capability indicated by higher layer parameter SRS-SwitchingTimeNR. |  |  |  |  |

Table 8.2.4.2.9-2: Interruption length X2 (slot)

|  | NR Slot | SRS carrie | Interruption length X2 (slots) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | length (ms) of victim cell | switching time (us) Note 1 | Sub carrier spacing for agressor cell (kHz) |  |  |  |
|  |  |  | 60 | 120 | 480 | 960 |
| 0 | 1 | ≤ 200 | 2 | 2 | 2 | 2 |
| 1 | 0.5 | ≤ 200 | 2 | 2 | 2 | 2 |
| 2 | 0.25 | ≤ 200 | 3 | 3 | 2 | 2 |
| 3 | 0.125 | ≤ 200 | 4 | 4 | 3 | 3 |
| 5 | 0.03125 | ≤ 200 | 11 | 10 | 8 | 8 |
| 6 | 0.015625 | ≤ 200 | 21 | 18 | 15 | 15 |
| Note1: NR SRS carrier switching time is UE capability indicated by higher layer parameter SRS-SwitchingTimeNR. |  |  |  |  |  |  |

Conclusion

This paper has presented Nokia’s views on RRM requirements for the extension to 71 GHz. From this discussion we have derived the following observations and proposals:

- Interruption requirements from RAN4 #100-e did not include NR-DC but mostly CA.

For most of the CA requirements agreed for interruption, the same logic can be reused for NR-DC.

- Update interruption duration at NR-DC PSCell/SCell addition/release on Table 8.2.4.2.1-2 to include 480 and 960 SCS as

Table 8.2.4.2.1-2: Interruption duration for SCell addition/release for intra-band DC/CA

|  | NR Slot length (ms) | Interruption length (slots) |
| --- | --- | --- |
| 0 | 1 | 1 + TSMTC_duration * Nslotsubframe,μ |
| 1 | 0.5 | 2 + TSMTC_duration * Nslotsubframe,μ |
| 2 | 0.25 | 4 + TSMTC_duration * Nslotsubframe,μ |
| 3 | 0.125 | 8 + TSMTC_duration * Nslotsubframe,μ |
| 5 | 0.03125 | 32 + TSMTC_duration * Nslotsubframe,μ |
| 6 | 0.015625 | 64 + TSMTC_duration * Nslotsubframe,μ |
| NOTE 1: TSMTC_duration measured in subframes is - the longest SMTC duration among all above activeserving cells and the SCell being added when one SCell is added; - the longest SMTC duration among all active serving cells in the same band when one SCell is released. NOTE 2: Nslotsubframe,μ is as defined in TS 38.211 [6] |  |  |

Update Table 8.2.4.2.2-1 for interband duration in NR-DC for NR-DC inter-band SCell activation/deactivation as

Table 8.2.4.2.2-1: Interruption duration for SCell activation/deactivation for inter-band DC/CA

|  | NR Slot length | Interruption length (slots) |  |  |
| --- | --- | --- | --- | --- |
|  | (ms) of victim cell | Sync |  | Async |
| 0 | 1 | 1 |  | 2 |
| 1 | 0.5 | 1 |  | 2 |
| 2 | 0.25 | Both aggressor cell and victim cell are on FR2 | 2 | 3 |
|  |  | Either aggressor cell or victim cell is on FR1 | 3 |  |
| 3 | 0.125 | Aggressor cell is on FR2 | 4 | 5 |
|  |  | Aggressor cell is on FR1 | 5 |  |
| 5 | 0.03125 | Aggressor cell is on FR1 | 17 | 17 |
| 6 | 0.015625 | Aggressor cell is on FR1 | 33 | 33 |

Update Table 8.2.4.2.2-1 for SCell activation/deactivation interband duration in NR-DC for inter-band SCell activation/deactivation as

Table 8.2.4.2.2-2: Interruption duration for SCell activation/deactivation for intra-band DC/CA

|  | NR Slot length (ms) | Interruption length (slots) |
| --- | --- | --- |
| 0 | 1 | 1 + TSMTC_duration * Nslotsubframe,μ |
| 1 | 0.5 | 1 + TSMTC_duration * Nslotsubframe,μ |
| 2 | 0.25 | 2 + TSMTC_duration * Nslotsubframe,μ |
| 3 | 0.125 | 4 + TSMTC_duration * Nslotsubframe,μ |
| 5 | 0.03125 | 16 + TSMTC_duration * Nslotsubframe,μ |
| 6 | 0.015625 | 32 + TSMTC_duration * Nslotsubframe,μ |
| NOTE 1: TSMTC_duration measured in subframes is - the longest SMTC duration among all above active serving cells and the SCell being activated when one SCell is activated; - the longest SMTC duration among all active serving cells in the same band when one SCell is deactivated. NOTE 2: Nslotsubframe,μ is as defined in TS 38.211 [6]. |  |  |

Update interruption SRS carrier based switching requirements for NR-DC in Table 8.2.4.2.9-1 as

Table 8.2.4.2.9-1: Interruption length X1 (slot)

|  | NR Slot length | SRS carrier | Interruption length X1 (slots) |  |
| --- | --- | --- | --- | --- |
|  | (ms) of victim cell | switching time (us)Note 1 | Sub carrier spacing for agressor cell (kHz) |  |
|  |  |  | 15 | 30 |
| 0 | 1 | ≤ 200 | 2 | 2 |
|  |  | 300, 500 | 2 | 2 |
|  |  | 900 | 3 | 3 |
| 1 | 0.5 | ≤ 200 | 3 | 2 |
|  |  | 300, 500 | 3 | 3 |
|  |  | 900 | 4 | 4 |
| 2 | 0.25 | ≤ 200 | 4 | 3 |
|  |  | 300, 500 | 5 | 4 |
|  |  | 900 | 7 | 6 |
| 3 | 0.125 | ≤ 200 | 7 | 5 |
|  |  | 300, 500 | 9 | 7 |
|  |  | 900 | 12 | 10 |
| 5 | 0.03125 | ≤ 200 | 22 | 15 |
|  |  | 300, 500 | 31 | 24 |
|  |  | 900 | 44 | 37 |
| 6 | 0.015625 | ≤ 200 | 42 | 28 |
|  |  | 300, 500 | 61 | 47 |
|  |  | 900 | 87 | 73 |
| Note1: NR SRS carrier switching time is UE capability indicated by higher layer parameter SRS-SwitchingTimeNR. |  |  |  |  |

Table 8.2.4.2.9-2: Interruption length X2 (slot)

|  | NR Slot | SRS carrie | Interruption length X2 (slots) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | length (ms) of victim cell | switching time (us) Note 1 | Sub carrier spacing for agressor cell (kHz) |  |  |  |
|  |  |  | 60 | 120 | 480 | 960 |
| 0 | 1 | ≤ 200 | 2 | 2 | 2 | 2 |
| 1 | 0.5 | ≤ 200 | 2 | 2 | 2 | 2 |
| 2 | 0.25 | ≤ 200 | 3 | 3 | 2 | 2 |
| 3 | 0.125 | ≤ 200 | 4 | 4 | 3 | 3 |
| 5 | 0.03125 | ≤ 200 | 11 | 10 | 8 | 8 |
| 6 | 0.015625 | ≤ 200 | 21 | 18 | 15 | 15 |
| Note1: NR SRS carrier switching time is UE capability indicated by higher layer parameter SRS-SwitchingTimeNR. |  |  |  |  |  |  |

References

- R4-2108353, Work plan for NR extension to 71 GHz - RRM, Qualcomm

- R4-2108354, WF on NR extension to 71 GHz - RRM, Qualcomm

- R4-2115351, WF on NR extension to 71 GHz – RRM - Part 1, Qualcomm

- R4-2115352, WF on NR extension to 71 GHz – RRM - Part 2, Intel

- R4-2120316, WF on NR extension to 71 GHz RRM requirements (Part 1), Qualcomm

- R4-2120317, WF on NR extension to 71 GHz RRM requirements (Part 2), Intel

- 3GPP TS 38.133: “NR; Requirements for support of radio resource management”

- R4-2113222, “Discussion on interruption requirements for extension to 71 GHz”, Nokia

- TS 38.213, NR; Physical layer procedures for control

- TS 38.211, NR; Physical channels and modulation

- 3GPP TR 38.808 v17.0.0, Study on supporting NR from 52.6 GHz to 71 GHz