3GPP TSG-RAN WG4 Meeting #101-bis-e	R4-2201332

# Electronic meeting, January 17-25, 2022

Source:		Ericsson

Title:			6 GHz licensed band – UE RF

Agenda item:		5.3.1

Document for:		Approval

# Introduction

The WI ([1]) on introducing new licensed band(s) in the 6 GHz was on hold, waiting for inputs from Regulators to start the work.

In last RAN#94-e meeting, RAN received a LS ([2]) from RCC informing 3GPP of the approval of “RCC Recommendation 1/21 Harmonization of the technical conditions for 5G-NR/IMT-2020 systems in the RCC countries in the frequency band 6 425-7 125 MHz or in its portions which contains the required regulatory requirements”.

It was then agreed to immediately start the specification work of this new 6GHz licensed band, answering RCC request.

This document is discussing this new 6GHz licensed band requirements, focusing on the UE RF requirements.

# Discussion

## System study on 6.425-7.125 GHz

In the scope of WRC-23, ITU-R WP5D requested the parameters for the 6.425-7.125 GHz and 10.0-10.5 GHz frequency ranges. RAN4 ran then a study and answered ITU-R with the requested parameters. The outcomes of this study have been captured in a TR ([3]) and could be used as input for the specification of this new 6GHz licensed band, together with the parameters specified by RCC Recommendation 1/21 ([2]).

## Tx requirements

In following Table 1, each UE Tx requirement is analyzed, suggesting corresponding limits or way forward for the new 6GHz licensed band.

| Requirement | Suggested limits |
| --- | --- |
| Maximum output power | RCC Recommendation 1/21 specifies that “the power of 5G-NR/IMT-2020 base stations and user equipment should not exceed the limits established by Article 21 of the Radio Regulations for the radio frequency band 6425-7125” Radio Regulations Article 21 doesn’t mention any output power restriction specific to the 6.425-7.125 GHz range for mobile service, the limit for bands above 1 GHz should then be applicable. Requirement specified in TS 38.101-1 sub-clause 6.2.1 shall also apply for the new 6GHz licensed band. |
| MPR | Even for 100MHz channel BW, the relative channel BW for this new band is less than 4%, no ∆MPR will be needed. |
| A-MPR | To be further studied. |
| Output power dynamics | Result of the study was captured in TR 38.921, sub-clause 7.1.1. Requirement specified in TS 38.101-1 sub-clause 6.3 shall also apply for the new 6GHz licensed band. |
| Transmit signal quality | This requirement is not depending on the considered frequency range. Requirement specified in TS 38.101-1 sub-clause 6.4 shall also apply for the new 6GHz licensed band. |
| Occupied bandwidth | This requirement is not depending on the considered frequency range. Requirement specified in TS 38.101-1 sub-clause 6.5.1 shall also apply for the new 6GHz licensed band. |
| Out of band emission – SEM | From the study done on 6.425-7.125GHz, an updated SEM has been agreed. See TR 38.921 sub-clause 7.1.2 |
| Out of band emission – ACLR | Based on the coexistence study, an ACLR value of 26dB should be specified. See TR 38.921 sub-clause 7.1.3 |
| Spurious emission - general | RCC Recommendation 1/21 specifies that “the unwanted emissions of 5G-NR/IMT-2020 base stations and user equipment in the spurious emissions domain shall be in compliance with the Category B limits for stations in the mobile service, as provided in the latest version of Recommendation ITU-R SM.329”. The spurious requirements in TS 38.101-1 are aligned with ITU-R SM.329. Result of the study was captured in TR 38.921, sub-clause 7.1.4. Requirement specified in TS 38.101-1 sub-clause 6.5.3.1 shall also apply for the new 6GHz licensed band. |
| Spurious emission - coexistence | The new 6GHz licensed band shall be added, considering coexistence with the other existing LTE/NR bands. |
| Transmit intermodulation | This requirement is not depending on the considered frequency range. Requirement specified in TS 38.101-1 sub-clause 6.5.4 shall also apply for the new 6GHz licensed band. |

Table 1: UE Tx requirement for 6.425-7.125GHz licensed band

## Rx requirements

In following Table 2, each UE Rx requirement is analyzed, suggesting corresponding limits or way forward for the new 6GHz licensed band.

| Requirement | Suggested limits |
| --- | --- |
| Reference sensitivity | The UE noise figure for 6.425-7.125 GHz has been discussed, the UE noise figure shall be in the [9, 13] dB range. Further discussion would be needed to finalize the value and investigate REFSENS value for this new band. FFS |
| Maximum input level | This requirement is not depending on the considered frequency range. Requirement specified in TS 38.101-1 sub-clause 6.3 shall also apply for the new 6GHz licensed band. |
| ACS | Based on the coexistence study, an ACS value of 32dBc should be specified. See TR 38.921 sub-clause 7.2.4 |
| In-band blocking | The conclusion of the study on 6.425-7.125GHz was that in-band blocking requirements for frequency larger than 3300 kHz could be applied, see TR 38.921 sub-clause 7.2.3 Requirement specified in TS 38.101-1 sub-clause 7.6.2 shall also apply for the new 6GHz licensed band. |
| Out-of-band blocking | The conclusion of the study on 6.425-7.125GHz was that in-band blocking requirements for frequency larger than 3300 kHz could be applied, see TR 38.921 sub-clause 7.2.3 Requirement specified in TS 38.101-1 sub-clause 7.6.3 shall also apply for the new 6GHz licensed band. |
| Narrow band blocking | The conclusion of the study on 6.425-7.125GHz was that in-band blocking requirements for frequency larger than 3300 kHz could be applied, see TR 38.921 sub-clause 7.2.3 Requirement specified in TS 38.101-1 sub-clause 7.6.4 shall also apply for the new 6GHz licensed band. |
| Spurious response | Requirement specified for frequency larger than 3300kHz in TS 38.101-1 sub-clause 7.6.4 shall also apply for the new 6GHz licensed band. |
| Intermodulation | This requirement is not depending on the considered frequency range. Requirement specified in TS 38.101-1 sub-clause 7.8 shall also apply for the new 6GHz licensed band. |
| Spurious emissions | Requirement specified in TS 38.101-1 sub-clause 7.9 shall also apply for the new 6GHz licensed band. |

Table 2: UE Rx requirement for 6.425-7.125GHz licensed band

# Conclusion

In this contribution, we studied the UE RF requirements for the new 6.425 – 7.125 GHz  licensed band and made following observation and proposal.

Observation: Most of the UE requirements are either already applicable or agreed for the new licensed 6.425 – 7.125 GHz band.

Proposal: Specify UE RF requirements based on the detailed analysis from Table 1 and Table 2.

# References

- RP-213696, WI Introduction of 6GHz NR licensed bands, Huawei

- RP-213605, “LS on inclusion of the 6425-7125 MHz frequency band in the 3GPP specification for 5G-NR/IMT-2000 systems”, Regional Commonwealth in the field of Communication (RCC).

- TR 38.921, Study on International Mobile Telecommunications (IMT) parameters for 6.425 - 7.025 GHz, 7.025 - 7.125 GHz and 10.0 - 10.5 GHz