3GPP TSG-RAN WG4 Meeting #101-bis-e	R4-2201550

Electronic Meeting, 17 January – 25 January 2022

Source:	Ericsson

Title:	TP for TR 38.717-04-01: CA_n41-n66-n70-n78

Agenda item:			5.10.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 38.717-04-01 to include CA_n41A-n66A-n70A-n78A as defined in WID [1].

# 2. Text Proposal

# ---Start of changes---

## 5.x	CA_n41-n66-n70-n78

### 5.x.1	Channel bandwidths per operating bands for CA

Table 5.x.1-1: Supported channel bandwidths per CA configuration for 4DL inter-band CA

| NR CA configuration | Uplink CA configuration | NR Band | Channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |  |  |  | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n41A-n66A-n70A-n78A | - | n41 |  | 10 | 15 | 20 |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | 0 |
|  |  | n66 |  | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  | n70 | 5 | 10 | 15 | 20 | 25 |  |  |  |  |  |  |  |  |  |
|  |  | n78 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |

### 5.x.2	∆TIB,c and ∆RIB,c values

For CA_n41-n66-n70-n78 the TIB,c and RIB,c  values are shown in table 5.x.2-1 and  table 5.x.2-2, respectively. Values are derived from DC_1-3-41_n78.

Table 5.x.2-1: ΔTIB,c for 4DL aggregation

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n41-n66-n70-n78 | n41 | 0.5 |
|  | n66 | 0.6 |
|  | n70 | 0.6 |
|  | n78 | 0.8 |

Table 5.x.2-2: ΔRIB,c for 4DL aggregation

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n41-n66-n70-n78 | n41 | 0 |
|  | n66 | 0.2 |
|  | n70 | 0.2 |
|  | n78 | 0.5 |

### 5.x.3	REFSENS requirements

MSD requirements are captured in lower order combinations.

---End of changes---

# Reference

[1] 		RP-212902, “Rel-17 NR inter-band Carrier Aggregation for 4 bands DL with 1 band UL”, Ericsson