3GPP TSG-RAN WG4 Meeting #101-bis-e	R4-2201566

Electronic Meeting, 17 January – 25 January 2022

Source:	Ericsson, BT plc

Title:	TP for TR 38.717-03-01 to include CA_n1-n20-n67

Agenda Item:	5.9.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 38.717-03-01 to include CA_n1A-n20A-n67A.

# 2. Text Proposal

# ---Start of changes---

## 6.X	CA_n1-n20-n67

### 6.X.1	Operating bands for CA

Table 6.X.1-1: 3DL Inter-band CA operating bands

| NR CA Band | NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n1-n20-n67 | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
|  | n67 | N/A |  |  | 738 MHz |  | 758 MHz | SDL |

### 6.X.2	Channel bandwidths per operating band for CA

Table 6.X.2-1: Supported channel bandwidths per CA configuration for 3DL inter-band CA

| NR CA configuration | Uplink CA configuration | NR Band | Channel bandwidth (MHz) (NOTE 3) |  |  |  |  |  |  |  |  |  |  |  |  | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n1A-n20A-n67A | CA_n1A-n20A | n1 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  | 0 |
|  |  | n20 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
|  |  | n67 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |

### 6.X.3	Co-existence studies

Table 6.x.3-1 gives harmonic issue for the 3DL bands CA with 1 UL. No issues can be seen.

Table 6.x.3-1: Harmonic Interference for 3DLs/1UL

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n1 | 1920 | 1980 | 2110 | 2170 | 3840 | 3960 | 5760 | 5940 | 7680 | 7920 |
| n20 | 832 | 862 | 791 | 821 | 1664 | 1724 | 2496 | 2586 | 3328 | 3448 |
| n67 | N/A | N/A | 738 | 758 | N/A | N/A | N/A | N/A | N/A | N/A |

Table 6.x.3-2 gives harmonic mixing issue for the 3DL bands CA with 1 UL. It can be seen that 3rd harmonic mixing of n20 might affect DL n1.

Table 6.x.3-2 Harmonic mixing for 3DLs/1UL

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n1 | 1920 | 1980 | 2110 | 2170 | 4220 | 4340 | 6330 | 6510 | 8440 | 8680 |
| n20 | 832 | 862 | 791 | 821 | 1582 | 1456 | 2151 | 2184 | 2868 | 2912 |
| n67 | N/A | N/A | 738 | 758 | 1476 | 1516 | 2214 | 2274 | 2952 | 3032 |

    - 6.X.4	∆TIB and ∆RIB values

For three simultaneous DLs and one UL of Band n41, n70 and n78, the TIB,c and RIB,c  values are shown in table 6.1.x.4-1 and  table 6.1.x.4-2, respectively. Values are derived from DC_1-20_n28.

Table 6.x.4-1: ΔTIB,c for 3DL aggregation

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n1-n20-n67 | n1 | 0.5 |
|  | n20 | 0.6 |
|  | n67 | 0.8 |

Table 6.x.4-2: ΔRIB,c for 3DL aggregation

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n1-n20-n67 | n1 | 0 |
|  | n20 | 0.2 |
|  | n67 | 0.2 |

### 6.X.5	MSD

The 3rd  harmonic mixing from n20 UL into n1 DL is addressed in lower order combination.

---End of changes---

# Reference

- RP-213299 TR 38.717-03-01, Rel-17 NR inter-band Carrier Aggregation for 3 bands DL with 1 band UL,v0.4.0