3GPP TSG-RAN WG4 Meeting #101bis-e                                                     R4-2201593

Electronic Meeting, 17-25 Jan., 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR |  | rev |  | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS38.101-1: Introduction of PC2 contiguous ULCA MPR for 2Tx |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Skyworks Solutions Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RF_FR1_enh-Core |  |  |  |  |  |  | Date: |  | 2022-01-15 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - Correction of DualPA IE signaling aspects - Introduction of TxD based PC2 2Tx delta MPR for 1LO case consistent with the introduction of the same configuration with additional UL MIMO |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Correct the restriction to dualPA-Architecture IE absent and add applicability of uplinkTxDC-TwoCarrierReport-r16 to bandwidth class C 2. Introduction of delta MPR for 2Tx in tables |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistency in signaling options and MPR for contiguous ULCA with or without UL MIMO |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2A.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS38.521-1 |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | These changes are related to proposals in R4-2200497 and R4-2200495 |  |  |  |  |  |  |  |  |

| This CR's revision history: |  |
| --- | --- |

## << Start of change >>

### 6.2A.2	UE maximum output power reduction for CA

#### 6.2A.2.1	UE maximum output power reduction for Intra-band contiguous CA

For intra-band contiguous carrier aggregation the allowed Maximum Power Reduction (MPR) for the maximum output power in Table 6.2A.1.1-1 with contiguous RB allocation is specified in Table 6.2A.2.1-1 for UE power class 3 CA bandwidth classes B and C. The MPR with contiguous RB allocation is specified in Table 6.2A.2.1-1a for power class 2 CA bandwidth classes B and C when the signalling is absent for dualPA-Architecture IE and CA bandwidth classes C when uplinkTxDC-TwoCarrierReport-r16 is reported. Additional MPR is applicable when TxD is reported.

In case the modulation format is different on different component carriers then the MPR is determined by the rules applied to higher order of those modulations.

Unless otherwise specified, pi/2 BPSK in following A-MPR tables refers to both variants of pi/2 BPSK referenced in 6.2.2 tables 6.2.2-1.

Table 6.2A.2.1-1: Contiguous RB allocation for Power Class 3

| Modulation |  | MPR for bandwidth class B(dB) |  | MPR for bandwidth class C(dB) |  |
| --- | --- | --- | --- | --- | --- |
|  |  | inner | outer | inner | outer |
| DFT-s-OFDM | Pi/2 BPSK | 1.0 | 3.5 | 2.5 | 7 |
|  | QPSK | 1.0 | 3.5 | 2.5 | 7 |
|  | 16QAM | 1.5 | 3.5 | 2.5 | 7 |
|  | 64QAM | 3.0 | 4.0 | 5 | 7 |
|  | 256QAM | 5.5 | 6.0 | 7 | 7.5 |
| CP-OFDM | QPSK | 2.0 | 4.0 | 3.5 | 8 |
|  | 16QAM | 2.5 | 4.0 | 3.5 | 8 |
|  | 64QAM | 3.5 | 4.0 | 5 | 8 |
|  | 256QAM | 6.5 | 6.5 | 7 | 8 |

Table 6.2A.2.1-1a: Contiguous RB allocation for Power Class 2

| Modulation |  | MPR for bandwidth class B(dB) |  | MPR for bandwidth class C(dB) |  |
| --- | --- | --- | --- | --- | --- |
|  |  | inner | Outer1 | inner | outer |
| DFT-s-OFDM | Pi/2 BPSK | 2.0 | 4.01 | 2.5 | 7 |
|  | QPSK | 2.0 | 4.01 | 2.5 | 7 |
|  | 16QAM | 2.5 | 4.01 | 2.5 | 7 |
|  | 64QAM | 3.0 | 4.51 | 5 | 7 |
|  | 256QAM | 5.5 | 6.0 | 7 | 7.5 |
| CP-OFDM | QPSK | 2.5 | 5.01 | 3.5 | 8 |
|  | 16QAM | 3.0 | 5.01 | 3.5 | 8 |
|  | 64QAM | 3.5 | 5.01 | 5 | 8 |
|  | 256QAM | 6.5 | 6.5 | 7 | 8 |
| NOTE 1: When 1 RB or 2 RB are allocated at the lower edge of lowest CC or upper edge of upper CC, MPR for outer is 5.5 dB. NOTE 2: When TxD is reported, an additional MPR of 0.5 dB for CP-OFDM and 1 dB for DFT-s-OFDM is applicable. |  |  |  |  |  |

For CA bandwidth class B and bandwidth class C with contiguous RB allocation, the following parameters are defined to specify valid RB allocation ranges for Inner and Outer RB allocations:

An RB allocation is contiguous if LCRB1 = 0 or LCRB2 = 0 or (LCRB1  0 and LCRB2  0 and RBStart1 + LCRB1 = NRB1 and RBStart2 = 0), where RBStart1, LCRB1, and NRB1 are for CC1, RBStart2, LCRB2, and NRB2 are for CC2, CC1 is the component carrier with lower frequency.

In contiguous CA, a contiguous allocation is an inner allocation if

RBStart,Low  ≤  RBStart_CA  ≤  RBStart,High, and NRB_alloc  ≤  ceil(NRB,agg /2),

where

RBStart,Low = max(1, floor(NRB_alloc /2))

RBStart,High = NRB,agg – RBStart,Low – NRB,alloc,

with

NRB_alloc= LCRB1 ∙ 2^µ1 + LCRB2 ∙ 2^µ2

NRB_alloc= (NRB1 - RBStart1)∙ 2^µ1 + (RBStart2 + LCRB2 ) ∙ 2^µ2,

NRB,agg=NRB1∙2^µ1+ NRB2∙2^µ2.

If LCRB1 =0, RBStart_CA = NRB1∙2^µ1+ RBStart2∙2^µ2,

if LCRB1 > 0, RBStart_CA = RBStart1∙2^µ1.

A contiguous allocation that is not an Inner contiguous allocation is an Outer contiguous allocation.

For intra-band contiguous carrier aggregation the allowed Maximum Power Reduction (MPR) for the maximum output power in Table Table 6.2A.1.1-1 with non-contiguous RB allocation is specified in Table 6.2A.2.1-2 for UE power class 3 CA bandwidth classes B and C. The MPR with non-contiguous RB allocation is specified in Table 6.2A.2.1-2a for power class 2 CA bandwidth classes B and C when the signalling is absent for dualPA-Architecture IE.

Table 6.2A.2.1-2: non-contiguous RB allocation for Power Class 3

| Modulation |  | MPR for bandwidth class B(dB) |  |  | MPR for bandwidth class C(dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | inner | Outer11 | Outer22 | inner | Outer11 | Outer22 |
| DFT-s-OFDM | Pi/2 BPSK | 2 | 5.5 | 11.5 | 2.5 | 6 | 13 |
|  | QPSK | 2 | 5.5 |  | 2.5 | 6 |  |
|  | 16QAM | 2.5 | 5.5 |  | 3 | 6 |  |
|  | 64QAM | 4.5 | 6 |  | 5 | 6 |  |
|  | 256QAM | 6 | 6.5 |  | 6.5 | 6.5 |  |
| CP-OFDM | QPSK | 2.5 | 6.5 | 12 | 3.5 | 7 | 14 |
|  | 16QAM | 3 | 7 |  | 3.5 | 7 |  |
|  | 64QAM | 5 | 7 |  | 5 | 7 |  |
|  | 256QAM | 7.5 | 7.5 |  | 7.5 | 7.5 |  |
| NOTE 1: Outer 1 MPR for Pi/2 BPSK and QPSK is reduced by 2dB for aggregated allocation bandwidth > 10MHz NOTE 2: Outer 2 MPR is reduced by 4.5dB for aggregated allocation bandwidth > 10MHz |  |  |  |  |  |  |  |

Table 6.2A.2.1-3: non-contiguous RB allocation for Power Class 2

| Modulation |  | MPR for bandwidth class B(dB) |  |  | MPR for bandwidth class C(dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | inner | Outer12 | Outer23 | Inner | Outer12 | Outer23 |
| DFT-s-OFDM | Pi/2 BPSK | 31 | 6.5 | 13 | 31 | 7.5 | 13.5 |
|  | QPSK | 31 | 6.5 |  | 31 | 7.5 |  |
|  | 16QAM | 31 | 6.5 |  | 31 | 7.5 |  |
|  | 64QAM | 5 | 6.5 |  | 5 | 7.5 |  |
|  | 256QAM | 6.5 | 7 |  | 6.5 | 7.5 |  |
| CP-OFDM | QPSK | 3.51 | 7 | 14 | 3.51 | 8 | 14.5 |
|  | 16QAM | 3.51 | 7 |  | 3.51 | 8 |  |
|  | 64QAM | 5 | 7 |  | 5 | 8 |  |
|  | 256QAM | 7.5 | 7.5 |  | 7.5 | 8 |  |
| NOTE 1: the allowed MPR is [4]dB for aggregated allocation bandwidth < [2MHz]. NOTE 2: Outer 1 MPR for Pi/2 BPSK and QPSK is reduced by 2dB for aggregated allocation bandwidth > 10MHz NOTE 3: Outer 2 MPR is reduced by 4.5dB for aggregated allocation bandwidth > 10MHz NOTE 4: When TxD is reported, an additional MPR of 0.5 dB for CP-OFDM inner and 1 dB for CP-OFDM outer 1 and 2 or DFT-s-OFDM is applicable. |  |  |  |  |  |  |  |

## << End change >>