3GPP TSG-RAN WG4 Meeting #101-bis-e		R4- 2201789

E-meeting, January 17-25, 2022

Agenda item:	6.16.7.4

Source:	Intel Corporation

Title:	Discussion on BWP switching delay for NR 52.6 – 71 GHz

Document for:	Discussion

# Introduction

In this paper we discuss the requirements for cross-carrier active BWP switching delay for new numerologies defined for NR 52.6 – 71 GHz operation.

# Discussion

During the RAN4 #101-e meeting the following was captured in the WF [1]

- Cross-carrier active BWP switching - MRTD value should be considered for BWP switching delay definition in cross-carrier scheduling case. - RAN4 to further discuss how to define requirements for cross-carrier BWP switching considering the following questions: - How to account MRTD in cross-carrier BWP switching delay: - Option 1: Several slots according to the MRTD length - Option 2: 1 slot to reserve misalignment in case of asynchronous between two carriers - Other options are not precluded - How to consider additional margin for cross-carrier scheduling - Option 1: Any option of previous question covers the margin as ceiling to the integer number provides additional time for cross-carrier processing - Option 2: 1 slot of 120 kHz when both scheduling carrier and scheduled carrier are in FR2-2 (aligned with Option 2 of the previous question) - Other options are not precluded - How to consider different SCS between scheduling cell and scheduled cell for cross-carrier BWP switching delay: - Option 1: the delay requirements to be defined considering the SCS of scheduled cell - Option 2: keep current assumption which says “TBWPswitchDelay + Y shall follow the smaller SCS of scheduling cell, scheduled cells before and scheduled cells after active BWP change”

In order to take into account full MRTD length, cross-carrier BWP switching delay need to be extended by

Y = ⌈MRTD/Tslot⌉

slots. Following the existing procedure, Y should be defined based on the smaller SCS among all the involved numerologies. Let’s consider possible scenarios for Y value

- FR2-2 + FR2-2.

In this scenario the MRTD value is expected to be 0.26us. This value is much smaller than the slot length of any numerology (MRTD < Tslot), so value Y will always be equal to 1

- FR1 + FR2-2

For this scenario the smaller SCS among all involved numerologies will be FR1 SCS. The slot length of any FR1 numerology is larger than any expected FR1 – FR2-2 MRTD (MRTD < Tslot), so value Y will always be equal to 1.

Based on the above, Option 1 and Option 2 in the first question raised in the WF lead to the same result – 1 slot is enough to account MRTD in cross-carrier BWP switching delay

Observation 1: One slot of the smaller SCS among all the involved numerologies is enough to cover MRTD for all considered scenarios.

As for the second question, we think that 1 slot from Observation 1 can also cover the margin for cross-carrier scheduling. In both FR2-2+FR2-2 and FR1+FR2-2 MRTD is very short comparing to that slot length, so almost the whole slot from Observation 1 can be used for cross-carrier processing.

Observation 2: One slot mentioned in Observation 1 is enough to also cover the margin for cross-carrier processing

Current specification already considers one extra slot of the smaller SCS among all the involved numerologies for the case of cross-carrier scheduling, so no changes are needed for FR2-2.

Proposal 1: Rel15/16 requirements for cross-carrier active BWP switching delay can be reused for FR2-2

# Conclusion

In this paper we discussed the requirements for cross-carrier active BWP switching delay for new numerologies defined for NR 52.6 – 71 GHz operation. The following proposals were made:

Observation 1: One slot of the smaller SCS among all the involved numerologies is enough to cover MRTD for all considered scenarios.

Observation 2: One slot mentioned in Observation 1 is enough to also cover the margin for cross-carrier processing

Proposal 1: Rel15/16 requirements for cross-carrier active BWP switching delay can be reused for FR2-2

# References

- R4-2120317, “WF on NR extension to 71 GHz RRM requirements (Part 2)”, RAN4 #101-e November 01-12, 2021