3GPP TSG RAN WG4 Meeting #101-bis-e	R4-2202007

Online, 17th – 25th Jan. 2022

Agenda item:	5.15.1

Source:	SoftBank Corp., LG Electronics

Title:	TP update for TR 37.717-21-11: EN-DC_8-11_n79

Document for:	Approval

1	Introduction

The contribution for EN-DC of 2B LTE and 1B NR of DC_8-11_n79 was approved in RAN4#101[1]. This TP is to update the MSD exception value.

7	Reference

[1] R4-2117657	TP for TR 37.717-21-11: EN-DC_8-11_n79, SoftBank Corp.

Annex.   Proposed text for TR37.717-21-11:

[Unchanged Parts Skipped]

5.x	DC_8-11_n79

### 5.x.1	Configurations for DC_8-11_n79

Table 5.x.1-1: Inter-band EN-DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_8A-11A_n79A5 | DC_8A_n79A DC_11A_n79A |
| NOTE 5: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability |  |

### 5.x.2	Co-existence studies

### When Uplink EN-DC configuration is DC_8A_n79A, (1) IMD5 of (B8 - n79) will fall into Rx band of Band 11. When Uplink EN-DC configuration is DC_11A_n79A, (2) IMD5 of (B11 - n79) will fall into Rx band of Band 8.

### 5.x.3	∆TIB and ∆RIB values

The following relaxation values are proposed:

Table 5.x.3-1: ΔTIB,c

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_8-11_n79 | 8 | 0.3 |
|  | 11 | 0.4 |
|  | n79 | 0 |

Table 5.x.3-2: ΔRIB

| Inter-band EN-DC configuration | NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_8-11_n79 | 8 | 0 |
|  | 11 | 0 |
|  | n79 | 0 |

### 5.x.4	Reference sensitivity exceptions

As mentioned above, IMD5 of B8 and n79 to Band11 Rx and IMD5 of B11 and n79 to Band8 Rx need to be addressed for REFSENS relaxation. The following values are proposed:

Table 5.x.4-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| DC_8A-11A_n79A | 8 | 882.5 | 5 | 25 | 927.5 | N/A | FDD | N/A |
|  | n79 | 4980 | 40 | 216 | 4980 | N/A | TDD | N/A |
|  | 11 | 1430.4 | 5 | 25 | 1478.4 | 1.2 | FDD | IMD5 |
| DC_8A-11A_n79A | 11 | 1435 | 5 | 25 | 1483 | N/A | FDD | N/A |
|  | n79 | 4810 | 40 | 216 | 4810 | N/A | TDD | N/A |
|  |  | 8 |  | 885 |  | 5 |  | 25 |  | 930 |  | 2.8 | FDD |  | IMD5 |  |  |

[Unchanged Parts Skipped]