3GPP TSG-RAN WG4 Meeting # 101-bis-e												R4-2202228

Electronic Meeting, January 17-25, 2022

Agenda item:			6.16.3

Source:	Moderator (Qualcomm)

Title:	Email discussion summary for [101-e][128] NR_ext_to_71GHz_Part_2

Document for:	Information

# Introduction

Scope is UE RX and TX requirements for 60 GHz.

# Topic #1: UE TX and RX

Main technical topic overview. Discuss EIRP proposals and observations from various companies. Seek some common ground

## Companies’ contributions summary TX

| T-doc number | title | Company | Proposals / Observations |
| --- | --- | --- | --- |
| R4-2200438 | UE antenna module with 60 GHz integration | Apple | Observation 1: Overlap of the patch antenna active areas cannot be avoided, that hinders maintaining inter-band isolation. Observation 2: Tight integration with lower frequency antennas has significant impact to performance and prevents effective beamforming. Observation 3: The maximum routing channel would support less than 12 transmission lines in each direction. An 8x2 array of dual-polarized elements is expected to mandate an additional 2 routing cores within the module Observation 4: A key aspect component of cost-effective mm-wave module implementation is designing to minimize the layer count within the module. Observation 5: Due to electrical thickness of device materials, strong frequency-dependent lensing and cavity effects can occur, which translated to pattern distortions and limitations in scanning range. Observation 6: While peak gain values may be within target ranges in simple models, the distortions in the Proposal 1: RAN4 should limit the number of antenna elements to 4 for handheld UE type and further discuss the number of antennas for other UE types. |
| R4-2200067 | Views on UE antenna elements for FR2-2 | Murata Manufacturing Co Ltd. | Observation 1: There were two comments on communication range in RAN4 #101-e. Condition 1: From system performance perspective > 20 dBm is required. Condition 2: Use case is Indoor like 30 m ISD. Observation 2: It seems severe to set the minimum peak EIRP over 20 dBm based on RAN4 #101-e data. If there is a system performance issue requiring 20dBm it needs clarification and possibly further study. Observation 3: It is challenging for 4 antenna element UE to communicate with local area BS over a 10m range. Observation 4: In UMi condition, it is difficult to communicate over 70 m. Observation 5: It is severe making 2x8 antenna array with over 4.0 dBi element gain in commercial FR2-1 antenna module physical dimension. Observation 6: Under the 50%-tile CDF condition, 1x8 antenna element UE with 1 panel can communicate less than 5m, but we will be able to improve communication range to 8m by using 2 panels. Observation 7: Under the 50%-tile CDF condition, 1x8 antenna element UE with 2 panels and 2x8 antenna element UE with 1 panel have almost the same communication range. Proposal 1: Use 8 antenna elements as base line in RAN4 discussions. Proposal 2: Use at least 2 panels for 1x8 antenna element UE in the RAN4 discussions. Proposal 3: We should change number of panels according to number of antenna elements in the RAN4 discussions. |
| R4-2200238 | 60GHz UE TX | Qualcomm Incorporated | Array losses: Observation: FR2-2 losses will be higher than FR2-1, based on the loss trend in FR2-1. Observation: Increasing the number of array elements helps to counteract the increased losses inherent in FR2-2. Handheld array size: Observation: FR2-2 8x2 element array fits in the same footprint as a commercial FR2-1 1x4 array Proposal: 8x2 array used as an assumption to develop FR2-2 requirements. Observation: 8 element FR2-2 arrays are approximately half the size of FR2-1 arrays, and 4 element arrays are approximately ¼ the size. Handheld min peak EIRP: Proposal: Handheld minimum peak EIRP is 15 dBm. Handheld spherical coverage: Observation: Handheld minimum TRP is -0.9 dBm. Observation: Peak to 50%ile gain drop is approximately 3.5 dB higher in FR2-2 than FR2-1 due to antenna pattern and other considerations, and n262 has a 13.1 dB drop. Therefore FR2-2 drop should be 16.6 dB. Proposal: 50th %ile spherical coverage point is -1.6 dBm (based on 15 dBm min peak EIRP, 2 panels) Reducing handheld antenna elements (from 16): Observation: Reducing to 8 elements degrades the EIRP by 6 dB and reducing to 4 elements degrades the EIRP by 12 dB. Observation: Reducing to 8 elements degrades the sensitivity by 6 dB and reducing to 4 elements degrades the sensitivity by 12 dB. 1 panel vs 2 panels (handheld UE): Observation: 1 panel covers at best a hemisphere, and 2 panels back-to-back work together to cover the majority of the sphere. Observation: Using 1 panel degrades the %ile coverage CDF by 3 to 4 dB in the 20 to 80% range. Observation: A single panel results in 6 dB SINR degradation of both the UL and DL CDFs. Proposal: Specifications should be derived assuming a minimum of 2 antenna panels PC1 UE min peak EIPR and antenna size: Observation: 64 elements provides good performance on the uplink. Proposal: Use 64 elements to develop the PC1 UE specification Proposal: PC1 UE minimum peak EIRP is 26 dBm EIRP PC1 UE spherical coverage: Proposal: Use the 1-panel curve and additional losses due to beamforming errors and radiated UE structure and material losses to develop minimum performance requirement. Proposal: Specify the 85%ile at 13.5 dB down from the peak EIRP value. This equates to 12.5 dB EIRP. UE max power limits TRP and max EIRP: Proposal: Use TRP to ensure the FCC total peak transmitter output power conducted requirement. Proposal: Use NS signalling to direct the UE to limit TRP for <= 100 MHz per the FCC directive. Proposal: 23 dBm/MHz maximum PSD enacted through NS signalling. UE ACLR: Proposal: UE ALCR is 17 dBc. SEM: Proposal: Adopt the combined FR2-1 and FR2-2 table. ON/ON transient periods Proposal: The transient period from FR2-1 is based on the capability of the UE to configure the transmitter and receiver. The same capability will exist in FR2-2. Use the same 5usec for FR2-2. PRACH ON power measurement period: Proposal: PRACH ON power measurement period table should be updated for 480 and 960 SCS as shown. Beam direction only switching time baseline assumption Observation: Our understanding is the UE can perform a beam direction change, or a power control change, or both during this 200 nsec time. |
| R4-2200312 | draft CR to 38.101-2 60 GHz UE TX | Qualcomm Incorporated |  |
| R4-2200453 | Remaining issues with transient requirements for FR2-2 | Apple | Observation 1: In terms of RF hardware control timelines, leveraging of FR2 based implementations implies reusing FR2 requirements on switching between DL and UL as well as Tx/Rx beam switching delays. Observation 2: Loss of throughput due to arbitrary SRS configurations ranges from 45% (at 480 kHz SCS) to 40% (at 960 kHz SCS). Observation 3: Alignment of SRS with the Tx/Rx boundary improves loss of throughput by nearly a factor of 2. Observation 4: Loss of throughput due to arbitrary SRS configurations with a 2 us ON/ON transition time improves comparied to the 5 us case and still exceeds the SRS at Tx/Rx boundary solution compared to the 5 us case. Proposal 1: Beam direction-only switching time assumption should be defined to be 200 nsec, and no UE capability is necessary for this parameter. Proposal 2: The motivation to reduce the ON/ON transition time for 480/960 kHz SCS does not seem well justified, given the potential for the network scheduler to mitigate the impact of scheduling SRS on overall network performance. |
| R4-2200470 | UE Array, EIRP level and Spherical Coverage at 60 GHz | Sony, Ericsson | Observation 1: At least a similar array aperture is needed to maintain the network coverage at FR2-2 as in FR2-1. Observation 2: With a common PA per polarization architecture, it is feasible to implement DPD technics to further improve the efficiency of the RF front end. Observation 3: It is possible for a common PA architecture array to achieve a similar power level to an array with distributed PA architecture but with better efficiency. Observation 4: the integration losses due to the packaging and phone housing impact will inevitability increase at higher frequencies. Observation 5: At least 16 element arrays in handheld devices would be needed to compensate for higher integration losses and lower output power from PA at FR2-2. Observation 6: UE types other than handheld UE, e.g., FWA, may surpass the EIRP level of 25 dBm. Observation 7: For FWA type of devices, the performance may be limited by the regulatory requirement rather than the antenna and RF component performances. Observation 8: The degradation between 50% and 100% array gain at 60 GHz is no worse than 28 GHz due to different array topologies are being used. Observation 9: Higher number of antenna panels is required to meet the nature of propagation and body blockage in FR2-2 than FR2-1. Proposal 1: RAN4 shall consider different RF architectures and consider their pros and cons when defines various Tx requirements in FR2-2. Proposal 2: Adopt 16 elements as a baseline to derive the minimum peak EIRP requirement and define the minimum EIRP = 20.5 dBm for handheld UEs. Proposal 3: RAN4 shall use at least 2 panels as baseline for deriving the spherical coverage requirement. Proposal 4: the upper limit of the power class for a UE form factor should not be determined by regulations of a particular region (like Europe) but be indicated by an NS value indicating the maximum output power applicable in the local regulation. |
| R4-2200570 | View on TX requirements of FR2-2 | MediaTek Beijing Inc. | Proposal: Antenna quantity assumption and Pout per RF chain assumption shall be agreed together as a package. |
| R4-2200945 | Discussion on Rx beam switch time | vivo | Proposal: The baseline of Rx beam switch time should be 200 ns in FR2. |
| R4-2200950 | Further discussion on handheld UE EIRP and spherical coverage requirements for 52.6~71 GHz | vivo | Proposal 1: For handheld UE, maximum 8 antenna elements in an antenna array is preferred for 52.6~71 GHz. Observation 1: For handheld UE, the minimum peak EIRP is 11.3dBm with 1x8 antenna array. Observation 2: For handheld UE supporting both FR2-1 and FR2-2 bands, multiband relaxation factors should be considered on the peak EIRP requirement. Observation 3: For the handheld UE spherical coverage, the peak to 50% percentile gain drop is 11.48 dB with 4 antenna elements based on one panel configuration. Observation 4: For the handheld UE spherical coverage, the peak to 50% percentile gain drop is 15.54 dB with 8 antenna elements based on one panel configuration. |
| R4-2201073 | On UE Tx RF aspects for a NR band in the range 52.6GHz – 71GHz | Nokia, Nokia Shanghai Bell | Observation 1: Implementation losses need special attention to guarantee high EIRP output and therefore good UL link budget. Observation 2: UE maximum output power limits considered agrees well with regulatory requirements. Observation 3: It needed to discuss if there are any concerns with typical number of elements for the targeted device form factors of either 1x4 or 2x2 antenna elements. Proposal 1: Minimum UE beamforming requirements shall be defined. Proposal 2: Minimum Peak ERIP shall be larger than 20dBm. Proposal 3: Assume a minimum of 2 antenna panels with the understanding that it can be chosen to operate only with the best panel selection (i.e., only one of the two panels). Proposal 4: Confirm the re-use of FR2-1 SEM requirements, scaled to the appropriate bandwidths Proposal 5: Use a UE beam direction switching time of 59 ns. |
| R4-2201209 | Discussion on Tx RF requirements in FR2-2 | LG Electronics Finland | For handheld UE Proposal 1: For handheld UE, consider the following observations for 8 antenna elements - Minimum peak EIRP - 14.7 dBm - EIRP at 50%-tile CDF is X dB lower than minimum peak EIRP - X = 14 dB for 1 panel - X = 9 dB for 2 panels For vehicular UE Proposal 2: For vehicular UE, consider the following observations for 16 antenna elements - Minimum peak EIRP - 22.7 dBm - EIRP at 60%-tile CDF is X dB lower than minimum peak EIRP - X = 15.1 dB for 1 panel - X = 7.5 dB for 2 panels |
| R4-2201534 | On 60GHz UE Tx RF requirements | Huawei, HiSilicon | Proposal 1: For 60GHz handheld UE, take 16 antenna elements as basic assumption of EIRP requirement derivation. Both 2*8 and 4*4 could be considered basing on UE implementation. Observation 1: Power consumption of 16 elements antenna array is comparable with FR2-1. Proposal 2: For 60GHz handheld UE spherical coverage requirement, adopt 13.1dB for the power degradation between peak EIRP and 50%-tile CDF. Observation 2: The number of antenna panels should be a matter of UE implementation and no need for a general assumption. |
| R4-2201594 | Discussion on UE Rx beam switch delay | Nokia | Proposal: Define a UE Rx beam switch delay of 60ns |
| R4-2201598 | System parameters for a NR band in the range 52.6GHz – 71GHz | Nokia, Nokia Shanghai Bell | Proposal 3: Apply same spectrum utilization for 120 kHz SCS in FR2-2 as in FR2-1 Proposal 4: Consider similar spectrum utilization for scenarios with 800MHz and 1600MHz as 120 kHz SCS in FR2-1 Proposal 5: Support reduced spectrum utilization for 960 kHz SCS & 2 GHz CBW |
| R4-2201925 | UE Tx requirements for 52.6 to 71 GHz | Intel Corporation | Handheld UE power class Observation 1: Considering the handheld form-factor, an 8-element array presents a reasonable compromise, providing increased performance while minimizing the integration impact. Proposal 1: Use an 8-element array assumption for PC3 in FR2-2 and define the minimum peak EIRP as 13.6 dBm. Observation 2: Aligning on a baseline array size assumption is preferred to reduce the range in proposed values. It also helps provide information needed to derive measurement uncertainty (which uses a larger array size than the core baseline assumption). [3,4] Proposal 2: If no consensus is reached for the array size assumption, RAN4 should discuss if having two power classes for handheld UEs in FR2-2 is feasible. FWA UE power class Observation 3: The 32-element array yields a reasonable ~26 dBm minimum peak EIRP. Proposal 3: Use a 32-element array assumption for PC1 in FR2-2 and define the minimum peak EIRP as 25.9 dBm. ON/ON transient period Observation 4: - Option 1: No gNB scheduling optimizations for ON/ON transient period - Using 5 µS ON/ON transient period leads to high throughput reduction due to corruption of the PUSCH data symbols. Up to 50% and 12% throughput loss can be expected for bundling size 2 and 8, respectively. - An improved ON/ON transient period faster than 5 µS is required to support at least full MCS for 16 QAM modulation. - Option 2: Optimized gNB scheduling for ON/ON transient period - Using 5 µS ON/ON transient period leads to high throughput loss even with optimized gNB scheduling without corrupted symbols on UE side. Up to 25% and 6% throughput loss can be expected for bundling size 2 and 8, respectively. - An improved ON/ON transient period faster than 5 µS allows better throughput performance with almost 20%, 10% and 5% improvement for scenarios with bundling size 2, 4 and 8, respectively. Proposal 4: Introduce {1, 2, 3} µS improved ON/ON transient period as the optional UE capabilities for 480 and 960 kHz SCS. |

## Companies’ contributions summary RX

| R4-2200239 | 60 GHz UE RX | Qualcomm Incorporated | REFSENS Proposal: REFSENS for PC1 based on 64 array elements Proposal: n263 REFSENS is [-87.1 dBm] under the assumption of 95% spectral occupancy. Proposal: REFSENS for PC3 based on 16 array elements Proposal: n263 REFSENS is [-79.6 dBm] under the assumption of 95% spectral occupancy. Spherical coverage Proposal: For PC1 specify the 85%ile at 13.5 dB down from the peak sensitivity value, based on 64 elements and 400MHz CBW/95% spectral occupancy. This equates to -73.6 dBm. Proposal: For PC3 specify the 50%ile at 16.6 dB down from the peak sensitivity value, based on 16 elements and 400MHz CBW/95% spectral occupancy. This equates to -63 dBm. |
| --- | --- | --- | --- |
| R4-2200307 | draft CR to 38.101-2 60GHz UE RX | Qualcomm Incorporated |  |
| R4-2200360 | Handheld UE antenna assumption for FR2-2 | NTT DOCOMO, INC. | Observation 1: One of the factors that divides the views of each company regarding antenna assumption is that the assumptions of the frequency range supported by one antenna may be different. Proposal 1: Peak EIRP(EIS) and Spherical coverage EIRP(EIS) should be defined separately for the following cases of the frequency range: - Case 1: 66-71GHz - Case 2: 57-71GHz - Other cases are not precluded. Observation 2: When deriving EIRP(EIS) requirements for FR2-2, whether multi-band implementation with FR2-1 bands is assumed or not should be clarified. Proposal 2: Peak EIRP(EIS) and Spherical coverage EIRP(EIS) for single FR2-2 band should be defined firstly. After that, multi-band relaxation should be discussed considering multi-band implementation with FR2-1 bands. Observation 3: As assumption for the antenna element number of handheld UE for FR2-2, 16 elements is more preferred to guarantee EIRP performance similar to FR2-1. However, it may be possible to assume the antenna element number to be 8 elements in order to solve the issues such as antenna size. Proposal 3: As assumption for the antenna element number of handheld UE, 8 or 16 antenna elements should be used for both 66-71GHz and 57-71GHz cases. Observation 4: It is difficult for us to imagine the motivation to support FR2-2 with one antenna panel considering a handheld UE is covered by human hands. In fact, it is popular that commercial UEs for FR2-1 have two or more antenna panels. In addition to that, FR2-2 may have narrower beam compared FR2-1. Proposal 4: As assumption for spherical coverage for FR2-2, 1 antenna panel is excluded from consideration for both 66-71GHz and 57-71GHz cases. It should be 2 or more. Proposal 5: We can reuse UE assumptions in R4-1801202 (e.g., materials used and device size) for both 66-71GHz and 57-71GHz cases except 1 antenna panel assumption ("Assumptions1" and "Assumptions2"). |
| R4-2200951 | Further discussion on handheld UE EIS requirements for 52.6~71 GHz | vivo | Observation 1: The single-band peak EIS requirement for 52.6~71 GHz is 74 dBm/100 MHz based on 1x8 antenna element number assumption. |
| R4-2201171 | Discussion on Rx RF requirements in FR2-2 | LG Electronics Finland | For handheld UE Proposal 1: For handheld UE, consider the following observations for 8 antenna elements. - Reference sensitivity - -75.3 dBm for CBW of 400MHz - EIS spherical coverage at 50%-tile CCDF is X dB higher than Reference sensitivity - X = 14 dB for 1 panel - X = 9 dB for 2 panels For vehicular UE Proposal 2: For vehicular UE, consider the following observations for 16 antenna elements. - Reference sensitivity - -80.3 dBm for CBW of 400MHz - EIS spherical coverage at 60%-tile CCDF is X dB higher than Reference sensitivity - X = 15.1 dB for 1 panel - X = 7.5 dB for 2 panels |
| R4-2201535 | On 60GHz UE EIS requirements | Huawei, HiSilicon | Proposal 1: For 60GHz handheld UE, take 16 antenna elements as basic assumption of EIS requirement derivation. Both 2*8 and 4*4 could be considered basing on UE implementation. Observation 1: The REFSENS requirement for 52.6~71 GHz is -78.5 dBm/100 MHz based on 16 elements antenna array assumption |
| R4-2201926 | UE EIS requirements for FR2-2 | Intel Corporation | Handheld UE Observation 1: PC3 minimum peak EIS (100MHz) for a 4-element array is -73.5 dBm. For 8 and 16-element arrays, the values are -76.0 dBm, and -78.5 dBm, respectively. Given the need for improved performance and size constraints of this form factor, an 8-element antenna array is a reasonable compromise. Proposal 1: Use an 8-element antenna array assumption for handheld UEs in FR2-2 and define the single-band minimum peak EIS as -76.0 dBm (100MHz). FWA UE Proposal 2: Use a 32-elment array assumption for PC1 in FR2-2 and define the minimum peak EIS single-band requirement as -82.2 dBm (100MHz). |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Handheld UE number of antenna element assumption

Sub-topic description:

Prior Agreements: Commercial FR2-1 antenna module physical dimension can be treated as the feasible FR2-2 antenna module dimension. Commercial FR2-1 antenna module is equipped with 1x4 or 2x2 antenna element. (R4-2120062)

Open issues and candidate options before e-meeting:

- Proposals

  - Proposal 1: Maximum 4

  - Proposal 2: 8 or ‘maximum 8’ (4)

  - Proposal 3: 16 (3)

  - Proposal 4: Antenna quantity assumption and Pout per RF chain assumption shall be agreed together as a package.

- Recommended WF

  - Companies may use assumptions between 8 and 16 elements for handheld UE requirement development.

| Company | Comments |
| --- | --- |
| Murata | We believe we need at least 8 antenna elements, because communication range will be too short if we use 4 antenna element UE. We prefer 8 antenna element UE (proposal 2), but we can also support proposal 3. |
| MediaTek | Proposal 4. As observation based on proposals until last meeting, it seems that companies’ views are quite diverse, for example, one company prefers larger antenna quantity, but much less Pout. However, we know the two factors are so key for EIRP requirement discussion. Maybe a compromise method is to treat the two factor assumptions as one package. Not prefer “Companies may use assumptions between 8 and 16 elements for handheld UE requirement development.”, because it doesn’t really solve the controversial part. |
| Nokia | We are fine to assume 8 antenna elements |
| Sony | 16 elements is our preference. The larger number of elements is needed to compensate the lower PA output power and higher integration loss. A 2*8 array at 60 GHz has a similar footprint as 1*4 at 28 GHz, and 1*16 can even be possible if a narrower footprint is preferred. Moreover, we also think higher directivity beam can ensure a better co-existence performance especially in unlicensed spectrum, which is also something can be taken into account. |
| LGE | Our preference is 8 antenna elements |
| Ericsson | Proposal 3 for deriving the power class requirements (reference architecture). Together with the peak EIRP requirement this also implies higher directivity, which is beneficial for coexistence (for higher-power devices in particular). |
| Apple | Proposal 1 (R4-2200438). We are open to discuss further option 4, since Pout is also a key parameter for the derivation of minimum peak EIRP |
| Intel | Our preference is 8-elements. If needed, we are ok to further discuss. |
| vivo | We prefer P2. P1 is also OK for us. With antenna quantity larger than 8, we believe it will result in more complexity for handheld UE intergradation. Even we agreed FR2-1 antenna size can be considered feasible for FR2-2, however, we also need to consider other RF components/devices to be added for cooperative work. It will bring more difficulty for handheld UE design. As a compromise, we can accept maximum 8 antenna element number. |
| HW | Propose to take 16 elements as working assumption to achieve reasonable EIRP level. |
| OPPO | Proposal 4 and 1 or 2. To derive the EIRP requirements, the antenna element number, antenna panel number, PA output power are all key parameters. We need to align them and get some middle ground. |
| DOCOMO | Our preference is 16 elements, but we can also support at least 8 elements. This discussion is for handheld UEs, so sufficient cell coverage is required. At least 16 elements is more preferred to guarantee EIRP performance similar to FR2-1. However, we can accept not to exclude 8 elements as compromise to solve the issues such as antenna size. |
| Xiaomi | Proposal 4 and 1. The achieved EIRP should be considered to be a package to have enough cell range. |
| QCOM | 16 elements are achievable and needed Responses to Apple 4038 comments on 8x2 array design Observation 1: Overlap of the patch antenna active areas cannot be avoided, that hinders maintaining inter-band isolation. This is not new or unique to FR2-2. Existing commercial products with dual-band capability have been able to manage this fine; see picture below for example. Some existing products actually have 100% overlap of the active antenna areas for the two frequency bands. If properly designed, mutual coupling can actually be used as a performance benefit instead of a hinder as speculated. Observation 2: Tight integration with lower frequency antennas has significant impact to performance and prevents effective beamforming. As mentioned above, with proper antenna design, mutual coupling can be used to boost performance instead of negatively impacting performance as suggested. Observation 3: The maximum routing channel would support less than 12 transmission lines in each direction. An 8x2 array of dual-polarized elements is expected to mandate an additional 2 routing cores within the module Routing and number of board layers required highly depend on specific antenna and RF front-end design so this information cannot simply be generalized as mentioned in Observation3. Observation 4: A key aspect component of cost-effective mm-wave module implementation is designing to minimize the layer count within the module. Once again, cost cannot be generalized without knowing the specifics of the antenna/RF design and implementation. Minimizing layer count can be done with good antenna/RF design and creative layout. Observation 5: Due to electrical thickness of device materials, strong frequency-dependent lensing and cavity effects can occur, which translated to pattern distortions and limitations in scanning range. Not necessarily true as suggested. A proper antenna design would not have the issue mentioned in Observation 5. Observation 6: While peak gain values may be within target ranges in simple models, the distortions in the radiation patterns results in designs that are not sufficiently robust for product integration Once again, proper antenna design would not have the pattern distortion issue mentioned in Observation 6. A wrong antenna design selection can certainly have the pattern distortion issue as mentioned. Proposal 1: RAN4 should limit the number of antenna elements to 4 for handheld UE type and further discuss the number of antennas for other UE types. There are no real technical reasons to limit the number of UE antenna elements for FR2-2 to less than 16. This proposal would limit the industry to low performance UEs for FR2-2 and increase the cost of gNB deployment to maintain the network link budget. The proposed limit of only 4 UE antenna elements would actually degrade user experience for FR2-2 relative to FR2-1. |

|  | Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e., WF assignment |
| --- | --- |
|  | Tentative agreements: Candidate options: Recommendations for 2nd round: discuss in GTW |

for GTW

Preferred number of elements from company comments :

16 (Huawei, DOCOMO, QCOM)

8 (Murata, Nokia, vivo, Intel)

4(?Apple?, Xiaomi)

Acceptable: vivo 8 -> 4, Murata 8 ->16, DOCOMO 16 ->8

### Handheld UE min peak EIRP

Sub-topic description:

Open issues and candidate options before e-meeting:

- Proposals

  - Proposal 1: 15 dBm EIRP

  - Proposal 2: 20.5 dBm EIRP

  - Proposal 3: > 20 dBm EIRP

  - Proposal 4: 14.7 dBm EIRP

  - Proposal 5: 13.6 dBm EIRP

  - Proposal 6: Possibly two handheld power classes

  - Proposal 7: 11.3 dBm EIRP (OBS)

- Recommended WF

  - Define two handheld power classes. One with 15 dBm min peak EIRP. The other with 20 dBm min peak EIRP.

| Company | Comments |
| --- | --- |
| Murata | We need agreeing number of antenna element before discussing min peak EIRP. Our proposal was 15.7 dBm with 8 antenna element UE and 20.1 dBm with 16 antenna element UE in R4-2117674. We can support proposal 1 or 2. |
| MediaTek | We share similar view as Murata, it’s better that we achieve certain consensus on antenna element firstly. As discussed in last meeting, consider UE implementation would be important. Currently, we don’t prefer to define two power classes for handheld UE. |
| Nokia | Our preference is to have only one UE power class with a min. peak EIRP of 20dBm |
| Sony | Proposal 2 and 3 are our preferences based on 16 elements. We are fine to agree first on the number of elements. However, we would like to highlight a few points: Based on some link-level simulations provided in this meeting, they confirm the UL coverage is extremely limited in FR2-2. Therefore, it is vital that the UE can provide sufficient Tx power to support the desired use cases. A peak EIRP of around 20 dBm provides a good compromise between UE implementation and network coverage aspects. It is also worth mentioning that we are only discussing the peak EIRP here, and it implies that the UE may have much lower value than the peak EIRP in the majority spatial angels. |
| LGE | Our preference is to focus at the moment on one power class. We think that ~15dBm is achievable with both 8 and 16 antennas but we think that 8 is better as baseline for handheld device. When discussing the minimum requirement we need to remember that the band is really wide and this means that frequency roll-off will be happening in many locations in the TX (and RX) chains. When minimum requirement is met at upper end of the band it is likely that higher output powers can be achieved at lower channels. |
| Ericsson | Proposal 2 and 3 for coverage in the 60 GHz range. |
| Apple | We share similar view as Murata, we should agree first on the number of antenna elements before discussing min peak EIRP. |
| Intel | Ideally and preferably, we want to agree on the number of antenna elements, but this will not be specified. Regarding the minimum peak EIRP value, we can support Proposal 5 (our proposal), and Proposal 7. As a compromise, we are ok with Proposal 4 and Proposal 1. |
| vivo | These proposals are quite diverse and are not based on the same antenna element number assumption. If we consider 8 element as the baseline to derive the requirement, we can derive the average value 13.2dBm for EIRP from P4,P5,P7. For the recommended WF, it is original to define two handheld power class. In FR2, power class is related to UE type. If we define two handheld power classes, does this mean we need to consider two different types of handheld UE? For one with 15dBm min peak EIRP, this value is from P1 and it is based on antenna element number 2*8. For the value 20dBm, I guess it is from P3, which is from the system performance perspective. It is not based on feasible link budget analysis. We suggest at lease adopt the value [13.2] dBm for one power class based on 8 antenna elements. |
| HW | Considering the R17 time frame, developing 2 power classes simultaneously might delay the completion of whole WI. |
| OPPO | Agree with other companies should agree antenna elements first. |
| DOCOMO | We need an agreement on antenna assumptions to discuss min peak EIRP. We recommend clarifying the following: - Number of antenna elements - Frequency range supported by one antenna - Whether Single-band for FR2-2 or Multi-band between FR2-1 and FR2-2 (*) (*) We prefer to assume single FR2-2 band firstly. After that, multi-band relaxation should be discussed considering multi-band implementation with FR2-1 bands. |
| Xiaomi | For minPeak EIRP, we agree that to have consensus on the antenna element first. Furthermore, we would like to see the potential use scenario if the min Peak EIRP is too low. |
| QCOM | We agree with the WF .. two power classes 15 and 20 |

|  | Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e., WF assignment |
| --- | --- |
|  | Tentative agreements: Only one handheld power class.. Candidate options: Recommendations for 2nd round: Continue discussion in GTW and discuss in WF on 60 GHz UE RF |

for GTW

can we make an agreement pending the number of element discussion above?

| Company | Min Peak EIRP |  |  |
| --- | --- | --- | --- |
|  | 4 elements | 8 elements | 16 elements |
| vivo |  | 13.2 |  |
| QCOM |  |  | 15 |
| LGE |  | 14.7 | 14.7 |
| Murata |  | 15.7 | 20.1 |
| Nokia |  | 20 | 20 |
| Sony, Ericsson |  |  | 20.5 |
| Intel |  | 13.6 |  |
| Average | n/a | 15.4 | 18.0 |

### Handheld UE number of antenna panels for spatial coverage

Sub-topic description:

Open issues and candidate options before e-meeting:

- Proposals

  - Proposal 1: Minimum 2 (4)

  - Proposal 2: Minimum 1 (observation)

- Recommended WF

  - Two panel assumption for handheld

| Company | Comments |
| --- | --- |
| Murata | We prefer Proposal 1. It will not be feasible using 1 panel from the viewpoint of communication range. It is also severe communicating with 1 panel in case that handheld UE is covered by human hands. |
| Nokia | We support a minimum of 2 antenna panels. |
| Sony | Proposal 1, two panels |
| LGE | We support proposal 1 |
| Ericsson | Proposal 1. |
| Apple | Proposal 2, minimum 1. Keeping the assumption as in FR2-1 |
| Intel | We can keep the assumption of minimum 1 panel. Ultimately, we will not specify this. |
| vivo | We tend to agree P2. Antenna panel number depends on UE implementation. No need to restrict it. |
| HW | Prefer Proposal 2. The number of antenna panels could be depending on UE implementation as long as the spherical coverage requirement is met. |
| OPPO | Proposal 2. One panel, align with FR2-1. More panels is challenging for UE to implement especially considering UE has to support also FR2-1 bands which probably are separate panels. Besides, we cannot require UE to support more panels while asking more elements in each panel. |
| DOCOMO | Proposal 1. It is difficult for us to imagine the motivation to support FR2-2 with one antenna panel considering a handheld UE is covered by human hands. |
| Xiaomi | Proposal 2. Larger number of panels can leave to UE implementation. |
| QCOM | Proposal 1. UE needs to have view around the sphere and this is not possible with 1 panel. |

|  | Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e., WF assignment |
| --- | --- |
|  | Minimum 2: (7) (Murata, Nokia, Sony, LGE, Ericsson, DOCOMO, QCOM) Minimum 1: (4) (Apple, vivo, Huawei, OPP) Tentative agreements: Candidate options: Recommendations for 2nd round: Continue discussion in round 2 |

for GTW

Companies are split on the number of panels

Minimum 2: (7) (Murata, Nokia, Sony, LGE, Ericsson, DOCOMO, QCOM)

Minimum 1: (4) (Apple, vivo, Huawei, OPPO)

### Handheld UE uplink spherical coverage

Sub-topic description:

Open issues and candidate options before e-meeting:

- Proposals

  - Proposal 1: 50 %ile, -1.6 dBm EIRP

  - Proposal 2: 50 %ile, 5.7 dBm w 2 panels

  - Proposal 3: 50%ile, 0.7 dBm with 1 panel

  - Proposal 4: 50 %ile, 13.1 dB drop from peak (peak EIRP not stated)

  - Proposal 5: 50 %ile, 11.48 dB drop, 1 panel, 4 elements

  - Proposal 6: 50 %ile, 15.54 dB drop, 1 panel, 8 elements

- Recommended WFs

  - 1 - Agree to use 50%ile for handheld

  - 2 – Further discuss values in the range of -1.6 and 5.7 dBm for 2 panel

| Company | Comments |
| --- | --- |
| Murata | We need agreeing number of antenna elements and number of antenna panels before discussing spherical coverage. Especially difference between peak and 50%-tile CDF EIRP is depend on number of panels. We can support proposal 2 because we will be able to communicate about 14m with this value. Communication range will be short in other proposals. |
| MediaTek | Echo “We need agreeing number of antenna elements and number of antenna panels before discussing spherical coverage” |
| Nokia | We are fine to consider the 50%ile for handheld UEs. The value is dependent on the agreement for min. peak EIRP. |
| Sony | We suggest focusing on single panel configuration and the number of panels first and then discussing this issue. If we assume two panels minimum, we don’t think the drop between peak EIRP and 50% EIRP is necessarily to be worst than FR2-1. In our simulation, an 8.5 dB drop from peak gain is observed with two panels |
| LGE | We are OK with 50% CDF point for handheld with 14dB and 9dB delta to minimum requirement with 1 and 2 panels respectively. |
| Ericsson | The number of panels should be decided first. We propose that two panels be assumed for adequate performance in view of the challenging propagation conditions at 60 GHz. |
| Apple | Similar as our comment in 1.3.1, we need to agree first on the number of antenna elements before discussing minimum peak EIRP and spherical coverage. |
| Intel | We need to agree on the minimum peak EIRP value first. We are ok to reuse the 50%-ile point. |
| vivo | We only agree to use 50%ile for handheld. The values for spherical coverage can be decided after we agree on antenna element number and panel number. |
| HW | Propose to agree on the power drop first. Below updated proposals include the power drop information. - Proposal 1: 50 %ile, -1.6 dBm EIRP, 16.6dB drop from peak EIPR - Proposal 2: 50 %ile, 5.7 dBm w 2 panels, 9dB drop from peak EIRP - Proposal 3: 50%ile, 0.7 dBm with 1 panel, 14dB drop from peak EIRP - Proposal 4: 50 %ile, 13.1 dB drop from peak (peak EIRP not stated) - Proposal 5: 50 %ile, 11.48 dB drop, 1 panel, 4 elements - Proposal 6: 50 %ile, 15.54 dB drop, 1 panel, 8 elements From above it could be seen the power drop ranges from 9dB~16.6dB. Proposal 4 is preferred, which is the worst power drop case in FR2-1. For Handheld UE, the similar spherical coverage performance as FR2-1 could be adopted as starting point. Also it could be a good compromise within the range of 9~16.6dB proposed from companies. How many panels are used to achieve this spherical requirement could be depending on UE’s implementation. |
| OPPO | Should be discussed after the panel numbers and elements, PA output powers are clear. Without these information, difficult to align requirements. |
| DOCOMO | We need an agreement on antenna assumptions to discuss spherical coverage. We recommend clarifying the following: - Number of antenna elements - Number of antenna panels - Frequency range supported by one antenna - Whether Single-band for FR2-2 or Multi-band between FR2-1 and FR2-2 (*) (*) We prefer to assume single FR2-2 band firstly. After that, multi-band relaxation should be discussed considering multi-band implementation with FR2-1 bands. |
| QCOM | We support WF. 50 %ile is the only %ile proposed. |

|  | Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e., WF assignment |
| --- | --- |
|  | Tentative agreements: 6 of 6 proposals use the 50%ile as the CDF point. Agree to use 50 %ile. Candidate options: Recommendations for 2nd round: Continue discussion in GTW and discuss in WF on 60 GHz UE RF |

for GTW

can we make an agreement pending the number of element discussion above?

| Company | 50 %ile spherical coverage dBm EIRP |  |  |
| --- | --- | --- | --- |
|  | 4 elements | 8 elements | 16 elements |
| 2 panels |  |  | -1.61, 5.72 |
| 1 panel |  | 0.72 |  |
| 1 – QCOM, 2 - LGE |  |  |  |

### Handheld UE REFSENS

Sub-topic description:

Open issues and candidate options before e-meeting:

- Proposals

  - Proposal 1: -79.6 dBm, CBW 400MHz, 95% SU assumption

  - Proposal 2: separate requirement for 57-71 and 66-71 GHz

  - Proposal 3: -75.3 dBm, CBW 400MHz

  - Proposal 4: -72.5 dBm CBW 400 MHz (from -78.5 dBm, CBW 100 MHz)

  - Proposal 5: -70.0 dBm CBW 400 MHz (from -76.0 dBm, CBW 100 MHz)

  - Proposal 6: -68 dBm CBW 400 MHz (from -74 dBm,100 MHz)

- Recommended WF

  - As a compromise, agree on the median -73.8 dBm

| Company | Comments |
| --- | --- |
| Murata | We prefer proposal 2. It is important confirming assumed frequency range, because frequency roll-off is depend on its frequency range. And we need agreeing number of antenna element before discussing REFSENSE. Our proposal was -71.8 dBm with 8 antenna element UE and -73.2 dBm with 16 antenna element UE. We set CBW 400MHz in R4-2117675, but frequency range was 52.6-71.0GHz. |
| MediaTek | Echo “we need agreeing number of antenna element before discussing REFSENSE.”. |
| Nokia | The proposed compromise of -73.8 is too relaxed in our view considering the link budget requirements. We can accept Proposal 1 or 3. |
| Sony | We suggest discussing this after we agreed on number of antenna elements and minimum peak EIRP. |
| LGE | Recommended WF (median) is a bit more relaxed than our analysis, but we can also agree with it as compromise. Different requirements (more stringent or looser) can be agreed for different bands and meeting improved performance on narrower band is easier than on wider one. Before going into details here RAN4 should discuss on more general level if sub-bands/ranges with different requirements (RX and TX) should be defined for unlicensed n263 band at 60GHz. |
| Ericsson | This is pending the decision on the number of antenna elements. |
| Apple | Similar as our comment in 1.3.1, we need to agree first on the number of antenna elements before discussing UE REFSENS. |
| Intel | May depend on outcome of previous topics. Our preference is Proposal 5. We are also ok with Murata’s 8-elements proposal of -71.8 dBm (R4-2117576). If needed, we can compromise to Proposal 4. |
| vivo | These proposals are based on different antenna assumption. Maybe we can further decide these EIS values after we agree on antenna assumption. For Proposal 2, we tend to consider the requirement for the whole band from 57~71GHz. |
| HW | This could be discussed after agreeing on number of antenna elements. |
| OPPO | Should be discussed after the panel numbers and elements are clear. Without these information, difficult to align requirements. |
| DOCOMO | We need an agreement on antenna assumptions to discuss REFSENS. We recommend clarifying the following: - Number of antenna elements - Frequency range supported by one antenna - Whether Single-band for FR2-2 or Multi-band between FR2-1 and FR2-2 (*) (*) We prefer to assume single FR2-2 band firstly. After that, multi-band relaxation should be discussed considering multi-band implementation with FR2-1 bands. |
| QCOM | Our proposal .. proposal 1 |

|  | Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e., WF assignment |
| --- | --- |
|  | Tentative agreements: Develop the requirement for n263. Candidate options: Recommendations for 2nd round: further discuss round 2 |

for GTW

can we make an agreement pending the number of element discussion above?

| Company | 400 MHz SENS |  |  |
| --- | --- | --- | --- |
|  | 4 elements | 8 elements | 16 elements |
| QCOM |  |  | -79.6 |
| vivo |  | -68 |  |
| LGE |  | -75.3 |  |
| Huawei |  |  | -72.5 |
| Intel |  | -70 |  |

### Handheld UE EIS spherical coverage

Sub-topic description:

Open issues and candidate options before e-meeting:

- Proposals

  - Proposal 1: -63 dBm, 400 MHz, 50%ile

  - Proposal 2: -64.3 dBm, 400 MHz, 50%ile

  - Proposal 3: -59.3 dBm, 400 MHz, 50%ile

- Recommended WF

  - As a compromise, agree on the median -61 dBm

| Company | Comments |
| --- | --- |
| Murata | We need agreeing number of antenna element and number of antenna panels before discussing EIS spherical coverage. |
| MediaTek | Echo “We need agreeing number of antenna element and number of antenna panels before discussing EIS spherical coverage.” |
| Nokia | The proposed compromise of -61dBM is too relaxed in our view considering the link budget requirements. We can accept Proposal 1 or 2. |
| Sony | Similar comments as previous issue, since EIS and EIRP are dual, it is better to consolidate the antenna assumption in EIRP discussion first and then discuss the receiver part. |
| LGE | We agree with Sony proposal above to agree first on EIRP and then apply similar approach and delta values for EIS spherical coverage. |
| Apple | Similar as our comment in 1.3.1, we need to agree first on the number of antenna elements before discussing EIS spherical coverage. |
| Intel | We need to agree on the minimum peak EIS value first |
| vivo | We need to align the antenna assumption first. |
| HW | Propose a new option: Deciding Rx Spherical coverage based on peak EIS value and required spherical performance drop while applying the same value for Tx spherical performance drop and Rx spherical performance drop. For bands in Rel-15 and Rel16, the Tx spherical performance drop is the same as Rx spherical performance drop except for band n259. This approach could be adopted for 60GHz bands. |
| OPPO | Should be discussed after the panel numbers and elements are clear. Without these information, difficult to align requirements. |
| DOCOMO | We need an agreement on antenna assumptions to discuss spherical coverage. We recommend clarifying the following: - Number of antenna elements - Number of antenna panels - Frequency range supported by one antenna - Whether Single-band for FR2-2 or Multi-band between FR2-1 and FR2-2 (*) (*) We prefer to assume single FR2-2 band firstly. After that, multi-band relaxation should be discussed considering multi-band implementation with FR2-1 bands. |
| QCOM | We are ok with proposal 1, or with the WF |

|  | Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e., WF assignment |
| --- | --- |
|  | Tentative agreements: Candidate options: Recommendations for 2nd round: discuss in WF on 60 GHz UE RF |

### FWA UE number of antenna element assumption

Sub-topic description:

Previous agreements: If a single power class is defined for FWA in Rel-17, the number of antenna element assumption is anywhere in the range between 32 and 64 elements. (R4-2120062)

Open issues and candidate options before e-meeting:

- Proposals

  - Proposal 1: 64 elements

  - Proposal 2: 32 elements

- Recommended WF

  - Allow assumptions anywhere in the range of 32 to 64 elements

| Company | Comments |
| --- | --- |
| XXX | YYYY |
| MediaTek | We don’t have strong view on 32 or 64 elements so far, however, it would be better to have consensus on either one compared to “ Allow assumptions anywhere in the range of 32 to 64 elements” |
| Intel | We agree with the recommended WF |
| HW | Proposal 1 is preferred. Open to further discussion on Proposal 2. |
| QCOM | Proposal 1 |

|  | Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e., WF assignment |
| --- | --- |
|  | Tentative agreements: Candidate options: Recommendations for 2nd round: discuss in WF on 60 GHz UE RF |

### FWA UE minimum peak EIRP

Sub-topic description:

Open issues and candidate options before e-meeting:

- Proposals

  - Proposal 1: 26 dBm EIRP

  - Proposal 2: 25.9 dBm EIRP

- Recommended WF

  - Agree 26 dBm FWA min peak EIRP

| Company | Comments |
| --- | --- |
| XXX | YYYY |
| MediaTek | Wait output of handheld UE and topic 1.3.7 (antenna element) would be easier to have progress on FWA min peak EIRP. |
| Nokia | We are fine with min. peak EIRP for FWA of 26dBm |
| Ericsson | This is also pending an agreement on the number of antenna elements for the reference architecture. |
| Intel | We support the recommended WF (26 dBm) |
| QCOM | Agree with the WF |

|  | Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e., WF assignment |
| --- | --- |
|  | Tentative agreements: Candidate options: Recommendations for 2nd round: discuss in WF on 60 GHz UE RF |

### FWA UE number of antenna panels for spatial coverage

Sub-topic description:

Open issues and candidate options before e-meeting:

- Proposals

  - Proposal 1: 1 panel

- Recommended WF

  - Proposal 1

| Company | Comments |
| --- | --- |
| XXX | YYYY |
| Nokia | We are okay with the recommended WF for FWA |
| Sony | Okay with this proposal. |
| Ericsson | One panel acceptable for this device type. |
| HW | OK with the recommended WF |
| QCOM | OK with the WF |

|  | Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e., WF assignment |
| --- | --- |
|  | Tentative agreements: One panel Candidate options: Recommendations for 2nd round: record agreement in in WF on 60 GHz UE RF |

### FWA UE uplink spherical coverage

Sub-topic description:

Open issues and candidate options before e-meeting:

- Proposals

  - Proposal 1: 85 %ile, 12.5 dBm EIRP

- Recommended WF

  - Agree Proposal 1

| Company | Comments |
| --- | --- |
| XXX | YYYY |
| MediaTek | FFS is better at current stage, as more typical discussion process. |
| Intel | More discussion is needed |
| HW | Input from more companies are needed to decide on FWA requirements. |
| QCOM | We agree with the WF |

|  | Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e., WF assignment |
| --- | --- |
|  | Tentative agreements: Discuss in future meeting Candidate options: Recommendations for 2nd round: No further discussion this meeting |

### FWA UE REFSENS

Sub-topic description:

Open issues and candidate options before e-meeting:

- Proposals

  - Proposal 1: -87.2 dBm. 400 MHz, 95% spectral occupancy assumption

  - Proposal 2: -76.2 dBm, 400 MHz (from -82.2 dBm, 100 MHz)

- Recommended WF

  - Further discuss

| Company | Comments |
| --- | --- |
| XXX | YYYY |
| MediaTek | Fine with “Further discuss” |
| Nokia | Comparing to proposals for handheld UE, proposal 2 is too relaxed and cannot be accepted. |
| Intel | Further discussion is needed. The specified value for 28GHz bands with 400MHz CBW is -88.5 dBm. For band n262 (400MHz CBW), the value is -83.5 dBm. We expect some degradation for band n263 compared to n262. |
| HW | Agree with recommended WF |
| QCOM | Proposal 1 |

|  | Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e., WF assignment |
| --- | --- |
|  | Tentative agreements: Discuss in future meeting Candidate options: Recommendations for 2nd round: No further discussion this meeting |

### FWA UE EIS spherical coverage

Sub-topic description:

Open issues and candidate options before e-meeting:

- Proposals

  - Proposal 1: -73.6 dBm, 400 MHz, 95% spectral occupancy assumption

- Recommended WF

  - Further discuss

| Company | Comments |
| --- | --- |
| XXX | YYYY |
| MediaTek | Fine with “Further discuss” |
| Intel | Need to agree on the minimum peak EIS first. Further discussion is needed. |
| HW | Agree with recommended WF |
| QCOM | Proposal 1 |

|  | Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e., WF assignment |
| --- | --- |
|  | Tentative agreements: Discuss in future meeting Candidate options: Recommendations for 2nd round: No further discussion this meeting |

### Vehicular UE minimum peak EIRP

Sub-topic description:

Prior Agreements: Vehicular UE antenna array size is >= handheld. (R4-2120062)

Open issues and candidate options before e-meeting:

- Proposals

  - Proposal 1: 22.7 dBm EIRP

- Recommended WF

  - Proposal 1

| Company | Comments |
| --- | --- |
| XXX | YYYY |
| MediaTek | If we can achieve consensus on antenna element of handheld and Vehicular UE, and also handheld min peak EIRP firstly, it would be easier to converge Vehicular UE min peak EIRP discussion. |
| LGE | We agree proposal 1, for 22.7dBm it was assumed that 16 antennas is used. |
| HW | Input from more companies are needed to decide on vehicular requirements. |

|  | Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e., WF assignment |
| --- | --- |
|  | Tentative agreements: Discuss in future meeting Candidate options: Recommendations for 2nd round: No further discussion this meeting |

### Vehicular UE uplink spherical coverage

Sub-topic description:

Open issues and candidate options before e-meeting:

- Proposals

  - Proposal 1: 60 %ile, 7.6 dBm 1 panel assumption

  - Proposal 2: 60 %ile, 15.2 dBm 2-panel assumption

- Recommended WF

  - Further discuss

| Company | Comments |
| --- | --- |
| MediaTek | Fine with “Further discuss” |
| LGE | We can further discuss this and focus first on agreeing the handheld requirements. |
| HW | Input from more companies are needed to decide on vehicular requirements. |
| QCOM | Companies should contribute on number of panels. We tend to support 2 or more panels. |

|  | Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e., WF assignment |
| --- | --- |
|  | Tentative agreements: Discuss in future meeting Candidate options: Recommendations for 2nd round: No further discussion this meeting |

### Vehicular UE REFSENS

Sub-topic description:

Open issues and candidate options before e-meeting:

- Proposals

  - Proposal 1: -80.3 dBm for CBW of 400MHz

- Recommended WF

  - Proposal 1

| Company | Comments |
| --- | --- |
| MediaTek | “Further discuss” is suggested |
| LGE | We support proposal 1. It is assumed that 16 antennas is used. |
| HW | Input from more companies are needed to decide on vehicular requirements. |
| QCOM | We support proposal 1 |

|  | Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e., WF assignment |
| --- | --- |
|  | Tentative agreements: Discuss in future meeting Candidate options: Recommendations for 2nd round: No further discussion this meeting |

### Vehicular UE EIS spherical coverage

Sub-topic description:

Open issues and candidate options before e-meeting:

- Proposals

  - Proposal 1: -72.8 dBm, 60%ile, 2 panels assumption, 400 MHz

  - Proposal 2: -65.1 dBm, 60%ile, 1 panel assumption, 400 MHz

- Recommended WF

  - Further discuss

| Company | Comments |
| --- | --- |
| MediaTek | Fine with “Further discuss” |
| LGE | We can further discuss this and focus first on agreeing the handheld requirements. |
| QCOM | Companies should contribute on number of panels. We tend to support 2 or more panels. |

|  | Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e., WF assignment |
| --- | --- |
|  | Tentative agreements: Discuss in future meeting Candidate options: Recommendations for 2nd round: No further discussion this meeting |

### UE ACLR

Sub-topic description:

Previous agreements: Specify 99% OBW requirements in CCBW and specify ACLR. Waive the ACLR in the test if the ACLR requirement is less stringent than OBW requirement (R4-2120062)

Open issues and candidate options before e-meeting:

- Proposals

  - Proposal 1: 17 dB

- Recommended WF

  - Discuss this topic in thread [129]

| Company | Comments |
| --- | --- |
| XXX | YYYY |

|  | Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e., WF assignment |
| --- | --- |
|  | Tentative agreements: Discuss in thread 129 Candidate options: Recommendations for 2nd round: If 129 concludes record agreement in in WF on 60 GHz UE RF |

### UE SEM

Sub-topic description:

Previous agreements: Agree the table from R4-2119122 as the starting point for SEM requirements (R4-2120062)

Open issues and candidate options before e-meeting:

- Proposals

  - Proposal 1: Adopt the combined FR2-1 and FR2-2 table from R4-2200238

- Recommended WF

  - Proposal 1

| Company | Comments |
| --- | --- |
| XXX | YYYY |
| Nokia | We are okay with recommended WF since the SEM is based on FR2-1 |
| vivo | This table misses 200M SEM for FR2-1. For ΔfOOB for 50MHz and 100MHz, they are not aligned with original FR2 table. |

|  | Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e., WF assignment |
| --- | --- |
|  | Tentative agreements: Candidate options: Recommendations for 2nd round: check vivo comments and possible correct in WF on 60 GHz UE RF |

### Spectral utilization

Sub-topic description:

Previous agreements:

Open issues and candidate options before e-meeting:

- Proposals

  - Proposal 1: Apply same spectrum utilization for 120 kHz SCS in FR2-2 as in FR2-1

  - Proposal 2: Consider similar spectrum utilization for scenarios with 800MHz and 1600MHz as 120 kHz SCS in FR2-1

  - Proposal 3: Support reduced spectrum utilization for 960 kHz SCS & 2 GHz CBW

- Recommended WF

  - 1 - Agree 120 kHz SU for 100 and 400 MHz the same as FR2-1.

  - 2 – Further discuss SU for other SCS and CCBW

| Company | Comments |
| --- | --- |
| Nokia | We are fine with the recommended WF |
| Ericsson | This requires further discussion and should be considered jointly for the gNB and UE. |
| vivo | We agree with the recommended WF. |
| QCOM | We agree with the WF |

|  | Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e., WF assignment |
| --- | --- |
|  | Tentative agreements: Candidate options: Recommendations for 2nd round: further discuss in WF on 60 GHz UE RF |

### FCC PSD Limit

Sub-topic description:

Open issues and candidate options before e-meeting:

- Proposals

  - Proposal 1: Use NS signalling to direct the UE to limit TRP for <= 100 MHz per the FCC directive; 23 dBm/MHz max

- Recommended WF

  - Proposal 1

| Company | Comments |
| --- | --- |
| Sony | Support NS signalling based method to define the upper limit. |
| Ericsson | We support using NS values for indicating the upper power-class limits that are subject to local regulation. |
| vivo | The proposal is OK. |
| HW | Need clarification on whether the PSD requirement is averaged on the whole bandwidth or scanned per 1MHz |
| QCOM | We are ok with the WF |

|  | Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e., WF assignment |
| --- | --- |
|  | Tentative agreements: Candidate options: Recommendations for 2nd round: further discuss in WF on 60 GHz UE RF |

### ON/ON transient periods for 480 and 960 SCS

Sub-topic description:

Open issues and candidate options before e-meeting:

- Proposals

  - Proposal 1: Use the same 5usec for FR2-2. (2)

  - Proposal 2: Introduce {1, 2, 3} µS improved ON/ON transient period as the optional UE capabilities for 480 and 960 kHz SCS.

- Recommended WF

  - Further discuss

| Company | Comments |
| --- | --- |
| Nokia | In case improved transient period is introduced, the number of possible values needs to be reduced. We prefer to maximum one optional value, it being 1us or 2us. |
| MediaTek | We continue to support Proposal 1. We do not support Proposal 2. |
| Ericsson | We support introduction of a capability for ON-ON transitions but can also accept 5 us. |
| Apple | Proposal 1. |
| Intel | According to our evaluations on ON/ON transient period impact on PUSCH performance, up to 20% throughput improvement can be achieved with transient periods smaller than 5us, even with optimized gNB scheduling to cope with transient period impact. Analysis from Apple on transient period between SRS and PUSCH also shows that the reduction of transient period from 5us to 2us allows to increase throughput up to 10%. Such significant benefits should be taken into account to not preclude FR2-2 products with smaller transient periods than 5us. Additionally, to allow gradual product development we suggest defining smaller transient periods values as optional UE capability. |
| AT&T | Proposal 2. However, we can consider the possibility of reducing the set of optional values. |
| HW | Proposal 1. |
| QCOM | Proposal 1. We don’t support proposal 2. |

|  | Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e., WF assignment |
| --- | --- |
|  | Tentative agreements: Candidate options: Recommendations for 2nd round: further discuss in WF on 60 GHz UE RF |

### UE beam direction switching time

Sub-topic description:

Open issues and candidate options before e-meeting:

- Proposals

  - Proposal 1: 200 nsec as in FR2-1

  - Proposal 2: 59 or 60 nsec.

- Recommended WF

  - Further discuss

| Company | Comments |
| --- | --- |
| QCOM from thread 119 | The gNB and UE face different operational criteria and require different procedures during a generalized beam change. It is not feasible to equate the two. Some significant differences are: - Power consumption allowance in case of gNB - Tx dynamic range requirement for UEs |
| OPPO from thread 119 | Option 1 |
| vivo from thread 119 | We support option 1, in TR 38.817-02, the worst case for intra-panel can be 100ns, and for CBM we should further consider the additional time for inter-panel or other latency, so 200 ns is preferred. In addition, based on the conclusion in this TR, 200ns beam switch time will not cause significant system performance degradation. |
| Ericsson from thread 119 | Option 2. |
| Nokia | We support 59ns and would like to ask for a reference to where 200ns is defined for FR2-1. |
| MediaTek | We continue to support Option 1. |
| Apple | Proposal 1: 200nsec. |
| vivo | OK with P1. |
| HW | Proposal 1. The beam switching time is not related to frequency range. Same value as FR2-1 shall be adopted. |
| QCOM | Proposal 1. Faster is not feasible |

|  | Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e., WF assignment |
| --- | --- |
|  | Tentative agreements: Candidate options: Recommendations for 2nd round: further discuss in WF on 60 GHz UE RF |

### PRACH ON power measurement period

Sub-topic description:

Open issues and candidate options before e-meeting:

- Proposals

  - Proposal 1: Proposal: PRACH ON power measurement period table should be updated for 480 and 960 SCS as shown in R4-2200238

- Recommended WF

  - Proposal 1

| Company | Comments |
| --- | --- |
| HW | The measurement period for 480 and 960 kHz SCS is just a few us, and probably not testable. It needs discussion first on whether to introduce the requirements considering the testability. |

|  | Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e., WF assignment |
| --- | --- |
|  | Tentative agreements: Further discuss in future meeting Candidate options: Recommendations for 2nd round: No further discussion this meeting |

## Companies views’ collection for 1st round

### CRs/TPs comments collection

For close-to-finalize Wis and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing Wis, suggest focusing on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2200312 | Nokia: We agree with the formatting used in the draftCR but many of the values are still under discussion. |
|  | Company B |
| R4-2200307 | Nokia: What is the motivation to create new tables for PC1? It would be still worth to check whether PC1 can re-use the existing tables similar to PC3. |
|  | Company B |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e., WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic #1 | Tentative agreements: Candidate options: Recommendations for 2nd round: |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2200312 | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |
| R4-2200307 | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round (if applicable)

# Recommendations for Tdocs

## 1st round

New tdocs

| Title | Source | Comments |
| --- | --- | --- |
| WF on 60 GHz UE RF | Qualcomm Incorporated |  |

Existing tdocs

| T-doc number | title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2200067 | Views on UE antenna elements for FR2-2 | Murata Manufacturing Co Ltd. | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-2200238 | 60GHz UE TX | Qualcomm Incorporated |  |  |
| R4-2200239 | 60 GHz UE RX | Qualcomm Incorporated |  |  |
| R4-2200307 | draft CR to 38.101-2 60GHz UE RX | Qualcomm Incorporated |  |  |
| R4-2200312 | draft CR to 38.101-2 60 GHz UE TX | Qualcomm Incorporated |  |  |
| R4-2200360 | Handheld UE antenna assumption for FR2-2 | NTT DOCOMO, INC. |  |  |
| R4-2200438 | UE antenna module with 60 GHz integration | Apple |  |  |
| R4-2200453 | Remaining issues with transient requirements for FR2-2 | Apple |  |  |
| R4-2200470 | UE Array, EIRP level and Spherical Coverage at 60 GHz | Sony, Ericsson |  |  |
| R4-2200570 | View on TX requirements of FR2-2 | MediaTek Beijing Inc. |  |  |
| R4-2200945 | Discussion on Rx beam switch time | vivo |  |  |
| R4-2200950 | Further discussion on handheld UE EIRP and spherical coverage requirements for 52.6~71 GHz | vivo |  |  |
| R4-2200951 | Further discussion on handheld UE EIS requirements for 52.6~71 GHz | vivo |  |  |
| R4-2201073 | On UE Tx RF aspects for a NR band in the range 52.6GHz – 71GHz | Nokia, Nokia Shanghai Bell |  |  |
| R4-2201171 | Discussion on Rx RF requirements in FR2-2 | LG Electronics Finland |  |  |
| R4-2201209 | Discussion on Tx RF requirements in FR2-2 | LG Electronics Finland |  |  |
| R4-2201534 | On 60GHz UE Tx RF requirements | Huawei, HiSilicon |  |  |
| R4-2201535 | On 60GHz UE EIS requirements | Huawei, HiSilicon |  |  |
| R4-2201594 | Discussion on UE Rx beam switch delay | Nokia |  |  |
| R4-2201598 | System parameters for a NR band in the range 52.6GHz – 71GHz | Nokia, Nokia Shanghai Bell |  |  |
| R4-2201925 | UE Tx requirements for 52.6 to 71 GHz | Intel Corporation |  |  |
| R4-2201926 | UE EIS requirements for FR2-2 | Intel Corporation |  |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on to/Cc WGs in the comments column

- Do not include hyper-links in the documents

## 2nd round

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| XXX | WF on TBD or whatever |  |  |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents

# Annex

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| Qualcomm Inc | Phil Coan | pcoan@qti.qualcomm.com |
| AT&T | Ron Borsato | ronald.borsato@att.com |
| vivo | Shuai Zhou | Shuai.zhou@vivo.com |
| DOCOMO | Ryu Kitagawa | ryuu.kitagawa.pn@nttdocomo.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e., Company A (XX, XX)