3GPP TSG-RAN WG4 Meeting # 101bis-e											     R4-2202279

Electronic Meeting, 17-25 January 2022

Source: 	Qualcomm Incorporated, Skyworks Solutions Inc.,

Title: 	WF on triple beat evaluation and specification framework

Agenda Item:	5.6.1

Document for:	Approval

# Background – Motivation for Way Forward

- Band combinations with 3 ULCCs were analysed in [1]

- Further analysis and measurements were completed recently in [4]

- 3 active UL CCs create triple beat distortion and can lead to RX de-sensitization.

- The table below captures the uplink configurations for which triple beat IMD products are generated. Eligible REL-17 band combinations from [1][2] that were detected to create triple beat distortion are highlighted in yellow in table below. According to RAN plenary guidelines [6], the combinations highlighted in orange are not eligible uplink configurations for REL-17 and should not be requested or specified.

| 3A_n7B | 3A_n41C | 3A_n78(2A) | 7C_n28A | 41C_n3A |
| --- | --- | --- | --- | --- |
| 28A_n7B | 3C_n1A | 3C_n77A | 7C_n78A | 41C_n77A |
| 1A_n7B | 3C_n5A | 3C_n78A | 8A_n79C | 41C_n78A |
| 1C_n3A | 3C_n7A | 3C_n79A | 25A_n41C | 41C_n79A |
| 2C_n41A | 3C_n28A | 7C_n1A | 30_n77(2A) | 42C_n28A |
| 2C_n71A | 3C_n41A | 7C_n3A | 39C_n41A | 42C_n3A |
| 2A_n77(2A) | 3A_n77(2A) | 7C_n5A | 40C_n78A | 66_n77(2A) |

# Background – Detection

- The detection mechanism for the 1st order triple beat is shown below from [2]

    - Duplex Offset is the duplex offset of the victim FDD band

    - ULCAMBW is the non-contiguous 2 tone allocation spacing

    - TXMBW and RXBW is the FDD victim band TX transmission measured BW and the RX DL channel BW respectively

# Background – Analysis

- Triple beat distortion occurs at RX and TX and is a 3rd order non-linearity that is mainly α (TX22TX1) and is more prominent for non-contiguous RB allocations.

- For TX

    - Both forward and reverse triple beat products can occur at the transmitter

    - It was shown that the triple beat distortion at the ULCA transmitter path is dominant for TX due to 2:1 variation with ULCA output power

    - PCB isolation from FDD band PA to ULCA band PA will affect forward TX distortion

    - ULCA Filter rejection and antenna isolation will affect the reverse TX distortion

- For RX

    - RX path filter rejection for both TX bands and the 3rd order non-linearity specification determines the RX triple beat distortion

- Power Back-off

    - For ULCA band, the power is backed off to meet the general emission requirements or CA_NS_04 requirement.

      - this can be a function of tone spacing.

    - For FDD band, the power is backed off to meet the NS_01 requirement

    - Backed off power must meet the power class requirement

    - Worst case triple beat will occur with equal power on the 3ULCCs if both power class and emission requirements are met.

# Background – Previous Framework for Specification [5]

- Mainly consider the 1st order triple beat α (TX22TX1) of 3rd order non-linearity -> this is shown in the table as triple beat order 1.

  - Detection -> Duplex Offset<1∙ULCAMBW+TXMBW2 +RXBW2

- Further check the 2nd order triple beat α (TX24TX1) of 5th order non-linearity -> this would be shown in the table as triple beat order 2.

  - Detection -> Duplex Offset<2∙ULCAMBW+TXMBW2 +RXBW2

  - This may come into play for higher TX power for power class 2 capable bands

  - Neglect the 3rd order triple beat α (TX26TX1) of 7th order non-linearity.

  - Detection -> Duplex Offset<3∙ULCAMBW+TXMBW2 +RXBW2

- Verify MSD test points including RB positions and consider following framework for specification.

  - If FDD victim TX band allocation is fully allocated, then RB positions in table are chosen so that ULCA tone spacing is at the duplex offset

  - Tone spacing and RB positions also chosen for least amount of back-off to meet emission requirements

# WF on TP Guidelines

## Triple Beat with 2TX band and 3ULCC with at least 1 FDD band

- ULCA band (TDD or FDD)

    - ULCA BW = CC1BW + W gap + CC2BW

    - In REL-17, eligible combinations are restricted to intra-band contiguous UL CA, so Wgap is not applicable for REL-17 combinations [6].

- Non-ULCA band

    - Must be FDD band

    - Duplex Offset

    - RX BW

    - TX MBW

- Triple Beat Condition

    - 1st order is TRUE if Duplex Offset < ULCA BW + FDD RXBW/2 + FDD TX MBW/2

    - and

    - The non-ULCA band must be a FDD band

    - Only Investigate 1st order triple beat and further check if MSD for 2nd order triple beat is required

    - Example of condition for band combinations is shown in table below

## Triple Beat Test Point Parameters

- ULCA band (TDD or FDD) Parameters

    - Must have non-contiguous RB allocation in UL LCRB (ie. 1RB + 1RB)

    - Duplex mode (ie. FDD or TDD)

    - Spacing between RB allocations in each CC must equal the FDD Victim band duplex offset

    - UL CC1 BW and UL CC1 Fc

    - UL CC2 BW and UL CC2 Fc

- Non-ULCA FDD band Parameters

    - DL Fc and UL Fc

    - TX LCRB

    - RX BW

    - TX BW

- Triple Beat Order

    - 1st order triple beat is for 3rd order non-linearity

    - Further investigate if 2nd order triple beat (5th order non-linearity) is required

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | Triple beat order |
| DC_3A-n41C | 3 | 1782.5 | 5 | 25 | 1877.5 | [12.7] | FDD | 1 |
|  | n41C | 2555 2635 | [80] [80] | [1 (RBstart=88)] [1 (RBstart=128)] | 2555 2635 | N/A | TDD | N/A |
| DC_25A-n41C | 25 | 1912.5 | 5 | 25 | 1992.5 | [5.4] | FDD | 1 |
|  | n41C | 2545 2595 | 90 100 | 1 (RBstart=150) 1 (RBstart=122) | 2545 2595 | N/A | TDD | N/A |
| DC_8A-n79C | 8 | 912.5 | 5 | 25 | 957.5 | [0] | FDD | 1 |
|  | n79C | 4545 4645 | 100 100 | 1 (RBstart=212) 1 (RBstart=60) | 4545 4645 | N/A | TDD | N/A |

## WF Recommendation of TB MSD Framework

- TB MSD evaluation is not trivial as several coupling paths need to be evaluated: FWD and REV IMD for each PA + LNA contribution. Depending on the coupling isolation assumptions, reverse TB IMD may play an important role.

- On one coupling path, there seem to agreements between two contributing companies for DC_3A_n41C. [1] [4]

- Considering the short time left to complete the missing REL-17 TB MSD analysis, the remaining task seems challenging.

- These guidelines are intended to help companies bring MSD analysis.

## Examples using minimum parameters for triple beat MSD

- Power backoff for each of 3CCs

  - Must not violate the power class

  - MSD is worst case when good filter is used to minimize back-off

- TB IMD at PA output

- PA-PA PCB rejection for forward PA triple beat

- Filter rejections

  - Triple beat rejection from PA to antenna

  - Reverse PA TX filter rejection

  - TX-RX filter rejections for TX at RX LNA input (to calculate triple beat for RX LNA)

- TB IP3 for LNA

- MSD with some example parameters is shown for DC_3_n41C and DC_25_n41C in table below

# References

[1] R4-2107627, Triple beat and 3ULCC MSD, Qualcomm Incorporated, R4#99-e

[2] R4-2111016 MSD Due to NR Intra-band ULCA IMD within Inter-band Combinations, Skyworks, R4#99-e

[3] R4-2105335 Way forward on analysis and framework of triple beat issue of 3CC UL with contiguous intra band UL CA , Qualcomm Inc., R4#98bis-e

[4] R4-2202154 rev of R4-2202034 Triple beat B3 MSD evaluation for DC_3A_n41C, Skyworks, R4#101Bis-e

[5] R4-2107803 WF on MSD due to triple beat of intra-band UL CA UL + FDD UL configurations, Skyworks, Qualcomm, R4#99-e

[6] RP-210639, Guidelines for FR1 CA/DC band combinations not subject to block approval, 3GPP TSG-RAN #91-e , Apple Inc., Skyworks Solutions Inc.