3GPP TSG-RAN WG4 Meeting #101-bis-e 												R4-2202554

Electronic Meeting, 17 - 25 January, 2022

Agenda item:			6.8.1 & 6.8.2

Source:	Moderator (CMCC)

Title:	Email discussion summary for [101-bis-e][203] NR_HST_FR1_enh_RRM

Document for:	Information

Introduction

This email discussion focuses on RRM for Rel-17 NR FR1 HST, and in particular the agenda items:

6.8.1 General

6.8.2 RRM core requirements

6.8.2.1 Intra-frequency measurements

6.8.2.2 Inter-frequency measurements

6.8.2.3 L1-SINR measurements

6.8.2.4 Other

Please note that the UE feature for FR1 HST RRM enhancement is discussed under Topic #5.

The targets of email discussion for 1st round and 2nd round are:

- 1st round: focus on discussing the open issues and strive to minimize the open issues

- 2nd round: according to 1st round discussion, discuss left open issues for 2nd round, and strive to agree on the draft CRs.

Topic #1: intra-frequency measurements

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2200626 | CMCC | Draft CR on NSCC_SSB for CSSFoutside_gap,i |

Open issues summary

N/A

Companies views’ collection for 1st round

Open issues

CRs/TPs comments collection

For close-to-finalize WIs and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2200626 (CMCC) | QC: This CR is agreeable. |
|  | Ericsson: This CR is agreeable. |
|  | MTK: This CR is agreeable. |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic #1 | N/A |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

Discussion on 2nd round (if applicable)

N/A

Topic #2: inter-frequency measurements

Main technical topic overview. The structure can be done based on sub-agenda basis.

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2200062 | MediaTek (Shenzhen) Inc. | Proposal 1: PSS/SSS detection time requirement for inter-frequency measurement with MG in connected state for HST when DRX cycle>320ms is defined as: - 6 x DRX cycle x CSSFinter if SMTC periodicity > 40 ms - 5 x DRX cycle x CSSFinter if SMTC periodicity ≤ 40 ms Observation 1: Considering the carrier frequency (2.1GHz for 15kHz and 3.6GHz for 30kHz) and UE speed (500 km/hr), the carrier frequency offset should be 1944Hz and 3333Hz. Observation 2: For the channel model in HST, it should be AWGN with 2*Doppler shift. Observation 3: For the frequency tracking, UE is not required to perform the frequency compensation for the SS-SINR and L1-SINR measurement. Observation 4: For L1-SINR in R16 eMIMO, the average accuracy are 2.1 dB and 1.9 dB in the baseband for 15kHz and 30kHz, respectively. Proposal 2: For L1-SINR measurement in HST, the upper bound of the side condition CSI-RS CMR Ês/Iot ≤ 5 dB should be introduced, for CMR only case at least. FFS the cases with dedicated IMR. Observation 5: The IE highSpeedMeasFlag-r16 cannot be used for the indication of the enhanced requirements for inter-frequency without modification. Proposal 3: For CONNECTED mode, introduce a new network signalling to indicate the application of the enhanced requirements for inter-frequency. |
| R4-2200095 | CATT | Proposal 1: PSS/SSS detection time requirement for inter-frequency measurement with MG in connected state for HST DRX cycle; T PSS/SSS_sync_inter for FR1 HST. No DRX; max(600ms, N1 Max(MGRP, SMTC period)) CSSFinter N1 = 7. DRX cycle ≤ 160ms; max(600ms, ceil(N2) x max(MGRP, SMTC period, DRX cycle)) x CSSFinter N2 = 7 x M2. 160ms < DRX cycle ≤ 320ms; ceil(N3) x DRX cycle x CSSFinter N3 = 7 x M2. DRX cycle>320ms; N4 DRX cycle CSSFinter N4 = 6 if SMTC periodicity>40ms N4 = 5 if SMTC periodicity≤40ms. NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: M2 = 1.5 if SMTC periodicity > 40 ms, otherwise M2=1. |
| R4-2200323 | Qualcomm, Inc. | Condition NOTE1,2; TPSS/SSS_sync_inter. No DRX; Max(600ms, 7 Max(MGRP, SMTC period)) CSSFinter. DRX cycle ≤ 320ms; Max(600ms, Ceil(7*M2 Note 3) Max(MGRP, SMTC period, DRX cycle)) CSSFinter. DRX cycle > 320ms; 7 DRX cycle CSSFinter. NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. NOTE 3: M2 = 1.5 if SMTC periodicity > 40 ms, otherwise M2=1. Table 2-3 PSS/SSS detection time for inter-frequency measurement requirement in HST Condition NOTE1,2; TSSB_time_index_inter. No DRX; Max(120ms, 3 Max(MGRP, SMTC period)) CSSFinter. DRX cycle ≤ 320ms; Max(120ms, Ceil(3 M2 Note 3) Max(MGRP, SMTC period, DRX cycle)) CSSFinter. DRX cycle > 320ms; 3 DRX cycle CSSFinter. NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. NOTE 3: M2 = 1.5 if SMTC periodicity > 40 ms, otherwise M2=1. Table 2-4 SSB index reading time for inter-frequency measurement requirement in HST Proposal 1: Set HST connected mode inter-frequency measurement requirement as Table 2-3,4. Proposal 2: New flag for inter-frequency measurement enhancement signaling is necessary. Proposal 3: SNR upper bound for L1-RSRP measurement requirement should follow R16 SS-SINR. |
| R4-2200635 | CMCC | Proposal 1: for inter-frequency measurement with MG in connected state for HST, for DRX cycle>320ms, PSS/SSS detection time is proposed to be 4 * M2, M2 = 1.5 if SMTC periodicity > 40 ms, otherwise M2=1 |
| R4-2200670 | Apple | Proposal 1: PSS/SSS detection time requirement for inter-frequency measurement with MG in connected state for HST for DRX cycle > 320ms, T PSS/SSS_sync_inter is defined as: DRX cycle; T PSS/SSS_sync_inter for FR1 HST. DRX cycle>320ms; N4 x DRX cycle x CSSFinter N4 = 4 x M2. NOTE: M2 = 1.5 if SMTC periodicity > 40 ms, otherwise M2=1. |
| R4-2200876 | Ericsson | Proposal 1: Regarding enhancement for inter-frequency measurement in idle mode, keep values in below table: DRX cycle length [s]; Tdetect,NR_Inter [s] (number of DRX cycles); Tmeasure,NR_Inter [s] (number of DRX cycles); Tevaluate,NR_Inter [s] (number of DRX cycles). 0.32; 3.2 x M2 (10 x M2); 0.32 x M3 (1 x M3); 0.96 x M4 (3 x M4). 0.64; 6.4 (10); 0.64 (1); 1.92 (3). 1.28; 10.24 (8); 1.28 (1); 3.84 (3). 2.56; 58.88 (23); 2.56 (1); 7.68 (3). Note 1: when SMTC < = 40 ms, M2 = M3 = M4 = 1; and when SMTC > 40 ms, M2 = 1.5, M3 = M4 = 2. Proposal 2: Regarding inter-frequency measurement with MG, T PSS/SSS_sync_inter when DRX cycle>320ms is specified as: DRX cycle; T PSS/SSS_sync_inter for FR1 HST. No DRX; max(600ms, N1 Max(MGRP, SMTC period)) CSSFinter N1 = 7. DRX cycle ≤ 160ms; max(600ms, ceil(N2) x max(MGRP, SMTC period, DRX cycle)) x CSSFinter N2 = 7 x M2. 160ms < DRX cycle ≤ 320ms; ceil(N3) x DRX cycle x CSSFinter N3 = 7 x M2. DRX cycle>320ms; N4 x DRX cycle x CSSFinter N4 = 4 x M2. NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: M2 = 1.5 if SMTC periodicity > 40 ms, otherwise M2=1. |
| R4-2201172 | Huawei, Hisilicon | Proposal 1: The PSS/SSS detection delay for inter-frequency with gap in FR1 HST can be defined as, DRX cycle; T PSS/SSS_sync_inter for FR1 HST. No DRX; max(600ms, N1 Max(MGRP, SMTC period)) CSSFinter N1 = 7. DRX cycle ≤ 160ms; max(600ms, ceil(N2) x max(MGRP, SMTC period, DRX cycle)) x CSSFinter N2 = 7 x M2. 160ms < DRX cycle ≤ 320ms; ceil(N3) x DRX cycle x CSSFinter N3 = 7 x M2. DRX cycle>320ms; N4 x DRX cycle x CSSFinter N4 = 4 x M2. NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: M2 = 1.5 if SMTC periodicity > 40 ms, otherwise M2=1. |
| R4-2201173 | Huawei, Hisilicon | Draft CR on inter-frequency measurements for FR1 HST |

Open issues summary

Sub-topic 2-1: inter-frequency measurement with MG, connected state

Background: agreements in last meeting (R4-2120285) PSS/SSS detection time requirement for inter-frequency measurement with MG in connected state for HST - Agreements: DRX cycle; T PSS/SSS_sync_inter for FR1 HST. No DRX; max(600ms, N1 Max(MGRP, SMTC period)) CSSFinter N1 = 7. DRX cycle ≤ 160ms; max(600ms, ceil(N2) x max(MGRP, SMTC period, DRX cycle)) x CSSFinter N2 = 7 x M2. 160ms < DRX cycle ≤ 320ms; ceil(N3) x DRX cycle x CSSFinter N3 = 7 x M2. DRX cycle>320ms; FFS. NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: M2 = 1.5 if SMTC periodicity > 40 ms, otherwise M2=1.

Issue 2-1-1: for DRX cycle > 320ms, PSS/SSS detection time requirement (TPSS/SSS_sync_inter) for inter-frequency measurement with MG in connected state for HST

- Proposals

  - Option 1 (MTK, CATT):

    - 6 x DRX cycle x CSSFinter if SMTC periodicity > 40 ms

    - 5 x DRX cycle x CSSFinter if SMTC periodicity ≤ 40 ms

  - Option 2 (QC): 7  DRX cycle  CSSFinter

  - Option 3 (CMCC, Apple, Ericsson, HW): 4 x M2 x DRX cycle x CSSFinter, M2 = 1.5 if SMTC periodicity > 40 ms, otherwise M2=1

- Recommended WF

  - More discussion is needed.

| Issue 2-1-1: for DRX cycle > 320ms, PSS/SSS detection time requirement (TPSS/SSS_sync_inter) for inter-frequency measurement with MG in connected state for HST |  |
| --- | --- |
| Company | Comments |
| QC | Intra-frequency HST PSS/SSS detection is DRX cycle>320ms; ceil(5 x Kp) x DRX cycle x CSSFintra. Option 3 runs faster than intra frequency. We are open to discuss between option 1 and 2, but option 3 requires modifications. |
| CMCC | Option 3. According to the existing inter-frequency measurement for non-HST, the number of samples for PSS/SSS detection are the same as that for measurement. It is suggested to follow the same approach for HST. From this point of view, option 3 is preferred. |
| Nokia | Support option 3 which is aligned with the inter-frequency measurement requirement for HST. The open issue raised by QC is legitimate but it is OK for SMTC > 40 ms. It is highly unlike network will configure DRX cycle > 320 ms in combination with SMTC ≤ 40 ms for HST. So, there is no concern. |
| Ericsson | Support Option 3. We also think that the situation raised by QC is rare. But it is OK to update note to mitigate the risk. |
| CATT | Support option 1. The difference between option 1 and option 3 is “4 or 5” if SMTC periodicity ≤ 40 ms. 4 is shorter than intra-frequency measurement which break the general rules. |
| MTK | Option 1. As pointed by QC and CATT, option 3 is faster than intra-frequency measurement. It is better to follow the general rule that inter-frequency measurement is not faster than intra-frequency measurement. |
| Huawei | Option 1 and option 3 are fine. |
| Apple | We proposed option 3 in our contribution by applying same requirement for inter-frequency measurement. However, we understand concern from QC, CATT and MTK that option 3 is faster than inter-frequency measurement. Some modification is acceptable. E.g. option 1 is also fine. |
| vivo | Support option 1. |

Companies views’ collection for 1st round

Open issues

CRs/TPs comments collection

For close-to-finalize Wis and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing Wis, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2201173 (Huawei, Hisilicon) | QC: May need to add UE capability when it is agreed. |
|  | nokia: The CR is pending the outcome of Issue 2-1-1. |
|  | MTK: Pending the outcome of Issue 2-1-1. |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic #2-1 inter-frequency measurement with MG, connected state | Issue 2-1-1: for DRX cycle > 320ms, PSS/SSS detection time requirement (TPSS/SSS_sync_inter) for inter-frequency measurement with MG in connected state for HST Candidate options: - Option 1 (MTK, CATT, QC, HW, Apple, vivo): - 6 x DRX cycle x CSSFinter if SMTC periodicity > 40 ms - 5 x DRX cycle x CSSFinter if SMTC periodicity ≤ 40 ms - Option 2 (QC): 7 DRX cycle CSSFinter - Option 3 (CMCC, Apple, Ericsson, HW, Nokia): 4 x M2 x DRX cycle x CSSFinter, M2 = 1.5 if SMTC periodicity > 40 ms, otherwise M2=1 The difference between option 1 and option 3 is “4 or 5” for SMTC periodicity <= 40 ms. Companies mentioned that inter-frequency measurement is not expected to be faster than intra-frequency measurement. From this point of view, option 1 seems more suitable. In 2nd round, companies are suggested to check whether option 1 is agreeable. Recommendations for 2nd round: Issue 2-1-1: for DRX cycle > 320ms, PSS/SSS detection time requirement (TPSS/SSS_sync_inter) for inter-frequency measurement with MG in connected state for HST: Companies are suggested to check whether following suggestion (option 1) is agreeable. - 6 x DRX cycle x CSSFinter, if SMTC periodicity > 40 ms - 5 x DRX cycle x CSSFinter, if SMTC periodicity ≤ 40 ms |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

Discussion on 2nd round (if applicable)

Sub-topic 2-1: inter-frequency measurement with MG, connected state

#### Issue 2-1-1: for DRX cycle > 320ms, PSS/SSS detection time requirement (TPSS/SSS_sync_inter) for inter-frequency measurement with MG in connected state for HST:

Companies are suggested to check whether following suggestion (option 1 in 1st round) is agreeable.

- 6 x DRX cycle x CSSFinter, if SMTC periodicity > 40 ms

- 5 x DRX cycle x CSSFinter, if SMTC periodicity ≤ 40 ms

| Issue 2-1-1: for DRX cycle > 320ms, PSS/SSS detection time requirement (TPSS/SSS_sync_inter) for inter-frequency measurement with MG in connected state for HST |  |
| --- | --- |
| Company | Comments |

Topic #3: other

Main technical topic overview. The structure can be done based on sub-agenda basis.

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2200096 | CATT | Proposal 1: Existing L1-SINR measurement requirements are reused for HST, and the upper bound of the side condition is 5dB. |
| R4-2200097 | CATT | Proposal 1: introduce a new network signalling to indicate the application of the enhanced requirements for inter-frequency. Proposal 2: The signalling can be a cell-level signalling. |
| R4-2200246 | Apple | Proposal 1: introduce a new network signalling to indicate the application of the enhanced requirements for inter-frequency. Proposal 2: if new signalling in P1 is agreed, the indication is per inter-frequency layer configuration. |
| R4-2200247 | Apple | CR on L1-RSRP measurement in FR1 HST |
| R4-2200596 | vivo | Proposal 1 L1-SINR measurements should be applicable to FR1 HST. Observation 1 RRM requirements was defined for CSIRS-based CMR-only L1-SINR measurements in R16 eMIMO. Observation 2 According to demod discussion in R16 HST, the baseline scenario for defining requirements should be DPS 1a, which is for UE only capable of tracking 1 TCI state. Observation 3 According to TS 38.101-4, 972Hz Doppler shift for SCS15kHz HST single tap is only for performance verification, and it is not meant to indicate the max Doppler shift UE needs to dealt with in real SCS15kHz HST deployment. Observation 4 According to TRS pattern specified in RAN1 specs, the frequency track ability of TRS is +/-1750Hz for SCS15kHz, and +/-3500Hz for SCS30kHz. Observation 5 In R16 SS-SINR accuracy evaluations for HST, no performance degradation is shown on the serving cell SS-SINR measurement if the one-tap scenario is considered, even assuming 1944Hz Doppler shift. Observation 6 The considered scenario in R17 is different, since CSI-RSs are UE-specific RS, but SSBs are cell-specific RSs. Proposal 2 RAN4 confirm to prioritize the case that UE is capable of tracking only 1 TCI for L1-SINR measurements in HST scenario. Proposal 3 RAN4 may add a note in TS 38.133 to the L1-SINR measurement accuracy requirements, ‘For L1-SINR measurements requirements in HST scenario, the considered max doppler shift should not beyond 1750Hz, at least for UE that supports tracking only 1 TCI state.’ Proposal 4 No need to add any upper bound to the side condition of L1-SINR requirements under FR1 HST scenario. Proposal 5 Remove the evaluation assumptions on propagation condition or revise it to Table B3.3-1 in TS 38.101-4. Observation 7 Significant performance degradation can be observed when Doppler spread is 1944Hz and SCS is 15kHz, which is caused by disfunction of TRS-based frequency tracking. |
| R4-2200634 | CMCC | Proposal 1: Existing L1-SINR measurement requirements are reused for HST, and the upper bound of the side condition is 5dB Proposal 2: for inter-frequency measurement enhancement in connected mode, it is proposed to introduce a new per-cell level network signalling to indicate UE that it is in the high speed condition and the enhanced requirements need to be applied (i.e. no need to reuse the network signalling for CA enhancement). Proposal 3: it is proposed that Rel-17 FR1 HST RRM enhancement, including CA enhancement and inter-frequency measurement enhancement, is release independent from Rel-15. |
| R4-2200671 | Apple | Proposal 1: Existing L1-SINR measurement requirements are reused for HST, and the upper bound of the side condition is 5dB. |
| R4-2200874 | Ericsson | Proposal 1: For Q1 in network signalling for CA enhancement, we can compromise and support Option 2. Proposal 2: For Q2 in network signalling for CA enhancement, support Option 1. With respect to agreement on Issue 3-4-2, the signaling is cell-level signaling. |
| R4-2200875 | Ericsson | Proposal 1: Support option 1, Existing L1-SINR measurement requirements are reused for HST. The upper bound of the side condition is slightly different from that for SS-SINR, but 5dB is acceptable for simplification. |
| R4-2201132 | OPPO | Proposal 1: If L1-SINR measurement is applicable to HST, existing L1-SINR accuracy requirements can be reused for high speed train scenario. |
| R4-2201133 | OPPO | Proposal 1: Introduce a new cell-level signalling to indicate the application of the enhanced requirements for inter-frequency. Proposal 2: Rel-17 NR HST RRM enhancement can be release independent from R17. |
| R4-2201174 | Huawei, Hisilicon | Proposal 1: If RAN4 is agreed to introduce L1-SINR in R17 HST FR1, the upper bound of the side condition can be 5dB. Proposal 2: In connected mode, the network signalling to indicate inter-frequency measurement can be per cell level. The concrete signalling design is up to RAN2. Proposal 3: Whether release independent is supported or not shall consider both RRM requirements and demodulation requirements |
| R4-2201582 | Nokia, Nokia Shanghai Bell | Proposal 1: Current L1-SINR measurement requirement can be reused in HST, no upper bound of side condition. |
| R4-2200062 | MediaTek (Shenzhen) Inc. | Proposal 1: PSS/SSS detection time requirement for inter-frequency measurement with MG in connected state for HST when DRX cycle>320ms is defined as: - 6 x DRX cycle x CSSFinter if SMTC periodicity > 40 ms - 5 x DRX cycle x CSSFinter if SMTC periodicity ≤ 40 ms Observation 1: Considering the carrier frequency (2.1GHz for 15kHz and 3.6GHz for 30kHz) and UE speed (500 km/hr), the carrier frequency offset should be 1944Hz and 3333Hz. Observation 2: For the channel model in HST, it should be AWGN with 2*Doppler shift. Observation 3: For the frequency tracking, UE is not required to perform the frequency compensation for the SS-SINR and L1-SINR measurement. Observation 4: For L1-SINR in R16 eMIMO, the average accuracy are 2.1 dB and 1.9 dB in the baseband for 15kHz and 30kHz, respectively. Proposal 2: For L1-SINR measurement in HST, the upper bound of the side condition CSI-RS CMR Ês/Iot ≤ 5 dB should be introduced, for CMR only case at least. FFS the cases with dedicated IMR. Observation 5: The IE highSpeedMeasFlag-r16 cannot be used for the indication of the enhanced requirements for inter-frequency without modification. Proposal 3: For CONNECTED mode, introduce a new network signalling to indicate the application of the enhanced requirements for inter-frequency. |
| R4-2200323 | Qualcomm, Inc. | Condition NOTE1,2; TPSS/SSS_sync_inter. No DRX; Max(600ms, 7 Max(MGRP, SMTC period)) CSSFinter. DRX cycle ≤ 320ms; Max(600ms, Ceil(7*M2 Note 3) Max(MGRP, SMTC period, DRX cycle)) CSSFinter. DRX cycle > 320ms; 7 DRX cycle CSSFinter. NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. NOTE 3: M2 = 1.5 if SMTC periodicity > 40 ms, otherwise M2=1. Table 2-3 PSS/SSS detection time for inter-frequency measurement requirement in HST Condition NOTE1,2; TSSB_time_index_inter. No DRX; Max(120ms, 3 Max(MGRP, SMTC period)) CSSFinter. DRX cycle ≤ 320ms; Max(120ms, Ceil(3 M2 Note 3) Max(MGRP, SMTC period, DRX cycle)) CSSFinter. DRX cycle > 320ms; 3 DRX cycle CSSFinter. NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. NOTE 3: M2 = 1.5 if SMTC periodicity > 40 ms, otherwise M2=1. Table 2-4 SSB index reading time for inter-frequency measurement requirement in HST Proposal 1: Set HST connected mode inter-frequency measurement requirement as Table 2-3,4. Proposal 2: New flag for inter-frequency measurement enhancement signaling is necessary. Proposal 3: SNR upper bound for L1-RSRP measurement requirement should follow R16 SS-SINR. |

Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Sub-topic 3-1: L1-SINR

Issue 3-1-1: how to specify the L1-SINR requirements for high speed scenario

- Proposals

  - Option 1 (MTK, QC, CATT, CMCC, Apple, Ericsson, HW): Existing L1-SINR measurement requirements are reused for HST, and the upper bound of the side condition is 5dB

    - Option 1a (MTK) the upper bound of the side condition CSI-RS CMR Ês/Iot ≤5 dB should be introduced, for CMR only case at least. FFS the cases with dedicated IMR

  - Option 2 (vivo, OPPO, Nokia): Existing L1-SINR measurement requirements are reused for HST, no need to add any upper bound to the side condition of L1-SINR requirements under FR1 HST scenario

- Recommended WF

  - More discussion is needed

| Issue 3-1-1: how to specify the L1-SINR requirements for high speed scenario |  |
| --- | --- |
| Company | Comments |
| QC | As we explained in our contribution, upper bound is due to measurement accuracy degradation in interference with Doppler shift. Therefore, we support option 1. |
| CMCC | Option 1. Our initial consideration is that no upper bound is needed. However, considering that even though L1-SINR measurement is performed on serving cell, the received signal may come from different RRH which may also result in ISI due to the larger Doppler spread between different RRHs. Based on this consideration, we are OK to have the side condition as SSB Ês/Iot ≤5 dB for L1-SINR. For option 1a, we do not see the necessity to have different consideration for CMR only case and dedicated IMR case. All the cases can be supported. |
| Nokia | Support Option 2 as there are no technical constraints in setting the side condition to 5 dB for the following reasons: - Even though UE experiences twice the maximum Doppler shifts, our simulation results clear show that the existing L1-SINR accuracy requirements can be reused if Doppler shift is compensated using, e.g., TRS which is made available by network. - The L1-SINR accuracy requirements were specified in Rel-16 eMIMO WI for beam management only. For FR1 HST, it is the same; the requirement should be applied to beam management and not to neighbouring cell measurement. |
| Ericsson | Support Option1. We have agreed to not prelude L1-SINR in HST scenario. Similar Doppler shift situation may happen on L1-SINR also. |
| CATT | Support option 1. The L1-SINR has the same Doppler shift issue. |
| MTK | Option 1. Due to large ICI, measurement accuracy degradation is expected. |
| OPPO | For R16 L1-SINR measurement, the requirements were defined without considering larger doppler shift than TRS tracking ability. The impact of pre-compensation and UE capability of tracking only 1 TCI for L1-SINR measurements in HST scenario could also be considered. At least we see no performance degradation of L1-SINR measurement on the serving cell. Regarding L1-SINR measurement is to be supported in R17 HST, we agree that existing L1-SINR measurement requirements are reused for HST. Furthermore, either involving some additional restrictions of doppler shift, network and UE assumption, or defining upper bound of the side condition (e.g., 5dB) is fine to us. |
| Huawei | Support option1. Even for serving cell, frequency pre-compensation is performed, there is still residual. |
| Apple | Support option 1. |
| vivo | Support option 2. Firstly, we do not think the doppler shift will impact the L1-SINR measurement accuracy if CMR is QCLed to the source RS of active TCI(s), as analyzed in both vivo’s paper (supporting option 2) and MTK’s paper (supporting option 1). Secondly, if CMR is not QCLed to the source RS of active TCI, but the corresponding QCL relation is correctly configured and the doppler shift is within TRS tracking ability, perhaps in our understanding UE is still able to compensate the residual, and hence there is no performance degradation. Thirdly, we see only one proponent of option 1 has provided evaluation results, while 2 proponents of option 2 have provided evaluation results. We have concerns if RAN4 just concludes on issues based on majority but not analysis and evaluations. |
| Intel | We support option 1. |

Sub-topic 3-2: signalling

Issue 3-2-1: whether the network signalling for CA enhancement can be reused for inter-frequency measurement

- Proposals

  - Option 1 (MTK, QC, CATT, Apple, CMCC, Ericsson, OPPO): No, introduce new network signalling

- Recommended WF

  - Companies are suggested to check whether following suggestion is agreeable

    - For inter-frequency measurement enhancement in connected mode, introduce new network signalling

| Issue 3-2-1: whether the network signalling for CA enhancement can be reused for inter-frequency measurement |  |
| --- | --- |
| Company | Comments |
| QC | Support recommended WF. |
| CMCC | OK with recommended WF. |
| Nokia | The recommended WF is Ok. |
| OPPO | OK with recommended WF. |
| Ericsson | OK with recommended WF. |
| CATT | Support Recommended WF. |
| MTK | Support Recommended WF. |
| Huawei | Support recommended WF. |
| Apple | Support recommended WF. |
| vivo | Support recommended WF. |

Issue 3-2-2: if new signalling is agreed for inter-frequency measurement, whether it is per cell level or per frequency layer level

Background: agreements in last meeting (R4-2120285) - For connected state, NW shall NOT indicate UE for which inter-frequency carriers, the enhanced inter-frequency measurement requirements shall be applied. - It is RAN4 understanding that the requirements will be defined under assumption that all carriers in the connected mode are high-speed train carriers and enhanced inter-frequency measurement requirements shall be applied

- Proposals

  - Option 1 (CATT, CMCC, Ericsson, OPPO, HW): The signalling is a cell-level signalling.

  - Option 2 (Apple): the indication is per inter-frequency layer configuration

- Recommended WF

  - In last meeting, it was agreed that no need to have per-carrier indication. Also, for Issue 3-2-1, it seems all companies are OK to introduce new network signalling for inter-frequency measurement. Based on above consideration, companies are suggested to check whether following suggestion is agreeable

    - For inter-frequency measurement enhancement in connected mode, the network signalling is a per-cell signalling

| Issue 3-2-2: if new signalling is agreed for inter-frequency measurement, whether it is per cell level or per frequency layer level |  |
| --- | --- |
| Company | Comments |
| QC | Support recommended WF. |
| CMCC | OK with the recommended WF. Considering the agreement in last meeting that no need to have per-carrier indication, we support the recommended WF. |
| Nokia | Option 1 or Option 2. |
| OPPO | OK with recommended WF. |
| Ericsson | OK with recommended WF. |
| CATT | Support Recommended WF. |
| MTK | Support Recommended WF. |
| Huawei | Support Recommended WF. |
| Apple | We propose per carrier indication just to align with idle mode design. Seems most companies have different view, we are fine with other alternative. One suggestion on the wording: change ‘per-cell’ to ‘cell specific’. Because ‘per-cell’ signaling implies NW may provide different indications for different cell. That’s not the intention according to contributions from companies. |
| vivo | Support recommended WF. |

Sub-topic 3-3: release independent

Issue 3-3-1: release independent

- Proposals

  - Option 1 (CMCC): Rel-17 FR1 HST RRM enhancement, including CA enhancement and inter-frequency measurement enhancement, is release independent from Rel-15

  - Option 2 (HW): Whether release independent is supported or not shall consider both RRM requirements and demodulation requirements

  - Option 3 (OPPO): Rel-17 NR HST RRM enhancement can be release independent from R17

- Recommended WF

  - More discussion is needed

| Issue 3-3-1: release independent |  |
| --- | --- |
| Company | Comments |
| QC | We suggest to consider release independent from R16, since R16 introduce SC feature, release independence from R16 for CA feature is more reasonable. |
| CMCC | Option 1. Firstly, we see the necessity to have Rel-17 FR1 HST RRM enhancement to be release independent from Rel-15. High speed scenario is an important deployed scenario. And the improvement of UE experience is necessary. Secondly, for option 2, we agree that whether HST enhancement can be release independent need to consider both RRM and demodulation. But for this RRM email thread, it is suggested to focus on the discussion from RRM point of view. It is not preferred to mix the discussion between RRM and demodulation. Once we have conclusion separately in demodulation session and RRM session, we can further consider whether the whole feature can be release independent taking the conclusions from both sessions into account. For option 3, since FR1 HST enhancement is a Rel-17 WI, no need to say that it is release independent from R17. |
| OPPO | The discussion on UE feature in issue 5-1-1 may have impact. For CA enhancement or inter-frequency measurement enhancement, they could be agreed as optional with or without capability signaling case by case. We can accept release independent from R16. |
| Huawei | Support option 2. Release independent for CA demodulation is also on-going in demodulation session. It is weird that RRM feature is from R15, and demodulation is from other release version (e.g., R17). From overall performance perspective, it means R15 UE can measure fast however actual data performance can not keep up after handover. |
| vivo | Option 2 make sense and needs to be further considered. |
| Intel | We respect the need from CMCC to introduce the requirements in early releases. One question is how does R15 UE indicate support for the requirements specified in R17? Is it allowed for the R15 to indicate a R17 signalling? Or we could apply the requirements from R16 where the prerequisite features are introduced and further specify the relation between R16 and R17 features. |

Companies views’ collection for 1st round

Open issues

CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2200247 (Apple) | QC: This CR is agreeable once the UE capability is confirmed. |
|  | Nokia: The CR is Ok. |
|  | MTK: The CR is ok once the signaling is confirmed. |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#3-1 L1-SINR | Issue 3-1-1: how to specify the L1-SINR requirements for high speed scenario Candidate options: - Option 1 (MTK, QC, CATT, CMCC, Apple, Ericsson, HW, OPPO, Intel): Existing L1-SINR measurement requirements are reused for HST, and the upper bound of the side condition is 5dB - Option 2 (vivo, OPPO, Nokia): Existing L1-SINR measurement requirements are reused for HST, no need to add any upper bound to the side condition of L1-SINR requirements under FR1 HST scenario According to companies’ comments, most companies prefer option 1. In 2nd round, companies are suggested to check whether option 1 is agreeable as compromise to move forward. Recommendations for 2nd round: Issue 3-1-1: how to specify the L1-SINR requirements for high speed scenario Companies are suggested to check whether following suggestion (option 1) is agreeable. - Existing L1-SINR measurement requirements are reused for HST, and the upper bound of the side condition is 5dB |
| Sub-topic#3-2 signalling | Issue 3-2-1: whether the network signalling for CA enhancement can be reused for inter-frequency measurement Tentative agreements: - For inter-frequency measurement enhancement in connected mode, introduce new network signalling (i.e. network signalling for CA enhancement cannot be reused) Issue 3-2-2: if new signalling is agreed for inter-frequency measurement, whether it is per cell level or per frequency layer level All companies are OK with the recommended WF, one company suggest to change the wording from “per cell” to “cell specific”. Tentative agreements: - For inter-frequency measurement enhancement in connected mode, the network signalling is a cell specific signalling |
| Sub-topic#3-3 release independent | Issue 3-3-1: release independent Candidate options: - Option 1 (CMCC): Rel-17 FR1 HST RRM enhancement, including CA enhancement and inter-frequency measurement enhancement, is release independent from Rel-15 - Option 2 (HW, vivo): Whether release independent is supported or not shall consider both RRM requirements and demodulation requirements - Option 3 (OPPO): Rel-17 NR HST RRM enhancement can be release independent from R17 - Option 4 (QC, OPPO): Rel-17 FR1 HST RRM enhancement, including CA enhancement and inter-frequency measurement enhancement, is release independent from Rel-16 Recommendations for 2nd round: Continue the discussion |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

Sub-topic 3-1: L1-SINR

#### Issue 3-1-1: how to specify the L1-SINR requirements for high speed scenario

Companies are suggested to check whether following suggestion (option 1) is agreeable.

- Existing L1-SINR measurement requirements are reused for HST, and the upper bound of the side condition is 5dB

| Issue 3-1-1: how to specify the L1-SINR requirements for high speed scenario |  |
| --- | --- |
| Company | Comments |

Sub-topic 3-3: release independent

#### Issue 3-3-1: release independent

- Option 1 (CMCC): Rel-17 FR1 HST RRM enhancement, including CA enhancement and inter-frequency measurement enhancement, is release independent from Rel-15

- Option 2 (HW, vivo): Whether release independent is supported or not shall consider both RRM requirements and demodulation requirements

- Option 3 (OPPO): Rel-17 NR HST RRM enhancement can be release independent from R17

- Option 4 (QC, OPPO): Rel-17 FR1 HST RRM enhancement, including CA enhancement and inter-frequency measurement enhancement, is release independent from Rel-16

| Issue 3-3-1: release independent |  |
| --- | --- |
| Company | Comments |

Topic #4: LS

Main technical topic overview. The structure can be done based on sub-agenda basis.

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2200634 | CMCC |  |

Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Sub-topic 4-1: LS to RAN2

Issue 4-1-1: content of LS

- Proposals

  - Option 1 (CMCC): draft LS in R4-2200634, Annex

- Recommended WF

  - Companies are suggested to provide comments to this draft LS

| Issue 4-1-1: content of LS |  |
| --- | --- |
| Company | Comments |
| QC | This LS is agreeable. |
| Nokia | The LS is pending the outcome of Issue 3-2-2. |
| OPPO | This LS is agreeable. |
| Ericsson | This LS is agreeable. |
| CATT | This LS is agreeable. |
| MTK | This LS is agreeable. |
| Huawei | This LS is agreeable. |
| Apple | In general the LS is agreeable to us. One suggestion on the wording as commented under issue 3-2-2: |
| vivo | This LS is agreeable. |

Companies views’ collection for 1st round

Open issues

CRs/TPs comments collection

Major close to finalize Wis and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going Wis, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| XXX | Company A |
|  | Company B |
| YYY | Company A |
|  | Company B |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#4-1 LS to RAN2 | All companies are OK with the content of LS (Annex in R4-2200634), one company provides suggestion on the wording. Recommendations for 2nd round: Responsible company updates the wording of the LS taking companies’ comments into account, and companies can have further check. |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

Topic #5: UE feature

Main technical topic overview. The structure can be done based on sub-agenda basis.

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2200641 | CMCC |  |
| R4-2200544 | Intel |  |

Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Sub-topic 5-1: UE feature for RRM enhancement for FR1 HST

Issue 5-1-1: UE feature for RRM enhancement for FR1 HST

- Proposals

  - Option 1 (CMCC):

| Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between Ues (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1-1 | Enhanced RRM requirements specified for CA for FR1 HST | The enhanced RRM requirements for CA to support high speed up to 500 km/h, as specified in TS 38.133 |  | Yes |  | The performance of RRM for CA in FR1 HST scenario cannot be guaranteed | Per UE | NO | FR1 only |  |  | optional |
| 1-2 | Enhanced RRM requirements specified for inter-frequency measurement in connected mode for FR1 HST | The enhanced RRM requirements for inter-frequency measurement in connected mode to support high speed up to 500 km/h, as specified in TS 38.133 |  | Yes |  | The performance of RRM for inter-frequency measurement in connected mode for FR1 HST cannot be guaranteed | Per UE | NO | FR1 only |  |  | optional |
| 1-3 | Enhanced RRM requirements specified for inter-frequency measurement in idle mode for FR1 HST | The enhanced RRM requirements for inter-frequency measurement in idle mode to support high speed up to 500 km/h, as specified in TS 38.133 |  | No |  | The performance of RRM for inter-frequency measurement in idle mode for FR1 HST cannot be guaranteed | Per UE | NO | FR1 only |  |  | Optional without UE capability |

  - Option 2 (Intel):

| Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between Ues (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| x-1 | Support of enhanced RRM requirements for CA in HST FR1 | 1) Support of enhanced RRM requirements for CA in HST FR1 | Rel-16 RAN4 feature 10-1, 10-4 or 10-5 | Yes | No | UE is not able to meet the enhanced RRM requirements for CA in HST FR1 | Per-UE | No | FR1 only | N/A |  | Optional with capability signalling |
| x-2 | Support of enhanced inter-frequency measurements in CONNECTED mode HST FR1 | 1) Support of enhanced inter-frequency measurements in HST FR1 in connected mode | Rel-16 RAN4 feature 10-1, 10-4 or 10-5 | Yes | No | UE is not able to meet the enhanced inter-frequency measurement requirements in CONNECTED mode HST FR1 | Per-UE | No | FR1 only | N/A |  | Optional with capability signalling |
| x-2a | Support of enhanced inter-frequency measurements in IDLE mode HST FR1 | 1) Support of enhanced inter-frequency measurements in HST FR1 in IDLE mode | Rel-16 RAN4 feature 10-1, 10-4 or 10-5 | No | No | UE is not able to meet the enhanced inter-frequency measurement requirements in IDLE mode HST FR1 | Per-UE | No | FR1 only | N/A |  | Optional without capability signalling |
| x-3 | Support of enhanced Demodulation requirements for CA in HST SFN FR1 | 1) Support of demodulation processing for HST SFN CA scenario in FR1 | Rel-16 RAN4 feature 10-2 | Yes | No | UE is not able to apply demodulation processing for HST SFN CA scenario in FR1 | Per-UE | No | FR1 only | N/A |  | Optional with capability signalling |

- Recommended WF

  - Companies are suggested to check whether following suggestion are agreeable:

| Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between Ues (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| x-1 | Enhanced RRM requirements specified for CA for FR1 HST | Support of the enhanced RRM requirements for CA to support FR1 high speed up to 500 km/h, as specified in TS 38.133 | [N/A] | Yes | No | The performance of RRM for CA in FR1 HST scenario cannot be guaranteed | Per UE | NO | FR1 only | N/A |  | Optional with capability signalling |
| x-2 | Enhanced RRM requirements specified for inter-frequency measurement in connected mode for FR1 HST | Support of the enhanced RRM requirements for inter-frequency measurement in connected mode to support FR1 high speed up to 500 km/h, as specified in TS 38.133 | [N/A] | Yes | No | The performance of RRM for inter-frequency measurement in connected mode for FR1 HST cannot be guaranteed | Per UE | NO | FR1 only | N/A |  | Optional with capability signalling |
| x-3 | Enhanced RRM requirements specified for inter-frequency measurement in idle mode for FR1 HST | Support of the enhanced RRM requirements for inter-frequency measurement in idle mode to support FR1 high speed up to 500 km/h, as specified in TS 38.133 | [N/A] | No | No | The performance of RRM for inter-frequency measurement in idle mode for FR1 HST cannot be guaranteed | Per UE | NO | FR1 only | N/A |  | Optional without capability signalling |

| Issue 5-1-1: UE feature for RRM enhancement for FR1 HST |  |
| --- | --- |
| Company | Comments |
| CMCC | OK with the recommended WF. |
| Ericsson | OK with the recommended WF. |
| MTK | OK with the recommended WF. |
| Huawei | OK with the recommended WF. |
| Apple | OK with the recommended WF. |
| vivo | Support option 1. OK with the recommended WF. |
| Intel | Clarification is needed on how the prerequisite features are not mentioned in the WF table? In addition for idle mode if we don’t have anything in the list it might cause confusion from previous experience. We don’t need a capability for IDLE mode but it could remain a feature in the list for everyone’s record. |

Companies views’ collection for 1st round

Open issues

CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| XXX | Company A |
|  | Company B |
| YYY | Company A |
|  | Company B |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#5-1 UE feature for RRM enhancement for FR1 HST | According to the comments, most parts of these UE features are agreeable, one company has concern on Prerequisite feature groups, which can be further discussed in 2nd round. Recommendations for 2nd round: Companies are suggested to provide comments on following questions: Q1: whether Prerequisite feature groups is needed or not - Option 1 (Intel): Yes, Prerequisite feature groups are Rel-16 RAN4 feature 10-1, 10-4 or 10-5 - Option 2: No Q2: moderator notices that for inter-frequency measurement, only connected state and idle state are mentioned, inactive state is missing. It is suggested to update the feature group on inter-frequency measurement in idle mode to cover inactive mode, and companies are suggested to check whether following updates are OK: |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

Sub-topic 5-1: UE feature for RRM enhancement for FR1 HST

#### Issue 5-1-1: UE feature for RRM enhancement for FR1 HST

| Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between Ues (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| x-1 | Enhanced RRM requirements specified for CA for FR1 HST | Support of the enhanced RRM requirements for CA to support FR1 high speed up to 500 km/h, as specified in TS 38.133 | [Rel-16 RAN4 feature 10-1, 10-4 or 10-5] | Yes | No | The performance of RRM for CA in FR1 HST scenario cannot be guaranteed | Per UE | NO | FR1 only | N/A |  | Optional with capability signalling |
| x-2 | Enhanced RRM requirements specified for inter-frequency measurement in connected mode for FR1 HST | Support of the enhanced RRM requirements for inter-frequency measurement in connected mode to support FR1 high speed up to 500 km/h, as specified in TS 38.133 | [Rel-16 RAN4 feature 10-1, 10-4 or 10-5] | Yes | No | The performance of RRM for inter-frequency measurement in connected mode for FR1 HST cannot be guaranteed | Per UE | NO | FR1 only | N/A |  | Optional with capability signalling |
| x-3 | Enhanced RRM requirements specified for inter-frequency measurement in idle/inactive mode for FR1 HST | Support of the enhanced RRM requirements for inter-frequency measurement in idle/inactive mode to support FR1 high speed up to 500 km/h, as specified in TS 38.133 | [Rel-16 RAN4 feature 10-1, 10-4 or 10-5] | No | No | The performance of RRM for inter-frequency measurement in idle/inactive mode for FR1 HST cannot be guaranteed | Per UE | NO | FR1 only | N/A |  | Optional without capability signalling |

Moderator: as highlighted in yellow in above feature group table, there are two questions, and companies are suggested to provide comments to the following questions.

#### Q1: whether Prerequisite feature groups is needed or not

  - Option 1 (Intel): Yes, Prerequisite feature groups are Rel-16 RAN4 feature 10-1, 10-4 or 10-5

  - Option 2: No

#### Q2: moderator notices that for inter-frequency measurement, only connected state and idle state are mentioned, inactive state is missing. It is suggested to update the feature group on inter-frequency measurement in idle mode to cover inactive mode, and companies are suggested to check whether following update is OK:

| Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between Ues (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| x-3 | Enhanced RRM requirements specified for inter-frequency measurement in idle/inactive mode for FR1 HST | Support of the enhanced RRM requirements for inter-frequency measurement in idle/inactive mode to support FR1 high speed up to 500 km/h, as specified in TS 38.133 |  | No | No | The performance of RRM for inter-frequency measurement in idle/inactive mode for FR1 HST cannot be guaranteed | Per UE | NO | FR1 only | N/A |  | Optional without capability signalling |

Recommendations for Tdocs

1st round

New tdocs

| Title | Source | Comments |
| --- | --- | --- |
| WF on RRM for FR1 HST | CMCC |  |
| LS on signalling for inter-frequency measurement enhancement in connected state for FR1 HST | CMCC | To: RAN2 |

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2200626 | Draft CR on Draft CR on NSCC_SSB for CSSFoutside_gap,i | CMCC | Agreeable |  |
| R4-2201173 | Draft CR on inter-frequency measurements for FR1 HST | Huawei, Hisilicon | Revised |  |
| R4-2200247 | CR on L1-RSRP measurement in FR1 HST | Apple | Agreeable |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

2nd round

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-210xxxx | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-210xxxx | WF on … | YYY | Agreeable, Revised, Noted |  |
| R4-210xxxx | LS on … | ZZZ | Agreeable, Revised, Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents

- Annex

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| CMCC | Jingjing Chen | chenjingjing@chinamobile.com |
| Ericsson | Ming Li | ming.l.li@ericsson.com |
| Intel | Meng Zhang | Meng.zhang@intel.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)