3GPP TSG-RAN4 Meeting #101bis-e	R4-2202652

# Electronic Meeting, January 17-25, 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | Draft CR | rev | 1 | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draft CR on interruption requirement for SL |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh-Core |  |  |  |  |  | Date: |  |  | 2022-01-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Add interruption requirements of NR SL enhancement. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add interruption requirement of NR SL enhancement in 12.7 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Miss interruption requirement of NR SL enhancment. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 12.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< START OF CHANGE #1 >

12	V2X Requirements

12.1	Introduction

This clause contains the requirements for the UE capable of V2X sidelink communication when the UE is out of coverage on the carrier used for V2X sidelink operation, as defined in TS 38.304 [1]. The requirements apply when the UE is:

-	in any cell selection state, or,

-	configured for V2X SL operation on a V2X carrier which is dedicated to only V2X SL operation and configured with only a PCell on WAN carrier.

Note:	Any cell selection state refers to a UE that is out of network coverage and is not associated with a serving cell on any carrier as defined in TS 38.304 [1].

Note:	When a UE in RRC_CONNECTED state is performing transmissions and/or reception for V2X sidelink communication, the UE shall meet all the requirements specified in Clause 9 assuming that UE has a dedicated RX/TX chain for V2X sidelink communication. Otherwise, the UE may interrupt the V2X sidelink communication in order to meet the measurement requirements specified in Clause 9.

For UE capable of Public Safety sidelink communication, unless explicitly stated, V2X requirements apply.

END OF CHANGE #1 >

< START OF CHANGE #2 >

12.7	Interruption

12.7.1	Interruptions to WAN due to V2X Sidelink Communication

This clause contains the requirements related to the interruptions on the PCell/serving cell due to V2X sidelink communication.

A UE capable of V2X sidelink communication may indicate its interest (initiation or termination) in V2X sidelink communication to the connected gNodeB using IE SidelinkUEInformationNR in TS38.331[2].

The UE is allowed an interruption of up to the duration shown in table 12.7.1-1 on the PCell/serving cell during the RRC reconfiguration procedure that includes the V2X sidelink communication configuration message SL-ConfigDedicatedNR in TS 38.331[2] (setup and release). This interruption is for both uplink and downlink of the PCell/serving cell.

Table 12.7.1-1: Interruption length at V2X RRC reconfiguration

|  | NR Slot length (ms) | Interruption length (number of slots) |
| --- | --- | --- |
| 0 | 1 | 2 |
| 1 | 0.5 | 3 |
| 2 | 0.25 | 5 |
| 3 | 0.125 | 9 |

12.7.2	V2X Sidelink Communication Dropping due to synchronization source change

This clause contains the requirements related to the interruptions on the V2X sidelink communication due to synchronization source change.

For NR V2X  UE not supporting gNB/eNB as synchronization reference source, UE is allowed to drop LTE and NR V2X SL transmission or reception for up to 1ms when synchronization source is changed, where the drop of  LTE V2X SL transmission or reception applies only to in-device coexistence scenario in TS38.213 [3]:

-	From GNSS

-	to syncRef UE that is synchronized to GNSS directly/in-directly

-	to syncRef UE that has the lowest priority

-	From syncRef UE that is synchronized to GNSS directly/in-directly

-	to GNSS

-	to syncRef UE that has the lowest priority

-	From syncRef UE that has the lowest priority

-	to GNSS

-	to syncRef UE that is synchronized to GNSS directly/in-directly

-	to syncRef UE that has the lowest priority

For NR V2X UE supporting gNB/eNB as synchronization reference source, UE is allowed to drop LTE and NR V2X SL transmission or reception for up to 1ms when synchronization source is changed, where the drop of  LTE V2X SL transmission or reception applies only to in-device coexistence scenario in TS38.213 [3]:

-	From GNSS

-	to syncRef UE that is synchronized to GNSS directly/in-directly

-	to gNB/eNB

-	to syncRef UE that is synchronized to gNB/eNB directly

-	to syncRef UE that is synchronized to gNB/eNB in-directly

-	to syncRef UE that has the lowest priority

-	From syncRef UE that is synchronized to GNSS directly/in-directly

-	to GNSS

-	to gNB/eNB

-	to syncRef UE that is synchronized to gNB/eNB directly

-	to syncRef UE that is synchronized to gNB/eNB in-directly

-	to syncRef UE that has the lowest priority

-	From gNB or eNB

-	to GNSS

-	to syncRef UE that is synchronized to GNSS directly/in-directly

-	to eNB or gNB

-	to syncRef UE that is synchronized to gNB or eNB directly

-	to syncRef UE that is synchronized to gNB or eNB in-directly

-	to syncRef UE that has the lowest priority

-	From syncRef UE that is synchronized to gNB/eNB directly

-	to GNSS

-	to syncRef UE that is synchronized to GNSS directly/in-directly

-	to gNB/eNB

-	to syncRef UE that is synchronized to gNB/eNB directly

-	to syncRef UE that is synchronized to gNB/eNB in-directly

-	to syncRef UE that has the lowest priority

-	From syncRef UE that is synchronized to gNB/eNB in-directly

-	to GNSS

-	to syncRef UE that is synchronized to GNSS directly/in-directly

-	to gNB/eNB

-	to syncRef UE that is synchronized to gNB/eNB directly

-	to syncRef UE that is synchronized to gNB/eNB in-directly

-	to syncRef UE that has the lowest priority

-	From syncRef UE that has the lowest priority

-	to GNSS

-	to syncRef UE that is synchronized to GNSS directly

-	to syncRef UE that is synchronized to GNSS in-directly

-	to gNB/eNB

-	to syncRef UE that is synchronized to gNB/eNB directly

-	to syncRef UE that is synchronized to gNB/eNB in-directly

-	to syncRef UE that has the lowest priority

UE is allowed to interruption any V2X sidelink signals including PSSCH, PSCCH, PSBCH, PSFCH and SLSS signals.

12.7.3	Interruptions to WAN due to switching between E-UTRA V2X Sidelink and NR V2X Sidelink

This sub-clause contains the requirements related to the interruptions on the PCell/serving cell due to switching between E-UTRA V2X sidelink and NR V2X sidelink transmissions on a dedicated carrier. It is applicable for UE capable of both NR V2X sidelink and E-UTRA V2X sidelink transmissions in TDM-ed manner.

When a UE capable of switching between E-UTRA V2X sidelink and NR V2X sidelink, the UE is allowed an interruption of up to the duration shown in table 12.7.3-1 on the PCell/serving cell during the E-UTRA V2X sidelink and NR V2X sidelink switch.

This interruption is for both uplink and downlink of the PCell/serving cell.

Table 12.7.3-1: Interruption length due to switching between E-UTRA V2X and NR V2X

|  | Slot length (ms) | Interruption length (number of slots) |
| --- | --- | --- |
| 0 | 1 | 2 |
| 1 | 0.5 | 2 |
| 2 | 0.25 | 2 |
| 3 | 0.125 | 3 |

12.7.4	Interruptions to WAN at transitions between active and non-active during SL-DRX

Interruption on PCell/serving cell if configured due to V2X transitions between active and non-active during SL-DRX when PCell/serving cell is in non-DRX are allowed with up to 1% probability of missed ACK/NACK when the configured SL-DRX cycle is less than 640 ms, and 0.625% probability of missed ACK/NACK is allowed when the configured SL-DRX cycle is 640 ms or longer. When multiple SL-DRX cycles are configured, a shortest SL-DRX cycle is applied. Each interruption shall not exceed X slot as defined in table 12.7.4-1.

Table 12.7.4-1: Interruption length X at transition between active and non-active during SL-DRX

|  | NR Slot | Interruption length X (slots) |  |
| --- | --- | --- | --- |
|  | length (ms) | Sync | Async |
| 0 | 1 | 1 | 2 |
| 1 | 0.5 | 1 | 2 |
| 2 | 0.25 | 3 |  |

Editor’s note : The followings will be treated in next meeting.

FFS on interruptions for the case when NR is in DRX and SL is in SL-DRX.

FFS whether interruptions are applicable for the following WAN conditions and impact on SL transitions between active and non-active SL DRX if interruptions are not applicable:

- reception of paging

- reception of system information

- while onDurationTimer is running

- while RLF timer is running

- while UE is performing CBD

12.7.5	Interruptions to V2X sidelink at transitions between active and non-active during DRX

Interruption on V2X sidelink if configured due to PCell transitions between active and non-active during DRX when V2X is in non SL-DRX are allowed with up to 1% probability of missed ACK/NACK when the configured DRX cycle is less than 640 ms, and 0.625% probability of missed ACK/NACK is allowed when the configured DRX cycle is 640 ms or longer. It is only applied when HARQ process on V2X sidelink is supported. Each interruption shall not exceed X slot as defined in table 12.7.5-1.

Table 12.7.5-1: Interruption length X at transition between active and non-active during DRX

|  | NR V2X Slot | Interruption length X (slots) |  |
| --- | --- | --- | --- |
|  | length (ms) | Sync | Async |
| 0 | 1 | 1 | 2 |
| 1 | 0.5 | 1 | 2 |
| 2 | 0.25 | 3 |  |

When PCell is DRX and V2X is in SL-DRX, no interruption is allowed.

12.7.6	Interruptions to V2X sidelink due to Active BWP switching Requirement

This clause contains the requirements related to the interruptions on the V2X sidelink due to BWP switch in FDM based intra-band concurrent V2X operation.

The requirements in clause 8.2.2.2.5 shall apply. The interrupted X slot is defined in Table 12.7.6-1.

Table 12.7.6-1: Interruption length X

|  | NR V2X Slot | Interruption length X (slots) |
| --- | --- | --- |
|  | length (ms) |  |
| 0 | 1 | 1 |
| 1 | 0.5 | 1 |
| 2 | 0.25 | 3 |

< END OF CHANGE #2 >