3GPP TSG-RAN WG4 Meeting #101-bis-e		R4-2203008

E-meeting, 17 January – 27 January, 2021

Agenda item:	6.12.2.1.1

Source:	Intel Corporation

Title:	WF on general and PDSCH demodulation requirements for inter-cell interference MMSE-IRC

Document for:	Approval

# Introduction

This document provides the way forward on general and PDSCH demodulation requirements for inter-cell interference MMSE-IRC.

# Common test parameters for scenario 1

## Network type

- Previous meeting status

  - Synchronized network is baseline assumption, interested companies are encouraged to bring results for async scenario under the baseline assumption of MMSE-IRC receiver.

    - For asynchronized scenario, reusing LTE configuration of time/frequency offset as starting point.

- Way forward

  - Option 1: Only consider synchronized network

  - Option 2: Include FDD asynchronized network type with applicability rule:

    - For 2Rx/4Rx UE that only support FDD mode, we can have 1 HomNet test for aync scenario and 1 HetNet test for sync scenario.

    - For 2Rx/4Rx UE that support both FDD and TDD modes, we can have 1 test for HomNet FDD async and 1 test for HetNet TDD sync respectively.

## SSB configuration

- Previous meeting status

  - Option 1: Use SSB Option 1 for all test

  - Option 2: Use SSB Option 2 for all test

  - Option 3: Use different assumptions for different deployment scenarios:

    - Option 3A: SSB Option 1 for homogeneous deployment assumptions and SSB Option 2 for heterogeneous deployment assumptions

    - Option 3B: SSB Option 2 for homogeneous deployment assumptions and SSB Option 1 for heterogeneous deployment assumptions

- Way forward

  - Option 1: Use SSB Option 1 (All SSBs are in the same time/frequency resources) for all test

  - Option 2: Use SSB Option 2 (Serving cell SSB and interference cell(s) SSB(s) are in the different time/frequency resources) for all test

  - Option 3: Use different assumptions for different deployment scenarios: SSB Option 1 for homogeneous deployment assumptions and SSB Option 2 for heterogeneous deployment assumptions

# Target PDSCH parameters for scenario 1

## MCS

- Previous meeting status

  - Use MCS 13 for homogeneous deployment assumptions

  - Use MCS 13 as baseline for heterogeneous deployment assumptions

    - It can be revised in the next RAN4 meeting in case any technical issue is raised

- Way forward

  - Use MCS 13 for requirements definition with homogeneous and heterogeneous deployment assumptions

# Interference model for scenario 1

## INR values for Homogeneous deployment assumptions and synchronous network

- Previous meeting status

  - Option 1: INRs 7.77 and 2.29 dB in case of 2 interference cells and INR 5.49 dB in case of 1 interference cell

  - Option 2: INRs 5.43 and -1.50 dB in case of 2 interference cells and INR 3.1 dB in case of 1 interference cell

- Way forward

  - INRs 7.77 and 2.29 dB in case of 2 interference cells and INR 5.49 dB in case of 1 interference cell

## INR values for HetNet deployment assumptions

- Previous meeting status

  - Baseline option: INRs 11.39 and 5.45 dB in case of 2 interference cells and INR 4.84 dB in case of 1 interference cell.

  - Baseline option can be updated in case technical issue will be observed

- Way forward

  - Use INRs 11.39 and 5.45 dB in case of 2 interference cells are modelled

  - Select one of the following options for scenario with 1 interference cell

    - Option 1: INR 4.84 dB.

    - Option 2: INR 7.58 dB

## Number of explicitly modeled interference cells

- Previous meeting status

  - Option 1: 1 interference cell for all tests

  - Option 2: 2 interference cells for all tests

  - Option 3: Use different assumptions for different deployment scenarios:

    - Option 3A: 2 interference cells for homogeneous deployment assumptions and 1 interference cell for heterogeneous deployment assumptions

    - Option 3B: 1 interference cell for homogeneous deployment assumptions and 2 interference cells for heterogeneous deployment assumptions

- Way forward

  - Option 1: 1 interference cell for all tests

  - Option 2: 2 interference cells for all tests

  - Option 3: Use different assumptions for different deployment scenarios: 2 interference cells for homogeneous deployment assumptions and 1 interference cell for heterogeneous deployment assumptions

## Time and frequency offsets for synchronized network with 30 kHz SCS

- Previous meeting status

  - Option 1: The serving cell is 3 us for interfering cell 1 and -1 us for interfering cell 2 (in case modeled)

  - Option 2: The serving cell is 1 us for interfering cell 1 and -0.25 us for interfering cell 2 (in case modeled)

  - Other options are not precluded

- Way forward

  - Companies are encouraged to bring simulation results for both options next meeting to identify whether significant performance difference can be observed

## INR values for asynchronous network (if introduced)

- Way forward

  - Option 1: DIP1= -2.23dB and DIP2= -8.06dB (INR1=3.87dB and INR2= -1.96dB)

  - Option 2: Reuse INR values options from sync scenario

  - Other options are not precluded

## Time and frequency offsets for asynchronous network (if introduced)

- Way forward

  - Use 0.33ms for interference cell 1 and 0.67ms for interference 2 as baseline in case requirements will be introduced

## Interference modelling in PDCCH region

- Way forward

  - Option 1: NR interference model to have unallocated RE’s in control region filled with QPSK randomly modulated symbols with random precoding for the number of antenna ports in the requirement scenario.

  - Option 2: No interference signal in PDCCH region

  - Option 3: Assume PDCCH transmission from interference cells

# Receiver assumptions for scenario 1

## TRS-IC/IM processing

- Previous meeting status

  - Further discuss whether to add in the simulation assumptions the clarification that no TRS interference cancellation/mitigation is considered for inter-cell MMSE-IRC requirements definition

  - No such clarification will be added in the TS 38.101-4

  - Interested companies are encouraged to check with performance for scenarios with and without TRS-IC/IM

- Way forward

  - Keep previous meeting agreement

# Requirements for scenario 2

## Definition of requirements

- Way forward

  - Further discuss whether to define requirements for scenario 2

  - For information, interested companies can check proposals 8-10 from R4-2200512 and comments in Sections 1.3.1.7 – 1.3.1.10 from R4-2203109

# UE feature list, capability signalling and release independence

- Way forward

  - Option 1: No need to introduce new UE feature, requirements release independent from Rel-15

  - Option 2: Optional without UE capability signalling and applicable from Rel-17

  - Option 3: Optional without UE capability signalling and applicable from Rel-15

  - Option 3a: Optional without UE capability signalling for Rel-15/16 UE and mandatory from Rel-17

  - RAN4 will make decision on RAN4#102-e meeting with above options

# References

- R4-2120707 “Way Forward on general and PDSCH demodulation requirements for inter-cell interference MMSE-IRC”, Intel Corporation, RAN4 #101-e, November 2021.

- R4-2115730 “Way Forward on general and PDSCH demodulation requirements for inter-cell interference MMSE-IRC”, Intel Corporation, RAN4 #100-e, August 2021.

- R4-2108664 “Way Forward on general and PDSCH demodulation requirements for inter-cell interference MMSE-IRC”, Intel Corporation, RAN4 #99-e, May 2021.